10 August, 2017

The Manager, Spectrum Planning Section

Spectrum Planning and Engineering Branch

Communications Infrastructure Division

Australian Communications and Media Authority

PO Box 78

BELCONNEN ACT 2616

COMMENTS ON IFC 9/2017

FUTURE USE OF THE 3.6 GHz BAND

In response to the release of the second options paper by the ACMA, DB Telecommunications Pty Ltd is pleased to be able to offer comment on the future use of the 3.6 GHz band.

As an ACMA Accredited Person, David Britt of DB Telecommunications has performed a significant number of assignments for 3.6 GHz BWA services in Tasmania and remote parts of Western Australia. This has provided DB Telecommunications with a good insight into the demand for 3.6 GHz services in these regions and the issues confronting incumbent and potential users of the band.

RESPONSE TO QUESTIONS RAISED IN THE FUTURE USE OF THE 3.6 GHz BAND OPTIONS PAPER

- Should the 3.6 GHz band be progressed from the preliminary replanning stage to the re-farming stage in the ACMA’s process for considering additional spectrum for MBB services? Why/Why not?

- Given current international developments potentially targeting the 3.6 GHz band for the initial deployment of 5G services, some re-farming of the 3.6 GHz band seems inevitable.

- The challenge for the ACMA is to find a way to achieve this in a timeframe and manner that does not unfairly disadvantage incumbent licensees providing a range of services from wireless internet services to satellite earth stations, while seeking to maximise the economic value of the spectrum.

- Do the areas identified in this analysis cover the likely areas of high demand for access to the 3.6 GHz band? Would smaller or larger areas be more appropriate? Why?

- DB Telecommunications believes that the areas depicted as Area 1probably represent the areas of highest demand for access to the 3.6 GHz band. There would also be some large regional towns in Area 2 that are likely to be areas of high demand, certainly from incumbent apparatus licensees. The major cities like Hobart, Launceston, Devonport and Burnie in Tasmania are likely to be areas of high demand, certainly from incumbent apparatus licensees.

- If any part of the 3.6 GHz band is re-allocated for the issue of spectrum licences is seven years a suitable re-allocation period? If not, what period of time would be appropriate?

- 7 years is probably a reasonable timeframe for most incumbent licensees to have recouped most of their current investment, but some WISPs operating in remote areas serving small population densities may require a longer period of time in which to recoup their investment.

- In its presentation at the recent 3.6 GHz band tune-up, Telstra had indicated that it had proposed a 10 year timeframe for the re-allocation of incumbents and VHA had indicated its desire to be flexible with incumbents where possible. These extended relocation times would be welcomed by incumbent licensees.

- A more vexing question facing incumbent licensees is that with increasing pressure to provide more spectrum licenced MBB and other services, plus a very limited range of options in other bands, will there be any viable spectrum for them to relocate to in 7-10 years’ time?

- Should different re-allocation periods be considered for different areas? For example, should a longer period be considered for services outside Area 1?

- There would appear to be some scope to offer longer re-allocation periods in parts of Area 2 where there may be no strong competing demand for spectrum, or where the rollout of MBB services is likely to occur over a more extended timeframe. This could possibly be dealt with through agreements between incumbent apparatus licensees and spectrum licensees.

- In many remote parts of Area 3 there would probably be scope to offer incumbent licensees extended re-allocation times, possibly on a rolling 5 year window basis where there is no competing demand from spectrum licensed services. However, also within Area 3 there would be some suburban type areas in smaller states like Tasmania where there is likely to be strong competing demand between incumbent licensees and spectrum licensees within the initial 7 year re-allocation period.

- Are these guidelines appropriate? Why?

- Due to time constraints, DB Telecommunications has not really had the time to properly consider the appropriateness of these guidelines.

- Are there any other issues that affect the usability of an area-wide licence that should be taken into account when defining the licence area?

- DB Telecommunications has no specific comments on this particular issue.

- If point-to-point licences are affected by replanning activities in the 3.6 GHz band, are the options identified for point-to-point licences suitable? Are there any alternative options that should be considered?

- The use of the 3.6 GHz band for fixed point-to-point services appears fairly limited, so the proposed arrangements would be considered adequate as there are many fixed point-to-point microwave bands that incumbent licensees could migrate to.

- Is the 5.6 GHz band a viable option for wireless broadband systems?

- DB Telecommunications does not believe that the 5.6 GHz band presents a viable option for wireless broadband systems for the following reasons.

  - The 5.6 GHz band offers BWA licensees only 40 MHz of spectrum as opposed to the 125 MHz in the 3.6 GHz band, which is likely to result in a high number of displaced licensees in some areas at the end of the 7 year re-allocation period. In some regional areas where demand for the 125 MHz of 3.6 GHz spectrum already exceeds supply, the proposed 40 MHz of 5.6 GHz spectrum would clearly be inadequate.

  - If all incumbent licensees were to be accommodated in the 40 MHz of available bandwidth, they would need to employ significantly more spectrally efficient technologies than they currently use and/or use significantly smaller network cell sizes, which would significantly increase their re-allocation costs.

  - The presence of BOM radar systems in the 5.6 GHz band is likely to limit the use of the band for BWA services in areas adjacent to the BOM radars. This is likely to include some urban areas in various cities on the mainland and parts of Tasmania.

- Under what circumstances should apparatus- and class-licensed arrangements be considered for the 5.6 GHz band?

- Under no circumstances should apparatus and class licenced services share the 5.6 GHz band. In urban areas where apparatus licensed spectrum for BWA services is generally unavailable, many wireless broadband operators have been forced to use 5.8 GHz class licensed spectrum to provide services.

- Due to the largely unregulated nature of the class licensed spectrum, many operators have been experiencing rising levels of interference is rising and have struggled to maintain proper carrier grade service standards.

- If apparatus licensed and class licensed services were to share the 5.6 GHz band, the apparatus licensed service operators are likely to see their grade of service significantly degraded by the influx of unregulated class licensed equipment and services over time.

- If apparatus licensing arrangements are developed for wireless broadband systems in the 5.6 GHz band, are the notional arrangements proposed in Appendix 3 suitable?

- Bandwidth – As noted above the proposed 40 MHz of bandwidth is likely to be inadequate in many regional areas but may be adequate for some of the more remote areas.

- Areas – The site based apparatus licensing for point-to-multipoint services is likely to be most viable in remote areas and smaller regional areas. It is not likely to be viable in metro areas, most likely due to supply outstripping demand and the risk of interference from unregulated class licensed equipment from the adjacent 5.8 GHz band.

- Power – The ACMA needs to clarify whether the proposed power of 4 W is TX power or EIRP. If the proposed power limit is 4 W EIRP this is likely to significantly reduce the coverage areas of incumbent networks, compared to coverage possible under the RALI FX 19 assignment rules which currently apply to the 3.6 GHz band. The use of lower powers would necessitate greater investment in infrastructure by network operators.

- While smaller cell sizes might suit a high capacity MBB system in urban areas, the lower powers would probably be less well suited to wireless broadband installations in regional areas, where operators are looking to service lower densities of subscribers spread over a larger geographic area.

- Coordination – DB Telecommunications believes that the existing RALI FX 19 could be adapted to accommodate the assignment of apparatus licensed point-to-multipoint services in the 5.6 GHz band.

- Pricing – DB Telecommunications believes that the licence tax for apparatus licensed services in the 5.6 GHz band should remain lower than that payable in the 3.6 GHz band, due to the inferior technical performance that network operators are likely to experience operating in the 5.6 GHz band.

- 3.6 GHz band point-to-multipoint priority – DB telecommunications agrees that licensees displaced from the 3.6 GHz band should be given priority access to any alternative band allocations, with new licensees prevented from accessing the band for a period of approximately 6 months.

- That initial priority access period needs to be for a finite period and be well managed from an ACMA perspective to avoid the similar issues to those experienced with the priority system that applied to the issue of apparatus licensed 1800 MHz LTE services in remote areas.

- Certainty – A reasonable degree of certainty of tenure is required to enable network operators to be confident to make the necessary investment in new network infrastructure in moving to the 5.8 GHz band or any alternative frequency.

- DB Telecommunications believes that the in-principle certainty of tenure should be at least 10 years.

- There was a suggestion that retention of 25 MHz of 3.6 GHz spectrum might be a reasonable outcome for point-to-multipoint users. This is only likely to be of significant benefit in smaller regional areas with perhaps one or two incumbents. In other areas 25 MHz of spectrum would be totally inadequate for incumbents unless they employed significantly more spectrally efficient technologies than they currently use.

- If point-to-multipoint licences are affected by replanning activities in the 3.6 GHz band, are the alternative options identified suitable? Are there any alternative options that should be considered?

- Overall DB Telecommunications considers that the alternative options proposed are likely to be inadequate for incumbent point-to-multipoint licensees with many being unable to continue offering broadband wireless services in many areas and providing no viable options for apparatus licensed point-to-multipoint to be offered in urban areas.

- With current international developments potentially targeting the 3.6 GHz band for the initial deployment of 5G services, the ACMA should consider whether this is likely to lead to some existing spectrum licensees in the 3.4 GHz band migrating to the 3.6 GHz band to deploy 5G services. This view might be supported by the ACMA’s suggestion that with the current sizes of metropolitan areas, 3.4 GHz spectrum licence bands are not ideal to deploy dense wide-area TDD MBB services.

- If such a migration were to occur, this might allow for an expansion of the existing apparatus licensing arrangements applying to the 3.4 GHz band in some regional areas, providing another option for apparatus licensees displaced by the 3.6 GHz band. It may also provide some scope to introduce some apparatus based point-to multipoint licensing in urban areas, where DB Telecommunications noted in its response to the ACMA’s previous discussion paper, there is currently an unsatisfied demand for access to spectrum.

- The ACMA seeks comment on the suitability of the current west coast earth station protection zone located near Mingenew, WA, for long-term satellite service use. Are the current regulatory arrangements effective?

- DB Telecommunications considers that the existing protective measures are probably adequate.

- In the event FSS earth stations are affected by replanning activities in the 3.6 GHz band, the ACMA seeks comment on:

- Any issues surrounding the development and establishment of an east coast earth station protection zone; particularly on what factors would be necessary to make it an attractive option for earth station operations.

- Whether there are any views on potential candidate locations to consider.

- Whether there should there be more than one earth station protection zone on the east and west coasts of Australia.

- If the identification of a central Australia earth station zone should be considered.

- DB Telecommunications has no specific comments on this particular issue. Those in the satellite industry are better qualified to comment.

- Are the approaches for amateurs, radiolocation services, class licensed devices and TVRO systems suitable?

- DB Telecommunications has no specific comments on this particular issue.

- Are there any other options for incumbent services, not identified in this paper, which should be considered?

- Apart from the suggestion of exploring opportunities for additional apparatus licensed spectrum in the 3.4 GHz band as some spectrum licensees migrate to the 3.6 GHz band, DB Telecommunications has no additional suggestions in relation to this point.

- Should any of the sharing arrangements discussed in this section be considered for implementation in the 3.6 GHz band? Why or why not?

- Given the likely strong competing demand between incumbent apparatus licensees and spectrum licensees for the 125 MHz of available spectrum in the 3.6 GHz band, there is probably little practical scope for sharing of the spectrum in urban and some of the larger regional towns. There is probably greater scope for sharing of spectrum in some of the less densely populated regional areas, as long as interference management can be adequately addressed.

- Negotiated agreements between spectrum licensees and incumbent apparatus probably represent the best opportunity for the sharing of 3.6 GHz spectrum going forward.

- Are there any other sharing arrangements that should be considered?

- DB Telecommunications has no specific comments on this particular issue.

- Are there any other replanning options that should be considered?

- DB Telecommunications has no specific comments on this particular issue.

- Which replanning option should be implemented in the band? Why?

- DB Telecommunications believes that Option 3a probably represents the best overall compromise as it will allow the ACMA to derive maximum value for the 3.6 GHz spectrum in densely populated urban areas and allowing carriers to proceed with the deployment of 5G services, while protecting the viability of incumbent wireless broadband operators operating outside of Area 1.

- Within Option 3a, some special treatment is probably warranted to protect the interests of incumbent FSS operators in areas such as Sydney and Perth, at least in the short to medium term.

- In the event an area-wide licensing option is implemented, in which of the defined areas (that is, Area 1, 2, 3 and Australia-wide as defined in Appendix 6) should these arrangements be implemented? Are the current area definitions appropriate? If not, what area should be defined?

- If an area-wide licensing option was adopted, DB Telecommunications believes that it would be most appropriate to restrict that to Area 1, as it is less likely to displace a large number of incumbent licensees in the 3.6 GHz band or have the utility of the area-wide licences adversely impacted by a large number of incumbent apparatus licensees.

- In terms of the current area definitions, because of its relatively small geographic area, DB Telecommunications believes that it may be appropriate to treat Tasmania as multiple areas rather than being part of a single area (Area 3). There will no doubt be strong demand for 3.6 GHz spectrum in the major cities like Hobart, Launceston, Devonport and to a lesser extent Burnie, but outside of those cities the rest of Tasmania could perhaps be classified as a remote area which may help to minimise the displacement of incumbent licensees.

- If Option 4a is implemented, what frequencies and areas should be re-allocated for the issue of spectrum licences? How much spectrum should remain subject to site-based apparatus licensing arrangements? Should different amounts be considered in different areas?

- DB Telecommunications does not consider Option 4a to be a highly viable option. If Option 4a was adopted, DB Telecommunications believes that the hybrid approach is only likely to be viable in Areas 2 and 3. The amount of spectrum retained for apparatus licensed services would probably need to be at least 50 MHz in many regional areas and even that may not be adequate in many areas with multiple incumbent licensees unless they employ significantly more spectrally efficient technologies than they currently use.

- If Option 4b is implemented, what frequencies and areas (that is, incumbent apparatus licence services) should remain subject to site-based apparatus licensing arrangements?

- DB Telecommunications believes that assessment of the viability of Option 4b would require a much more detailed study of the incumbent licensees and their spectrum holdings in the various areas.

- Comment is sought on the ACMA’s preferred option (Option 3c) for the 3.6 GHz band.

- DB Telecommunications is not in favour of the preferred option as it has the potential to displace a significant of incumbent licensed services without providing them with viable alternative spectrum at this time.

- The proposed use of the 5.6 GHz band is not considered a viable technical option for these incumbent licensees for reasons discussed in earlier sections of this response.

If you would like additional information or wish to discuss any aspect of my submission, please do not hesitate to contact me on (03) 9331 3170 or by email dbritt@dbtelecomm.com.au.

Yours sincerely,

David Britt

Director