Radiocommunications site data requirements

August 2012

Canberra Purple Building Benjamin Offices Chan Street Belconnen ACT PO Box 78 Belconnen ACT 2616 T +61 2 6219 5555 F +61 2 6219 5353; Melbourne Level 44 Melbourne Central Tower 360 Elizabeth Street Melbourne VIC PO Box 13112 Law Courts Melbourne VIC 8010 T +61 3 9963 6800 F +61 3 9963 6899; Sydney Level 5 The Bay Centre 65 Pirrama Road Pyrmont NSW PO Box Q500 Queen Victoria Building NSW 1230 T +61 2 9334 7700 1800 226 667 F +61 2 9334 7799.

© Commonwealth of Australia 2012 This work is copyright. Apart from any use as permitted under the Copyright Act 1968, no part may be reproduced by any process without prior written permission from the Commonwealth. Requests and inquiries concerning reproduction and rights should be addressed to the Manager, Editorial Services, Australian Communications and Media Authority, PO Box 13112 Law Courts, Melbourne Vic 8010. Published by the Australian Communications and Media Authority

1	Introduction	1

1.1	Purpose	1

1.2	Background	1

1.3.1	Implementation dates	2

1.3.2	Interim Arrangements	2

1.4	Aligning existing site records with new requirements	2

1.5	Questions and advice on application of requirements	3

1.6	Requests to revise site name examples and options for site configuration	3

2	Objectives for radiocommunications site data rules		4

3	What is a radiocommunications site?	5

3.1	Explanation of radiocommunications site terms	5

4	Site coordinates	7

4.1	Datum used for site coordinates	7

4.2	Accuracy of site coordinates	7

4.3	Determining site coordinates using a surveyor or GPS receiver	7

4.3.1	Correct GPS usage	8

4.4	Measurements of accuracy and quality of site coordinates	9

4.5	The RADCOM 10m rule	9

5	Site name	10

6	Site configuration	11

7	What site is that antenna on?	12

7.1	Points to consider when determining site location and 
coordinates	12

8	Site name and site configuration examples	14

8.1	Individual towers at a site separated by more than  10m	14

8.2	Localities, town names and geographic features	14

8.3	Building with a tower at each end of the building	15

8.4	Sites on sides of buildings	15

8.5	Buildings with towers and microwave dishes that are grouped together	16

8.6	Nominal location terminology for sites on buildings	16

9	Responsibilities for site data	17

9.1	Overview	17

9.2	Licensees	17

9.3	Accredited persons	17

9.4	The ACMA	17

9.4.1	Frequency assignment and licensing	17

9.4.2	Auditing and quality control of site data	17

10	Entering radiocommunications site data into RADCOM	18

Mandatory	18

Optional		18

11	Online site data management	19

11.1	Online site creation	19

11.2	Online site deletion	19

11.3	Online site updating	20

Appendices	21

Appendix 1: Site data conversion	22

Purpose		22

Background and explanation	22

Background to GDA	22

GDA and AGD	23

Datum data and the ACMA	23

Appendix 2: Site data entry requirements	24

Mandatory	24

# 1	Introduction

## 1.1	Purpose

The purpose of this document is to specify requirements for creating and managing radiocommunications site data in the Australian Communications and Media Authority (the ACMA) radiocommunications licensing database (RADCOM) and via the accredited person’s online submission system.

The document replaces the Business Operating Procedures “Ensure that consistent site coordinate standards and naming conventions are maintained for sites in RADCOM” and “Site Data Conversion”.

## 1.2	Background

In September 2002 the then ACA proposed revisions to the ACA’s radiocommunications site data requirements. The main objective of the proposed revisions was to develop a set of practical clearly defined requirements for site data that would minimise site data errors and improve the quality of recorded data.

In parallel, the then ACA also commenced discussions with accredited persons on proposed requirements to collect radiocommunications site data using an online submission system.

Finalised requirements are contained in this document. Main changes from previous arrangements are:

Recording of site name information in three fields (building/tower name, street address and town/geographic location). Previously this information was recorded in one field. Providing three fields better supports use with computer applications and allows specification of optional and mandatory data.

Collection of new information “Coordinate Source” – to describe how site coordinates were determined (eg map, GPS, or surveyor) and “Configuration” to provide a description layout of antennas on a site (eg tower centre, rooftop centre)

Inclusion of site manager email and site contact person email fields

Inclusion of more examples on application of site naming and selection requirements.

1.3	Implementation dates and interim arrangements

Under a phased implementation arrangement ACMA systems will be progressively updated to support the following new fields:

radiocommunications site data (ie, building/tower name;

street address;

geographic location;

coordination source;

configuration;

site manager email; and

contact person email.

See the Background section for further detail. In the meantime, the interim arrangements set out in paragraph 1.3.2 will apply

### 1.3.1	Implementation dates

Accredited persons online submission system

Inclusion of site management functionality expected by 1st quarter 2005.

ACMA staff RADCOM interface

Inclusion of three fields for site name (building/tower name, street address, and geographic location) site configuration and coordinate source expected by 1st quarter 2005, at the same time as the update to the accredited person’s online system.

RADCOM CDROM and Daily update service

No changes planned

New site fields will support backwards compatibility and changes will not impact on RADCOM CDROM and Daily Update Service

Users will be consulted before any changes are made.

Register of Radiocommunications Licences

Timing yet to be determined.

New site fields will support backwards compatibility and changes will not impact the on Register.

ACMA RADCOM Database

All new fields to be included in database by 1st quarter 2005, at the same time as update to accredited persons online system to include the site management facility.

Access limited to ACMA RADCOM database administrators.

### 1.3.2	Interim Arrangements

From now until the inclusion of the site management functionality in the accredited person’s online submission system:

The requirements specified in this document are to apply

Sites in RADCOM should be created with two spaces between site name elements to assist migration of site name to new arrangements (ie two spaces between building/tower name, street address and town/geographic location).

## 1.4	Aligning existing site records with new requirements

With around 50,000 existing sites in the RADCOM database it will take some years before existing sites can be updated to reflect the revised requirements outlined in this paper. Initially, site information will be updated on an opportunity basis (eg as a part of ACMA site audit activities). Once the revised requirements are well established the ACMA will look at whether there is a need to develop a specific site update program.

## 1.5	Questions and advice on application of requirements

For advice on the application of this document or matters that appear to fall outside the express policy please contact the ACMA’s Radiocommunications Licensing and Telecommunications Deployment Section (lais@acma.gov.au).

## 1.6	Requests to revise site name examples and options for site configuration

Requests to revise site naming and options for site configuration and coordinate source should be sent to the ACMA’s Radiocommunications Licensing and Telecommunications Deployment Section (lais@acma.gov.au).

# 2	Objectives for radiocommunications site data rules

The radiocommunications site data requirements outlined in this document are intended to meet the following objectives:

Be practical to use

Be simple and easy to understand in order to facilitate accurate and consistent application

Improve the standard of recorded site data over time

Support the use of sites for radiocommunications apparatus licensing and spectrum licensing device registration using common site data criteria

Support more efficient frequency coordination processes through improved and consistent data

Support site data auditing against clearly defined criteria.

# 3	What is a radiocommunications site?

A radiocommunications site records the location where a licensee is authorised to operate a radiocommunications station. A station includes transmitters, receivers and antennas. However, it is the station’s antenna location that is used to define the site where the station is authorised to operate, not the transmitter location, which is likely to be in a different location to the antenna (for example 20-100m away).

Conceptually, the site where the station is authorised to operate is a point on the ground above which that station’s antenna is located. Usually, the antenna is located at least 2m above the ground, via a supporting structure (eg, a pole, a pole on a roof, a tower or a tower on a building). The height of the antenna above ground (which includes the support structure) is known as the antenna height. The height of the ground above sea level is known as the site height. The site height does not include the height of the antenna support structure.

The site recorded on a radiocommunications licence may not be the exact location of an antenna. Often a site is the representation of the location of a number of antennas.

## 3.1	Explanation of radiocommunications site terms

The following terms are used in this document to describe radiocommunications sites in RADCOM.

Antenna location: The location of the phase centre of the antenna. Generally speaking an antenna phase centre is the point where an electromagnetic wave would originate from if the antenna was transmitting.

In practice, antenna location is determined and represented using a lay interpretation of phase centre that is more understandable by a wider audience. Some examples are provided below.

Antenna Type; Lay Definition of Antenna Location. Yagi; Mount point on antenna support structure. Microwave dish; Centre of dish. Dipole array; Centre point of array. Steerable antenna; Mounting point about which the antenna can rotate.

Commonly, a site is the representation of the location of a number of antennas. In such cases a site may represent the antenna support structure (for example, centre of a tower) or a point about which a number of antennas are located.

What a site is representing can be determined from the “site layout/configuration” field. Note that the site layout/configuration field was only introduced in July 2004 and is not recorded for most sites prior to that date.

Site location: The point on the ground that is directly below the antenna location.

Site coordinates: The coordinates of the site location. Coordinates may be in geographic (latitude and longitude) or Cartesian (zone, easting, northing) format.

Datum: A reference point for coordinate systems. For more information see the coordinates, datums & ellipsoids information of the geodesy section of the Geosciences Australia website.

Site elevation: The height of the ground above sea level at the site (with reference to the Australian Height Datum). This does not include the height of any antenna support structure (that is, it does not include building height or tower height).

Antenna height: The height of the antenna location (the phase centre) above ground at the site. Antenna height includes the height of any antenna support structure (that is, includes building height or tower height).

Site layout/configuration: This field is used to describe what the “site” is representing and how antennas are located on the site. For more information see Section 6 “Site configuration”.

Site Name: Records the building/tower name, street address and town/geographic location of the site. All sites should have town/geographic location specified. Only one of building/tower name and street address needs to be specified, though both have been specified for a large number of sites.

Precision: This field records the accuracy of the recorded site coordinates in RADCOM. That is, the maximum expected error between the recorded site coordinates in RADCOM and the physical site location. Coordinate precision is dependent on the method used to determine site coordinates. For example, coordinates determined using a registered surveyor should be more accurate than those determined by using a 1:100,000 scale map.

Coordinate source: Records how the site was determined (eg GPS reading or map). Combined with the precision field it provides a guide to the quality and reliability of coordinates recorded in RADCOM.

# 4	Site coordinates

## 4.1	Datum used for site coordinates

The ACMA database uses the 1966 Australian Geodetic Datum (AGD66) as the datum for its site information. All site information is stored using this datum, and printed to licence documentation where applicable. Sites supplied in 1984 Australian Geodetic Datum (AGD84) and Geocentric Datum of Australia 1994 (GDA94) are converted and stored in AGD66.

The ACMA is planning to convert AGD66 coordinates within ACMA database to GDA94. In preparation for that event all coordinates supplied in GDA94 are stored in both AGD66 and GDA94.

For more information see the Appendix 1 “Site data conversion”.

## 4.2	Accuracy of site coordinates

The aim is to record radiocommunications site coordinates that are accurate to within 10 metres (equivalent to approximately 0.3 of a second of latitude and longitude) of the site location.

Coordinate accuracy may be relaxed in rural areas (ie low fee density areas) to that provided by available maps, but not exceeding 100 metres.

In some cases (for example, on a large tower) the recorded location will be greater than 10m away from the physical antenna location. In those cases the recorded coordinates should be within 10m of the location described by the combination of site name and site configuration fields.

For example:

If:

Site Name is: North Tower 350 Elizabeth Street MELBOURNE

Configuration is: Tower Centre

Then:

Recorded coordinates should be within 10m of the centre of the north tower at 350 Elizabeth Street MELBOURNE.

The use of common site coordinates to represent multiple antenna locations is directly dependent on having coordinates accurate to within 10 metres. In cases where coordinate accuracy is within 100m without an inspection of the site it would be difficult to determine whether common site coordinates can be used. For example, two antennas recorded on the one site could be on one tower or on different towers separated by up to 200 meters (worst case).

## 4.3	Determining site coordinates using a surveyor or GPS receiver

The ACMA prefers that site coordinates are determined by using:

a surveyor (a person who holds a licence or registration as a Surveyor in an Australian State or Territory or in New Zealand) or

a GPS measurement of adequate quality that has an accuracy of 10 metres or better.

In some cases in high and medium density fee areas (Adelaide, Brisbane/Gold Coast, Melbourne/Geelong, Perth, Newcastle, and Sydney/Wollongong) the use of a surveyor or GPS may not be practical. For example

on small sites (eg antenna height less than 10m) it may not be cost effective to use a surveyor

the local environment may limit the achievable accuracy and confidence of GPS measurements (the degree of limitation is largely dependent on the type of GPS being used)

high accuracy maps may be available (eg 1:1000).

In such cases site data should be provided with advice about the accuracy and reliability of the site coordinates and information on how the coordinates were determined.

In rural areas (low density fee areas), where it may not be feasible to determine site coordinates using a surveyor or GPS measurement with the required accuracy and reliability, alternative methods (eg maps or less accurate GPS measurements) may be used provided that the resulting accuracy does not exceed 100 metres.

## 4.3.1	Correct GPS usage

Care must be taken to ensure that the position being recorded with a GPS receiver is at the point where the site is located and appropriate techniques are used. There is no value in taking a reading that is somewhere nearby the site location, the accuracy that results in such cases in only equivalent to that obtained using a map of scale between 1:50,000 to 1:100,000 to 1:250,000 depending how far away the GPS receiver is from the “site”.

It is also important to ensure that the GPS receiver has had time to settle, and that the geometry of the visible GPS satellites will provide the required accuracy and quality (sometimes known as the “dilution of precision” indicator).

For this reason the ACMA and accredited persons may need to request documentation on the measurement technique used to determine the site coordinates.

When using a GPS receiver care must be taken to ensure that correct datum is used. By default GPS system uses the World Geodetic System 1984 (WGS84) datum. Before collecting data with a GPS receiver that receiver’s datum should be a datum supported by the ACMA. To support the ACMA’s preparation for moving to the GDA94 datum, it is preferred that the datum used by GPS applications is GDA94.

According to the Intergovernmental Committee on Surveying and Mapping, when accuracy of only a few metres is required GDA94 coordinates can be considered the same as WGS84. For site data purposes, in most cases coordinates determined using a GPS with a WGS84 datum can be considered to be the same as GDA94 coordinates.

For more information see the GPS & GLONASS section of the Geosciences Australia website and the GDA FAQ section of the Intergovernmental Committee on Surveying and Mapping website.

## 4.4	Measurements of accuracy and quality of site coordinates

The ACMA’s radiocommunications licensing database (RADCOM) records the accuracy of supplied site coordinates and the methodology used to determine coordinates. Current values are:

Accuracy (known as precision in database): within 10m, within 100m, unknown

Coordinate source: GPS; Surveyor; Map 1:100,000; Map 1:50,000; Map 1:25,000; Map 1:10,000; Map 1:5,000; Map 1:2,500.

When combined, the accuracy and the coordinate source provide a guide to the accuracy and reliability of the recorded site coordinates to assist users in validating site data.

## 4.5	The RADCOM 10m rule

The ACMA’s RADCOM database has a data integrity rule that does not allow the creation of a new site within 10 metres of an existing site.

# 5	Site name

A site name comprises three elements - building/tower name, street address and town/geographic location. For detailed examples of site name see Section 8 “Site name and site configuration examples”. A simple description of each element is provided below:

Building/tower name: name of the building or tower at the site if applicable (entered in Title Case format). Examples are:

Rialto Building

NTA Tower or

NTA Tower Rialto Building.

Street address: is the street number and street name within a suburb or town (entered in Title Case format). This field can also be used to provide supplementary information to describe more precisely a specific location in conjunction with the town, suburb or geographical location name.

Examples of the various ways this element can be recorded are as follows:

within a town or suburb - 101 Collins Street

supplementary information (remote locations) - via Bourke and

if the site has no street address (remote from a town) - 12 km SW of

Town or geographical location: is the proper name of the town/locale of the site (entered in UPPERCASE format). Site name construction for this element follows:

Localities having postcode status should be recorded as listed. Suburbs within metropolitan areas should be recorded under the suburb name not the city name, eg, Carlton not Melbourne

In rural areas a site located outside town limits but which is used to provide radiocommunications facilities for the town should be recorded using the site name rather than the town. (The site name should be consistent with records in the Gazetteer or AUSLIG 1:1000000 scale NATMAP Topographic Maps. If relevant names cannot be found then the site name by which it is commonly known may be used.)

Standard abbreviations for particular situations:

mountain sites which are geographical features and not town names should be recorded as Mt (eg, Mt Macedon)

town names should be recorded as Mount (eg, Mount Macedon)

geographical features such as `Black Mountain' should be recorded as Mtn (eg, Black Mtn)

town names which include mountain as part of their name should not be abbreviated (eg, Mountain Gate or Mountain River).

# 6	Site configuration

The aim of the site design/layout/configuration field is to allow additional information to be provided that describes how the recorded site coordinates relate to the physical location of antennas on the site. Complete examples with reference to site name are at Section 8 “Site name and site configuration examples”.

Note: the list of descriptions will be expanded over time to accommodate new descriptions.

Site Configuration Description; Meaning. Tower centre; Site coordinates represent the centre point of a tower base. There maybe one or more antennas located on the tower, or nearby the tower.. Rooftop centre; Site coordinates represent the centre of a rooftop. There may be one or more antennas located about the centre of the rooftop.. Rooftop nominal; Site coordinates represent a point on a building rooftop. To be used in cases where building geometry or antenna location mean that rooftop centre is not a meaningful description.. Broadcasting Planning; Transmission site identified for the purpose of broadcast planning. This may or may not correspond to an established transmission site. There may or may not be licensed broadcasting services operating at this site.. Nominal point; Site coordinates represent a nominated point about which a number of antennas are located. To be used when the more specific descriptions are not appropriate..

# 7	What site is that antenna on?

To determine what site a new antenna should be recorded on, or whether to create a new site the following general principles should be considered.

Separate towers on rooftops or hilltops should always be recorded as separate sites

Antennas distributed across large complexes (for example, shopping centres, industrial complexes, warehouses, and universities) should not be represented as one site. Instead the site name field should be used to indicate on what part of the complex the antenna is located and the configuration field used to describe the site at that location

A building rooftop with a single tower and antennas on the side of the building could be recorded as one site for the tower and other sites that represent the layout of the antennas on the sides of the building, or as a single site. The approach depends on the size and layout of the building

Antennas at different levels on the side of a building should be recorded at the same site with a different antenna height to record where the antenna is located

Where an antenna is more than 30m from a tower, consideration should be given to recording the antenna on a separate site

Note: this list may be expanded as other examples are identified.

## 7.1	Points to consider when determining site location and coordinates

The following points should be considered in determining site location and coordinates for a new service:

As a starting point obtain site coordinates to the accuracy of available maps or use coordinates supplied with the request. Surveyed data or data taken using a GPS unit are preferred

Ensure that a common coordinate datum is used for coordinate comparison. Note that the ACMA database records coordinates in 1966 Australian Geodetic Datum (AGD66)

Search for existing sites within the vicinity of the proposed site as the proposed site may already be recorded in the ACMA database (this avoids unnecessary creation of additional site records). A search radius consistent with available map coordinate plotting accuracy and the expected density of radio sites should be used. Commonly used initial search radii are:

50m to 100m in capital city central business districts

100m to 500m in suburbs and other cites and

500m to 2000m in rural areas.

Using a listing of adjacent services on the site (a list of all services within a certain frequency range and distance of the proposed site) as a guide to assessing the accuracy of proposed site coordinates compared with those of database site coordinates

For possible sites review the site configuration field to see if the location of the new service can be covered by the description of the physical location of antennas on the existing sites

If by using coordinate searches, no site can be identified as being that of the proposed site, a site name search should occur before creating a new database site record. Site name searches may include building/tower names, street addresses, or geographic location/suburb

If any doubts exist about the accuracy of site coordinates obtained from the above, additional advice should be sought (eg contact the licensee or licence applicant). Accredited Persons may also contact the ACMA office responsible for the site for advice. In cases where there are still doubts about the accuracy of the site coordinates, ACMA staff should consider whether an audit of the sites involved is warranted.

# 8	Site name and site configuration examples

## 8.1	Individual towers at a site separated by more than  10m

Tower/Building Name; Street Number/Address; Town/Geographical Location; Site Configuration. CFA Tower; BASS HILL; Tower Centre. NTL Tower; BASS HILL; Tower Centre. Telstra CMTS Tower; BASS HILL; Tower Centre. NTA Tower; MT NARDI; Tower Centre.

## 8.2	Localities, town names and geographic features

| Tower/Building Name |  | Street Number/Address | Town/Geographical Location | Site Configuration |
| --- | --- | --- | --- | --- |
|  |  | 105 Miller Street | NORTH SYDNEY | As appropriate |
| BHP House |  | 11 Rundle Street | ADELAIDE | As appropriate |
|  |  | 55 Edward Street | CARLTON | As appropriate |
| Motorola Site |  |  | DARK MTN (where MTN is the standard for a mountain geographical feature) | As appropriate |
|  |  | 11 Alfred Street | CLEAR MOUNTAIN (where mountain forms part of the town name) | As appropriate |
|  |  | via Bourke | DYNEVOR HOMESTEAD (where the homestead is a gazetted location and `via Bourke' provides useful information) | As appropriate |
|  |  | Survey Camp 15 km SW of | DIRRANBANDI (where the location has no recognised name and the nearest geographical location is recorded) | As appropriate |

## 8.3	Building with a tower at each end of the building

Tower/Building Name; Street Number/Address; Town/Geographical Location; Site Configuration. North Tower; 350 Elizabeth Street; MELBOURNE; Tower Centre. South Tower; 350 Elizabeth Street; MELBOURNE; Tower Centre.

## 8.4	Sites on sides of buildings

The preferred approach is to use a common site name and antenna height to distinguish where the antenna is located. This is consistent with the approach that the building is the antenna support structure.

Tower/Building Name; Street Number/Address; Town/Geographical Location; Site Configuration; Site Height; Antenna Height. North Face, 286 Victoria Street; PERTH; Nominal; 100; 10. North Face, 286 Victoria Street; PERTH; Nominal; 100; 40.

## 8.5	Buildings with towers and microwave dishes that are grouped together

Tower/Building Name; Street Number/Address; Town/Geographical Location; Site Configuration. Bourke Place; 600 Bourke Street *; MELBOURNE; Tower Centre. 120 Collins Street **; Tower Centre. 101 Collins Street ***; MELBOURNE; Tower Centre.

* 	Left picture

**	Left tower in right picture

*** 	Right tower in right picture

## 8.6	Nominal location terminology for sites on buildings

Sites on Sides of a Building; Sites on Top of a Building. North Face; North; SE Cnr. South Face; South; SW Cnr. East Face; East; NE Cnr. West Face; West; NW Cnr.

or as appropriate depending on orientation of building.

Where the above terminology is not used, it should be assumed that the site is in the centre of the building.

# 9	Responsibilities for site data

## 9.1	Overview

An apparatus licensed radiocommunications service authorises the licensee to operate the service at the location specified on the licence. If a licensee operates from a location not specified on the licence, the licensee may be subject to regulatory action by the ACMA.

Devices to be operated under a spectrum licence cannot be operated unless the location of the device is recorded in the Register of Radiocommunications Licences. If a licensee of a device operates from a location not specified in the register, the licensee may be subject to regulatory action by the ACMA.

## 9.2	Licensees

It is the licensee’s responsibility to ensure that the specified site (ie the combination of coordinates, site name, site configuration) accurately reflect an antenna’s location as determined using the procedures outlined in this document.

## 9.3	Accredited persons

The responsibility of accredited persons is limited to checking to see if the supplied data is “reasonable” and asking the licensee/licence applicant to check coordinates when in doubt. An example of a “reasonable” test is whether the site name, site coordinates and site configuration description are consistent.

## 9.4	The ACMA

### 9.4.1	Frequency assignment and licensing

The responsibility of the ACMA is limited to checking to see if the supplied data is “reasonable” and asking the licensee/licence applicant to check coordinates when in doubt. An example of a “reasonable” test is whether the site name, site coordinates and site configuration description are consistent.

### 9.4.2	Auditing and quality control of site data

The ACMA approach to ensure quality control of site data is to assume that the requirements and procedures of this document are being consistently applied, and to audit site data as a part of the ACMA audit program of frequency assignment certificates and through random site audits.

# 10	Entering radiocommunications site data into RADCOM

Information required to create a site in the ACMA RADCOM database is outlined below. More detailed information is at Appendix 2.

## Mandatory

Site Name which comprises of the fields building/tower name, street number/name, town/geographic location

Coordinate Datum

Coordinates

Coordinate accuracy/precision

Coordinate source

Site Layout/Configuration.

## Optional

Site Elevation

Site Manager Surname (or trading name)

Site Manager Given name(s)

Site Manager Phone number

Site Manager E-mail

Contact Person Surname

Contact Person Given name(s)

Contact Person Phone number

Contact Person E-mail.

Note that the ACMA collects site contact details to assist with interference investigations and site audit activities. Currently, due the privacy requirements, site manager and contact person details are not made available once collected as that information is supplied on the basis that it is not publicly available.

# 11	Online site data management

The site data management facilities will be included in the “Online Services for Accredited Persons” application in the next phase of the application due for release in early 2005. The online site data management facility will support site creation, site updating and site deletion. When creating a site online an accredited person will be required to state that the site has been created from information supplied by a licence applicant using the ACMA’s radiocommunications site data requirements specified in this document.

The online system will also allow an accredited person to request the ACMA to assess submitted site information. An accredited person may use that facility in cases where the site information provided by a licensee conflicts with information recorded in the ACMA database, and that conflict cannot be resolved using the ACMA site data procedures. Coordination of those assessments will be managed by the Radiocommunications Licensing and Telecommunications Deployment Section in the ACMA’s Canberra Central Office.

The online system is only available to accredited persons who have registered to use the system. More detailed information about the online system (including how to register) is contained in the online services user guide available on the monthly Radiocommunications Record of Licences (RRL) Database on CDROM or by contacting the ACMA’s ICT Service Desk ictservicedesk@acma.gov.au.

## 11.1	Online site creation

The creation of a new site using the accredited person’s online submission system is subject to the following rules:

That the information listed in Section 10 “Entering radiocommunications site data into RADCOM” is provided

There is no other site within 10m of the proposed new site

That in high and medium fee density areas coordinate source is a surveyor’s certificate or GPS and that the accuracy is 10m or 100m.

Note: mobile sites (used in spectrum licensing) cannot be created online.

## 11.2	Online site deletion

The deletion of an existing site in the RADCOM database using the accredited person’s online submission system is subject to the following rules:

No sites with devices attached can be deleted

No mobile sites (used in spectrum licensing) can be deleted

No site can be deleted that was created or updated in the last 30 days unless the user created or updated the site.

## 11.3	Online site updating

The updating of an existing site in the RADCOM database using the accredited person’s online submission system is subject to the following rules:

No mobile sites (used in spectrum licensing) can be updated

Site manager and contact person details cannot be updated. (As this information is not publicly available it is unlikely that an accredited person would know what information would require updating.)

Coordinates and site elevation cannot be updated for sites with devices attached

No site can be updated with devices attached unless:

all the devices are spectrum licensing devices and

the user is authorised to supply technical information for the licences that the devices are attached too

No site can be updated that was created or updated in the last 7 days unless the user created or updated the site

Coordinates cannot be updated if the new coordinates would be within 10m of an existing site

If the coordinates are updated the precision:

in high density fee areas must be 10m

in medium density fee areas must be:

if already 10m, 10m

otherwise (ie if 100m, or unknown), 10m or 100m

in low density fee area must be either 10m or 100m.

# Appendices

# Appendix 1: Site data conversion

## Purpose

This Appendix provides information relating to Site Data, and the conversion of this data into a standard information base within the RADCOM system.

## Background and explanation

The Geocentric Datum of Australia (GDA) is the new Australian coordinate system, replacing the Australian Geodetic Datum (AGD). GDA is part of a global coordinate reference frame and is directly compatible with the Global Positioning System (GPS). It is the culmination of more than a decade of anticipation and work by the Intergovernmental Committee on Surveying and Mapping (ICSM) and its predecessor the National Mapping Council (NMC). When the NMC adopted the AGD84 coordinate set in 1984, it "recognised the need for Australia to eventually adopt a geocentric datum". This was further recognised in 1988 when ICSM recommended the adoption of an appropriate geocentric datum by 1 January 2000.

## Background to GDA

In 1992, as part of the world-wide International GPS Service (IGS) campaign, continuous GPS observations were undertaken on eight geologically stable marks at sites across Australia, which form the Australian Fiducial Network (AFN). During this campaign, GPS observations were also carried out at a number of existing geodetic survey stations across Australia. These were supplemented by further observations in 1993 and 1994, producing a network of about 70 well determined GPS sites, with nominal 500km spacing across Australia. These sites are collectively known as the Australian National Network (ANN).

The GPS observations at both the AFN and ANN sites were combined in a single regional GPS solution in terms of the International Terrestrial Reference Frame 1992 (ITRF92) and the resulting coordinates were mapped to a common epoch of 1994.0. The positions for the AFN sites are estimated to have an absolute accuracy of about 2cm at 95% confidence, while the ANN positions are estimated to have an absolute accuracy of about 5cm. These positions of the AFN sites were used to define the GDA, and were published in the Commonwealth of Australia Government Gazette of 6 September 1995.

The positions of both the AFN and ANN sites were used to constrain a re-adjustment of the Australian geodetic networks, which included all observations from the previous AGD66 and AGD84 adjustments, conventional observations added since that time, and the extensive GPS networks established by the State and Territory authorities at about 100km spacing between the ANN sites. This resulted in a data set of more than 70,000 observations and produced GDA94 coordinates at almost 8,000 stations. These GDA94 coordinates are now used by the State and Territory authorities to adjust their subsidiary survey networks onto GDA.

## GDA and AGD

ITRF92, on which GDA is based, was realised using Very Long Baseline Interferometry (VLBI), GPS and Satellite Laser Ranging (SLR) observations at 287 globally distributed stations. However, the coordinates for Johnston, the original station for the AGD, were based on a selection of 275 astro-geodetic stations distributed over most of Australia. The adoption of this option, and the best fitting local ellipsoid, the Australian National Spheroid (ANS) meant that the centre of the ANS did not coincide with the centre of mass of the earth, but lay about 200 metres from it. Hence the GDA94 coordinates of a point appear to be about 200 metres north east of the AGD coordinates of the same point.

The precise size and orientation of the difference will vary from place to place.

## Datum data and the ACMA

The Radcom database uses AGD66 as the basis for its site information and consequently all site information is stored using this datum, and printed to licence documentation where applicable.

Computing Services have changed the "Location Management" frame within the RADCOM application that will allow the entering if site information in Zoning Easting Northing, Decimal Degrees and Degrees Minutes Seconds in AGD66, AGD84 and GDA94.

Where site coordinates are entered using GDA94, the application will use a formulae (known as Molodensky's Formulae) to convert the coordinates into AGD66 to be stored within the GIS, and store the original GDA94 coordinates in new fields within the existing site table in RADCOM. Similarly, when site coordinates are entered in AGD84, the application will firstly use Molodensky's formulae to convert the coordinates to GDA94 to be stored in the new fields within the existing site table, and again to convert the GDA94 coordinates into AGD66 to be stored in the GIS.

The ACMA Form RF77 has been amended to allow clients to stipulate what Datum reference they have used when providing site information. The RADCOM site management screen will allow for the selection of the Datum specified, and the entry of subsequent details.

Sometime in the future, all AGD66 coordinates within RADCOM will be converted to GDA94. However, before then it should be noted that AGD66 will continue to be the Datum of choice within RADCOM.

# Appendix 2: Site data entry requirements

The following data is required when creating a new radiocommunications site. Fields to be introduced in the 1st quarter 2005 are indicated as “(2005)”.

## Mandatory

Field; Values; Comments. Building/tower name (2005); Character, maximum combined length 96 characters; Must provide one of Building/tower name or street address. Street address (2005). Town/geographic location (2005); Mandatory. Datum; AGD66, AGD84 or GDA94; Mandatory. Coordinates as either: Zone, Easting, Northing Latitude, longitude (DD) Latitude, longitude (DMS); As below for details; Mandatory. Zone, Easting, Northing; Zone (two digits) valid range 41-59 Easting (six digits in metres) Northing (seven digits in metres); Latitude should be south (negative) Longitude should be east (positive). Latitude, longitude (DD) Decimal degrees; Up to six decimal places Eg Lat -43.123456 Long 143.123456. Latitude, longitude (DMS) Degrees Minutes Seconds; To 3 decimal places Eg Lat: -43 34 56.678 Long: 145 45 567.345; Latitude should be south (negative) Longitude should be east (positive). Coordinate Source (2005); Surveyor, GPS, Map 1:100,000; Map 1:50,000; Map 1:25,000; Map 1:10,000; Map 1:5,000; Map 1:2,500; Mandatory, addition values can be added by ACMA if required. Accuracy/Precision; Within 10 metres, or Within 100 metres; Mandatory. Configuration (2005); Tower, Rooftop centre, Rooftop nominal, reference point; Mandatory, addition values can be added by ACMA if required. Site Elevation; Numeric; Optional. Site Manager Surname (or trading name); Up to 50 characters; Optional. Site Manager Given name(s); Up to 40 characters; Optional. Site Manager Phone number; Up to 15 characters; Optional. Site Manager E-mail (2005); Up to 50 characters; Optional. Contact Person Surname; Up to 50 characters; Optional. Contact Person Given name(s); Up to 40 characters; Optional. Contact Person Phone number; Up to 15 characters; Optional. Contact Person E-mail (2005); Up to 50 characters; Optional.