Business operating procedure

Submission and processing of applications for earth and earth receive apparatus licences for fixed earth stations

DECEMBER 2017

Canberra

Red Building 
Benjamin Offices
Chan Street 
Belconnen ACT

PO Box 78
Belconnen ACT 2616

T	+61 2 6219 5555
F	+61 2 6219 5353

Melbourne

Level 32 
Melbourne Central Tower
360 Elizabeth Street 
Melbourne VIC

PO Box 13112
Law Courts 
Melbourne VIC 8010

T	+61 3 9963 6800
F	+61 3 9963 6899

Sydney

Level 5 
The Bay Centre
65 Pirrama Road 
Pyrmont NSW

PO Box Q500
Queen Victoria Building 
NSW 1230

T	+61 2 9334 7700 or 1800 226 667
F	+61 2 9334 7799

Amendment history

| Version | Date of effect | Comments |
| --- | --- | --- |
| 1.0 | 6 August 2014 | Initial release |
| 1.1 | 3 May 2016 | Update to include procedures to be followed by APs |
| 2.0 | 19 December 2017 | Updated to include further detail and information on procedures and checks undertaken by the ACMA. Refer IFC 19/2017 Update of licensing procedures for space-based communications systems |

Copyright notice

http://creativecommons.org/licenses/by/3.0/au/

With the exception of coats of arms, logos, emblems, images, other third-party material or devices protected by a trademark, this content is licensed under the Creative Commons Australia Attribution 3.0 Licence.

We request attribution as © Commonwealth of Australia (Australian Communications and Media Authority) 2017.

All other rights are reserved.

The Australian Communications and Media Authority has undertaken reasonable enquiries to identify material owned by third parties and secure permission for its reproduction. Permission may need to be obtained from third parties to re-use their material.

Written enquiries may be sent to:

Manager, Editorial and Design
PO Box 13112
Law Courts
Melbourne VIC 8010
Email: info@acma.gov.au

1	Purpose	1

2	Background	1

3	Regulatory considerations	1

3.1	Determining the ITU regulatory status	2

3.1.1	Recorded in the MIFR with favourable finding	2

3.1.2	Not recorded in the MIFR	2

3.2	Checking consistency with ITU registration details of the satellite network and proposed use	3

3.3	Launch and transfer orbit support services and/or in-orbit testing	3

3.4	The information required for regulatory assessment	3

4	Consultation with relevant Government organisations	4

5	Frequency coordination	4

6	Licensing arrangements for fixed earth stations	5

7	Special licence conditions and advisory notes	5

8	Further information	5

Appendix A: Standard special conditions and 
advisory notes 	6

A.1	Recorded in the MIFR with favourable finding	6

A.1.1	Special Conditions	6

A.1.2	Advisory Notes	6

A.2	Not recorded in the MIFR	6

A.2.1	Special Conditions	6

A.2.2	Advisory Notes	7

Appendix B: Special conditions and advisory notes for launch and TOSS/IOT use	8

B.1	A specific space mission	8

B.1.1	Special Conditions	8

B.1.2	Advisory Notes	8

B.2	Any number of space missions	8

B.2.1	Special Conditions	8

B.2.2	Advisory Notes	9

Appendix C: Information required for regulatory assessment	10

Appendix D:	Letter of assurance pro forma	11

Applicants or accredited persons must read the Disclaimer in conjunction with the procedures set out below.

# Purpose

This business operating procedure (BOP) outlines the technical and regulatory framework applicable to the apparatus licensing of fixed earth stations, as well as the procedures to be followed when seeking the issue of licences for fixed earth stations. It covers the apparatus licensing options of fixed earth and earth receive.

The procedures outlined in this BOP cover the assessment of the space-related aspect of the licence application process only. The accredited person or applicant must provide the response from this process with the subsequent apparatus licence application. The ACMA will only issue an apparatus licence where the space-related assessment supports the application.

The procedures in this BOP are reflective of the status of satellite networks associated with typical licence applications and cover the majority of licensing scenarios only. Cases outside those outlined will be treated on a case-by-case basis.

# Background

In considering whether to grant an application for a fixed earth or earth receive apparatus licence, the ACMA is required to consider a range of matters. Depending on the application, the assessment process may require:

consideration of consistency with current regulatory arrangements, including the international (International Telecommunications Union—ITU) regulatory status and the international (ITU) registration details of the satellite network that will cover operation of the proposed fixed earth station

relevant government organisations to be consulted in relation to the application

frequency coordination for the proposed fixed earth station with existing licensed radiocommunications services

the licence type(s), necessary to authorise operation of the fixed earth station, to be determined

appropriate special condition(s) and/or advisory note(s) to be applied to the licence.

This BOP provides an overview of the processes associated with each of these steps.

Additional information on the arrangements for fixed earth and earth receive stations can be found from the Information directory for apparatus licensing and the Space system regulation areas on the ACMA website.

# Regulatory considerations

As with all radiocommunications services, frequency assigning and licensing needs to be consistent with current regulatory arrangements. At a high level, this means consistency with requirements as specified in the Radiocommunications Act 1992, the Australian Radiofrequency Spectrum Plan (the Spectrum Plan), frequency band plans (both legislative and administrative) and spectrum embargoes.

For the licensing of space based communication systems (in this case fixed earth and earth receive), there is an additional requirement to consider the international (ITU) regulatory status and the international (ITU) registration details of the satellite network that will cover operation of the proposed fixed earth station. This assessment is undertaken by the ACMA’s satellite coordination area using information provided by the applicant or accredited person (AP). The applicant or AP is advised of the result of the assessment and the appropriate special conditions and advisory notes that are to be included on the licence. More information on the special conditions and advisory notes can be seen in Section 7.

Further information on the assessment of the ITU regulatory status and the ITU registration details of satellite system is below.

## Determining the ITU regulatory status

Prior to technical coordination, it is necessary to ascertain the ITU regulatory status of the satellite network that the proposed fixed earth station is part of.

Information about the ITU regulatory status of a satellite network can be found by checking the ITU’s Master International Frequency Register (MIFR). The results of these checks are used in part to determine appropriate special condition(s) and/or advisory note(s) to be applied to the licence.

### Recorded in the MIFR with favourable finding

If a satellite network is recorded in the ITU’s MIFR with a favourable finding, then the risk of interference may be considered acceptably low. Special conditions and advisory notes that are specific to the situation must be applied to the apparatus licence (refer to section 7).

### Not recorded in the MIFR

In the case where a satellite network is not yet recorded in the MIFR but is progressing through the ITU coordination process, the ACMA may seek information using a ‘Letter of Assurance’ (LOA) about its ITU coordination status from the satellite operator (via the applicant or AP) to ascertain whether or not the proposed satellite network is likely to be recorded in the MIFR.

The LOA is where the satellite operator advises the ACMA of any ITU published references, including coordination request and/or notification notices and particulars of any unresolved coordination issues. The LOA will also include a statement to the effect that measures will be taken to ensure that the operation of the satellite network in Australia will not cause interference to other satellite networks/systems operating in accordance with the ITU Radio Regulations. Details on the measures to be undertaken to not cause interference should be included in the LOA. The format of the LOA is at Appendix D.

Where the ACMA is assured of a likely successful outcome, the ACMA will progress the earth station licence application. However, special conditions and advisory notes that are specific to the situation will be applied to the apparatus licence (refer to Section 7).

## Checking consistency with ITU registration details of the satellite network and proposed use

Upon the determination of the ITU regulatory status of the satellite network, the ACMA needs to further check whether the ITU registration details of the satellite network are consistent with the characteristics of the proposed fixed earth station, as specified in the licence application, and that the service is consistent with the Spectrum Plan.

Checking consistency of ITU registration details with the proposed service and the Spectrum Plan may include (but is not limited to) the following factors:

the frequency ranges:

the frequency ranges of the proposed fixed earth station must be covered by the frequency ranges of the satellite network

the class of station:

the service purpose/function of the proposed fixed earth station must be consistent with the station class(es) of the satellite network

the service area:

the location of the proposed fixed earth station must be within the service area of the satellite network

technical characteristics of the associated earth station:

the technical characteristics of the proposed fixed earth station must be within the envelope of the associated earth stations of the satellite network.

## Launch and transfer orbit support services and/or in-orbit testing

Due to the transitory nature of launch and transfer orbit support services (TOSS) and/or in-orbit testing (IOT) services, such services are often not identified in the ITU regulatory information. Consequently, the related licence applications are considered on a case-by-case basis, with conditions determined depending on the specifics of the applications.

Consideration of launch/TOSS/IOT are divided into two sub categories:

Temporary fixed earth stations that provide launch and TOSS services and/or IOT services for a specific space mission. Such licences are normally limited to the period of mission event and are non-renewable with maximum licence period of one year.

Ongoing fixed earth stations that provide launch and TOSS services and/or IOT services for any number of space missions.

Applicable special conditions and advisory notes that must be applied to the apparatus licence for these types of applications are discussed at Section 7.

## The information required for regulatory assessment

In order to assess the proposed fixed earth stations against the above requirements for the ITU regulatory status and the ITU registration details of the related satellite network, applicants or APs may submit completed earth and/or earth receive application form(s) (depending on the type of licence being sought), or alternatively, provide the information as listed at Appendix C in a summary table format to satellite.coordination@acma.gov.au.

In general, application forms or the summary tables should include all necessary information for most applications. However, depending the specifics of certain licence applications, the ACMA may seek further information from the applicant or AP to assist with the assessment.

# Consultation with relevant government organisations

Due to possible security issues associated with foreign ownership of aspects of space communications, some applications may be subject to wider government consultation. In general, the ACMA will consult with relevant organisations in the following situations:

new missions by existing ground stations that support (or suggest support) of foreign space systems, including the launch or early orbit phases

new foreign owned, or partly foreign owned, earth stations and space support equipment

new Australian-owned earth stations that will provide support to foreign space systems including launch or early orbit phases, except where the foreign space system is used solely for commercial communications (for example, television broadcasting).

The applicants or APs should note that additional time may be required to process applications that are subject to wider government consultation.

# Frequency coordination

Assignments in support of fixed earth station apparatus licences must be consistent with the Australian Radiofrequency Spectrum Plan (the Spectrum Plan).

Depending on the frequency band of operation, the proposed fixed earth station may need to be coordinated with incumbent radiocommunications services operating in or adjacent to the proposed band of operation. Coordination criteria for some bands are detailed in relevant Radiocommunications Assignment and Licensing Instructions (RALIs).

For frequency bands or sharing scenarios for which the ACMA has not published coordination criteria, the relevant ITU Recommendations should be considered. APs should also consider the matters detailed in section 5 of the Radiocommunications (Frequency Assignment Certificates) Determination 2014.

# Licensing arrangements for fixed earth stations

The ACMA website has a detailed explanation of the framework that applies to the licensing of space networks.

For fixed earth station operation, the fixed earth licence type may be used to authorise operation of uplink frequencies (that is, Earth-to-space direction) and earth receive licence type to authorise operation of downlink frequencies (that is, space-to-Earth direction) for one or more fixed earth stations. These licence types are generally used to support:

ongoing general communications between an individual earth station and a satellite

ongoing telemetry, tracking and control (TT&C) for the purpose of providing permanent ‘station keeping’ services for a specific satellite

one-off requirement to provide launch and TOSS and/or IOT for a specific space mission; or

ongoing requirement to provide launch and TOSS and/or IOT for any number of space missions.

# Special licence conditions and advisory notes

The Radiocommunications Licence Conditions (Apparatus Licence) Determination 2015 (LCD) specifies general conditions of operation that are common to most apparatus licence types. Additional conditions not in the LCD can be included on individual apparatus licences to address issues specific to an assigned service. These conditions are printed on the licence under the heading ‘Special Conditions’.

In addition to the above, ‘Advisory Notes’ may also be included on licences to inform licensees of matters relevant to a particular service. Unlike licence conditions, advisory notes do not impose a legal obligation on the licensee.

Appendixes A and B detail the special conditions and advisory notes that are to be included on fixed earth and earth receive apparatus licences. Where a special condition or advisory note is not pre-defined, an AP will need to request the ACMA to include the appropriate conditions or advisory notes on the licence.

# Further information

Please contact the ACMA’s Customer Service Centre at info@acma.gov.au.

Standard special conditions and advisory notes

This appendix applies to ongoing communications by individual fixed earth stations with a specific satellite system (including ongoing telemetry, tracking and control for the purpose of providing permanent station keeping services for a specific satellite system).

Recorded in the MIFR with favourable finding

The following special conditions and advisory notes are to be applied where the satellite system has been successfully coordinated and is recorded (notified) in the ITU’s Master International Frequency Register (MIFR) with favourable finding:

Special Conditions

Pre-defined (EC)

- Operation of this earth station must be in accordance with frequency assignments recorded in the Master International Frequency Register (MIFR) of the International Telecommunication Union.

Advisory Notes

Pre-defined (EI)

- The Master International Frequency Register (MIFR) is maintained by the International Telecommunication Union (ITU) in accordance with the Radio Regulations.

User-defined

- This licence authorises communications with [ITU satellite network name].

Not recorded in the MIFR

The following special conditions and advisory notes are to be applied where the satellite system is still under coordination and has not as yet been recorded (notified) in the ITU’s Master International Frequency Register (MIFR):

Special Conditions

Pre-defined (EJ)

- Prior to the frequency assignments being recorded in the Master International Frequency Register (MIFR), this earth station may operate in accordance with the operating parameters published by the ITU in Special Sections of International Frequency Information Circulars, and in accordance with any agreements reached as a result of an ITU frequency coordination process.

Pre-defined (EH)

- Transmissions must not occur in circumstances that result in harmful interference to stations outside of Australia where these stations are operating in accordance with the Radio Regulations of the ITU, except where the transmissions are in accordance with any agreements reached as a result of an ITU international frequency coordination process.

User-defined

- This earth station is authorised to communicate with space station(s) of the [ITU satellite network name] satellite network as published by the International Telecommunication Union (ITU) in Special Section [ITU reference—take the most recent document number] of International Frequency Information Circular [IFIC number].

Advisory Notes

Pre-defined (EI)

- The Master International Frequency Register (MIFR) is maintained by the International Telecommunication Union (ITU) in accordance with the Radio Regulations.

Pre-defined (ED)

- Coordination agreements reached as a result of an ITU international frequency coordination process are intended to minimise the potential for harmful interference to radiocommunications stations. A radiocommunications station operated prior to a frequency assignment being recorded in the MIFR cannot necessarily claim protection from harmful interference from radiocommunications stations of other countries.

Special conditions and advisory notes for launch and TOSS/IOT use

This appendix applies to launch, and transfer orbit support services (TOSS) and/or in-orbit testing (IOT).

A specific space mission

The following special conditions and advisory notes are to be applied where the temporary fixed earth station provides launch and transfer orbit services and/or in-orbit testing for a specific space mission:

Special Conditions

Pre-defined (E8)

- This licence authorises radiocommunications with space stations for the purpose of performing transfer orbit support services and/or in-orbit testing.

Pre-defined (EA)

- No harmful interference shall be caused to radiocommunications stations operating in accordance with the provisions of the Constitution, the Convention and the Radio Regulations of the International Telecommunication Union.

Advisory Notes

Pre-defined (EB)

- Protection is not afforded for interference caused by radiocommunications stations operating in accordance with the provisions of the Constitution, the Convention and the Radio Regulations of the International Telecommunication Union.

User-defined

- This licence authorises communications with [ITU satellite network name].

Any number of space missions

The following special conditions and advisory notes are to be applied where the ongoing fixed earth station provides launch and transfer orbit support services and/or in-orbit testing for any number of space missions:

Special Conditions

Pre-defined (E8)

- This licence authorises radiocommunications with space stations for the purpose of performing transfer orbit support services and/or in-orbit testing.

Pre-defined (E9)

- Before commencing transmissions to each space station, the licensee must inform ACMA of the International Telecommunication Union identity of the space station and the dates of commencement and completion of transmissions.

Pre-defined (EA)

- No harmful interference shall be caused to radiocommunications stations operating in accordance with the provisions of the Constitution, the Convention and the Radio Regulations of the International Telecommunication Union.

Advisory Notes

Pre-defined (EB)

- Protection is not afforded for interference caused by radiocommunications stations operating in accordance with the provisions of the Constitution, the Convention and the Radio Regulations of the International Telecommunication Union.

Information required for regulatory assessment

Information required for fixed earth and earth receive licences

|  |  | Example 1 | Example 2 |
| --- | --- | --- | --- |
| Licence type: |  | Fixed earth | Earth receive |
| Licence renewability (ongoing/ non-ongoing) and licence period: |  | Non-ongoing (6 months) | Ongoing (initial 1 year) |
| Direction: |  | Uplink | Downlink |
| Frequency lower bound (MHz): |  | 14301 | 1526 |
| Frequency upper bound (MHz): |  | 14302 | 1527 |
| Service purpose (Communications, TT&C, Launch/TOSS, or IOT): |  | Launch/TOSS | Communications |
| The related ITU satellite network name: |  | SAT-A | SAT-B |
| The orbital longitude of the satellite network: |  | NGSO | 140E |
| The location of the fixed earth station: |  | Landsdale, WA | Sydney, NSW |
| The maximum e.i.r.p. of the earth station transmitter (for fixed earth licence only): |  | –3 dBW | N/A |
| The antenna 3dB beamwidth (for fixed earth licence only): |  | 0.5 degree | N/A |
| The antenna maximum gain (for fixed earth licence only): |  | 30 dBi | N/A |

Note: depending on the specifics of certain licence applications, the ACMA may seek further information from the applicant or AP to assist with the assessment.

Letter of assurance pro forma

This pro forma is to be used when a satellite filing has not been successfully recorded in the Master International Frequency Register (MIFR).

The application for a licence to communicate with [satellite commercial name] in the frequency bands [insert frequency bands] will use the [ITU satellite filing name] satellite filing, for which the International Telecommunication Union (ITU) has published 'special sections' such as [CR/C xxx in IFIC XXXX (only include one–preferably the CR/C publication)]. This filing has been made by the Administration of [country name], and has orbital position/characteristics of [XXX E/W (GSO) or contains a constellation of P satellites in Q planes at R inclination and S altitude (NGSO)].

[Satellite operator] provides the following assurances to the ACMA to support applications for licences within Australia:

- Coordination with other administrations for the [ITU satellite filing name] satellite filing has begun, but has not yet resulted in a 'favourable finding' (Part II-s) notification published by the ITU in a fortnightly international frequency information circular (IFIC).

To the best of our knowledge, operation in accordance with the [ITU satellite filing name] satellite filing will not cause harmful interference to other satellite networks operating as per the ITU Radio Regulations.

That in the event of any actual case of harmful interference (to other satellite networks operating as per the ITU Radio Regulations), all efforts will be made to immediately address and resolve such interference.

The term space-related used here is intended to mean space-based network or space system, and not to mean space or space receive apparatus licence type.

See lists of space station class of station and earth station class of station in Preface to the BR IFIC (Space services).

The application forms can be downloaded via this link.

Space support equipment includes equipment that assists in the calibration of early orbit and on-orbit systems.

Failure to provide accurate ITU satellite network information will delay the process. The ITU SNL database (Part B) is a useful tool for checking the accuracy of the satellite network name. See http://www.itu.int/net/ITU-R/space/snl/bsearchb/spublication.asp.

If the above assurances do not cover all communications to/from the satellite, please indicate the limits of this assurance (for example, if it only applies to a particular frequency band or company).