Frequency assignment practice Guideline No. 1 Area wide land mobile services used for voice communications or low duty cycle data applications

DECEMBER 2012

Canberra Purple Building Benjamin Offices Chan Street Belconnen ACT PO Box 78 Belconnen ACT 2616 T +61 2 6219 5555 F +61 2 6219 5353; Melbourne Level 44 Melbourne Central Tower 360 Elizabeth Street Melbourne VIC PO Box 13112 Law Courts Melbourne VIC 8010 T +61 3 9963 6800 F +61 3 9963 6899; Sydney Level 5 The Bay Centre 65 Pirrama Road Pyrmont NSW PO Box Q500 Queen Victoria Building NSW 1230 T +61 2 9334 7700 1800 226 667 F +61 2 9334 7799.

Amendment History Version; Date of Effect; Comments. 1.0; 23 July 2012; Initial release of guideline. 1.1; 1 December 2012; Requirement relating to high power operation in the vicinity of sited services clarified.

© Commonwealth of Australia 2012 This work is copyright. Apart from any use as permitted under the Copyright Act 1968, no part may be reproduced by any process without prior written permission from the Commonwealth. Requests and inquiries concerning reproduction and rights should be addressed to the Manager, Editorial Services, Australian Communications and Media Authority, PO Box 13112 Law Courts, Melbourne Vic 8010. Published by the Australian Communications and Media Authority

## Purpose

To provide information to assist with the assignment and licensing of frequencies for area wide land mobile services used for voice communications or low duty cycle data applications.  In addition to this guideline, reference should be made to the Land Mobile Licence Licences - Guidelines which provides an overview of the licensing arrangements applicable to the land mobile service.

This guideline applies to all services operating in non-government spectrum, including government services operating within non-government spectrum.

Arrangements for area wide services employing high duty cycle, including Differential Global Positioning Systems (DGPS), are contained in Frequency Assignment Practice: Guideline No.2.

It should be noted that the ACMA has made provision for the use of certain UHF frequencies by companies supplying personal mobile radiocommunications equipment for short term rental.  The use of those frequencies is not subject to the arrangements outlined in this document.

Requests for exemptions to the arrangements detailed in this document, or licence applications that appear to fall outside the enunciated framework, should be referred to the ACMA at FACPolicyExemptions@acma.gov.au.

## Background

The ACMA occasionally receives applications from organisations seeking to operate radiocommunications services at varying locations for short periods of time.  The areas of operation are usually confined, but the locations may be widely dispersed, sometimes extending State or Australia wide.  Applicants seeking to licence these ‘area wide’ services are typically organisations involved in exploration, construction or the provision of communications facilities at event venues.  Given the nature of these activities, the locations of operation are not normally known in advance.

Area wide land mobile services may be licensed as either single frequency or two frequency (repeater) services, and may comprise a base station and mobiles/personal mobiles or just mobiles/personal mobiles.  Most area wide services are single frequency voice communication services operating on a low duty cycle, however, data services employing low or high duty cycle may also be licensed for area wide operation.

Over the years many channels in the VHF and UHF land mobile bands have been assigned and licensed for area wide use.  In some States, area wide services were initially assigned on channels reserved specifically for area wide use.  Despite these efforts to group area wide services on specific channels it is now the case that the majority of single frequency channels are licensed to both area wide and sited land mobile services.

In May 2012, the ACMA undertook an audit of VHF and UHF channels with a view to identifying a suitable set of frequencies that could be reserved for area wide use.  While the identified channels also contain assignments to sited services the low duty cycle applications envisaged, together with the assignment and licensing arrangements detailed below, should allow for the expanded deployment of area wide services on these channels while minimising the likelihood of interference to existing sited services.

## Interference considerations

As an area wide service operates at unspecified locations, it is not possible to perform detailed coordination.  Consequently, there is potential for the proposed service to cause interference to, or receive interference from, existing sited and area wide services.  Due to the transitory nature of area wide services (and, in this case, the low duty cycle), any interference is usually of a short duration, but nevertheless still has potential to cause significant disruption in certain situations.

A base station, in particular, has the potential to cause interference to other radiocommunication services due to receiver desensitisation and/or generation of intermodulation products, particularly in situations where high transmit powers are utilised at, or near, major radiocommunications sites.  The potential for interference is greatest in areas of high spectrum demand due to the higher density of licensed services.  For this reason the ACMA will limit the radiated power of base stations, associated with area wide services, when operating in high or medium density areas.  Outside of these areas higher radiated powers may be employed, but restrictions will apply to operations in the vicinity of sited services.

# Frequency assignment and licensing arrangements

In order to minimise the potential for interference, the following frequency assignment and licensing arrangements have been formulated for area wide services supporting voice communications or low duty cycle data applications.

- The single and two frequency channels listed in Table 1 and 2, respectively, have been reserved exclusively for area wide operation.  Sited services or area wide services employing high duty cycle (including DGPS transmissions) should not be assigned on these channels.

- Area wide services may only be assigned on those land mobile channels identified in Tables 1 and 2.

- Attachment 1 provides a list of spectrum access areas for which area wide operation may be authorised.  An area wide service should only be assigned to an area for which there is an operational requirement.  As an example, if operation is not required in high and medium density areas of Australia, then the area wide service should be restricted to low and remote density areas only (Area ID 74).

- An area wide licence authorises operation on a ‘short term’ basis at unspecified locations within the designated geographic area(s).  The ACMA will generally consider operations of 4 weeks or less at a given location as ‘short term’. Restricting the period of operation at any one location ensures that area wide frequencies are not used to provide semi-permanent services, assists with interference mitigation by limiting the period of uncoordinated operation at any one location and maximises flexibility in the use of area wide frequencies.

- For extended operation at a particular location, the licensee should apply for a sited licence.  A sited service has the benefit of being coordinated with other radiocommunications services in the area of operation, and is therefore able to operate without the constraints applied to area wide services.  Given requirements (1) and (2) above, and the need to coordinate with surrounding sited services, the frequency assigned to the service will be different to that made available for area wide use.

- An area wide service may operate only on condition that it does not cause interference to other radiocommunications services, and cannot claim protection from any interference caused by other services.  This requirement is enforced through the application of a special condition on the licence (refer Special Condition 27 at Attachment 2).

- A base station authorised under an area wide licence is restricted to low power (maximum 8.3 Watts EIRP) in high and medium density areas as defined in the Apparatus licence fees.  In low and remote density areas powers up to 83 Watts EIRP can be authorised, however Assigners should endeavour to restrict the power to that necessary for the given application.

For areas of operation which encompass a single area density (for example, Sydney/Wollongong HD), the restriction is enforced directly through the licensed power. For areas of operation which encompass more than one area density (for example Australia wide), and which include a high density or medium density area, the restriction is enforced through the licensed power in conjunction with Special Condition LM02 (refer Attachment 2).

- A base station authorised under an area wide licence, operating with a radiated power exceeding 8.3 Watts EIRP, may not be sited within 200 metres of a radiocommunications site with licensed services operating in the same band.  This requirement will be enforced by application of Special Condition LM03 or LM04, whichever is applicable (refer Attachment 2).

- An area wide land mobile service comprising a base station and mobiles/personal mobiles will be licensed as a Land Mobile System.  An area wide service comprising only of portables and/or mobiles will be licensed as a Land Mobile – Ambulatory System.  Further information on licensing arrangements is contained in the Land Mobile Licence Licences - Guidelines and Apparatus licence fees.

- Assignments made in support of area wide land mobile services must comply with all other applicable requirements, including those detailed in Radiocommunications Assignment and Licensing Instruction LM08 (RALI LM08), Spectrum Embargoes and Band Plans.

## Further information

Please contact the Radiocommunications Licensing and Telecommunications Deployment Section.

| BAND Segment / Channel No. | Tx/Rx Frequency |  |
| --- | --- | --- |
|  | 12.5 kHz Bandwidth | 25 kHz Bandwidth |
| VHF HIGH BAND |  |  |
| Segment G Ch 71 | 153.3750 MHz |  |
| Segment I Ch 5 | 154.4125 MHz |  |
| Segment O Ch 38 | 161.4500 MHz |  |
| Segment T Ch 9 | 168.3000 MHz |  |
| Segment V Ch 11 | 172.9250 MHz |  |
| 400 MHz BAND |  |  |
| Segment G Ch 13 | 410.7000 MHz | 410.70625 MHz |
| Segment G Ch 14 | 410.7125 MHz |  |
| Segment P Ch 7 | 450.1000 MHz | 450.10625 MHz |
| Segment P Ch 8 | 450.1125MHz |  |
| Segment W Ch 9 | 462.1125 MHz | 462.11875 MHz |
| Segment W Ch 10 | 462.1250 MHz |  |
| Segment AA Ch 33 | 471.6250 MHz | 471.63125 MHz |
| Segment AA Ch 34 | 471.6375 MHz |  |
| Segment AA Ch 55 | 471.9000 MHz | 471.90625 MHz |
| Segment AA Ch 56 | 471.9125 MHz |  |
| Segment CC Ch 21 | 475.0500 MHz | 475.05625 MHz |
| Segment CC Ch 22 | 475.0625 MHz |  |
| Segment HH Ch 25 | 485.1000 MHz | 485.10625 MHz |
| Segment HH Ch 26 | 485.1125 MHz |  |
| Segment KK Ch 17 | 495.0000 MHz | 495.00625 MHz |
| Segment KK Ch 18 | 495.0125 MHz |  |
| Segment QQ Ch 27 | 509.8750 MHz | 509.88125 MHz |
| Segment QQ Ch 28 | 509.8875 MHz |  |

Table 1: Frequencies reserved for single frequency area wide operation

| BAND Paired Segments / Channel No. | Tx / Rx Frequency Pair |  |
| --- | --- | --- |
|  | 12.5 kHz Band Width | 25 kHz Band Width |
| VHF HIGH BAND |  |  |
| Segment B/H Ch 20 | 149.3750/153.9750 MHz (6.25 kHz bandwidth only) |  |
| Segment E/J Ch 52 | 150.7000/155.3000 MHz |  |
| Segment R/M Ch 74 | 163.8125/159.2125 MHz |  |
| 400 MHz BAND |  |  |
| Segment X/S Ch 273 | 465.9125/455.9125 MHz | 465.91875/455.91875 MHz |
| Segment X/S Ch 274 | 465.9250/455.9250 MHz |  |
| Segment Z/DD Ch 81 | 471.0000/476.2000 MHz | 471.00625/476.20625 MHz |
| Segment Z/DD Ch 82 | 471.0125/476.2125 MHz |  |
| Segment BB/FF Ch 165 | 474.2750/479.4750 MHz | 474.28125/479.48125 MHz |
| Segment BB/FF Ch 166 | 474.2875/479.4875 MHz |  |
| Segment II/GG Ch 95 | 486.3750/481.1750 MHz | 486.38125/481.18125 MHz |
| Segment II/GG Ch 96 | 486.3875/481.1875 MHz |  |
| Segment JJ/LL Ch 191 | 492.3750/497.5750 MHz | 492.38125/497.58125 MHz |
| Segment JJ/LL Ch 192 | 492.3875/497.5875 MHz |  |
| Segment OO/TT Ch 95 | 506.1750/516.1750 MHz | 506.18125/516.18125 MHz |
| Segment OO/TT Ch 96 | 506.1875/516.1875 MHz |  |
| Segment PP/UU Ch 61 | 507.7750/517.7750 MHz | 507.78125/517.78125 MHz |
| Segment PP/UU Ch 62 | 507.7875/517.7875 MHz |  |
| Segment NN/SS Ch 91 | 502.1250/512.1250 MHz |  |
| Segment NN/SS Ch103 | 502.2750/512.2750 MHz |  |
| Segment NN/SS Ch 115 | 502.4250/512.4250 MHz |  |
| Segment NN/SS Ch 127 | 502.5750/512.5750 MHz |  |
| Segment NN/SS Ch 139 | 502.7250/512.7250 MHz |  |

Table 2: Frequencies reserved for two frequency area wide operation

Attachment 1

SPECTRUM ACCESS AREAS

| Area ID | Name | Density Area |
| --- | --- | --- |
| 1 | Australia Wide | High/Medium/Low/Remote |
| 2 | New South Wales | High/Medium/Low/Remote |
| 3 | Victoria | High/Low/Remote |
| 4 | Queensland | High/Low/Remote |
| 5 | South Australia | Medium/Low/Remote |
| 6 | Western Australia | Medium/Low/Remote |
| 7 | Tasmania | Low/Remote |
| 8 | Northern Territory | Low/Remote |
| 9 | Antarctica | Remote |
| 10 | Norfolk Island | Remote |
| 11 | Cocos/Keeling Island | Remote |
| 12 | Christmas Island | Remote |
| 16 | Macquarie Island | Remote |
| 17 | Heard Island | Remote |
| 18 | Australian Capital Territory | Low |
| 36 | Lord Howe Island | Remote |
| 68 | Brisbane/Gold Coast HD | High |
| 69 | Newcastle MD | Medium |
| 70 | Sydney Wollongong HD | High |
| 71 | Melbourne/Geelong HD | High |
| 72 | Adelaide MD | Medium |
| 73 | Perth MD | Medium |
| 74 | Low and Remote Density Areas | Low/Remote |
| 75 | Remote Density Area | Remote |
| 76 | Low Density Areas | Low |

Table A1: Spectrum access and corresponding licence density areas

Table A1-1 provides a list of spectrum access areas for which area wide operation may be authorised.  The licence fee density area(s), corresponding to each spectrum access area, are identified in the corresponding ‘Density Area’ column.  Fee density areas are defined in the Apparatus licence fees.  The Area ID is a unique identification number used in the ACMA’s licensing system to reference a particular spectrum access area.

Attachment 2

# SPECIAL CONDITIONS TEXT

Conditions of operation which apply to an individual licence are printed on the apparatus licence under the heading ‘Special Conditions’.

The ACMA’s policy is that an accredited person should specify on the Frequency Assignment Certificate all the conditions the frequency assignment will be subject to, including those conditions required by ACMA documentation.  Information about this policy is available on the ACMA website.

Special Condition 27 – no interference / no protection

The following special condition must be applied to all licences authorising the operation of an area wide land mobile service:

No interference shall be caused to any radiocommunication station or service and no protection from interference by such stations or services shall be afforded.

Special Condition LM02 – high power operation

Where the area of operation encompasses more than one area density, of which one is high density or medium density, the service is to be licensed at a maximum EIRP of 8.3 Watts.  Operation at higher powers (maximum 83 Watts) outside of high density and medium density areas is authorised through application of the following condition:

To the extent permitted by the licensed area of operation, the land (base) station authorised under this licence may operate at a maximum equivalent isotropic radiated power (EIRP) of 83 Watts when operating in low or remote density areas, as defined in the Apparatus Licence Fee Schedule.

Special Condition LM03 – siting of 400 MHz area wide services

If the licence authorises the operation of a Land Mobile System in the 400 MHz band at powers exceeding 8.3 Watts EIRP, the following special condition is to be applied to the licence:

A land (base) station authorised under this licence, when operating with an equivalent isotropic radiated power (EIRP) exceeding 8.3 Watts EIRP, shall not be sited within 200 metres of a radiocommunications site with licensed 400 MHz band services, as recorded on the Register of Radiocommunications Licences (RRL).  Reference to 400 MHz band services is taken to mean radiocommunications services licensed in accordance with Radiocommunications Assignment and Licensing Instruction MS22 (the 400 MHz Plan).

Special Condition LM04 – siting of VHF (high band) area wide services

If the licence authorises the operation of a Land Mobile System in the VHF high band at powers exceeding 8.3 Watts EIRP, the following special condition is to be applied to the licence:

A land (base) station authorised under this licence, when operating with an equivalent isotropic radiated power (EIRP) exceeding 8.3 Watts EIRP, shall not be sited within 200 metres of a radiocommunications site with licensed VHF high band services, as recorded on the Register of Radiocommunications Licences (RRL).  Reference to VHF high band services is taken to mean radiocommunications services licensed in accordance with the VHF High Band Frequency Band Plan (148 – 174 MHz).

Table A2-1 below lists the maximum licensed power and corresponding special condition(s), applicable to each spectrum access area for which area wide operation may be authorised.

Attachment 2

(Continued)

| Area ID | Name | Density Area | Maximum Licensed EIRP | Special Conditions# |
| --- | --- | --- | --- | --- |
| 1 | Australia Wide | High/Medium/Low/Remote | 8.3 | 27, LM02, LM03/04 |
| 2 | New South Wales | High/Medium/Low/Remote | 8.3 | 27, LM02, LM03/04 |
| 3 | Victoria | High/Low/Remote | 8.3 | 27, LM02, LM03/04 |
| 4 | Queensland | High/Low/Remote | 8.3 | 27, LM02, LM03/04 |
| 5 | South Australia | Medium/Low/Remote | 8.3 | 27, LM02, LM03/04 |
| 6 | Western Australia | Medium/Low/Remote | 8.3 | 27, LM02, LM03/04 |
| 7 | Tasmania | Low/Remote | 83 | 27, LM03/04 |
| 8 | Northern Territory | Low/Remote | 83 | 27, LM03/04 |
| 9 | Antarctica | Remote | 83 | 27, LM03/04 |
| 10 | Norfolk Island | Remote | 83 | 27, LM03/04 |
| 11 | Cocos/Keeling Island | Remote | 83 | 27, LM03/04 |
| 12 | Christmas Island | Remote | 83 | 27, LM03/04 |
| 16 | Macquarie Island | Remote | 83 | 27, LM03/04 |
| 17 | Heard Island | Remote | 83 | 27, LM03/04 |
| 18 | Australian Capital Territory | Low | 83 | 27, LM03/04 |
| 36 | Lord Howe Island | Remote | 83 | 27, LM03/04 |
| 68 | Brisbane/Gold Coast HD | High | 8.3 | 27 |
| 69 | Newcastle MD | Medium | 8.3 | 27 |
| 70 | Sydney Wollongong HD | High | 8.3 | 27 |
| 71 | Melbourne/Geelong HD | High | 8.3 | 27 |
| 72 | Adelaide MD | Medium | 8.3 | 27 |
| 73 | Perth MD | Medium | 8.3 | 27 |
| 74 | Low and Remote Density Areas | Low/Remote | 83 | 27, LM03/04 |
| 75 | Remote Density Area | Remote | 83 | 27, LM03/04 |
| 76 | Low Density Areas | Low | 83 | 27, LM03/04 |

Table A2-1: Maximum licensed power and corresponding special condition(s) for each spectrum access area.

# Where LM03/04 is indicated, apply either special condition LM03 or LM04, whichever is applicable.

Arrangements for data services employing high duty cycle, including Differential Global Positioning Systems (DGPS), are detailed in Frequency Assignment Practice: Guideline No. 2.

In this guideline, reference to density areas is taken to mean the density areas defined in the Apparatus licence fees.

The 400 MHz band frequencies are identified in Table 2 (Appendix A) of the 400 MHz Plan (RALI MS22).