Proposed Amendments to RALI FX3: Protection Ratios Assumptions and Methodology

Spectrum Planning Report SPP 2014/07

JuLY 2014

Canberra Red Building Benjamin Offices Chan Street Belconnen ACT PO Box 78 Belconnen ACT 2616 T +61 2 6219 5555 F +61 2 6219 5353; Melbourne Level 32 Melbourne Central Tower 360 Elizabeth Street Melbourne VIC PO Box 13112 Law Courts Melbourne VIC 8010 T +61 3 9963 6800 F +61 3 9963 6899; Sydney Level 5 The Bay Centre 65 Pirrama Road Pyrmont NSW PO Box Q500 Queen Victoria Building NSW 1230 T +61 2 9334 7700 1800 226 667 F +61 2 9334 7799.

Copyright notice http://creativecommons.org/licenses/by/3.0/au/ With the exception of coats of arms, logos, emblems, images, other third-party material or devices protected by a trademark, this content is licensed under the Creative Commons Australia Attribution 3.0 Licence. We request attribution as: © Commonwealth of Australia (Australian Communications and Media Authority) 2013. All other rights are reserved. The Australian Communications and Media Authority has undertaken reasonable enquiries to identify material owned by third parties and secure permission for its reproduction. Permission may need to be obtained from third parties to re-use their material. Written enquiries may be sent to: Manager, Editorial and Design PO Box 13112 Law Courts Melbourne VIC 8010 Tel: 03 9963 6968 Email: candinfo@acma.gov.au

1	Introduction	1

2	Protection Ratio Methodology	2

2.1	Co-channel Protection Ratio calculations	2

2.1.1	Multipath fading	2

2.1.2	Rain fading	3

2.1.3	Correction Factors calculations	3

2.2	Adjacent Channel Protection Ratio calculations	4

2.2.1	Adjacent Channel Protection Ratio Assumption	4

2.2.2	Frequency Dependant Rejection	4

3	Equipment Performance Assumptions	6

3.1	Transmitter assumptions	6

3.1.1	Radiofrequency spectrum mask	6

3.1.2	Spectral efficiency classes	6

3.1.3	Emission masks	7

3.2	Receiver assumptions	8

3.2.1	Receiver threshold and noise figure	8

3.2.2	Receiver selectivity	9

4	Bibliography	10

Appendix A Transmitter and Receiver Characteristics	11

Appendix B Transmitter Emission Masks and Receiver Selectivity Graphs	16

# Introduction

This paper outlines the methodology and assumptions used to calculate the protection ratios in support of the June 2014 proposed amendments to RALI FX 3 Appendix 1 “RF Channel Arrangements and Assignments Instructions” (1).  For details of the proposed changes refer to SP 2014/05, “Proposed changes to channel arrangements for microwave fixed point-to-point links” (2) and supplement “Proposed amendments to RALI 3 Appendix 1 “RF Channel Arrangements and Assignments Instruction” (3).

Consistent with the approach used in first developing RALI FX 3, for new channel rasters protection ratios have been calculated by first determining the co-channel protection for a notional path length and then using  frequency dependent rejection (FDR) technique as a model of the interference between a transmitter and receiver calculate protections for various frequency offsets.

Calculation of co-channel protection ratios requires assumptions about required fade margin, receiver noise floor, receive threshold level (receiver selectivity) and transmitted emission spectral density.

The required information has been sourced from ITU-R and ETSI documents.

To calculate protections between new channel rasters and existing channel raster, the FDR approach has been used.  For calculation purposes, for existing channel rasters receiver selectivity and transmitted emission spectral density have been determined (reversed engineered) using existing protection ratios and ITU-R and ETSI information.

# Protection Ratio Methodology

This section details the methodology used to calculate the protection ratios for new channel rasters. The co-channel protection ratio is first calculated and then adjacent channel protection ratio determined from it.

## Co-channel Protection Ratio calculations

As outlined in earlier versions of RALI FX3 co-channel protection is calculated in terms of the allowable level of interference which degrades a typical receiver threshold and path fade margin.  The equation is:

PR=Tmx+FM-Iall

where

Tmx is receiver threshold for specified S/N and BER  [dBm],

FM is fade margin in [dB] and

Iall  is minimum allowable interference level [dBm].

With reference ETSI EN 302 217-2-2 (4)  Tmx (RSL - receiver signal level in the ESTI standard) can be determined for a specified bandwidth, spectral efficiency (spectral class) and required BER.

The maximum allowable interference power can be calculated as follows:

Iall=10log10kTB-IN+NF

where

k is Boltzmann constant [W/(Hz K)]

T is absolute temperature [K],

B is receiver bandwidth [Hz],

NF is noise figure [dB],

I/N is interference to noise level [dB].

For the range of frequency bands 1-50 GHz noise figures (NF) values can be found in the ETSI TR 101 854 (5). For new channel rasters, protection ratio calculation has been based on 10-6 BER, compared to 10-3 BER when RALI FX 3 was first developed. The interference-to-noise (I/N) value generally depends of frequency bands and sharing conditions. It is taken to be 6 dB as recommended in the ITU-R F.758-5 (6).

Fading Mechanisms

Two fading mechanisms need to be considered when determining fade margin (FM): multipath and rain fading. Multipath fading describes the combination of various clear-air fading mechanisms such as surface and atmospheric multipath, beam spreading etc. Rain fading occurs as a result of absorption and scattering by hydrometeors as rain, snow, hail and fog. Multipath fading is the dominant fading factor below 10 GHz. Rain fading is the dominant fading factor for frequencies above 18 GHz. Between 10 GHz and 18 GHz both fading mechanisms need to be considered.

Fade margin are to be calculated for the notional path lengths as listed in

### Multipath fading

As outlined in Appendix 4 of RALI FX 3, ITU-R Rec.530-6 (7) is used to calculate multipath fade margin and hence path length correction factors for paths length different than the benchmarked notional path length.  While path length correction factors will be continued to reference against ITU-R Rec.530-6, for the new channel rasters co-channel protections ratios will be calculated using the more recent  ITU-R Rec 530-15 [10].  It is acknowledged that a review of RALI FX 3 is required to consider the latest ITU-R documentation; however that is a larger task that will be considered at later date.

From Rec 530-15 the fade margin can be calculated as follows:

FM=10log10K*f0.8*d3.1*(1+εp)-1.29-10log10pW-0.0089hL

where:

hL is altitude of the lower antenna (above sea level) [m];

|ɛp |=|hr-he|/d is the magnitude of the path inclination in mrad;

hr and he are emitting and receiving antenna heights (above sea level) [m];

K is geoclimatic factor calculated by using the point refractivity gradient (dN1).

K=10-4.6-0.0027dN1

where:

dN1 is point refractivity gradient in the lowest 65 m of the atmosphere not exceeded for 1% of an average year. It is provided on a 1.5 grid in latitude and longitude in Recommendation ITU-R P.453 (8).

### Rain fading

Rain attenuation, R [db/km] depends of frequency, polarisation and rain rate R0.01 and is given by:

γR=αR0.01β

Parameters α and β are provided in Rec. ITU-R P.838 (9) and rain rate statistics are detailed in Rec. ITU-R P.837 (10).

As a rain is not uniform, only a part of path is affected by rain, the effective path length has to be calculated.

From the Rec ITU-R P.530-6, the effective path length factor can be calculated as:

r=11+dd0   and d0=35e-0.015R0.01

From the Rec ITU-R P.530-15, the effective path length factor can be calculated as:

r=10.477d0.633R0.010.073αf0.123-10.579(1-e-0.024d)

Finally, an estimate of the path attenuation exceeded for 0.01% of the time is given by:

A0.01=γRdeff=γRdr

For other percentage of time in the range 0.001% to 1%, the attenuation can be calculated from Rec ITU-R 530.

For path length and geoclimatic factor that are different from reference values, the correction factors are then used to calculate the protection ratios.

### Correction Factors calculations

No changes to path length correction factors, the existing graphs continue to apply.

## Adjacent Channel Protection Ratio calculations

The adjacent channel protection ratio is calculated by subtracting the frequency dependant rejection (FDR) factor from co-channel protection ratio.

PRadj-ch=PRco-ch-FDR

### Adjacent Channel Protection Ratio Assumption

The approach for determining the spectral separation for the specification of protection ratios is the same as has been used previously in RALI FX 3. The FDR calculations are calculated:

Using transmitter emission masks and receiver selectivity curves as detailed in section 3;

at frequency offsets referenced to channel centre frequencies;

for closest possible co-channel permutation to the furthest spectral limit of both transmitter emission mask and receiver selectivity curve.  These are modelled out to ±250 precent of channel bandwidth around the channel centre frequency.  Hence the maximum offset between channel centre is 2.5 times the transmitter and receiver bandwidth; and

for all possible discrete channel offsets between the above two points.

### Frequency Dependant Rejection

Recommendation ITU-R SM.337 (11) defines frequency dependant rejection (FDR) as a measure of the interference coupling mechanism between interferer and receiver.  The goal is to provide an estimate of the minimum frequency and distance separation between interferer and receiver required for acceptable receiver performance.  The FDR is defined by:

FDR∆f=10log0∞Pfdf0∞P(f)H(f+∆f)2df

where

P(f) is power spectral density (emission mask) of the interfering signal [W/Hz],

H(f) is frequency response of the receiver (receiver selectivity) and

∆f=ft-fr  is frequency offset between transmitter’s – ft and receivers’ frequency - fr. [Hz]

The FDR can be divided into two terms, the on-tune rejection (OTR) and off-frequency rejection (OFR) as follows:

FDR∆f=OTR+OFR(∆f)

where

OTR=10log0∞Pfdf0∞PfHf+∆f2df

and

OFR(∆f)=10log0∞PfHf2df0∞PfHf+∆f2df

The on-tune rejection, also called the correction factor, can often be approximated by:

OTR≈20logBTBR  for BR  BT

where

BT and BR are interferer and receivers bandwidths [MHz] respectively.

Transmitter power spectral density masks have been determined using ETSI EN 302 217 (4) and receiver selectivity using ETSI TR 101 854 (5).

# Equipment Performance Assumptions

This section details the assumptions related to the applicability of the interference protection methodology. A summary of parameters is at Appendix A

## Transmitter assumptions

### Radiofrequency spectrum mask

The emission masks, proposed for the updated version of Appendix 1 of RALI FX3, are:

applicable for digital services only;

based on the ETSI EN 302 217 standard, which lists a set of radiofrequency spectrum masks in accordance with their associated spectral efficiency classes;

determined by applying already defined protection ratios from Appendix 1 of RALI FX3 for currently available channel arrangements in order to maintain consistency for all current and future licensees,

specified, for proposed wider channel arrangements, using a higher order spectral efficiency where consistent with the channel arrangements in other frequency bands;

defined for the limits up to and including ±250 precent of channel bandwidth around the channel centre frequency ;

for 50 MHz channel, as not been defined by ETSI standards, it has been obtained from the 56 MHz mask by scaling with the 50/56 factor;

specified for 80 MHz channel. Based on CEPT/ERC/REC 12-06 E Recommendation (12), the same spectral efficiency (class 5) as for 40 MHz should be adopted for 80 MHz channels.

### Spectral efficiency classes

The ETSI EN 302 217 standard specifies different classes of equipment in accordance to their spectral efficiency. The spectral efficiency classes are defined for typical modulation formats and are limited by a “minimum radio interface capacity (RIC) density” (Mbit/s/MHz).

The list of spectral efficiency classes used for protection ratio evaluation, their minimum RIC density and typical modulation are given in Table 1.

Table 1 — Spectral efficiency classes details

Spectral efficiency class; Minimum RIC density (Mbit/s/MHz); Modulation. 1*; 0.57; 2-states modulation schemes (2 FSK, 2 PSK etc). 2; 1.14; 4-states modulation schemes (4 FSK, 4 QAM etc). 3; 1.7; 8-states modulation schemes (8 PSK etc). 4L; 2.28; 16-states modulation schemes (16 QAM, 16 APSK etc). 4H; 3.5; 32-states modulation schemes (32 QAM, 32 APSK etc). 5L; 4.2; 64-states modulation schemes (64 QAM etc). 5H; 4.9; 128-states modulation schemes (128 QAM etc).

* Note: class 1 spectral efficiency is below the minimum requirement defined by the RALI FX3.

### Emission masks

Emission masks are specified in ETSI EN 302 217 standard.  For the purpose of coordination, the emission masks are determined as minimum requirements as shown in Table 2.  Emission masks are show at Appendix B.

Table 2 — Recommended emission masks

Frequency band; Channel width; Recommended class. 6 GHz; 29.65 MHz; class 4L. 59.3 MHz; class 4H. 6.7 GHz; 40 MHz; class 5. 80 MHz; class 5. 7.5 GHz; 7 MHz; class 1-3. 14 MHz; class 4L. 28 MHz; class 4L. 8 GHz; 29.65 MHz; class 4L. 59.3 MHz; class 4H. 10 GHz; 29.65 MHz; class 4L. 59.3 MHz; class 4H. 11 GHz; 40 MHz; class 5. 80 MHz; class 5. 13 GHz; 28 MHz; class 4L. 15 GHz; 7 MHz; class 1-3. 14 MHz; class 4L. 28 MHz; class 4L. 18 GHz; 7.5 MHz; class 1-3. 13.75 MHz; class 4L. 27.5 MHz; class 4L. 55 MHz; class 4H. 22 GHz; 7 MHz; class 1-3. 14 MHz; class 4L. 28 MHz; class 4L. 50 MHz; class 4H. 56 MHz; class 4H. 28 GHz; 28 MHz; class 5LB. 56 MHz; class 5LB. 128 MHz; class 5LB. 38 GHz; 7 MHz; class 1-3. 14 MHz; class 4L. 28 MHz; class 4L. 50 GHz; 40 MHz; class 5.

## Receiver assumptions

### Receiver threshold and noise figure

ETSI EN 302 217-2 provides receiver threshold, as receiver input signal level RSL, in terms of particular receiving bandwidth.  Values used are show in Appendix A, Table 6.

For the range of frequency bands 1-50 GHz receiver noise figures (NF) values can be found in the ETSI TR 101 854. For new channel rasters, protection ratio calculation has been based on 10-6 BER. Values used are show in Appendix A, Table 5.

### Receiver selectivity

For the purpose of the protection ratio calculation, the receiver selectivity has been derived from the corresponding emission mask as specified in the ETSI TR 101 854. The ETSI technical report specifies a conservative and more realistic approach.  Following the analysis undertaken by the ACMA on the resulting protection ratio calculations for both the conservative and more realistic receiver selectivity, the proposed protection ratios are based on the more realistic receiver approach.

The resulting receiver masks are show at Appendix B.  For these, the receiver filter is a combination of a Nyquist filtering and 2 straight lines until the limits of 2.5 x channel bandwidth.

Nyquist receiver filter is defined by equation:

HRXf=121+cosπ2rofffn-1+rof       f<fn1-roffn1-rof≤f≤fn1+roff>fn1+rof

where

rof  is cosine roll-of factor and

fn  is Nyquist frequency defined by the following expression:

fn=payload2*N* 100+overheadoverhaed

where

N equals to modulation order 2N.

Details related to payload, overhead and modulation order are provided in the Appendix A, Table 4.

# Bibliography

1. Australian Communications and Media Authority. Microwave Fixed Services Frequency Coordination. 2006. RALI FX 3.

2. Australian Communicaitons and Media Authority. Proposed changes to channel arrangements for microwave fixed point-to-point links. 2014. SPP 2014/05.

3. Australian Communications and Media Authority. Proposed amendments to RALI 3 Appendix 1 “RF Channel Arrangements and Assignments Instruction". 2014. SPP 2014/06.

4. ETSI. Fixed Radio System; Characteristics and requirements for point-to-point equipment and antennas; Part 2-2: Digital systems operating in frequency bands where frequency coordination is applied; . Harmonized EN covering the essential requirements of article 3.2 of the R&TTE Directive. ETSI EN 302 217-2-2 .

5. —. Fixed Radio Systems; Point-to-point equipment; Derivation of receiver interference parameters useful for planning fixed service point-to-point systems operating different equipment classes and/or capacities. ETSI TR 101 854.

6. ITU-R Reccomendation F.758-5. System parameters and considerations in the development of criteria for sharing or compatibility between digital fixed wireless systems in the fixed service and systems in other services and other sources of interference. s.l. : ITU-R. F.758-5.

7. ITU-R Recommendation P.530. Propagation data and prediction methods required for design of terrestrial line-of-sight systems. s.l. : ITU. P.530.

8. ITU-R Recommendation P.453-10. The radio refractive index: its formula and refractivity data. s.l. : ITU. P.453-10.

9. ITU-R Recommendation P.838-3. Specific attenuation model for rain for use in prediction methods. s.l. : ITU. P.838-3.

10. ITU-R Recommendation P.837-6. Characteristics of precipitation for propagation modelling. s.l. : ITU. P.837-6.

11. ITU-R Recommendation SM.337-6. Frequency and distance separation. s.l. : ITU. SM.337-6.

12. CEPT/ERC/RECOMMENDATION 12-06 E. Preferred channel arrangements for fixed service systems operating in the frequency band 10.7 - 11.7 GHz. s.l. : CEPT, Rome 1996, revised Rottach Egern, February 2010. ERC 12-06E.

# Appendix A Transmitter and Receiver Characteristics

| Table 3 – Emission masks details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Channel width (MHz) | Modulation |  | Recommended class | k1 | f1 |  | k2 |  | f2 |  | k3 |  | f3 |  | k4 |  | f4 |  | k5 |  | f5 |  | k6 |  | f6 |  | k7 |  | f7 |  |
| 6 GHz | 29.65 | 16 QAM, 16 APSK | 4L | 2 | 12.8 |  |  | -27 |  | 17 |  | -55 |  | 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 59.3 | 32 QAM, 32 APSK | 4H | 2 | 24 |  |  | -10 |  | 30 |  | -33 |  | 33.6 |  | -40 |  | 70 |  | -55 |  | 110 |  |  |  |  |  |  |  |  |  |
| 6.7 GHz | 40 | 64QAM | 5 | 2 | 18 |  |  | -10 |  | 21.5 |  | -32 |  | 24.5 |  | -35 |  | 29 |  | -45 |  | 57 |  | -55 |  | 77 |  | -55 |  | 77 |  |
|  | 80 | 64QAM | 5 | 2 | 36 |  |  | -10 |  | 43 |  | -32 |  | 49 |  | -35 |  | 58 |  | -45 |  | 114 |  | -55 |  | 154 |  | -55 |  | 154 |  |
| 7.5 GHz | 7 | 4 FSK, 4QAM and 8PSK | 2,3 | 1 | 3.4 |  |  | -23 |  | 4.2 |  | -23 |  | 6.8 |  | -45 |  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14 | 16 QAM, 16 APSK | 4L | 1 | 6.4 |  |  | -28 |  | 8.8 |  | -55 |  | 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 GHz | 29.65 | 16QAM, 16APSK etc | 4L | 2 | 12.8 |  |  | -27 |  | 17 |  | -55 |  | 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 59.3 | 32QAM, 32APSK etc | 4H | 2 | 24 |  |  | -10 |  | 30 |  | -33 |  | 33.6 |  | -40 |  | 70 |  | -55 |  | 110 |  |  |  |  |  |  |  |  |  |
| 10 GHz | 7 | 4 FSK, 4QAM and 8PSK | 2,3 | 1 | 3.4 |  |  | -23 |  | 4.2 |  | -23 |  | 6.8 |  | -45 |  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14 | 16 QAM, 16 APSK | 4L | 1 | 6.4 |  |  | -28 |  | 8.8 |  | -55 |  | 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 GHz | 40 | 64QAM | 5 | 2 | 18 |  |  | -10 |  | 21.5 |  | -32 |  | 24.5 |  | -35 |  | 29 |  | -45 |  | 57 |  | -55 |  | 77 |  |  |  |  |  |
|  | 80 | 64QAM | 5 | 2 | 36 |  |  | -10 |  | 43 |  | -32 |  | 49 |  | -35 |  | 58 |  | -45 |  | 114 |  | -55 |  | 154 |  |  |  |  |  |
| 13 GHz | 28 | 16 QAM, 16 APSK | 4L | 2 | 12.8 |  |  | -27 |  | 17 |  | -55 |  | 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 GHz | 7 | 4 FSK, 4QAM and 8PSK | 2,3 | 1 | 3.4 |  |  | -23 |  | 4.2 |  | -23 |  | 6.8 |  | -45 |  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14 | 16 QAM, 16 APSK | 4L | 1 | 6.4 |  |  | -28 |  | 8.8 |  | -55 |  | 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 | 16 QAM, 16 APSK | 4L | 2 | 12.8 |  |  | -27 |  | 17 |  | -55 |  | 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 GHz | 7.5 | 4 FSK, 4QAM and 8PSK | 2, 3 | 3.4 | -23 |  |  | 4.2 |  | -23 |  | 6.8 |  | -45 |  | 12 |  | 3.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13.75 | 16 QAM, 16 APSK | 4L | 1 | 6.4 |  |  | -28 |  | 8.8 |  | -55 |  | 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27.5 | 16 QAM, 16 APSK | 4L | 2 | 12.8 |  |  | -27 |  | 17 |  | -55 |  | 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 55 | 32 QAM, 32 APSK | 4H | 2 | 24 |  |  | -10 |  | 30 |  | -33 |  | 33.6 |  | -40 |  | 70 |  | -50 |  | 96.6 |  |  |  |  |  |  |  |  |  |
| 22 GHz | 7 | 4 FSK, 4QAM and 8PSK | 2,3 | 1 | 3.4 |  |  | -23 |  | 4.2 |  | -23 |  | 6.8 |  | -45 |  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14 | 16 QAM, 16 APSK | 4L | 1 | 6.4 |  |  | -28 |  | 8.8 |  | -50 |  | 24.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 | 16 QAM, 16 APSK | 4L | 2 | 12.8 |  |  | -27 |  | 17 |  | -50 |  | 49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50 | 32 QAM, 32 APSK | 4H | 2 | 21.43 |  |  | -10 |  | 26.79 |  | -33 |  | 30 |  | -40 |  | 62.5 |  | -50 |  | 86.25 |  |  |  |  |  |  |  |  |  |
|  | 56 | 32 QAM, 32 APSK | 4H | 2 | 24 |  |  | -10 |  | 30 |  | -33 |  | 33.6 |  | -40 |  | 70 |  | -50 |  | 96.6 |  |  |  |  |  |  |  |  |  |
| 28 GHz | 28 | 64 QAM | 5LB | 2 | 12 |  |  | -10 |  | 14.5 |  | -32 |  | 15.5 |  | -36 |  | 17 |  | -45 |  | 40 |  | -50 |  | 47 |  |  |  |  |  |
|  | 56 | 64 QAM | 5LB | 2 | 24 |  |  | -10 |  | 29 |  | -32 |  | 31 |  | -36 |  | 34 |  | -45 |  | 80 |  | -50 |  | 94 |  |  |  |  |  |
|  | 112 | 64 QAM | 5LB | 2 | 48 |  |  | -10 |  | 58 |  | -32 |  | 62 |  | -36 |  | 68 |  | -45 |  | 160 |  | -50 |  | 188 |  |  |  |  |  |
| 38 GHz | 7 | 4 FSK, 4QAM and 8PSK | 2,3 | 1 | 3.4 |  |  | -23 |  | 4.2 |  | -23 |  | 6.8 |  | -45 |  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14 | 16 QAM, 16 APSK | 4L | 1 | 6.4 |  |  | -28 |  | 8.8 |  | -50 |  | 24.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 | 16 QAM, 16 APSK | 4L | 2 | 12.8 |  |  | -27 |  | 17 |  | -50 |  | 49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50 GHz | 40 | 64QAM | 5 | 2 | 18 |  |  | -10 |  | 21.5 |  | -32 |  | 24.5 |  | -35 |  | 29 |  | -45 |  | 57 |  | -55 |  | 77 |  |  |  |  |  |

| Table 4 - Receiver characteristics details |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | Channel width (MHz) | Modulation | Class | Payload | Overhead factor | Modulation order |
| 6 GHz | 29.65 | 16QAM, 16APSK etc | 4L | 67 | 20 | 4 |
|  | 59.3 | 32QAM, 32APSK etc | 4H | 197 | 5 | 5 |
| 6.7 GHz | 40 | 64QAM | 5 | 149 | 20 | 6 |
|  | 80 | 64QAM | 5 | 298 | 20 | 6 |
| 7.5 GHz | 7 | 4 FSK, 4QAM and 8PSK | 2,3 | 12.5 | 20 | 3 |
|  | 14 | 16 QAM, 16 APSK | 4L | 34 | 20 | 4 |
| 8 GHz | 29.65 | 16QAM, 16APSK etc | 4L | 67 | 20 | 4 |
|  | 59.3 | 32QAM, 32APSK etc | 4H | 197 | 5 | 5 |
| 10 GHz | 7 | 4 FSK, 4QAM and 8PSK | 2,3 | 12.5 | 20 | 3 |
|  | 14 | 16 QAM, 16 APSK | 4L | 34 | 20 | 4 |
| 11 GHz | 40 | 64QAM | 5 | 149 | 20 | 6 |
|  | 80 | 64QAM | 5 | 298 | 20 | 6 |
| 13 GHz | 28 | 16 QAM, 16 APSK | 4L | 67 | 20 | 4 |
| 15 GHz | 7 | 4 FSK, 4QAM and 8PSK | 2,3 | 12.5 | 20 | 3 |
|  | 14 | 16 QAM, 16 APSK | 4L | 34 | 20 | 4 |
|  | 28 | 16 QAM, 16 APSK | 4L | 67 | 20 | 4 |
| 18 GHz | 7.5 | 4 FSK, 4QAM and 8PSK | 2, 3 | 12.5 | 20 | 3 |
|  | 13.75 | 16 QAM, 16 APSK | 4L | 34 | 20 | 4 |
|  | 27.5 | 16 QAM, 16 APSK | 4L | 67 | 20 | 4 |
|  | 55 | 32 QAM, 32 APSK | 4H | 186 | 10 | 5 |
| 22 GHz | 7 | 4 FSK, 4QAM and 8PSK | 2,3 | 12.5 | 20 | 3 |
|  | 14 | 16 QAM, 16 APSK | 4L | 34 | 20 | 4 |
|  | 28 | 16 QAM, 16 APSK | 4L | 67 | 20 | 4 |
|  | 50 | 32 QAM, 32 APSK | 4H | 168 | 10 | 5 |
|  | 56 | 32 QAM, 32 APSK | 4H | 188 | 10 | 5 |
| 28 GHz | 28 | 64 QAM | 5LB | 100 | 20 | 6 |
|  | 56 | 64 QAM | 5LB | 200 | 20 | 6 |
|  | 112 | 64 QAM | 5LB | 400 | 20 | 6 |
| 38 GHz | 7 | 4 FSK, 4QAM and 8PSK | 2,3 | 12.5 | 20 | 3 |
|  | 14 | 16 QAM, 16 APSK | 4L | 34 | 20 | 4 |
|  | 28 | 16 QAM, 16 APSK | 4L | 67 | 20 | 4 |
| 50 GHz | 40 | 64QAM | 5 | 149 | 20 | 6 |

Table 5 — Additional parameters required for protection ratio calculation

Frequency band (GHz); Path length (km); NF (dB). 6; 50; 4. 6.7; 50; 4. 7.5; 50; 4. 8; 50; 4. 10; 30; 4.5. 11; 30; 4.5. 13; 20; 5. 15; 20; 5. 18; 10; 5. 22; 5; 5.5. 28; 2; 6.5. 38; 2; 7.5. 50; 2; 10.

Notes:

- Path length (d km) is the notional path length used in FX 3 for calculating path length correct factors

- Noise figure from ETSI TR 101 854 V1.2.1 (2005-01), Table 2: Typical Noise Figures (NF) and associated Industrial Margins (IMF), column 2  “Typical Noise Figure (NF) (dB)

| Table 6 – Parameters related to protection ratio calculation for new channels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band (GHz) | Channel width (MHz) | Modulation | S/N based on ITU-R F.1101 (dB) | Fade margin based on ITU-R 530-15 (dB) | Rain fade based on ITU-R 530-15 (p=0.01%,R=80) (dB) |  | Receiver input signal level RSL (upper bound) for BER  10-6 (dBm) | Co-channel protection ratio based on ITU-R 530-15 (dB) | RSL ETSI Reference: ETSI EN 302 217-2-2 v2.1.0 |
| 6 | 59.3 | 32QAM, 32APSK etc | 23.5 | 38.05 |  | - | -68 | 68.1 | Table b.6, Reference index 5, Class 4H |
| 6.7 | 80 | 64QAM | 23.8 - 26.5 | 38.4 |  | - | -66 | 69.2 | Based on Table c.6, Reference index 6, Class 5LB |
| 8 | 59.3 | 32QAM, 32APSK etc | 23.5 | 40.02 |  | - | -68 | 69.1 | Table b.6, Reference index 5, Class 4H |
| 11 | 80 | 64QAM | 23.8 - 26.5 | 32.8 |  | 40.2 | -65 | 70.5 | Based on Table c.6, Reference index 6, Class 5LB |
| 22 | 56 | 32 QAM, 32 APSK | 20.6 - 23.5* | - |  | 35.7 | -67 | 65.5 | Table E.8B, Reference index 5, Class 4H |
| 28 | 28 | 64 QAM | 23.8 - 26.5* | - |  | 27.81 | -63 | 64.6 | Table E.8B, Reference index 7, Class 5HA/5HB |
|  | 56 | 64 QAM | 23.8 - 26.5* | - |  | 27.81 | -60 | 64.6 |  |
|  | 112 | 64 QAM | 23.8 - 26.5* | - |  | 27.81 | -57 | 64.6 |  |

# Appendix B Transmitter Emission Masks and Receiver Selectivity Graphs

THE 6 GHz BAND (5925 - 6425 MHz)

Emission Masks

|  | f1 | f2 | f3 |
| --- | --- | --- | --- |
| Offset (MHz) | 12.8 | 17 | 56 |
| Attenuation (dB) | 2 | -27 | -55 |

|  | f1 | f2 | f3 | f4 | f5 |
| --- | --- | --- | --- | --- | --- |
| Offset (MHz) | 24 | 30 | 33.6 | 70 | 110 |
| Attenuation (dB) | 2 | -10 | -33 | -40 | -55 |

THE 6 GHz BAND (5925 - 6425 MHz)

Receiver Masks

THE 6.7 GHz BAND (6425 - 7110 MHz)

Transmitter Emission Masks

|  | f1 | f2 | f3 | f4 | f5 | f6 |
| --- | --- | --- | --- | --- | --- | --- |
| Offset (MHz) | 18 | 21.5 | 24.5 | 29 | 57 | 77 |
| Attenuation (dB) | 2 | -10 | -32 | -35 | -45 | -55 |

|  | f1 | f2 | f3 | f4 | f5 | f6 |
| --- | --- | --- | --- | --- | --- | --- |
| Offset (MHz) | 36 | 43 | 49 | 58 | 114 | 154 |
| Attenuation (dB) | 2 | -10 | -32 | -35 | -45 | -55 |

THE 6.7 GHz BAND (6425 - 7110 MHz)

Receiver Masks

THE 8 GHz BAND (7725 - 8275 MHz)

Transmitter Emission Masks

|  | f1 | f2 | f3 |
| --- | --- | --- | --- |
| Offset (MHz) | 12.8 | 17 | 56 |
| Attenuation (dB) | 2 | -27 | -55 |

|  | f1 | f2 | f3 | f4 | f5 |
| --- | --- | --- | --- | --- | --- |
| Offset (MHz) | 24 | 30 | 33.6 | 70 | 110 |
| Attenuation (dB) | 2 | -10 | -33 | -40 | -55 |

THE 8 GHz BAND (7725 - 8275 MHz)

Receiver Masks

THE 10 GHz BAND (10.55 - 10.68 GHz)

Transmitter Emission Masks

|  | f1 | f2 | f3 | f4 |
| --- | --- | --- | --- | --- |
| Frequency (MHz) | 3.4 | 4.2 | 6.8 | 12 |
| Attenuation (dB) | 1 | -23 | -23 | -45 |

|  | f1 | f2 | f3 |
| --- | --- | --- | --- |
| Frequency (MHz) | 6.4 | 8.8 | 28 |
| Attenuation (dB) | 1 | -28 | -55 |

THE 10 GHz BAND (10.55 - 10.68 GHz)

Receiver Masks

THE 11 GHz BAND (10.7 - 11.7 GHz)

Transmitter Emission Masks

|  | f1 | f2 | f3 | f4 | f5 | f6 |
| --- | --- | --- | --- | --- | --- | --- |
| Offset (MHz) | 18 | 21.5 | 24.5 | 29 | 57 | 77 |
| Attenuation (dB) | 2 | -10 | -32 | -35 | -45 | -55 |

|  | f1 | f2 | f3 | f4 | f5 | f6 |
| --- | --- | --- | --- | --- | --- | --- |
| Offset (MHz) | 36 | 43 | 49 | 58 | 114 | 154 |
| Attenuation (dB) | 2 | -10 | -32 | -35 | -45 | -55 |

THE 11 GHz BAND (10.7 - 11.7 GHz)

Receiver Masks

THE 15 GHz BAND (14.5 - 15.35 GHz)

Transmitter Emission Masks

|  | f1 | f2 | f3 | f4 |
| --- | --- | --- | --- | --- |
| Frequency (MHz) | 3.4 | 4.2 | 6.8 | 12 |
| Attenuation (dB) | 1 | -23 | -23 | -45 |

|  | f1 | f2 | f3 |
| --- | --- | --- | --- |
| Frequency (MHz) | 6.4 | 8.8 | 28 |
| Attenuation (dB) | 1 | -28 | -55 |

|  | f1 | f2 | f3 |
| --- | --- | --- | --- |
| Frequency (MHz) | 12.8 | 17 | 56 |
| Attenuation (dB) | 2 | -27 | -55 |

THE 15 GHz BAND (14.5 - 15.35 GHz)

Receiver Masks

THE 22 GHz BAND (21.2 - 23.6 GHz)

|  | f1 | f2 | f3 | f4 |
| --- | --- | --- | --- | --- |
| Frequency (MHz) | 3.4 | 4.2 | 6.8 | 12 |
| Attenuation (dB) | 1 | -23 | -23 | -45 |

|  | f1 | f2 | f3 |
| --- | --- | --- | --- |
| Frequency (MHz) | 6.4 | 8.8 | 24.8 |
| Attenuation (dB) | 1 | -28 | -50 |

|  | f1 | f2 | f3 |
| --- | --- | --- | --- |
| Frequency (MHz) | 12.8 | 17 | 49 |
| Attenuation (dB) | 2 | -27 | -50 |

|  | f1 | f2 | f3 |
| --- | --- | --- | --- |
| Frequency (MHz) | 22.8 | 30.4 | 87.5 |
| Attenuation (dB) | 2 | 27 | -50 |

|  | f1 | f2 | f3 | f4 | f5 |
| --- | --- | --- | --- | --- | --- |
| Frequency (MHz) | 24 | 30 | 33.6 | 70 | 96.6 |
| Attenuation (dB) | 2 | -10 | -33 | -40 | -50 |

THE 22 GHz BAND (21.2 - 23.6 GHz)

Receiver Masks

THE 28 GHz BAND (27.5 – 29.5 GHz)

|  | f1 | f2 | f3 | f4 | f5 | f6 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency (MHz) | 12 | 14.5 | 15.5 | 17 | 40 | 47 |
| Attenuation (dB) | 2 | -10 | -32 | -36 | -45 | -50 |

|  | f1 | f2 | f3 | f4 | f5 | f6 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency (MHz) | 24 | 29 | 31 | 34 | 80 | 94 |
| Attenuation (dB) | 2 | -10 | -32 | -36 | -45 | -50 |

|  | f1 | f2 | f3 | f4 | f5 | f6 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency (MHz) | 48 | 58 | 62 | 68 | 160 | 188 |
| Attenuation (dB) | 2 | -10 | -32 | -36 | -45 | -50 |

THE 28 GHz BAND (27.5 – 29.5 GHz)

Receiver Masks

Frequency dependent rejection:  a measure of the rejection produced by the receiver selectivity curve on an unwanted transmitter emission spectra.