frequency plan for the VHF bands

70 - 87.5 MHz and 148 - 174 MHz

Amendment History

| Date | Comments |
| --- | --- |
| August 2015 | Initial draft covering material taken from the VHF Mid Band Frequency Band Plan and the VHF High Band Frequency Band Plan |
| September 2015 | Finalised RALI following public consultation. See Issue for Comment: Sunsetting of the VHF mid band, VHF high band and 1.9 GHz frequency band plans. |
| October 2015 | Correction of typographical errors in 1st channel centre frequency for VHF High Band in table 2. |
| June 2016 | Consultation draft proposing changes to the VHF High band to: - Incorporate changes for paging services (refer proposed updates to LM 2). - Allow outside of HSD areas segments G and V to also be used for low powered two frequency land mobile systems. - Provide additional flexibility by allowing use of trunking segments in low and remote areas for non-trunked spectrally efficient technologies. |
| August 2016 | Finalised RALI following consultation (IFC 17/2016) |

Suggestions for improvements to Radiocommunications Assignment and Licensing Instruction MS 42 may be addressed to The Manager, Spectrum Engineering, ACMA at PO Box 78, Belconnen, ACT, 2616, or by e-mail to freqplan@acma.gov.au.  It would be appreciated if notification to ACMA of any inaccuracy or ambiguity found be made without delay in order that the matter may be investigated and appropriate action taken.

# Table of Contents

1	Purpose	1

1.1	Commencement	1

2	Spectrum arrangements	1

2.1	The VHF Mid-Band (70 to 87.5 MHz)	1

2.2	The VHF High-band (148 to 174 MHz)	2

3	RALI Authorisation	3

Appendix A: Allocation and Channelling Arrangements	4

frequency plan for the VHF bands 70 - 87.5 MHz and 148 - 174 MHz

# Purpose

The purpose of this Radiocommunications Assignment and Licensing Instruction (RALI) is to set out spectrum planning arrangements for radiocommunications services in the frequency bands 70 to 87.5 MHz (known as the VHF Mid Band) and 148-174 MHz (known as the VHF High Band).  The RALI specifies the service allocations including the frequency limits of each allocation segment, and the channelling arrangements for the two bands.  The RALI also sets out the High Spectrum Demand (HSD) and Rural areas for these bands across Australia.

This RALI replaces the VHF Mid Band Frequency Band Plan (70 to 87.5 MHz) 1991  and the VHF High Band Frequency Band Plan (148 to 174 MHz) 1991 which ceased (will cease) on 1 October 2015 under “sunsetting” provisions of the Legislative Instruments Act 2003.

In making decisions, accredited frequency assigners and the ACMA’s officers should take all relevant factors into account and decide each case on its merits.  Issues relating to this document that appear to fall outside the enunciated policy should be referred to the Manager, Spectrum Engineering Section, PO Box 78, Belconnen, ACT, 2616, or by e-mail to freqplan@acma.gov.au.

## Commencement

This RALI will commence on cessation of the VHF Mid Band Frequency Band Plan (70 to 87.5 MHz) 1991 and the VHF High Band Frequency Band Plan (148 to 174 MHz) 1991 on 1 October 2015.

# Spectrum arrangements

This section set out the spectrum allocation and channelling arrangements for the frequency bands 70 to 87.5 MHz (the VHF Mid Band) and 148-174 MHz (the VHF High Band).  A service may be operated in the 70 to 87.5 MHz and 148-174 MHz frequency bands provided its operation would be consistent with the requirements listed in this RALI and the Australian Radiofrequency Spectrum Plan (the Spectrum Plan).

## The VHF Mid-Band (70 to 87.5 MHz)

Spectrum allocation and channelling arrangements in the VHF Mid-Band are contained in Table 1 of Appendix A and illustrated in Figure 1 of the Appendix.  Services are to be assigned in accordance with the arrangement contained in Table 1.

The following exceptions to the requirements of Table 1 of Appendix A apply:

- Existing services may continue to operate as authorised under the VHF Mid Band Frequency Band Plan (70 to 87.5 MHz) 1991.

- New services may operate under channelling arrangements not in accordance with Appendix A Table 1 of this RALI as a primary service provided that the service is considered to be spectrally efficient.  In determining the spectrum efficiency of a service, without limiting the range of matters which may be taken into account, the following matters may be considered:

- occupied bandwidth;

- adjacent channel performance;

- the distance from the transmitter that the channel may be used again without causing harmful interference; and

- the impact that introduction of the service will have upon existing services.

- New services assigned outside a High Spectrum Density (HSD) area specified in Appendix A Table 3 may operate under channelling arrangements not in accordance with Appendix A Table 1 of this RALI as a secondary service.

- Exceptions under 2 and 3 will be considered by the ACMA on a case-by-case basis.

## The VHF High-band (148 to 174 MHz)

Spectrum allocation and channelling arrangements for services in the VHF High-Band are contained in Table 2 of Appendix A and illustrated in Figure 2 of the Appendix.  Fixed and mobile services are to be assigned in accordance with the arrangement contained in Table 2.

The following exceptions to the requirements of Table 2 of Appendix A apply:

- Existing services may continue to operate as authorised under the VHF High Band Frequency Band Plan (148 to 174 MHz) 1991.

- New services may operate under channelling arrangements not in accordance with Appendix A Table 2 of this RALI as a primary service provided that the service is considered to be spectrally efficient.  In determining the spectrum efficiency of a service, without limiting the range of matters which may be taken into account, the following matters may be considered:

- occupied bandwidth;

- adjacent channel performance;

- the distance from the transmitter that the channel may be used again without causing harmful interference; and

- the impact that introduction of the service will have upon existing services.

- New services assigned outside a High Spectrum Density (HSD) area specified in Appendix A Table 3 may operate under channelling arrangements not in accordance with Appendix A Table 2 of this RALI as a secondary service.

- Exceptions under 2 and 3 will be considered by the ACMA on a case-by-case basis.

# RALI Authorisation

Approved	18 August 2016

Mark Arkell

Manager
Spectrum Engineering and Space Section
Spectrum Infrastructure Branch
Australian Communications and Media Authority

# Appendix A: Allocation and Channelling Arrangements

The allocation and channelling arrangements for narrowband services are set out in the following tables:

Figure 1: Illustration of VHF Mid-Band spectrum arrangements

Table 1: Service allocations and channelling arrangements in the VHF Mid-Band

Figure 2: Illustration of VHF High-Band spectrum arrangements

Table 2: Service allocations and channelling arrangements in the VHF High-Band

Table 3: High Spectrum Demand and Rural Area contour centre coordinates and radii

Figure 3: VHF Mid-Band HSD area contour diagram

Figure 4: VHF High-Band HSD and Rural area contour diagram

Figure 1: Illustration of VHF Mid-Band spectrum arrangements

Table 1: Service allocations and channelling arrangements in the VHF Mid-Band

| Segment | Service allocation | Allowed Areas | Segment frequency limits (MHz) | Paired segment | Channel bandwidth (kHz) | Channel centre frequency formula | Range of integer values for variable ‘n’ | First / last channel centre frequency |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Miscellaneous service (see note 2) | Australia Wide | 70.00000 to 70.24375 | - | 12.5 (see note 2) | 70.0000 + n (0.0125) (see note 2) | 1 to 19 | 70.01250 MHz 70.2375 MHz |
| B | Land mobile service (two frequency, base transmit) | Australia Wide | 70.24375 to 72.29375 | D | 12.5 | 70.2375 + n (0.0125) | 1 to 164 | 70.25 MHz 72.2875 MHz |
| C | Land mobile service (single frequency) | Australia Wide | 72.29375 to 72.74375 |  | 12.5 | 72.2875 + n (0.0125) | 1 to 36 | 72.3 MHz 72.7375 MHz |
| D | Land mobile service (two frequency, base receive) | Australia Wide | 72.74375 to 74.80000 | B | 12.5 | 72.7375 + n (0.0125) | 1 to 164 | 72.75 MHz 74.7875 MHz |
| E | Aeronautical radionavigation service | Australia Wide | 74.80000 to 75.20000 |  | See note 3 | See note 3 | See note 3 | See note 3 |
| F | Land mobile service (single frequency) | Australia Wide | 75.20000 to 77.29375 |  | 12.5 | 75.2000 + n (0.0125) | 1 to 167 | 75.2125 MHz 77.2875 MHz |
| G | Miscellaneous service (see note 2) | Australia Wide | 77.29375 to 77.49375 |  | 12.5 (see note 2) | 77.2875 + n (0.0125) (see note 2) | 1 to 16 | 77.3 MHz 77.4875 MHz |
| H | Land mobile service (two frequency, base transmit) | Australia Wide | 77.49375 to 79.80625 | J | 12.5 | 77.4875 + n (0.0125) | 1 to 185 | 77.5 MHz 79.8 MHz |
| I | Land mobile service (single frequency) |  | 79.80625 to 79.99375 |  | 12.5 | 79.8000 + n (0.0125) | 1 to 15 | 79.8125 MHz 79.9875 MHz |
| J | Land mobile service (two frequency, base receive) | Australia Wide | 79.99375 to 82.30625 | H | 12.5 | 79.9875 + n (0.0125) | 1 to 185 | 80 MHz 82.3 MHz |
| K | Land mobile service (single frequency) | Australia Wide | 82.30625 to 82.49375 |  | 12.5 | 82.3000 + n (0.0125) | 1 to 15 | 82.3125 MHz 82.4875 MHz |
| L | Land mobile service (two frequency, base receive) | Australia Wide | 82.49375 to 83.74375 | O | 12.5 | 82.4875 + n (0.0125) | 1 to 100 | 82.5 MHz 83.7375 MHz |
| M | Land mobile service (two frequency, base receive) OR (single frequency) | Australia Wide | 83.74375 to 84.69375 | P | 12.5 | 83.7375 + n (0.0125) (see note 5) | 1 to 76 | 83.75 MHz 84.6875 MHz |
| N | Miscellaneous service (see note 2) | Australia Wide | 84.69375 to 84.99375 | Q | 12.5 (see note 2) | 84.6875 + n (0.0125) (see note 2) | 1 to 24 | 84.7 MHz 84.9875 MHz |
| O | Land mobile service (two frequency, base transmit) | Australia Wide | 84.99375 to 86.24375 | L | 12.5 (see note 4) | 84.9875 + n (0.0125) (see note 4) | 1 to 100 | 85 MHz 86.2375 MHz |
| P | Land mobile service (two frequency, base transmit) OR (single frequency) | Australia Wide | 86.24375 to 87.19375 | M | 12.5 (see note 4) | 86.2375 + n (0.0125) (see notes 4 and 5) | 1 to 76 | 86.25 MHz 87.1875 MHz |
| Q | Miscellaneous service (see notes 2 and 4) | Australia Wide | 87.19375 to 87.50000 | N | 12.5 | 87.1875 + n (0.0125) (see notes 2 and 4) | 1 to 24 | 87.2 MHz 87.4875 MHz |

Notes to Table 1:

1.	The Department of Defence and the Australian Defence Force are allowed secondary use of the 70 MHz to 87.5 MHz band for the purpose of land mobile communications and may operate with channelling arrangements other than those specified in Table 1.

2.	The channelling arrangements for miscellaneous services are nominal only.  A user of a miscellaneous service may be authorised to operate with a channel centre frequency and channel bandwidth other than that specified.  Segments A, G, N and Q are predominantly allocated to miscellaneous services however, they may also be used for narrowband area services.  The channelling arrangements for narrowband area services are not defined.

3.	The Spectrum Plan allocates the frequency range 74.7-75.2 MHz on a primary basis to the aeronautical radionavigation service.  No other services are to be assigned in this frequency range.  The operation of aeronautical marker beacons within the aeronautical radionavigation service is subject to the provisions of Annex 10 to the Convention on International Civil Aviation and the Standards and Recommended Practices of the International Civil Aviation Organisation.

4.	The Spectrum Plan allocates the frequency range 85 to 87.5 MHz on a primary basis to the broadcasting service.  Broadcasting services may operate with channelling arrangements other than those specified in Table 1.  The band may be used for other services, as indicated in Table 1, as secondary services.

5.	Land mobile segments referenced by this note may be used in a single frequency mode or a two frequency mode in non-HSD areas. In HSD areas, either single or two frequency modes may be used, but not both.  The mode of operation for these segments in HSD areas may be determined by the ACMA, based upon relative demand for these modes of operation in each State and Territory.  See Frequency assignment practice guideline No.3 “Sited single frequency operation in two frequency land mobile segments” available on the ACMA website.

Figure 2: Illustration of VHF High-Band spectrum arrangements

Table 2: Service allocations and channelling arrangements in the VHF High-Band

| Segment | Service allocation | Allowed Areas | Segment frequency limits (MHz) | Paired segment | Channel bandwidth (kHz) | Channel centre frequency formula | Range of integer values for variable ‘n’ | First / last channel centre frequency |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Land mobile service (paging) (see notes 1 and 8) | Australia Wide | 148.00000 to 149.25000 | - | 25 | 147.9875 + (n x (0.025)) | 1 to 50 | 148.0125 MHz 149.2375 MHz |
| B | Land mobile service (two frequency, base transmit) (see note 8) | Australia Wide | 149.25000 to 149.75625 | H | 6.25 | 149.2500 + (n x (0.00625)) | 1 to 80 | 149.25625 MHz 149.75 MHz |
| C | Land mobile service (single frequency) (see notes 2 and 8) | Australia Wide | 149.75625 to 149.90000 | - | 12.5 | 149.7500 + (n x (0.0125)) | 1 to 11 | 149.7625 MHz 149.8875 MHz |
| D | Radionavigation satellite service (see note 9) | Australia Wide | 149.90000 to 150.05000 | - | See note 9 | See note 9 | See note 9 | See note 9 |
| E | Land mobile service (two frequency, base transmit) and | Australia Wide | 150.05000 to 151.39375 | J | 12.5 | 150.0500 + (n x (0.0125)) | 1 to 107 | 150.0625 MHz 151.3875 MHz |
| E | and Fixed service (two frequency) | Rural Areas ( note 3) | 150.05000 to 151.39375 | J | 12.5 | 150.0500 + (n x (0.0125)) | 1 to 107 | 150.0625 MHz 151.3875 MHz |
| F | Miscellaneous service (see note 10) | Australia Wide | 151.39375 to 152.49375 | - | 12.5 (see note 10) | 151.3875 + (n x (0.0125)) (see note 10) | 1 to 88 | 151.4 MHz 152.4875 MHz |
| G | Land mobile service (single frequency) (see notes 4 and 12) | Australia Wide | 152.49375 to 153.85000 | - | 12.5 | 152.4875 + (n x (0.0125)) | 1 to 108 | 152.5 MHz 153.8375 MHz |
| H | Land mobile service (two frequency, base receive) | Australia Wide | 153.85000 to 154.35625 | B | 6.25 | 153.8500 + (n x (0.00625)) | 1 to 80 | 153.85625 MHz 154.35 MHz |
| I | Land mobile service (single frequency) | Australia Wide | 154.35625 to 154.65625 | - | 12.5 (see note 11) | 154.3500 + (n x (0.0125)) (See note 11) | 1 to 23 | 154.3625 MHz 154.6375 MHz |
| J | Land mobile service (two frequency, base receive) and | Australia Wide | 154.65625 to 156.00000 | E | 12.5 | 154.6500 + (n x (0.0125)) | 1 to 107 | 154.6625 MHz 155.9875 MHz |
| J | And Fixed service (two frequency) | Rural Areas ( note 3) | 154.65625 to 156.00000 | E | 12.5 | 154.6500 + (n x (0.0125)) | 1 to 107 | 154.6625 MHz 155.9875 MHz |
| K | Maritime mobile service (see note 5) | Australia Wide | 156.00000 to 157.45000 | N, P | See note 5 | See note 5 | See note 5 | See note 5 |
| L | Land mobile service (two frequency, base receive) OR (single frequency) (see note 6) | Australia Wide | 157.45000 to 158.29375 | Q | 12.5 | 157.4500 + (n x (0.0125)) | 1 to 67 | 157.4625 MHz 158.2875 MHz |
| M | Land mobile service (two frequency, base receive) | Australia Wide | 158.29375 to 160.60000 | R | 12.5 | 158.2875 + (n x (0.0125)) | 1 to 184 | 158.3 MHz 160.5875 MHz |
| N | Maritime mobile service (see note 5) | Australia Wide | 160.60000 to 160.97500 | K | See note 5 | See note 5 | See note 5 | See note 5 |
| O | Land mobile service (single frequency) | Australia Wide | 160.97500 to 161.47500 |  | 12.5 | 160.9750 + (n x (0.0125)) | 1 to 39 | 160.9875 MHz 161.4625 MHz |
| P | Maritime mobile service (see note 5) | Australia Wide | 161.47500 to 162.05000 | K | See note 5 | See note 5 | See note 5 | See note 5 |
| Q | Land mobile service (two frequency, base transmit) OR (single frequency) (see note 6) | Australia Wide | 162.05000 to 162.89375 | L | 12.5 | 162.0500 + (n x (0.0125)) | 1 to 67 | 162.0625 MHz 162.8875 MHz |
| R | Land mobile service (two frequency, base transmit) | Australia Wide | 162.89375 to 165.19375 | M | 12.5 | 162.8875 + (n x (0.0125)) | 1 to 184 | 162.9 MHz 165.1875 MHz |
| S | Land mobile service (trunked, base transmit) (see note 7) | Australia Wide | 165.19375 to 168.19375 | U | 12.5 | 165.1875 + (n x (0.0125)) | 1 to 240 | 165.2 MHz 168.1875 MHz |
| T | Land mobile service (single frequency) | Australia Wide | 168.19375 to 169.79375 |  | 12.5 | 168.1875 + (n x (0.0125)) | 1 to 128 | 168.2 MHz 169.7875 MHz |
| U | Land mobile service (trunked, base receive) (see note 7) | Australia Wide | 169.79375 to 172.79375 | S | 12.5 | 169.7875 + (n x (0.0125)) | 1 to 240 | 169.8 MHz 172.7875 MHz |
| V | Land mobile service (single frequency) (see note 12) | Australia Wide | 172.79375 to 173.29375 |  | 12.5 | 172.7875 + (n x (0.0125)) | 1 to 40 | 172.8 MHz 173.2875 MHz |
| W | Miscellaneous service (see note 10) | Australia Wide | 173.29375 to 174.00000 |  | 12.5 (see note 10) | 173.2875 + (n x (0.0125)) (see note 10) | 1 to 56 | 173.3 MHz 173.9875 MHz |

Notes to Table 2:

1.	The land mobile service (paging) in segment A is a primary service when used for exterior paging and is a secondary service when used for interior paging except in channels with the centre frequencies 148.1625 MHz, 148.3375 MHz, 149.1125 MHz or 149.1875 MHz where it is a primary service when used for interior paging and is a secondary service when used for exterior paging.

2.	In a channel with centre frequency 149.7875, 149.8375 or 149.8875 MHz, existing land mobile service (paging) may continue to operate  as a primary service when used for exterior paging or as a secondary service when used for interior paging.  No new paging  services are to be authorised.

3.	Fixed services (two frequency) may be operated in non-rural areas on a secondary basis.

4.	The frequency 153.8 MHz may be used as the centre frequency for an interior paging talkback channel with a bandwidth of 25 kHz.

5.	The operation of maritime mobile services is subject to the provisions of Appendix 18 of the International Telecommunication Union Radio Regulations.

6.	Land mobile segments referenced by this note may be used in a single frequency mode or a two frequency mode in non-HSD areas.  In HSD areas of each State or Territory either single or two frequency modes may be used, but not both.  The mode of operation for these segments in HSD areas may be determined by the ACMA, based upon relative demand for these modes of operation in each State and Territory.  See Frequency assignment practice guideline No.3 “Sited single frequency operation in two frequency land mobile segments” available on the ACMA website

7.	Segments referenced by this note may be authorised for use by users of:

(a) other land mobile services that make equivalent or more efficient use of the spectrum, as compared to the designated service allocation;

(b) land mobile services (two frequency), if such services are used in conjunction with a land mobile service (trunked); or

(c) land mobile services (two frequency) in low and remote areas the utilising technologies that are more spectrally efficient than analog modulation.

8.	Segments A, B and C may also be used for a mobile-satellite service.  The channelling arrangements for a mobile-satellite service are not defined.

9.	Segment D may also be used for a land mobile-satellite service.  The channelling arrangements for the radionavigation satellite service and land mobile-satellite service are not defined.

10.	Segments F and W may also be used for narrowband area services.  The channelling arrangements are for miscellaneous services are nominal only.  Assignments in the miscellaneous service may be authorised to operate with a channel centre frequency and channel bandwidth other than that specified.  The channelling arrangements for narrowband area services are not defined.

11.	Any user of a land mobile service (single frequency) may be authorised to operate their service with a channel bandwidth of 25 kHz and a channel centre frequency other than that specified in this segment, if their service is used in conjunction with a maritime mobile service.

12.	Outside of HSD areas segments G and V may also be used for low powered two frequency land mobile systems.

Table 3: High Spectrum Demand and Rural Area contour centre coordinates and radii

| Area | State | Town | Coordinates (Easting, Northing, Zone) | HSD Radius | Rural Radius (see note 2) |
| --- | --- | --- | --- | --- | --- |
| 1 | New South Wales | Newcastle | 384722, 6355447, 56 | 50 km | 100 km |
| 2 | New South Wales | Sydney | 335103, 6249367, 56 | 60 km | 100 km |
| 3 | New South Wales | Penrith | 279726, 6267448, 56 | 35 km | 100 km |
| 4 | New South Wales | Wollongong | 301116, 6177146, 56 | 35 km | 100 km |
| 5 | Queensland | Maroochydore | 509952, 7052315, 56 | 35 km | 100 km |
| 6 | Queensland | Brisbane | 501674, 6961834, 56 | 55 km | 100 km |
| 7 | Queensland | Surfers Paradise | 542065, 6902340, 56 | 35 km | 100 km |
| 8 | South Australia | Adelaide | 280600, 6132250, 54 | 40 km (see note 1) | 100 km |
| 9 | Victoria | Melbourne | 320605, 5812740, 55 | 50 km | 100 km |
| 10 | Victoria | Mornington | 328000, 5768000, 55 | 50 km | 100 km |
| 11 | Victoria | Geelong | 268300, 5774265, 55 | 20 km | 100 km |
| 12 | Western Australia | Perth | 391314, 6464517, 50 | 50 km | 100 km |

Notes to Table 3:

1.	The South Australia HSD area is only applicable to the VHF high-band.  There is no South Australia HSD area for the VHF mid-band.

2.	The Rural area is only applicable to the VHF high-band.  There are no Rural areas for the VHF mid-band.

Figure 3: VHF Mid-Band HSD contour diagram

Figure 4: VHF High-Band HSD contour diagram

See https://www.comlaw.gov.au/Series/F2005B01396

See https://www.legislation.gov.au/Series/F2005B01397.