ACMA LODGEMENT FACILITY

User Guide

MAY 2020

Canberra Red Building Benjamin Offices Chan Street Belconnen ACT PO Box 78 Belconnen ACT 2616 T +61 2 6219 5555 F +61 2 6219 5353; Melbourne Level 32 Melbourne Central Tower 360 Elizabeth Street Melbourne VIC PO Box 13112 Law Courts Melbourne VIC 8010 T +61 3 9963 6800 F +61 3 9963 6899; Sydney Level 5 The Bay Centre 65 Pirrama Road Pyrmont NSW PO Box Q500 Queen Victoria Building NSW 1230 T +61 2 9334 7700 1800 226 667 F +61 2 9334 7799.

Copyright notice http://creativecommons.org/licenses/by/3.0/au/ With the exception of coats of arms, logos, emblems, images, other third-party material or devices protected by a trademark, this content is licensed under the Creative Commons Australia Attribution 3.0 Licence. We request attribution as: © Commonwealth of Australia (Australian Communications and Media Authority) 2017. All other rights are reserved. The Australian Communications and Media Authority has undertaken reasonable enquiries to identify material owned by third parties and secure permission for its reproduction. Permission may need to be obtained from third parties to re-use their material. Written enquiries may be sent to: Manager, Editorial and Design PO Box 13112 Law Courts Melbourne VIC 8010 Tel: 03 9963 6968 Email: candinfo@acma.gov.au

ACMA Lodgement Facility	5

1.	Introduction	5

2.	Using ALF	6

2.1	Issue of One-time Token	6

2.2	General Conditions	6

3.	Machine credential	7

3.1	What is a machine credential?	7

3.2	How to install the machine credential	7

3.3	 Machine credential Security	7

4.	Authorisations for device registrations	7

5.	Lodgement of Submissions via ALF	7

5.1 Key Steps for submitting application via ALF	8

5.2 Notification of submission results	8

5.3	ALF Lodgement URL in Production	9

6.	ALF Testing Software	9

7.	Summary of Checks carried out on Submissions	9

8.	Submission Construction	10

9.	Legal Statements and Agreements	11

9.1	Legal Statement References	11

9.2	Legal statements for different submission types	11

9.3	Apparatus Licence with Frequency Assignment Certificate statement	12

9.4	Compliance with the S145(4) determination statement for Device Registrations	12

9.5	Guard Space Statement for Device Registrations	12

9.6	Agreement Statement for Device Registrations	12

9.7	Core Condition Agreement Statement for Device Registrations	12

9.8 	Deregistration of Devices for Spectrum Licences	13

9.9	Agreements for Device Registrations	13

10	Apparatus Licensing – submission elements	14

10.1	ApplicationInfoTypes.xsd	14

10.2	CommonTypes.xsd	15

10.3	LicenceDetailsTypes.xsd	15

10.4	StationTypes.xsd	16

10.5	EquipmentTypes.xsd	17

10.6	AntennaDetailsTypes.xsd	18

10.7	FrequencyTypes.xsd	19

10.8	PowerTypes.xsd	20

11	Site Registrations – submission elements	21

11.1	SitesRegistration_V12.xsd	21

12	Antenna Registrations – submission elements	23

12.1	AntennaRegistration_V7.xsd	23

13	Station Registrations for PTS 900 MHz licences – submission elements	24

13.1	ApplicationInfoTypes.xsd	24

13.2	Common Types	24

13.3	LicenceDetailsTypes	24

13.4	StationTypes	24

13.5	Equipment Types	24

13.6	AntennaDetailsTypes.xsd	24

13.7	Frequency Types	25

13.8	PowerTypes	25

13.9	PTSSubmission	25

13.10	ReceiverTypes	25

13.11	TechnicalDetailsTypes	25

13.12	TransmitterTypes	25

14	Device Registrations – submission elements	26

14.1	CommonRadioCommunicationsComplexTypes.xsd	26

14.2	spectrumDeviceRegistration_V201404.xsd	29

# ACMA Lodgement Facility

## Introduction

The ACMA Lodgement Facility (ALF) system provides Accredited Persons (AP’s) with the facility to submit licence application and technical data via XML for the following:

- Apparatus licensing submissions;

- Site registrations;

- Antenna registrations;

- Station registrations for PMTS 900 MHz apparatus licences; and

- Device registrations for spectrum licences.

AP’s can submit compliant data files automatically via a “Web URL” by writing some custom ALF client software. The key steps for creating the custom software are detailed under section 5 - Lodgement of Submissions via ALF.  A developer’s guide is also available on the ACMA’s website to assist AP’s with developing software for the automated process. Alternatively, AP’s may wish to use the ACMA’s ALF Testing Software and ALF PROD Clients for test and production environments, respectively.

It is important to note that the ACMA does not supply software to create XML files for submissions via ALF.

Further information about the ACMA’s online services for accredited persons is available on the ACMA website.

## Using ALF

In order to use the ALF system, the following pre-requisites must be met:

- be accredited by the ACMA under Section 263 of the Radiocommunications Act 1992 ;

- have a current Australian Business Number (ABN);

- hold a valid machine credential and related software (see section 3);

- have a one-time token issued by the ACMA (see section 2.1)

- where applicable, complete authorisation process for device registrations with the ACMA (see section 4);

- have the means to construct a submission file that complies with the format and construction rules contained in this guide.

### 2.1	Issue of One-time Token

Before being able to use the ALF system, AP’s will need to be provided with an initial one-time token from the ACMA and complete the Register One Time Token form. AP’s can request the allocation of a one-time token through the ACMA’s Customer Service Centre via email at info@acma.gov.au. The ACMA will then email the one-time token to the nominated user(s).

### 2.2	General Conditions

Information lodged electronically to the ACMA using the ALF system must:

- comply with the requirements set out in this guide; and.

- meet the same regulatory and administrative requirements as information sent to the ACMA in written form (refer below for submission of non-compliant information).

As part of the lodgement process, the file is electronically signed by using a digital certificate (machine credential – refer to Section 3). This enables the ACMA to authenticate the submission and confirm that the data has not been compromised during transmission. Submissions lodged electronically have the same legal standing as if sent to the ACMA in paper form.

All submissions lodged are subject to a variety of formatting, data integrity and core business rules checks. Feedback on the processing of submissions will be emailed to the submitter.

Whilst the ALF system performs some basic validation of the data submitted, the responsibility continues with AP’s to ensure all requirements are met.

## Machine credential

### 3.1	What is a machine credential?

A machine credential is a digital certificate used to interact with government online services through business software. This is setup and installed using Relationship Authorisations Manager (RAM).

### 3.2	How to install the machine credential

https://info.authorisationmanager.gov.au/guide-how-to-install-a-machine-credential

ALF requires that the Credential name be the same identifier as your MyGovID identifier. The ACMA will be able to provide this identifier after the Register One Time Token form has been submitted (refer to section 2.1)

### 3.3	 Machine credential Security

ALF users are responsible for the security of the machine credential used to sign submissions. Should the confidentiality of the machine credential or any other user details become comprised, the ACMA’s Customer Service Centre should be contacted via e-mail to info@acma.gov.au or by telephone on 1300 850 115.

## Authorisations for device registrations

For spectrum licence device registrations, an additional stakeholder authorisation process is required prior to using ALF. The authorisation process identifies which spectrum licences an AP has been given authorisation, by the licensee, to register or de-register device registrations on their behalf.

APs will need to inform the ACMA if they wish to update their existing authorisations or create new authorisations for device registrations. AP’s must also provide evidence that a licensee has authorised the AP to act on their behalf. All relevant information should be sent via email to the ACMA Customer Service Centre at info@acma.gov.au.

Note: The authorisation process is not required for apparatus licensing, site, antenna or PTS 900 MHz station registration submissions.

## Lodgement of Submissions via ALF

This section outlines the major steps for submitting applications through the ALF portal. To assist AP’s with this process, a .NET development guide is also available on the ACMA website.

In order to automate the ALF process, the following pre-requisites are required:

- a Microsoft .NET development environment (recommended);

- hold a valid machine credential (see section 3);

- Web Services Description Language (WSDL) for the ACMA’s ALF web service; and

- The schemas for the device registrations, apparatus licences, sites and antennas.

The WSDL’s and ALF Schema’s are also available on the ACMA’s website.

### 5.1 Key Steps for submitting application via ALF

The key steps to making submissions via ALF are as follows:

1. Extract data from AP’s database and compose the required XML’s in accordance with the relevant XSD schemas and validate the XML files. There is a range of XML validation software available for this purpose. Due to the 5 MB file size limit, the XML file should be limited to a fixed number of applications to fit within the size limitation. If there are more applications to be submitted, multiple payloads will need to be created and the ALF web service invoked multiple times.

2. Sign the XML payload using a valid AUSkey. AP’s can also use the signing function provided by the CertificateSigner.cs (this file is also available on the ACMA website).

3. Based on the WSDL for the ALF service, create an instance of the ALF web service request. As well as the signed XML payload, a valid email address is required.

4. Invoke the ALF web service and process the response. The response will contain a GUID and a Response Code. A positive response merely indicates that the submission has reached the ACMA successfully. As such, AP’s may need to keep the GUID to correlate with the email notification that will be received following the successful processing of the submission. The email notification will reference the same GUID.

5. AP’s to update own databases based on responses received from the ACMA. This will ensure that the status of applications are up to date and avoid the duplication of submissions after an application has been submitted.

### 5.2 Notification of submission results for Device Registrations, Site/Antenna Registrations and PTS Station Registrations

Submissions for these requests  are processed automatically. For these requests, AP’s will receive three email messages from the ACMA:

- First email will be from the ACMA’s Customer Service Centre notifying the AP that the application has been received by the ACMA and will be processed shortly.

- Second email notifies the AP that the submission is currently being processed. The submitted payload (XML file) will be attached to this email.

- Third and final email notifies the AP that the submission has been successful and includes the new reference number for the site, antenna or station registration(s).

If the submission fails, an email will be sent notifying the AP that the application was unsuccessful, including the reason for the failure.

### 5.3 Notification of submission results for Apparatus Licence applications

For submissions that do NOT require manual assessment and comply with all validation checks are processed automatically, AP’s will receive three email messages from the ACMA:

- First email will be from the ACMA’s Customer Service Centre notifying the AP that the application has been received by the ACMA and will be processed shortly.

- Second email notifies the AP that the submission is currently being processed. The submitted payload (XML file) will be attached to this email

- Third and final email notifies the AP that the submission has been successful. The successful email contains details of the new licence number, service/subservice of the request and the submission ID.

For submissions that fail to comply with the validation checks, the AP will receive a not successful email instead of the third successful email mentioned above. The email will also include an attachment which outlines the reason for the failure (e.g. Invalid Emission designator).

For submissions that require manual assessment or fail to meet a system embargo or check against Section 152 applications, the AP will only receive the first two emails.

In these case, the ACMA will manually process the application and contact the AP if required.

In cases, where submissions include applications that pass and fail the validation checks, the AP will receive a not successful email. The email will provide details of which applications were successful or unsuccessful.

Functionality is available for AP’s to receive email notification for apparatus licence submissions once an invoice has been issued and when the apparatus licence has been issued to the customer. To use this functionality, AP’s should contact the ACMA’s Customer Service Centre on 1300 850 115 or info@acma.gov.au.

### 5.4 ALF Lodgement URL in Production

The URL used to lodge submissions to the ACMA’s ALF gateway for device registrations is as follows:

https://eport.acma.gov.au/DataService/SpectraDeviceRegistration/v1.1/Service.svc

The URL used to lodge submissions to the ACMA’s ALF gateway for apparatus license submissions is as follows:

https://eport.acma.gov.au/DataService/ApparatusLicenseSubmission/v1.0/Service.svc

## ALF Testing Software

The ACMA has also developed testing software to allow AP’s to test the submission of submissions via ALF to the ACMA. This software can also be configured for use in a production environment (if required).

The URL to be used with the ALF testing software for device registrations is as follows:

https://uat-eportal.acma.gov.au/ALFOuter/DataService/SpectraDeviceRegistration/v1.1/Service.svc

The URL to be used with the ALF testing software for apparatus license submissions is as follows:

https://uat-eportal.acma.gov.au/ALFOuter/DataService/ApparatusLicenseSubmission/v1.0/Service.svc

Further information about the testing software can be found in the ALF Testing Software Guide. This guide is also available on the ACMA website.

## Summary of Checks carried out on Submissions

The ALF system is based on the premise that accredited persons have responsibility for ensuring the accuracy of submitted data (consistent with the requirements of the accreditation scheme). Consequently, only minimal data integrity and key frequency coordination checks are performed. These include ensuring:

- AP is authorised to submit or delete device registrations for a specific spectrum licence.

- The device is inside the frequency and area bounds of a spectrum licence.

- Data file format conforms to ‘XSD’ definition and data is in a valid format (e.g. valid units, emission designator correctly constructed, and emission bandwidth consistent channel width).

- Data is consistent (e.g. a site should not be specified for an area wide service, a transmitter should not be included in the data for a receive only licence type).

- Conformance with ACMA embargo requirements.

- Conformance with the channelling formulas specified in band and channel plans.

- The Site/Area is valid;

- The Antenna is valid

- Conformance with maximum transmitter power and radiated power requirements specified in ACMA planning models

- Conformance with minimum path length requirements for point-to-point services

- For variations to apparatus licences, the licence is issued and current.

- The licensee is current.

- The correct service/subservice for the licence has been specified.

After a submission is digitally signed, the ALF gateway will conduct some basic file format checks, including verification of the user who signed the file. If there are errors with the verification of the user or with the XSD, an error message will be immediately displayed to the user.

## Submission Construction

The ALF system utilises industry standard ‘XML’ language for data exchange. The basic elements that form the five submission types are provided in this document:

- Apparatus licensing submissions (refer section 9);

- Site registrations (refer section 10);

- Antenna registrations (refer section 11);

- Station registrations for PMTS 900 MHz apparatus licences (refer section 12); and

- Device registrations for spectrum licences (refer section 13).

Example XML’s for the five submission types listed above can be found on the ACMA website in the relevant schema folder.

## Legal Statements and Agreements

All submissions must refer to the legal statement relevant to the submission type. Submissions must contain only applications/devices certified using the same certification type/legal statement.

In constructing an XML file, AP’s only need to provide the appropriate Legal Statement Reference shown in 9.1 below, instead of the complete legal statement(s) as detailed in 9.3 –9.8.

### 9.1	Legal Statement References

| Element Name | Submission Type | Legal Statement Reference |
| --- | --- | --- |
| Legal Statement | Apparatus Licence with Frequency Assignment Certificate | Apparatus Licensing statement applicable |
| Legal Statement 1 | Register Device: Compliance with S145(4) Determination | Register Device: Compliance with S145(4) Determination Statement applicable |
| Legal Statement 1 | Register Device: Guard Space | Register Device: Guard Space Statement applicable |
| Legal Statement 1 | Register Device: Agreement | Register Device: Agreement Statement applicable |
| Legal Statement 1 | De-register Device | Deregistration of Devices Statement applicable |
| Legal Statement 2 | All Device Registrations, except De-register Device | Register Device: Core Condition Agreement Statement applicable |

### 9.2	Legal statements for different submission types

| Submission Type | Legal Statement | Legal Statement 1 | Legal Statement 2* |
| --- | --- | --- | --- |
| Apparatus Licence with Frequency Assignment Certificate | Refer to section 8.1.3 | N/A | N/A |
| Register Device: Compliance with S145(4) Determination | N/A | Refer to section 8.1.4 | Refer to section 8.1.7* |
| Register Device: Guard Space | N/A | Refer to section 8.1.5 | Refer to section 8.1.7* |
| Register Device: Agreement | N/A | Refer to section 8.1.6 | Refer to section 8.1.7* |
| De-register Device | N/A | Refer to section 8.1.8 | N/A |

* Legal Statement 2 is an optional field and is only used when a core condition agreement exists for the purposes of device registrations for spectrum licences.

### 9.3	Apparatus Licence with Frequency Assignment Certificate statement

As a person accredited under subsection 263(1) of the Radiocommunications Act 1992, I certify that a) the proposed operation of the described device(s) have been assessed in accordance with the conditions that are required to be satisfied under section 5 of the Radiocommunications (Frequency Assignment Certificates) Determination; and b) that I believe, on reasonable grounds, that the described device(s) are, at the time this certificate is issued, device(s) for the operation of which the ACMA would issue an apparatus licence; and c) that the information detailed in this certificate is not false or misleading; and d) that the assessment of the described device(s) is in accordance with ACMA procedural documents relevant at the time this certificate is issued.

### 9.4	Compliance with the S145(4) determination statement for Device Registrations

As a person authorised by the licensee of the described spectrum licence(s), I apply to the Australian Communications and Media Authority to have included in the Register of Radiocommunications Licences the details of the radiocommunications device(s) described in this application that is proposed to be operated under the licence(s).

As a person accredited under subsection 263(1) of the Radiocommunications Act 1992, I am satisfied that, where the device(s) are operated in accordance with the information set out in the application, the operation of these device(s) meets the condition contained in subclause 6(a) of the Radiocommunications (Subsection 145(3) Certificates) Determination.

### 9.5	Guard Space Statement for Device Registrations

As a person authorised by the licensee of the described spectrum licence(s), I apply to the Australian Communications and Media Authority to have included in the Register of Radiocommunications Licences the details of the radiocommunications device(s) described in this application that is proposed to be operated under the licence(s).

As a person accredited under subsection 263(1) of the Radiocommunications Act 1992, I am satisfied that, where the device(s) are operated in accordance with the information set out in the application, the operation of these device(s) meets the condition contained in subclause 6(b) of the Radiocommunications (Subsection 145(3) Certificates) Determination.

### 9.6	Agreement Statement for Device Registrations

As a person authorised by the licensee of the described spectrum licence(s), I apply to the Australian Communications and Media Authority to have included in the Register of Radiocommunications Licences the details of the radiocommunications device(s) described in this application that is proposed to be operated under the licence(s).

As a person accredited under subsection 263(1) of the Radiocommunications Act 1992, I am satisfied that, where the device(s) are operated in accordance with the information set out in the application, the operation of these device(s) meets the condition contained in subclause 6(c) of the Radiocommunications (Subsection 145(3) Certificates) Determination.

### 9.7	Core Condition Agreement Statement for Device Registrations

As a person authorised by the licensee of the described spectrum licence(s), I apply to the Australian Communications and Media Authority to have included in the Register of Radiocommunications Licences the details of the radiocommunications device(s) described in this application that is proposed to be operated under the licence(s).

As a person accredited under subsection 263(1) of the Radiocommunications Act 1992, I am satisfied that, for the purpose of the core conditions in the licence(s), all spectrum licensees that may be affected by the operation of the device(s) have entered into a written agreement in the form set out in the licence(s) that the device(s) described may be operated in accordance with the information set out in the application.

### 9.8 	Deregistration of Devices for Spectrum Licences

As a person authorised by the licensee of the described spectrum licence(s), I apply to the Australian Communications and Media Authority to have removed from the Register of Radiocommunications Licences the radiocommunications device(s) described in this application and attached under the licence(s).

### 9.9	Agreements for Device Registrations

For device registrations, the following types of agreements are available:

- Core condition agreements

- IIC Agreements

A core condition agreement relates to an agreement between affected licensees to allow a device to be registered that would exceed either the Out-of-area or Out-of-band core conditions stated on the licence. This is not an agreement for the purposes of an IIC. A transmitter and/or receiver subject to a core condition agreement still needs to have an IIC submitted stating it either adheres to the section 145(4) determination, there is sufficient guard space or there is agreement with all affected licensees to exceed the criteria of the section 145(4) determination.

An IIC agreement relates to the registration of a transmitter under a spectrum licence via agreement with all affected parties. This path is usually followed when a device does not meet the section 145(4) determination of unacceptable levels of interference – meaning the device fails either the device boundary criteria or one more of the deployment constraints.

The ACMA has created predefined agreement IDs for the various core condition and IIC agreement scenarios. If the scenario required is not specified in the table below, a new agreement ID can be requested by contacting the ACMA’s Customer Service Centre via email to info@acma.gov.au.

Summary of agreement types used for different submissions

| Agreement ID | Agreement Type |
| --- | --- |
| 20131 | Out-of-area core condition agreement in force |
| 20132 | Out-of-band core condition agreement in force |
| 20133 | Agreement to exceed the device boundary criterion in force |
| 20134 | Agreement to exceed the deployment constraints in force |

## 10	Apparatus Licensing – submission elements

### 10.1	ApplicationInfoTypes.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| SubmissionID | YES | NMTOKEN |  |  | A field containing the submitter's identification for the job application. Each job requires a unique value. |
| SubmissionDate | YES | DateTime |  |  | Date of submission of the request. |
| CustomerID | YES | Integer |  |  | Unique Identifier of the customer. |
| ServiceID | YES | Token |  |  | SPECTRAplus Service Name (ACMA Licence Type). |
| SubServiceID | YES | Token |  |  | SPECTRAplus Sub-Service Name (ACMA Licence Sub-Type). |
| AccredPersonClientNumber | YES | Integer |  |  | Client number of the accredited person. |
| AccredPersonFirstName | YES | String |  |  | First name of the accredited person |
| AccredPersonLastName | YES | String |  |  | Last name of the accredited person |
| PolicyProvider | YES | Token |  |  | The name of the insurance policy provider |
| PolicyNumber | YES | NMTOKEN |  |  | The number of the insurance policy. |
| PolicyCeaseDate | YES | Date |  |  | The cessation date of the insurance policy. |
| GovernmentEmployee | YES | Boolean |  |  | A boolean field indicating whether the accredited person is from a government agency and therefore covered by the relevant government insurance. |
| LegalStatement | YES | Token |  |  | Refer to section 8 for required value |
| Comment | NO | String |  |  | A field for AP’s to enter comments about the application |
| PaymentOffice | NO | Integer |  |  | Unique identifier of the payment office |
| PayOfficeContactFirstName | NO | String |  |  | First name of the contact for the payment office |
| PayOfficeContactLastName | NO | String |  |  | Last name of the contact for the payment office |
| APContact | NO | Integer |  |  | Unique identifier for the AP Contact |
| APContactFirstName | NO | String |  |  | First name of the contact for the AP Contact |
| APContactLastName | NO | String |  |  | Last name of the contact for the AP Contact |

### 10.2	CommonTypes.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| APStationRef | YES | Token |  |  | Unique Identifier of the Station generated by the Licensing subsystem. |
| FrequencyAssignmentID | NO | Token |  |  | Unique Identifier of the Frequency generated by the Licensing subsystem. |
| AttachmentDescription | NO | Token |  |  | A field for general comments or description of the attached files. |
| FileName | NO | NMTOKEN |  |  | Name of the attachment file. |
| FileBlob | NO | Base64Binary |  |  | The file in base64Binary format. |

### 10.3	LicenceDetailsTypes.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| LicenceTermCategory | YES | NMTOKEN |  |  | Indicates whether or not the licence is renewable or non-renewable |
| LicenceTerm | YES | Token |  |  | Indicates the term of the licence. |
| LicenceTermUnit | YES | NMTOKEN |  |  | Indicates the term of the licence - Unit. |
| LicenceStartDate | NO | Date |  |  | Indicates the start date of the licence. |
| LicenceExpiryDate | NO | Date |  |  | Indicates the expiry date of the licence |
| CallSign | NO | Token |  |  | Callsign to be manually entered or generated from callsign list. |
| ShipName | NO | Token |  |  | Free text field for Ship Name to be entered onto licence. |
| TypeOfPayment | YES | NMTOKEN |  |  | Indicates if application is to be paid for in advance (up front) or to be an instalment licence. |

### 10.4	StationTypes.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| APStationRef | NO | Token |  |  | Unique Identifier (5 digit if provided by AP) of the Station generated by the Licensing subsystem. |
| FrequencyAssignmentID | NO | NMTOKEN |  |  | Unique Identifier of the Frequency generated by the Licensing subsystem. |
| TypeOfStation | NO | NMTOKEN |  |  | Indicates the type of Station e.g. Base or Supplementary or Satellite etc. |
| ServicePurpose | NO | NMTOKEN |  |  | Select from List: 1. Communicaitons (Internet, VOIP etc.) 2. Broadcasting (TV etc) 3. Launch/TOSS (Transfer Orbit Support Services) 4. TTC (Tracking, Telemetry, Commonand & Monitoring) 5. Testing 6. Other Where other is selected, user can manually enter details (--> Combobox required) |
| Operation | NO | NMTOKEN |  |  | Indicates whether the station is simplex or duplex. |
| ManualAssessment | YES | Boolean |  |  | Manual assessment if required. Boolean value. |
| StationAssociationID | NO | NMTOKEN |  |  | Unique identifier (5 digit) indicating the linkage between two stations under the same technical details. |
| PreDefinedSpecialConditions | NO | TOKEN |  |  | 1.Pre-defined special conditions to be added to licence; 2. APs to provide the code from the RRL. |
| PreDefinedAdvisoryNotes | NO | TOKEN |  |  | 1. Pre-defined advisory notes to be added to licence. 2. APs to provide the code from the RRL. |
| SiteID | YES | Integer |  |  | The ACMA assigned site identification number which the antenna is located. |
| AreaID | YES | Integer |  |  | The ACMA assigned area identification number which the antenna is located. |
| SatelliteID | NO | Integer |  |  | Unique identifier of the satellite |

### 10.5	EquipmentTypes.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| ServiceNature | NO | Token |  |  | As per ITU |
| ServiceRadius | NO | Double |  |  | Applicable to only couple of subservices |
| ITUEmissionDesignator | YES | Token |  |  | A string field containing the emission designator of the transmission as defined by Appendix 1 of the ITU-R Radio Regulations |
| ModeHFPropogation | NO | NMTOKEN |  |  | Skywave, Groundwave or NVIS |
| EqualORLessTWOkms | YES | Boolean |  |  | Communication Distance in km. Either less than or greater than 2 km. |
| OperationStartHrs | NO | Time |  |  | Start time of the operation |
| OperationEndHrs | NO | Time |  |  | End time of the operation |

### 10.6	AntennaDetailsTypes.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| AntennaID | YES | Token |  |  | The unique identification number assigned to an antenna by the ACMA. An antenna may be referenced by this number. |
| Multimode | NO | Boolean |  |  | To indicate whether an antenna is mounted multiple times. To be used by SPECTRAemc. Default to No. Applicable to all assigned Subservices |
| Azimuth | NO | Double |  |  | The azimuthal angle for the orientation of the antenna relative to true north. Units in degrees. Default value of 0 if not known. |
| Minimum | NO | Double |  |  | Relative to true north,the start azimuthal angle of the inteval over which the radiated power applies. Units of degrees. |
| Maximum | NO | Double |  |  | Relative to true north, the end azimuthal angle of the inteval over which the above radiated power applies.Units in degrees. |
| Elevation | NO | Double |  |  | The tilt (elevation) for the orientation of the antenna. Units in degrees; positive value for tilt above the horizontal plane, negative value for tilt below the horizontal plane. |
| Minimum | NO | Double |  |  | Minimum Range of the Antenna Elevation. |
| Maximum | NO | Double |  |  | Maximum Range of the Antenna Elevation. |
| AntennaHeight | YES | Double |  |  | Height of the antenna above ground level (AGL) (metres). |
| Polarisation | YES | Token |  |  | A field with enumerated type values for polarisation of the antenna. Values include: Linear Vertical, Linear Horizontal, RH Circular, LH Circular, RH Slant, LH Slant, Slant, Linear, Mixed, Other. |
| PolarisationAngle | NO | Integer |  |  | A field to indicate the anle of polarisation. |
| FixedLosses | NO | Token |  |  | Attenuation in the signal path due to feeder and branching power losses. Units of dB. Default value of 0 if not known. |

### 10.7	FrequencyTypes.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| CarrierFrequency | NO | Double |  |  | Carrier frequency of the emitted channel. |
| CarrierFrequencyUnit | NO | NMTOKEN |  |  | Units for these frequency parameters; values of Hz, kHz, MHz, GHz |
| OperatingFrequency | YES | Double |  |  | Centre frequency of the emitted channel. |
| OperatingFrequencyUnit | YES | NMTOKEN |  |  | Units for these frequency parameters; values of Hz, kHz, MHz, GHz |
| ChannnelBandwidth | YES | Double |  |  | A decimal field containing the effective occupied bandwidth of the signal. As defined by the ITU-R Radio Regulations |
| ChannelBandwidthUnit | YES | NMTOKEN |  |  | Units for these frequency parameters; values of Hz, kHz, MHz, GHz |
| CarrierFrequency | NO | Double |  |  | Carrier frequency of the emitted channel. |
| CarrierFrequencyUnit | NO | NMTOKEN |  |  | Units for these frequency parameters; values of Hz, kHz, MHz, GHz |
| OperatingFrequency | YES | Double |  |  | Centre frequency of the emitted channel. |
| OperatingFrequencyUnit | YES | NMTOKEN |  |  | Units for these frequency parameters; values of Hz, kHz, MHz, GHz |
| ChannnelBandwidth | YES | Double |  |  | A decimal field containing the effective occupied bandwidth of the signal. As defined by the ITU-R Radio Regulations |
| ChannelBandwidthUnit | YES | NMTOKEN |  |  | Units for these frequency parameters; values of Hz, kHz, MHz, GHz |

### 10.8	PowerTypes.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| OutputPower | YES | Double |  |  | Transmitter output power (at the connector of the device). Specified in Watts per effective occupied bandwidth. |
| OutputPowerUnit | YES | NMTOKEN |  |  | Units for transmitter output power. Watts |
| RadiatedPower | YES | Double |  |  | The maximum transmitted radiated power (at the output of the antenna connected to the device). Specified in Watts per effective occupied bandwidth. |
| RadiatedPowerUnit | YES | NMTOKEN |  |  | Units for radiated transmitter power, Watts. |

## 11	Site Registrations – submission elements

### 11.1	SitesRegistration_V12.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| SubmissionID | YES | String |  |  | A field containing the submitter's identification for the job application. Each job requires a unique value. |
| AccreditPersonID | YES | String |  |  | Client number of the accredited person. |
| SiteName | YES | String |  |  | Name of site |
| Street | YES | String |  |  | Name of street |
| Locality | YES | String |  |  | Site town/suburb |
| State | YES | NMTOKEN |  | VIC ACT NSW QLD TAS SA WA NT EXT |  |
| Latitude | YES | Double |  |  | Latitude coordinates of site |
| Longitude | YES | Double |  |  | Longitude coordinates of site |
| CoordinatePrecisionInMeters | YES | String |  | 10 100 Unknown |  |
| SiteCategory | YES | NMTOKEN |  | TowerCentre BroadcastingPlanning RooftopCentre NominalPoint RooftopNominal |  |

## 12	Antenna Registrations – submission elements

### 12.1	AntennaRegistration_V7.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| SubmissionID | YES | String |  |  | A field containing the submitter's identification for the job application. Each job requires a unique value. |
| AccreditPersonID | YES | String |  |  | Client number of the accredited person. |
| AntennaModel | YES | Token |  |  |  |
| AntennaType | YES | Token |  |  |  |
| Manufacturer | YES | String |  |  |  |
| AntennaGain | YES | Double |  |  |  |
| HorizontalBeamWidth | YES | Double |  |  |  |
| FrontToBack | YES | Double |  |  |  |
| DirectionalAntenna | YES | Boolean |  |  |  |

## 13	Station Registrations for PTS 900 MHz licences – submission elements

### 13.1	ApplicationInfoTypes.xsd

Refer to ApplicationInfoTypes.xsd under Apparatus Licensing – submission elements.

### 13.2	Common Types

Refer to CommonTypes.xsd under Apparatus Licensing– submission elements.

### 13.3	LicenceDetailsTypes

Refer to CommonTypes.xsd under Apparatus Licensing – submission elements.

### 13.4	StationTypes

Refer to StationTypes.xsd under Apparatus Licensing – submission elements.

### 13.5	Equipment Types

Refer to EquipmentTypes.xsd under Apparatus Licensing – submission elements.

### 13.6	AntennaDetailsTypes.xsd

Refer to AntennaDetailsTypes.xsd under Apparatus Licensing – submission elements.

### 13.7	Frequency Types

Refer to FrequencyTypes.xsd under Apparatus Licensing – submission elements.

### 13.8	PowerTypes

Refer to PowerTypes.xsd under Apparatus Licensing – submission elements.

### 13.9	PTSSubmission

Refer to relevant schemas under Apparatus Licensing.

### 13.10	ReceiverTypes

Refer to ReceiverTypes.xsd under Apparatus Licensing.

### 13.11	TechnicalDetailsTypes

Refer to TechnicalDetailsTypes.xsd under Apparatus Licensing.

### 13.12	TransmitterTypes

Refer to TransmitterTypes.xsd under Apparatus Licensing.

## 14	Device Registrations – submission elements

### 14.1	CommonRadioCommunicationsComplexTypes.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| LegalStatement1 | YES | String | - | Refer to section 8 | Reference to legal statement used by accredited persons. The choice of legal statement depends on the certification method used for devices submitted or whether devices are to be deregistered. |
| LegalStatement2 | NO | String | - | Refer to section 8 | Reference to legal statement used by accredited persons when a core condition agreement is in effect for the purposes of device registration. |
| AttachmentDescription | NO | String | - | - | A field for general comments or description of attached files |
| FileName | NO | NMTOKEN |  |  | Name of the attachment file. |
| FileBlob | NO | Base64Binary |  |  | The file in base64Binary format. |
| AccredPersonName | YES | String | - | - | Name of the accredited person. |
| AccredPersonClientNumber | YES | Integer | - | - | ACMA identification number for the accredited person. |
| PolicyProvider | YES | String | - | - | Insurance policy provider name or code. |
| PolicyNumber | YES | String | - | - | Insurance policy number. |
| CeaseDate | YES | Date | - | - | Date of cessation of the insurance policy. |
| GovernmentEmployee | NO | Boolean | - | True, False | Indication of whether the AP is a government employee. |
| AntennaID | YES | Integer | - | - | The ACMA unique identification number assigned to an antenna by the ACMA. An antenna may be referenced by this number. |
| NonDirectionalAntenna | YES | String | - | ND | A field that specifies that the antenna is non-directional. Also used for group registrations to indicate directionality is either random or effectively providing 360 degree coverage. |
| AntennaAzimuth | YES for Directional Antennas | Decimal | Degrees | 0-360 | The azimuthal angle (in the horizontal plane) of the antenna relative to true north. |
| AntennaTilt | YES for Directional Antennas | Decimal | Degrees | -90 to +90 | The tilt or elevation angle (in the vertical plane) of the antenna. Positive angle for tilt above the horizontal plane, negative angle for tilt below the horizontal plane. |
| AntennaHeight | YES | Decimal | Metres | - | Height of the antenna above ground level (AGL) (metres). |
| Polarisation | YES | String | - | Linear Vertical, Linear Horizontal, RH Circular, LH Circular, RH Slant, LH Slant, Slant, Mixed, Other. | Polarisation setting of the antenna. |
| Feeder Loss | YES | Decimal | dB | - | Attenuation in the signal path between Antenna and Transmitter/Receiver due to feeder and branching power losses. |
| LevelOfProtection | NO | Decimal | dBm | - | The power level in the receiver that represents the maximum level of interference, or the protection attributed to the receiver device. |
| LevelOfProtectionUnit | NO | String | - | dBm | Units for level of protection. |
| ReferenceBandwidth | YES where LOP stated | Integer | Hz, kHz, MHz, GHz. | - | Reference bandwidth for level of protection. |
| ReferenceBandwidthUnit | YES where LOP stated | String | - | Hz, kHz, MHz, GHz. | Units for the Reference bandwidth |
| OutputPower | YES | Decimal | W (watts) per effective occupied bandwidth. | - | Transmitter output power (at the output connector of the device). |
| OutputPowerUnit | YES | String | - | W (watts) | Units for transmitter output power. |
| PowerIndicator | YES | String | - | pX, pY, pZ | Power indicator for specified power as defined by the ITU-R Radio Regulations. |
| RadiatedPower | YES | Decimal | W (watts) per effective occupied bandwidth. | - | The maximum transmitted radiated power (at the output of the antenna connected to the device). |
| RadiatedPowerUnit | YES | String | - | W (watts) | Units for radiated transmitter power. |
| PowerIndicator | YES | String | - | pX, pY, pZ | Power indicator for specified power as defined by the ITU-R Radio Regulations. |
| CentreFrequency | YES | Decimal | Hz |  | Centre frequency of the emitted signal. |
| CentreFrequencyUnit | YES | String |  | Hz | Units for the centre frequency. |
| EffectiveOccupiedBandwidth | YES | Decimal | Hz |  | A decimal field containing the effective occupied bandwidth of the signal as defined in the ITU-R Radio Regulations. |
| EffectiveOccupiedBandwidthUnit | YES | String |  | Hz | Units for the Effective Occupied Bandwidth. |

### 14.2	spectrumDeviceRegistration_V201404.xsd

| Element | Required | Type | Units | Values | Comments |
| --- | --- | --- | --- | --- | --- |
| Submitter | YES | crt:SubmitterType | - | - | Name and client number of accredited person (see CommonRadioCommunicationsComplexTypes.xsd |
| SpectrumLicenceNumber | YES | Integer | - | - | The spectrum licence number that the submission relates to. One spectrum licence per submission. |
| SubmissionID | YES | Token | - | - | A field containing the submitter’s identification for the submission. Each job requires a unique value. |
| DeviceRecord | YES | DeviceRecordType | - | - | An individual device submission element – either contains details of a new device registration or details of a device to be de-registered. There may be one or more multiple requests of either type. |
| CustomerDeviceID | YES | Token | - | - | Customer defined identification number attached to each device submission (should be unique). Used to identify the success or failure of individual device submissions in ACMA report to customer. |
| ACMARegisteredDeviceId | YES | Integer | - | - | The unique ACMA assigned number identifying the device to be deregistered. |
| SiteID | YES | Integer | - | - | The unique ACMA assigned identification number for the site the device’s antenna is located. |
| GroupStatus | YES | Boolean | - | True, False | A Boolean field indicating whether the device is to be registered as part of a group registration as defined in the relevant S145(4) determination for each spectrum licensed band. |
| EffectiveRadius | NO | Decimal | km |  | The effective radius of a registered device as defined in the relevant S145(4) Determination for each spectrum licensed band. Units in kilometres. Only used for group registrations. |
| AntennaID | YES | Integer | - | - | The unique identification number assigned to an antenna by the ACMA. An antenna may be referenced by this number. |
| EmissionDesignator | YES | String | - | - | The designation of the transmitter’s emission as defined by Appendix 1 of the ITU-R Radio Regulations. |
| ReceiverProtection | YES | crt:ReceiverProtectionType | - | - | As defined in CommonRadioCommunicationsComplexTypes.xsd |
| TransmitterOutputPower | YES | crt:TransmitterOutputPowerType | - | - | As defined in CommonRadioCommunicationsComplexTypes.xsd |
| TransmitterRadiatedPower | YES | crt:TransmitterRadiatedPowerType | - | - | As defined in CommonRadioCommunicationsComplexTypes.xsd |
| HorizontalRadiatedPowerPattern | YES | SpecLicRadiatedPowerPatternType | - | - | The radiated power pattern from the device in the horizontal plane specified for every degree of azimuthal angles from 0 through to 360 degrees. This should match the horizontal radiated pattern used in determining the device boundary as defined in the relevant S145(4) determination. |
| RadiatedPower | YES | Decimal | As defined in RadiatedPowerUnitsType | - | The radiated power from the antenna in the horizontal plane between the start and end azimuthal angles. |
| AzimuthStart | YES | Decimal | Degrees | - | Relative to true North, the start azimuthal angle of the interval over which the radiated power applies. |
| AzimuthEnd | YES | Decimal | Degrees | - | Relative to true North, the end azimuthal angle of the interval over which the radiated power applies. |
| RadiatedPowerUnits | YES | String | - | dBm | Units used for the Horizontal Radiated Power Pattern. Set to dBm for all submissions. |
| ReferenceBandwidth | YES | Integer | Hz, kHz, MHz, or GHz. | - | The reference bandwidth of the Horizontal Radiated Power (HRP) Pattern. The reference bandwidth that applies for a particular spectrum licence band is the same as that defined for the HRP in the relevant S145(4) determination. |
| ReferenceBandwidthUnit | YES | String | - | Hz, kHz, MHz, or GHz. | Units for the Reference Bandwidth. |
| S145 | YES | Boolean | - | True, False | A Boolean field indicating whether the device has been registered in compliance with the S145(4) Determination that applies to the spectrum licensed band. |
| GuardSpace | NO | GuardSpaceType- | - | - | Where a device does not comply with the S145(4) determination, it may be registered where there is sufficient guard space in order to mitigate interference. |
| GuardSpaceIndicator | YES for GuardSpace | Boolean | - | True, False | A Boolean field indicating whether the device has been registered through the use of guard space. |
| MitigationValue | YES for GuardSpace | Decimal | dB | - | A decimal field containing an attenuation value representing the additional mitigation between the registered device and an affected service. |
| Agreement | NO | AgreementType- | - | - | Where a device does not comply with the S145(4) Determination, it may be registered where an agreement exists between the spectrum licensees which interference may be caused to. A unique identifying number for this agreement is used for the purposes of device registration. The same agreement ID can be used to register as many devices as it applies to. |
| AgreementNumber | YES for Agreements | Integer | - | 20133 20134 | An integer field specifying the identification number of the IIC agreement type in force (refer to section 8.9). |
| CoreConditionAgreement | NO | Integer | - | 20131 20132 | Where a core condition agreement is in place (as defined on the spectrum licence), a device may be registered with core conditions agreed to by all affected frequency-adjacent licences and area-adjacent licences. A unique identifying number for this agreement is used for the purposes of device registration. The same agreement can be used to register as many devices as it applies to. |
| Frequency | YES | crt:FrequencyType |  |  | As defined in CommonRadioCommunicationsComplexTypes.xsd |
| Manual Assessment | YES | Boolean | - | True / False | A field indicating whether manual assessment by ACMA is requested. |