ACMA compliance priority 2019–20

Small cell deployments—carrier compliance with licence conditions relating to electromagnetic energy (EME)

JUlY 2020

Canberra

Red Building 
Benjamin Offices
Chan Street 
Belconnen ACT

PO Box 78
Belconnen ACT 2616

T	+61 2 6219 5555
F	+61 2 6219 5353

Melbourne

Level 32 
Melbourne Central Tower
360 Elizabeth Street 
Melbourne VIC

PO Box 13112
Law Courts 
Melbourne VIC 8010

T	+61 3 9963 6800
F	+61 3 9963 6899

Sydney

Level 5 
The Bay Centre
65 Pirrama Road 
Pyrmont NSW

PO Box Q500
Queen Victoria Building 
NSW 1230

T	+61 2 9334 7700 or 1800 226 667
F	+61 2 9334 7799

Copyright notice

https://creativecommons.org/licenses/by/4.0/

With the exception of coats of arms, logos, emblems, images, other third-party material or devices protected by a trademark, this content is made available under the terms of the Creative Commons Attribution 4.0 International (CC BY 4.0) licence.

We request attribution as © Commonwealth of Australia (Australian Communications and Media Authority) 2020.

All other rights are reserved.

The Australian Communications and Media Authority has undertaken reasonable enquiries to identify material owned by third parties and secure permission for its reproduction. Permission may need to be obtained from third parties to re-use their material.

Written enquiries may be sent to:

Manager, Editorial Services
PO Box 13112
Law Courts
Melbourne VIC 8010
Email: info@acma.gov.au

Executive summary	1

Background	2

Methodology	4

Site selection	4

Conducting the audits	5

Preliminary assessment	6

Audit results	7

Telstra	7

Optus	7

Vodafone	8

Summary	10

Appendix A—List of sites audited	11

Appendix B—Detailed audit findings	19

Executive summary

The deployment of small cell base stations has increased in recent years, with a corresponding increase in community concern with the technology and carrier deployment practices. With a more expansive small cell and 5G rollout expected over the next few years, in April 2019, the ACMA nominated the deployment of small cell base stations a compliance priority for 2019–20. The findings will provide a baseline for the ACMA to continue its focus on 5G and electromagnetic energy (EME) emissions (including small cells deployments) as part of its compliance priorities for 2020–21.

There are two main elements of the regulation of small cells under the legislation administered by the ACMA:

carrier consultation and notification requirements when deploying small cells under the C564:2018 Mobile Phone Base Station Deployment (the Deployment Code)

- carrier compliance with the mandatory standards for EME emissions that are given effect through licence conditions under the Radiocommunications Licence Conditions (Apparatus Licence) Determination 2015 (the Licence Determination).

This report focuses on the carrier’s compliance with the Licence Determination for small cell deployments.

In late 2019 and early 2020, we conducted an audit of carriers’ records to assess their compliance with the Licence Determination. A total of 153 small cell sites across Australia were selected and carrier records were gathered from Telstra, Optus and Vodafone for analysis against the requirements of the Licence Determination.

This compliance activity aimed to determine whether the carriers were complying with their record-keeping licence conditions under the Licence Determination which, among other things, are intended to ensure the EME levels emitted by transmitters do not exceed safe levels for general public exposure. This audit coincided with our audit of carriers’ small cells consultation and notification requirements under the Deployment Code undertaken in early 2020.

Findings from this audit do not suggest there is any evidence of non-compliance with EME emission levels from the small cells audited. The EME measurement program undertaken as part of this compliance priority found the average overall EME levels near all small cell sites measured were less than one per cent of the safe levels for general public exposure set by the Australian Radiation Protection and Nuclear Safety Agency (ARPANSA).

The audit provided some assurance that carriers are compliant with their record-keeping licence conditions under the Licence Determination. However, some issues were identified in the carriers’ maintenance of site information on the ACMA Register of Radiocommunications Licences (RRL) and the Radio Frequency National Site Archive (RFNSA). Although these findings were not of material consequence to the reporting of EME levels at each of these sites, it is intended that the findings of this audit will inform practice-improvements by the carriers for their record-keeping and information provision requirements.

# Background

Small cells are low-powered base stations that provide additional capacity and coverage in highly populated areas and strengthen local coverage in regional and rural areas. They are generally placed on existing infrastructure such as light and power poles. Small cells are generally used to deliver 4G mobile services and will play an important role in providing 5G mobile telecommunications.

In April 2019, noting the continuing public interest and concern about public health impacts from the level of EME emissions from small cell facilities, the ACMA announced carrier compliance with the EME Standard for small cell deployments as a compliance priority for 2019–20  (the small cells compliance program).

The announcement followed an audit in January 2019 of TPG Telecom Limited’s (TPG) compliance with its EME obligations for its small cell deployments (the TPG compliance assessment). The audit found no significant issues of non-compliance.

Consistent with the TPG compliance assessment, the smalls cells compliance program has continued our focus on the role of carriers, specifically Telstra, Optus and Vodafone, to ascertain their compliance with their EME-related licence conditions under the Licence Determination for small cell deployments.

There are two main elements of the regulation of small cells under the legislation administered by the ACMA:

carrier consultation and notification requirements when deploying small cells, under the Deployment Code

carrier compliance with the mandatory standards for EME emissions that are given effect through licence conditions, which relate to the operation of transmitters and associated record-keeping obligations under the Licence Determination.

This report focuses on the carriers’ compliance with their record-keeping obligations under the Licence Determination for small cell deployments. A separate report on the carriers’ compliance with their consultation and notification requirements under the Deployment Code is also available on the ACMA website.

Where telecommunications carriers plan to deploy mobile phone radiocommunication facilities, including small cells, they need to comply with the Deployment Code, which sets out consultation and EME reporting requirements for proposed sites. The Deployment Code requires that carriers respond to all reasonable requests for predicted EME levels at specific locations. Carriers also maintain a website with information of installed base stations, including site specific environmental EME reports. This information is made available on the RFNSA.

The ACMA, through the Licence Determination, sets licence conditions with which carriers must comply, to ensure EME levels emitted by transmitters do not exceed the safe levels for general public exposure. ARPANSA sets the EME human exposure limits in its Radiation Protection Standard for Maximum Exposure Levels to Radiofrequency Fields – 3 kHz to 300 GHz (ARPANSA Standard). The ARPANSA Standard specifies limits of human exposure to radiofrequency fields in the range 3 kHz to 300GHz to prevent adverse effects, and basic restrictions for occupational exposure, general public exposure, and equipment and usage parameters.

The Licence Determination stipulates that the level of emissions from a transmitter must not exceed the limits for the general public exposure category of the ARPANSA Standard at places accessible to a member of the general public.

Part 4 of the Licence Determination also requires carriers to maintain records and provide information to the ACMA on request. This audit has been undertaken in accordance with these obligations to ascertain each of the carrier’s compliance with their EME-related licence conditions for small cells.

The audit focuses on the Licence Determination but also draws upon obligations relating to publishing EME reports, including those made available on the RFNSA. We utilised information available on the RFNSA and records provided by the carriers to perform an audit of Telstra, Optus and Vodafone’s records.

# Methodology

The audit program consisted of the following activities:

preliminary engagement with the carriers to inform them of the audit program

site selection, by collecting information on the location of small cells nationally and applying a sampling method

conducting the audit, through formal requests for the carriers’ records for a sample of small cells and then assessing those records against the requirements of the Licence Determination

engaging with the carriers about discrepancies identified in a preliminary assessment of the carrier’s records

assessing any new information provided by the carriers to make our audit findings

identifying any issues of non-compliance and other emerging issues

reporting on the findings of the audit

engaging with the carriers about our findings and associated compliance actions required to be undertaken.

## Site selection

In August 2019, we met with the carriers and the Australian Mobile Telecommunications Association to discuss the scope of the audit program. We then wrote to each of the carriers and requested a complete list of their active small cells nationally as at 1 October 2019. Telstra advised of 1,116 active 4G small cells, Optus 128 active 4G small cells and Vodafone 37 active 4G small cells. The lists were cross-checked with records available on the RFNSA and the RRL.

An initial desk-based assessment was undertaken to establish criteria for determining a sample of small cell sites for the audits. This comprised an assessment of all the EME environmental reports, where available, on the RFNSA for all the 4G small cells, an assessment of a selection of sites in CBD locations, with and without points of interest, and an assessment of the facilities with the highest-predicted EME levels.

The following criteria were then applied to each of the carrier’s compiled lists of small cells to derive a sample for audit:

small cells where there was no EME environmental report available on the RFNSA

small cells in the most densely populated cities nationally, namely locations in Victoria, New South Wales and Queensland

small cells with the highest predicted EME levels across all states and territories nationally.

On application of these criteria, an initial sample of 154 small cells was selected for audit. When we began the audit, several small cell sites were excluded because they were found not to have been activated or were ‘in-building systems’. Most were substituted with another site that met the abovementioned criteria.

The final sample of 153 small cells selected for this audit comprised:

- 95 of Telstra’s small cells (approximately 8.5 per cent of Telstra’s 4G small cells nationally)

- 42 of Optus’s small cells (approximately 32 per cent of Optus’s 4G small cells nationally)

- 16 of Vodafone’s small cells (approximately 43 per cent of Vodafone’s 4G small cells nationally).

A list of small cells sites audited is at Appendix A.

## Conducting the audits

All records were assessed against the EME requirements for the operation of transmitters under Part 3 of the Licence Determination, which imposes conditions to ensure the EME levels from operational transmitters do not exceed the reference levels for general public exposure at places accessible to members of the general public.

In doing so, carriers were formally asked to provide to the ACMA the compliance records required to be kept under section 15 of the Licence Determination for each of the small cells selected for audit.

Subsections 15(1)–(3) of the Licence Determination provides:

(1)   A licensee must keep the following records:

(a)    a declaration of conformity, for the transmitter, that includes the information mentioned in subsection (2);

(b)   if the licensee uses an agent under section 18—a copy of the agency agreement;

(c)    the name and qualifications of any person who has assessed the transmitter for compliance;

(d)   the dates of any assessments;

(e)    the results of any assessments obtained in accordance with paragraph 10(2)(a) or subsection 13(1), including any method of assessment; and

(f)    details of the transmitter including its power level, gain, size, tilt, manufacturer, model number and emission designator.

(2)   For paragraph (1)(a), the information is:

(a)    the licensee’s name, address, ACN (if any), ABN (if any) or ARBN (if any), transmitter licence type and licence number; and

(b)   details of the transmitter, including its location, antenna type and height above the ground or other accessible surface; and

(c)    a statement that the RF field produced by the transmitter meets the requirements of subsection 8(1); and

(d)   the name and position in the organisation of the person who signs the declaration.

(3)   The licensee must amend the declaration of conformity in accordance with any changes to the information included in it.

…

In response to the ACMA’s request for compliance documentation, Optus predominantly relied on EME reports and information available on the RFNSA, Telstra provided internal data and Vodafone provided a combination of its reports on the RFNSA and internal data.

## Preliminary assessment

We undertook a preliminary assessment of all the carriers’ records against the requirements of the Licence Determination and identified a number of discrepancies.  We then wrote to the carriers to seek clarification about each of the discrepancies, including whether they were administrative in nature or related to non-compliance with the Licence Determination. It is important to note the discrepancies did not imply non-compliance, but that further information was required to inform our final assessment of the carrier records, described further below.

In Telstra’s records, discrepancies were initially identified in records provided for 44 of its small cells during the preliminary assessment. The nature of the discrepancies was:

the antenna height, type, or orientation for a number of the small cells differed in the records provided when compared to the corresponding site report on the RFNSA

details of the transmitter/s and/or technology provided in the EME Guide, compliance report and certificate differed from the actual site configuration

the ACMA site identification number provided did not correspond with records held by the ACMA for that site

for one site, an antenna appeared to have been mounted close to a rooftop parapet, potentially posing an EME hazard.

For Optus’s records, discrepancies were initially identified in records for 22 of its small cells audited during the preliminary assessment. The nature of those discrepancies was:

technical information in the declaration of conformity and in its report on the RFNSA differed to information in the RRL

power levels in the declaration of conformity differed from the power level in the corresponding EME report for those small cells on the RFNSA

the compliance inspection date in the declaration of conformity differed from the information available on the RFNSA for a number of small cell sites

potential EME hazards at several sites.

For Vodafone’s records, discrepancies were initially identified in records for 16 of its small cells audited during the preliminary assessment. The nature of the discrepancies was:

technical information in the declaration of conformity and in its report on the RFNSA differed from information in the RRL

power levels in the declaration of conformity differed from the power level in the corresponding EME report for those small cells on the RFNSA

additional information was required in support of the Declaration of Conformity.

The carriers were cooperative in providing information to the ACMA in response to the preliminary assessment.

## Audit results

Our audit findings are documented below for the records provided by each of the carriers in accordance with the Licence Determination.

The detailed audit findings are set out in the table at Appendix B.

### Telstra

Telstra demonstrated a high level of compliance with the Licence Determination, with no substantive non-compliance found. However, a number of discrepancies were found, with some requiring further action. These are documented below.

For 25 of the small cells audited, the Declaration of Conformity, which included technical data to support the declaration, contained incomplete or inaccurate data. This related to omitted or inaccurate information in the Declaration of Conformity submitted for the ACMA licence number or site identification number, or licensed power levels or technologies required to be updated in accordance with licence information in the RRL and, in some cases, the EME Guide on the RFNSA. We have found these discrepancies have no material consequence to the EME levels at each of these sites, however, the information supplied does require correction. We will request Telstra correct its records and submit them to us for further assessment.

While the focus of this audit was on the accuracy of the EME-related records provided under section 15 of the Licence Determination, other observations were made:

For four of the small cells audited, the information in the RFNSA did not reflect the operational configuration of the small cell, including details of the technology or sector orientations described in the records provided or in the ACMA licence records. The information in the RFNSA should be consistent with the operational configuration of the site. We will request this information be updated and monitor Telstra’s progress with this.

For 15 of the small cells audited, the operational site configuration was not recorded correctly in the RRL, including details of the technology or sector orientation. Telstra is required to apply to the ACMA to update its licence records to reflect the operational small cell configuration. We will request that this information be updated and monitor Telstra’s progress with this.

We will also be seeking assurance from Telstra about its compliance with its record-keeping obligations for all other sites.

### Optus

Optus demonstrated a high level of compliance with the Licence Determination, with no substantive non-compliance found. However, a number of discrepancies were found, with some requiring further action. These are documented below.

For one of the small cells audited, the Declaration of Conformity, which included technical data to support the declaration, contained details of a non-operational transmitter. We found this discrepancy has no material consequence to the EME levels at each of these sites, however, the information supplied does require correction. We will request Optus correct its records and submit them to the ACMA for further assessment.

While the focus of this audit was on the accuracy of the EME-related records provided under section 15 of the Licence Determination, other observations were made:

For four of the small cells audited, the information in the RFNSA (EME Guide) did not reflect the information in the Declaration of Conformity. For three of these sites, the EME Guide did not reflect the higher maximum output power of the transmitters provided in the Declaration of Conformity. For another site, the location of the site was incorrectly illustrated in the EME Guide. The information in the RFNSA should be consistent with the Declaration of Conformity. We will engage with Optus regarding the accuracy of information on the RFNSA.

We will also be seeking assurance from Optus about its compliance with its record-keeping obligations for all other sites.

### Vodafone

Vodafone demonstrated a high level of compliance with the Licence Determination, with no substantive non-compliance found. However, a number of discrepancies were found, with some requiring further action. These are documented below.

For four of the small cells audited, the Declaration of Conformity, which included technical data to support the declaration, contained incomplete or inaccurate data relating to the technology. We have found that these discrepancies have no material consequence to the EME levels at each of these sites. However, the information supplied does require correction and we will request Vodafone correct its records and submit them to the ACMA for further assessment.

While the focus of this audit was on the accuracy of the EME-related records provided under section 15 of the Licence Determination, other observations were made:

For eight of the small cells audited, the information in the RFNSA (EME Guide) did not reflect the information provided in the Declaration of Conformity. For seven of these sites, the EME Guide did not reflect the higher maximum output power of the transmitters provided in the Declaration of Conformity. For three of these sites, the EME Guide did not reflect the site configuration specified in the Declaration of Conformity. The information in the RFNSA should be consistent with the Declaration of Conformity. We will engage with Vodafone regarding the accuracy of information on the RFNSA.

For three of the small cells audited, the operational site configuration was not recorded correctly in the RRL. Vodafone is required to apply to the ACMA to update its licence records to reflect the technology or power levels of these sites. We will request that this information be updated and monitor Vodafone’s progress with this.

We will also be seeking assurance from Vodafone about its compliance with its record-keeping obligations for all other sites.

# Summary

The audit findings demonstrate the carrier’s compliance with their EME-related licence conditions under the Licence Determination.

On conclusion of the audit, we found:

All of the carriers have demonstrated the required knowledge of their EME obligations and associated record-keeping requirements under the Licence Determination.

All of the carriers have processes in place to comply with their record-keeping obligations under the Licence Determination.

There were no significant issues of non-compliance identified with the record-keeping obligations under s15 of the Licence Determination. We will request carriers correct any discrepancies found in their records and provide them to us for further assessment. We will also seek assurances of their compliance with the record-keeping obligations for other sites.

The EME measurement program undertaken as part of this compliance priority found the average overall EME levels near all small cell sites measured were less than one per cent of the ARPANSA levels for general public exposure.

While the focus of this audit was to ascertain carriers’ compliance with their record-keeping conditions under the Licence Determination, a number of other observations were made:

administrative issues regarding maintaining accurate site information in the RRL.

inaccuracies in site information available in the RFNSA

information in the RRL and the RFNSA should be consistent with the Declaration of Conformity for each site.

We will request carriers update information in the RRL and in the RFNSA where inaccuracies or discrepancies have been identified and monitor their progress.

# Appendix A—List of sites audited

Click on the hyperlinks attached to the RFNSA site number to access individual site reports.

| Carrier | RFNSA site number | Address | Suburb | State | Postcode | Relevant reference in the Licensing Determination |
| --- | --- | --- | --- | --- | --- | --- |
| Telstra | 2068022 | 24 Edinburgh Rd | WILLOUGHBY | NSW | 2068 | Section 15 |
| Telstra | 2000574 | 2 Alfred St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000578 | 94 King St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000582 | 39–41 York St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000598 | 53–65 Sussex St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000601 | 465 Kent St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000602 | 110 Elizabeth St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000606 | 54 Pitt St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000608 | 155 Macquarie St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000609 | Cnr of Hunter St & Chifley Square | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000611 | 19–29 Martin Place | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000617 | 101 York St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000627 | 1 Lang St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000639 | 151 Castlereagh St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2000641 | 2 Market St | SYDNEY | NSW | 2000 | Section 15 |
| Telstra | 2011012 | 22 Wylde St | POTTS POINT | NSW | 2011 | Section 15 |
| Telstra | 2046011 | 249 Great North Rd | FIVE DOCK | NSW | 2046 | Section 15 |
| Telstra | 2060011 | Waverton Railway Station Bay Rd | WAVERTON | NSW | 2060 | Section 15 |
| Telstra | 2065014 | Wollstonecraft Railway Station Shirley Rd | WOLLSTONE- CRAFT | NSW | 2065 | Section 15 |
| Telstra | 2068013 | 14 Artarmon Rd | WILLOUGHBY | NSW | 2068 | Section 15 |
| Telstra | 2069006 | 84 Boundary St | ROSEVILLE | NSW | 2069 | Section 15 |
| Telstra | 2071007 | Eastern Arterial Rd | EAST KILLARA | NSW | 2071 | Section 15 |
| Telstra | 2076011 | 48 Lucinda Ave | WAHROONGA | NSW | 2076 | Section 15 |
| Telstra | 2095007 | 7340 West Esplanade | MANLY WEST | NSW | 2095 | Section 15 |
| Telstra | 2099020 | 6 Jersey Place | CROMER | NSW | 2099 | Section 15 |
| Telstra | 2100019 | 65 Tristram Rd | BEACON HILL | NSW | 2100 | Section 15 |
| Telstra | 2105001 | 2A McCarrs Creek Rd | CHURCH POINT | NSW | 2105 | Section 15 |
| Telstra | 2121023 | Cnr of Epping Rd & Pembroke St | EPPING | NSW | 2121 | Section 15 |
| Telstra | 2122003 | 60A Balaclava Rd | EASTWOOD | NSW | 2122 | Section 15 |
| Telstra | 3000068 | 394–398 Queen St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000075 | 1 Bourke St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000086 | 180 Bourke St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000087 | 238 Russell St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000102 | Cnr of Collins & Elizabeth St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000104 | 236 Queen St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000271 | 176 Swanston St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000280 | 393 Little Bourke St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000282 | 433 Little Collins St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000285 | 180 Flinders Lane | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000287 | 535 Little Lonsdale St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000288 | 609–619 Little Bourke St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000293 | 128 Little Bourke St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3000302 | 306 Little Collins St | MELBOURNE | Vic. | 3000 | Section 15 |
| Telstra | 3002019 | Cnr of Batman Ave & Swan St | MELBOURNE | Vic. | 3002 | Section 15 |
| Telstra | 3053010 | 693 Elizabeth St | NORTH MELBOURNE | Vic. | 3053 | Section 15 |
| Telstra | 3031039 | Flemington Birdcage | FLEMINGTON | Vic. |  | Section 15 |
| Telstra | 3031037 | 500 Epsom Rd | FLEMINGTON | Vic. | 3031 | Section 15 |
| Telstra | 3121032 | Cnr of Wixon St & Barkers Rd | HAWTHORN WEST | Vic. | 3121 | Section 15 |
| Telstra | 3141041 | 10 Luxton Rd | SOUTH YARRA | Vic. | 3141 | Section 15 |
| Telstra | 3152010 | 390 Burwood Hwy | WANTIRNA SOUTH | Vic. | 3152 | Section 15 |
| Telstra | 3186016 | Cnr of Hampton & Dendy | BRIGHTON EAST | Vic. | 3187 | Section 15 |
| Telstra | 3199055 | 88 Kars St | FRANKSTON SOUTH | Vic. | 3199 | Section 15 |
| Telstra | 3205069 | 221 Pickles St | PORT MELBOURNE | Vic. | 3207 | Section 15 |
| Telstra | 3797011 | 1590 Little Yarra Rd | POWELLTOWN | Vic. | 3797 | Section 15 |
| Telstra | 3941013 | 2629 Point Nepean Rd | RYE | Vic. | 3941 | Section 15 |
| Telstra | 5041006 | 112 Dundas St | PANORAMA | SA | 5041 | Section 15 |
| Telstra | 7000036 | 118A Hill St | WEST HOBART | Tas. | 7000 | Section 15 |
| Telstra | 7004007 | 24 Salamanca Square | BATTERY POINT | Tas. | 7004 | Section 15 |
| Telstra | 6000044 | 16 Milligan St | PERTH | WA | 6000 | Section 15 |
| Telstra | 6515002 | 14 Main St | COOROW | WA | 6515 | Section 15 |
| Telstra | 4000214 | 144 George St | BRISBANE | Qld | 4000 | Section 15 |
| Telstra | 4017018 | 64 Brighton Terrance | SANDGATE | Qld | 4017 | Section 15 |
| Telstra | 4019007 | 25 Ernest St | WOODY POINT | Qld | 4019 | Section 15 |
| Telstra | 4020009 | 146 Prince Edward Parade | SCARBOROUGH | Qld | 4020 | Section 15 |
| Telstra | 4034030 | 19 Chartwell St | ASPLEY | Qld | 4034 | Section 15 |
| Telstra | 4070007 | 11 Leopardwood Close | MOGGILL | Qld | 4070 | Section 15 |
| Telstra | 4101054 | 19 Musgrave St | WEST END | Qld | 4101 | Section 15 |
| Telstra | 4101055 | 21A Hampstead Rd | HIGHGATE HILL | Qld | 4101 | Section 15 |
| Telstra | 4101056 | 6 Ambleside St | WEST END | Qld | 4101 | Section 15 |
| Telstra | 4101057 | 31 Vulture St | WEST END | Qld | 4101 | Section 15 |
| Telstra | 4101059 | 154 Boundary St | WEST END | Qld | 4101 | Section 15 |
| Telstra | 4101060 | 39 Skinner St | WEST END | Qld | 4101 | Section 15 |
| Telstra | 4101095 | 144 Montague Rd | WEST END | Qld | 4101 | Section 15 |
| Telstra | 4101058 | 36 Vulture St | WEST END | Qld | 4101 | Section 15 |
| Telstra | 4101061 | 54 Hardgrave Rd | WEST END | Qld | 4101 | Section 15 |
| Telstra | 4128008 | 18 Voltaire St | SHAILER PARK | Qld | 4128 | Section 15 |
| Telstra | 4128010 | 60–62 Commercial Drive | SHAILER PARK | Qld | 4128 | Section 15 |
| Telstra | 4159005 | 7 Agnes St | BIRKDALE | Qld | 4159 | Section 15 |
| Telstra | 4165019 | 850–938 Mt Cotton Rd | MOUNT COTTON | Qld | 4165 | Section 15 |
| Telstra | 4170004 | 169 Barwon St | MORNINGSIDE | Qld | 4170 | Section 15 |
| Telstra | 4216009 | 265 Bayview St | HOLLYWELL | Qld | 4216 | Section 15 |
| Telstra | 4224005 | Boyd St South West of Desalination Plant | TUGUN | Qld | 4224 | Section 15 |
| Telstra | 4224007 | 15 Singh St | TUGUN | Qld | 4224 | Section 15 |
| Telstra | 4364002 | 5 Lord St | BROOKSTEAD | Qld | 4364 | Section 15 |
| Telstra | 4500017 | 126 Samsonvale Rd | STRATHPINE | Qld | 4500 | Section 15 |
| Telstra | 4560011 | 44 Flaxton Drive | MAPLETON | Qld | 4560 | Section 15 |
| Telstra | 4563011 | 4 Garnet St | COOROY | Qld | 4563 | Section 15 |
| Telstra | 4573007 | 16 Tea Tree Grove | PEREGIAN SPRINGS | Qld | 4573 | Section 15 |
| Telstra | 4610002 | 29–45 Alford St | KINGAROY | Qld | 4610 | Section 15 |
| Telstra | 4810030 | 1526 Cape Cleveland Rd | CAPE CLEVELAND | Qld | 4810 | Section 15 |
| Telstra | 4810026 | 26 Sallows St | PALLARENDA | Qld | 4810 | Section 15 |
| Telstra | 4861003 | 88–92 Munro St | BABINDA | Qld | 4861 | Section 15 |
| Telstra | 0835014 | 284 Whitewood Rd | HOWARD SPRINGS | NT | 0835 | Section 15 |
| Telstra | 0839001 | 20 Henning Rd | COOLALINGA | NT | 0839 | Section 15 |
| Telstra | 0870062 | 55 Heath Rd | KILGARIFF | NT | 0873 | Section 15 |
| Optus | 2000473 | 470 Pitt St | HAYMARKET | NSW | 2000 | Section 15 |
| Optus | 2000474 | 8 Lee St | HAYMARKET | NSW | 2000 | Section 15 |
| Optus | 2000642 | Smart pole ID: SP02259, Pitt St & Martin Place | SYDNEY | NSW | 2000 | Section 15 |
| Optus | 2000643 | Pole ID: SY18029 Northeast corner of, Martin Place and Elizabeth St | SYDNEY | NSW | 2000 | Section 15 |
| Optus | 2000476 | 112 Bathurst St | SYDNEY | NSW | 2000 | Section 15 |
| Optus | 2000414 | Royal Botanic Gardens (40 small cell structures throughout the Gardens | SYDNEY | NSW | 2000 | Section 15 |
| Optus | 2011013 | Cowper Wharf Rd | POTTS POINT | NSW | 2011 | Section 15 |
| Optus | 2074032 | 14 Kissing Point Rd | TURRAMURRA | NSW | 2074 | Section 15 |
| Optus | 2074033 | 61 Kissing Point Rd | TURRAMURRA | NSW | 2074 | Section 15 |
| Optus | 2074034 | 91 Kissing Point Rd | TURRAMURRA | NSW | 2074 | Section 15 |
| Optus | 2075017 | 307 Mona Vale Rd | ST IVES | NSW | 2075 | Section 15 |
| Optus | 2076020 | Light Pole adjacent to 34 Fox Valley Rd | WAHROONGA | NSW | 2076 | Section 15 |
| Optus | 2093010 | 197–215 Condamine St | BALGOWLAH | NSW | 2093 | Section 15 |
| Optus | 2134039 | Pole LE32086, near 36 Park Ave | BURWOOD | NSW | 2134 | Section 15 |
| Optus | 2134040 | Pole LE21541, opposite 90 Burwood Rd | BURWOOD | NSW | 2134 | Section 15 |
| Optus | 2250092 | 23 York St | POINT FREDERICK | NSW | 2250 | Section 15 |
| Optus | 3000254 | Corner Flinders & Russell St | MELBOURNE | Vic. | 3000 | Section 15 |
| Optus | 3000255 | King & Flinders St, North East corner | MELBOURNE | Vic. | 3000 | Section 15 |
| Optus | 3000237 | Corner Flinders & Swanston St | MELBOURNE | Vic. | 3000 | Section 15 |
| Optus | 3000361 | Corner Spencer & Lonsdale St | MELBOURNE | Vic. | 3000 | Section 15 |
| Optus | 3000362 | Corner Bourke & Queen St | MELBOURNE | Vic. | 3000 | Section 15 |
| Optus | 3000238 | Corner Bourke & Queen St | MELBOURNE | Vic. | 3000 | Section 15 |
| Optus | 3000246 | Corner Spring & Victoria St | MELBOURNE | Vic. | 3000 | Section 15 |
| Optus | 3000247 | North West corner Spring & Flinders St | MELBOURNE | Vic. | 3000 | Section 15 |
| Optus | 3000250 | 99 Spencer St | DOCKLANDS | Vic. | 3008 | Section 15 |
| Optus | 4217083 | 44 Esplanade | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Optus | 4217084 | 26–32 Esplanade | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Optus | 4217085 | 20 Esplanade | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Optus | 4217086 | 2 Esplanade | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Optus | 4217087 | 28 Cavill Ave | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Optus | 4217088 | 4 Esplanade | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Optus | 4217089 | 20 Cavill Ave | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Optus | 4217090 | Esplanade | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Optus | 4217091 | Esplanade | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Optus | 4217092 | Northcliffe Terrace | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Optus | 4217093 | 41 Northcliffe Terrace | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Optus | 4218029 | 71 Broadbeach Boulevard | BROADBEACH | Qld | 4218 | Section 15 |
| Optus | 4218030 | Gold Coast Oceanway | BROADBEACH | Qld | 4218 | Section 15 |
| Optus | 4218031 | Gold Coast Oceanway | BROADBEACH | Qld | 4218 | Section 15 |
| Optus | 4218032 | 175 Old Burleigh Rd | BROADBEACH | Qld | 4218 | Section 15 |
| Optus | 6151019 | Light Pole No.1018 on Road Reserve, adjacent to 80 Mill Point Rd | SOUTH PERTH | WA | 6151 | Section 15 |
| Optus | 6151020 | Light Pole No. 1007 on Road Reserve opposite to 44 Mill Point Rd | SOUTH PERTH | WA | 6151 | Section 15 |
| Vodafone | 2000011 | EA Light Pole SY20232, NW cnr of Dixon and Hay St | HAYMARKET | NSW | 2000 | Section 15 |
| Vodafone | 2000118 | Smart Pole #SY18158 Eddy Ave, opposite Central Station | HAYMARKET | NSW | 2000 | Section 15 |
| Vodafone | 2000414 | 2B Macquarie St | SYDNEY | NSW | 2000 | Section 15 |
| Vodafone | 2000619 | Control Room Bldg, near Tumbalong Park, (31 Wheat Rd) | SYDNEY | NSW | 2000 | Section 15 |
| Vodafone | 2000623 | Light Pole opposite Copenhagen Ice cream, 1A Circular Quay Sydney 2000 | SYDNEY | NSW | 2000 | Section 15 |
| Vodafone | 2017031 | Road Reserve, near 18–20 O'Dea Ave | WATERLOO | NSW | 2017 | Section 15 |
| Vodafone | 2049020 | Road Reserve adjacent, 824–826 Parramatta Rd | LEWISHAM | NSW | 2049 | Section 15 |
| Vodafone | 2132003 | Cnr Meta St. & Paisley Rd | CROYDON | NSW | 2132 | Section15 |
| Vodafone | 2194006 | Ausgrid Pole # LE33432, cnr Beamish & Lilian St | CAMPSIE | NSW | 2194 | Section 15 |
| Vodafone | 2208020 | Trainline Reserve, cnr Kingsgrove & Commercial Rd | KINGSGROVE | NSW | 2208 | Section 15 |
| Vodafone | 2210017 | Road Reserve opposite, 58 Thurlow St | RIVERWOOD | NSW | 2210 | Section 15 |
| Vodafone | 2212005 | Cnr Marco Av & The River Rd | REVESBY | NSW | 2212 | Section 15 |
| Vodafone | 3000020 | Cnr of Spencer and Bourke St | MELBOURNE | Vic. | 3000 | Section 15 |
| Vodafone | 3000149 | Cnr of La Trobe & Swanston St | MELBOURNE | Vic. | 3000 | Section 15 |
| Vodafone | 4217004 | The Promenade Hotel, 20 Orchid Ave | SURFERS PARADISE | Qld | 4217 | Section 15 |
| Vodafone | 4217014 | Cnr of Main Beach Pde & Esplanade, pole # 331083 | SURFERS PARADISE | Qld | 4217 | Section 15 |

# Appendix B—Detailed audit findings

Telstra’s detailed audit findings

| Issues relating to record-keeping provisions in the LCD | Nature of non-compliance |
| --- | --- |
| 15-1 (f) details of the transmitter including its power level, gain, size, tilt, manufacturer, model number and emission designator | The Declaration of Conformity (including supplied technical data to support the declaration) contains incomplete or inaccurate data. Sites affected: RFNSA site numbers 2000606, 2000627, 2000639, 2000641, 2065014, 2071007, 2095007, 2105007, 3000068, 3000075, 3000086, 3000087, 3000102, 3000104, 3000271, 3000280, 3000282, 3000285, 3000287, 3000288, 3000302, 3186016, 3031037, 3121032, 4070007 |
| Issues relating to inaccurate RFNSA data | Nature of discrepancy |
|  | The data in the RFNSA (EME Guide, EME Compliance Report or EME Compliance Certificate) does not reflect the operational configuration of the small cell. Sites affected: RFNSA site numbers 2046011, 2069006, 3031039, 5041006 |
| Issues relating to site licensing | Nature of discrepancy |
|  | The operational site configuration is not recorded correctly in the Register of Radiocommunications Licences database. The ACMA licence records based on the RRL do not reflect the operational small cell configuration. Sites affected: RFNSA site numbers 2069006, 2071007, 2011012, 2060011, 3002019, 3031039, 3053010, 3941013, 4019007, 4101056, 4101058, 4101059, 4101060, 4101095, 4165019 |

Optus’s detailed audit findings

| Issues relating to record keeping provisions in the LCD | Nature of non-compliance |
| --- | --- |
| 15-1 (f) details of the transmitter including its power level, gain, size, tilt, manufacturer, model number and emission designator | The Declaration of Conformity (including supplied technical data to support the declaration) contains inaccurate data. Sites affected: RFNSA site number 3000237 |
| Issues relating to inaccurate RFNSA data and potential issues relating to best practice | Nature of discrepancy |
|  | The RFNSA documentation reflects the (lower) operational transmitter power, however the Declaration of Conformity reflects the maximum licenced power of the site. Sites affected: RFNSA site numbers 2134039, 2134040, 3000361 The RFNSA documentation illustrates incorrect site location. Sites affected: RFNSA site number 3000362 |

Vodafone’s initial audit findings

| Issues relating to record keeping provisions in the LCD | Nature of non-compliance |
| --- | --- |
| 15-1 (f) details of the transmitter including its power level, gain, size, tilt, manufacturer, model number and emission designator | The Declaration of Conformity (including supplied technical data to support the declaration) contains incomplete or inaccurate data. Sites affected: RFNSA site numbers RFNSA site numbers 2000414, 2212005, 3000020, 4217014 |
| Issues relating to RFNSA data and potential issues relating to best practice | Nature of discrepancy |
|  | The RFNSA documentation reflects the (lower) operational transmitter power, however the Declaration of Conformity reflects the maximum licenced power of the site. Sites affected: RFNSA site numbers 2000011, 2000623, 2049020, 2132003, 2194006, 3000149, 4217014 The EME Guide did not reflect the technology provided in the Declaration of Conformity. Sites affected: RFNSA site numbers 2132003, 2210017, 4217014 |
| Issues relating to site licensing | Nature of discrepancy |
|  | The operational site configuration is not recorded correctly in the Register of Radiocommunications Licences database. The ACMA licence records based on the RRL do not reflect the operational small cell configuration. Sites affected: RFNSA site numbers 2049020, 2210017, 3000020 |

Small cells may also be used to provide fixed telecommunications services.

In November 2020, a new search mechanism was added to the RFNSA to enable searches for information of small cell facilities.

Points of interest refer to ‘community sensitive locations’, as defined under the Deployment Code as, “land uses such as residential areas, childcare centres, schools, aged care centres, hospital and regional icons may be considered as sensitive uses in some communities. It is acknowledged that each location should be evaluated on a site by site basis to determine community sensitive locations.”

Sites affected: RFNSA site numbers 2000606, 2000627, 2000639, 2000641, 2065014, 2071007, 2095007, 2105007, 3000068, 3000075, 3000086, 3000087, 3000102, 3000104, 3000271, 3000280, 3000282, 3000285, 3000287, 3000288, 3000302, 3186016, 3031037, 3121032, 4070007.

Sites affected: RFNSA site numbers 2046011, 2069006, 3031039, 5041006.

EME Guide, EME Compliance Report or EME Compliance Certificate.

Sites affected: RFNSA site numbers 2069006, 2071007, 2011012, 2060011, 3002019, 3031039, 3053010, 3941013, 4019007, 4101056, 4101058, 4101059, 4101060, 4101095, 4165019.

Sites affected: RFNSA site numbers 3000237.

Sites affected: RFNSA site numbers 2134039, 2134040, 3000361.

Site affected: RFNSA site number 3000362.

Sites affected: RFNSA site numbers 2000414, 2212005, 3000020, 4217014.

Sites affected: RFNSA site numbers 2000011, 2000623, 2049020, 2132003, 2194006, 2210017, 3000149, 4217014.

Sites affected: RFNSA site numbers 2000011, 2000623, 2049020, 2132003, 2194006, 3000149, 4217014.

Sites affected: RFNSA site numbers 2132003, 2210017, 4217014.

Sites affected: RFNSA site numbers 2049020, 2210017, 3000020.