Mapping of allotment numbers to frequency blocks

Allotments 1 to 6 are for the commercial/community (Cat. 1) multiplex or commercial/community/national (Cat. 2) multiplex.

Note that allotments 1 and 2 are co-channel with the Brisbane, Melbourne and Sydney metropolitan Cat. 1 multiplexes.  Allotment 2 is also co-channel with Adelaide and Perth metropolitan Cat. 1 multiplexes.  Allotments 3 and 4 are adjacent to planned or operating VHF television services.

Allotments 7 and 8 are for the national (Cat. 3) multiplex.

Note that allotment 8 is co-channel with the Cat. 3 multiplexes in all five mainland state capital cities.

Frequency blocks for digital radio services (195-209 MHz)

| Digital Radio (DAB) Frequency Block Identifier | Allotment code number | Australian television channel number | Centre frequency (MHz) | Frequency range (MHz) |
| --- | --- | --- | --- | --- |
| 8A | 3 | 9 | 195.936 | 195.168-196.704 |
| 8B | 7 | 9 | 197.648 | 196.880-198.416 |
| 8C | 6 | 9 | 199.360 | 198.592-200.128 |
| 8D | 5 | 9 | 201.072 | 200.304-201.840 |
| 9A | 1 | 9A | 202.928 | 202.160-203.696 |
| 9B | 2 | 9A | 204.640 | 203.872-205.408 |
| 9C | 8 | 9A | 206.352 | 205.584-207.120 |
| 9D | 4 | 9A | 208.064 | 207.296-208.832 |

Note: The following ‘non-standard’ frequency blocks may also be used in some licence areas. They are only ‘non-standard’ in the sense that they were not determined by automated algorithm process.  Receivers available for sale in Australia should have the capability of tuning all frequency blocks from 174 to 230 MHz.

‘Non-standard’ frequency blocks for digital radio services

| Digital Radio (DAB) Frequency Block Identifier | Allotment code number | Australian television channel number | Centre frequency (MHz) | Frequency range (MHz) |
| --- | --- | --- | --- | --- |
| 7B | - | 8 | 190.640 | 189.872-191.408 |
| 7C | - | 8 | 192.352 | 191.584-193.120 |
| 10B | - | 10 | 211.648 | 210.880-212.416 |
| 10C | - | 10 | 213.360 | 212.592-214.128 |
| 11B | - | 11 | 218.640 | 217.872-219.408 |
| 11C | - | 11 | 220.352 | 219.584-221.120 |