Frequency assignment requirements for narrowband single channel two frequency point-to-point services in the VHF high and 400 MHz bands

Radiocommunications Assignment and Licensing Instruction

rali: FX 17

date of effect: 1 July 2020

Amendment history

| Date | Comments |
| --- | --- |
| October 2001 | Initial RALI |
| December 2019 | Updates to add 50 kHz and the VHF high band. See IFC 31/2019. |
| July 2020 | Update to remove 800/900MHz bands. See IFC 12/2020. |

Suggestions for improvements to Radiocommunications Assignment and Licensing Instruction FX 17 may be addressed to:

The Manager, Spectrum Planning Section
Australian Communications and Media Authority
PO Box 78 
Belconnen ACT 2616

or by email to: freqplan@acma.gov.au.

Please notify the ACMA of any inaccuracy or ambiguity found in this RALI, so that it can be investigated, and appropriate action taken.

Contents

1	Introduction	1

1.1	Purpose	1

2	Service description	2

3	Service model	3

3.1	Target grade of service	3

4	Planning rules	4

4.1	Protection ratios	4

4.2	Reduced protection ratios for services operating close to minimum useable receiver sensitivity	5

5	Frequency assignment policy	6

5.1	Application of planning rules	6

5.2	Channelling arrangements	6

5.3	Channel loading strategy for SCTF services in the VHF high and 400 MHz bands	6

5.4	Inter-service co-ordination procedures	6

6	Frequency coordination procedures	7

6.1	Frequency Selection	7

7	Local environment	9

8	Exceptions	10

9	RALI Authorisation	11

Appendix A: Channelling arrangements	12

A.1	Preface	12

A.2	Combining Channels	12

# Introduction

## Purpose

This Radiocommunications Assignment and Licensing Instruction (RALI) provides advice on frequency assignment policy and coordination procedures for Narrowband Single Channel Two Frequency (SCTF) Fixed Point to Point (P-P) services operating in accordance with:

the VHF High Band Plan (part of RALI MS42); and

the 400 MHz Plan (RALI MS22).

The information in this document reflects the ACMA’s statement of current policy in relation to frequency assignment requirements for narrowband SCTF services operating in the VHF High and 400 MHz bands. In making decisions, accredited frequency assigners and the ACMA’s officers should take all relevant factors into account and decide each case on its merits. Issues relating to this document that appear to fall outside the enunciated policy should be referred to:

The Manager, Spectrum Planning Section
Australian Communications and Media Authority
PO Box 78 
Belconnen ACT 2616

or by email to: freqplan@acma.gov.au.

# Service description

SCTF P-P services in the VHF high and 400 MHz bands are typically used to provide simultaneous bi-directional (full duplex) narrowband communications between two fixed points. Services may use either analogue or digital modulation.

Functionally a SCTF system may be characterised as featuring:

Line-of-site links between stations at high sites employing directional antennas;

Full duplex operation; and

Carriage of either:

Voice services; or

Data services.

The channelling arrangements for SCTF services are provided in the relevant band plans. The method for combining channels for 25 kHz and 50 kHz systems is detailed in Appendix A.

# Service model

The following section presents a service model that is to be applied, with certain allowances to interservice sharing, to all new frequency assignments for narrowband SCTF services operating in the VHF High and 400 MHz bands.

The model is intended to ensure a satisfactory grade of service, while still facilitating a reasonable density of co-channel services within the geographic vicinity.

The service model for SCTF services in the VHF High and 400 MHz bands is notionally characterised as a radiocommunications system where full duplex communications, meeting or exceeding a target grade of service, occur between two terminal stations utilising directional antennas over a substantially unobstructed path.

## Target grade of service

The notional target grade of service for SCTF services is:

a received signal quality of 30 dB SINAD or better for analogue services for more than 90% of the time; and

a BER of better than 10-6 for digital services for more than 90% of the time.

Section 4 specifies planning rules intended to ensure that new SCTF services achieve at least the notional target grade of service specified in the service model.

# Planning rules

SCTF services in the VHF High and 400 MHz bands must comply with the following planning rules:

operation is limited to frequencies authorised under the appropriate frequency plan;

where channelling arrangements cater for 12.5 kHz, 25kHz and 50 kHz channels, 12.5 kHz channels should be used wherever operationally feasible. 25 kHz channels should only be assigned to new services operating at bit rates of at least 16 kbps. 50 kHz channels may only be assigned in low or remote density areas and for data rates exceeding 32 kbps;

a flat co-channel protection ratio of 30 dB applies. However, this may be reduced in some circumstances - see Section 4.2;

a maximum [average] transmitter output power (into the antenna) of 1 W, except for services where the link distance is less than 10 km where the maximum transmitter output power (into the antenna) shall be 100 mW; and

actual antenna radiation performance is to be at least equal to the following notional antenna characteristics:

In High and Medium Spectrum Density Areas:

in the 400 MHz band: a directional antenna with a mid-band gain of least 13 dBi, a minimum front-to-back ratio of 17 dB and a maximum beam width (in E-plane) of 46.

Outside of High and Medium Spectrum Density Areas:

in the VHF High band: a directional antenna with a mid-band gain of least 7 dBi, a minimum front-to-back ratio of 12 dB and a maximum beam width (in E-plane) of 60; and

in the 400 MHz band: a directional antenna with a mid-band gain of least 9 dBi, a minimum front-to-back ratio of 15 dB and a maximum beam width (in E-plane) of 47.

## Protection ratios

The service model applies a flat co-channel protection ratio of 30 dB for all co-channel SCTF services, which reduces by a calculated amount for cases where a victim receiver is operating near its minimum useable sensitivity - see Section 4.2.

Due to the high level of adjacent channel isolation that is inherent to narrowband SCTF receivers, the service model only considers co-channel interference. This assumes that services will operate at no more than the EIRP limits imposed by this RALI and that isolation between channels is 50 dB or better.

The basic 30 dB co-channel protection ratio is intended to ensure that SCTF services achieve actual grades of service that are a least equal to those specified in Section 3.1.

Note: the same co-channel protection ratios apply to 12.5 kHz, 25 kHz and 50 kHz channels.

## Reduced protection ratios for services operating close to minimum useable receiver sensitivity

Where a SCTF receiver is operating close to its noise limited receive threshold the co-channel protection that it can be afforded is reduced. Without such reductions the effective co-channel re-use distance for adjacent services becomes unreasonably large.

Table 1 is to be used to determine the level of co-channel protection that can be afforded to a receiver operating close to its minimum useable sensitivity.

Reduced co-channel protection will apply to:

a proposed new receiver where its own "wanted" signal will be weaker than - 99 dBm; or

any potential victim receiver with a "wanted" signal weaker than - 99 dBm.

| "Wanted" Signal Level (WL) | Co-channel protection ratio applying |
| --- | --- |
| WL -99 dBm | = 30 dB; |
| - 99 > WL > - 129 dBm | = 30 minus "X" dB; where X= (- 99 minus WL) |
| WL ≤ -129 dBm | No protection. |

Calculation of co-channel protection ratios applying to SCTF receivers.

# Frequency assignment policy

## Application of planning rules

SCTF services operating in the VHF High and 400 MHz bands are to comply with the planning rules specified in Section 4.

## Channelling arrangements

Frequency assignments for SCTF services may only be made in accordance with the channelling arrangements for SCTF services made in:

RALI MS42; and

the 400 MHz Plan (RALI MS22).

Frequency assigners are to give full attention to the notes applying in each band plan.

## Channel loading strategy for SCTF services in the VHF high and 400 MHz bands

A bottom-up vertical channel loading strategy is to be applied to all new SCTF services.

A bottom-up vertical channel loading strategy is one in which the lowest available channel must be assigned.

## Inter-service co-ordination procedures

Coordination of P-P services with wideband fixed services in the 400 MHz band is addressed in RALI FX1. In any other cases, because of the diversity and complexity of sharing situations that may arise, it is not possible to provide rigorous and explicit procedures covering all inter-service coordination requirements. In these cases, coordination should be performed in accordance with good engineering practice based on fundamental interference mitigation principles.

# Frequency coordination procedures

The following section outlines a coordination procedure for the frequency assignment of SCTF services in the VHF High and 400 MHz bands.

Note: the procedure given below only outlines the general principles that should be applied when selecting and co-ordinating frequencies. It is the responsibility of the assigner to apply sound engineering principles and take full account of all embargoes, relevant RALIs and Band Plans.

## Frequency Selection

After identifying the prospective frequency range(s) that meet the basic engineering requirements for the proposed link the frequency selection process may be carried out in the following 9 general stages:

- Obtain a current list of licensed services covering the frequency range of interest.

Referring to the list, perform a radial cull around the mean position (mid-point) of the proposed service. The cull is carried out in order to exclude services too far away to affect or be affected by the proposed new service. The choice of a suitable cull radius will depend on terrain and the frequency band but should not be less than 200 km.

Select the most appropriate site sense arrangements for the proposed service.

- Note: assigners should endeavour to follow established transmitter site sense wherever possible. This not only improves the availability of channels later, but also helps to optimise the productivity of prime sites. Rather than mixing site sense assigners should consider using an alternative frequency band. However, as a general guide when it is not possible to avoid mixing site sense, the selected transmission frequency should be at least four channels away from frequencies occupied by receivers nearby. This reduces potential intermodulation problems, which can also affect site productivity.

Referring to the list of current embargoes exclude any channels that cannot be used.

For the 400 MHz band, carry out necessary engineering assessments to exclude channels that are unavailable due to the potential for interference to wideband P-P Services - see RALI FX1.

Using the parameters specified in Table 2 and propagation loss determined by the use of ITU-R P.525 + P.526. The path loss may be determined by use of Table 3 to generate a list of wanted-to-unwanted ratios comparing the proposed service to each co-channel service in the cull area.

| Relative angle off bore-sights | Assumed discrimination |
| --- | --- |
| within ±18 degrees | - 15 dB |
| any other angle | 0 dB |

Assumed cross-polar discrimination for notional antennas.

- Note: the assigner is free to select the antenna polarisation (either horizontal or vertical) that provides the best co-ordination results. However, horizontal polarisation is preferred as additional onsite isolation is then available to and from adjacent vertically polarised land mobile services.

| Propagation Distance | Path Loss |
| --- | --- |
| 0.003 to 40 km | Free space loss + 10dB |
| > 40 km | 104 + 0.55*(distance in km) |

Path loss calculations for SCTF services.

- Note: Formulas are derived from the Longley Rice model applied to an unobstructed line-of-sight path.

Exclude from the list those channels where the co-channel protection ratio required by Section 4.2 is not met. Then select the lowest available channel using the bottom-up vertical channel loading strategy described in Section 5.3.

- Note: selection of a suitable SCTF channel will require finding a pair of available duplex frequencies.

- Document the final details of the service for licensing purposes.

Complete licensing requirements.

- Note: antenna details must be shown on licences in cases where the authorised antenna does not meet the requirements of Schedule 1 of the Radiocommunications Licence Conditions (Fixed Licence) Determination 2015.

# Local environment

There may be circumstances where the channel selected using the procedure above is not the optimal channel to be assigned due to the local environment. Examples are:

a large mountain range offering additional propagation loss to/from a service in an adjacent area;

a transmitter located on a site at a height much greater than the planning model assumes; or

an anomalous propagation mode occurring due to a path over water.

Under such circumstances, propagation path loss may be determined by the use of any appropriate method described in section 4 of ITU-R P.526 (versions 4 through 14). All methods must use a 9 second digital elevation model (such as RadDEM) or better. Other methods for determining the propagation path loss may also be used pending ACMA agreement.

# Exceptions

Exceptions to the requirements of this RALI for prospective assignments require case-by-case consideration by the Manager, Spectrum Planning Section.

A request for exemption from the requirements of this RALI would need to be accompanied by evidence to support the request.

All requests for exemptions should be submitted to freqplan@acma.gov.au.

# RALI Authorisation

Approved     	30/06/2020

Chris Worley
Manager
Spectrum Planning Section
Spectrum Planning and Engineering Branch

Communications Infrastructure Division
Australian Communications and Media Authority

# Appendix A: Channelling arrangements

- A.1	Preface

Each SCTF channel comprises of a pair of frequencies with a fixed frequency separation - a duplex pair. Depending on the site sense (see stage 3 of section 6.1) one end of every SCTF link must be chosen to transmit on the higher frequency of the duplex pair (i.e. to Transmit High). The related receiver at that same end receives on the lower frequency of the pair. The transmitter/receiver at the opposite end of the link operates in complement - transmitting on the lower frequency of the pair (Transmit Low) and receiving on the higher frequency. Tables A1 to A3 list the duplex pairs allocated in each band in terms of transmitter frequency. By convention, the higher duplex frequency is denoted as Transmit High and the lower duplex frequency is denoted as Transmit Low.

- A.2	Combining Channels

Where two 12.5 kHz channels are being assigned contiguously, combine channels 1 and 2, 3 and 4 etc. The 25 kHz channel centre frequency should be selected as follows:

- Calculate the centre frequency of each odd numbered channel (where n=1, 3, 5, etc.);

- Next add 6.25 kHz to each calculated centre frequency. This yields the centre frequency of each possible 25 kHz channel; then

Consistent with the bottom up channel loading strategy expressed in section 5.3, use the frequency selection process outlined in section 6.1 to find the lowest available 25 kHz centre frequency.

Where four 12.5 kHz channels are being assigned contiguously (VHF High and 400 MHz bands), combine channels 1-4, 5-8 etc. The 50 kHz channel centre frequency should be selected as follows:

- Note the centre frequency of each even numbered channel (where n=2, 6, 10, etc);

- Add 6.25 kHz to that centre frequency. This yields the centre frequency of each possible 50 kHz channel; then

Consistent with the bottom up channel loading strategy expressed in section 5.3, use the frequency selection process outlined in section 6.1 to find the lowest available 50 kHz centre frequency.

Note: the same co-channel protection ratios apply to 12.5 kHz, 25 kHz and 50 kHz channels.

| Channel | 12.5 kHz Centre Frequency | 25 kHz Centre Frequency | 50 kHz Centre Frequency | 12.5 kHz Centre Frequency | 25 kHz Centre Frequency | 50 kHz Centre Frequency |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 150.0625 | 150.06875 | 150.08125 | 154.6625 | 154.66875 | 154.68125 |
| 2 | 150.075 |  |  | 154.675 |  |  |
| 3 | 150.0875 | 150.09375 |  | 154.6875 | 154.69375 |  |
| 4 | 150.1 |  |  | 154.7 |  |  |
| 5 | 150.1125 | 150.11875 | 150.13125 | 154.7125 | 154.71875 | 154.73125 |
| 6 | 150.125 |  |  | 154.725 |  |  |
| 7 | 150.1375 | 150.14375 |  | 154.7375 | 154.74375 |  |
| 8 | 150.15 |  |  | 154.75 |  |  |
| 9 | 150.1625 | 150.16875 | 150.18125 | 154.7625 | 154.76875 | 154.78125 |
| 10 | 150.175 |  |  | 154.775 |  |  |
| 11 | 150.1875 | 150.19375 |  | 154.7875 | 154.79375 |  |
| 12 | 150.2 |  |  | 154.8 |  |  |
| 13 | 150.2125 | 150.21875 | 150.23125 | 154.8125 | 154.81875 | 154.83125 |
| 14 | 150.225 |  |  | 154.825 |  |  |
| 15 | 150.2375 | 150.24375 |  | 154.8375 | 154.84375 |  |
| 16 | 150.25 |  |  | 154.85 |  |  |
| 17 | 150.2625 | 150.26875 | 150.28125 | 154.8625 | 154.86875 | 154.88125 |
| 18 | 150.275 |  |  | 154.875 |  |  |
| 19 | 150.2875 | 150.29375 |  | 154.8875 | 154.89375 |  |
| 20 | 150.3 |  |  | 154.9 |  |  |
| 21 | 150.3125 | 150.31875 | 150.33125 | 154.9125 | 154.91875 | 154.93125 |
| 22 | 150.325 |  |  | 154.925 |  |  |
| 23 | 150.3375 | 150.34375 |  | 154.9375 | 154.94375 |  |
| 24 | 150.35 |  |  | 154.95 |  |  |
| 25 | 150.3625 | 150.36875 | 150.38125 | 154.9625 | 154.96875 | 154.98125 |
| 26 | 150.375 |  |  | 154.975 |  |  |
| 27 | 150.3875 | 150.39375 |  | 154.9875 | 154.99375 |  |
| 28 | 150.4 |  |  | 155 |  |  |
| 29 | 150.4125 | 150.41875 | 150.43125 | 155.0125 | 155.01875 | 155.03125 |
| 30 | 150.425 |  |  | 155.025 |  |  |
| 31 | 150.4375 | 150.44375 |  | 155.0375 | 155.04375 |  |
| 32 | 150.45 |  |  | 155.05 |  |  |
| 33 | 150.4625 | 150.46875 | 150.48125 | 155.0625 | 155.06875 | 155.08125 |
| 34 | 150.475 |  |  | 155.075 |  |  |
| 35 | 150.4875 | 150.49375 |  | 155.0875 | 155.09375 |  |
| 36 | 150.5 |  |  | 155.1 |  |  |
| 37 | 150.5125 | 150.51875 | 150.53125 | 155.1125 | 155.11875 | 155.13125 |
| 38 | 150.525 |  |  | 155.125 |  |  |
| 39 | 150.5375 | 150.54375 |  | 155.1375 | 155.14375 |  |
| 40 | 150.55 |  |  | 155.15 |  |  |
| 41 | 150.5625 | 150.56875 | 150.58125 | 155.1625 | 155.16875 | 155.18125 |
| 42 | 150.575 |  |  | 155.175 |  |  |
| 43 | 150.5875 | 150.59375 |  | 155.1875 | 155.19375 |  |
| 44 | 150.6 |  |  | 155.2 |  |  |
| 45 | 150.6125 | 150.61875 | 150.63125 | 155.2125 | 155.21875 | 155.23125 |
| 46 | 150.625 |  |  | 155.225 |  |  |
| 47 | 150.6375 | 150.64375 |  | 155.2375 | 155.24375 |  |
| 48 | 150.65 |  |  | 155.25 |  |  |
| 49 | 150.6625 | 150.66875 | 150.68125 | 155.2625 | 155.26875 | 155.28125 |
| 50 | 150.675 |  |  | 155.275 |  |  |
| 51 | 150.6875 | 150.69375 |  | 155.2875 | 155.29375 |  |
| 52 | 150.7 |  |  | 155.3 |  |  |
| 53 | 150.7125 | 150.71875 | 150.73125 | 155.3125 | 155.31875 | 155.33125 |
| 54 | 150.725 |  |  | 155.325 |  |  |
| 55 | 150.7375 | 150.74375 |  | 155.3375 | 155.34375 |  |
| 56 | 150.75 |  |  | 155.35 |  |  |
| 57 | 150.7625 | 150.76875 | 150.78125 | 155.3625 | 155.36875 | 155.38125 |
| 58 | 150.775 |  |  | 155.375 |  |  |
| 59 | 150.7875 | 150.79375 |  | 155.3875 | 155.39375 |  |
| 60 | 150.8 |  |  | 155.4 |  |  |
| 61 | 150.8125 | 150.81875 | 150.83125 | 155.4125 | 155.41875 | 155.43125 |
| 62 | 150.825 |  |  | 155.425 |  |  |
| 63 | 150.8375 | 150.84375 |  | 155.4375 | 155.44375 |  |
| 64 | 150.85 |  |  | 155.45 |  |  |
| 65 | 150.8625 | 150.86875 | 150.88125 | 155.4625 | 155.46875 | 155.48125 |
| 66 | 150.875 |  |  | 155.475 |  |  |
| 67 | 150.8875 | 150.89375 |  | 155.4875 | 155.49375 |  |
| 68 | 150.9 |  |  | 155.5 |  |  |
| 69 | 150.9125 | 150.91875 | 150.93125 | 155.5125 | 155.51875 | 155.53125 |
| 70 | 150.925 |  |  | 155.525 |  |  |
| 71 | 150.9375 | 150.94375 |  | 155.5375 | 155.54375 |  |
| 72 | 150.95 |  |  | 155.55 |  |  |
| 73 | 150.9625 | 150.96875 | 150.98125 | 155.5625 | 155.56875 | 155.58125 |
| 74 | 150.975 |  |  | 155.575 |  |  |
| 75 | 150.9875 | 150.99375 |  | 155.5875 | 155.59375 |  |
| 76 | 151 |  |  | 155.6 |  |  |
| 77 | 151.0125 | 151.01875 | 151.03125 | 155.6125 | 155.61875 | 155.63125 |
| 78 | 151.025 |  |  | 155.625 |  |  |
| 79 | 151.0375 | 151.04375 |  | 155.6375 | 155.64375 |  |
| 80 | 151.05 |  |  | 155.65 |  |  |
| 81 | 151.0625 | 151.06875 | 151.08125 | 155.6625 | 155.66875 | 155.68125 |
| 82 | 151.075 |  |  | 155.675 |  |  |
| 83 | 151.0875 | 151.09375 |  | 155.6875 | 155.69375 |  |
| 84 | 151.1 |  |  | 155.7 |  |  |
| 85 | 151.1125 | 151.11875 | 151.13125 | 155.7125 | 155.71875 | 155.73125 |
| 86 | 151.125 |  |  | 155.725 |  |  |
| 87 | 151.1375 | 151.14375 |  | 155.7375 | 155.74375 |  |
| 88 | 151.15 |  |  | 155.75 |  |  |
| 89 | 151.1625 | 151.16875 | 151.18125 | 155.7625 | 155.76875 | 155.78125 |
| 90 | 151.175 |  |  | 155.775 |  |  |
| 91 | 151.1875 | 151.19375 |  | 155.7875 | 155.79375 |  |
| 92 | 151.2 |  |  | 155.8 |  |  |
| 93 | 151.2125 | 151.21875 | 151.23125 | 155.8125 | 155.81875 | 155.83125 |
| 94 | 151.225 |  |  | 155.825 |  |  |
| 95 | 151.2375 | 151.24375 |  | 155.8375 | 155.84375 |  |
| 96 | 151.25 |  |  | 155.85 |  |  |
| 97 | 151.2625 | 151.26875 | 151.28125 | 155.8625 | 155.86875 | 155.88125 |
| 98 | 151.275 |  |  | 155.875 |  |  |
| 99 | 151.2875 | 151.29375 |  | 155.8875 | 155.89375 |  |
| 100 | 151.3 |  |  | 155.9 |  |  |
| 101 | 151.3125 | 151.31875 | 151.33125 | 155.9125 | 155.91875 | 155.93125 |
| 102 | 151.325 |  |  | 155.925 |  |  |
| 103 | 151.3375 | 151.34375 |  | 155.9375 | 155.94375 |  |
| 104 | 151.35 |  |  | 155.95 |  |  |
| 105 | 151.3625 | 151.36875 |  | 155.9625 | 155.96875 |  |
| 106 | 151.375 |  |  | 155.975 |  |  |
| 107 | 151.3875 |  |  | 155.9875 |  |  |

Table A1 – VHF High Band Point-to Point Channels

| Channel | 12.5 kHz Centre Frequency | 25 kHz Centre Frequency | 50 kHz Centre Frequency | 12.5 kHz Centre Frequency | 25 kHz Centre Frequency | 50 kHz Centre Frequency |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 403.99375 | 404 | 404.0125 | 413.44375 | 413.45 | 413.4625 |
| 2 | 404.00625 |  |  | 413.45625 |  |  |
| 3 | 404.01875 | 404.025 |  | 413.46875 | 413.475 |  |
| 4 | 404.03125 |  |  | 413.48125 |  |  |
| 5 | 404.04375 | 404.05 | 404.0625 | 413.49375 | 413.5 | 413.5125 |
| 6 | 404.05625 |  |  | 413.50625 |  |  |
| 7 | 404.06875 | 404.075 |  | 413.51875 | 413.525 |  |
| 8 | 404.08125 |  |  | 413.53125 |  |  |
| 9 | 404.09375 | 404.1 | 404.1125 | 413.54375 | 413.55 | 413.5625 |
| 10 | 404.10625 |  |  | 413.55625 |  |  |
| 11 | 404.11875 | 404.125 |  | 413.56875 | 413.575 |  |
| 12 | 404.13125 |  |  | 413.58125 |  |  |
| 13 | 404.14375 | 404.15 | 404.1625 | 413.59375 | 413.6 | 413.6125 |
| 14 | 404.15625 |  |  | 413.60625 |  |  |
| 15 | 404.16875 | 404.175 |  | 413.61875 | 413.625 |  |
| 16 | 404.18125 |  |  | 413.63125 |  |  |
| 17 | 404.19375 | 404.2 | 404.2125 | 413.64375 | 413.65 | 413.6625 |
| 18 | 404.20625 |  |  | 413.65625 |  |  |
| 19 | 404.21875 | 404.225 |  | 413.66875 | 413.675 |  |
| 20 | 404.23125 |  |  | 413.68125 |  |  |
| 21 | 404.24375 | 404.25 | 404.2625 | 413.69375 | 413.7 | 413.7125 |
| 22 | 404.25625 |  |  | 413.70625 |  |  |
| 23 | 404.26875 | 404.275 |  | 413.71875 | 413.725 |  |
| 24 | 404.28125 |  |  | 413.73125 |  |  |
| 25 | 404.29375 | 404.3 | 404.3125 | 413.74375 | 413.75 | 413.7625 |
| 26 | 404.30625 |  |  | 413.75625 |  |  |
| 27 | 404.31875 | 404.325 |  | 413.76875 | 413.775 |  |
| 28 | 404.33125 |  |  | 413.78125 |  |  |
| 29 | 404.34375 | 404.35 | 404.3625 | 413.79375 | 413.8 | 413.8125 |
| 30 | 404.35625 |  |  | 413.80625 |  |  |
| 31 | 404.36875 | 404.375 |  | 413.81875 | 413.825 |  |
| 32 | 404.38125 |  |  | 413.83125 |  |  |
| 33 | 404.39375 | 404.4 | 404.4125 | 413.84375 | 413.85 | 413.8625 |
| 34 | 404.40625 |  |  | 413.85625 |  |  |
| 35 | 404.41875 | 404.425 |  | 413.86875 | 413.875 |  |
| 36 | 404.43125 |  |  | 413.88125 |  |  |
| 37 | 404.44375 | 404.45 | 404.4625 | 413.89375 | 413.9 | 413.9125 |
| 38 | 404.45625 |  |  | 413.90625 |  |  |
| 39 | 404.46875 | 404.475 |  | 413.91875 | 413.925 |  |
| 40 | 404.48125 |  |  | 413.93125 |  |  |
| 41 | 404.49375 | 404.5 | 404.5125 | 413.94375 | 413.95 | 413.9625 |
| 42 | 404.50625 |  |  | 413.95625 |  |  |
| 43 | 404.51875 | 404.525 |  | 413.96875 | 413.975 |  |
| 44 | 404.53125 |  |  | 413.98125 |  |  |
| 45 | 404.54375 | 404.55 | 404.5625 | 413.99375 | 414 | 414.0125 |
| 46 | 404.55625 |  |  | 414.00625 |  |  |
| 47 | 404.56875 | 404.575 |  | 414.01875 | 414.025 |  |
| 48 | 404.58125 |  |  | 414.03125 |  |  |
| 49 | 404.59375 | 404.6 | 404.6125 | 414.04375 | 414.05 | 414.0625 |
| 50 | 404.60625 |  |  | 414.05625 |  |  |
| 51 | 404.61875 | 404.625 |  | 414.06875 | 414.075 |  |
| 52 | 404.63125 |  |  | 414.08125 |  |  |
| 53 | 404.64375 | 404.65 | 404.6625 | 414.09375 | 414.1 | 414.1125 |
| 54 | 404.65625 |  |  | 414.10625 |  |  |
| 55 | 404.66875 | 404.675 |  | 414.11875 | 414.125 |  |
| 56 | 404.68125 |  |  | 414.13125 |  |  |
| 57 | 404.69375 | 404.7 | 404.7125 | 414.14375 | 414.15 | 414.1625 |
| 58 | 404.70625 |  |  | 414.15625 |  |  |
| 59 | 404.71875 | 404.725 |  | 414.16875 | 414.175 |  |
| 60 | 404.73125 |  |  | 414.18125 |  |  |
| 61 | 404.74375 | 404.75 | 404.7625 | 414.19375 | 414.2 | 414.2125 |
| 62 | 404.75625 |  |  | 414.20625 |  |  |
| 63 | 404.76875 | 404.775 |  | 414.21875 | 414.225 |  |
| 64 | 404.78125 |  |  | 414.23125 |  |  |
| 65 | 404.79375 | 404.8 | 404.8125 | 414.24375 | 414.25 | 414.2625 |
| 66 | 404.80625 |  |  | 414.25625 |  |  |
| 67 | 404.81875 | 404.825 |  | 414.26875 | 414.275 |  |
| 68 | 404.83125 |  |  | 414.28125 |  |  |
| 69 | 404.84375 | 404.85 | 404.8625 | 414.29375 | 414.3 | 414.3125 |
| 70 | 404.85625 |  |  | 414.30625 |  |  |
| 71 | 404.86875 | 404.875 |  | 414.31875 | 414.325 |  |
| 72 | 404.88125 |  |  | 414.33125 |  |  |
| 73 | 404.89375 | 404.9 | 404.9125 | 414.34375 | 414.35 | 414.3625 |
| 74 | 404.90625 |  |  | 414.35625 |  |  |
| 75 | 404.91875 | 404.925 |  | 414.36875 | 414.375 |  |
| 76 | 404.93125 |  |  | 414.38125 |  |  |
| 77 | 404.94375 | 404.95 | 404.9625 | 414.39375 | 414.4 | 414.4125 |
| 78 | 404.95625 |  |  | 414.40625 |  |  |
| 79 | 404.96875 | 404.975 |  | 414.41875 | 414.425 |  |
| 80 | 404.98125 |  |  | 414.43125 |  |  |
| 81 | 404.99375 | 405 |  | 414.44375 | 414.45 |  |
| 82 | 405.00625 |  |  | 414.45625 |  |  |

Table A2 – 400 MHz Segments B/J Point-to Point Channels

| Channel | 12.5 kHz Centre Frequency | 25 kHz Centre Frequency | 50 kHz Centre Frequency | 12.5 kHz Centre Frequency | 25 kHz Centre Frequency | 50 kHz Centre Frequency |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 450.49375 | 450.5 | 450.5125 | 459.99375 | 460 | 460.0125 |
| 2 | 450.50625 |  |  | 460.00625 |  |  |
| 3 | 450.51875 | 450.525 |  | 460.01875 | 460.025 |  |
| 4 | 450.53125 |  |  | 460.03125 |  |  |
| 5 | 450.54375 | 450.55 | 450.5625 | 460.04375 | 460.05 | 460.0625 |
| 6 | 450.55625 |  |  | 460.05625 |  |  |
| 7 | 450.56875 | 450.575 |  | 460.06875 | 460.075 |  |
| 8 | 450.58125 |  |  | 460.08125 |  |  |
| 9 | 450.59375 | 450.6 | 450.6125 | 460.09375 | 460.1 | 460.1125 |
| 10 | 450.60625 |  |  | 460.10625 |  |  |
| 11 | 450.61875 | 450.625 |  | 460.11875 | 460.125 |  |
| 12 | 450.63125 |  |  | 460.13125 |  |  |
| 13 | 450.64375 | 450.65 | 450.6625 | 460.14375 | 460.15 | 460.1625 |
| 14 | 450.65625 |  |  | 460.15625 |  |  |
| 15 | 450.66875 | 450.675 |  | 460.16875 | 460.175 |  |
| 16 | 450.68125 |  |  | 460.18125 |  |  |
| 17 | 450.69375 | 450.7 | 450.7125 | 460.19375 | 460.2 | 460.2125 |
| 18 | 450.70625 |  |  | 460.20625 |  |  |
| 19 | 450.71875 | 450.725 |  | 460.21875 | 460.225 |  |
| 20 | 450.73125 |  |  | 460.23125 |  |  |
| 21 | 450.74375 | 450.75 | 450.7625 | 460.24375 | 460.25 | 460.2625 |
| 22 | 450.75625 |  |  | 460.25625 |  |  |
| 23 | 450.76875 | 450.775 |  | 460.26875 | 460.275 |  |
| 24 | 450.78125 |  |  | 460.28125 |  |  |
| 25 | 450.79375 | 450.8 | 450.8125 | 460.29375 | 460.3 | 460.3125 |
| 26 | 450.80625 |  |  | 460.30625 |  |  |
| 27 | 450.81875 | 450.825 |  | 460.31875 | 460.325 |  |
| 28 | 450.83125 |  |  | 460.33125 |  |  |
| 29 | 450.84375 | 450.85 | 450.8625 | 460.34375 | 460.35 | 460.3625 |
| 30 | 450.85625 |  |  | 460.35625 |  |  |
| 31 | 450.86875 | 450.875 |  | 460.36875 | 460.375 |  |
| 32 | 450.88125 |  |  | 460.38125 |  |  |
| 33 | 450.89375 | 450.9 | 450.9125 | 460.39375 | 460.4 | 460.4125 |
| 34 | 450.90625 |  |  | 460.40625 |  |  |
| 35 | 450.91875 | 450.925 |  | 460.41875 | 460.425 |  |
| 36 | 450.93125 |  |  | 460.43125 |  |  |
| 37 | 450.94375 | 450.95 | 450.9625 | 460.44375 | 460.45 | 460.4625 |
| 38 | 450.95625 |  |  | 460.45625 |  |  |
| 39 | 450.96875 | 450.975 |  | 460.46875 | 460.475 |  |
| 40 | 450.98125 |  |  | 460.48125 |  |  |
| 41 | 450.99375 | 451 | 451.0125 | 460.49375 | 460.5 | 460.5125 |
| 42 | 451.00625 |  |  | 460.50625 |  |  |
| 43 | 451.01875 | 451.025 |  | 460.51875 | 460.525 |  |
| 44 | 451.03125 |  |  | 460.53125 |  |  |
| 45 | 451.04375 | 451.05 | 451.0625 | 460.54375 | 460.55 | 460.5625 |
| 46 | 451.05625 |  |  | 460.55625 |  |  |
| 47 | 451.06875 | 451.075 |  | 460.56875 | 460.575 |  |
| 48 | 451.08125 |  |  | 460.58125 |  |  |
| 49 | 451.09375 | 451.1 | 451.1125 | 460.59375 | 460.6 | 460.6125 |
| 50 | 451.10625 |  |  | 460.60625 |  |  |
| 51 | 451.11875 | 451.125 |  | 460.61875 | 460.625 |  |
| 52 | 451.13125 |  |  | 460.63125 |  |  |
| 53 | 451.14375 | 451.15 | 451.1625 | 460.64375 | 460.65 | 460.6625 |
| 54 | 451.15625 |  |  | 460.65625 |  |  |
| 55 | 451.16875 | 451.175 |  | 460.66875 | 460.675 |  |
| 56 | 451.18125 |  |  | 460.68125 |  |  |
| 57 | 451.19375 | 451.2 | 451.2125 | 460.69375 | 460.7 | 460.7125 |
| 58 | 451.20625 |  |  | 460.70625 |  |  |
| 59 | 451.21875 | 451.225 |  | 460.71875 | 460.725 |  |
| 60 | 451.23125 |  |  | 460.73125 |  |  |
| 61 | 451.24375 | 451.25 | 451.2625 | 460.74375 | 460.75 | 460.7625 |
| 62 | 451.25625 |  |  | 460.75625 |  |  |
| 63 | 451.26875 | 451.275 |  | 460.76875 | 460.775 |  |
| 64 | 451.28125 |  |  | 460.78125 |  |  |
| 65 | 451.29375 | 451.3 | 451.3125 | 460.79375 | 460.8 | 460.8125 |
| 66 | 451.30625 |  |  | 460.80625 |  |  |
| 67 | 451.31875 | 451.325 |  | 460.81875 | 460.825 |  |
| 68 | 451.33125 |  |  | 460.83125 |  |  |
| 69 | 451.34375 | 451.35 | 451.3625 | 460.84375 | 460.85 | 460.8625 |
| 70 | 451.35625 |  |  | 460.85625 |  |  |
| 71 | 451.36875 | 451.375 |  | 460.86875 | 460.875 |  |
| 72 | 451.38125 |  |  | 460.88125 |  |  |
| 73 | 451.39375 | 451.4 | 451.4125 | 460.89375 | 460.9 | 460.9125 |
| 74 | 451.40625 |  |  | 460.90625 |  |  |
| 75 | 451.41875 | 451.425 |  | 460.91875 | 460.925 |  |
| 76 | 451.43125 |  |  | 460.93125 |  |  |
| 77 | 451.44375 | 451.45 | 451.4625 | 460.94375 | 460.95 | 460.9625 |
| 78 | 451.45625 |  |  | 460.95625 |  |  |
| 79 | 451.46875 | 451.475 |  | 460.96875 | 460.975 |  |
| 80 | 451.48125 |  |  | 460.98125 |  |  |
| 81 | 451.49375 | 451.5 |  | 460.99375 | 461 |  |
| 82 | 451.50625 |  |  | 461.00625 |  |  |

Table A3 – 400 MHz Segments B/J Point-to-Point Channels

See RALI FX01 "Frequency Assignment Requirements for Narrowband Fixed and Mobile Services with Wideband Fixed Services in the 403-420 MHz Band".

Refer to paragraph 2.3 of Spectrum Planning Report 3-86.

Typical narrowband receivers will deliver output BERs of at least 10-6 at signal-to-noise ratios as low as 15 dB at the receiver input.

See ACMA's current "Apparatus Licence Fee Schedule" for definitions and maps for "High", "Medium" and "Low" Spectrum Density Areas.

Consistent with Schedule 1 of the Radiocommunications Licence Conditions (Fixed Licence) Determination 2015.

Protection Ratio: the minimum wanted-to-unwanted ratio that must be achieved for a successful co-ordination with a nearby service.

Limited by intrinsic system noise to around - 129 dBm in typical 25 kHz receivers.

Free Space Loss (FSL) = 32.5+ 20*Log (Distance in km) + 20*Log (Frequency in MHz).

The model estimates loss to a 90% confidence level.

https://www.legislation.gov.au/Details/F2018C00890