Frequency plan for Services in the 
800 MHz band

(803–890 MHz)

Amendment History

| Date | Comments |
| --- | --- |
| August 2015 | Initial draft covering material taken from the Radiocommunications 900 MHz Band Plan 1992 |
| September 2015 | Finalised RALI following public consultation. |
| July 2016 | Updated to prepare for the transition of single frequency fixed services as per Milestone 1 of the 803-960 MHz review implementation plan, see: The ACMAs long-term strategy for the 803-960 MHz band. The primary change is to provide a new channelling raster for fixed services in the 845-849 MHz segment, and to remove redundant allocations in the 825-845 and 870-890 MHz segments. |
| August 2018 | Updated to provide incremental allocation changes (contained in appendix A-D) to support the 803-960 MHz review implementation plan until completion. |
| July 2020 | Update to include provisions previously contained in Embargo 64. These include provisions to give priority access to services required to relocate as a result of the 803–960 MHz review. See IFC 12/2020. |

Suggestions for improvements to Radiocommunications Assignment and Licensing Instruction MS 40 may be addressed to The Manager, Spectrum Planning Section, ACMA at PO Box 78, Belconnen, ACT, 2616, or by e-mail to freqplan@acma.gov.au.  It would be appreciated if notification to ACMA of any inaccuracy or ambiguity found be made without delay in order that the matter may be investigated, and appropriate action taken.

# Table of Contents

1	Purpose	3

2	Future arrangements in this frequency band	3

3	Spectrum arrangements	4

3.1	Assignment Conditions	4

4	Relationship with RALI MS 41	4

5	RALI Authorisation	5

Appendix A: Allocation and Channelling Arrangements from 1 July 2020 to 30 June 2023	6

Appendix B: Allocation and Channelling Arrangements from 1 July 2023 to 30 June 2024	14

Appendix C: Allocation and Channelling Arrangements – effective from 1 July 2024	21

frequency plan for the 800 MHz band 803 - 890 MHz

# Purpose

The purpose of this Radiocommunications Assignment and Licensing Instruction (RALI) is to set out spectrum planning arrangements for radiocommunications services in the frequency band 803 to 890 MHz (known as the 800 MHz Band).  The RALI specifies the service allocations including the frequency limits of each allocation segment, and the channelling arrangements within these segments (where applicable).

This RALI, together with the related RALI MS 41 (for the 900 MHz band) replaces the Radiocommunications 900 MHz Band Plan 1992 which ceased on 1 October 2015 under “sunsetting” provisions of the Legislative Instruments Act 2003.

In making decisions, accredited frequency assigners and the ACMA’s officers should take all relevant factors into account and decide each case on its merits.  Issues relating to this document that appear to fall outside the enunciated policy should be referred to the Manager, Spectrum Engineering Section, PO Box 78, Belconnen, ACT, 2616, or by e-mail to freqplan@acma.gov.au.

# Future arrangements in this frequency band

The ACMA commenced a review of arrangements in the 803–960 MHz frequency band in May 2011 (the Review). The scope of the Review included services operating within the band 820-960 MHz, as well as consideration of future use of the 803-820 MHz segment vacated as part of the digital dividend. In November 2015, the ACMA completed the Review and released its decision in the paper: ‘The ACMA’s long-term strategy for the 803-960 MHz band’ (the Decision paper). The Decision paper contains a range of decisions on reforms to the 803-960 MHz band, as well as a detailed plan for the implementation of these reforms (see Section 3.3 of the Decision paper).

This RALI provides incremental changes to the spectrum allocations in the 800 MHz band, effective at different points in time between 1 July 2020 and 1 July 2024 (contained in appendix A-C), to support the implementation of the new arraignments in the 800 MHz band. These incremental changes align with the Review implementation plan detailed in the Decision paper.  As such the spectrum allocations in Appendix C reflect the final arrangements in the band at the completion of the review implementation process.

Appendix A-C identify some frequency segments as being ‘not allocated’. As detailed in the Decision paper, these segments are being cleared of legacy services with the intention of being designated for spectrum licensing in the future.

# Spectrum arrangements

Spectrum allocation and channelling arrangements for the 800 MHz band are contained in the appendices to this RALI, and incrementally come into effect from the following dates.

- Appendix A – effective from 1 July 2020 until 30 June 2023

- Appendix B – effective from 1 July 2023 until 30 June 2024

- Appendix C – effective from 1 July 2024

As detailed in the Decision paper, arrangements may be introduced to allow two frequency fixed links to access spectrum in the adjacent trunked land mobile segment (806-809/851-854 MHz) on a secondary basis as a means of alleviating potential congestion in the future (see section 3.2.3.1 of the Decision paper). These arrangements are currently not included in the spectrum allocations detailed in the appendices to this RALI. The ACMA will consider introduction of these arrangements at a future date if congestion issues arise.

## Assignment Conditions

Services are to be assigned in accordance with this RALI.

Channelling arrangements other than those specified in this RALI may be authorised where such arrangements provide for more efficient use of the spectrum, when compared to the channelling arrangements specified for that segment in this RALI. In determining the spectrum efficiency of a service, without limiting the range of matters which may be taken into account, the following matters may be considered:

- Occupied bandwidth;

- Adjacent channel performance;

- The distance from the transmitter that the channel may be used again without causing harmful interference; and

- The impact that introduction of the service will have upon existing services

# Relationship with RALI MS 41

One outcome of the Review was to separate planning arrangements for the 803-960 MHz band into two distinct band plans – RALI MS 40 for the 803-890 MHz band and RALI MS 41 for the 890-960 MHz band. The reasons for creating two separate band plans are twofold:

- To provide the flexibility needed to make future revisions, in particular to implement the outcomes of the Review; and

- To reflect a general change in community and industry-accepted nomenclature on the frequency extremes of both bands.

Although the planning arrangements in the range 803-960 MHz are now split between the two new administrative band plans, the following services are currently covered by both plans:

- Two frequency fixed services in the range 852-857/928-933 MHz (until 30 June 2021); and

- Radiolocation services in the range 850-915 MHz.

# RALI Authorisation

Approved     	30/06/2020

Chris Worley

Manager
Spectrum Planning Section
Spectrum Planning and Engineering Branch

Communications Infrastructure Division
Australian Communications and Media Authority

# Appendix A: Allocation and Channelling Arrangements from 1 July 2020 to 30 June 2023

The allocation and channelling arrangements for the 800 MHz band, effective from 1 July 2020 until 30 June 2023, are set out in the following figure and tables:

Figure 1: 800 MHz band services (803 - 890 MHz) diagram

Table 1: Service allocations and channelling arrangements for primary services

Table 2: Service allocations and channelling arrangements for secondary services

Figure 1: 800 MHz band (803 – 890 MHz) services diagram – effective from 1 July 2020 to 30 June 2023

Table 1: Service allocations and channelling arrangements for primary services (note 1) – effective from 1 July 2020 to 30 June 2023

| Item | Segment frequency limits (MHz) (lower limit exclusion, upper limit inclusive) | Service allocation | Paired segment (MHz) | Maximum channel bandwidth (kHz) | Channel centre frequency formula | Range of integer values for variable ‘n’ (inclusive) | First channel/last channel centre frequency (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 803 – 804 | Guard band (see note 2) |  |  |  |  |  |
| 2 | 804 – 805.5 | Fixed point-to-point (two frequency) (see note 3) | 849 – 850.5 | 200 | 803.99375 + n (0.0125) | 1 to 120 | 804.00625 805.49375 |
| 3 | 805.5 – 806 | Fixed point-to-multipoint (two frequency, base receive) (see note 3) | 850.5 – 851 | 25 | 805.49375 + n (0.0125) | 1 to 40 | 805.50625 805.99375 |
| 4 | 806 – 809 | Land Mobile Service (trunked, base receive) (see notes 4 and 5) | 851 – 854 | 25 | 805.99375 + n (0.0125) | 1 to 240 | 806.00625 808.99375 |
| 5 | 809 – 820 | Not allocated (see note 6) |  |  |  |  |  |
| 6 | 820 – 825 | Land Mobile Service (trunked, base receive) (see notes 4, 7 and 8) | 865 – 870 | 25 | 819.9875 + n (0.025) | 1 to 200 | 820.0125 824.9875 |
| 7 | 825 – 845 | Subject to spectrum licensing (see note 9) |  |  |  |  |  |
| 8 | 845 - 846.5 | Fixed point-to-point (single frequency) (see notes 10 and 11) |  | 400 | 844.99375 + n (0.0125) | 1 to 120 | 845.00625 846.49375 |
|  |  | Sound Outside Broadcast Link |  |  |  |  |  |
| 9 | 846.5 – 849 | Fixed point-to-point (single frequency) (see notes 10 and 11) |  | 400 | 846.49375 + n (0.0125) | 1 to 200 | 846.50625 848.99375 |
| 10 | 849 – 850.5 | Fixed point-to-point (two frequency) (see note 3) | 804 – 805.5 | 200 | 848.99375 + n (0.0125) | 1 to 120 | 849.00625 850.49375 |
| 11 | 850.5 – 851 | Fixed point-to-multipoint (two frequency, base transmit) (see note 3) | 805.5 – 806 | 25 | 850.49375 + n (0.0125) | 1 to 40 | 850.50625 850.99375 |
| 12 | 851 – 852 | Land Mobile Service (trunked, base transmit) (see notes 4 and 5) | 806 – 807 | 25 | 850.99375 + n (0.0125) | 1 to 80 | 851.00625 851.99375 |
| 13 | 852 – 853.5 | Land Mobile Service (trunked, base transmit) (see notes 4 and 5) | 807 – 808.5 | 25 | 851.99375 + n (0.0125) | 1 to 120 | 852.00625 853.49375 |
|  |  | Fixed point-to-point (two frequency) (see notes 12 and 13) |  |  |  |  |  |
| 14 | 853.5 – 854 | Land Mobile Service (trunked, base transmit) (see notes 4 and 5) | 808.5 – 809 | 25 | 853.49375 + n (0.0125) | 1 to 40 | 853.50625 853.99375 |
|  |  | Fixed point-to-multipoint (two frequency, base receive) (see notes 12 and 13) |  |  |  |  |  |
| 15 | 854 – 857 | Fixed point-to-point (two frequency) (see notes 13 and 14) |  |  |  |  |  |
| 16 | 857 – 861 | Fixed point-to-point (single frequency) (see notes 7, 8 and 15) |  | 25 | 856.9875 + n (0.025) | 1 to 160 | 857.0125 860.9875 |
|  |  | Cordless Telephone Service (see notes 7, 8 and 15) |  |  |  |  |  |
| 17 | 861 – 865 | Cordless Telephone Service (see notes 7, 8 and 16) |  | 100 | 860.95 + n (0.1) | 1 to 40 | 861.05 864.95 |
| 18 | 865 – 870 | Land Mobile Service (trunked, base transmit) (see notes 4, 7 and 8) | 820 – 825 | 25 | 864.9875 + n (0.025) | 1 to 200 | 865.0125 869.9875 |
| 19 | 870 – 890 | Subject to spectrum licensing (see note 9) |  |  |  |  |  |

Notes:

1.	The allocations for these primary services are represented in Figure 1.

2.	No services are to be assigned in the 803-804 MHz guard band.

3.	Until 1 July 2021, no new assignments are to be made under this allocation, except where the assignment is to a licensee seeking to move an existing licensed service from within the frequency bands 852–857 MHz and 928–933 MHz. A new licence in the range 804–806/849–851 MHz can be taken out before the existing licence it is to replace (in the range 852–857/928–933 MHz) has been surrendered. From 1 July 2021, new assignments can be made under this allocation.

4.	Segments referenced by this note may be authorised for use by users of:

- land mobile services (two frequency), if such services are used in conjunction with a land mobile service (trunked); or

- other land mobile services that make equivalent or more efficient use of the spectrum, as compared to the designated service allocation.

5.	No new assignments are to be made under this allocation, except where the assignment is to a licensee seeking to move an existing licensed service from within the frequency bands 820–825 MHz and 865–870 MHz. A new licence in the range 806–809/851–854 MHz can be taken out before the existing licence it is to replace (in the range 820–825/865–870 MHz) has been surrendered.

6.	No new assignments are to be made in this segment.

7.	Stations operating under this allocation must cease operating in this segment by 30 June 2024. Existing apparatus licences under this allocation will not be extended beyond 30 June 2024.

8.	No new assignments are to be made under this allocation, except where the assignment is for the expansion of an existing system or where one assignment is exchanged for another within the band and within the same area, contingent on minimal changes in overall spectrum use. Where an assignment is made on the basis that it is for the expansion of an existing system, or when one assignment is exchanged for another, the licensee shall acknowledge that:

- As the result of the 803–960 MHz band review allocations and systems in this segment are required to be spectrally relocated or for operations to cease in the future. Timeframes are available in the ACMA’s long-term strategy for the 803–960 MHz band.

- The assignment does not provide any additional protection or rights and they will be subject to the same transition times as all other assignments.

9.	On 15 July 1997, the Minister for Communications and the Arts declared that:

- throughout Australia, the bands 825-830 MHz, 835-845 MHz, 870-875 MHz and 880-890 MHz; and

- in parts of Australia, the bands 830-835 MHz and 875-880 MHz,

are subject to re-allocation by issuing spectrum licences. Furthermore, on 15 October 2000, the Minister for Communications, Information Technology and the Arts declared that further parts of Australia in the bands 830-835 MHz and 875-880 MHz are subject to re-allocation. No apparatus licences may be issued in bands and areas which are subject to spectrum licensing provisions, see spectrum embargo 26.

10.	Services in this segment which were licensed prior to 1 July 2016 do not need to comply with the specified channelling arrangements, with the exception of the maximum channel bandwidth.

11.	No new assignments are to be made under this allocation except where the assignment is to a licensee seeking to move an existing licenced service from within the frequency band 857–861 MHz. A new licence in the range 845–849 MHz can be taken out before the existing licence it is to replace (in the range 857–861 MHz) has been surrendered.

12.	Services operated under this allocation are paired with 928–930 MHz which is allocated under the 900 MHz Plan.

13.	No new assignments are to be made under this allocation in this segment. Existing assignments under this allocation must cease operating in this segment by 30 June 2021. Existing apparatus licences under this allocation will not be extended beyond 30 June 2021. This allocation will conclude in this segment on 30 June 2021.

14.	Services operated under this allocation are paired with 930–933 MHz, which is allocated under the 900 MHz Plan.

15.	Special licensing and coordination arrangements apply to this service.

16.	Equipment used in this service must comply with relevant ACMA mandated technical standards.

Table 2: Service allocations and channelling arrangements for secondary services (note 1) – effective from 1 July 2020 to 30 June 2023

| Item | Segment frequency limits (MHz) (lower limit exclusion, upper limit inclusive) | Service allocation |
| --- | --- | --- |
| 1 | 846.5 - 850 | Sound Outside Broadcast Link |
| 2 | 850 – 851 | Sound Outside Broadcast Link |
|  |  | Radiolocation (see note 2) |
| 3 | 851 - 890 | Radiolocation (see note 2) |

Notes:

1.	The allocations for these secondary services are represented in Figure 1.

2.	Assignments to users other than the Department of Defence or the Australian Defence Force will not normally be authorised for this service. In this segment the service is primary in offshore areas.

# Appendix B: Allocation and Channelling Arrangements from 1 July 2023 to 30 June 2024

The allocation and channelling arrangements for the 800 MHz band, effective from 1 July 2023 until 30 June 2024, are set out in the following figure and tables:

Figure 2: 800 MHz band services (803 - 890 MHz) diagram

Table 3: Service allocations and channelling arrangements for primary services

Table 4: Service allocations and channelling arrangements for secondary services

Figure 2: 800 MHz band (803 – 890 MHz) services diagram – effective from 1 July 2023 to 30 June 2024

Table 3: Service allocations and channelling arrangements for primary services (note 1) – effective from 1 July 2023 to 30 June 2024

| Item | Segment frequency limits (MHz) (lower limit exclusion, upper limit inclusive) | Service allocation | Paired segment (MHz) | Maximum channel bandwidth (kHz) | Channel centre frequency formula | Range of integer values for variable ‘n’ (inclusive) | First channel/last channel centre frequency (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 803 - 804 | Guard band (see note 2) |  |  |  |  |  |
| 2 | 804 – 805.5 | Fixed point-to-point (two frequency) | 849 – 850.5 | 200 | 803.99375 + n (0.0125) | 1 to 120 | 804.00625 805.49375 |
| 3 | 805.5 - 806 | Fixed point-to-multipoint (two frequency, base receive) | 850.5 – 851 | 25 | 805.49375 + n (0.0125) | 1 to 40 | 805.50625 805.99375 |
| 4 | 806 - 809 | Land Mobile Service (trunked, base receive) (see notes 3 and 4) | 851 – 854 | 25 | 805.99375 + n (0.0125) | 1 to 240 | 806.00625 808.99375 |
| 5 | 809 - 820 | Not allocated (see note 5) |  |  |  |  |  |
| 6 | 820 – 825 | Land Mobile Service (trunked, base receive) (see notes 3 and 6) | 865 – 870 |  |  |  |  |
| 7 | 825 - 845 | Subject to spectrum licensing (see note 7) |  |  |  |  |  |
| 8 | 845 - 846.5 | Fixed point-to-point (single frequency) (see note 8) |  | 400 | 844.99375 + n (0.0125) | 1 to 120 | 845.00625 846.49375 |
|  |  | Sound Outside Broadcast Link |  |  |  |  |  |
| 9 | 846.5 - 849 | Fixed point-to-point (single frequency) (see note 8) |  | 400 | 846.49375 + n (0.0125) | 1 to 200 | 846.50625 848.99375 |
| 10 | 849 – 850.5 | Fixed point-to-point (two frequency) | 804 – 805.5 | 200 | 848.99375 + n (0.0125) | 1 to 120 | 849.00625 850.49375 |
| 11 | 850.5 - 851 | Fixed point-to-multipoint (two frequency, base transmit) | 805.5 - 806 | 25 | 850.49375 + n (0.0125) | 1 to 40 | 850.50625 850.99375 |
| 12 | 851 - 854 | Land Mobile Service (trunked, base transmit) (see notes 3 and 4) | 806 – 809 | 25 | 850.99375 + n (0.0125) | 1 to 240 | 851.00625 853.99375 |
| 13 | 854 - 857 | Not allocated (see note 9) |  |  |  |  |  |
| 14 | 857 – 861 | Fixed point-to-point (single frequency) (see note 6) |  |  |  |  |  |
|  |  | Cordless Telephone Service (see note 6) |  |  |  |  |  |
| 15 | 861 - 865 | Cordless Telephone Service (see notes 6 and 10) |  |  |  |  |  |
| 16 | 865 – 870 | Land Mobile Service (trunked, base transmit) (see notes 3 and 6) | 820 – 825 |  |  |  |  |
| 17 | 870 – 890 | Subject to spectrum licensing (see note 7) |  |  |  |  |  |

Notes:

1.	The allocations for these primary services are represented in Figure 2.

2.	No services are to be assigned in the 803-804 MHz guard band.

3.	Segments referenced by this note may be authorised for use by users of:

- land mobile services (two frequency), if such services are used in conjunction with a land mobile service (trunked); or

- other land mobile services that make equivalent or more efficient use of the spectrum, as compared to the designated service allocation.

4.	No new assignments are to be made under this allocation, except where the assignment is to a licensee seeking to move an existing licensed service from within the frequency bands 820–825 MHz and 865–870 MHz. A new licence in the range 806–809/851–854 MHz can be taken out before the existing licence it is to replace (in the range 820–825/865–870 MHz) has been surrendered.

5.	No new assignments are to be made in this segment.

6.	No new assignments are to be made under this allocation in this segment. Existing services under this allocation must cease operating in this segment by 30 June 2024. Existing apparatus licences under this service will not be extended beyond 30 June 2024. This allocation will conclude in this segment on 30 June 2024.

7.	On 15 July 1997, the Minister for Communications and the Arts declared that

- throughout Australia, the bands 825-830 MHz, 835-845 MHz, 870-875 MHz and 880-890 MHz; and

- in parts of Australia, the bands 830-835 MHz and 875-880 MHz,

are subject to re-allocation by issuing spectrum licences. Furthermore, on 15 October 2000, the Minister for Communications, Information Technology and the Arts declared that further parts of Australia in the bands 830-835 MHz and 875-880 MHz are subject to re-allocation. No apparatus licences may be issued in bands and areas which are subject to spectrum licensing provisions, see spectrum embargo 26.

8.	Services in this segment which were licensed prior to 1 July 2016 do not need to comply with the specified channelling arrangements, with the exception of the maximum channel bandwidth.

9.	No new assignments are to be made in this segment, other than secondary radiolocation services (see Table 4).

10.	Equipment used in this service must comply with relevant ACMA mandated technical standards.

Table 4: Service allocations and channelling arrangements for secondary services (note 1) – effective from 1 July 2023 to 30 June 2024

| Item | Segment frequency limits (MHz) (lower limit exclusion, upper limit inclusive) | Service allocation |
| --- | --- | --- |
| 1 | 846.5 - 850 | Sound Outside Broadcast Link |
| 2 | 850 – 851 | Sound Outside Broadcast Link |
|  |  | Radiolocation (see note 2) |
| 3 | 851 - 890 | Radiolocation (see note 2) |

Notes:

1.	The allocations for these secondary services are represented in Figure 2.

2.	Assignments to users other than the Department of Defence or the Australian Defence Force will not normally be authorised for this service. In this segment the service is primary in offshore areas.

# Appendix C: Allocation and Channelling Arrangements – effective from 1 July 2024

The allocation and channelling arrangements for the 800 MHz band, effective from 1 July 2024, are set out in the following figure and tables:

Figure 3: 800 MHz band services (803 - 890 MHz) diagram

Table 5: Service allocations and channelling arrangements for primary services

Table 6: Service allocations and channelling arrangements for secondary services

Figure 3: 800 MHz band (803 – 890 MHz) services diagram – effective from 1 July 2024

Table 5: Service allocations and channelling arrangements for primary services (note 1) – effective from 1 July 2024

| Item | Segment frequency limits (MHz) (lower limit exclusion, upper limit inclusive) | Service allocation | Paired segment (MHz) | Maximum channel bandwidth (kHz) | Channel centre frequency formula | Range of integer values for variable ‘n’ (inclusive) | First channel/last channel centre frequency (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 803 - 804 | Guard band (see note 2) |  |  |  |  |  |
| 2 | 804 – 805.5 | Fixed point-to-point (two frequency) | 849 – 850.5 | 200 | 803.99375 + n (0.0125) | 1 to 120 | 804.00625 805.49375 |
| 3 | 805.5 - 806 | Fixed point-to-multipoint (two frequency, base receive) | 850.5 – 851 | 25 | 805.49375 + n (0.0125) | 1 to 40 | 805.50625 805.99375 |
| 4 | 806 - 809 | Land Mobile Service (trunked, base receive) (see note 3) | 851 – 854 | 25 | 805.99375 + n (0.0125) | 1 to 240 | 806.00625 808.99375 |
| 5 | 809 - 825 | Not allocated (see note 4) |  |  |  |  |  |
| 6 | 825 - 845 | Subject to spectrum licensing (see note 5) |  |  |  |  |  |
| 7 | 845 - 846.5 | Fixed point-to-point (single frequency) (see note 6) |  | 400 | 844.99375 + n (0.0125) | 1 to 120 | 845.00625 846.49375 |
|  |  | Sound Outside Broadcast Link |  |  |  |  |  |
| 8 | 846.5 - 849 | Fixed point-to-point (single frequency) (see note 6) |  | 400 | 846.49375 + n (0.0125) | 1 to 200 | 846.50625 848.99375 |
| 9 | 849 – 850.5 | Fixed point-to-point (two frequency) | 804 – 805.5 | 200 | 848.99375 + n (0.0125) | 1 to 120 | 849.00625 850.49375 |
| 10 | 850.5 - 851 | Fixed point-to-multipoint (two frequency, base transmit) | 805.5 – 806 | 25 | 850.49375 + n (0.0125) | 1 to 40 | 850.50625 850.99375 |
| 11 | 851 - 854 | Land Mobile Service (trunked, base transmit) (see note 3) | 806 – 809 | 25 | 850.99375 + n (0.0125) | 1 to 240 | 851.00625 853.99375 |
| 12 | 854 - 870 | Not allocated (see note 7) |  |  |  |  |  |
| 13 | 870 – 890 | Subject to spectrum licensing (see note 5) |  |  |  |  |  |

Notes:

1.	The allocations for these primary services are represented in Figure 3.

2.	No services are to be assigned in the 803-804 MHz guard band.

3.	Segments referenced by this note may be authorised for use by users of:

- land mobile services (two frequency), if such services are used in conjunction with a land mobile service (trunked); or

- other land mobile services that make equivalent or more efficient use of the spectrum, as compared to the designated service allocation.

4.	No new assignments are to be made in this segment.

5.	On 15 July 1997, the Minister for Communications and the Arts declared that

- throughout Australia, the bands 825-830 MHz, 835-845 MHz, 870-875 MHz and 880-890 MHz; and

- in parts of Australia, the bands 830-835 MHz and 875-880 MHz,

are subject to re-allocation by issuing spectrum licences. Furthermore, on 15 October 2000, the Minister for Communications, Information Technology and the Arts declared that further parts of Australia in the bands 830-835 MHz and 875-880 MHz are subject to re-allocation. No apparatus licences may be issued in bands and areas which are subject to spectrum licensing provisions, see spectrum embargo 26.

6.	Services in this segment which were licensed prior to 1 July 2016 do not need to comply with the specified channelling arrangements, with the exception of the maximum channel bandwidth.

7.	No new assignments are to be made in this segment, other than secondary radiolocation services (see Table 6).

Table 6: Service allocations and channelling arrangements for secondary services (note 1) – effective from 1 July 2024

| Item | Segment frequency limits (MHz) (lower limit exclusion, upper limit inclusive) | Service allocation |
| --- | --- | --- |
| 1 | 846.5 - 850 | Sound Outside Broadcast Link |
| 2 | 850 – 851 | Sound Outside Broadcast Link |
|  |  | Radiolocation (see note 2) |
| 3 | 851 - 890 | Radiolocation (see note 2) |

Notes:

1.	The allocations for these secondary services are represented in Figure 3.

2.	Assignments to users other than the Department of Defence or the Australian Defence Force will not normally be authorised for this service. In this segment the service is primary in offshore areas.

It should be noted that the new two-frequency fixed link (804-806/849-851 MHz) and trunked land mobile (806-809/851-854 MHz) allocations will be created a year earlier than originally indicated in the Decision paper. This is to aid an earlier transition to the new frequency segments if licensees wish to do so. This does not impact the timeframes established in the Decision paper for incumbent services to vacate these frequency segments.