Frequency plan for Services in the 900 MHz band

890 - 960 MHz

Amendment History

| Date | Comments |
| --- | --- |
| August 2015 | Initial draft covering material taken from the Radiocommunications 900 MHz Band Plan 1992 |
| September 2015 | Finalised RALI following public consultation. |
| August 2018 | Updated to provide incremental allocation changes (contained in appendix A-B) to support the 803-960 MHz review implementation plan until completion. |
| July 2020 | Draft for consultation: Update to include provisions previously contained in Embargo 64. See IFC 12/2020. |

Suggestions for improvements to Radiocommunications Assignment and Licensing Instruction MS 41 may be addressed to The Manager, Spectrum Planning Section, ACMA at PO Box 78, Belconnen, ACT, 2616, or by e-mail to freqplan@acma.gov.au.  It would be appreciated if notification to ACMA of any inaccuracy or ambiguity found be made without delay in order that the matter may be investigated, and appropriate action taken.

# Table of Contents

1	Purpose	3

2	Future arrangements in this frequency band	3

3	Spectrum arrangements	3

3.1	Assignment Conditions	4

4	Relationship with RALI MS 40	4

5	RALI Authorisation	5

Appendix A: Allocation and Channelling Arrangements – effective 1 July 2020 to 30 June 2021	6

Appendix B: Allocation and Channelling Arrangements – effective from 1 July 2021	12

frequency plan for the 900 MHz band 890 - 960 MHz

# Purpose

The purpose of this Radiocommunications Assignment and Licensing Instruction (RALI) is to set out spectrum planning arrangements for radiocommunications services in the frequency band 890 to 960 MHz (known as the 900 MHz Band).  The RALI specifies the service allocations including the frequency limits of each allocation segment, and the channelling arrangements within these segments (where applicable).

This RALI, together with the related RALI MS 40 (for the 800 MHz band) replaces the Radiocommunications 900 MHz Band Plan 1992 which ceased on 1 October 2015 under “sunsetting” provisions of the Legislative Instruments Act 2003.

In making decisions, accredited frequency assigners and the ACMA’s officers should take all relevant factors into account and decide each case on its merits.  Issues relating to this document that appear to fall outside the enunciated policy should be referred to the Manager, Spectrum Planning Section, PO Box 78, Belconnen, ACT, 2616, or by e-mail to freqplan@acma.gov.au.

# Future arrangements in this frequency band

The ACMA commenced a review of arrangements in the 803–960 MHz frequency band in May 2011 (the Review). The scope of the Review included services operating within the band 820-960 MHz, as well as consideration of future use of the 803-820 MHz segment vacated as part of the digital dividend. In November 2015, the ACMA completed the Review and released its decision in the paper: ‘The ACMA’s long-term strategy for the 803-960 MHz band’ (the Decision paper). The Decision paper contains a range of decisions on reforms to the 803-960 MHz band, as well as a detailed plan for the implementation of these reforms (see Section 3.3 of the Decision paper).

This RALI provides incremental changes to the spectrum allocations in the 900 MHz band, effective at different points in time between 1 July 2020 and 1 July 2021 (contained in appendix A-B), to support the implementation of the new arraignments in the 900 MHz band. These incremental changes align with the Review implementation plan detailed in the Decision paper.  As such the spectrum allocations in Appendix B reflect the final arrangements in the band at the completion of the review implementation process.

# Spectrum arrangements

Spectrum allocation and channelling arrangements for the 900 MHz band are contained in the appendices to this RALI, and incrementally come into effect from the following dates:

- Appendix A – effective from 1 July 2020 until 30 June 2021

- Appendix B – effective from 1 July 2021

## Assignment Conditions

Services are to be assigned in accordance with this RALI.

Channelling arrangements other than those specified in this RALI may be authorised where such arrangements provide for more efficient use of the spectrum, when compared to the channelling arrangements specified for that segment in this RALI. In determining the spectrum efficiency of a service, without limiting the range of matters which may be taken into account, the following matters may be considered:

- Occupied bandwidth;

- Adjacent channel performance;

- The distance from the transmitter that the channel may be used again without causing harmful interference; and

- The impact that introduction of the service will have upon existing services

# Relationship with RALI MS 40

One outcome of the Review was to separate planning arrangements for the 803-960 MHz band into two distinct band plans – RALI MS 40 for the 803-890 MHz band and RALI MS 41 for the 890-960 MHz band. The reasons for creating two separate band plans are twofold:

- To provide the flexibility needed to make future revisions, in particular to implement the outcomes of the Review; and

- To reflect a general change in community and industry-accepted nomenclature on the frequency extremes of both bands.

Although the planning arrangements in the range 803-960 MHz are now split between the two new administrative band plans, the following services are currently covered by both plans:

- Two frequency fixed services in the range 852-857/928-933 MHz (until 30 June 2021); and

- Radiolocation services in the range 850-915 MHz.

# RALI Authorisation

Approved	30/06/2020

Chris Worley

Manager
Spectrum Planning Section
Spectrum Infrastructure Branch

Communications Infrastructure Division
Australian Communications and Media Authority

# Appendix A: Allocation and Channelling Arrangements – effective 1 July 2020 to 30 June 2021

The allocation and channelling arrangements for the 900 MHz band, effective from 1 July 2020 until 30 June 2021, are set out in the following figure and tables:

Figure 1: 900 MHz band services (890 - 960 MHz) diagram

Table 1: Service allocations and channelling arrangements for primary services

Table 2: Service allocations and channelling arrangements for secondary services

Figure 1: 900 MHz band (890 – 960 MHz) services diagram – effective 1 July 2020 to 30 June 2021

Table 1: Service allocations and channelling arrangements for primary services (note 1) – effective 1 July 2019 to 30 June 2021

| Item | Segment frequency limits (MHz) (lower limit exclusion, upper limit inclusive) | Service allocation | Maximum channel bandwidth (kHz) | Channel centre frequency formula | Range of integer values for variable ‘n’ (inclusive) | First channel/last channel centre frequency (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 890 – 915 | Considered for future spectrum licensing (see notes 2, 3 and 4) |  |  |  |  |
| 2 | 915 – 918 | Radiolocation (see note 5) |  |  |  |  |
| 3 | 918 – 926 | Radiolocation (see note 5) |  |  |  |  |
|  |  | Industrial, Scientific and Medical Applications |  |  |  |  |
| 4 | 926 – 928 | Radiolocation (see note 5) |  |  |  |  |
| 5 | 928 – 929 | Fixed point-to-point (two frequency) (see notes 6 and 7) | 25 | 927.9875 + n (0.025) | 1 to 40 | 928.0125 928.9875 |
| 6 | 929 – 929.5 |  | 12.5 | 928.99375 + n (0.0125) (see note 8) | 1 to 40 | 929.00625 929.49375 |
| 7 | 929.5 - 930 | Fixed point-to-multipoint (two frequency, base transmit) (see notes 6 and 7) | 12.5 | 929.49375 + n (0.0125) (see note 8) | 1 to 40 | 929.50625 929.99375 |
| 8 | 930 – 933 | Fixed point-to-point (two frequency) (see notes 7 and 9) | 200 | 930 + n (0.05) | 1 to 59 | 930.05 932.95 |
| 9 | 933 – 935 | Digital Short Range Radio Service (see note 7) |  |  |  |  |
| 10 | 935 – 960 | Considered for future spectrum licensing (see notes 2, 3 and 4) |  |  |  |  |

Notes:

1.	The allocations for these primary services are represented in Figure 1.

2.	No new assignments are to be made under this allocation, except where the assignment is for the expansion of an existing system or where one assignment is exchanged for another within the band and within the same area, contingent on minimal changes in overall spectrum use. Where an assignment is made on the basis that it is for the expansion of an existing system, or when one assignment is exchanged for another, the licensee shall acknowledge that:

- As the result of the 803–960 MHz band review allocations and systems in this segment are requirement to be spectrally relocated or for operations to cease in the future. Timeframes are available in the ACMA’s long-term strategy for the 803–960 MHz band.

- The assignment does not provide any additional protection or rights and that they will be subject to the same transition times as all other assignments.

3.	Existing apparatus licences within this segment are not to be renewed for a period exceeding one year.

4.        The band 890–915/935–960 MHz is undergoing reform and is considered for future spectrum licensing. Details of the consultation outcome can be found in ‘Reconfiguring the 900 MHz band’.

5.	Assignments to users other that the Department of Defence or the Australian Defence Force will not normally be authorised for this service.

6.	Services operated in this segment are paired with 852–854 MHz which is allocated under the 800 MHz Plan.

7.	No new assignments are to be made under this allocation. Stations operating under this allocation must cease operating in this segment by 30 June 2021. Existing apparatus licences under this service will not be extended beyond 30 June 2021. This allocation will conclude in this segment on 30 June 2021.

8.	Where a demonstrated need exists for 25 kHz operation, two contiguous 12.5 kHz channels may be assigned frequencies offset by 6.25 kHz between the channel centre frequencies given by this formula.

9.	Services operated in this segment are paired with 854-857 MHz which is allocated under the 800 MHz Plan.

Table 2: Service allocations and channelling arrangements for secondary services (note 1) – effective 1 July 2020 to 30 June 2021

| Item | Segment frequency limits (MHz) (lower limit exclusion, upper limit inclusive) | Service allocation |
| --- | --- | --- |
| 1 | 890 - 915 | Radiolocation (see note 2) |
| 2 | 928 – 942 | Radiolocation (see note 2) |

Notes:

1.	The allocations for these secondary services are represented in Figure 1.

2.         Assignments to users other than the Department of Defence or the Australian Defence Force will not normally be authorised for this service. In this segment the service is primary in offshore areas.

# Appendix B: Allocation and Channelling Arrangements – effective from 1 July 2021

The allocation and channelling arrangements for the 900 MHz band, effective from 1 July 2021, are set out in the following figure and tables:

Figure 2: 900 MHz band services (890 - 960 MHz) diagram

Table 3: Service allocations and channelling arrangements for primary services

Table 4: Service allocations and channelling arrangements for secondary services

Figure 2: 900 MHz band (890 – 960 MHz) services diagram – effective from 1 July 2021

Table 3: Service allocations and channelling arrangements for primary services (note 1) – effective 1 July 2021

| Item | Segment frequency limits (MHz) (lower limit exclusion, upper limit inclusive) | Service allocation | Maximum channel bandwidth (kHz) | Channel centre frequency formula | Range of integer values for variable ‘n’ (inclusive) | First channel/last channel centre frequency (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 890 – 915 | Planned for future spectrum licensing (see notes 2, 3 and 4) |  |  |  |  |
| 2 | 915 – 918 | Radiolocation (see note 5) |  |  |  |  |
| 3 | 918 – 926 | Radiolocation (see note 5) |  |  |  |  |
|  |  | Industrial, Scientific and Medical Applications |  |  |  |  |
| 4 | 926 – 928 | Radiolocation (see note 5) |  |  |  |  |
| 5 | 928 – 933 | Low power/duty-cycle fixed links/mesh nodes (see notes 6) |  |  |  |  |
| 6 | 933 – 935 | Low power/duty-cycle fixed links/mesh nodes (see notes 6 and 7) |  |  |  |  |
|  |  | Cognitive radio system research (see note 6) |  |  |  |  |
| 7 | 935 – 960 | Planned for future spectrum licensing (see notes 2, 3 and 4) |  |  |  |  |

Notes:

1.	The allocations for these primary services are represented in Figure 2.

2.	No new assignments are to be made under this allocation, except where the assignment is for the expansion of an existing system or where one assignment is exchanged for another within the band and within the same area, contingent on minimal changes in overall spectrum use. Where an assignment is made on the basis that it is for the expansion of an existing system, or when one assignment is exchanged for another, the licensee shall acknowledge that:

- As the result of the 803–960 MHz band review, allocations and systems in this segment are requirement to be spectrally relocated or for operations to cease in the future. Timeframes are available in the ACMA’s long-term strategy for the 803–960 MHz band.

- The assignment does not provide any additional protection or rights and they will be subject to the same transition times as all other assignments.

3.	Existing apparatus licences within this segment are not to be renewed for a period exceeding one year.

4.        The band 890–915/935–960 MHz has been identified for future spectrum licensing. Details can be found in the ACMA’s ‘Reconfiguring the 900 MHz band’ paper.

5.	Assignments to users other that the Department of Defence or the Australian Defence Force will not normally be authorised for this service.

6.	Stations operating in this allocation are authorised under the Radiocommunications (Low Interference Potential Devices) Class Licence 2015, as in force from time to time.

7.	This allocation is limited to low and remote density areas only (as defined in the ACMA’s apparatus licence fee density area maps).

Table 4: Service allocations and channelling arrangements for secondary services (note 1) – effective 1 July 2021

| Item | Segment frequency limits (MHz) (lower limit exclusion, upper limit inclusive) | Service allocation |
| --- | --- | --- |
| 1 | 890 - 915 | Radiolocation (see note 2) |
| 2 | 928 – 942 | Radiolocation (see note 2) |

Notes:

1.	The allocations for these secondary services are represented in Figure 2.

2.        Assignments to users other than the Department of Defence or the Australian Defence Force will not normally be authorised for this service. In this segment the service is primary in offshore areas.