Proposed update to the

Australian Radiofrequency Spectrum Plan

Consultation paper

August 2020

Canberra

Red Building 
Benjamin Offices
Chan Street 
Belconnen ACT

PO Box 78
Belconnen ACT 2616

T	+61 2 6219 5555
F	+61 2 6219 5353

Melbourne

Level 32 
Melbourne Central Tower
360 Elizabeth Street 
Melbourne VIC

PO Box 13112
Law Courts 
Melbourne VIC 8010

T	+61 3 9963 6800
F	+61 3 9963 6899

Sydney

Level 5 
The Bay Centre
65 Pirrama Road 
Pyrmont NSW

PO Box Q500
Queen Victoria Building 
NSW 1230

T	+61 2 9334 7700 or 1800 226 667
F	+61 2 9334 7799

Copyright notice

https://creativecommons.org/licenses/by/4.0/

With the exception of coats of arms, logos, emblems, images, other third-party material or devices protected by a trademark, this content is made available under the terms of the Creative Commons Attribution 4.0 International (CC BY 4.0) licence.

We request attribution as © Commonwealth of Australia (Australian Communications and Media Authority) 2020.

All other rights are reserved.

The Australian Communications and Media Authority has undertaken reasonable enquiries to identify material owned by third parties and secure permission for its reproduction. Permission may need to be obtained from third parties to re-use their material.

Written enquiries may be sent to:

Manager, Editorial Services
PO Box 13112
Law Courts
Melbourne VIC 8010
Email: info@acma.gov.au

Executive summary	1

Issues for comment	2

Introduction	3

The regulatory framework	4

International obligations—WRC-19 Final Acts	4

Spectrum review	4

Changes to the Spectrum Plan	6

Chapter 1—General information	6

Chapter 2—Australian Radiofrequency Spectrum Plan	6

Chapter 2—Part 2: Table of frequency band allocations	6

Chapter 2—Part 3: Australian footnotes	7

Chapter 2—Part 4: International footnotes	8

Invitation to comment	9

Making a submission	9

Executive summary

Following each International Telecommunication Union (ITU) World Radiocommunication Conference (WRC), the Australian Communications and Media Authority (the ACMA) typically updates the Australian Radiofrequency Spectrum Plan (the Spectrum Plan).

The Spectrum Plan, made under section 30 of the Radiocommunications Act 1992 (the Act), divides the spectrum into frequency bands and specifies the general purpose or purposes for which the bands may be used. The Spectrum Plan reflects both international and domestic arrangements for radiofrequency management.

The Spectrum Plan:

provides a basis for management of the radiofrequency spectrum in Australia

informs radiocommunication users about the services allocated to each frequency band and the conditions attached to those allocations

reflects Australia’s treaty obligations as a member of the ITU, as set out in the ITU’s Radio Regulations

provides details of international frequency allocations agreed by the ITU for the three world Regions.

The last major changes to the Spectrum Plan were made in January 2017, following the 2015 WRC (WRC-15). Proposed changes to the Spectrum Plan are outlined in this paper and the attached marked-up version of the draft Spectrum Plan. The proposed changes reflect changes to the ITU Radio Regulations at the WRC held in Sharm El Sheik, Egypt, in late 2019. They include new identifications for International Mobile Telecommunications (IMT) (mobile broadband), maritime and satellite services to reflect the decisions of WRC-19, as well as some minor editorial changes and clarification of a number of Australian footnotes.

It is proposed that the new Spectrum Plan discussed in this paper and its attachment be made before 1 January 2021, so as to commence on that date.

# Issues for comment

The ACMA welcomes comment from interested stakeholders on the issues raised in this consultation, or any other issues relevant to the update of the Australian Radiofrequency Spectrum Plan. In particular, the ACMA seeks comment on whether changes need to be made to particular Australian footnotes as described in the section Chapter 2—Part 3: Australian footnotes.

# Introduction

The Spectrum Plan specifies the general purpose or purposes for which frequency bands may be used. The Spectrum Plan provides a framework within which a diversity of radiocommunications services are able to operate.

The Spectrum Plan, as published by the ACMA, is divided into two chapters. The first chapter provides an introduction to the Spectrum Plan, guidance on its use and information relevant to the international framework from which the Spectrum Plan has been developed. This first chapter has been prepared to assist understanding of the Spectrum Plan and does not form part of the Spectrum Plan published on the Federal Register of Legislation. The second chapter contains the legal instrument, including the table of frequency allocations and associated footnotes.

The ACMA proposes to remake the Spectrum Plan to reflect changes to the ITU’s Radio Regulations made at the WRC held in Sharm El Sheik, Egypt in late 2019. Australia participated in this international treaty-level negotiation, agreeing in principle to changes that are mirrored in the Spectrum Plan in column 1 of the Table of Frequency Band Allocations (Part 2 of Chapter 2) and the international footnotes (Part 4 of Chapter 2). The ACMA typically updates the Spectrum Plan following each WRC.

In addition to reflecting updated international arrangements, the changes proposed to the Spectrum Plan:

include updates reflecting domestic changes that have occurred over the past four years

correct a small number of minor typographical errors.

# The regulatory framework

The Spectrum Plan is a legislative instrument made under section 30 of the Act.

A spectrum plan must:

divide the portion of the spectrum to be regulated into the number of frequency bands that the ACMA thinks necessary to effectively regulate that spectrum

designate one or more bands to be used primarily for the general purposes of defence

specify the general purpose or purposes for which each other band may be used.

## International obligations—WRC-19 Final Acts

The Spectrum Plan is usually updated every three to four years, following each ITU WRC. Australia is a member of the International Telecommunication Union (ITU) and has ratified the Constitution and Convention of the ITU. Under the constitution, member states are bound to comply with the ITU’s Radio Regulations. The WRC involves decisions that change the ITU’s Radio Regulations. These changes are recorded in ‘Final Acts’ from the conference. Australia agreed in principle to these changes by signing the Final Acts of the Conference.

The Spectrum Plan is updated to reflect changes agreed upon at the conference for parts of the international frequency allocation table that are referenced in the Spectrum Plan, as well as other international changes that Australia has agreed to in principle.

Domestic approval of the treaty amendments is currently being sought—that process is being managed by the Department of Infrastructure, Transport, Regional Development and Communications (DITRDC). Subject to DITRDC’s progress on this matter, the domestic process for consideration of the Final Acts may not occur by the end of 2020. The ACMA is working towards remaking the Spectrum Plan to coincide with the Final Acts coming into force.

## Spectrum review

In October 2019, the Australian Government committed to progressing a set of targeted amendments which will update and streamline the Act. These amendments will achieve this by removing existing unnecessary prescription and legislative barriers. This includes:

Clarifying the object of the Act, and the roles of the Minister and the Australian Communications and Media Authority (ACMA).

Streamlining the spectrum allocation and re-allocation processes.

Improving flexibility and reducing regulatory barriers between licence types.

Better reflecting modern spectrum needs and supply chains.

Introducing a modernised compliance and enforcement regime.

An exposure draft of a Bill to implement these reforms was released for public consultation. The exposure draft did not include any changes to Section 30 of the Act and therefore does not affect this proposed update of the Spectrum Plan.

# Changes to the Spectrum Plan

## Chapter 1—General information

Only minor editorial changes are proposed for Chapter 1—General information.

The ACMA seeks comment from interested parties on Chapter 1—General information (see Attachment A).

## Chapter 2—Australian Radiofrequency Spectrum Plan

### Chapter 2—Part 1: Introductory

The only changes proposed for Chapter 2—Part 1: Introductory are:

updating the name of the instrument

adding a commencement date

updating revocation provisions to reflect the change from 2017 to 2021.

Additionally, the definition of the land mobile-satellite service is removed as the allocations to this service in the table of allocations are also proposed for removal.

## Chapter 2—Part 2: Table of frequency band allocations

### Column 1: ITU Radio Regulations table of allocations

These changes, agreed to in principle by Australia at WRC-19 in November 2019, are now subject to consideration by Parliament as well as the Joint Standing Committee on Treaties (JSCOT) and the Federal Executive Council. Stakeholder consideration of the various issues reflected in the changes occurred prior to WRC-19, as part of developing Australian positions on WRC-19 Agenda items.

### Column 2: Australian table of allocations

The changes proposed reflect international changes affecting Australia and minor editorial changes. These include:

Identification of the frequency band 495–505 kHz for use by the international NAVDAT system—a digital file broadcasting system for ships at sea, for navigation, safety and security and a new primary allocation to the maritime mobile-satellite service in the band 1,621.35–1,626.5 MHz (as outcomes of WRC-19 Agenda item 1.8).

Removal of the primary allocation to the broadcasting service in 7,100–7,200 kHz and consequential update to Australian footnote AUS54. This allocation was suppressed internationally from 29 March 2009.

Arrangements in the 137–138 MHz band for downlink and the 148–149.9 MHz band for uplink for tracking, telemetry and control (TT&C) of short duration non-geostationary orbit (non-GSO) satellites in the space operation service (as an outcome of WRC-19 Agenda item 1.7).

Correcting an editorial error in the 137–138 MHz band where international footnote 208 was applied only to the mobile-satellite service whereas the footnote applies to the entire band.

A secondary allocation to the maritime mobile-satellite service in the frequency bands 157.1875–157.3375 MHz and 161.7875–161.9375 MHz for non-GSO satellite systems to enable a new VHF data exchange system (VDES) satellite component (as an outcome of WRC-19 Agenda item 1.9.2).

New footnotes to implement in-band power limits for earth stations operating in the frequency bands 401–403 MHz and 399.9–400.05 MHz (as an outcome of WRC-19 Agenda item 1.2).

Suppression of international footnotes 311A and 530D (as an outcome of WRC-19 Agenda item 4).

Removal of the secondary allocation to the land mobile-satellite (space to earth) service in the 12.2–12.75 GHz band. This allocation does not appear in the ITU Radio Regulations. Such use is facilitated by the fixed-satellite service allocations in this band in combination with subsection 10(9) of the ARSP.

Identification of the 17.7–19.7 GHz (space to earth) and 27.5-29.5 GHz (earth to space) bands for the operation of earth stations in motion communicating with geostationary fixed-satellite service space stations (as an outcome of WRC-19 Agenda item 1.5).

Identification of the 24.25–27.5 GHz, 37–43.5 GHz, 47.2–48.2 GHz and 
66–71 GHz bands for the terrestrial component of International Mobile Telecommunications (IMT) (as an outcome of WRC-19 Agenda item 1.13).

Identification of the 31–31.2 GHz and 38–39.5 GHz bands for use by high-altitude platform stations (HAPS) (as an outcome of WRC-19 Agenda item 1.14).

New footnotes to establish a regulatory framework for non-GSO FSS satellite systems that may operate in the frequency bands 37.5–39.5 GHz (space to earth), 39.5–42.5 GHz (space to earth), 47.2–50.2 GHz (earth to space) and 50.4–51.4 GHz (earth-to-space) (as an outcome of WRC-19 Agenda item 1.6).

A primary allocation to the fixed-satellite service in the 51.4-52.4 GHz band (as an outcome of WRC-19 Agenda item 9.1 issue 9.1.9).

## Chapter 2—Part 3: Australian footnotes

The following changes are proposed to the following Australian footnotes:

AUS98: the frequency bands listed in this footnote have been corrected to align with Radiocommunications Assignment and Licensing Instruction (RALI) MS22.

AUS53:

the removal of a reference to Appendix 13 of the Radio Regulations which was suppressed by WRC-07

the addition of the bands 18,900–19,800kHz and 25,210–26,175 kHz to the text of the footnote

the addition of the footnote to the 26,100–26,175 kHz band (noting the footnote already appears in the table of frequency band allocations for the 19,680–19,800kHz band).

The proposal reflects structural changes to the Radio Regulations that were not captured in previous updates to the Spectrum Plan and also expands the frequency ranges within the scope of the footnote to incorporate all applicable frequency segments under Appendix 17 of Radio Regulations.

We are also seeking comment on potential changes to the following Australian footnotes:

AUS98: the National Coordinating Committee for Government Radiocommunications (NCCGR) is named in this footnote for the purposes of consultation on the use of harmonised government frequency ranges.

- We seek comment on whether this is the most appropriate organisation to be referred to in this footnote, or whether there are more appropriate state-based points of contact for considering access to harmonised government frequencies.

## Chapter 2—Part 4: International footnotes

Proposed changes to Part 4 directly reflect those made at WRC-19.

The ACMA seeks comment from interested parties on Chapter 2 (see Attachment A).

# Invitation to comment

## Making a submission

The ACMA invites comments on the issues set out in this consultation paper.

Online submissions can be made by uploading a document. Submissions in PDF, Microsoft Word or Rich Text Format are preferred.

Submissions by post can be sent to:

The Manager

Spectrum Planning Section

Australian Communications and Media Authority

PO Box 78

Belconnen ACT 2616

The closing date for submissions is COB, Tuesday 29 September 2020.

Consultation enquiries can be emailed to freqplan@acma.gov.au.

#### Publication of submissions

The ACMA publishes submissions on our website, including personal information (such as names and contact details), except for information that you have claimed (and we have accepted) is confidential.

Confidential information will not be published or otherwise released unless required or authorised by law.

#### Privacy

View information about our policy on the publication of submissions, including collection of personal information during consultation and how we handle that information.

Information on the Privacy Act 1988, how to access or correct personal information, how to make a privacy complaint and how we will deal with the complaint, is available in our privacy policy.

The latest version of the Spectrum Plan is the Australian Radiofrequency Spectrum Plan 2017.

The term ‘used’ includes reserved for future use and reserved for the prevention or control of interference to radiocommunications.

Copies of the ITU-R Radio Regulations are available on the ITU-R website.