Spectrum Embargoes

Background	6

Embargoes	8

Annex A: Amendment History	113

Annex B: Index of bands	118

Annex C: Withdrawn Embargoes	122

Disclaimer

The Australian Communications and Media Authority (ACMA) advise that these instructions reflect the current policies of the ACMA.

Prospective applicants for licences should take all necessary steps to ensure that they have access to appropriate technical or other specialist advice independently of the ACMA concerning their applications, the operation of radiocommunications equipment and services, and any other matters relevant to the operation of transmitters and services under the licences in question.

The policies of the ACMA, and the laws of the Commonwealth may change from time to time, and prospective licensees should ensure that they have informed themselves of the current policies of the ACMA and of any relevant legislation (including subordinate instruments). Prospective applicants for licences should not rely on statements made in these instructions about the policies that may be followed by other government authorities or entities, nor about the effect of legislation. These instructions are not a substitute for independent advice (legal or otherwise) tailored to the circumstances of individual applicants.

Radiocommunications Assignment and Licensing Instructions are subject to periodic review and are amended as the ACMA considers necessary. To keep abreast of developments, it is important that users ensure that they are in possession of the latest edition.

No liability is or will be accepted by the Minister or Department of Communications and the Arts, the ACMA, the Commonwealth of Australia, or its officers, servants or agents for any loss suffered, whether arising directly or indirectly, due to reliance on the accuracy or contents of these instructions.

Suggestions for improvements to Radiocommunications Assignment and Licensing Instructions may be addressed to The Manager, Spectrum Planning Section, ACMA at PO Box 78, Belconnen, ACT, 2616, or by e-mail to freqplan@acma.gov.au. It would be appreciated if notification to ACMA of any inaccuracy or ambiguity found be made without delay in order that the matter may be investigated and appropriate action taken.

RALI AUTHORISATION

Approved		26 October 2020

Chris Worley

Manager

Spectrum Planning Section

Australian Communications and Media Authority

# Background

One of the objects of the Radiocommunications Act 1992 is the management of the radiofrequency spectrum in order to maximise, by ensuring the efficient allocation and use of the spectrum, the overall public benefit derived from using the radiofrequency spectrum.

In managing the spectrum, the ACMA uses a number of tools including the placement of embargoes on parts of the spectrum to support planning and other purposes. Embargoes place restrictions on frequency assignments for apparatus-licensed services in certain bands an in certain geographical areas. Embargoes, together with planning, are intended to ensure that the status of the band remains stable for the durations of the planning process. The overall public benefit from spectrum use can be maximised where such planning is not constrained by the introduction of unplanned services or by their premature introduction.

Embargoes are an effective and efficient administrative tool used in conjunction with other planning tools. Their application is part of a transparent decision-making process, and the application of an embargo is reviewable.

Information about the principles that the ACMA uses with making administrative decision can be found in the policy information paper Principles for Decision Making (available from the ACMA’s website at www.acma.gov.au/theACMA/principles-for-decision-making.

EXEMPTIONS

Exemptions may be given to an embargo where there is sufficient justification. All applications for frequency assignments in embargoed bands should be forwarded to the:

The Manager

Spectrum Planning Section, ACMA

PO Box 78, Belconnen, ACT, 2616

or by e-mail to freqplan@aca.gov.au

for consideration on a case-by-case basis.

REMARKS

This RALI replaces MS 03 dated 22 July 2020. Embargo numbers are not re-used to maintain historical reference. Each new embargo is authorised by the RALI Authorisation of the delegated officer. The authorisation is updated when the embargo is amended.

# Embargoes

EMBARGO 1

Status: Lifted

EMBARGO 2

Status: Lifted

EMBARGO 3

Status: Lifted

EMBARGO 4

Status: Lifted

EMBARGO 5

Status: Lifted

EMBARGO 6

Status: Lifted

EMBARGO 7

Status: Lifted

EMBARGO 8

Status: Lifted

EMBARGO 9

Status: Lifted

EMBARGO 10

Status: Lifted

EMBARGO 11

Status: Lifted

EMBARGO 12

Status: Replaced by Embargo 15

EMBARGO 13

Status: Lifted

EMBARGO 14

Status: Lifted

EMBARGO 15

Status: Lifted

EMBARGO 16

Status: Lifted

EMBARGO 17

Status: Lifted

EMBARGO 18

Status: Replaced by Embargo 26

EMBARGO 19

FREQUENCY RANGE(S):	406.11875–406.61875 MHz
408.11875–408.61875 MHz
415.56875–416.06875 MHz
417.56875–418.06875 MHz

SUBJECT:	Trunked land mobile radio service (TLMRS)—restriction on the assignment of certain channels in specific areas

DATE OF EFFECT:	September 1990 (last revised September 2007)

COVERAGE:	Any area outside a 100 km radius of Sydney, Melbourne, Brisbane, Adelaide, Perth and Canberra.

TIME FRAME:	Ongoing

INSTRUCTIONS

No assignments are to be made to the TLMRS on the following channels except with the approval of the Manager, Spectrum Engineering Section: channels 1–40 and 161–200 for Sydney, Melbourne, Brisbane, Adelaide, Perth, and channels 1–15 for Canberra (406.11875–406.30625 MHz and 415.56875–415.75625 MHz). Refer to RALI MS22 and RALI LM8 Annex B for more detail.

REASONS

Restrictions on the use of these channels will provide protection to 400 MHz wideband fixed service channels 2–2' and 3–3' (as detailed in RALI MS22), which overlap the TLMRS bands. As these wideband links are permitted only in areas beyond a 200 km radius of capital cities, there is effectively a 100 km "buffer zone" existing between the two areas.

This zone should ensure that mobile transmitters do not interfere with wideband links. It also allows for reasonable discretion to be used in considering cases where a proposed base station may be beyond the 100 km limit.

HISTORY

In September 2007 the embargo was revised to include Canberra in the list of excluded areas, in recognition of an increase in the use of the TLMRS and no growth in the use of wideband links in this area over the last several years. Provision for the protection of existing links remains. The frequency range previously covered by this embargo (403–420 MHz) was refined to be more specific. Some editorial and formatting changes were also made.

EMBARGO AUTHORISATION:

[signed]			27/09/07

Geoff McMillen
Manager
Spectrum Engineering Section
Spectrum Planning Branch
Australian Communications and Media Authority

EMBARGO 20

Status: Replaced by Embargo 23

EMBARGO 21

Status: Replaced by restrictions imposed by the 1.5 GHz Band Plan

EMBARGO 22

Status: Lifted

EMBARGO 23

FREQUENCY RANGE(S):	1980–2010 MHz
2010–2110 MHz
2170–2200 MHz
2200–2300 MHz

SUBJECT:	Embargo on new assignments to support television outside broadcast and mobile-satellite services

DATE OF EFFECT:	April 1996 (last revised September 2013)

COVERAGE:	See instructions below

TIME FRAME:	Ongoing

INSTRUCTIONS

- No new assignments are to be made Australia-wide in the 1980–2010 MHz and 2170–2200 MHz frequency bands except for those for television outside broadcasting services.

- No assignments of fixed or mobile services are to be made in the 2010–2110 MHz and 2200–2300 MHz frequency bands inside the areas detailed in Attachment 1. Application for television outside broadcast services will be considered on a case-by-case basis by the Manager Spectrum Engineering Section.

- Assignments for Earth stations (Earth receive and Fixed earth licence options) in the frequency range 2010–2110 MHz and 2200–2300 MHz will be considered on a case-by-case basis by the Manager Spectrum Engineering Section.

- No frequency assignments are to be made Australia-wide in the frequency ranges 1980–2010 MHz and 2170–2200 MHz for Ancillary Terrestrial Component (ATC) and Complementary Ground Component (CGC) services where they support or form part of a mobile-satellite service.

REASONS

Instruction 1 & 2:  This serves to facilitate the introduction of television outside broadcast services in accordance with the Television Outside Broadcast Services (1980–2110 MHz and 2170–2300 MHz) Frequency Band Plan 2012. It also serves to preserve planning options for 1980–2010 MHz and 2170–2200 MHz while the future use of the band is under review.

Instruction 3:  This provision supports potential planning activity associated with current public consultation on the siting of Earth stations particularly in relation to those within propagation distance of areas of high density radiocommunications.

Instructions 4:  This serves to limit the further use of the 1980–2010 and 2170–2200 MHz bands by Ancillary Terrestrial Component (ATC) and Complementary Ground Component (CGC) services while the ACMA considers its position on international developments for use of these bands. Note while this restriction is covered by instruction 1, it is maintained to provide clarity.

HISTORY

Embargo 23 was originally issued in January 1993. The embargo has been revised several times between April 1994 and June 2002 and has changed significantly from its original form. More recent changes of relevance include the following:

In August 2005 the embargo was extended to include fixed and mobile assignments in the 2025–2110 and 2200–2300 MHz bands.

In October 2005 the embargo was removed from remote density areas in the 2025–2110 and 2200–2300 MHz bands.

In September 2007 the embargo was revised to remove the reference to exemptions for the Melbourne 2006 Commonwealth Games, and to include some editorial and formatting changes.

In November 2010, the embargo was extended to include Ancillary Terrestrial Component (ATC) and Complementary Ground Component (CGC) services in the 1980–2010 and 2170–2200 MHz ranges. These services provide terrestrially based supplementation of services provided by stations in the mobile-satellite service. While there was international consideration of the approval for these services, this embargo provided the ACMA the ability to consider its position.

In April 2012, the embargo was revised in included the 2010–2025 MHz frequency band.  That band was previously included in embargo 38 which has now been revoked.  The embargo was also extend to facilitate the introduction of television outside broadcast services in the 1980–2110 MHz and 2170–2300 MHz frequency bands.

In May 2012, the embargo was revised to remove the area around Darwin for the 2200–2300 MHz frequency band that had been included by error.

In September 2013, instruction 1 of the embargo was revised to remove the restriction on television outside broadcast services in the bands 1980–2010 MHz and 2170–2200 MHz. The embargo on all other services is to preserve planning options while the future use of the band is under review as part of considerations future spectrum requirements for mobile broadband.

In September 2013, instruction 2 was revised to allow application for television outside broadcast services to be considered on case-by-case basis by the Manager Spectrum Engineering Section pending finalisation of coordination arrangements for television outside broadcast services in the bands 2010–2110 MHz and 2200–2300 MHz

EMBARGO AUTHORISATION:

[signed]			16/09/2013

Mark Arkell
Manager
Spectrum Engineering Section
Spectrum Planning and Engineering Branch 
Australian Communications and Media Authority

Attachment 1:

- No assignments are to be made for fixed or mobile services in the 2010–2110 MHz frequency band in the geographic area described by a circle with a radius of 210 kilometres whose centre is located at a point specified in Table 1.

- No assignments are to be made for fixed or mobile services in the 2200–2300 MHz frequency band in the geographic area described by a circle with a radius of 210 kilometres whose centre is located at a point specified in Table 2.

- No assignments are to be made for fixed or mobile services in the 2010–2110 MHz and 2200–2300 MHz frequency bands inside or within 60 kilometres of the geographic area whose boundary is described by the coordinates specified in Table 3.

The datum used for all geographic coordinates in this attachment is the Geocentric Datum of Australia 1994.

Table 1

| ° South | ° East |
| --- | --- |
| 31.95075 | 115.87204 |
| 31.953254 | 115.855373 |
| 32.012419 | 116.061762 |
| 32.008252 | 116.083985 |
| 31.878253 | 115.859817 |
| 32.057978 | 115.751210 |
| 12.463580 | 130.835066 |
| 12.464135 | 130.844233 |
| 12.448302 | 130.836455 |

Table 2

| ° South | ° East |
| --- | --- |
| 31.95075 | 115.87204 |
| 31.953254 | 115.855373 |
| 32.012419 | 116.061762 |
| 32.008252 | 116.083985 |
| 31.878253 | 115.859817 |
| 32.057978 | 115.751210 |

Table 3

| ° South | ° East |
| --- | --- |
| 31.998556 | 136.001359 |
| 31.998546 | 137.001345 |
| 31.998537 | 138.001335 |
| 31.998521 | 139.001320 |
| 31.998513 | 140.001305 |
| 31.998499 | 141.001291 |
| 32.998503 | 141.001301 |
| 32.998492 | 142.001289 |
| 32.998484 | 143.001274 |
| 33.998489 | 143.001286 |
| 33.998479 | 144.001273 |
| 33.998470 | 145.001258 |
| 33.998459 | 146.001242 |
| 33.998450 | 147.001227 |
| 32.998449 | 147.001211 |
| 32.998441 | 148.001196 |
| 31.998441 | 148.001188 |
| 30.998441 | 148.001176 |
| 30.998435 | 149.001159 |
| 29.998435 | 149.001149 |
| 28.998437 | 149.001143 |
| 28.998431 | 150.001125 |
| 27.998425 | 150.001112 |
| 26.998429 | 150.001100 |
| 25.998434 | 150.001089 |
| 24.998427 | 150.001086 |
| 24.998434 | 149.001105 |
| 23.998440 | 149.001093 |
| 23.998448 | 148.001103 |
| 22.998453 | 148.001095 |
| 21.998461 | 148.001086 |
| 20.998461 | 148.001080 |
| 20.998461 | 147.001099 |
| 20.998467 | 146.001113 |
| 19.998468 | 146.001105 |
| 18.998468 | 146.001098 |
| 18.998472 | 145.001108 |
| 17.998480 | 145.001094 |
| 16.998479 | 145.001085 |
| 15.998479 | 145.001090 |
| 15.998478 | 146.001078 |
| 15.998474 | 147.001067 |
| 16.998469 | 147.001072 |
| 17.998465 | 147.001078 |
| 18.998465 | 147.001089 |
| 18.998456 | 148.001071 |
| 18.998451 | 149.001058 |
| 19.998451 | 149.001064 |
| 19.998441 | 150.001050 |
| 20.998438 | 150.001056 |
| 20.998432 | 151.001042 |
| 21.998429 | 151.001049 |
| 22.998434 | 151.001058 |
| 22.998420 | 152.001041 |
| 23.998428 | 152.001046 |
| 23.998411 | 153.001033 |
| 23.998405 | 154.001018 |
| 24.998402 | 154.001025 |
| 25.998401 | 154.001033 |
| 26.998397 | 154.001041 |
| 27.998398 | 154.001049 |
| 28.998397 | 154.001059 |
| 29.998395 | 154.001068 |
| 30.998395 | 154.001078 |
| 31.998395 | 154.001088 |
| 31.998405 | 153.001103 |
| 32.998404 | 153.001116 |
| 32.998415 | 152.001132 |
| 33.998414 | 152.001145 |
| 34.998416 | 152.001158 |
| 34.998426 | 151.001172 |
| 35.998427 | 151.001188 |
| 36.998431 | 151.001203 |
| 37.998434 | 151.001218 |
| 37.998444 | 150.001236 |
| 37.998457 | 149.001255 |
| 38.998459 | 149.001268 |
| 39.998464 | 149.001286 |
| 40.998469 | 149.001304 |
| 41.998475 | 149.001323 |
| 42.998481 | 149.001343 |
| 43.998488 | 149.001364 |
| 43.998499 | 148.001382 |
| 43.998511 | 147.001401 |
| 43.998522 | 146.001418 |
| 43.998534 | 145.001436 |
| 42.998527 | 145.001413 |
| 41.998522 | 145.001384 |
| 41.998531 | 144.001408 |
| 40.998524 | 144.001387 |
| 40.998536 | 143.001403 |
| 39.998529 | 143.001383 |
| 38.998522 | 143.001358 |
| 38.998534 | 142.001379 |
| 38.998546 | 141.001393 |
| 38.998557 | 140.001407 |
| 37.998545 | 140.001384 |
| 37.998562 | 139.001401 |
| 36.998554 | 139.001381 |
| 36.998567 | 138.001396 |
| 36.998578 | 137.001408 |
| 36.998590 | 136.001420 |
| 35.998576 | 136.001402 |
| 34.998562 | 136.001392 |
| 33.998560 | 136.001384 |
| 32.998557 | 136.001369 |
| 31.998556 | 136.001359 |

EMBARGO 24

Status: Lifted

EMBARGO 25

Status: Lifted

EMBARGO 26

FREQUENCY RANGE(S):

1.	703–748 MHz

2.	758–803 MHz

3.	825–845 MHz

4.	870–890 MHz

5.	1710–1785 MHz

6.	1805–1880 MHz

7.	1900–1980 MHz

8.	2110–2170 MHz

9.	2302–2400 MHz

10.	2500–2570 MHz

11.	2570–2620 MHz

12.	2620–2690 MHz

13.	3425–3492.5 MHz

14.	3542.5–3575 MHz

15.	25.1-27.5 GHz

SUBJECT:		Restrictions on assignments for apparatus-licensed services due to spectrum licensing provisions

DATE OF EFFECT:		May 1996 (last revised March 2020)

TIME FRAME:	Ongoing

INSTRUCTIONS

No apparatus licences are to be issued in frequency bands and areas subject to spectrum licensing provisions as detailed in the following section.

REASONS

1,2.	On 1 November 2011 the Minister for Broadband, Communications and the Digital Economy declared that in parts of Australia the bands 703–748 MHz and 758–803 MHz are subject to re-allocations by issuing spectrum licences. For details of the 703–748 MHz and 758–803 MHz bands and areas see the Radiocommunication (Spectrum Re-allocation) Declaration No. 1 of 2011

3,4.	On 15 July 1997 the Minister for Communications and the Arts declared that:

- throughout Australia, the bands 825–830 MHz, 835–845 MHz, 870–875 MHz and 880–890 MHz; and

- in parts of Australia, the bands 830–835 MHz and 875–880 MHz,

are subject to re-allocation by issuing spectrum licences. For details of these bands and areas see the Spectrum Re-allocation Declaration No. 1 of 1997 and Spectrum Re-allocation Declaration No. 2 of 1997.

On 7 October 2000 the Minister for Communications, Information Technology and the Arts declared that further parts of Australia in the bands 830–835 MHz and 875–880 MHz are subject to re-allocation. For details of these bands and areas see the Radiocommunications (Spectrum Re-allocation) Declaration No. 3 of 2000.

5,6.		On 15 July 1997 the Minister for Communications and the Arts declared that in parts of Australia the bands 1710–1755 MHz and 1805–1850 MHz are subject to re-allocation by issuing spectrum licences. For details of the 1710–1755 MHz and 1805–1850 MHz bands and areas see the Spectrum Re-allocation Declaration No. 3 of 1997 and the Spectrum Re-allocation Declaration No. 4 of 1997.

On 27 September 1999 the Minister for Communications, Information Technology and the Arts declared that in parts of Australia the bands 1755–1785 MHz and 1850–1880 MHz are subject to re-allocation by issuing spectrum licences. For details of the 1755–1785 MHz and 1850–1880 MHz bands and areas see the Spectrum Re-allocation Declaration 1999 (No. 2).

On 26 May 2015 the Minister for Communications declared that in regional Australia the bands 1725–1785 MHz and 1820–1880 MHz should be re-allocated by issuing spectrum licences under Subdivision B of Division 1 of Part 3.2 of the Act, for details see Spectrum Re-allocation Declaration 2015.

7,8.	On 7 October 2000 the Minister for Communications, Information Technology and the Arts declared that in parts of Australia the bands 1900–1980 MHz and 2110–2170 MHz are subject to re-allocation by issuing spectrum licences. For details of the 1900–1980 MHz and 2110–2170 MHz bands and areas see the Radiocommunications (Spectrum Re-allocation) Declaration No. 2 of 2000.

9.	On 14 January 2000 the Minister for Communications, Information Technology and the Arts designated the band 2302–2400 MHz throughout Australia as part of the spectrum to be allocated by issuing spectrum licences. For details see the Radiocommunications (Spectrum Designation) Notice No. 1 of 2000.

10,12.	On 1 November 2011 the Minister for Broadband, Communication and the Digital Economy declared that in parts of Australia the bands 2500–2570 MHz and 2620–2690 MHz are subject to re-allocation by issuing spectrum licences. For details of the 2500–2570 MHz and 2620–2690 MHz bands and areas see the Radiocommunication (Spectrum Re-allocation) Declaration No.2 of 2011.

11.	On 24 February 2012 the Minister for Broadband, Communication and the Digital Economy designated the frequency band between 2570 MHz and 2620 MHz in parts of Australia to be allocated by issuing spectrum licences. For details see the Radiocommunications (Spectrum Designation) Notice No.1 of 2012.

13,14.	On 11 April 2000 the Minister for Communications, Information Technology and the Arts declared that the part of the spectrum from 3425–3492.5 MHz and 3542.5–3575 MHz is subject to re-allocation by issuing spectrum licences. For details see the Radiocommunications (Spectrum Re-allocation) Declaration 2000.

15.	On 18 October 2019 the Minister for Communications, Cyber Safety and the Arts declared that 25.1-27.5 GHz is subject to re-allocation in specific areas by issuing spectrum licences. For details see the Radiocommunications (Spectrum Re-allocation—26 GHz Band) Declaration 2019.

HISTORY

The embargo was initially issued in August 1997 to provide a consolidating reference to those bands subject to spectrum licensing provisions at that time.

The May 1998 revision included the bands 27.5–28.35 GHz and 31–31.3 GHz.

The October 1999 revision included the bands 3425–3492.5 MHz and 3542.5–3575 MHz.

The December 1999 revision included the bands 1755–1785 MHz and 1850–1880 MHz.

The May 2000 revision included the band 2302–2400 MHz.

The July 2000 revision included the band 26.5–27.5 GHz, which is now also subject to spectrum licensing provisions.

The November 2000 revision included a change to the areas covered by the bands 830–835 MHz and 875–880 MHz.  It also included the bands 1900–1980 MHz and 2110–2170 MHz.

The August 2005 revision included the band 2010–2025 MHz.

In September 2007 the embargo was revised to include minor editorial and formatting changes.

The April 2012 revision included the bands 703–748 MHz, 758–803 MHz, 2500–2570 MHz, 2750–2620 MHz and 2620–2690 MHz.  The frequency range 2500–2690 MHz was previously included in embargo 43 which has now been revoked.  The frequency range 2010–2025 MHz was excluded as the relevant declaration (Radiocommunications (Spectrum Re-allocation) Declaration No. 1 of 2005) was revoked on 1 January 2007 by virtue of section 153K of the Radiocommunications Act because no licences were issued before the re-allocation deadline.

The September 2012 revision removed the bands 500.99375–504.99375 MHz and 510.99375–514.99375 MHz because the Radiocommunications (Spectrum Designation) Notice No. 1 of 1996 was revoked by the Radiocommunications (Spectrum Designation) Notice No. 1 of 1996 Instrument of Revocation No. 1 of 2010 on 1 June 2012.

The January 2014 revision removed the band 27.5–28.35 GHz because the Radiocommunications (Spectrum Designation) Notice No. 1 of 1998 was revoked by the Radiocommunications (Spectrum Designation) Notice No. 1 of 1998 Instrument of Revocation No. 1 of 2013 on 31 January 2014. The band 31–31.3 GHz was also removed as existing spectrum licences expired and were not re-issued.

February 2016 revision removed the band 26.5–27.5 GHz because the Radiocommunications (Unacceptable Levels of Interference - 27 GHz Band) Determination 2000 was revoked by the Radiocommunications (Spectrum Designation—27 GHz Band) Repeal Notice 2015 on 5 November 2015.

Made reference to Spectrum re-allocation declaration of 2015 under Reason 5,6.

The March 2020 revision included the band 25.1 – 27.5 GHz.

EMBARGO AUTHORISATION

[signed]			27/03/2020

Chris Worley

Manager

Spectrum Planning Section

Spectrum Planning and Engineering Branch

Australian Communications and Media Authority

EMBARGO 27

Status: Replaced by Embargo 26

EMBARGO 28

Status: Lifted

EMBARGO 29

Status: Lifted

EMBARGO 30

Status: Lifted

EMBARGO 31

Status: Replaced by provisions in the Australian Radiofrequency Spectrum Plan

EMBARGO 32

FREQUENCY RANGE(S):	168–174 MHz

SUBJECT:	Embargo on new assignments to support the introduction of digital terrestrial television broadcasting

DATE OF EFFECT:	September 2000 (last revised September 2007)

COVERAGE:	Around the geographic locations specified in Attachment A

TIME FRAME:	Until further notice

INSTRUCTIONS

No assignments for new fixed or mobile services are to be made in the frequency band 168–174 MHz within the limits specified in Table A around the sites listed in Table B of Attachment A.

REASONS

Operation of fixed or mobile services in this band in the vicinity of the television broadcasting sites listed in Table B of Attachment A may be affected by the introduction of digital terrestrial television broadcasting (DTTB).

COMMENTS

The VHF land mobile spectrum adjacent to TV channel 6 may be affected by out-of-band emissions from DTTB services on TV channel 6. As planning of DTTB services Australia-wide is not complete, the list of sites has been compiled from completed digital channel plans and a list of existing analog TV channel 7 sites which could potentially have channel 6 DTTB services associated with them.

Table B will be revised as DTTB planning progresses.

HISTORY

In December 2001 the embargo was revised to include an update of the frequency bands and exclusion distances in Table A based on new information on combiner filtering of the DTTB transmitter out-of-band emissions. The list of sites in Table B was also updated to include information available in digital channel plans completed since the initial release of this embargo.

The March 2002, July 2002, May 2003 and August 2003 revisions also included updates to Table B based on digital channel plans completed since the previous update of this embargo.

In September 2007 the embargo was revised to update Table B based on information available in digital channel plans completed since the previous release of this embargo, and has limited the inclusion of sites to those where channel 7 analog television services currently operate with ERPs of 500 W or more. Some editorial and formatting changes were also made.

EMBARGO AUTHORISATION:

[signed]			27/09/2012

Geoff McMillen
Manager
Spectrum Engineering Section
Spectrum Planning Branch
Australian Communications and Media Authority

Attachment A:

No assignments for new fixed or mobile services should be made in the frequency band 168–174 MHz within the frequency/distance limits specified in Table A of the sites listed in Table B.

Table A

| FREQUENCY BAND | EXCLUSION DISTANCE FROM SITE IN TABLE B |
| --- | --- |
| 168.0–172.8 MHz 172.8–173.3 MHz 173.3–174.0 MHz | 3 km 10 km 60 km |

Table B

| SITE NAME | GEOGRAPHIC COORDINATES |  |
| --- | --- | --- |
|  | LATITUDE (dms) | LONGITUDE (dms) |
| ACT Canberra – Black Mountain NSW Sydney – Artarmon Sydney – Willoughby Murrumbidgee Irrigation Area – Mt Bingar Bourke – Mt Oxley VIC Melbourne – Mt Dandenong – HSV7 site Melbourne – Mt Dandenong – ATV10 site Western Victoria – Mt Dundas QLD Brisbane – Mt Cootha – BTQ7 site Brisbane – Mt Cootha – Q10 site Cairns – Mt Bellenden Ker Roma – Timbury Hills Jericho – Colorado Weipa SA Adelaide – Mt Lofty – 7 / 10 site Adelaide – Mt Lofty – 9 site Ceduna WA Perth – Bickley Perth – Carmel Port Hedland Katanning – Fairfield Carnarvon NT Katherine Alice Springs – West Gap Tennant Creek | -35 16 38 -33 48 25 -33 48 48 -34 07 23 -30 12 03 -37 50 13 -37 50 20 -37 27 37 -27 28 05 -27 27 53 -17 15 57 -26 34 27 -23 37 35 -12 37 30 -34 58 57 -34 59 02 -32 09 14 -32 00 34 -32 00 50 -20 22 15 -33 47 34 -24 54 25 -14 28 25 -23 43 26 -19 38 13 | 149 05 48 151 10 49 151 11 41 146 14 02 146 14 22 145 20 47 145 20 43 141 54 53 152 56 32 152 56 49 145 51 09 148 50 56 146 15 18 141 53 00 138 42 24 138 42 25 133 45 14 116 04 58 116 03 37 118 33 42 117 30 58 113 43 07 132 16 40 133 51 19 134 13 48 |

EMBARGO 33

Status: Replaced by provisions in the Australian Radiofrequency Spectrum Plan

EMBARGO 34

Status: Superseded by Embargo 64

EMBARGO 35

Status: Superseded by Embargo 50

EMBARGO 36

Status: Superseded by Embargo 50

EMBARGO 37

Status: Replaced by Embargo 23

EMBARGO 38

Status: Lifted

EMBARGO 39

FREQUENCY RANGE(S):	5725–5850 MHz

SUBJECT:	Embargo on fixed service assignments to provide limited support to 5.8 GHz fixed links and to protect radiolocation receivers

DATE OF EFFECT:	25 November 2004 (last revised September 2007)

COVERAGE:	As specified in Tables A and B of Attachment 1

TIME FRAME:	Ongoing

INSTRUCTIONS

No fixed service assignments are to be made in the frequency ranges/geographic areas specified in Table A and Table B of Attachment 1.

REASONS

This embargo has been introduced to:

- preserve opportunities for devices authorised under class licensing arrangements in the subject band (in particular in larger population areas defined in Table B1 of Attachment 1); and

- provide protection for radiolocation receivers used for defence purposes within areas defined in Table B2 of Attachment 1.

COMMENTS

The 5725–5850 MHz band is used, or is available for use, by:

- the radiolocation service (under a primary allocation that provides that the use is principally for the purposes of defence);

- the amateur and amateur-satellite (space-to-Earth) services (under secondary allocations);

- a wide variety of short range devices under class licensing arrangements; and

- industrial, scientific and medical (ISM) applications which use radiofrequency energy locally for non-radiocommunication purposes.

HISTORY

In September 2007 the embargo was revised to include editorial and formatting changes.

EMBARGO AUTHORISATION:

[signed]			27/09/2007

Geoff McMillen
Manager
Spectrum Engineering Section
Spectrum Planning Branch
Australian Communications and Media Authority

Attachment 1

1. No fixed service assignments should be made in the frequency ranges specified in Table A.

Table A

| FREQUENCY RANGE | EXCLUSION AREA |
| --- | --- |
| 5725–5735 MHz 5755–5775 MHz 5795–5850 MHz | Australia-wide Australia-wide Australia-wide |

2. In the frequency ranges specified in Table B, no fixed service assignments should be made in the geographic areas specified in Table B.

Table B

| FREQUENCY RANGE | EXCLUSION ZONES (to be applied simultaneously) |
| --- | --- |
| 5735–5755 MHz 5775–5795 MHz | High density areas and medium density areas as defined in Schedule 1 of the Radiocommunications (Transmitter Licence Tax) Determination 2003 (No.2). Within the circles defined by the centroid coordinates and radii listed in Table B1. Within the areas enclosed by polygons defined by coordinates listed in Table B2. |

Table B1

| State/ territory | Site Name | Coordinates of centroid |  | Radius (km) |
| --- | --- | --- | --- | --- |
|  |  | Latitude (dec degrees South ) | Longitude (dec degrees East) |  |
| ACT/NSW | Canberra/Queanbeyan (Capital Hill) | 35.310000 | 149.120000 | 25 |
| NSW | Armidale | 30.516667 | 151.666667 | 20 |
| NSW | Bathurst | 33.416667 | 149.583333 | 20 |
| NSW | Coffs Harbour | 30.300000 | 153.133333 | 20 |
| NSW | Dubbo | 32.250000 | 148.616667 | 20 |
| NSW | Goulburn | 34.750000 | 149.716667 | 20 |
| NSW | Lismore | 28.816667 | 153.266667 | 20 |
| NSW | Nowra | 34.883333 | 150.600000 | 20 |
| NSW | Orange | 33.283333 | 149.100000 | 20 |
| NSW | Port Macquarie | 31.450000 | 152.916667 | 20 |
| NSW | Tamworth | 31.083333 | 150.933333 | 20 |
| NSW | Wagga Wagga | 35.116667 | 147.366667 | 20 |
| NSW/VIC | Albury/Wodonga (Wodonga town centre) | 36.116667 | 146.883333 | 20 |
| VIC | Ballarat | 37.566667 | 143.850000 | 20 |
| VIC | Bendigo | 36.766667 | 144.283333 | 20 |
| VIC | Mildura | 34.183333 | 142.166667 | 20 |
| VIC | Shepparton/Mooroopna | 36.383333 | 145.400000 | 20 |
| VIC | Warrnambool | 38.383333 | 142.483333 | 20 |
| QLD | Bundaberg | 24.866667 | 152.350000 | 20 |
| QLD | Cairns | 16.916667 | 145.766667 | 20 |
| QLD | Caloundra/Kawana Waters | 26.800000 | 153.133333 | 20 |
| QLD | Gladstone | 23.850000 | 151.266667 | 20 |
| QLD | Hervey Bay | 25.290000 | 152.850000 | 20 |
| QLD | Mackay | 21.150000 | 149.183333 | 20 |
| QLD | Maroochydore/Mooloolaba/Buderim | 26.650000 | 153.100000 | 20 |
| QLD | Maryborough | 25.533333 | 152.700000 | 20 |
| QLD | Mt Isa | 20.733333 | 139.483333 | 20 |
| QLD | Rockhampton | 23.366667 | 150.533333 | 20 |
| QLD | Tewantin-Noosa | 26.400000 | 153.066667 | 20 |
| QLD | Toowoomba | 27.566667 | 151.950000 | 20 |
| QLD | Townsville/Thuringowa (centre of Townsville urban area) | 19.260000 | 146.810000 | 20 |
| SA | Mt Gambier | 37.833333 | 140.783333 | 20 |
| SA | Whyalla | 33.033333 | 137.600000 | 20 |
| WA | Albany | 35.000000 | 117.866667 | 20 |
| WA | Bunbury | 33.333333 | 115.633333 | 20 |
| WA | Geraldton | 28.766667 | 114.616667 | 20 |
| WA | Mandurah | 32.533333 | 115.716667 | 20 |
| WA | Kalgoorlie/Boulder | 30.750000 | 121.466667 | 20 |
| TAS | Devonport | 41.183333 | 146.350000 | 20 |
| TAS | Hobart—New Town | 42.860000 | 147.300000 | 25 |
| TAS | Launceston | 41.450000 | 147.166667 | 20 |
| NT | Alice Springs | 23.700000 | 133.866667 | 20 |
| NT | Darwin-Palmerston (Palmerston town centre) | 12.482222 | 130.982778 | 25 |

Table B2

| Shoalwater Bay-QLD |  |
| --- | --- |
| Latitude | Longitude |
| (south) | (east) |
| 22.767211 | 150.765770 |
| 22.758374 | 150.767852 |
| 22.744160 | 150.720385 |
| 22.757631 | 150.715294 |
| 22.743752 | 150.641557 |
| 22.750821 | 150.620010 |
| 22.759017 | 150.624735 |
| 22.850781 | 150.613412 |
| 22.850338 | 150.584197 |
| 22.897435 | 150.594082 |
| 22.884900 | 150.474451 |
| 22.844289 | 150.475202 |
| 22.818367 | 150.334494 |
| 22.832383 | 150.309875 |
| 22.849077 | 150.220907 |
| 22.807859 | 150.195470 |
| 22.765399 | 150.236231 |
| 22.754388 | 150.234519 |
| 22.752759 | 150.143073 |
| 22.730292 | 150.147919 |
| 22.709445 | 150.143000 |
| 22.634488 | 150.090591 |
| 22.457819 | 150.094766 |
| 22.252500 | 150.011111 |
| 22.100000 | 150.500000 |
| 22.100000 | 150.750000 |
| 22.801876 | 150.983349 |

| ## Zone 1-NT |  |
| --- | --- |
| Latitude | Longitude |
| (south) | (east) |
| 12.004700 | 130.914700 |
| 9.912200 | 130.868600 |
| 10.403800 | 132.419700 |
| 11.683600 | 133.344400 |
| 12.314700 | 131.317500 |
| 12.086900 | 131.159700 |

| ## Woomera-SA |  |
| --- | --- |
| Latitude | Longitude |
| (south) | (east) |
| 27.833333 | 133.833333 |
| 28.566667 | 133.833333 |
| 29.116667 | 134.366667 |
| 29.116667 | 134.950000 |
| 28.716667 | 135.166667 |
| 28.716667 | 135.750000 |
| 29.300000 | 136.600000 |
| 30.325556 | 136.983056 |
| 30.533333 | 137.283333 |
| 30.678333 | 137.333889 |
| 30.866667 | 137.400000 |
| 31.197222 | 137.357222 |
| 31.581389 | 137.009722 |
| 31.387778 | 136.352222 |
| 31.111667 | 136.214167 |
| 30.561667 | 133.921944 |
| 30.430278 | 132.260000 |
| 30.083333 | 132.000000 |
| 29.900000 | 131.500000 |
| 28.133333 | 131.500000 |
| 28.133333 | 132.000000 |
| 27.833333 | 132.000000 |

| ## Mt Bundy-NT |  |
| --- | --- |
| Latitude | Longitude |
| (south) | (east) |
| 12.823300 | 131.968800 |
| 12.920200 | 131.968800 |
| 12.920200 | 132.070000 |
| 13.173000 | 132.072200 |
| 13.173000 | 131.875000 |
| 12.918600 | 131.685000 |
| 12.918000 | 131.783600 |
| 12.857700 | 131.783600 |

| Zone 2-NT |  |
| --- | --- |
| Latitude | Longitude |
| (south) | (east) |
| 14.000000 | 131.000000 |
| 14.000000 | 131.923000 |
| 14.171900 | 132.007500 |
| 14.780800 | 131.936400 |
| 15.023000 | 132.376600 |
| 16.194700 | 132.376600 |
| 16.230962 | 132.212901 |
| 16.245936 | 131.810228 |
| 16.046290 | 131.235872 |
| 15.350000 | 131.000000 |
| 15.350000 | 130.801410 |
| 15.537610 | 130.801410 |
| 15.556580 | 130.746620 |
| 15.625150 | 130.685830 |
| 15.591170 | 130.497560 |
| 15.629770 | 130.428150 |
| 15.559850 | 130.363980 |
| 15.472980 | 130.365055 |
| 15.418750 | 130.003790 |
| 15.457090 | 129.943370 |
| 15.431370 | 129.848300 |
| 15.177530 | 129.768400 |
| 14.830000 | 129.461600 |
| 11.500500 | 128.171600 |
| 12.289100 | 130.503000 |
| 12.822222 | 130.777500 |
| 14.784428 | 130.027045 |
| 14.789580 | 130.175340 |
| 14.893410 | 130.266630 |
| 14.825000 | 130.633300 |
| 14.929100 | 130.741600 |
| 14.929100 | 131.000000 |

EMBARGO 40

Status: Lifted

EMBARGO 41

FREQUENCY RANGE(S):	70 MHz–25.25 GHz

SUBJECT:	Embargo for a Radio Quiet Zone

DATE OF EFFECT:	11 April 2005 (last revised and 19 December 2014)

COVERAGE:	For the frequency range 70 MHz–25.25 GHz, within 70 km radius, of Latitude 26° 42’ 15” South, longitude 116° 39’ 32” East (Near Boolardy Station, Western Australia).

TIME FRAME:	Ongoing

INSTRUCTIONS

No new assignments are to be made within the frequency range 70 MHz–25.25 GHz, within 70 km radius, of Latitude 26° 42’ 15” South, longitude 116° 39’ 32” East2.  The embargo applies to all apparatus licensed, coordinated terrestrial stations and earth stations that are located within the specified zone.  Existing licensed services may continue to operate.  However, assignment proposals that seek to expand or modify existing apparatus licensed services are subject to the embargo.

Exceptions to this embargo require case-by-case consideration and the approval of the Manager, Spectrum Planning and Engineering Section.  For cases involving the bands managed under the Broadcasting Services Act 1992, approval must be obtained from the Manager, Broadcast Engineering Section.  Sections 100B, 102 and 102A of the Radiocommunications Act 1992 continue to apply.

REASONS

This embargo establishes a Radio Quiet Zone (RQZ) within the areas described above, facilitating the development and use of new radio astronomy technologies.  Subject to planning processes this embargo may be replaced by more detailed policy documents.

On 30 November 2005, subject to sections 100B, 102 and 102A of the Radiocommunications Act 1992, the scope of the embargo was extended to include the bands managed under the Broadcasting Services Act 1992, and the centre coordinates of the embargoed areas was adjusted to more accurately define the RQZ area.

On 30 April 2007 the central coordinates of the embargoed areas were changed to accommodate potential interference issues with new mining operations near the original site.

EMBARGO AUTHORISATION:

[signed]			19/12/2014

Mark Arkell
Manager
Spectrum Engineering Section
Spectrum Planning and Engineering Branch
Australian Communications and Media Authority

EMBARGO 42

FREQUENCY RANGE(S):	3575–3710 MHz

SUBJECT:	Embargo on all new frequency assignments to support future re-planning options.

DATE OF EFFECT:	26 May 2005 (last revised 15 November 2019)

COVERAGE:	Metropolitan and regional areas specified in Attachment 1 for point-to-multipoint licences.

Australia-wide for all other services.

TIME FRAME:	Until further notice.

INSTRUCTIONS

No assignments are to be made for point-to-multipoint licences in the frequency range 3575–3710 MHz inside the metropolitan and regional areas detailed in Attachment 1.

For all other services, no assignments are to be made Australia-wide in the frequency range 3575–3710 MHz.  This includes assignments for existing licensees seeking to expand or modify their radiocommunications systems in this range.

Licences may only be issued or re-issued in the frequency range 3575–3710 MHz (Australia-wide) for a maximum one-year period at a time until further notice. This includes point-to-multipoint licences issued and re-issued in remote areas.

The restrictions of Embargo 42 do not apply to new or existing satellite earth stations located inside the Earth Station Protection Zones defined in Appendix C of RALI MS44.

Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Engineering Section for consideration.

REASONS

The ACMA is currently reviewing arrangements in the 3575–3700 MHz band. The purpose of the embargo is to preserve future planning options and minimise the impact that any future possible change in use might cause. The current restriction will be reconsidered once the outcomes of the 3575–3700 MHz band review are known.

COMMENTS

Interest in the 3.6 GHz band for fixed/mobile broadband has progressed due to spectrum harmonisation developments in the International Telecommunication Union–Radiocommunication Sector (ITU-R) and Asia–Pacific Telecommunity (APT), technology standardisation progress in industry bodies such as 3GPP and considerations within individual countries leading to potential economies of scale.  The progress of this consideration means it is timely to consider the potential re-planning of this band in Australia.

HISTORY

In September 2007 the embargo was revised to include minor editorial changes.

In September 2009 the embargo was revised to allow wireless access services (point-to-multipoint services) to be licensed in regional and remote areas of Australia.

In November 2016 the embargo was revised to restrict access to the 3575–3710 MHz frequency for point to multipoint services in metropolitan and regional areas.

In September 2019 Embargo 42 was revised to allow satellite earth stations to be licenced inside the Earth Station Protection Zones defined in RALI MS44.

EMBARGO AUTHORISATION:

Approved 	15/11/2019

Zarko Krusevac

Acting Manager
Spectrum Engineering Section
Spectrum Planning & Engineering Branch
Communications Infrastructure Division
Australian Communications & Media Authority

ATTACHMENT 1

For the purposes of Embargo 42, metropolitan and regional areas are defined by the following HCIS area description:

BV, CV, DV, IV, IW, JV, JW, KQ, KV, KW, LR, LV, LW, LX, LY, MS, MT, MU, MV, MW, NT, NU, AV9, AW3, BV3, BV6, BV7, BV8, BV9, BW1, BW2, BW3, BW5, BW6, CW1, CW2, CW3, CW4, DW1, DW2, DW3, EV1, EV2, EV3, EV4, EV5, EV6, EV7, FV1, FV2, FV3, FV4, FV5, GV1, GV2, GV3, GV6, HV1, HV2, HV3, HV4, HV5, HV6, HV8, HV9, HW3, HW6, IW1, IW2, IW4, IW5, IW7, IW8, IW9, JW2, JW3, JW4, JW5, JW6, JW7, JW8, JW9, JX1, JX2, JX3, JX5, JX6, KO1, KO4, KO5, KO7, KO8, KP1, KP2, KP4, KP5, KP6, KP7, KP8, KP9, KX1, KX2, KX3, KX4, KX5, KX6, KX8, KX9, KY2, KY3, KY6, LP4, LP7, LQ1, LQ2, LQ4, LQ5, LQ7, LQ8, LZ1, LZ2, LZ3, MR1, MR4, MR5, MR7, MR8, MR9, MV1, MV2, MV3, MV4, MV5, MV6, MV7, MV8, MW1, MW2, MW6, MW7, MW8, MW9, MX1, MX2, MX3, MX4, MX7, MY1, MY4, MY7, MZ1, NS4, NS7, NS8, NS9, NT1, NT2, NT3, NT4, NT7, NU1, NU4, NU5, NU6, NU7, NU8, NU9, NV1, NV2, NV3, NV4, NV5, NV7, NW1

HCIS area descriptions can be converted into a Placemark file (viewable in Google Earth) through the Convert HCIS area description to Placemark facility on the ACMA website.

The HCIS is described in the Australian Spectrum Map Grid 2012.

EMBARGO 43

Status: Replaced by Embargo 26

EMBARGO 44

FREQUENCY RANGE(S):	5950–6200 kHz
7100–7300 kHz
9500–9900 kHz
11650–12050 kHz
13600–13800 kHz
15100–15600 kHz
17550–17900 kHz
21450–21850 kHz
25670–26100 kHz

SUBJECT:	Embargo on new frequency assignments to support domestic broadcasting services using DRM technology

DATE OF EFFECT:	18 September 2006 (last revised September 2007)

COVERAGE:	Australia-wide

TIME FRAME:	Ongoing

INSTRUCTIONS

No new assignments are to be made Australia-wide in the frequency bands listed above. This includes assignments for existing licensees seeking to expand or modify their communications systems in the bands.

The frequency requirements of existing licensees of overseas HF broadcasting services will continue to be licensed for those frequencies coordinated in accordance with Article 12 of the Radio Regulations of the International Telecommunication Union (ITU).

Exceptions to this embargo require case-by-case consideration and the approval of the Manager, Spectrum Engineering Section. ACMA will consider licence applications for trials to investigate use of the bands for Digital Radio Mondiale (DRM).

REASONS

The purpose of this embargo is to support planning of the bands to accommodate domestic broadcasting services using DRM technology. ACMA is currently considering international developments of DRM using these bands and needs to preserve the availability of the bands for potential future use by DRM services in Australia.

The bands are embargoed to ensure that ACMA’s ability to implement the resulting planning arrangements is not constrained by the use of the bands by additional radiocommunications services.

COMMENTS

DRM is a digital broadcasting system that has received wide international support, including the development of receivers by international equipment manufacturers and international trials of the technology since 2003. The extent of the developments, trial outcomes, and interest in use of the band for DRM by Australian parties requires ACMA to assess DRM developments and preserve the bands while planning occurs.

Existing ITU arrangements for the bands will need to be considered in light of current interests in the band for DRM.

HISTORY

In September 2007 the embargo was revised to include editorial changes.

EMBARGO AUTHORISATION:

[signed]			27/09/2007

Geoff McMillen
Manager
Spectrum Engineering Section
Spectrum Planning Branch
Australian Communications and Media Authority

EMBARGO 45

FREQUENCY RANGE(S):	518–520 MHz

SUBJECT:	Embargo on new frequency assignments to support the expansion of UHF TV channel 27 and to preserve planning options for adjacent channel sharing

DATE OF EFFECT:	6 October 2006 (last revised September 2007)

COVERAGE:	Australia-wide

TIME FRAME:	Until further notice

INSTRUCTIONS

No new assignments are to be made Australia-wide in the frequency band 518–520 MHz. This includes assignments for existing licensees seeking to expand or modify their communications systems in the bands.

Exemptions to this embargo require case-by-case consideration and the approval of the Manager, Spectrum Engineering Section. Applications for exemption in remote areas will be considered; it is anticipated that no exemptions will be granted in high, medium or low density geographic locations.

REASONS

The purpose of the embargo on 519–520 MHz is to support the expansion of UHF television channel 27. This channel is currently 6 MHz wide; a channel of 7 MHz width is required to accommodate a standard analog or digital television emission.

The purpose of the embargo on 518–519 MHz is to preserve planning options for adjacent channel sharing. These parameters will be established once the future of channel 27 is decided.

The band is embargoed to ensure that ACMA’s ability to implement the resulting planning arrangements is not constrained by the use of the bands by additional radiocommunications services.

HISTORY

In September 2007 the embargo was revised to include editorial and formatting changes.

EMBARGO AUTHORISATION:

[signed]			27/09/2007

Geoff McMillen
Manager
Spectrum Engineering Section
Spectrum Planning Branch
Australian Communications and Media Authority

EMBARGO 46

FREQUENCY RANGE(S):	5900–5950 kHz
7300–7350 kHz
9400–9500 kHz
11600–11650 kHz
12050–12100 kHz
13570–13600 kHz
13800–13870 kHz
15600–15800 kHz
17480–17550 kHz
18900–19020 kHz

SUBJECT:	Embargo on new frequency assignments to encourage the introduction of digitally modulated emissions for broadcasting services in HF bands

DATE OF EFFECT:	6 February 2007 (last revised September 2007)

COVERAGE:	Australia-wide

TIME FRAME:	Ongoing

INSTRUCTIONS

No new frequency assignments are to be made for any location in Australia or its Territories and adjacent areas in the frequency bands listed above. This includes frequency assignments for existing licensees seeking to expand or modify their radiocommunications systems in the bands.

Exceptions to this embargo require case-by-case consideration and the approval of the Manager, Spectrum Engineering Section. ACMA will consider licence applications for frequency assignments for existing licensees of overseas HF broadcasting services using digitally modulated emissions in accordance with the provisions of Resolution 517 of the International Telecommunication Union (ITU) for those frequencies coordinated in accordance with Article 12 of the Radio Regulations of the ITU.

REASON

The World Administrative Radio Conference of the ITU held in 1992 made decisions concerning the above frequency bands that from 1 April 2007 will result in changed status for existing fixed and mobile services with the introduction of broadcasting services. In Australia, existing services may continue to operate provided that their communication is limited to within the boundary of Australia and on the condition that harmful interference is not caused to the broadcasting service. In addition, existing services must accept interference from broadcasting services.

The ITU World Radiocommunication Conference held in 2003 urged administrations to use the above bands to facilitate the introduction of digitally modulated emissions for broadcasting services in accordance with the provisions of ITU Resolution 517 (Rev. WRC-03).

In addition, ACMA intends to implement the intention of the Australian Government to facilitate the introduction of digital radio by inclusion of the above bands in its planning for the introduction of digital radio.

Considering these reasons it is necessary to regulate use of the bands to facilitate these outcomes. This embargo is intended to do this by encouraging the introduction of digitally modulated emissions for broadcasting services and limiting the use of other emissions.

COMMENTS

Digitally modulated emissions for broadcasting services potentially provide efficiencies in spectrum use and provide significant benefits for users including greater program quality and service availability. The radiofrequency spectrum is a finite resource of significant value to the Australian community. ACMA responsibilities include its management for the overall public benefit to which end ACMA facilitates its efficient use and methods to improve services it supports.

HISTORY

In September 2007 the embargo was revised to include minor editorial changes.

EMBARGO AUTHORISATION:

[signed]			27/09/2007

Geoff McMillen
Manager
Spectrum Engineering Section
Spectrum Planning Branch
Australian Communications and Media Authority

EMBARGO 47

FREQUENCY RANGE(S):	7250–7750 MHz
7900–8400 MHz
10.95–12.75 GHz
13.75–14.50 GHz

SUBJECT:	Embargo on new frequency assignments for stations in terrestrial services near Geraldton

DATE OF EFFECT:		24 September 2007

COVERAGE:	Within 75 km of latitude 28° 41’ 38” south, longitude 114° 50’ 43” east (GDA94)

TIME FRAME:	Until further notice

INSTRUCTIONS

No new assignments are to be made in the frequency bands 7250–7750 MHz, 7900–8400 MHz, 10.95–12.75 GHz and 13.75–14.5 GHz within 75 km of latitude 28° 41’ 38” south, longitude 114° 50’ 43” east (near Kojarena, Western Australia). The embargo applies to all apparatus-licensed terrestrial stations (including those in the fixed and mobile services) located within the specified zone. This includes assignments for existing licensees seeking to expand or modify their radiocommunications systems in the bands.

This embargo does not apply to assignments for satellite Earth stations consistent with relevant planning arrangements.

Exceptions to this embargo require case-by-case consideration and the approval of the Manager, Spectrum Engineering Section. Exemptions will only be granted if the proposed assignments can be shown to successfully coordinate with planned geostationary satellite orbit Earth station operations at this location.

REASON

The purpose of the embargo is to support planned activities at the Australian Defence Satellite Communication Station (ADSCS) located at Kojarena, near Geraldton, WA.

EMBARGO AUTHORISATION:

[signed]			26/09/2007

Geoff McMillen
Manager
Spectrum Engineering Section
Spectrum Planning Branch
Australian Communications and Media Authority

EMBARGO 48

FREQUENCY RANGE:	5850–5925 MHz

SUBJECT:	Embargo on new frequency assignments to support planning for the introduction of intelligent transport systems

DATE OF EFFECT:	24 April 2008

COVERAGE:	Australia-wide

TIME FRAME:	Until further notice

INSTRUCTIONS

No new frequency assignments are to be made in the frequency band 5850–5925 MHz. The embargo applies to all apparatus-licensed stations for any location in Australia or its Territories. This includes frequency assignments for existing licensees seeking to expand or modify their radiocommunications systems in the band.

This embargo also applies to proposed services, any part of whose necessary emission bandwidth is within the embargoed frequency band, and includes channel 1 of the interleaved channelling plan for the 6 GHz band described in the Radiocommunications Assignment and Licensing Instruction (RALI) FX 3.

Exceptions to this embargo require case-by-case consideration and the approval of the Manager, Spectrum Engineering Section.

REASON

The purpose of the embargo is to support planning for the introduction of intelligent transport systems (ITS) into Australia. ACMA is considering the international development of regulatory and channelling arrangements, and the development of Australian requirements, for ITS in the band; and needs to preserve the availability of the band for future use by ITS in Australia.

COMMENTS

ITS involves the application of information and communications technology (ICT) to solve transport issues such as safety, mobility and pollution. ITS uses a broad range of wireless technologies intended to be incorporated in transport system infrastructure and in vehicles. ITS uses dedicated short-range communications (DSRC) technologies and typically involves data transfer over short distances between infrastructure and vehicles, and between vehicles. The introduction of ITS and the planning of its spectrum requirements is supported by Australian road safety and planning authorities.

EMBARGO AUTHORISATION:

[signed]			24/04/2008

Geoff McMillen
Manager
Spectrum Engineering Section
Spectrum Planning Branch
Australian Communications and Media Authority

EMBARGO 49

FREQUENCY RANGE(S):	2015–2100 MHz
2100–2130 MHz
2190–2280 MHz
2280–2310 MHz
6700–7075 MHz
7135–7245 MHz
7250–7750 MHz
7900–8390 MHz
8390–8460 MHz
8460–8510 MHz
8540–8660 MHz
10700–14800 MHz
15349–15410 MHz
15430–15630 MHz
17200–21400 MHz
22200–22510 MHz
24750–25250 MHz
25500–31000 MHz
33400–36000 MHz
37500–43500 MHz
47200–51400 MHz
52590–59300 MHz

SUBJECT:	Embargo on new frequency assignments for terrestrial radiocommunication services

DATE OF EFFECT:	2 April 2009 (last revised 2 August 2019)

COVERAGE:	Within the following distances from Depot Hill Road site, Yarragadee, 18.5 kilometres NW of Mingenew, Western Australia (29 degrees, 2 minutes, 47 seconds South Latitude and 115 degrees, 20 minutes, 35 seconds East Longitude):

- 300 kilometres for 2100–2130 MHz, 2280–2310 MHz;

- 190 kilometres for 7135–7200 MHz and 8390–8460 MHz; otherwise

- 150 kilometres for bands below 12 GHz;

- 100 kilometres for bands above 12 GHz.

TIME FRAME:	Until further notice

INSTRUCTIONS

No new frequency assignments for terrestrial services are to be made within the specified distances of the Mingenew site, Western Australia in the frequency bands listed above. This includes assignments for existing licensees seeking to expand or modify their communications systems in the bands.

Terrestrial services are all services other than the space research service and Earth stations communicating with space objects (GSO and non-GSO communications satellites).

The embargo does not apply to stations operated by Australian Defence Force or the Department of Defence in the band 8500–8510 MHz.

Exceptions to this embargo require case-by-case consideration and the approval of the Manager, Spectrum Planning Section.

REASONS

The purpose of this embargo is to support the development of space communications facilities in the general area of the Mingenew site. Space communications stations typically have particular interference protection requirements and can result in unacceptably large areas of spectrum denial to terrestrial services. In addition, consequences of communications failure due to interference can have unacceptable consequences for some space activities particularly space exploration in the space research service. The potential impact of space communications services on spectrum availability for other services is such that ACMA encourages restriction of their operation to locations beyond propagation range of areas of significant spectrum use by other services.

COMMENTS

Lead times for planning space communications, including deep space for space research, are typically many years. In order for such planning to have certainty of spectrum access it is necessary for spectrum to be withheld from other services for considerable periods of time. This requirement can be difficult to satisfy in areas of spectrum demand by terrestrial services. Considering this, ACMA encourages the siting of such stations at locations beyond propagation range of areas of spectrum demand in order to minimise the impact on spectrum availability for other services. The Mingenew site is one such location which ACMA intends to protect for space related communications activities.

The 300 kilometres criterion for 2100–2130 MHz and 2280–2310 MHz and the 190 kilometres criterion for 7135–7200 MHz and 8390–8460 MHz recognise the particular requirements for deep-space communications involving space exploration missions and the interference protection levels specified in the Radio Regulations of the International Telecommunication Union.

Note that embargo 23 also applies limitation on the bands 2190–2300 MHz and 2015–2110 MHz to facilitate the introduction of television outside broadcast services.  Arrangements for the coordination and operation of TOB services with respect to Mingenew are contained in RALI FX 21 and that RALI should be referred to when considering assignments for TOB services in bands covered by this embargo.

HISTORY

This embargo was put in place in April 2009 to support the development of space communications facilities in low spectrum impact areas.

This embargo was updated in July 2015 to include minor additions (2015–2025 MHz, 2190–2200 MHz, 7200–7245 MHz and 8500–8510 MHz) to existing frequency ranges to further encourage the support the development of space communications facilities in the general area of the Mingenew site.

In August 2019, 3400–4200 MHz and 5850-6700 MHz bands were removed from embargo 49.  These bands are now subject to the coordination procedures contained in RALI MS 44.

EMBARGO AUTHORISATION:

Approved 2/08/2019

Zarko Krusevac

Manager, Spectrum Planning Section

Spectrum Planning and Engineering Branch
Australian Communications & Media Authority

EMBARGO 50

FREQUENCY RANGES:	403–403.9875 MHz
405.0125–406 MHz
409.0375–410.5375 MHz
412.4625–413.4375 MHz
414.4625–415.5625 MHz
418.4875–420 MHz
420–420.75 MHz
421.25–424.75 MHz
425.25–427.75 MHz
428.25–430 MHz
457.5–459.9875 MHz
467.5–469.9875 MHz

SUBJECT:	Embargo on new frequency assignments to support arrangements for harmonised government spectrum primarily to support national security, law enforcement and emergency services

DATE OF EFFECT:	2 April 2009 (last revised 30 April 2010)

COVERAGE:	Australia-wide

TIME FRAME:	Ongoing

INSTRUCTIONS

No new assignments are to be made in the frequency bands 403–403.9875 MHz, 405.0125–406 MHz, 408.6375–410.5375 MHz, 412.4625–413.4375 MHz, 414.4625–415.4375 MHz, 418.0875–420 MHz, 420–420.75 MHz, 421.25–424.75 MHz, 425.25–427.75 MHz and 428.25–430 MHz, except to Federal, State and Territory government agencies involved in national security, law enforcement or the provision of emergency services operating in accordance with the C.O.A.G approved national framework for government radiocommunications interoperability.

New assignments to other parts of Federal, State and Territory government are permitted provided assignments for government agencies involved in the national security, law enforcement or the provision of emergency services have been accommodated.

Licensees other than Federal, State and Territory government agencies involved in national security, law enforcement or the provision of emergency services and other government agencies accommodated in the band must cease operation or relocate outside of this spectrum by 31 December 2015 in high and medium density areas and 31 December 2018 outside high and medium density areas except in the 457.5–459.9875 MHz and 467.5–469.9875 MHz where users must cease operation or relocate outside of this spectrum by 31 December 2014 in high and medium density areas and 31 December 2015 outside high and medium density areas. Authorisation from the relevant state/territory government NCCGR authority stating that the user will be accommodated in the government band will be required to continue operation in this spectrum.

Exceptions to this embargo require case-by-case consideration and approval of the Manager, Spectrum Engineering Section.

REASONS

The purpose of this embargo is to support arrangements for harmonised government spectrum for Federal, State and Territory governments primarily to meet the strategic communications needs of national security, law enforcement and emergency services organisations, including interoperability objectives.

If in doubt about the applicability of this embargo to any particular frequency assignment, please contact the Manager, Government Planning Section for further advice.

HISTORY

The embargo was put in place in April 2009 to preserve planning options associated with the review of the 400 MHz band (403–430 MHz and 440–520 MHz).

The embargo was reviewed and amended in July 2009 to clarify its intent and applicability.

The embargo was again reviewed and amended in April 2010 to reflect the ACMA’s final decision regarding the identification of harmonised government spectrum.

EMBARGO AUTHORISATION:

[signed]			16/07/2010

Geoff McMillen
Manager
Spectrum Engineering Section
Spectrum Infrastructure Branch
Australian Communications and Media Authority

EMBARGO 51

FREQUENCY RANGE(S):	403–518 MHz

SUBJECT:	Embargo on new land mobile frequency assignments requiring channel bandwidths greater than 12.5 kHz, and on high power single frequency assignments

DATE OF EFFECT:	2 April 2009 (last revised September 2012)

COVERAGE:	Australia-wide

TIME FRAME:	Until further notice

INSTRUCTIONS

No new frequency assignments for the land mobile service requiring channel bandwidths greater than 12.5 kHz are to be made in the frequency band 403–518 MHz within high density areas (HDAs) and medium density areas (MDAs).

No new high power frequency assignments for the single frequency land mobile service within HDAs and MDAs or on communal sites (defined as a site with more than a single licensee with assignments in the 400 MHz band at that site) outside HDAs and MDAs.

Exceptions to this embargo require case-by-case consideration and approval of the Manager, Spectrum Engineering Section.

REASONS

The purpose of this embargo is to support measures to address congestion in HDAs and MDAs as part of planning activities in the 400 MHz band (403–430 MHz and 450–520 MHz).

If in doubt about the applicability of this embargo to any particular frequency assignment, please contact the Manager, National Interest Planning Section for further advice.

HISTORY

The embargo was put in place in April 2009 to support planning options associated with the review of the 400 MHz band, and subsequently reviewed and amended in July 2009.

The embargo was reviewed and amended in April 2010 to reflect the ACMA’s final decision regarding the identification of harmonised government spectrum.

The embargo was amended in September 2010 to limit its frequency scope to 518 MHz, to remove the wideband fixed point-to-point service from its scope and to clarify that its scope does not include to point-to-multipoint systems in any part of the 400 MHz band.

The embargo was amended in September 2012 to clarify requirements for communal sites in low and remote density areas and to remove information pertaining to existing assignments as these requirements are covered by MS22.

EMBARGO AUTHORISATION:

[signed]			25/09/2012

Mark Arkell
Manager
Spectrum Engineering and Space
Australian Communications and Media Authority

EMBARGO 52

FREQUENCY RANGE(S):	3400–3575 MHz
3600–3700 MHz

SUBJECT:	Embargo on new frequency assignments in the Woomera Prohibited Area (WPA)

DATE OF EFFECT:	14 September 2009 (last revised Nov. 2009)

COVERAGE:	Inside the designated areas, for point-to multipoint services, as specified in Attachment 1.

TIME FRAME:	Ongoing

INSTRUCTIONS

No assignments are to be made for wireless access services (WAS - point-to-multipoint services) where any part of whose necessary bandwidth is in the frequency ranges 3400–3575 MHz or 3600–3700 MHz inside the designated areas detailed in Attachment 1.

Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Engineering Section for consideration.

REASONS

The purpose of the embargo is to facilitate ongoing use of the WPA by the Department of Defence for activities that are subject to Part 1.4, Division 4 of the Radiocommunications Act 1992.

COMMENTS

In regional and remote areas of Australia the 3.4 GHz (3400–3575 MHz) and 3.6 GHz (3575–3700 MHz) bands have been identified for use by wireless access services (point-to-multipoint services). This embargo limits the use of these bands by WAS to 25 MHz (3575–3600 MHz) inside the WPA.

HISTORY

This embargo was put in place in 14th September 2009.

This embargo was amended on the 26th November 2009 to include the 3.4 GHz (3400–3575 MHz) band.

EMBARGO AUTHORISATION:

[signed]			26/11/2009

Geoff McMillen
Manager
Spectrum Engineering Section
Spectrum Planning Branch
Australian Communications and Media Authority

Attachment 1

INSTRUCTIONS

No assignments are to be made for wireless access services (point-to-multipoint services) in the frequency ranges 3400–3575 MHz and 3600–3700 MHz inside the geographical areas whose boundaries are described by the locations specified in the tables below:

The datum used for all geographic coordinates in this attachment is the Geocentric Datum of Australia 1994 (GDA94).

| LONGITUDE | LATITUDE |
| --- | --- |
| 132 | -30.41 |
| 132.002 | -30.331 |
| 132.002 | -30.081 |
| 131.503 | -30.081 |
| 131.502 | -29.999 |
| 131.502 | -29.747 |
| 131.501 | -29.498 |
| 131.502 | -29.001 |
| 131.502 | -28.499 |
| 131.502 | -28.117 |
| 132.001 | -28.117 |
| 132.001 | -28.075 |
| 132.002 | -27.999 |
| 132.002 | -27.839 |
| 132.5 | -27.839 |
| 133 | -27.84 |
| 133.501 | -27.839 |
| 133.697 | -27.84 |
| 133.835 | -27.839 |
| 133.836 | -27.999 |
| 133.839 | -28.531 |
| 134.095 | -28.53 |
| 134.095 | -28.746 |
| 134.344 | -29.001 |
| 134.358 | -29.011 |
| 134.357 | -29.115 |
| 134.95 | -29.115 |
| 134.951 | -28.999 |
| 134.951 | -28.742 |
| 135.001 | -28.742 |
| 135.173 | -28.742 |
| 135.172 | -28.721 |
| 135.507 | -28.722 |
| 135.838 | -28.723 |
| 135.893 | -28.769 |
| 136.277 | -28.959 |
| 136.432 | -29.07 |
| 136.601 | -29.292 |
| 136.602 | -29.781 |
| 136.636 | -29.856 |
| 136.636 | -29.974 |
| 136.669 | -29.993 |
| 136.735 | -29.993 |
| 136.735 | -30 |
| 136.736 | -30.532 |
| 136.842 | -30.532 |
| 136.843 | -30.624 |
| 136.913 | -30.624 |
| 136.905 | -30.98 |
| 136.869 | -31.193 |
| 136.822 | -31.235 |
| 136.396 | -31.232 |
| 135.884 | -31.099 |
| 135.817 | -31.001 |
| 135.762 | -30.912 |
| 135.554 | -30.912 |
| 135.361 | -30.908 |
| 135.36 | -30.844 |
| 132 | -30.41 |

EMBARGO 53

FREQUENCY RANGES:	406.1–408.6375 MHz
410.5375–412.4625 MHz
415.5625–418.0875 MHz
450–450.4875 MHz
452.5–457.50625 MHz
462–467.50625 MHz
469.9875–476.4125 MHz
477.41875–518 MHz

SUBJECT:	Embargo on new frequency assignments to national security, law enforcement and emergency services

DATE OF EFFECT:	30 April 2010 (last revised September 2012)

COVERAGE:	Australia-wide

TIME FRAME:	Ongoing

INSTRUCTIONS

No new assignments are to be made to Federal, State and Territory government agencies for the purposes of national security, law enforcement or the provision of emergency services in the frequency bands 406.1–408.6375 MHz, 410.5375–412.4625 MHz, 415.5625–418.0875 MHz, 450–450.4875 MHz, 452.5–457.50625 MHz, 462–467.50625 MHz, 469.9875–476.4125 MHz and 477.41875–518 MHz.

New assignments to Federal, State and Territory government agencies are permitted in the frequency ranges above provided it can be demonstrated that they cannot be accommodated in spectrum set aside for government purposes.

Exemptions from this embargo require case-by-case consideration and approval by the Manager, Spectrum Engineering Section.

REASONS

The purpose of this embargo is to support arrangements for harmonised government spectrum for Federal, State and Territory governments primarily to meet the strategic communications needs of national security, law enforcement and emergency services organisations, including interoperability objectives.

If in doubt about the applicability of this embargo to any particular frequency assignment, please contact the Manager, Spectrum Engineering Section for further advice.

HISTORY

The embargo was put in place in April 2010 to support the identification of harmonised government spectrum in the 400 MHz band (403–430 MHz and 450–520 MHz).

The embargo was amended in July 2011 to extend its applicability to all government services, to excise segments Q, R, U and V and its upper limit was reduced to 518 MHz from 520 MHz.

The embargo was amended in June 2012 to exclude the UHF Citizen Band (segment EE) from its scope.

EMBARGO AUTHORISATION:

[signed]			25/09/2012

Mark Arkell 
Manager
Spectrum Engineering Section
Australian Communications and Media Authority

EMBARGO 54

Status: Lifted

EMBARGO 55

Status: Lifted

EMBARGO 56

Status: Lifted

EMBARGO 57

Status: Not used

EMBARGO 58

Status: Not used

EMBARGO 59

FREQUENCY RANGE(S):	7250–7750 MHz
7900–8400 MHz

SUBJECT:	Embargo on new frequency assignments for stations in terrestrial services near Kapooka, NSW

DATE OF EFFECT:		22 June 2010

COVERAGE:	Within 75 km of latitude 35° 10’ 16” south, longitude 147° 15’ 43” east (GDA94)

TIME FRAME:	Until further notice

INSTRUCTIONS

No new assignments are to be made in the frequency bands 7250–7750 MHz and 7900–8400 MHz within 75 km of latitude 35° 10’ 16” south, longitude 147° 15’ 43” east (Kapooka, NSW). The embargo applies to all apparatus-licensed terrestrial stations (including those in the fixed and mobile services) located within the specified zone. This includes assignments for existing licensees seeking to expand or modify their radiocommunications systems in the bands.

This embargo does not apply to assignments for satellite Earth stations consistent with relevant planning arrangements.

Exceptions to this embargo require case-by-case consideration and approval of the Manager, Spectrum Engineering Section. Exemptions will only be granted if the proposed assignments can be shown to successfully coordinate with planned geostationary satellite orbit Earth station operations at this location.

REASON

The purpose of the embargo is to support potential development of a Defence satellite Earth station at Kapooka, near Wagga Wagga, NSW.

EMBARGO AUTHORISATION:

[signed]			22/06/2010

Geoff McMillen 
Manager
Spectrum Engineering Section
Spectrum Infrastructure Branch
Australian Communications and Media Authority

EMBARGO 60

FREQUENCY RANGES:	408.6375–409.04375 MHz
418.0875–418.49375 MHz
450.050 MHz

SUBJECT:	Embargo on new frequency assignments to support formalising arrangements for the rail industry

DATE OF EFFECT:	16 July 2010 (revised September 2012)

COVERAGE:	Australia-wide

TIME FRAME:	Ongoing

INSTRUCTIONS

No new assignments are to be made in the frequency bands 408.6375–409.04375 MHz and 418.0875–418.49375 MHz and the frequency 450.050 MHz except to organisations in the rail industry. In determining these organisations, the ACMA will be guided by advice from the Australasian Railway Association. All assignments for the land mobile service on the frequency 450.050 MHz shall be 12.5 kHz bandwidth or less.

Exceptions to this embargo require case-by-case consideration and approval of the Manager, Spectrum Engineering Section. The Australasian Railway Association will be consulted in considering any exception to this embargo.

REASONS

The purpose of this embargo is to support planning options to support formalising arrangements for the rail industry as discussed in the ACMA paper The Way Ahead- Decisions and implementation options for the 400 MHz Band.

If in doubt about the applicability of this embargo to any particular frequency assignment, please contact the Manager, Spectrum Engineering Section for further advice.

HISTORY

This embargo was put in place in July 2010 to preserve planning options associated with the outcomes of the review of the 400 MHz band (403–430 MHz and 440–520 MHz). It was amended in August 2012 to correct frequency ranges.

EMBARGO AUTHORISATION:

[signed]			25/09/2012

Mark Arkell
Manager
Spectrum Engineering Section
Australian Communications & Media Authority

EMBARGO 61

FREQUENCY RANGE(S):	3492.5–3542.5 MHz

SUBJECT:	Embargo on all new frequency assignments to support future planning for terrestrial fixed and mobile services

DATE OF EFFECT:	21 January 2011

COVERAGE:	Australia-wide

TIME FRAME:	Until further notice

INSTRUCTIONS

No assignments are to be made Australia-wide in the frequency range 3492.5–3542.5 MHz.

Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Engineering Section for consideration.

REASONS

The purpose of the embargo is preserve future planning options in the band for terrestrial fixed and mobile services.

COMMENTS

Internationally the band 3400–3600 MHz was identified for the implementation of IMT at WRC-07 by over 100 countries and, in a number of these and other countries, for other types of terrestrial wireless access. Currently there are relatively few assignments in the 3492.5–3542.5 MHz band Australia-wide.

HISTORY

This embargo was put in place in January 2011

EMBARGO AUTHORISATION:

[signed]			14/01/2011

Geoff McMillen
Manager
Spectrum Engineering Section
Australian Communications and Media Authority

EMBARGO 62

FREQUENCY RANGE(S):	1710–1785 MHz
1805–1880 MHz

SUBJECT:	Embargo on apparatus licence frequency assignments to support use of the band for PTS

DATE OF EFFECT:	21 January 2011 (last revised January 2016)

COVERAGE:	Major metropolitan and regional areas for PTS apparatus licences, Australia-wide for all other services

TIME FRAME:	Until further notice

INSTRUCTIONS

No assignments are to be made for Apparatus Licences Australia-wide in the frequency ranges 1710–1785 MHz and 1805–1880 MHz, with the exception of Apparatus Licences for Public Telecommunication Services (PTS) outside the area described in Attachment 1. This limits PTS licences to remote areas of Australia.

Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Engineering Section for consideration.

REASONS

The purpose of the embargo is to preserve future planning options in the defined frequency ranges for terrestrial mobile services.

The purpose of the January 2016 update is to facilitate PTS in remote Australia.

COMMENTS

Consultation on the use of the band for PTS began in 2012 see: 1800 MHz: a shared strategy. As a result of which, Apparatus Licences for PTS in the 1800 MHz Band in remote Australia are permitted. Coordination and licensing procedures for Apparatus Licensed mobile services in the 1800 MHz Band are detailed in RALI MS 34.

EMBARGO AUTHORISATION:

Approved	12/01/2016

Mark Arkell
Manager
Spectrum Engineering Section
Australian Communications and Media Authority

ATTACHEMENT 1:

Apparatus Licences for PTS are permitted outside the area described by the following HCIS:

BV, IW, LX, LY, MW, NT, NU, AU9, AV9, AW3, BU7, BU8, BU9, BW1, BW2, BW3, BW5, BW6, CV4, CV7, CW1, CW4, GV1, GV2, GV3, GV6, HV1, HV2, HV4, HV5, HV6, HV8, HV9, HW3, HW6, IV4, IV5, IV6, IV7, IV8, IV9, JV4, JV5, JV7, JV8, JW1, JW2, JW4, JW5, JW6, JW7, JW8, JW9, JX1, JX2, JX3, JX5, JX6, KW4, KW5, KW6, KW7, KW8, KW9, KX1, KX2, KX3, KX4, KX5, KX6, KX8, KX9, KY2, KY3, KY6, LQ1, LQ2, LQ4, LQ5, LQ7, LQ8, LR2, LR3, LR5, LR6, LV9, LW2, LW3, LW4, LW5, LW6, LW7, LW8, LW9, LZ1, LZ2, LZ3, MR1, MR4, MR5, MR7, MR8, MR9, MS1, MS2, MS3, MS4, MS5, MS6, MS8, MS9, MT3, MT6, MT9, MU3, MU5, MU6, MU8, MU9, MV2, MV3, MV4, MV5, MV6, MV7, MV8, MV9, MX1, MX2, MX3, MX4, MX7, MY1, MY4, MY7, MZ1, NS4, NS7, NS8, NS9, NV1, NV2, NV3, NV4, NV5, NV7, NW1, GO7C, GO7D, GO7G, GO7H, GO7K, GO7L, GO8A, GO8E, GO8I

The HCIS is described in the Australian Spectrum Map Grid 2012. The Australian Spectrum Map Grid 2012 is available on the ACMA website at: www.acma.gov.au.

EMBARGO 63

Status: Lifted

EMBARGO 64

Status: Lifted

EMBARGO 65

FREQUENCY RANGE(S):	2300–2302 MHz

SUBJECT:	Embargo on all new frequency assignments to support expansion of the 2.3 GHz spectrum licence.

DATE OF EFFECT:	25 September 2012

COVERAGE:	Australia-wide

TIME FRAME:	Until further notice

INSTRUCTIONS

No new frequency assignments are to be made in the band 2300–2302 MHz.

Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Engineering and Space Section for consideration.

REASONS

The purpose of the embargo is to support the designation of the band 2300–2302 MHz for spectrum licensing Australia-wide. It is intended that this designation will extend the existing 2.3 GHz (2302–2400 MHz) spectrum licence band to cover the 2300–2400 MHz band Australia-wide.

COMMENTS

On 14 January 2000 the band 2302–2400 MHz was designated for spectrum licensing throughout Australia in the Radiocommunications (Spectrum Designation) Notice No. 1 of 2000.

The ACMA has previously indicated it would review options to expand the 2.3 GHz spectrum licence band from 98 MHz to 100 MHz Australia-wide, most recently in the Five-year spectrum outlook 2012–2016.

EMBARGO AUTHORISATION:

[signed]			25/09/2012

Mark Arkell
Manager
Spectrum Engineering and Space
Australian Communications and Media Authority

EMBARGO 66

Status: Lifted

EMBARGO 67

FREQUENCY RANGE(S):	45–52 MHz
56–70 MHz
85–87.5 MHz
137–144 MHz

SUBJECT:	Embargo on new frequency assignments for broadcasting licence types

DATE OF EFFECT:	6 February 2014 (last revised 31 March 2017)

COVERAGE:	Australia-wide

TIME FRAME:	Until further notice

INSTRUCTIONS

No new frequency assignments for any broadcasting licence types (including narrowcasting) are to be made in the above mentioned bands corresponding to former analog television channels 0, 1, 2, 3 (in part) and 5A Australia-wide.

An exemption applies to licences issued for 30 days or less to facilitate the use of this spectrum for special events.

Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Engineering Section for consideration.

REASONS

The purpose of the embargo is to preserve planning options for the future use of these bands while they are under review.

COMMENTS

The ACMA intends to undertake a review into future use of VHF broadcasting bands vacated by analog television services.

Non-broadcasting usage of these bands is permitted, consistent with the Australian Radiofrequency Spectrum Plan and any other planning and licensing requirements.  Such use in each band is enabled by section 34 ‘drop throughs’ (Broadcasting Services Act).

The embargo does not cover the 87.5–108 MHz FM radio broadcasting band in which a limited number of analog television services operated on channels 3, 4 and 5.

HISTORY

In March 2017, this embargo was revised to include the band 85–87.5 MHz.  When the initial embargo was put in place this band was governed by a statutory band plan VHF mid band frequency band plan 1991 which limited the potential usage of this band for broadcasting.  That frequency band plan has sunset and been replaced by RALI MS42 which under some circumstances would appear to permit broadcasting assignments.  To clarify that broadcasting is not intended to occur in this band until a review is undertaken the embargo was extended to explicitly include the 85–87.5 MHz band.

EMBARGO AUTHORISATION:

Approved			31/03/2017

Mark Arkell
Manager
Spectrum Engineering and Space
Australian Communications and Media Authority

EMBARGO 68

FREQUENCY RANGE(S):	3400–3425 MHz

SUBJECT:	Embargo on all new frequency assignments to support future planning for possible terrestrial fixed and mobile services.

DATE OF EFFECT:	12 May 2014

COVERAGE:	Australia-wide

TIME FRAME:	Until further notice

INSTRUCTIONS

No assignments are to be made Australia-wide in the frequency range 3400–3425 MHz.

Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Engineering Section for consideration.

REASONS

The purpose of the embargo is preserve future planning options in the band for possible terrestrial fixed and mobile services.

COMMENTS

Internationally the band 3400–3600 MHz was identified for the implementation of IMT at WRC-07 by over 100 countries and, in a number of these and other countries, for other types of terrestrial wireless access.

Currently there are relatively few assignments in the 3400–3425 MHz band Australia-wide.

EMBARGO AUTHORISATION:

[signed]			12/05/2014

Mark Arkell
Manager
Spectrum Engineering
Australian Communications and Media Authority

EMBARGO 69

FREQUENCY RANGE(S):	24.25–24.7 GHz

SUBJECT:	Restriction on all new frequency assignments until new licensing arrangements are implemented.

DATE OF EFFECT:	2 March 2016 (updated 26 July 2020)

COVERAGE:	Australia-wide.

TIME FRAME:	Until further notice

INSTRUCTIONS

No new apparatus assignments in the frequency range 24.25-24.7 GHz Australia-wide. Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Planning Section for consideration.

REASONS

The purpose of this embargo is to preserve the band for future class licensing arrangements. The scope of this embargo has been updated to remove frequencies above 24.7 GHz where area-wide apparatus licences will be available for issue (now covered under Embargo 79).

COMMENTS

The 26 GHz band (24.25-27.5 GHz) is one of the key frequency bands being considered for the delivery of millimetre wave (mmWave) fifth generation (5G) wireless broadband services, both internationally and domestically. The ACMA has conducted a review of the band, which concluded with the release of the paper Future use of the 26 GHz band—Planning decisions and preliminary views (the decision paper). This release signalled the progression of the 26 GHz band to the ‘re-planning’ phase under the ACMA’s planning process and sets out an agenda for the implementation of a range of new measures, including:

- the introduction of new wireless broadband services in the band, including the band 25.1- 27.5 GHz proposed to be made available for wide-area deployments in populated areas under spectrum licensing

- a suite of spectrum, apparatus and class licensing measures to enable participation from a broad cross-section of industry, including sectors not traditionally associated with wireless broadband network deployments.

Given the band will undergo a change in near future, any services deployed in the 26 GHz band may be affected. Of particular concern are those services with long term requirements and/or significant infrastructure associated with them.  Due to the relatively low level use of the band at present, the ACMA therefore considers it prudent to restrict the issue of new licences to facilitate the ‘re-planning’ phase and deter any potential new licensees that would be adversely affected by planned new use of the band.

This embargo is expected to be incrementally lifted when the new class licensing arrangements are implemented, expected late-Q4 2020.

EMBARGO AUTHORISATION:

Approved			26/10/2020

Chris Worley
Manager
Spectrum Planning
Australian Communications and Media Authority

EMBARGO 70

FREQUENCY RANGE(S):	1427–1518 MHz

SUBJECT:	Restriction on all new frequency assignments to preserve future planning options in the identified frequency range.

DATE OF EFFECT:	20 April 2016

COVERAGE:	Metropolitan and Regional Areas

TIME FRAME:	Until further notice

INSTRUCTIONS

No assignments are to be made in defined Metropolitan and Regional Areas (refer to Attachment 1 for a description) for apparatus licences in the frequency range 1427–1518 MHz. Case-by-case exemptions are to be referred to the Manager, Spectrum Engineering Section for consideration.

Any new/existing licences in the 1427–1518 MHz band will be issued/renewed for a maximum period of one-year at a time. This policy applies Australia-wide (i.e. including remote areas).

REASONS

The purpose of the embargo is to preserve future planning options in the identified frequency range and minimise the impact that a future possible change in highest value use of the band might cause.

COMMENTS

The 1427–1518 MHz frequency range is currently used for a mix of mobile (aeronautical) and terrestrial fixed services. WRC-15 identified the 1427–1452 MHz and 1492–1518 MHz frequency ranges world-wide for the implementation of International Mobile Telecommunications (IMT). The 1452–1492 MHz frequency range was also identified in Region 2 (North and South America), Region 3 (Asia-Pacific) and numerous Region 1 countries (African and Middle East Nations) for the implementation of IMT.

Given significant global interest in the 1427–1518 MHz frequency range for IMT, the ACMA intends to review arrangements in the bands (refer to the ACMA’s Mobile Broadband Work Program – February 2016 update).

It is noted that the Radiocommunications 1.5 GHz Frequency Band Plan 2015 already applies to the 1452–1492 MHz frequency band. This embargo restricts the issue of any new licences except point-to-multipoint services operated on specific frequencies and used for the delivery of public telecommunications services in a rural or remote area.

EMBARGO AUTHORISATION:

[approved]			20/04/2016

Mark Arkell
Manager
Spectrum Engineering
Australian Communications and Media Authority

ATTACHMENT 1:

For the purpose embargo 70, Metropolitan and Regional areas are described by the following HCIS:

BV, JW, IW, KW, LX, LY, MV, MW, NT, NU, AU2, AU3, AU6, AU9, AV9, AW3, BU1, BU2, BU4, BU5, BU7, BU8, BU9, BW1, BW2, BW3, BW5, BW6, CV4, CV7, CW1, CW4, GV1, GV2, GV3, GV6, HV1, HV2, HV4, HV5, HV6, HV8, HV9, HW3, HW6, IV4, IV5, IV6, IV7, IV8, IV9, JV4, JV5, JV7, JV8, JV9, JX1, JX2, JX3, JX5, JX6, KV7, KX1, KX2, KX3, KX4, KX5, KX6, KX8, KX9, KY2, KY3, KY6, LQ1, LQ2, LQ4, LQ5, LQ7, LQ8, LR2, LR3, LR5, LR6, LV9, LW1, LW2, LW3, LW4, LW5, LW6, LW7, LW8, LW9, LZ1, LZ2, LZ3, MR1, MR4, MR5, MR7, MR8, MR9, MS1, MS2, MS3, MS4, MS5, MS6, MS8, MS9, MT3, MT6, MT9, MU3, MU5, MU6, MU8, MU9, MX1, MX2, MX3, MX4, MX7, MY1, MY4, MY7, MZ1, NS4, NS7, NS8, NS9, NV1, NV2, NV3, NV4, NV5, NV7, NW1, GO7C, GO7D, GO7G, GO7H, GO7K, GO7L, GO8A, GO8E, GO8I

The HCIS is described in the Australian Spectrum Map Grid 2012. The Australian Spectrum Map Grid 2012 is available on the ACMA website at: www.acma.gov.au.

EMBARGO 71

Status: Lifted

EMBARGO 72

FREQUENCY RANGES:	2025–2110 MHz
2200–2290 MHz

SUBJECT:	Embargo on new frequency assignments for terrestrial radiocommunication services in parts of regional NSW and regional Queensland.

DATE OF EFFECT:	18 July 2018 (Last revised 2 August 2019)

COVERAGE:	Area 1, Area 2, Area 3 and Area 4. Refer to Attachment 1 for descriptions of Area 1, Area 2, Area 3 and Area 4.

TIME FRAME:	It is expected that embargo 72 will be reviewed before December 2020.

INSTRUCTIONS

For the bands listed above no new frequency assignments for terrestrial services are to be made within Area 1, Area 2, Area 3 and Area 4 for all bands.

This includes assignments for existing licensees seeking to expand or modify their communications systems in the bands. Existing licences are not affected by this embargo.

For the purposes of this embargo, terrestrial services include all services other than Earth stations communicating with space objects (GSO and non-GSO satellites).

Exceptions to this embargo require case-by-case consideration and the approval of the Manager, Spectrum Planning Section.

Other embargoes and assignment restrictions should be considered in conjunction with this embargo. Particularly attention is drawn to embargo 23 which contains guidance regarding assignments for Earth station in 2025–2110 and 2200–2290 MHz.

REASONS

The purpose of this embargo is to support the development of one or more areas providing long-term certainty and flexibility for the operation of commercial space communications teleport facilities in east Australia.  Following on from the outcomes of the consultation on the future use of the 3.6 GHz band, the ACMA has initially identified areas around Moree, Quirindi and Roma for further consideration as possible locations.  These are described by Area 2, Area 3 and Area 4 respectively.  Area 1 has been developed as protection zones of 150 km around each of Area 2, Area 3 and Area 4 to facilitate earth stations facilities being deployed in the areas.

It is recommended that, where possible, prospective commercial earth station licensees consider locating their facilities within Area 2, Area 3 or Area 4 and engaging with the ACMA in the process of choosing one or more eventual long-term earth station protection zones.  These areas (2, 3 and 4) are currently the most likely areas to be identified to support the deployment and long-term operation of space communications teleport facilities in east Australia.

It is noted that if one or more of these sites are found to be unsuitable, embargo 72 will be reviewed to remove any assignment restrictions associated with the location(s).

COMMENTS

Lead times for planning space communications are typically many years. In order for such planning to have certainty of spectrum access it is necessary for spectrum to be withheld from other services for considerable periods of time.  This requirement can be difficult to satisfy in areas of high demand for spectrum by terrestrial services.  Considering this, the ACMA encourages the siting of such stations at locations of low demand for access to spectrum in order to minimise the impact on spectrum availability for other services.

As consideration of each area (2, 3 and 4) progresses, the ACMA, where appropriate, will work with industry to replace the embargo with coordination requirements.  It is expected that any such arrangements will be developed on a band by band basis.  This will protect space communications and limit the effect on access to spectrum for terrestrial services within the listed bands.  The bands included in embargo 72 will continue to be revised and reviewed depending on demand and associated protection requirements for satellite services.

HISTORY

In July 2018 this embargo was revised to remove frequency ranges subject to defence footnotes AUS100, AUS100A (FIXED SATELLITE), AUS101 or AUS101A (FIXED SATELLITE) that is the 4500–4800, 7250–7750, 7900–8400, 147145–14800, 20200–21200, 30000–31000 and 33400–36000 MHz bands.  Previously this advice was contained in the instructions by reference to the footnotes only.  The July 2018 revision avoids any ambiguity as to what frequency ranges are subject to the embargo.

Also in July 2018 the 3575–4200 and 5850–6700 MHz bands were removed from embargo 72.  These bands are now subject to the coordination procedures contained in RALI MS44.

In August 2019, the 5091–5250 MHz, 6700–7075 MHz, 10700–11700 MHz, 12200–13250 MHz, 13750–14714.5 MHz, 15430–15630 MHz, 17300–20200 MHz, 24650–25250 MHz, 27000–30000 MHz, 37500–43500 MHz, 47200–50200 MHz and 50400–51400 MHz bands were removed from embargo 72.  These bands are now subject to the coordination procedures contained in RALI MS44.

EMBARGO AUTHORISATION:

Approved		2/08/2019

Zarko Krusevac

Manager
Spectrum Planning Section 
Australian Communications and Media Authority

ATTACHMENT 1:

For the purpose of embargo 72, Area 1 is described by the following HCIS:

LT6, MT1, MT7, MT8, MU2, MU4, MU8, MU9, MV2, MV6, NV1, LT3H, LT3K, LT3L, LT3N, LT3O, LT3P, LT9B, LT9C, LT9D, LT9F, LT9G, LT9H, LT9K, LT9L, LT9P, MT2A, MT2B, MT2E, MT2F, MT2G, MT2H, MT2I, MT2J, MT2K, MT2L, MT2M, MT2N, MT2O, MT2P, MT3I, MT3M, MT4A, MT4B, MT4C, MT4D, MT4E, MT4I, MT4M, MT4N, MT5A, MT5B, MT5C, MT5D, MT5F, MT5G, MT5H, MT5J, MT5K, MT5L, MT5N, MT5O, MT5P, MT6A, MT6B, MT6E, MT6F, MT6I, MT6J, MT6M, MT6N, MT9A, MT9B, MT9E, MU1C, MU1D, MU1G, MU1H, MU1J, MU1K, MU1L, MU1N, MU1O, MU1P, MU3A, MU3B, MU3C, MU3E, MU3F, MU3G, MU3H, MU3I, MU3J, MU3K, MU3L, MU3M, MU3N, MU3O, MU3P, MU5A, MU5B, MU5E, MU5F, MU5I, MU5J, MU5M, MU5N, MU5O, MU5P, MU6B, MU6C, MU6D, MU6F, MU6G, MU6H, MU6J, MU6K, MU6L, MU6M, MU6N, MU6O, MU6P, MU7B, MU7C, MU7D, MU7F, MU7G, MU7H, MU7K, MU7L, MV1H, MV1L, MV3A, MV3B, MV3E, MV3I, MV3M, MV3N, MV5A, MV5B, MV5C, MV5D, MV5F, MV5G, MV5H, MV5K, MV5L, MV5P, NU1I, NU1M, NU1N, NU4A, NU4B, NU4E, NU4F, NU4G, NU4I, NU4J, NU4K, NU4M, NU4N, NU7A, NU7B, NU7E, NU7F, NU7G, NU7H, NU7I, NU7J, NU7K, NU7L, NU7M, NU7N, NU7O, NU7P, NU8I, NU8M, NV2A, NV2B, NV2E, NV2F, NV2I, NV2J, NV2M, NV2N, NV4A, NV4B, NV4C, NV4D, NV4E, NV4F, NV4G, NV4H, NV4I, NV4J, NV4K, NV4L, NV4M, NV4N, NV5A, NV5B, NV5E, LT3C9, LT3D5, LT3D6, LT3D7, LT3D8, LT3D9, LT3F9, LT3G2, LT3G3, LT3G4, LT3G5, LT3G6, LT3G7, LT3G8, LT3G9, LT3I9, LT3J2, LT3J3, LT3J4, LT3J5, LT3J6, LT3J7, LT3J8, LT3J9, LT3M3, LT3M5, LT3M6, LT3M8, LT3M9, LT5H3, LT5H6, LT5H9, LT5L3, LT5L6, LT5L9, LT5P3, LT5P6, LT9A1, LT9A2, LT9A3, LT9A4, LT9A5, LT9A6, LT9A8, LT9A9, LT9E2, LT9E3, LT9E6, LT9E9, LT9J1, LT9J2, LT9J3, LT9J5, LT9J6, LT9J9, LT9O1, LT9O2, LT9O3, LT9O5, LT9O6, LU3D2, LU3D3, LU6H3, LU6H6, LU6H9, LU6L3, LU6L6, LU6L9, MS7N8, MS7N9, MS7O7, MS7O8, MS7O9, MS7P7, MS7P8, MS7P9, MS8M7, MS8M8, MS8M9, MT2C4, MT2C5, MT2C6, MT2C7, MT2C8, MT2C9, MT2D7, MT3E4, MT3E7, MT3E8, MT3J4, MT3J7, MT3J8, MT3N1, MT3N2, MT3N4, MT3N5, MT3N6, MT3N7, MT3N8, MT3N9, MT4F1, MT4F2, MT4F3, MT4F4, MT4F5, MT4F6, MT4F7, MT4F8, MT4G1, MT4J1, MT4J2, MT4J4, MT4J5, MT4J7, MT4J8, MT4J9, MT4O4, MT4O5, MT4O7, MT4O8, MT4O9, MT4P6, MT4P7, MT4P8, MT4P9, MT5E1, MT5E2, MT5E3, MT5E5, MT5E6, MT5E8, MT5E9, MT5I3, MT5I6, MT5I8, MT5I9, MT5M2, MT5M3, MT5M4, MT5M5, MT5M6, MT5M7, MT5M8, MT5M9, MT6C1, MT6C4, MT6C7, MT6G1, MT6G4, MT6G7, MT6G8, MT6K1, MT6K2, MT6K4, MT6K5, MT6K7, MT6K8, MT6O1, MT6O4, MT6O7, MT9C1, MT9C4, MT9F1, MT9F2, MT9F3, MT9F4, MT9F5, MT9F7, MT9F8, MT9I1, MT9I2, MT9I3, MT9I4, MT9I5, MT9I6, MT9I7, MT9I8, MT9J1, MT9M1, MT9M4, MT9M5, MT9M6, MT9M7, MT9M8, MT9M9, MT9N4, MT9N5, MT9N7, MT9N8, MT9N9, MT9O7, MT9O8, MU1A1, MU1A2, MU1A3, MU1A5, MU1A6, MU1B1, MU1B2, MU1B3, MU1B4, MU1B5, MU1B6, MU1B9, MU1F6, MU1F8, MU1F9, MU1I6, MU1I9, MU1M2, MU1M3, MU1M5, MU1M6, MU1M7, MU1M8, MU1M9, MU3D1, MU3D4, MU3D5, MU3D6, MU3D7, MU3D8, MU3D9, MU5C1, MU5C2, MU5C3, MU5C4, MU5C5, MU5C6, MU5C7, MU5D1, MU5D2, MU5D3, MU5D4, MU5D5, MU5D6, MU5K7, MU6A1, MU6A2, MU6A3, MU6A4, MU6A5, MU6A6, MU6A8, MU6A9, MU6E3, MU6E6, MU6E9, MU6I3, MU6I6, MU6I8, MU6I9, MU7A1, MU7A2, MU7A3, MU7A5, MU7A6, MU7A8, MU7A9, MU7E3, MU7J2, MU7J3, MU7J6, MU7O3, MU7P1, MU7P2, MU7P3, MU7P4, MU7P5, MU7P6, MU7P8, MU7P9, MV1D2, MV1D3, MV1D4, MV1D5, MV1D6, MV1D7, MV1D8, MV1D9, MV1P1, MV1P2, MV1P3, MV1P4, MV1P5, MV1P6, MV1P8, MV1P9, MV3C1, MV3C2, MV3C3, MV3C4, MV3C5, MV3C6, MV3C7, MV3D1, MV3D2, MV3D3, MV3D4, MV3D5, MV3D6, MV3D8, MV3D9, MV3F1, MV3F2, MV3F4, MV3F7, MV3J1, MV3J4, MV3J7, MV3J8, MV3O4, MV3O5, MV3O6, MV3O7, MV3O8, MV3O9, MV3P2, MV3P3, MV3P4, MV3P5, MV3P6, MV3P7, MV3P8, MV3P9, MV4D2, MV4D3, MV4D5, MV4D6, MV4D9, MV4H3, MV5E1, MV5E2, MV5E3, MV5E4, MV5E5, MV5E6, MV5E8, MV5E9, MV5I3, MV5I6, MV5J1, MV5J2, MV5J3, MV5J4, MV5J5, MV5J6, MV5J8, MV5J9, MV5N3, MV5O1, MV5O2, MV5O3, MV5O4, MV5O5, MV5O6, MV5O9, MV8D3, MV9A1, MV9A2, MV9A3, MV9A6, MV9B1, MV9B2, MV9B3, MV9B4, MV9B5, MV9B6, MV9C1, MV9C2, MV9C3, MV9C4, MV9C5, MV9C6, MV9D1, MV9D2, MV9D3, MV9D4, MV9D5, MV9D6, NU1A7, NU1E1, NU1E2, NU1E4, NU1E5, NU1E6, NU1E7, NU1E8, NU1E9, NU1F7, NU1J1, NU1J2, NU1J4, NU1J5, NU1J6, NU1J7, NU1J8, NU1J9, NU1O1, NU1O4, NU1O7, NU1O8, NU4C1, NU4C2, NU4C4, NU4C5, NU4C7, NU4C8, NU4C9, NU4O1, NU4O2, NU4O3, NU4O4, NU4O5, NU4O7, NU4O8, NU7C1, NU7C2, NU7C4, NU7C5, NU7C6, NU7C7, NU7C8, NU7C9, NU7D7, NU7D8, NU8E4, NU8E7, NU8E8, NU8J4, NU8J7, NU8J8, NU8N1, NU8N2, NU8N4, NU8N5, NU8N6, NU8N7, NU8N8, NU8N9, NV2C1, NV2C4, NV2C7, NV2G1, NV2G4, NV2G7, NV2K1, NV2K4, NV2K7, NV2O1, NV2O4, NV2O7, NV4O1, NV4O2, NV4O3, NV4O4, NV4O5, NV4O6, NV4O7, NV4O8, NV4P1, NV4P2, NV4P4, NV5F1, NV5F2, NV5F4, NV5F7, NV5I1, NV5I2, NV5I3, NV5I4, NV5I5, NV5I7, NV7A1, NV7A2, NV7A3, NV7A4, NV7A5, NV7B1, NV7B2, NV7B3

For the purpose of embargo 72 Area 2, which surrounds the town of Moree, is described by the following HCIS:

MU5G, MU5H, MU5L, MU5C8, MU5C9, MU5D7, MU5D8, MU5D9, MU5K1, MU5K2, MU5K3, MU5K4, MU5K5, MU5K6, MU5K8, MU5K9, MU6A7, MU6E1, MU6E2, MU6E4, MU6E5, MU6E7, MU6E8, MU6I1, MU6I2, MU6I4, MU6I5, MU6I7

For the purpose of embargo 72 Area 3, which surrounds the town of Quirindi, is described by the following HCIS:

MV3G, MV3H, MV3K, MV3L, MV3C8, MV3C9, MV3D7, MV3F3, MV3F5, MV3F6, MV3F8, MV3F9, MV3J2, MV3J3, MV3J5, MV3J6, MV3J9, MV3O1, MV3O2, MV3O3, MV3P1

For the purpose of embargo 72 Area 4, which surrounds the town of Roma, is described by the following HCIS:

MT4H, MT4K, MT4L, MT4F9, MT4G2, MT4G3, MT4G4, MT4G5, MT4G6, MT4G7, MT4G8, MT4G9, MT4J3, MT4J6, MT4O1, MT4O2, MT4O3, MT4O6, MT4P1, MT4P2, MT4P3, MT4P4, MT4P5, MT5E4, MT5E7, MT5I1, MT5I2, MT5I4, MT5I5, MT5I7, MT5M1

The HCIS is described in the Australian Spectrum Map Grid 2012. The Australian Spectrum Map Grid 2012 is available on the ACMA website at: www.acma.gov.au.

EMBARGO 73

STATUS:	Replaced by Embargo 78.

EMBARGO 74

Status: Lifted

EMBARGO 75

FREQUENCY RANGE:	3425–3442.5 MHz and 3475–3492.5 MHz

SUBJECT:	Embargo on all new frequency assignments to support the implementation of new planning arrangements.

DATE OF EFFECT:	9 May 2019 (last revised 15 November 2019).

COVERAGE:	Metropolitan and regional areas.

TIME FRAME:	This embargo will be reviewed in December 2020.

INSTRUCTIONS

No new frequency assignments for apparatus licences are to be issued in metropolitan and regional areas (defined at Attachment 1) in the 3425–3442.5 MHz and 3475–3492.5 MHz frequency ranges. The exception to this is for licensees seeking to move an existing licensed service from the 3425–3442.5 MHz frequency range to the 3475–3492.5 MHz (or 3492.5–3510 MHz) frequency range.

Licences issued or reissued in the 3425–3442.5 MHz frequency range must expire on or before 30 November 2020.

Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Planning Section for consideration.

REASONS

The purpose of this embargo is to support the implementation of the decisions made as a result of the review of the 3400–3575 MHz band, and to aid the transition of affected services.  This includes restacking point-to-multipoint licences from the 3425–3442.5 MHz frequency range to the 3475–3510 MHz frequency range in affected areas.

Details of the final planning arrangements for the 3400–3575 MHz band are available on the ACMA’s website.

COMMENTS

This embargo will be reviewed after incumbent apparatus licences have been restacked to the new planning arrangements.  This is planned to occur no later than 30 November 2020.

EMBARGO AUTHORISATION:

Approved 15/11/2019

Andrew Stewart
Acting Manager
Spectrum Planning Section
Communications Infrastructure Division
Australian Communications & Media Authority

ATTACHEMENT 1:

For the purpose of Embargo 75, metropolitan and regional areas are defined by the following HCIS area description:

BV, CV, DV, IV, IW, JV, JW, KQ, KV, KW, LR, LV, LW, LX, LY, MS, MT, MU, MV, MW, NT, NU, AU9, AV9, AW3, BU7, BU8, BW1, BW2, BW3, BW5, BW6, CW1, CW2, CW3, CW4, DW1, DW2, DW3, EV1, EV2, EV3, EV4, EV5, EV6, EV7, FV1, FV2, FV3, FV4, FV5, GV1, GV2, GV3, GV6, HV1, HV2, HV3, HV4, HV5, HV6, HV8, HV9, HW3, HW6, JX1, JX2, JX3, JX5, JX6, KO1, KO4, KO5, KO7, KO8, KP1, KP2, KP4, KP5, KP6, KP7, KP8, KP9, KX1, KX2, KX3, KX4, KX5, KX6, KX8, KX9, KY2, KY3, KY6, LP4, LP7, LQ1, LQ2, LQ4, LQ5, LQ7, LQ8, LZ1, LZ2, LZ3, MR1, MR4, MR5, MR7, MR8, MR9, MX1, MX2, MX3, MX4, MX7, MY1, MY4, MY7, MZ1, NS4, NS7, NS8, NS9, NV1, NV2, NV3, NV4, NV5, NV7, NW1, AU6I, AU6J, AU6K, AU6L, AU6M, AU6N, AU6O, AU6P, BU4H, BU4I, BU4J, BU4K, BU4L, BU4M, BU4N, BU4O, BU4P, BU5E, BU5F, BU5G, BU5H, BU5I, BU5J, BU5K, BU5L, BU5M, BU5N, BU5O, BU5P, BU9A, BU9B, BU9E, BU9F, BU9I, BU9J, BU9M, BU9N

HCIS area descriptions can be converted into a Placemark file (viewable in Google Earth) through the Convert HCIS area description to Placemark facility on the ACMA website.

The HCIS is described in the Australian Spectrum Map Grid 2012.

EMBARGO 76

FREQUENCY RANGE:	1900–1920 MHz

SUBJECT:	Embargo on all new apparatus licences in specified areas.

DATE OF EFFECT:	15 November 2019

COVERAGE:	Metropolitan areas defined in Attachment 1.

TIME FRAME:	Indefinite.

INSTRUCTIONS

For the 1900–1920 MHz band, no new assignments for apparatus licences are to be made within any of the areas defined in Attachment 1 to this embargo.

This includes assignments for existing licensees seeking to expand or modify their communications systems in the bands.

Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Planning Section for consideration.

REASONS

The purpose of Embargo 76 is to preserve planning options in the 1900–1920 MHz band in metropolitan areas defined in Attachment 1.

COMMENTS

Prior to 12 October 2017 the 1900–1920 MHz band was subject to spectrum licencing within the areas defined at Attachment 1. This restricted the issue of apparatus licences within these areas.

Since licences expired in October 2017, spectrum licence arrangements no longer apply. Embargo 76 has been put in place whilst long-term arrangements are developed.

EMBARGO AUTHORISATION

Approved	15/11/2019

Andrew Stewart
Acting Manager
Spectrum Planning Section
Communications and Infrastructure Division
Australian Communications and Media Authority

# EMBARGO 78

# FREQUENCY RANGE:	3700-4000 MHz

SUBJECT:	Embargo on all new frequency assignments to support future re-planning options.

# DATE OF EFFECT:	22 July 2020

# COVERAGE:	Australia-wide

TIME FRAME:	This embargo will be reviewed in December 2021

# INSTRUCTIONS

No new frequency assignments for apparatus licences are to be issued in the 3700-4000 MHz range Australia-wide.

The restrictions of Embargo 78 do not apply to new or existing satellite earth stations located inside the Earth Protection Zones as defined in Appendix C of RALI MS44.

Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Planning Section for consideration using the freqplan@acma.gov.au email address.

# REASONS

The ACMA is currently reviewing arrangements in the 3700–4000 MHz range. The purpose of this embargo is to preserve future planning options and minimise the effect that any future possible change in use might cause. The current restrictions will be reconsidered once the outcomes of the 3700–4200 MHz band are known.

# COMMENTS

The ACMA is currently in the process of consulting on possible changes to the wider 3700–4200 MHz band. This may result in a change to planning arrangements in the band that may affect incumbent services. Of particular concern are those services with long term requirements and/or significant infrastructure associated with them. The ACMA therefore considers it prudent to restrict the issue of new apparatus licences to preserve future planning options.

# EMBARGO AUTHORISATION:

APPROVED			15/07/2020

Chris Worley

Manager

Spectrum Planning Section

Australian Communications and Media Authority

ATTACHMENT 1

For the purposes of Embargo 76, the following areas are defined by HCIS area descriptions.

HCIS area descriptions can be converted into a Placemark file (viewable in Google Earth) through the Convert HCIS area description to Placemark facility on the ACMA website.

The HCIS is described in the Australian Spectrum Map Grid 2012.

Area 1—Adelaide

IW3J, IW3K, IW3L, IW3N, IW3O, IW3P, IW6B, IW6C, IW6D, IW6F, IW6G, IW6H, IW3E5, IW3E6, IW3E8, IW3E9, IW3F4, IW3F5, IW3F6, IW3F7, IW3F8, IW3F9, IW3G4, IW3G5, IW3G6, IW3G7, IW3G8, IW3G9, IW3H4, IW3H5, IW3H6, IW3H7, IW3H8, IW3H9, IW3I2, IW3I3, IW3I5, IW3I6, IW3I8, IW3I9, IW3M2, IW3M3, IW3M5, IW3M6, IW3M8, IW3M9, IW6A2, IW6A3, IW6A5, IW6A6, IW6A8, IW6A9, IW6E2, IW6E3, IW6E5, IW6E6, IW6E8, IW6E9, JW1E4, JW1E7, JW1I1, JW1I4, JW1I7, JW1M1, JW1M4

Area 2—Brisbane

NT9, NT5G, NT5H, NT5K, NT5L, NT5O, NT5P, NT6E, NT6F, NT6G, NT6H, NT6I, NT6J, NT6K, NT6L, NT6M, NT6N, NT6O, NT6P, NT8C, NT8D, NT8G, NT8H, NT8K, NT8L, NT8O, NT8P, NU3A, NU3B, NU3C, NU3D, NU3F, NU3G, NU3H, NT5C4, NT5C5, NT5C6, NT5C7, NT5C8, NT5C9, NT5D4, NT5D5, NT5D6, NT5D7, NT5D8, NT5D9, NT6A4, NT6A5, NT6A6, NT6A7, NT6A8, NT6A9, NT6B4, NT6B5, NT6B6, NT6B7, NT6B8, NT6B9, NT6C4, NT6C5, NT6C6, NT6C7, NT6C8, NT6C9, NT6D4, NT6D5, NT6D6, NT6D7, NT6D8, NT6D9, NU2C1, NU2C2, NU2C3, NU2D1, NU2D2, NU2D3, NU2D5, NU2D6, NU2D8, NU2D9, NU2H2, NU2H3, NU3E1, NU3E2, NU3E3, NU3E5, NU3E6, NU3E8, NU3E9, NU3I2, NU3I3, NU3J1, NU3J2, NU3J3, NU3K1, NU3K2, NU3K3, NU3L1, NU3L2, NU3L3

Area 3—Canberra

MW4D, MW4H, MW4L, MW5A, MW5B, MW5E, MW5F, MW5I, MW5J, MW1P4, MW1P5, MW1P6, MW1P7, MW1P8, MW1P9, MW2M4, MW2M5, MW2M6, MW2M7, MW2M8, MW2M9, MW2N4, MW2N5, MW2N6, MW2N7, MW2N8, MW2N9, MW4P1, MW4P2, MW4P3, MW5M1, MW5M2, MW5M3, MW5N1, MW5N2, MW5N3

Area 4—Darwin

GO7C, GO7D, GO7G, GO7H, GO7K, GO7L, GO8A, GO8E, GO8I

Area 5—Hobart

LY8L, LY8P, LY9I, LY9J, LY9K, LY9L, LY9M, LY9N, LY9O, LY9P, LZ2D, LZ2H, LZ3A, LZ3B, LZ3C, LZ3D, LZ3E, LZ3F, LZ3G, LZ3H, LY8H4, LY8H5, LY8H6, LY8H7, LY8H8, LY8H9, LY9E4, LY9E5, LY9E6, LY9E7, LY9E8, LY9E9, LY9F4, LY9F5, LY9F6, LY9F7, LY9F8, LY9F9, LY9G4, LY9G5, LY9G6, LY9G7, LY9G8, LY9G9, LY9H4, LY9H5, LY9H6, LY9H7, LY9H8, LY9H9, LZ2L1, LZ2L2, LZ2L3, LZ3I1, LZ3I2, LZ3I3, LZ3J1, LZ3J2, LZ3J3, LZ3K1, LZ3K2, LZ3K3, LZ3L1, LZ3L2, LZ3L3

Area 6—Melbourne

KX3J, KX3K, KX3L, KX3N, KX3O, KX3P, KX6B, KX6C, KX6D, KX6F, KX6G, KX6H, KX6J, KX6K, KX6L, LX1I, LX1M, LX1N, LX1O, LX4A, LX4B, LX4C, LX4E, LX4I, KX3F7, KX3F8, KX3F9, KX3G7, KX3G8, KX3G9, KX3H4, KX3H5, KX3H6, KX3H7, KX3H8, KX3H9, KX3M6, KX3M8, KX3M9, KX6A2, KX6A3, KX6A5, KX6A6, KX6A8, KX6A9, KX6E2, KX6E3, KX6E5, KX6E6, KX6E8, KX6E9, KX6I2, KX6I3, KX6I5, KX6I6, KX6I8, KX6I9, LX1E4, LX1E7, LX1E8, LX1E9, LX1J1, LX1J4, LX1J5, LX1J6, LX1J7, LX1J8, LX1J9, LX1K4, LX1K7, LX4F1, LX4F2, LX4F4, LX4F5, LX4F7, LX4F8, LX4J1, LX4J2, LX4J4, LX4J5, LX4J7, LX4J8

Area 7—Perth

BV1I, BV1J, BV1K, BV1L, BV1M, BV1N, BV1O, BV1P, BV2I, BV2J, BV2M, BV2N, BV4A, BV4B, BV4C, BV4D, BV4E, BV4F, BV4G, BV4H, BV4I, BV4J, BV4K, BV4L, BV5A, BV5B, BV5E, BV5F, BV5I, BV5J, BV1E7, BV1E8, BV1E9, BV1F7, BV1F8, BV1F9, BV1G7, BV1G8, BV1G9, BV1H7, BV1H8, BV1H9, BV2E7, BV2E8, BV2E9, BV2F7, BV2F8, BV2F9, BV4M1, BV4M2, BV4M3, BV4N1, BV4N2, BV4N3, BV4O1, BV4O2, BV4O3, BV4P1, BV4P2, BV4P3, BV5M1, BV5M2, BV5M3, BV5N1, BV5N2, BV5N3

Area 8—Sydney

NW1, MV9I, MV9J, MV9K, MV9L, MV9M, MV9N, MV9O, MV9P, MW3C, MW3D, MW3G, MW3H, MW3K, MW3L, MW3O, MW3P, NV4N, NV4O, NV4P, NV5M, NV5N, NV5O, NV5P, NV7B, NV7C, NV7D, NV7E, NV7F, NV7G, NV7H, NV7I, NV7J, NV7K, NV7L, NV7M, NV7N, NV7O, NV7P, MV9D6, MV9D9, MV9E4, MV9E5, MV9E6, MV9E7, MV9E8, MV9E9, MV9F4, MV9F5, MV9F6, MV9F7, MV9F8, MV9F9, MV9G4, MV9G5, MV9G6, MV9G7, MV9G8, MV9G9, MV9H3, MV9H4, MV9H5, MV9H6, MV9H7, MV9H8, MV9H9, MW3B2, MW3B3, MW3B5, MW3B6, MW3B8, MW3B9, MW3F2, MW3F3, MW3F5, MW3F6, MW3F8, MW3F9, MW3J2, MW3J3, NV4I5, NV4I6, NV4I8, NV4I9, NV4J4, NV4J5, NV4J6, NV4J7, NV4J8, NV4J9, NV4K4, NV4K5, NV4K6, NV4K7, NV4K8, NV4K9, NV4L4, NV4L5, NV4L6, NV4L7, NV4L8, NV4L9, NV4M2, NV4M3, NV4M5, NV4M6, NV4M8, NV4M9, NV5I4, NV5I5, NV5I6, NV5I7, NV5I8, NV5I9, NV5J4, NV5J5, NV5J6, NV5J7, NV5J8, NV5J9, NV5K4, NV5K5, NV5K6, NV5K7, NV5K8, NV5K9, NV5L4, NV5L5, NV5L6, NV5L7, NV5L8, NV5L9, NV7A2, NV7A3, NV7A4, NV7A5, NV7A6, NV7A7, NV7A8, NV7A9

# EMBARGO 79

# FREQUENCY RANGE:	25.1 – 30 GHz.

SUBJECT:	Restriction on assignment of certain apparatus licence types.

# DATE OF EFFECT:	26 October 2020

# COVERAGE:	25.1-27.5 GHz outside the areas defined in Attachment 1.

# 24.7-25.1 GHz or 27.5-30 GHz Australia-wide.

TIME FRAME:	Until further notice.

# INSTRUCTIONS

Only area-wide and space receive apparatus licences are to be issued in the frequency range in 25.1-27.5 GHz outside the areas defined in Attachment 1 or in 24.7-25.1 GHz and 27.5-30 GHz Australia wide. All existing apparatus licences are permitted to continue to operate and be renewed.

Any applications for case-by-case exemptions are to be referred to the Manager, Spectrum Planning Section for consideration.

REASONS

The purpose of this embargo is to preserve the band for allocation via area-wide apparatus licences (AWLs) and Space Receive apparatus licences. The technical framework for AWLs in the range 25.1-30 GHz has been optimised to support wireless broadband services and fixed-satellite service earth stations. New space receive apparatus licences are also permitted to maintain ongoing access for ubiquitous earth stations operated under the Radiocommunications (Communication with Space Object) Class Licence 2015 (CSO class licence).

COMMENTS

The 26 GHz band (24.25-27.5 GHz) and 28 GHz band (27.5-29.5 GHz) are key bands planned for the delivery of millimetre wave fifth generation (5G) wireless broadband services. The frequency range 27-30 GHz is also a key band for the delivery of fixed-satellite services (FSS) uplinks.

The ACMA has decided that new wireless broadband services and FSS earth stations (not otherwise authorised by space receive apparatus/CSO class licence arrangements) in the frequency range 24.7-30 GHz are to be authorised using area-wide licences. Additional information is available in the Allocation of apparatus licences in the 26 GHz and 28 GHz bands – Applicant information pack and in RALI MS 46.

EMBARGO AUTHORISATION:

Approved		26/10/2020

Chris Worley

Manager

Spectrum Planning

Australian Communications and Media Authority

ATTACHEMENT 1:

| Named area | HCIS identifiers |
| --- | --- |
| Adelaide | IW3J, IW3K, IW3L, IW3N, IW3O, IW3P, IW6B, IW6C, IW6D, IW6F, IW6G, IW6H, IW3E5, IW3E6, IW3E8, IW3E9, IW3F4, IW3F5, IW3F6, IW3F7, IW3F8, IW3F9, IW3G4, IW3G5, IW3G6, IW3G7, IW3G8, IW3G9, IW3H4, IW3H5, IW3H6, IW3H7, IW3H8, IW3H9, IW3I2, IW3I3, IW3I5, IW3I6, IW3I8, IW3I9, IW3M2, IW3M3, IW3M5, IW3M6, IW3M8, IW3M9, IW6A2, IW6A3, IW6A5, IW6A6, IW6A8, IW6A9, IW6E2, IW6E3, IW6E5, IW6E6, IW6E8, IW6E9, IW6K1, IW6K2, IW6K3, IW6K4, IW6K5, IW6K6, IW6L1, JW1E4, JW1E7, JW1I1, JW1I4, JW1I7, JW1M1, JW1M4 |
| Canberra | MW5E, MW4D6, MW4D9, MW4H3, MW4H9, MW4L3, MW5A4, MW5A5, MW5A6, MW5A7, MW5A8, MW5A9, MW5B4, MW5B7, MW5F1, MW5F4, MW5F7, MW5I1, MW5I2, MW5I3, MW5J1 |
| Darwin | GO7C, GO7D, GO7G, GO7H, GO7K, GO7L, GO8A, GO8E, GO8I |
| Greater Brisbane/Lismore | NT9, NT5G, NT5H, NT5K, NT5L, NT5O, NT5P, NT6E, NT6F, NT6G, NT6H, NT6I, NT6J, NT6K, NT6L, NT6M, NT6N, NT6O, NT6P, NT7H, NT7L, NT8C, NT8D, NT8E, NT8F, NT8G, NT8H, NT8I, NT8J, NT8K, NT8L, NT8O, NT8P, NU3A, NU3B, NU3C, NU3D, NU3F, NU3G, NU3H, NU3J, NU3K, NU3L, NU3O, NU3P, NT5C4, NT5C5, NT5C6, NT5C7, NT5C8, NT5C9, NT5D4, NT5D5, NT5D6, NT5D7, NT5D8, NT5D9, NT6A4, NT6A5, NT6A6, NT6A7, NT6A8, NT6A9, NT6B4, NT6B5, NT6B6, NT6B7, NT6B8, NT6B9, NT6C4, NT6C5, NT6C6, NT6C7, NT6C8, NT6C9, NT6D4, NT6D5, NT6D6, NT6D7, NT6D8, NT6D9, NT7G2, NT7G3, NT7G5, NT7G6, NT7G8, NT7G9, NT7K2, NT7K3, NT7K5, NT7K6, NT7K8, NT7K9, NT7O2, NT7O3, NT7O5, NT7O6, NT7P1, NT7P2, NT7P3, NT7P4, NT7P5, NT7P6, NT8M1, NT8M2, NT8M3, NT8M4, NT8M5, NT8M6, NT8N1, NT8N2, NT8N3, NT8N4, NT8N5, NT8N6, NU2C1, NU2C2, NU2C3, NU2D1, NU2D2, NU2D3, NU2D5, NU2D6, NU2D8, NU2D9, NU2H2, NU2H3, NU3E1, NU3E2, NU3E3, NU3E5, NU3E6, NU3E8, NU3E9, NU3I2, NU3I3, NU3I6, NU3I9, NU3M3, NU3M6, NU3N1, NU3N2, NU3N3, NU3N4, NU3N5, NU3N6 |
| Greater Perth Note: For the frequency range 25.1 GHz–27 GHz | BV1I, BV1J, BV1K, BV1L, BV1M, BV1N, BV1O, BV1P, BV2I, BV2J, BV2M, BV2N, BV4A, BV4B, BV4C, BV4D, BV4E, BV4F, BV4G, BV4H, BV4I, BV4J, BV4K, BV4L, BV4O, BV5A, BV5B, BV5E, BV5F, BV5I, BV5J, BV7C, BV7G, BV1E7, BV1E8, BV1E9, BV1F7, BV1F8, BV1F9, BV1G7, BV1G8, BV1G9, BV1H7, BV1H8, BV1H9, BV2E7, BV2E8, BV2E9, BV2F7, BV2F8, BV2F9, BV4M1, BV4M2, BV4M3, BV4N1, BV4N2, BV4N3, BV4P1, BV4P2, BV4P3, BV4P4, BV4P5, BV4P7, BV4P8, BV5M1, BV5M2, BV5M3, BV5N1, BV5N2, BV5N3, BV7D1, BV7D2, BV7D4, BV7D5, BV7D7, BV7D8, BV7H1, BV7H2, BV7H4, BV7H5, BV7H7, BV7H8, BV7K1, BV7K2, BV7K3, BV7L1, BV7L2 |
| Greater Perth Note: For the frequency range 27 GHz–27.5 GHz | BV7G, BV7C4, BV7C5, BV7C6, BV7C7, BV7C8, BV7C9, BV7D4, BV7D5, BV7D7, BV7D8, BV7H1, BV7H2, BV7H4, BV7H5, BV7H7, BV7H8, BV7K1, BV7K2, BV7K3, BV7L1, BV7L2 BV1I, BV1J, BV1K, BV1L, BV1M, BV1N, BV1O, BV1P, BV2I, BV2J, BV2M, BV2N, BV4A, BV4B, BV4C, BV4D, BV4E, BV4F, BV4G, BV4H, BV4I, BV4J, BV4K, BV4L, BV5A, BV5B, BV5E, BV5F, BV5I, BV5J, BV1E7, BV1E8, BV1E9, BV1F7, BV1F8, BV1F9, BV1G7, BV1G8, BV1G9, BV1H7, BV1H8, BV1H9, BV2E7, BV2E8, BV2E9, BV2F7, BV2F8, BV2F9, BV4M1, BV4M2, BV4M3, BV4N1, BV4N2, BV4N3, BV4O1, BV4O2, BV4O3, BV4P1, BV4P2, BV4P3, BV5M1, BV5M2, BV5M3, BV5N1, BV5N2, BV5N3 |
| Hobart | LY9N, LY9I8, LY9I9, LY9J7, LY9J8, LY9J9, LY9K7, LY9K8, LY9M2, LY9M3, LY9M5, LY9M6, LY9M8, LY9M9, LY9O1, LY9O2, LY9O4, LY9O5, LY9O7, LY9O8, LZ3A2, LZ3A3, LZ3B1, LZ3B2, LZ3B3, LZ3C1, LZ3C2 |
| Melbourne/Ballarat | KX2L, KX3I, KX3J, KX3K, KX3L, KX3N, KX3O, KX3P, KX6B, KX6C, KX6D, KX6F, KX6G, KX6H, KX6J, KX6K, KX6L, LX1I, LX1M, LX1N, LX1O, LX4A, LX4B, LX4C, LX4E, LX4I, KX2G9, KX2H7, KX2H8, KX2H9, KX2K3, KX2K6, KX2K9, KX3E7, KX3E8, KX3E9, KX3F7, KX3F8, KX3F9, KX3G7, KX3G8, KX3G9, KX3H4, KX3H5, KX3H6, KX3H7, KX3H8, KX3H9, KX3M2, KX3M3, KX3M5, KX3M6, KX3M8, KX3M9, KX6A2, KX6A3, KX6A5, KX6A6, KX6A8, KX6A9, KX6E2, KX6E3, KX6E5, KX6E6, KX6E8, KX6E9, KX6I2, KX6I3, KX6I5, KX6I6, KX6I8, KX6I9, LX1E4, LX1E7, LX1E8, LX1E9, LX1J1, LX1J4, LX1J5, LX1J6, LX1J7, LX1J8, LX1J9, LX1K4, LX1K7, LX4F1, LX4F2, LX4F4, LX4F5, LX4F7, LX4F8, LX4J1, LX4J2, LX4J4, LX4J5, LX4J7, LX4J8 |
| Sydney/Bathurst | NW1, MV8G, MV9I, MV9J, MV9K, MV9L, MV9M, MV9N, MV9O, MV9P, MW3C, MW3D, MW3G, MW3H, MW3K, MW3L, MW3O, MW3P, MW6C, MW6D, NV4N, NV4O, NV4P, NV5M, NV5N, NV5O, NV5P, NV7B, NV7C, NV7D, NV7E, NV7F, NV7G, NV7H, NV7I, NV7J, NV7K, NV7L, NV7M, NV7N, NV7O, NV7P, MV8F3, MV8F6, MV8F9, MV8H4, MV8H5, MV8H6, MV8H7, MV8H8, MV8H9, MV8J3, MV8K1, MV8K2, MV8K3, MV8L1, MV8L2, MV8L3, MV9D6, MV9D9, MV9E4, MV9E5, MV9E6, MV9E7, MV9E8, MV9E9, MV9F4, MV9F5, MV9F6, MV9F7, MV9F8, MV9F9, MV9G4, MV9G5, MV9G6, MV9G7, MV9G8, MV9G9, MV9H3, MV9H4, MV9H5, MV9H6, MV9H7, MV9H8, MV9H9, MW3B2, MW3B3, MW3B5, MW3B6, MW3B8, MW3B9, MW3F2, MW3F3, MW3F5, MW3F6, MW3F8, MW3F9, MW3J2, MW3J3, NV4I5, NV4I6, NV4I8, NV4I9, NV4J4, NV4J5, NV4J6, NV4J7, NV4J8, NV4J9, NV4K4, NV4K5, NV4K6, NV4K7, NV4K8, NV4K9, NV4L4, NV4L5, NV4L6, NV4L7, NV4L8, NV4L9, NV4M2, NV4M3, NV4M5, NV4M6, NV4M8, NV4M9, NV5I4, NV5I5, NV5I6, NV5I7, NV5I8, NV5I9, NV5J4, NV5J5, NV5J6, NV5J7, NV5J8, NV5J9, NV5K4, NV5K5, NV5K6, NV5K7, NV5K8, NV5K9, NV5L4, NV5L5, NV5L6, NV5L7, NV5L8, NV5L9, NV7A2, NV7A3, NV7A4, NV7A5, NV7A6, NV7A7, NV7A8, NV7A9 |
| Albany | BW3P7, BW3P8, BW3P9, BW6D1, BW6D2, BW6D3, BW6D4, BW6D5, BW6D6, CW1M7, CW4A1, CW4A4 |
| Albury | LW8D, LW5P7, LW5P8, LW5P9, LW6M7, LW9A1, LW9A4, LW9A7 |
| Armidale | NU7G8, NU7G9, NU7K2, NU7K3 |
| Bendigo | KW9I5, KW9I6, KW9I8, KW9I9, KW9J4, KW9J5, KW9J6, KW9J7, KW9J8, KW9J9, KW9M2, KW9M3, KW9M5, KW9M6, KW9N1, KW9N2, KW9N3, KW9N4, KW9N5, KW9N6 |
| Bundaberg/Hervey Bay | NS8N, NS8O, NT2B, NT2C, NS8M2, NS8M3, NS8M5, NS8M6, NS8M8, NS8M9, NT2A2, NT2A3, NT2D7, NT2D8, NT2D9, NT2G1, NT2G2, NT2G3, NT2G4, NT2G5, NT2G6, NT2H1, NT2H2, NT2H3, NT2H4, NT2H5, NT2H6 |
| Cairns | LQ1O, LQ1P, LQ1K7, LQ1K8, LQ1K9, LQ1L7, LQ1L8, LQ1L9, LQ4C1, LQ4C2, LQ4C3, LQ4C4, LQ4C5, LQ4C6, LQ4D1, LQ4D2, LQ4D3, LQ4D4, LQ4D5, LQ4D6 |
| Coffs Harbour | NU9A, NU9E, NU8D9, NU8H3, NU8H6, NU8H9 |
| Forster/Tuncurry | NV5B6, NV5B9, NV5C4, NV5C5, NV5C7, NV5C8, NV5F3, NV5G1, NV5G2 |
| Launceston | LY6E, LY5H3, LY5H6, LY5H9, LY5L3, LY5L6, LY6F1, LY6F4, LY6F7, LY6I1, LY6I2, LY6I3, LY6I4, LY6I5, LY6I6, LY6J1, LY6J4 |
| Mackay | MR8A, MR5M7, MR5M8, MR5M9 |
| Margaret River | AV9P6, AV9P9, AW3D3, BV7M4, BV7M5, BV7M7, BV7M8, BW1A1, BW1A2 |
| Mildura | KW1A4, KW1A5, KW1A6, KW1A7, KW1A8, KW1A9, KW1E1, KW1E2, KW1E3 |
| Port Macquarie | NV2H, NV2L1, NV2L2, NV2L3 |
| Rockhampton | MS6F, MS6G, MS6B7, MS6B8, MS6B9, MS6C7, MS6C8, MS6C9, MS6J1, MS6J2, MS6J3, MS6K1, MS6K2, MS6K3 |
| Shepparton/Mooroopna | LW7F, LW7G1, LW7G4, LW7G7, LW7J1, LW7J2, LW7J3, LW7K1 |
| Townsville | LR2C, LR2D, LR2G, LR2H |
| Traralgon/Morwell | LX5A6, LX5A8, LX5A9, LX5B4, LX5B5, LX5B6, LX5B7, LX5B8, LX5B9, LX5C4, LX5C5, LX5C7, LX5C8, LX5F1, LX5F2, LX5F3, LX5G1, LX5G2 |
| Wagga Wagga | LW6B |
| Warrnambool | KX4F2, KX4F3, KX4F5, KX4F6, KX4F8, KX4F9, KX4G1, KX4G2, KX4G4, KX4G5, KX4G7, KX4G8 |

# Annex A: Amendment History

| DATE OF EFFECT | COMMENTS |
| --- | --- |
| 26 October 2020 | Embargo 69 revised Embargo 74 withdrawn Embarog 79 created |
| 22 July 2020 | Embargo 78 created Embargo 73 withdrawn |
| 1 July 2020 | Embargo 64 withdrawn Embargo 69 revised Embargo 74 revised |
| 27 March 2020 | Embargo 26 revised |
| 15 November 2019 | Embargo 42 revised Embargo 74 revised Embargo 75 revised Embargo 76 created |
| 2 August 2019 | Embargo 49 revised Embargo 71 withdrawn Embargo 72 revised |
| 16 May 2019 | Embargo 75 created |
| 16 April 2019 | Embargo 74 created |
| 14 February 2018 | Embargo 64 revised |
| 11 December 2017 | Embargo 73 created |
| 11 December 2017 | Embargo 72 created |
| 11 December 2017 | Embargo 71 revised |
| 18 August 2017 | Embargo 71 revised |
| 03 May 2017 | Embargo 71 created |
| 31 March 2017 | Embargo 67 revised |
| 9 November 2016 | Embargo 42 revised |
| 20 April 2016 | Embargo 70 created |
| 2 March 2016 | Embargo 69 created |
| 03 February 2016 | Embargo 26 revised Embargo 54 withdrawn Embargo 56 withdrawn |
| 12 January 2016 | Embargo 62 revised |
| 27 July 2015 | Embargo 49 revised |
| 19 December 2014 | Embargo 41 revised |
| 24 October 2014 | Embargo 66 withdrawn |
| 12 May 2014 | Embargo 68 created |
| 6 February 2014 | Embargo 55 withdrawn Embargo 67 created |
| 1 February 2014 | Embargo 26 revised Embargo 66 created |
| 19 September 2013 | Embargo 23 revised |
| 25 September 2012 | Embargo 26 revised Embargo 51 revised Embargo 53 revised Embargo 60 revised Embargo 65 created |
| 25 May 2012 | Embargo 23 revised |
| 24 April 2012 | Embargo 23 revised Embargo 26 revised Embargo 38 withdrawn Embargo 43 withdrawn |
| 6 December 2011 | Embargo 64 created Embargo 24 withdrawn Embargo 34 withdrawn |
| 29 August 2011 | Embargo 63 created |
| 19 July 2011 | Embargo 53 revised Embargo 54 revised Embargo 55 revised Embargo 56 revised |
| 21 January 2011 | Embargo 61 created Embargo 62 created Embargo 38 revised |
| 16 July 2010 | Embargo 60 created Embargo 50 revised |
| 22 June 2010 | Embargo 59 created |
| 30 April 2010 | Embargo 50 revised Embargo 51 revised Embargo 53 created Embargo 54 created Embargo 55 created Embargo 56 created Embargo 35 withdrawn Embargo 36 withdrawn |
| 14 September 2009 | Embargo 52 created Embargo 42 revised |
| 6 August 2009 | Embargo 51 revised Embargo 50 revised |
| 2 April 2009 | Embargo 51 created Embargo 50 created Embargo 49 created |
| 24 April 2009 | Embargo 48 created |
| 28 September 2007 | RALI completely updated All embargoes revised, with exception of the following: Embargo 22 withdrawn Embargo 25 withdrawn Embargo 28 withdrawn Embargo 33 withdrawn Embargo 47 created |
| 6 February 2007 | Embargo 46 created |
| 6 October 2006 | Embargo 45 created Embargo 37 withdrawn Embargo 40 withdrawn |
| 18 September 2006 | Embargo 44 created |
| 5 December 2005 | Embargo 41 revised |
| 27 October 2005 | Embargo 23 revised |
| 23 August 2005 | Embargo 23 revised Embargo 25 revised Embargo 26 revised |
| 27 June 2005 | Embargo 43 created |
| 27 May 2005 | Embargo 24 revised |
| 26 May 2005 | Embargo 42 created |
| 11 April 2005 | Embargo 41 created |
| 14 February 2005 | Embargo 38 revised |
| 8 February 2005 | Embargo 37 revised Embargo 31 withdrawn |
| 20 December 2004 | Embargo 38 revised |
| 17 December 2004 | Embargo 36 revised |
| 2 December 2004 | Embargo 40 created |
| 26 November 2004 | Embargo 39 created |
| 24 September 2004 | Embargo 38 revised |
| 30 June 2004 | Embargo 36 revised |
| 25 June 2004 | Embargo 38 revised |
| 26 March 2004 | Embargo 38 created |
| 16 March 2004 | Embargo 37 revised |
| 18 December 2003 | Embargo 36 created |
| 14 February 2003 | Embargo 35 created |
| 16 July 2002 | Embargo 32 created |
| 3 July 2002 | Embargo 34 created |
| 31 May 2002 | Embargo 23 revised |
| 22 March 2002 | Embargo 30 withdrawn |
| 19 December 2001 | Embargo 32 revised |
| 10 October 2001 | Embargo 23 revised |
| 26 September 2001 | Embargo 26 revised |
| 5 September 2001 | Embargo 33 created |
| 13 February 2001 | Embargo 23 revised |
| 11 November 2000 | Embargo 26 revised |
| 5 September 2000 | Embargo 32 created |
| 20 July 2000 | Embargo 26 revised |
| 20 July 2000 | Embargo 24 revised |
| 2 June 2000 | Embargo 26 revised |
| 4 May 2000 | Embargo 31 created |
| 22 February 2000 | Embargo 29 withdrawn |
| 24 January 2000 | Embargo 26 revised |
| 13 January 2000 | Embargo 24 revised |
| 12 January 2000 | Embargo 30 created |
| 21 December 1999 | Embargo 26 revised |
| 29 October 1999 | Embargo 26 revised Embargo 27 withdrawn |
| 30 July 1999 | Embargo 28 revised |
| 15 July 1999 | Embargo 11 withdrawn and RALI completely updated |
| 31 May 1999 | Embargo 23 revised |
| 18 December 1998 | Embargo 24 revised Embargo 25 revised |
| 31 August 1998 | Embargo 27 revised |
| 10 August 1998 | Embargo 29 created |
| 3 August 1998 | Embargo 28 created |
| 19 June 1998 | Embargo 11 revised |
| 8 May 1998 | Embargo 24 revised Embargo 26 revised |
| 10 March 1998 | Embargo 27 created |
| 7 November 1997 | Embargo 24 revised |
| 24 September 1997 | Embargo 23 revised |
| 3 September 1997 | Embargo 24 revised |
| 6 August 1997 | Embargo 18 revised Embargo 26 created |
| 14 February 1997 | Embargo 11 revised |
| 6 May 1994 | RALI completely updated |

# Annex B: Index of bands

| FREQUENCY BAND | EMBARGO NUMBER | COMMENT |
| --- | --- | --- |
| 5900–5950 kHz | 46 | Revised Sept 2007 |
| 5950–6200 kHz | 44 | Revised Sept 2007 |
| 7100–7300 kHz | 44 | Revised Sept 2007 |
| 7300–7350 kHz | 46 | Revised Sept 2007 |
| 9400–9500 kHz | 46 | Revised Sept 2007 |
| 9500–9900 kHz | 44 | Revised Sept 2007 |
| 11600–11650 kHz | 46 | Revised Sept 2007 |
| 11650–12050 kHz | 44 | Revised Sept 2007 |
| 12050–12100 kHz | 46 | Revised Sept 2007 |
| 13570–13600 kHz | 46 | Revised Sept 2007 |
| 13600–13800 kHz | 44 | Revised Sept 2007 |
| 13800–13870 kHz | 46 | Revised Sept 2007 |
| 15100–15600 kHz | 44 | Revised Sept 2007 |
| 15600–15800 kHz | 46 | Revised Sept 2007 |
| 17480–17550 kHz | 46 | Revised Sept 2007 |
| 17550–17900 kHz | 44 | Revised Sept 2007 |
| 18900–19020 kHz | 46 | Revised Sept 2007 |
| 21450–21850 kHz | 44 | Revised Sept 2007 |
| 25670–26100 kHz | 44 | Revised Sept 2007 |
| 45–52 MHz | 67 | Revised Mar 2017 |
| 56–70 MHz | 67 | Revised Mar 2017 |
| 85–87.5 MHz | 67 | Revised Mar 2017 |
| 70 MHz–25.25 GHz | 41 | Revised Dec 2014 |
| 137–144 MHz | 67 | Revised Mar 2017 |
| 168–174 MHz | 32 | Revised Sept 2007 |
| 403–403.9875 MHz | 50 | Revised April 2010 |
| 403–518 MHz | 51 | Revised Sept 2012 |
| 403–520 MHz | 71 | Revised Dec 2017 |
| 405.0125–406 MHz | 50 | Revised April 2010 |
| 406.1–408.6375 MHz | 53 | Revised Sept 2012 |
| 406.11875–406.61875 MHz | 19 | Revised Sept 2007 |
| 408.11875–408.61875 MHz | 19 | Revised Sept 2007 |
| 408.6375–409.04375 MHz | 60 | Revised Sept 2012 |
| 409.0375–410.5375 MHz | 50 | Revised April 2010 |
| 410.5375–412.4625 MHz | 53 | Revised Sept 2012 |
| 412.4625–413.4375 MHz | 50 | Revised April 2010 |
| 414.4625–415.5625 MHz | 50 | Revised April 2010 |
| 415.5625– 418.0875 MHz | 53 | Revised Sept 2012 |
| 415.56875–416.06875 MHz | 19 | Revised Sept 2007 |
| 417.56875–418.06875 MHz | 19 | Revised Sept 2007 |
| 418.0875–418.49375 MHz | 60 | Revised Sept 2012 |
| 418.4875–420 MHz | 50 | Revised April 2010 |
| 420–420.75 MHz | 50 | Revised April 2010 |
| 421.25–424.75 MHz | 50 | Revised April 2010 |
| 425.25–427.75 MHz | 50 | Revised April 2010 |
| 428.25–430 MHz | 50 | Revised April 2010 |
| 450–450.4875 MHz | 53 | Revised Sept 2012 |
| 450.050 MHz | 60 | Revised Sept 2012 |
| 452.5–457.50625 MHz | 53 | Revised Sept 2012 |
| 457.5–459.9875 MHz | 50 | Revised April 2010 |
| 462–467.50625 MHz | 53 | Revised Sept 2012 |
| 467.5–469.9875 MHz | 50 | Revised April 2010 |
| 469.99375–476.4125 MHz | 53 | Revised Sept 2012 |
| 477.41875–518 MHz | 53 | Revised Sept 2012 |
| 518–520 MHz | 45 | Revised Sept 2007 |
| 520–694 MHz | 71 | Revised Dec 2017 |
| 694–703 MHz | 71 | Revised Dec 2017 |
| 703–748 MHz | 26 | Revised Feb 2016 |
| 748–758 MHz | 71 | Revised Dec 2017 |
| 758–803 MHz | 26 | Revised Feb 2016 |
| 803–820 MHz | 71 | Revised Dec 2017 |
| 825–845 MHz | 26 | Revised Feb 2016 |
| 870–890 MHz | 26 | Revised Feb 2016 |
| 1427–1518 MHz | 70 | Created April 2016 |
| 1710–1785 MHz | 26 | Revised Feb 2016 |
| 1710–1785 MHz | 62 | Revised Jan 2016 |
| 1805–1880 MHz | 26 | Revised Feb 2016 |
| 1805–1880 MHz | 62 | Revised Jan 2016 |
| 1900–1920 MHz | 76 | Created Nov 209 |
| 1900–1980 MHz | 26 | Revised Feb 2019 |
| 1980–2010 MHz | 23 | Revised Sept 2013 |
| 1980–2010 MHz | 71 | Revised Dec 2017 |
| 2010–2110 MHz | 23 | Revised Sept 2013 |
| 2015–2100 MHz | 49 | Revised Aug 2019 |
| 2025–2110 MHz | 72 | Revised Aug 2019 |
| 2100–2130 MHz | 49 | Revised Aug 2019 |
| 2110–2170 MHz | 26 | Revised Feb 2016 |
| 2170–2200 MHz | 23 | Revised Sept 2013 |
| 2170–2300 MHz | 71 | Revised Dec 2017 |
| 2190–2280 MHz | 49 | Revised Aug 2019 |
| 2200–2290 MHz | 72 | Revised Aug 2019 |
| 2200–2300 MHz | 23 | Revised Sept 2013 |
| 2280–2310 MHz | 49 | Revised Aug 2019 |
| 2300–2302 MHz | 65 | Created Sept 2012 |
| 2302–2400 MHz | 26 | Revised Feb 2016 |
| 2500–2570 MHz | 26 | Revised Feb 2016 |
| 2570–2620 MHz | 26 | Revised Feb 2016 |
| 2620–2690 MHz | 26 | Revised Feb 2016 |
| 3400–3425 MHz | 68 | Created May 2014 |
| 3400–3575 MHz | 52 | Revised Nov 2009 |
| 3425–3442.5 MHz | 75 | Revised Nov 2019 |
| 3425–3492.5 MHz | 26 | Revised Feb 2016 |
| 3475–3492.5 MHz | 75 | Revised Nov 2019 |
| 3492.5–3542.5 MHz | 61 | Created Jan 2011 |
| 3542.5–3575 MHz | 26 | Revised Feb 2016 |
| 3575–3710 MHz | 42 | Revised Nov 2019 |
| 3600–3700 MHz | 52 | Revised Nov 2009 |
| 3700–4000 MHz | 78 | Created July 2020 |
| 5725–5850 MHz | 39 | Revised Sept 2007 |
| 5850–5925 MHz | 48 | Created April 2008 |
| 6700–7075 MHz | 49 | Revised Aug 2019 |
| 7100–7425 MHz | 71 | Revised Dec 2017 |
| 7135–7245 MHz | 49 | Revised Aug 2019 |
| 7250–7750 MHz | 47 | Created Sept 2007 |
| 7250–7750 MHz | 49 | Revised Aug 2019 |
| 7250–7750 MHz | 59 | Created June 2010 |
| 7900–8390 MHz | 49 | Revised Aug 2019 |
| 7900–8400 MHz | 47 | Created Sept 2007 |
| 7900–8400 MHz | 59 | Created June 2010 |
| 8390–8460 MHz | 49 | Revised Aug 2019 |
| 8460–8510 MHz | 49 | Revised Aug 2019 |
| 8540–8660 MHz | 49 | Revised Aug 2019 |
| 10700–14800 MHz | 49 | Revised Aug 2019 |
| 10.95–12.75 GHz | 47 | Created Sept 2007 |
| 13.75–14.50 GHz | 47 | Created Sept 2007 |
| 15349–15410 MHz | 49 | Revised Aug 2019 |
| 15430–15630 MHz | 49 | Revised Aug 2019 |
| 17200–21400 MHz | 49 | Revised Aug 2019 |
| 22200–22510 MHz | 49 | Revised Aug 2019 |
| 24250–27500 MHz | 69 | Revised Oct 2020 |
| 24750–25250 MHz | 49 | Revised Aug 2019 |
| 25100-27500 MHz | 26 | Revised Mar 2020 |
| 25100-30000 MHz | 79 | Created Oct 2020 |
| 25500–31000 MHz | 49 | Revised Aug 2019 |
| 33400–36000 MHz | 49 | Revised Aug 2019 |
| 37500–43500 MHz | 49 | Revised Aug 2019 |
| 47200–51400 MHz | 49 | Revised Aug 2019 |
| 52590–59300 MHz | 49 | Revised Aug 2019 |

# Annex C: Withdrawn Embargoes

| EMBARGO NUMBER | FREQUENCY BAND | COMMENT |
| --- | --- | --- |
| 31 | 3776–3800 kHz | REPLACED By provisions in the Australian Radiofrequency Spectrum Plan |
| 22 | 169.3875–169.7875 MHz | LIFTED |
| 15 | 402 MHz | LIFTED |
| 1 | 403–420 MHz | LIFTED |
| 2 | 403–520 MHz | LIFTED |
| 71 | 403–520 MHz | LIFTED |
| 11 | 410.75–412.25 MHz | LIFTED |
| 35 | 418.0875–420 MHz | SUPERSEDED by Embargo No 50 |
| 13 | 419.0–420.2 MHz | REPLACED by Embargo No 15 |
| 37 | 420–430 MHz | LIFTED |
| 40 | 450–520 MHz | LIFTED |
| 13 | 451.50–452.50 MHz | LIFTED |
| 3 | 451.95–453.40 MHz | LIFTED |
| 14 | 452.50–453.50 MHz | LIFTED |
| 55 | 452.50–453 MHz | LIFTED |
| 54 | 452.5–457.50625 MHz | LIFTED |
| 4 | 456.35–457.325 MHz | LIFTED |
| 36 | 458.3375–459.9375 MHz | SUPERSEDED by Embargo No 50 |
| 13 | 461–462 MHz | LIFTED |
| 3 | 461.45–462.90 MHz | LIFTED |
| 14 | 462–463 MHz | LIFTED |
| 55 | 462–462.5 MHz | LIFTED |
| 54 | 462.5–467.50625 MHz | LIFTED |
| 4 | 465.85–466.825 MHz | LIFTED |
| 36 | 467.8375–469.4375 MHz | SUPERSEDED by Embargo No 50 |
| 55 | 469.4875–469.9875 | LIFTED |
| 56 | 469.9875–476.4125 MHz | LIFTED |
| 5 | 472.025–472.60 MHz | LIFTED |
| 56 | 477.41875–484.79375 MHz | LIFTED |
| 56 | 485.19375–489.99375 MHz | LIFTED |
| 6 | 500–501 MHz | LIFTED |
| 18 | 501–505 MHz | REPLACED by Embargo No 26 |
| 7 | 505.5–507 MHz | LIFTED |
| 6 | 510–511 MHz | LIFTED |
| 18 | 511–515 MHz | REPLACED by Embargo No 26 |
| 7 | 515.5–517.0 MHz | LIFTED |
| 71 | 520–694 MHz | LIFTED |
| 71 | 694–703 MHz | LIFTED |
| 71 | 748–758 MHz | LIFTED |
| 71 | 803–820 MHz | LIFTED |
| 64 | 803–825 MHz | LIFTED |
| 8 | 820–960 MHz | LIFTED |
| 29 | 820–825 MHz | LIFTED |
| 64 | 845–870 MHz | LIFTED |
| 34 | 857–859 MHz 861–865 MHz 933–935 MHz | SUPERSEDED By Embargo 64 |
| 29 | 865–870 MHz | LIFTED |
| 64 | 890–960 MHz | LIFTED |
| 21 | 1427–1535 MHz | REPLACED by restrictions in 1.5 GHz Band Plan, December 1996 |
| 20 | 1700–1900 MHz | REPLACED by Embargo No 23 |
| 38 | 1785–1805 MHz | LIFTED |
| 38 | 1900–1920 MHz | LIFTED |
| 63 | 1960–1920 MHz | LIFTED |
| 71 | 1980–2110 MHz | LIFTED |
| 38 | 2010–2025 MHz | REPLACED by Embargo No 23 |
| 9 | 2076–2111 MHz | LIFTED |
| 71 | 2170-2300 MHz | LIFTED |
| 9 | 2300–2400 MHz | LIFTED |
| 43 | 2500–2690 MHz | REPLACED by Embargo No 26 |
| 49 | 3400–4200 MHz | Revised Aug 2019 |
| 27 | 3425–3442 MHz | REPLACED by Embargo No 26 |
| 27 | 3475–3492 MHz | REPLACED by Embargo No 26 |
| 72 | 3575–4200 MHz | Revised Aug 2019 |
| 72 | 4500–4800 MHz | Revised Aug 2019 |
| 72 | 5091–5250 MHz | Revised Aug 2019 |
| 49 | 5850-6700 MHz | Revised Aug 2019 |
| 72 | 5850–7075 MHz | Revised Aug 2019 |
| 71 | 7100–7425 MHz | LIFTED |
| 30 | 7250–7375 MHz | LIFTED |
| 72 | 7250–7750 MHz | Revised Aug 2019 |
| 72 | 7900–8400 MHz | Revised Aug 2019 |
| 72 | 10.7–11.7 GHz | Revised Aug 2019 |
| 33 | 11.7–12.2 GHz | REPLACED By provisions in the Australian Radiofrequency Spectrum Plan |
| 72 | 12.2–13.25 GHz | Revised Aug 2019 |
| 10 | 12.75–13.25 GHz | LIFTED |
| 16 | 12.75–13.27 GHz | LIFTED |
| 72 | 13.75–14.8 GHz | Revised Aug 2019 |
| 17 | 14.5–15.35 GHz | LIFTED |
| 72 | 15.43–15.63 GHz | Revised Aug 2019 |
| 72 | 17.3–21.2 GHz | Revised Aug 2019 |
| 25 | 18.8–19.3 GHz | LIFTED |
| 24 | 24.5–26.5 GHz | LIFTED |
| 26 | 26.5–27.5 GHz | Revised February 2016 |
| 72 | 24.65–25.25 GHz | Revised Aug 2019 |
| 72 | 27–31 GHz | Revised Aug 2019 |
| 66 | 27.5–28.35 GHz | LIFTED |
| 74 | 27.5–28.3 GHz | LIFTED |
| 24 | 28.6–29.1 GHz | LIFTED |
| 24 | 29.1–29.5 GHz | LIFTED |
| 72 | 33.4–36 GHz | Revised Aug 2019 |
| 72 | 37.5–43.5 GHz | Revised Aug 2019 |
| 28 | 40.5–43.5 GHz | LIFTED |
| 72 | 47.2–50.2 GHz | Revised Aug 2019 |
| 72 | 50.4–51.4 GHz | Revised Aug 2019 |
| 73 | 3710—3790 MHz | Replaced by Embargo 78 July 2020 |

Diagrams illustrating these areas can be found in Appendix F of the Apparatus Licence Fee Schedule at:
https://www.acma.gov.au/theACMA/About/Making-payments/Apparatus-licence-fees/apparatus-licence-fees-acma

GDA94 datum

Stations as defined in Article 1, ITU Radio Regulations (2004).

As defined in the ACMA Apparatus Licence Fee Schedule - www.acma.gov.au/theACMA/About/Making-payments/Apparatus-licence-fees/apparatus-licence-fees-acma

Available at www.acma.gov.au/theACMA/ifc-112010-the-way-ahead-decisions-and-implementation-options-for-the-400-mhz-band

FYSO 2012-2026 is available at: www.acma.gov.au/Industry/Spectrum/Five-Year-Spectrum-Outlook/About-the-five-year-outlook/fiveyear-spectrum-outlook

https://www.acma.gov.au/theACMA/future-approach-to-the-3_6-ghz-band