3.4 GHz urban excise area 
coexistence studies

Spectrum planning report 01/2021

May 2021

Canberra

Red Building 
Benjamin Offices
Chan Street 
Belconnen ACT

PO Box 78
Belconnen ACT 2616

T	+61 2 6219 5555
F	+61 2 6219 5353

Melbourne

Level 32 
Melbourne Central Tower
360 Elizabeth Street 
Melbourne VIC

PO Box 13112
Law Courts 
Melbourne VIC 8010

T	+61 3 9963 6800
F	+61 3 9963 6899

Sydney

Level 5 
The Bay Centre
65 Pirrama Road 
Pyrmont NSW

PO Box Q500
Queen Victoria Building 
NSW 1230

T	+61 2 9334 7700
F	+61 2 9334 7799

Copyright notice

https://creativecommons.org/licenses/by/4.0/

With the exception of coats of arms, logos, emblems, images, other third-party material or devices protected by a trademark, this content is made available under the terms of the Creative Commons Attribution 4.0 International (CC BY 4.0) licence.

We request attribution as © Commonwealth of Australia (Australian Communications and Media Authority) 2021.

All other rights are reserved.

The Australian Communications and Media Authority has undertaken reasonable enquiries to identify material owned by third parties and secure permission for its reproduction. Permission may need to be obtained from third parties to re-use their material.

Written enquiries may be sent to:

Manager, Editorial Services
PO Box 13112
Law Courts
Melbourne VIC 8010
Email: info@acma.gov.au

Introduction	5

Background	6

Study parameters	7

nbn fixed wireless broadband base station parameters	7

nbn CPE receiver parameters	8

New wireless broadband base station parameters	9

Terrain and propagation modelling	10

Study methodology	12

Studies on deploying new services in urban excise areas	12

Simulation implementation	13

Study of existing deployments in the 3.4 GHz band	14

Results	15

Studies on deploying new services in urban excise areas	15

ACT results	17

NSW results	19

Queensland results	21

SA results	23

Victoria results	25

WA results	27

Study of existing deployments in the 3.4 GHz band	29

Discussion	36

Discussion of new services in urban excise studies	36

Overall observations	36

ACT	38

NSW	39

Queensland	39

SA	40

Victoria	41

WA	42

Discussion on existing service deployments	43

Analysis of macro cell deployments on the RRL	43

Impact on macro cell deployments within urban excise areas	43

Summary	44

Appendix A: Urban excise areas	46

# Introduction

As part of the process to optimise arrangements in the 3400–3575 MHz band, the ACMA announced plans to excise 75 MHz of unused spectrum in urban areas from nbn’s existing licence holdings (referred to as ‘urban excise’). There are a variety of options available for the use of this excised spectrum. To determine which approach best meets spectrum management objectives, an assessment of the technical coexistence issues for uses/users in these urban excise areas with surrounding areas and adjacent spectrum is required.

The objective of this report is to assess the potential interference from wireless access services in the urban excise areas to nbn’s existing fixed broadband service areas. The use of both a ‘macro cell’ model and a ‘restricted deployment’ model (also referred to as a small cell model) within urban excise areas are considered in the analysis performed.

The studies included in this paper are intended to inform:

Views on the utility of the urban excise and associated risk of interference from the macro-cell and restricted deployment scenarios.

The development of interference management criteria for the urban excise. This includes the development of specific parameters for the different deployment models to manage interference to and from services deployed in urban excise areas.

# Background

In November 2019, the ACMA finalised its review of arrangements in the 
3400–3575 MHz band with the release of the Optimising arrangements for the 
3400–3575 MHz band: Planning decisions and preliminary views outcomes paper. One of the key announcements in the paper was plans to excise 75 MHz of unused spectrum, from 3400–3475 MHz, in urban areas from nbn’s existing 3.5 GHz licence holdings (referred to as ‘urban excise’). Implementation of this outcome is pending conversion of nbn apparatus licences in the 3.4 GHz band to spectrum licences and subsequent defragging of nbn’s spectrum holdings.

A definition of the urban excise areas is provided in Appendix A of this paper.

In the outcomes paper, we also indicated we would work with stakeholders to determine criteria to manage interference to and from urban excise areas. Once determined, the ACMA will use these criteria to examine the utility of the spectrum. If the urban excise can be reasonably used, an appropriate licensing approach to make it available for use by other wireless broadband operators would then be determined.

When developing measures to manage interference to and from urban excise areas, the ACMA has identified the following desired outcomes:

Provide sufficient protection to existing nbn services and ensure they are not unreasonably constrained in their ability to deploy new (and more spectrally efficient) technologies in the future.

Maximise the utility of urban excise areas for new wireless broadband services.

Minimise the impact any new measures have on existing 3.4 GHz spectrum licensees (other than nbn spectrum licences in the 3400–3475 MHz band) occupying areas directly adjacent to urban excise areas.

# Study parameters

This section summarises parameters used in the coexistence studies in this paper.

## nbn fixed wireless broadband base station parameters

Parameters used for the nbn base stations are a combination of information from the Register of Radiocommunications Licences (RRL) and from information provided to the 3.4 GHz TLG by nbn. Parameters used in studies are summarised in Table 1.

The base stations used in the study have been restricted to those inside the nbn provided service areas that are within 64 km of urban excise area boundaries. nbn base station locations (including sector azimuths) and service areas used in the study are detailed in the embedded files below. nbn base stations are used to determine the wanted signal level to nbn consumer premises equipment (CPE).

(Please ensure you have the correct software/programs to open the embedded files below.)

nbn base station parameters

| Parameter | Value | Note |
| --- | --- | --- |
| Antenna gain | From RRL data Values: [17.5, 19.6, 20.1, 25] dBi | If a base station does not have a value recorded an antenna gain of 17.5 dBi is used. This provides a worst-case scenario for C/N calculations. |
| Antenna pattern | ITU-R Rec F.1336 | Antenna parameters used in ITU-R F.1336 are those defined in Table 4 of ITU-R Report M.2292. |
| Antenna tilt | Network config document | nbn deployment parameter Sum of mechanical tilt and electrical tilt. |
| Antenna height | network config document | nbn deployment parameter. |
| Antenna azimuth | network config document | nbn deployment parameter. |
| Tx power | 9 dBW/MHz | From RRL data 160 W/20MHz. |
| Losses | 0.5 dB | From RRL data. |
| Equivalent isotropic radiated power | [26, 28.1, 28.6, 33.5] dBW/MHz | Calculated from T x power, antenna gain values (from RRL data) and losses. |
| Bandwidth | 20 MHz | Based on RRL registrations (normalised to 1 MHz for study). |

## nbn CPE receiver parameters

The CPE receiver parameters were provided by nbn along with the receive antenna pattern. These are summarised in Table 2.

(Please ensure you have the correct software/program to open the embedded file below.)

The receive antenna pattern provided by nbn consists of azimuth and elevation pattern slices for horizontal and vertical polarisations. Worst-case azimuth and elevation patterns of the 2 polarisations were identified. These were then used to develop a cross-weighted approximate 3D radiation pattern which was modelled in MATLAB R2019b and imported into Visualyse Professional v7.979.

Notional CPE parameters

| Parameter | Value | Note |
| --- | --- | --- |
| Antenna gain | 20 dBi | Antenna gain for nbn B42 CPE. |
| Antenna pattern | - | Worst-case azimuth and elevation polarisation patterns of nbn CPE antenna pattern used to generate 3D antenna pattern. |
| Antenna tilt | Point to closest BS |  |
| Antenna height | 5 m | Typical deployment scenario, can be greater for few non-standard installs. |
| Antenna azimuth | Point to closest BS |  |
| Noise figure | 7 dB | Representative noise figure. |
| Losses | 1 dB | Representative value. |
| Bandwidth | 20 MHz | Based on RRL registrations (normalised to 1 MHz for study). |
| Target C/(N+I) | 13 dB | Representative value provided by nbn. |
| Cell radius | Max 15 km | Based on nbn deployments. |

## New wireless broadband base station parameters

The parameters used in the studies for macro base stations (BS) and outdoor small cells in the urban excise areas are summarised in Table 3 and Table 4 respectively. Base station locations used in studies are detailed in the embedded file below (refer to study methodology for how these were determined).

(Please ensure you have the correct software/program to open the embedded file below.)

New macro base station parameters

| Item | Parameter | Value | Note |
| --- | --- | --- | --- |
| A | Antenna gain | Case 1 & 3: 23.1 dBi Case 2 & 4: 24.8 dBi | Case 1: Dependent on predefined AAS antenna pattern in Visualyse Professional. Case 2: Typical gain, based on RRL data for 3.4 GHz band. |
| B | Antenna pattern | Case 1: ITU-R M.2101 Case 2: Omnidirectional | Case 1 used the Visualyse Professional IMT Model BS Suburban antenna pattern (based on ITU-R M.2101). Each site had 3 sectors. |
| C | Base station horizontal coverage range relative to azimuth | Case 1: ±60° Case 2: 360° (omnidirectional) | Case 1: Table 6, Attachment 4.5 WP 5D Chair report of 36th meeting (5D/360). AAS beam is randomised in horizontal direction between -60° to 60° from antenna pointing azimuth. |
| D | Base station vertical coverage range (degrees) | Case 1: 90-100° Case 2: 180° (omnidirectional) | Case 1: Table 6, Attachment 4.5 WP 5D Chair report of 36th meeting (5D/360). AAS beam is randomised in vertical direction between 90° to 100°. |
| E | Antenna height | 25 m | Macro suburban, Table 4, ITU-R Report M.2292. |
| F | Total radiated power (TRP) | 5.23 dBW/MHz | From typical RRL data for IMT BS in 3.6 GHz band 200 W/60 MHz. |
| G | Losses | 0 dB | It is assumed ohmic losses are included in TRP value. |

New outdoor small cell parameters

| Item | Parameter | Value | Note |
| --- | --- | --- | --- |
| A | Antenna gain | 5 dBi | Outdoor small cell, Table 4, ITU-R Report M.2292 |
| B | Antenna pattern | Omnidirectional | Assumed |
| C | Antenna height | 6 m | Outdoor small cell, Table 4, ITU-R Report M.2292 |
| D | Total radiated power (TRP) | -6 dBW/20MHz (-19 dBW/MHz) | Outdoor small cell, Table 4, ITU-R Report M.2292 |
| E | Losses | 0 dB | Assumed |

## Terrain and propagation modelling

The studies were completed using the Visualyse Professional v7.979 modelling tool using digital elevation model (DEM) terrain data from the Shuttle Radar Topography Mission (SRTM). The SRTM DEM used provides for 3 second (approximately 
90 m) resolution.

The propagation models used to model wanted and unwanted signals are detailed in Table 5. Clutter loss is used to model propagation from unwanted signals in Case 3.

Propagation models

| Parameter | Value | Note |
| --- | --- | --- |
| Propagation Model | ITU-R Rec P.525, ITU-R Rec P.526-13 | Used for all cases |
| Clutter loss | ITU-R Rec P.2108 | Used for Case 3 only. Used to model propagation losses from unwanted signals only. The statistical clutter model was used with a random % of locations applied. Clutter loss is only applied at the restricted cell Tx. |

# Study methodology

To assess potential interference from, and utility of, urban excise the following studies were conducted:

Studies on deploying new services in urban excise areas: These studies assess potential interference into nbn services from new wireless broadband services deployed within urban excise areas.

Study of existing deployments in the 3.4 GHz band: This study analyses existing deployments by nbn and Optus near the boundary of their area adjacent 3.4 GHz spectrum licences. The intention is to assess how the current technical framework affects deployments close to shared licence boundaries as well as typical separation distances between services in adjacent areas. This serves as a reference to compare the results of studies on new services in urban excise areas.

## Studies on deploying new services in urban excise areas

For the purposes of the studies in this paper, the operation of services deployed within urban excise areas are assumed to be synchronised with nbn services. For this reason, the main interference mechanism that needs to be considered is interference from new wireless broadband base stations (BS) operating in urban excise areas into nbn CPEs.

To investigate the potential interference from new services in urban excise areas, the following cases were modelled:

Case 1: A large network of AAS macro base stations, with sectored antennas and random beam pointing directions.

Case 2: 2 to 5 new macro base stations, with omnidirectional antennas, located in centre of urban excise areas.

Case 3: A large network of outdoor small cells within urban excise areas, with omnidirectional antennas, height 6m.

Case 4: 5 to 6 outdoor small cells, with omnidirectional antennas, height 6 m, located close to nbn service areas, modelled without clutter loss.

The new base station locations used in studies are based on RRL data of existing stations in the 3.6 GHz band as follows:

For Cases 1 and 3, a number of base station locations were identified either across an entire city or within a defined area (depending on the size of the urban excise area). This was done to understand the impact of interference from a network of BS into the nbn service area. Registered 3.5 GHz BS locations on the RRL for Optus were used as this represented a middle ground on network deployment density for existing operators. For each city, only sites greater than 5 km from the urban excise boundaries were used. To further reduce the number of BS (and therefore simulation complexity) in Sydney and Melbourne, only those BS within defined HCIS were considered. Where specific HCIS were used, these were chosen to cover a encompass a number of suburbs (with ≈90 base stations) that are close to the most populated areas outside of the urban excise area.

For Case 2, locations of 3.4 GHz spectrum licensee Mobile JV’s base stations were filtered out in increasing 5 km increments from the nbn service areas until 2-5 sites remain. This was to give an indication of the potential interference from new BS at a reasonable distance from the boundary of the urban excise areas. Mobile JV sites were used as there were fewer registrations close to the boundary to 
choose from.

For Case 4, locations of 3.4 GHz spectrum licensee Mobile JV’s base stations within 5-10 km of the nbn service area were identified. Five to 6 sites were chosen so they were spread along the boundary of the urban excise areas. This was to give an indication of the potential interference from a new BS at several locations within close proximity to nbn service areas. Mobile JV sites were used as there were fewer registrations close to the boundary to choose from.

For each of the cases detailed above, the carrier to noise plus interference (C/(N+I)) ratio of the nbn fixed wireless service links (base stations to CPE) is calculated in the presence of new wireless broadband base stations operating inside urban excise areas. A Monte Carlo simulation was used to model interference in each case.

### Simulation implementation

The Monte Carlo simulation was implemented using Visualyse Professional v7.979. This included modelling existing nbn fixed wireless base stations within nbn service areas (including individual sectors). A nbn CPE was randomly distributed at 10,000 test locations around the nbn base stations with a new location for each timestep of the simulation. At each timestep the nbn CPE antenna points towards the nearest base station. The maximum distance between a CPE test location and the nearest base station is 15 km.

New wireless broadband base stations were modelled as follows:

- Case 1: A network of new wireless broadband stations is modelled. Each site has 3 sectors with advanced antenna systems (AAS) and beamforming is assumed. Each beam is randomly steered in the azimuthal and elevation plains. Only interference from antennas using beamforming is assessed.

- Case 2: 2 to 5 new wireless broadband stations are modelled using omni directional antennas. This assumes the maximum gain of an interfering base station antenna is always pointing at a nbn CPE. This was done to show locations where possible failures could occur under a worst-case scenario.

- Case 3: A network of new wireless broadband stations is modelled. Each site has a small cell base station at a height of 6m with an omni direction antenna pattern located at it. Random values for clutter loss are modelled between each new wireless broadband station and the nbn CPE.

- Case 4: 5 to 6 new wireless broadband stations are modelled. Each site has a small cell base station at a height of 6m with an omni direction antenna pattern located at it. Clutter loss is not applied in this case. This was done to show locations where possible failures could occur under a worst-case scenario.

For each of the case above, at each timestep of the simulation, the carrier to noise (C/N), C/(N+I) ratio at each CPE location as well as its geographic coordinates are recorded. The carrier strength (C) is the strength of the signal from a nbn base station to the CPE. The interference (I) recorded is the aggregate interference into the CPE from all new wireless broadband base stations modelled for a particular case.

Post-processing of the simulation data is conducted in MATLAB R2019b. Using the provided service areas from nbn, test locations not within the service areas are filtered out and removed. Invalid test locations where the recorded C/N fail the C/(N+I) target are removed. These invalid locations represent situations where in the absence of interference the link between the CPE and the nbn fixed wireless station failed and therefore would not have been used. Examples of these can be locations where the links are blocked by terrain.

## Study of existing deployments in the 3.4 GHz band

This study analyses existing deployments by nbn and Optus near the boundary of their area adjacent spectrum licences in the 3.4 GHz band. The intention is to assess how the current technical framework affects deployments close to shared licence boundaries as well as typical separation distances between services in adjacent areas. To do this the following was done:

The existing spectrum licence boundaries between nbn and Optus were identified. These were then overlayed with the urban excise area boundaries to provide a visual comparison of the 2 areas.

2 images were then created for each urban excise area:

In the first image, stations registered on the RRL under nbn and Optus 3.4 GHz spectrum licences were plotted. Contours in 5 km increments (out to 20 km) was incorporated on either side of the shared boundary between existing nbn and Optus 3.4 GHz spectrum licences. This was done to assist in gauging the distance services are deployed from the licence boundary as well as the distance between services located on either side of the boundary. The azimuths of some sectorised base stations within 10km of the boundary are also provided for information.

In the second image, the location of base stations operating under nbn’s 3.5 GHz PTS licences are plotted. A buffer zone in 5 km increments (out to 20 km) was incorporated on either side of the urban excise area boundary. This was done to assist in gauging the distance services are deployed from the area boundary as well as the size of urban excise areas.

# Results

## Studies on deploying new services in urban excise areas

A summary of the simulation results for each study case can be found in the Table 6. Full study results are also provided in the embedded files below in ‘csv’ and ‘kml’ formats. These represent the results after removing CPE simulation locations that are outside the nbn service areas and locations where the C/N fails the C/(N+I) target defined in Table 2. Pictorial and graphical representations of the results for each area are provided in the Figures 1–18.

It is noted that the term ‘failed sites’ is used when presenting results. A ‘failed site’ refers to the exceedance of the C(/N+I) criteria for an individual timestep of the simulation. For Cases 1 and 3 it is possible for a specific test site to fail one timestep but pass another. This is due to the dynamic nature of the AAS beam steering used for Case 1 and the random clutter values used for Case 3. For these cases the results show the percentage of timesteps a failure occurs. For Cases 2 and 4, if a site fails for one timestep it will fail for any future timesteps. This is because all relevant parameters are static. For these cases the results show the percentage of locations that fail.

In the KMLs embedded below, placemark descriptions are the C/(N+I) ratio at the location. For pass locations the name of the point is the C/(N+I) level above the target and for fail locations the name is the C/(N+I) level below the target.

(Please ensure you have the correct software/programs to open the embedded files below.)

Summary of study results

|  |  | Case 1 | Case 2 | Case 32 | Case 4 |
| --- | --- | --- | --- | --- | --- |
| ACT | Number of CPE locations after filtering | 3669 | 3742 | 3742 | 3742 |
|  | Percentage of failed sites | 0.87% | 0.13% | 0% | 0% |
| NSW | Number of CPE locations after filtering | 5508 | 5583 | 5583 | 5583 |
|  | Percentage of failed sites | 1.7% | 0.61% | 0 | 0.2% |
| Qld | Number of CPE locations after filtering | 6554 | 6439 | 6439 | 6439 |
|  | Percentage of failed sites | 2.18% | 0.92% | 0% | 0% |
| SA | Number of CPE locations after filtering | 7370 | 7352 | 7352 | 7352 |
|  | Percentage of failed sites | 1.56% | 4.18% | 0% | 0% |
| Vic | Number of CPE locations after filtering | 7010 | 6884 | 6884 | 6884 |
|  | Percentage of failed sites | 2.34% | 3.27% | 0% | 0% |
| WA | Number of CPE locations after filtering | 5815 | 5887 | 5887 | 5887 |
|  | Percentage of failed sites | 3.96% | 4.77% | 0% | 0.1% |

### ACT results

Failed sites for Cases 1-4, amount C/(N+I) criteria is exceeded (dB)

CDF of amount C/(N+I) criteria is exceeded for Cases 1-4

Scatter plot of C/N vs C/(N+I) for Cases 1-4

### NSW results

Failed sites for Cases 1-4, amount C/(N+I) criteria is exceeded (dB)

CDF of amount C/(N+I) criteria is exceeded for Cases 1-4

Scatter plot of C/N vs C/(N+I) for Cases 1-4

### Queensland results

Failed sites for Cases 1-4, amount C/(N+I) criteria is exceeded (dB)

CDF of amount C/(N+I) criteria is exceeded for Cases 1-4

Scatter plot of C/N vs C/(N+I) for Cases 1-4

### SA results

Failed sites for Cases 1-4, amount C/(N+I) criteria is exceeded (dB)

CDF of amount C/(N+I) criteria is exceeded for Cases 1-4

Scatter plot of C/N vs C/(N+I) for Cases 1-4

### Victoria results

Failed sites for Cases 1-4, amount C/(N+I) criteria is exceeded (dB)

CDF of amount C/(N+I) criteria is exceeded for Cases 1-4

Scatter plot of C/N vs C/(N+I) for Cases 1-4

### WA results

Failed sites for Cases 1-4, amount C/(N+I) criteria is exceeded (dB)

CDF of amount C/(N+I) criteria is exceeded for Cases 1-4

Scatter plot of C/N vs C/(N+I) for Cases 1-4

## Study of existing deployments in the 3.4 GHz band

The results of the analysis of existing deployments are provided in this section. The legend for the figures in this section is provided below.

Existing 3.4 GHz deployments in and around Canberra (ACT)

Existing 3.4 GHz deployments in and around the Sydney (NSW)

Existing 3.4 GHz deployments in and around Brisbane (Qld)

Existing 3.4 GHz deployments in and around Adelaide (SA)

Existing 3.4 GHz deployments in and around Melbourne (Vic)

Existing 3.4 GHz deployments in and around Perth (WA)

# Discussion

This section discusses the results of the studies conducted. This includes analysis of the results for each area as well as general observations.

## Discussion of new services in urban excise studies

### Overall observations

#### Macro cell studies

Case 1 only modelled base stations using beamforming. It is expected that different results would apply if either the antennas were not operating in beamforming mode or passive antennas (with fixed tilt) were modelled. However, for purposes of this report, it was assumed that the main use case for macro cells in the 3.4 GHz band will be AAS implementing beamforming.

The analysis shows that the CPE sites where the C/(N+I) criteria is most likely to be exceeded are those where:

- the CPE antenna is pointed in the direction of the urban excise area, particularly if it is located on hill/mountain

- the CPE antenna is located close to the urban excise boundary

- there are terrain obstructions between the CPE antenna and nbn base station that reduce the starting C/N.

Most clusters of failure sites, including sites with the largest failure margin, tended to associate with (1) above. This includes cases where the nbn CPE is higher above sea level than the AAS macro base station.

A number of failure sites appear to be located in within national parks, oceans, lakes and within major suburbs or towns. It is possible that many of these are not areas nbn CPEs would be deployed. A more accurate nbn service area description, that removes these areas, would be required if a macro cell approach is adopted.

Brisbane, Canberra, Sydney were the only areas where Case 2 results had a lower percentage of failures than Case 1. This is likely due to these urban excise areas being larger than in other cities. Consequently, new macro cells were located further form the boundary in these cases.

A more detailed analysis of the macro-cell case for each urban excise area is provided later in this section.

Study results suggest it would be possible to deploy a macro cell network in Canberra if some minor mitigation measures are employed to protect nbn services. There may also be an opportunity to deploy a small macro cell network in Sydney with some mitigation measures employed. Though this would likely be limited to sites some distance from the urban excise boundary. Studies suggest that it would be difficult to deploy a macro cell network in other capital cities without significant mitigation measures being employed. This would then start to represent a form of restricted cell model.

#### Restricted cell studies

Case 3 modelled interference from a network of low powered cells. A random value for clutter loss was incorporated into the unwanted signals propagation losses from each transmitting cell site.

Analysis of Case 3 showed no site failure. The lower EIRPs coupled with the large range of clutter loss values applied by using ITU-R Recommendation P.2108, as shown in Figures 25 and 26, are the likely reasons there were no site failures. While such a large range of clutter loss values may apply for mobile services (located close to the ground), it is uncertain whether such large values are reasonable to assume for base stations located 6 m above the ground. Further advice from industry will be sought on this point.

Case 4 modelled interference from small number of low powered cells. No clutter loss was assumed in this case. The results of studies showed one site failure outside the Sydney urban excise area and 7 outside the Perth urban excise area – though half of these appear to fall within national parklands. These results suggest that there is potential for a restricted cell model to cause interference in nbn service areas when there is little to no clutter loss in certain cases.

The analysis shows that the CPE sites where the C/(N+I) criteria is most likely to be exceeded are the same as those identified for the macro cell case. Similarly, the worst cases tend to be when nbn CPEs are much higher above sea level than the restricted cell base stations.

It is noted that the amount the C/(N+I) criteria is exceeded in sites that fail is typically low. For sites that are not within national parks, 3 fail by less than 0.5 dB. However, one site failed by 9.2 dB outside the Perth urban excise area.

The restricted cell studies assumed an omni directional antenna for all base stations. This is considered to provide a worse case result as no antenna is perfectly omni directional. For example, antennas with lower gains above the horizon may result in fewer sites that fail.

Given the small percentage of sites that failed, coupled with the typically low exceedance of the C/(N+I) criteria, it may be possible for a restricted cell model to be developed (with suitable conditions) that present a low risk of interference to nbn services.

CDF of random clutter loss values used in Case 3 (for Sydney study)

Histogram of random clutter loss values used in Case 3 
(for Sydney study)

### ACT

#### Macro cell studies

Out of all urban excise areas, the Canberra area had the lowest percentage of failures. This is likely due to the size of the area, the mountainous terrain and the reasonably large separation between the city and suburbs of Canberra and the nbn service areas.

The only sites where the C/(N+I) criteria was exceeded are located to the north of Canberra where there is limited terrain shielding. The analysis shows that the most at risk nbn sites are those where the CPE antenna is pointed in the direction of Canberra and located on a hill/mountain.

0.87% of Case 1 timesteps (i.e., 32 timesteps) failed. The largest group of failures were for sites located near the tops of hills around Bango (NSW). A total of 87.5% of these failed by a margin less than 13 dB, but margins as large as 23.5 dB were recorded. Failures within a 13 dB range will result in a positive C/(N+I) at the CPE, which may mean data could still be sent (albeit at a lower data rate).

For Case 2, only 5 sites failed. Failure margins were less than 1.7 dB.

The Case 1 and Case 2 study results suggest a reasonable ability to deploy macro cells within Canberra, particularly in central and southern Canberra. This could be done be taking advantage of a mixture of terrain shielding and various deployment mitigation measures to manage interference. However, there may be some sites where significant restrictions will apply or where macro cells cannot be deployed, particularly on hills with limited terrain shielding with the norther nbn service area.

The nbn service areas used in this study are considered broadly representative at this stage. However, it is noted they cover parts of the ocean, parkland (i.e., national and state parks) where it would seem unlikely services would be deployed. Also, they encompass some reasonably sized towns and suburbs of cities where it fixed line solutions may be more likely to be deployed. A reasonable portion of the total failure sites appear to fall within these locations. A more accurate nbn service area description, that removes these areas, would be required if a macro cell approach is adopted.

### NSW

#### Macro cell studies

It is noted that the macro cell network only covered a portion of the Sydney urban excise area. This portion was chosen as it covered a significant amount of Sydney suburbs and was closest to the most populated areas outside of the urban excise area. Different results could be obtained if different portion of Sydney was chosen or the network modelled covered a larger area.

Sites where the C/(N+I) criteria was exceeded were distributed around the nbn service areas, though, when ignoring those that fail within parkland, they were mostly concentrated to the west of Sydney. The low failure rates in other areas are likely due to the mountainous terrain, location of national parks and larger separation distances from the macro cell network.

The analysis shows that the most at risk nbn sites are those where the CPE antenna is pointed in the direction of the Sydney urban excise area and located on hill/mountain. Most clusters of fail sites, including sites with the largest failure margin, are located on hills/mountains.

There are several fail sites that fall within reasonably sized population centres, such as North Richmond, Silverdale, South Penrith. It is likely that fibre solutions would be used in these areas.

1.7% of Case 1 timesteps (i.e., 94 timesteps) failed. Around one-third of failures were in parkland. Ignoring sites within parkland, 79.3% of sites failed by a margin less than 13 dB, but margins as large as 25 dB were recorded. Failure margins of 13 dB or less result in a positive C/(N+I) which could still enable a connection between CPE and BS albeit at a lower data rate.

0.6% of Case 2 sites (i.e., 34 sites) failed. Around one-third of failures were in parkland or had significant terrain obstructions between CPE and nbn BS. Ignoring these sites, 57% of sites failed by a margin less than 13 dB, but margins as large as 19.4 dB were recorded. Failure margins of 13 dB or less result in a positive C/(N+I) which could still enable a connection between CPE and BS albeit at a lower data rate.

The Case 1 study results suggest it may be difficult to deploy macro cells within the Sydney urban excise area, particularly within 10–15 km of the boundary. Based on Case 2 study results, interference reduces the further new BS are deployed from urban excise boundaries. Due to the small number of failure sites, it may be possible to deploy macro cells further from the area boundary. This could be done by taking advantage of a mixture of terrain shielding and various deployment mitigation measures to manage interference. However, there may be some sites where significant restrictions will apply or where macro cells cannot be deployed.

### Queensland

#### Macro cell studies

Sites where the C/(N+I) criteria was exceeded were mainly distributed to the north and south of Brisbane, with a small number to the west. The low failure rates in other areas are likely due to the mountainous terrain, location of national parks and larger separation distances from macro cell network.

The analysis shows that the most at risk nbn sites are those where the CPE antenna is pointed in the direction of the Brisbane urban excise area and located on hill/mountain. There are also a number of failures from CPE location close to the urban excise boundary. However, most clusters of fail sites, including sites with the largest failure margin, are located on hills/mountain.

There are several fail sites that fall within reasonably sized population centres, such as areas north of Caboolture, Eden’s Landing, Loganholme, New Beith, Ormeau, Undullah and Windaroo. It is likely that fibre solutions would be used in these areas.

2.2% of Case 1 timesteps (i.e., 143 timesteps) failed. Around one-third of failures were in parkland. Ignoring sites within parkland, 86% of sites failed by a margin less than 13 dB, but margins as large as 20 dB were recorded. Failure margins of 13 dB or less result in a positive C/(N+I) which could still enable a connection between CPE and BS albeit at a lower data rate.

0.92% of Case 2 sites (i.e., 59 sites) failed. Nearly half of failures were in parkland. Ignoring these sites, 84.8% of sites failed by a margin less than 13 dB, but margins as large as 20.3 dB were recorded. Failure margins of 13 dB or less result in a positive C/(N+I) which could still enable a connection between CPE and BS, albeit at a lower data rate.

The Case 1 and Case 2 study results suggest it may be difficult to deploy macro cells within the Brisbane urban excise area, particularly within 10–15 km of the boundary. Based on Case 2 study results, interference reduces the further new BS are deployed from urban excise boundaries. Consequently, it may be possible to deploy a limited number of macro cells further from the area boundary. This could be done be taking advantage of a mixture of terrain shielding and various deployment mitigation measures to manage interference. However, there may be some sites where significant restrictions will apply or where macro cells cannot be deployed.

### SA

#### Macro cell studies

Sites where the C/(N+I) criteria was exceeded were mainly distributed to the north of Adelaide, with a some along the Mount Lofty ranges east of Adelaide and a few across the St Vincent Gulf west of Adelaide. The low failure rates west of Adelaide are likely due to the larger separation distance and areas in the southeast of Adelaide have no failure points likely due to the shielding provided by the Mount Lofty ranges.

The analysis shows that the most at risk nbn sites are those where the CPE antenna is pointed in the direction of the Adelaide urban excise area. There are also a number of failures from CPE location close to the urban excise boundary. However, most clusters of fail sites, including sites with the largest failure margin, are located on hills/mountain. The worst affected nbn sites are those in the relatively flat terrain north of the Adelaide. These locations are in close proximity to the macro cell network and areas of limited terrain shielding. There are also a number of failures from CPE location close to the urban excise boundary east of Adelaide in mountainous terrain.

There are several fail sites that fall with reasonably sized population centres, such as Aberfoyle Park, Angel Vale, Evanston and Willaston. It is likely that fibre solutions would be used in these areas.

1.6% of Case 1 timesteps (i.e., 115 timesteps) failed. There were only a handful of failure sites that were either within parkland or the ocean. Ignoring sites within parkland and the ocean, 86% of sites failed by a margin less than 13 dB, but margins as large as 29.8 dB were recorded. The largest failure margins were seen to the north of Adelaide and close to the boundary. Failure margins of 13 dB or less result in a positive C/(N+I) which could still enable a connection between CPE and BS, albeit at a lower data rate.

4.2% of Case 2 sites (i.e., 307 sites) failed. The general location of failures in Case 2 are similar to those in Case 1. Half of all failures are situated in the flat terrain north of Adelaide. A total of 7.8 % of sites where either within parkland or the ocean. Ignoring sites within parkland and the ocean, 79% of sites failed by a margin less than 13 dB, but margins as large as 33.6 dB were recorded. Failure margins of 13 dB or less result in a positive C/(N+I) which could still enable a connection between CPE and BS albeit at a lower data rate.

Due to the small size of the Adelaide urban excise area, there is little difference in the distance from the boundary that new base stations are deployed in Cases 1 and 2.

The Case 1 and Case 2 study results suggest it may be difficult to deploy macro cells within the Adelaide urban excise area. While it might be possible to address failures in the east and west of Adelaide, taking advantage of terrain shielding and various deployment mitigation measures, protecting areas north of Adelaide would be more difficult even with mitigation measures.

### Victoria

#### Macro cell studies

Sites where the C/(N+I) criteria was exceeded were distributed across the service area with the exception of southeast Melbourne. The low failure rates in the southeast are likely due to the mountainous terrain and larger separation distance from the macro cell network.

Most failures south and northeast of the macro cell network are those where the CPE antenna is pointed in the direction of the of the Melbourne urban excise area and located on mountainous terrain. In the west of Melbourne, there are a few fail sites that are on mountainous terrain, in Lerderderg State Park and around Mount Macedon. There are also a reasonable number of failures to the south and southwest across Port Phillip Bay.

It is noted that the Melbourne Case 1 base stations are only located in the western side of the urban excise area. This may result in a lower level of possible site failures to nbn services located to the east of Melbourne.

There are several fail sites that fall with reasonably sized population centres, such as Frankston South, Geelong, Hoppers Crossing, Mount Eliza, Rosebud, Tarneit, Werribee. It is likely that fibre solutions would be used in these areas.

The analysis shows that the most at risk sites are those situated in the relatively flat terrain west and northwest of Melbourne. These areas do not benefit from terrain shielding and are in close proximity of the macro cell networks.

2.3% of Case 1 timesteps (i.e., 164 timesteps) failed. Approximately 40% of failures were in either within parkland (mainly Kinglake National Park and Lederberg State Park) or the ocean. These accounted for many of the largest margin failures. Ignoring sites within parkland and the ocean, 81.7% of sites failed by a margin less than 13 dB, but margins as large as 24.6 dB were recorded. Failure margins of 13 dB or less result in a positive C/(N+I) which could still enable a connection between CPE and BS albeit at a lower data rate. The largest failure margins were at locations to the west and northwest of Melbourne within a few kilometres of the boundary.

3.3% of Case 2 sites (i.e., 225 sites) failed. The general location of failures in Case 2 are similar to those in Case 1. Approximately one-quarter of failures were in either within parkland (mainly Kinglake National Park and Lerderderg State Park) or the ocean/lakes. These accounted for many of the largest margin failures. Ignoring sites within parkland and the ocean, 82.6% of sites failed by a margin less than 13 dB, but margins as large as 28 dB were recorded. Failure margins of 13 dB or less result in a positive C/(N+I) which could still enable a connection between CPE and BS albeit at a lower data rate. The largest failure margins were at locations to the west and northwest of Melbourne within a few kilometres of the boundary.

The Case 1 and Case 2 study results suggest it may be difficult to deploy macro cells within the Melbourne urban excise area. Case 2 results show that situating macro cells further away from the area boundary can still result in a high risk of interference. While it might be possible to address failures in the mountainous areas of Melbourne, by taking advantage of terrain shielding and various deployment mitigation measures, protecting areas west and northwest of Melbourne would be more difficult even when implementing different mitigation measures.

### WA

#### Macro cell studies

Sites where the C/(N+I) criteria was exceeded were mainly distributed along the Darling Ranges to the east of the Perth urban excise area. The low failure rates in other areas are likely due to the larger separation distances and possible terrain shielding from the macro cell network.

The analysis shows that the most at risk nbn sites are those where the CPE antenna is pointed down and in the direction of the Perth urban excise area and located on a hill/mountain. This can be seen with most failures occurring on and adjacent to the Darling Ranges.

There are a few failure sites on areas west of the Darling Scarp, they are likely due to the small separation distance of the CPEs from the macro cell network.

There are several fail sites that fall within reasonably sized population centres, such as Armidale, Aveley, Brabham, Byford, Lesmurdie, Stratton, Viveash. It is likely that fibre solutions would be used in these areas.

4.0% of Case 1 timesteps (i.e., 230 timesteps) failed. Of these around 60% are situated on the mountainous area of the Darling Scarp and 37% adjacent to it (Byford and Middle Swan). Around one-third of failures were in parkland. Ignoring sites within parkland, 61.8% of sites failed by a margin less than 13 dB, but margins as large as 46 dB were recorded. Failure margins of 13 dB or less result in a positive C/(N+I) which could still enable a connection between CPE and BS, albeit at a lower data rate.

4.8% of Case 2 sites (i.e., 281 sites) failed. Of these around 65% are situated on the mountainous area of the Darling Scarp and 37% are located adjacent to it (Byford and Middle Swan). Around one-third of failures were in parkland. Ignoring sites within parkland, 74% of sites failed by a margin less than 13 dB, but margins as large as 49 dB were recorded. Failure margins of 13 dB or less result in a positive C/(N+I) which could still enable a connection between CPE and BS albeit at a lower data rate.

Due to the small size of the Adelaide urban excise are, there is little difference in the distance from the boundary that new base stations are deployed in Cases 1 and 2.

The Case 1 and Case 2 study results suggest it may be difficult to deploy macro cells within the Perth urban excise area. While it might be possible to address failures in the north and south of Perth, protecting nbn service locations along the Darling Ranges would be more difficult even with mitigation measures.

## Discussion on existing service deployments

This study analysed existing deployments by nbn and Optus near the boundary of their area adjacent 3.4 GHz spectrum licences. This was based on information from the RRL which is limited to macro cell deployments. The intention is to assess how the current technical framework affects deployments close to shared licence boundaries as well as typical separation distances between services in adjacent areas. This serves as a reference to compare the results of studies on new services in urban excise areas.

### Analysis of macro cell deployments on the RRL

3.4 GHz spectrum licence base stations:

nbn and Optus have 23 and 5 base stations within 5 km of their shared 3.4 GHz spectrum licence boundaries respectively.

nbn and Optus have 26 and 51 base stations within 5–10 km of their shared 3.4 GHz spectrum licence boundaries respectively.

Deployments within 10 km of the shared licence boundaries employ different mitigation techniques to reduce out of area emissions. This often involves sectors that point away from or along their shared boundaries.

nbn and Optus base stations are almost always separated by more than 10 km.

nbn PTS base stations:

25 within 5 km of urban excise area boundaries.

111 within 5–10 km of urban excise area boundaries.

No mitigation measures have been employed to deploy close to the urban excise boundary. This is because services were mainly designed and deployed before urban excise areas were defined and before a decision was made to excise them. This will likely impact how close new services within urban excise areas can be deployed (and the degree of mitigation measures required) to nbn service areas.

### Impact on macro cell deployments within urban excise areas

Other than Canberra, urban excise areas are significantly smaller than existing Optus 3.4 GHz spectrum licence areas in capital cities.

Analysis of existing 3.4 GHz base stations suggests that deployment of macro cells within 10 km of a licence boundary is difficult.

The Canberra urban excise area is similar, and in some places larger, in size to the existing Optus 3.4 GHz spectrum licence area. The areas base stations are most likely to be deployed are more than 20 km from the urban excise boundary.

The Sydney urban excise area encompasses some areas that are more than 20 km from the urban excise boundary, this includes the Botany Bay area. There is also a reasonable amount of area that is 10–20 km from the boundary, this includes the central business district (CDB). Macro cell deployments within these areas may be possible while managing interference into nbn service areas. Macro cell deployments around Wollongong, Newcastle and Gosford present a higher risk of interference as they are within 5 km of the urban excise boundary.

Most of the Brisbane urban excise area is within 10 km of the boundary, this includes the CBD. While parts of North Stradbroke Island and Moreton Island are more than 20 km from the boundary, none of Brisbane itself is. Macro cell deployments in most parts of Brisbane are likely to be difficult while managing interference into nbn service areas. Macro cell deployments in the Gold Coast may also be difficult as they are within 10 km of the urban excise boundary.

All land areas within the Adelaide urban excise area are within 10 km of the boundary. This will likely restrict the deployment of macro cells within Adelaide while managing interference into nbn service areas.

A majority of the Melbourne urban excise area is within 10 km of the boundary. Only the suburb of Hampton is more than 20 km from the boundary. Areas in and around the CBD, St Kilda, Bentleigh and Cheltenham are within 10–20 km from the boundary. Macro cell deployments in most parts of Melbourne are likely to be difficult while managing interference into nbn service areas.

A majority of the Perth urban excise area is within 10 km of the boundary, this includes the CBD. While Rottnest Island is more than 20 km from the boundary, none of Perth itself is. There are parts of Perth within 15 km of the boundary in the Karrinyup and City beach areas. Macro cell deployments in Perth are likely to be difficult while managing interference into nbn service areas.

## Summary

A summary of the key outcomes of the studies conducted in this paper are provided in Table 7. These outcomes are expressed in terms of the risk of interference a macro cell and restricted cell model presents to nbn services in each area. Importantly they consider the viability of deploying a reasonable number of base stations rather than the viability of deploying a single or small number of them.

A key factor in considerations is that nbn has an existing network deployed outside urban excise areas. This network was designed and largely deployed before urban excise areas were identified and investigated for use by other operators. This requires different attention than a scenario where only greenfield deployments are being considered. The former of which would allow all operators to design and deploy a network that accounted for potential interference across a licence boundary.

When analysing the results of the deploying new services in urban excise areas studies, it is not expected that 100% of sites or timesteps will pass in order for an area to be deemed low risk of interference to nbn service areas (or potentially viable for macro or restricted cell deployments). If a small enough number of sites fail and or the failure margin is low, an area could be considered low risk and it may be possible for services to be deployed with some mitigation measures employed. This paper does not attempt to quantify what exactly could be acceptable in this case but does identify margins of failure and percentage of site failures.

In addition, further analysis could be conducted to determine whether failure sites are viable locations for nbn CPEs if failure sites are:

- located on tops of specific hills/mountains

- located within a cluster of housing or towns where fibre may be deployed.

While a rudimentary analysis was performed to identify locations detailed above, only those within national parks, oceans or lakes were considered to not be viable in this paper. A more accurate nbn service area description, that removes these areas, would be required if a macro cell approach is adopted.

Studies suggest that the deployment of macro cells within 10 km of the nbn service areas will be difficult to do while managing interference into the nbn service area. Since many nbn service areas are within 1 km of an urban excise boundary, this means there will be restrictions on macro cell deployments within, at least, 10 km of an urban excise boundary in many cases. It is consequently expected that, due to their small size, utility of many of the urban excise areas will be low or unviable for any macro cell use. While it may be possible to implement numerous mitigation techniques to enable deployments in some case, it is unlikely a significant macro cell network could be deployed.

In such cases a restricted cell model may be more viable. The restricted cell model in this paper represents an example of what deployment constraints could be applied. For example, it may be possible to allow higher EIRPs by defining suitable height and licence boundary conditions. However, further investigation into the application of ITU-R Recommendation P.2108 to model clutter loss for cells located at heights of 6 m above the ground is required.

Summary of analysis for small and macro cell deployment scenarios

| Sub-area name | Small cell scenario | Macro cell scenario |
| --- | --- | --- |
| ACT | Small cell use may present a low risk of interference, provided suitable conditions are defined. | Macro cell use likely possible with low interference risk, provided suitable conditions are defined. |
| NSW | Small cell use may present a low risk of interference, provided suitable conditions are defined. | Macro cell use may be possible in some parts of Sydney, provided suitable conditions are defined. Macro cell deployments closer to the urban excise boundary have a higher risk of interference. Deployments in and near Wollongong, Newcastle and Gosford are unlikely to be viable. |
| Qld | Small cell use may present a low risk of interference, provided suitable conditions are defined. | Macro cell use presents a high risk of interference. Deployments in the Gold Coast area are unlikely to be viable. |
| SA | Small cell use may present a low risk of interference, provided suitable conditions are defined. | Macro cell use presents a high risk of interference and is unlikely to be viable. |
| WA | Small cell use may present a low risk of interference, provided suitable conditions are defined. | Macro cell use presents a high risk of interference. |
| Vic | Small cell use may present a low risk of interference, provided suitable conditions are defined. | Macro cell use presents a high risk of interference. |

# Appendix A: Urban excise areas

The geographical areas covered by urban excise areas are:

- Described in Table 8.

- Illustrated in Figure 27.

HCIS description of urban excise areas

| Sub-area name | HCIS |
| --- | --- |
| Adelaide | IW3N, IW3O4, IW3O5, IW3O7, IW3O8, IW6B1, IW6B2, IW6B3, IW6B5, IW6B6 |
| Brisbane | NT9B, NT9C, NT9D, NT9E, NT9F, NT9G, NT9H, NT9K, NT9L, NT8H3, NT8L2, NT8L3, NT8L5, NT8L6, NT8L8, NT8L9, NT9A6, NT9A7, NT9A8, NT9A9, NT9I1, NT9I2, NT9I3, NT9I4, NT9I5, NT9I6, NT9J1, NT9J2, NT9J3, NT9J4, NT9J5, NT9J6, NT9J9, NT9N5, NT9N6, NT9N8, NT9N9, NT9O4, NT9O7, NU3B2, NU3B3, NU3C1, NU3C4 |
| Canberra | MW4D, MW4H, MW5A, MW5B, MW5E, MW5F, MW2M5, MW2M6, MW2M7, MW2M8, MW2M9, MW2N4, MW2N5, MW2N7, MW2N8, MW2N9, MW4L1, MW4L2, MW4L3, MW4L5, MW4L6, MW5I1, MW5I2, MW5I3, MW5I4, MW5I5, MW5I6, MW5J1, MW5J2, MW5J4, MW5J5 |
| Melbourne | KX3P, KX3L6, KX3L7, KX3L8, KX3L9, KX6D1, KX6D2, KX6D3, KX6D5, KX6D6, LX1M, LX1I7, LX1N4, LX4A1, LX4A2, LX4A3, LX4A4, LX4A5, LX4B1 |
| Perth | BV1M, BV1N, BV1O, BV4A, BV4B, BV4C, BV1L5, BV1L8, BV1P1, BV1P2, BV1P4, BV1P5, BV1P7, BV1P8, BV4D1, BV4D2, BV4E1, BV4E2, BV4E3, BV4F1, BV4F2, BV4F3, BV4G1, BV4G2, BV1P9, BV4D3, BV4D4, BV4G3 |
| Sydney | NV7G, NV7H, NV7J, NV7K, NV7L, NV7M, NV7N, NV7O, NV7P, NW1A, NW1B, NW1C, NW1D, NW1E, NW1F, NW1G, NW1H, MV9P2, MV9P3, MV9P5, MV9P6, MV9P7, MV9P8, MV9P9, MW3D1, MW3D2, MW3D3, MW3D5, MW3D6, MW3D8, MW3D9, MW3H2, MW3H3, MW3H5, MW3H6, MW3H9, MW3L2, MW3L3, NV4O7, NV4O8, NV4O9, NV4P7, NV7F6, NV7F8, NV7F9, NV7I6, NV7I8, NV7I9, NW1I1, NW1I2, NW1I3, NW1J1, NW1J2, NW1J3, NW1K1, NW1K2, NW1K3, NW1L1, NW1L2, NW1L3 |

The HCIS coordinates can be converted into a Placemark file (viewable in Google Earth) through a facility on the ACMA website.

Illustration of urban excise areas (purple areas)

The following HCIS were used for Sydney: MV9P2, MV9P3, MV9P5, MV9P6, MV9P7, MV9P8, MV9P9, MW3D1, MW3D2, NV7M1, NV7M4, NV7M7; and Melbourne: KX3L7, KX3L8, KX3L9, KX3P1, KX3P2, KX3P3, KX3P4, KX3P5, KX3P6.

For Cases 1 and 3, the term ‘percentage of failed sites’ should be read as the ‘percentage of timesteps a failure occurred’.