ACMA compliance priority 2020–21

Electromagnetic energy (EME) near 5G mobile base stations

AUGUST 2021

Canberra

Red Building 
Benjamin Offices
Chan Street 
Belconnen ACT

PO Box 78
Belconnen ACT 2616

T	+61 2 6219 5555

Melbourne

Level 32 
Melbourne Central Tower
360 Elizabeth Street 
Melbourne VIC

PO Box 13112
Law Courts 
Melbourne VIC 8010

T	+61 3 9963 6800

Sydney

Level 5 
The Bay Centre
65 Pirrama Road 
Pyrmont NSW

PO Box Q500
Queen Victoria Building 
NSW 1230

T	+61 2 9334 7700

Copyright notice

https://creativecommons.org/licenses/by/4.0/

With the exception of coats of arms, logos, emblems, images, other third-party material or devices protected by a trademark, this content is made available under the terms of the Creative Commons Attribution 4.0 International (CC BY 4.0) licence.

We request attribution as © Commonwealth of Australia (Australian Communications and Media Authority) 2021.

All other rights are reserved.

The Australian Communications and Media Authority has undertaken reasonable enquiries to identify material owned by third parties and secure permission for its reproduction. Permission may need to be obtained from third parties to re-use their material.

Written enquiries may be sent to:

Manager, Editorial Services
PO Box 13112
Law Courts
Melbourne VIC 8010
Email: info@acma.gov.au

Executive summary	1

Our 5G and EME compliance program	1

EME measurement program	2

Summary of EME measurement results	3

Background	4

Methodology	5

Part 1: Environmental EME measurements	9

Purpose		9

Methodology	9

Outcomes	10

Part 2: Comparisons to carriers’ predicted EME	13

Purpose		13

Methodology	13

Outcomes	14

Part 3: Investigation of 5G beamform technology	17

Purpose		17

Methodology	17

Outcomes	19

Summary	20

Appendix A: Site list	21

Appendix B: Predicted maximum EME vs measured maximum EME	26

Appendix C: Active vs idle maximum beamformed EME	29

Executive summary

Regarded as the next step in the evolution of mobile wireless communication, 5G promises improved connectivity and increased data download speeds. There is considerable interest in this technology and concern from some members of the community about potential health effects from electromagnetic energy (EME) emissions from 5G mobile phone base stations.

The EME exposure standard in Australia is based on scientific assessment and established limits to human exposure to radiofrequency fields (RF). They are set by the Australian Radiation Protection and Nuclear Safety Agency (ARPANSA). The Australian Communications and Media Authority (ACMA) regulates these limits by, among other things, imposing conditions on network operators for the operation of radiocommunications transmitters.

The ARPANSA limits are set well below levels at which harm to people may occur. At exposure levels below the limits set within the ARPANSA safety standard, it is the assessment of ARPANSA and international organisations, that there is no established scientific evidence to support any adverse health effects from EME exposures to populations or individuals.

We actively monitor to ensure that network operators are following the rules.

## Our 5G and EME compliance program

With the continuation of the expansive 5G rollout by Optus, Telstra and TPG Telecom Limited (the carriers) expected over the next few years, in April 2020, we announced 5G and EME as one of our compliance priorities for 2020–21.

There are two main elements of the regulation of 5G mobile phone base stations under the legislation we administer:

carrier consultation and notification requirements under the C564:2018 Mobile Phone Base Station Deployment Code (the deployment code) when deploying base stations

carrier compliance with the mandatory standard for EME emissions that are given effect through licence conditions under the Radiocommunications Licence Conditions (Apparatus Licence) Determination 2015 (the licence determination).

We have published 3 reports outlining the outcomes of our 5G and EME compliance programs in 2020–21. They provide information on our audits to assess carriers’ compliance with:

The EME exposure level standard

consultation and notification requirements under the deployment code when rolling out 5G technology

licence conditions.

Components of our 2020–21 compliance programs

This EME measurement program report focuses on carriers’ compliance with the EME standard for 5G deployments.

While this compliance program has focused on 5G sites in Victoria, the findings from this work provide a baseline for our continuing focus on 5G and EME deployments in other Australian states and territories in the coming year.

## EME measurement program

In early 2021, we measured EME public exposure levels at 129 5G-enabled mobile telecommunications base station sites in Victoria.

The purpose of the current measurement program is to assess compliance by the carriers with their regulatory obligations to ensure that EME levels in areas accessible to the public are below the ARPANSA limits.

As part of our 5G EME measurement program we undertook 3 different types of activity to assess carriers’ compliance with the EME standard:

Environmental EME measurements – measuring the average cumulative EME at a site.

Comparisons of actual maximum measured levels with carrier-predicted maximum EME levels.

EME measurements using 5G beamforming to consider the impact of beamforming technology on EME measurements.

## Summary of EME measurement results

### Environmental EME measurements

Average EME at all sites was less than 1.5% of the ARPANSA standard, the majority of sites were under 1%.

### Comparisons of actual vs predicted maximum EME levels

Carrier EME predicted levels were, in all cases, higher than the actual levels measured. For 96% of sites, the measured total EME was less than 50% of the carrier predicted EME.

### Beamforming EME maximum measurements

The maximum beamformed EME measured was 2.5% of the ARPANSA standard.

# Background

The ACMA is responsible for ensuring communications equipment and devices comply with the EME standard. We undertake a range of compliance activities, including examining areas of specific interest or concern as new technologies are rolled out in Australia.

Recent examples include the deployment of mobile network small cell 4G infrastructure and the rapid rollout of 5G technology in the 3500 MHz band deployed on mobile network rooftop and tower sites.

In 2019–20, in response to public interest about health impacts from the level of EME emissions from small cell base stations, we assessed compliance with the EME standard in relation to small cell deployments as part of our compliance priorities.

We measured 59 small cell base stations across Australia and found the EME levels were well below 1% of the ARPANSA limit. The highest level recorded was 0.737%.

In this year’s 5G and EME compliance program, we’ve continued our focus on the role of the carriers – to ascertain their awareness and compliance with their licence conditions under the licence determination, and with their consultation requirements under the deployment code.

Our objectives for the 2020–21 5G and EME compliance priority program were to:

assess the effectiveness of current regulatory measures in managing the risks associated with EME emissions from 5G deployments

ensure the carriers complied with their obligations to notify and consult interested stakeholders before deploying 5G technology

ensure the carriers complied with the EME standard in relation to the operation of their mobile base stations and through the provision and maintenance of accurate records

ascertain and mitigate any identified risk of EME exposure to the public from 5G deployments

make sure the public were effectively educated and informed about EME exposure levels.

This report presents the results of our EME measurements at a selection of 5G-enabled base stations in Victoria, undertaken between February 2021 and April 2021, as part of the 5G and EME compliance program.

# Methodology

We took measurements in 3 separate categories to assess a range of components of carriers’ compliance with the EME standard:

Part 1: Environmental EME measurements

- The environmental EME measurement category seeks to establish whether the measured EME at each base station is within the limits set by the ARPANSA standard with respect to whole body exposure to unperturbed average (RMS) RF electromagnetic fields.

Part 2: Comparisons to carriers' predicted EME

- For each base station facility, the Carriers provide a calculated prediction of the maximum level of EME and a distance from the base station where that maximum occurs. The Comparisons to the Carriers predicted EME category seeks to ascertain the integrity of the Carriers EME predictions by comparing them with field-based measurements taken at, or as close as practically possible to, the predicted distance from the base station provided by the Carriers.

Part 3: Investigation of 5G beamforming technology

- The purpose of our Investigation of 5G beamforming technology is to utilise alternative measurement techniques to provide an indication of the maximum level of EME that may be contained within a 5G beamformed transmission from base stations installed with beamforming capable antennas.

Each category is discussed in detail in the following sections of this report.

### Limitations

ACMA staff conducting field measurements were largely based in Victoria, and due to various travel restrictions and lockdowns during the 2020–21 financial year, the COVID-19 pandemic meant that a nationwide measurement program was not feasible. We chose to conduct measurements only in Victoria. Our program will extend to other states of Australia in 2021–22 to provide a broader sample of measured sites.

### Base station site selection

We selected a sample of mobile telecommunication sites for audit and measurement from the Radio Frequency National Site Archive (RFNSA), a public database of all Australian mobile phone base stations, hosted by the Australian Mobile Telecommunications Association (AMTA).

In February 2021, we obtained a list of all base stations in Victoria from the RFNSA and filtered the list to only include sites with active 5G services, and by site operator (Optus, Telstra and TPG Telecom Limited).

Each operator has progressed the rollout of 5G services at different rates, and at the time we selected sites to measure, we determined that TPG Telecom Limited had the smallest number of active 5G sites in Victoria at 43 sites. We used 43 as a base figure to determine the number of sites to measure for each network operator to maintain fairness.

A random sample was then applied to Optus and Telstra sites in Victoria to produce a selection of 43 sites for each carrier to arrive at a final figure of 129 sites.

The sites are shown below in Figure 2 and 3, site numbers correlate with the list of sites at Appendix A.

Site locations in outer Melbourne and regional Victoria

Site locations in greater Melbourne

### Selecting a suitable measurement location

We selected a suitable location for conducting measurements at each base station on all of the following criteria:

At a distance from the base station accessible to the public that is as close as possible to where the highest level of predicted EME is specified in the environmental EME report available from the RFNSA.

Has a clear and uninterrupted line-of-sight view to the base station antennas.

In a direction not more than ±30 degrees from the centre line of the azimuth of one of the base station’s antenna sectors.

Before conducting each EME measurement, ACMA staff identified known characteristics of the transmitters installed at each site, including the likely propagation and physical environment characteristics.

Physical site characteristics were visually inspected and compared with corresponding information for each base station in the environmental EME report and the EME guide available from the RFNSA. Photographs of each of the base stations were also taken. Observations made at each site were recorded and utilised to inform both the EME records and base station deployment code audit programs.

As shown in Figure 4 below, measurements in all 3 categories were conducted using a selective field strength measuring device – a Narda SRM-3006 selective radiation meter – along with an isotropic electric field probe – a Narda three axis antennae (420 MHz to 6 GHz). The meter was mounted on a tripod at a height of 1.5 metres above ground level.

Narda SRM-3006 measurement probe at a typical measurement location

# Part 1: Environmental EME measurements

## Purpose

The purpose of conducting environmental EME measurements was to measure the average overall EME exposure levels presented between 420 MHz and 6 GHz at, or as close as practically possible to, the point of maximum predicted or calculated EME level at the selected sample of 129 active 5G base station sites. This was to ascertain whether EME is within the limits set by Radiation Protection Standard for Maximum Exposure Levels to Radiofrequency Fields – 3 KHz to 300 GHz (2002) (the ARPANSA RPS3 standard) as required by the reference levels for time-averaged exposure to RMS electric and magnetic fields (unperturbed fields).

### Limitations

On 25 February 2021, ARPANSA released a new standard, the Standard for Limiting Exposure to Radiofrequency Fields – 100 KHz to 300 GHz (the ARPANSA RPS S-1 standard). This new standard updates the ARPANSA RPS3 standard which was first published in 2002. For frequencies between 420 MHz and 6 GHz, the new standard specifies a change to the averaging time for whole body exposure to EME from 6 minutes to 30 minutes, to better match the time taken for body core temperature to rise. The EME power density exposure limits for frequencies between 420 MHz and 6 GHz specified in the ARPANSA RPS3 standard remain the same in the new ARPANSA RPS S-1 standard.

We are updating our legislative instruments to reflect the new standard, specifically the Radiocommunications Licence Conditions (Apparatus Licence) Determination 2015 which, among other things, sets out the licence conditions for mobile network operator licensees, and currently references the ARPANSA RPS3 standard.

We conducted environmental EME measurements using the ARPANSA RPS3 standard currently referenced in our legislative instruments. However, we have chosen to depart from the standard in so far as adopting the increased whole of body exposure measurement time of 30 minutes specified in the new ARPANSA RPS S-1 standard.

The increased exposure measurement time was adopted in the interests of applying a best practice approach. The adoption of the 30-minute measurement period will provide for compatibility in our measurement methodology to enable us to make comparisons between results collected during this current measurement program with results collected in future programs when ARPANSA RPS S-1 is incorporated into the regulatory arrangements.

## Methodology

Measurements were conducted in accordance with the Standards Australia document, Radiofrequency fields, Part 2: Principles and methods of measurement and computation – 3 kHz to 300 GHz – AS/NZS 2772.2 Standard (the AS/NZS 2772.2 Standard) and the ARPANSA RPS3 Standard.

At each measurement location, measurements were conducted between 420 MHz and 6 GHz, which includes all public mobile telecommunication service bands currently used in Australia.

The equipment provided an overall exposure level made up of individual contributions made by the mobile base station and other sources present at the measurement location transmitting between 420 MHz and 6 GHz. The average EME expressed as a percentage of the ARPANSA RPS3 limit was measured over a period of 30 minutes.

## Outcomes

Having conducted measurements at 129 individual base station sites in Victoria, we observed very low levels of average EME, measured at under 1.5% of the ARPANSA limit.

Measurements were taken in publicly accessible locations and we are satisfied that the carriers are demonstrating compliance with respect to the exposure limits of the safety standard:

Inside the Melbourne metropolitan area, the highest average EME measured was 1.43% at Werribee (RFNSA site 3030001) and the lowest measured was 0.04% at Carlton (RFNSA site 3053020). See Figure 5.

- Outside of the Melbourne metropolitan area, the highest average EME measured was: 0.24% at Diggers Rest (RFNSA site 3427001) and the lowest measured was 0.07% at Pakenham (RFNSA site 3810020). See Figure 6.

Greater Melbourne average EME site-by-site as of 31 May 2021

Outer Melbourne and regional Victoria average EME site-by-site as of 31 May 2021

# Part 2: Comparisons to carriers’ predicted EME

## Purpose

For each mobile base station site, an assessment of the EME environment is conducted in accordance with the deployment code, which requires the supply of specified information as part of the consultative process with the local community and government. The environmental EME report is part of this process and is produced according to a methodology developed by ARPANSA. It provides conservative estimates of the maximum levels of EME from a wireless base station for both existing base stations and proposed upgrades to base stations at the site.

As part of our measurement program, we compared the calculated maximum EME predicted in the environmental EME report with the actual maximum EME measured at each site. The intention in doing so was to provide assurance that the calculations predicting cumulative EME at individual sites are conservative and do not under-represent the level of emissions at a site.

### Limitations

While the environmental EME report provides a calculated prediction of the maximum cumulative EME for all active and proposed transmitters at an individual site, it was not possible in practice in a field environment to only measure the emissions from one individual carrier’s deployment of a particular technology, such as 5G, at a site. Additionally, the environmental EME report provides for proposed transmitters which may or may not have been active at the time we conducted the measurement. Measurements conducted in the field will be made up of all components from all transmitter sources present at the measurement location at the time of measurement.

However, it is anticipated that the majority component of measured EME will be the emissions of the base station of interest and that only a small component of additional EME will come from other sources. Other sources could include more distant mobile phone base stations, as well as other nearby transmitters in the 420 MHz to 6 GHz band. As a result, the ACMA is of the view that a comparison between the EME levels it has measured at sites and predicted cumulative maximums will provide an acceptable point of comparison for evaluating the reasonableness of these calculations.

## Methodology

For the purposes of this assessment, at each location we have measured the maximum cumulative EME between 420 MHz and 6 GHz, as well as the maximum EME contained within all individual public mobile telecommunications service bands including other defined service bands between 420 MHz and 6 GHz.

These bands include:

UHF LMRS

UHF TV

700 MHz 4G

800 MHz TRUNK

800 MHz 3G/4G

915 MHz LIPD

900 MHz 3G/4G

1800 MHz 4G

2100 MHz 3G/4G

2300 MHz 4G

2400 MHz LIPD

2600 MHz 4G

3500 MHz 5G

5800 MHz LIPD

The maximum level of cumulative EME, expressed as a percentage of the ARPANSA RPS3 limit, was measured over a period of 30 minutes between 420 MHz to 6 GHz, as well as the maximum EME within each of the defined service bands.

The results compare predicted maximum with the measured maximum of all emissions between 420 MHz and 6 GHz, as well as the measured maximum of the 3500 MHz 5G service band.

## Outcomes

Having conducted measurements at 129 individual base station sites in Victoria, we observed very low levels of EME with maximum measured levels below 5% of the ARPANSA limit. Figure 7 below shows that for the majority of sites, EME levels were below 2.5% of the ARPANSA limit.

Top 10 Maximum EME levels measured at sites as of 31 May 2021

When the measured EME levels were compared to carrier-predicted levels, we found that the carrier predictions have been modelled on worst case exposure assessments and the predictions are highly conservative.

Measurements were taken in publicly accessible locations, and we are satisfied that the carriers are demonstrating compliance with respect to the exposure limits of the safety standard:

For TPG Telecom Limited, the highest measured EME was 4.47% at a site in Werribee (RFNSA site 3030001). The predicted EME was 13.02%.

For Optus, the highest measured EME was 2.15% at a site in Thornbury (RFNSA site 3071005). The predicted EME was 6.54%.

Telstra’s highest measured EME was 0.80% at a site in Thornbury (RFNSA site 3071001). The predicted EME was 6.45%.

Comparison of predicted vs highest measured maximum EME by carrier as of 31 May 2021

For 96% of sites, the measured total EME was less than 50% of the carrier’s predicted EME, and for 55% of sites, the measured total EME was less than 10% of the carrier’s predicted EME.

Full details of the predicted vs measured maximum EME levels for each carrier are at Appendix B.

# Part 3: Investigation of 5G beamform technology

## Purpose

We are aware of work within industry and by international regulators to develop alternative 5G measurement techniques to ascertain more closely the impact of 5G beamforming technology on EME levels.

Beamforming techniques are used for efficient delivery of 5G services where one or more focused radiofrequency beams are directed to the mobile user’s handset from the base station’s antenna. By using beamforming capable antennas, the transmitter power of the base station is typically lower than when non-beamforming capable antennas are used because the transmitter’s energy can be directed more efficiently.

Beamformed radiofrequency emissions for 5G are typically very short in nature and are only present when the mobile user’s handset is actively performing a transaction using 5G. Therefore an EME measurement technique that activates the 5G transmitter may provide a more accurate representation of 5G EME levels.

### Limitations

Having reviewed field measurement trials conducted by the carriers and international regulators, we chose to conduct a series of measurements, based on best practice knowledge gathered from industry and other regulators globally, to provide an indication of the maximum 5G EME contained within an active beamformed transmission.

It is important to recognise that there is absence of agreed international measurement standards for measuring beamformed EME at this time. In this context, 5G beamforming measurements should be considered experimental and are based on current best practice knowledge gathered from mobile network operators and other regulators. Undertaking this work allows the ACMA to observe the impact of 5G beamforming technology on EME levels and will inform the development of future measurement methodologies.

## Methodology

5G mobile base station transmitters adjust their power up and down to be just sufficient to manage user demand. To attract as much of the transmitter power resource available at any point in time, it is necessary to provide as much traffic load as possible on the 5G transmitter.

To achieve a high traffic load scenario, we used a 5G capable smart phone handset utilising the Ookla speed-test application to activate the 5G transmitter. Maximum base station transmitter demand from the mobile handset occurs during the download part of the speed test transaction, and EME is measured during this short interval of time during the download.

As shown in Figure 9, to avoid measuring EME from the smart phone as much as possible, the phone is positioned no less than 3 metres behind the measurement probe in such a way as to create a direct path for the beamform that intersects with the measurement probe. It should also be noted that the smart phone transmitter is least active during the Ookla speed-test download transaction, further reducing its contribution to the EME measured result.

The positioning of the measurement probe with respect to the base station and the smart phone

For the selected carrier at each site, we took 6 individual measurements of the active beamformed transmission for 10 seconds each, corresponding to the duration of the Ookla speed-test download transaction. We also took 6 idle measurements of 10 seconds each, in between active measurements, to measure the non-activated level of EME from the base station.

We recorded the highest of the six active and idle measurements, expressed as a percentage of the ARPANSA RPS3 limit.

Measurements were made in the 3500 MHz band where all the carriers in Australia are currently active with their 5G services. Of the 43 sites selected for each carrier, the frequency bands measured are listed below:

Telstra 5G Melbourne and Geelong (3575 MHz to 3635 MHz).

Telstra 5G Victoria Regional (3545 MHz to 3625 MHz).

Optus 5G Melbourne and Geelong (3475 MHz to 3575 MHz).

TPG Telecom Limited 5G Melbourne and Geelong (3635 MHz to 3695 MHz).

## Outcomes

Having conducted measurements at 129 individual base station sites in Victoria, we observed very low levels of beamformed EME with maximum levels at 2.5% of the ARPANSA limit.

Measurements were taken in publicly accessible locations and we are satisfied that the carriers are demonstrating compliance with respect to the exposure limits of the safety standard.

In some cases, we were not able to consistently attract a 5G beamform. We expect that in these cases the mobile handset was utilising a combination of 3G, 4G and/or 5G connections from the measured site or from other nearby sites.

We observed, at a small number of sites, that measured idle levels were close to, or higher than, active levels, suggesting that we were measuring other nearby user traffic at the time of the idle measurement.

Figure 10 shows the sites with the highest maximum active beamformed EME for Optus, Telstra and TPG Telecom Limited.

Highest active maximum beamformed EME measured by network operator as of 31 May 2021

For 92%% of sites measured, the idle beamformed EME was less than 50% of the active beamformed EME, and for 40% of sites measured, the idle beamformed EME was less than 10% of the active beamformed EME.

A full breakdown of the maximum EME measured for both idle and active beamformed 5G at all sites is at Appendix C.

# Summary

For all the 129 sites visited, and in all measurement categories, we observed very low levels of EME well below the public exposure limits and that the network operators are compliant with the ARPNSA standard with respect to those limits.

Our environmental EME measurements determined that the average EME from sites was less than 1.5% of the ARPANSA standard. The majority (98%) of sites measured under 1%.

Top 10 average measured EME levels by site as of 31 May 2021

Our comparison to the carriers’ predicted EME measurements determined that the measured levels were significantly lower than the predicted levels. The carriers’ predictions have been modelled on worse-case exposure assessments and are very conservative.

Our investigation into 5G beamforming indicated peak levels of EME of up to 2.5% of the ARPANSA standard under a heavily loaded base station scenario, with 96% of sites measured under 1%.

We also observed that sites with the highest EME levels for both environmental and beamforming measures were typically sites with multiple operators and multiple technologies in place. All sites were well within the ARPANSA standard and lower than predicted EME levels. However, noting the small sample size, these observations will need to be further evaluated as we increase the number of 5G sites measured.

This work will provide a baseline for our continuing focus on 5G and EME deployments in 2021–22. In the coming year we’ll expand our 5G and EME measurement program to other Australian states and territories.

# Appendix A: Site list

| Audit site number | Carrier | RFNSA site ID | Address | Suburb |
| --- | --- | --- | --- | --- |
| 1 | Optus | 3002005 | Melbourne Cricket Ground, Brunton Avenue | East Melbourne |
| 2 | Optus | 3012005 | 8 Cromwell Parade | West Footscray |
| 3 | Optus | 3015005 | 120 Mason Street | Newport |
| 4 | Optus | 3018002 | Bluegum Drive and Fresno Street | Altona |
| 5 | Optus | 3020008 | 78 Berkshire Road | Sunshine North |
| 6 | Optus | 3020011 | Judge Street | Sunshine |
| 7 | Optus | 3025005 | 17 Chambers Road | Altona North |
| 8 | Optus | 3029014 | 1 Alexandra Avenue | Hoppers Crossing |
| 9 | Optus | 3030021 | Werribee Racecourse, 12-16 Bulban Road | Werribee |
| 10 | Optus | 3039001 | 641-649 Mt Alexander Road | Moonee Ponds |
| 11 | Optus | 3058005 | 61 Spry Street | Coburg North |
| 12 | Optus | 3065003 | 105 Victoria Street | Fitzroy |
| 13 | Optus | 3071005 | 364 Rossmoyne Street | Thornbury |
| 14 | Optus | 3074002 | 391 Settlement Road | Thomastown |
| 15 | Optus | 3074005 | 23 Macquarie Drive | Thomastown |
| 16 | Optus | 3081008 | Coomalie Crescent | Heidelberg West |
| 17 | Optus | 3082002 | The Stables Shopping Centre, Childs Road | Mill Park |
| 18 | Optus | 3101009 | Kew Reservoir, 378 Cotham Road | Kew |
| 19 | Optus | 3103009 | 255A Belmore Road | Balwyn |
| 20 | Optus | 3121001 | 100 Madden Grove | Burnley |
| 21 | Optus | 3121041 | 36 Crown Street | Richmond |
| 22 | Optus | 3133001 | TOC Depot, Morack Road | Vermont South |
| 23 | Optus | 3140003 | 122-124 Beresford Road | Lilydale |
| 24 | Optus | 3144002 | 322 Glenferrie Road | Malvern |
| 25 | Optus | 3149002 | Melbourne Water Site, St Albans Street | Mount Waverley |
| 26 | Optus | 3151002 | 643 Highbury Road | Burwood East |
| 27 | Optus | 3152004 | 1166 High Street Road | Wantirna South |
| 28 | Optus | 3158001 | Guy Road | Upwey |
| 29 | Optus | 3161004 | 230 Balaclava Road | Caulfield North |
| 30 | Optus | 3166015 | Railway Avenue | Oakleigh |
| 31 | Optus | 3175006 | 56 Soden Road | Bangholme |
| 32 | Optus | 3175011 | 121 Stud Road | Dandenong |
| 33 | Optus | 3178003 | Stud Park Shopping Centre, Stud Road | Rowville |
| 34 | Optus | 3186004 | 120 Church Street | Brighton |
| 35 | Optus | 3190001 | 1123A Nepean Highway | Highett |
| 36 | Optus | 3199022 | 183 Cranbourne Road | Frankston |
| 37 | Optus | 3216009 | 209 High Street | Belmont |
| 38 | Optus | 3226001 | 1-13 Adco Grove | Ocean Grove |
| 39 | Optus | 3427001 | 1376 Calder Highway | Diggers Rest |
| 40 | Optus | 3910008 | 7 Duiker Court | Langwarrin |
| 41 | Optus | 3930005 | Peninsula School, Wooralla Drive | Mount Eliza |
| 42 | Optus | 3975006 | 505 Thompsons Road | Lyndhurst |
| 43 | Optus | 3977022 | Wedge Road | Sandhurst |
| 44 | Telstra | 3000262 | 522-552 Flinders Lane | Melbourne |
| 45 | Telstra | 3002017 | 282-336 Victoria Parade | East Melbourne |
| 46 | Telstra | 3023005 | 27 Station Road | Deer Park |
| 47 | Telstra | 3026005 | 167 Cherry Lane | Laverton North |
| 48 | Telstra | 3040007 | 53 Brewster Street | Essendon |
| 49 | Telstra | 3046008 | 2 Hartington Street | Glenroy |
| 50 | Telstra | 3053020 | 720 Swanston Street | Carlton |
| 51 | Telstra | 3064002 | 1327 Hume Freeway | Mickleham |
| 52 | Telstra | 3071001 | 739-741 High Street | Thornbury |
| 53 | Telstra | 3072007 | 6B Rennick Street | Preston |
| 54 | Telstra | 3073014 | 229 Broadhurst Avenue | Reservoir |
| 55 | Telstra | 3074003 | 25 Lawson Crescent | Thomastown |
| 56 | Telstra | 3078003 | 36-38 Montefiore Street | Fairfield |
| 57 | Telstra | 3083007 | 289W McKimmies Road | Bundoora |
| 58 | Telstra | 3083020 | La Trobe University, 1040 Plenty Road | Bundoora |
| 59 | Telstra | 3101010 | 2 Chandler Highway | Kew |
| 60 | Telstra | 3124020 | 2-4 Bright Street | Camberwell |
| 61 | Telstra | 3125005 | 301 Burwood Highway | Burwood |
| 62 | Telstra | 3149014 | 8-12 Gilby Road | Mount Waverley |
| 63 | Telstra | 3156006 | 830 Wellington Road | Lysterfield |
| 64 | Telstra | 3159001 | 14 Maskells Hill Road | Selby |
| 65 | Telstra | 3173007 | 48-60 Popes Road | Keysborough |
| 66 | Telstra | 3174002 | 41 Plan 10582 Heatherton Road | Noble Park |
| 67 | Telstra | 3175014 | 8-10 Kitchen Road | Dandenong South |
| 68 | Telstra | 3189008 | 56 Keys Road | Cheltenham |
| 69 | Telstra | 3199014 | 211-279 Skye Road | Frankston |
| 70 | Telstra | 3205042 | 220 Albert Road | South Melbourne |
| 71 | Telstra | 3212002 | 290 Elcho Road | Lovely Banks |
| 72 | Telstra | 3212005 | 130 Coonawarra Drive | Avalon |
| 73 | Telstra | 3212010 | 15 McHarrys Road | Lara |
| 74 | Telstra | 3219005 | 471-489 Wilsons Road | St Albans Park |
| 75 | Telstra | 3228008 | 45 A Geelong Road | Torquay |
| 76 | Telstra | 3340016 | 29 Holts Lane | Darley |
| 77 | Telstra | 3356002 | Miles Street (near Todd Street) | Sebastopol |
| 78 | Telstra | 3429007 | The Heights, Jacksons Hill Estate | Sunbury |
| 79 | Telstra | 3752002 | 152 Cravens Road | Mernda |
| 80 | Telstra | 3759001 | 5 Church Road | Panton Hill |
| 81 | Telstra | 3765002 | Montrose Recreational Reserve, Aileen Avenue | Montrose |
| 82 | Telstra | 3796003 | 138 York Road | Mount Evelyn |
| 83 | Telstra | 3805004 | 1 Fullard Road | Narre Warren |
| 84 | Telstra | 3809002 | Officer Recreation Reserve, Princes Highway | Officer |
| 85 | Telstra | 3810020 | 2-8 Bate Close | Pakenham |
| 86 | Telstra | 3941004 | 745 Truemans Road | Fingal |
| 87 | TPG Telecom Limited | 3000042 | RMIT (Building 51), 80 Victoria Street | Carlton |
| 88 | TPG Telecom Limited | 3011013 | Victoria University, Buckley Street | Footscray |
| 89 | TPG Telecom Limited | 3012015 | Victoria University, 70-100 Ballarat Road | Footscray |
| 90 | TPG Telecom Limited | 3012028 | 260-262 Geelong Road | West Footscray |
| 91 | TPG Telecom Limited | 3016021 | 3-7 Fink Street | Williamstown North |
| 92 | TPG Telecom Limited | 3019009 | 85 Ashley Street | Braybrook |
| 93 | TPG Telecom Limited | 3020020 | 74-76 Sydney Street | Albion |
| 94 | TPG Telecom Limited | 3020026 | 42 Harvester Avenue | Sunshine North |
| 95 | TPG Telecom Limited | 3021008 | 176 Furlong Road | St Albans |
| 96 | TPG Telecom Limited | 3026011 | 1-11 Little Boundary Road | Laverton North |
| 97 | TPG Telecom Limited | 3028003 | 30 Triholm Avenue | Laverton |
| 98 | TPG Telecom Limited | 3028004 | 380 Sayers Road | Tarneit |
| 99 | TPG Telecom Limited | 3029006 | 428 Old Geelong Road | Hoppers Crossing |
| 100 | TPG Telecom Limited | 3030001 | 75 Watton Street | Werribee |
| 101 | TPG Telecom Limited | 3030044 | 30 Fulton Drive | Derrimut |
| 102 | TPG Telecom Limited | 3030047 | 29-31 Windorah Drive | Point Cook |
| 103 | TPG Telecom Limited | 3032004 | Highpoint City Shopping Centre, 120-200 Rosamond Road | Maribyrnong |
| 104 | TPG Telecom Limited | 3039001 | 641-649 Mt Alexander Road | Moonee Ponds |
| 105 | TPG Telecom Limited | 3047001 | 61 Riggall Street | Broadmeadows |
| 106 | TPG Telecom Limited | 3047003 | 1100 Pascoe Vale Road | Broadmeadows |
| 107 | TPG Telecom Limited | 3048002 | 55-63 Paringa Boulevard | Meadow Heights |
| 108 | TPG Telecom Limited | 3051005 | 163-171 Flemington Road | North Melbourne |
| 109 | TPG Telecom Limited | 3073013 | 92 Newlands Road | Reservoir |
| 110 | TPG Telecom Limited | 3074014 | 59 Strong Avenue | Thomastown |
| 111 | TPG Telecom Limited | 3075002 | 50T Gillwell Road | Lalor |
| 112 | TPG Telecom Limited | 3108001 | Westfield Doncaster, 619 Doncaster Road | Doncaster |
| 113 | TPG Telecom Limited | 3124010 | 965-969 Burke Road | Camberwell |
| 114 | TPG Telecom Limited | 3151002 | 643 Highbury Road | Burwood East |
| 115 | TPG Telecom Limited | 3152005 | Knox City Shopping Centre, Burwood Highway | Wantirna South |
| 116 | TPG Telecom Limited | 3163002 | 2 Railway Parade | Murrumbeena |
| 117 | TPG Telecom Limited | 3175038 | 270 Stud Road | Dandenong North |
| 118 | TPG Telecom Limited | 3175039 | 208 Princes Highway | Dandenong |
| 119 | TPG Telecom Limited | 3177005 | 1 Lace Street | Eumemmerring |
| 120 | TPG Telecom Limited | 3181003 | 55 Commercial Road | Melbourne |
| 121 | TPG Telecom Limited | 3182001 | 21 The Esplanade | St Kilda |
| 122 | TPG Telecom Limited | 3189015 | 488-490 South Road | Moorabbin |
| 123 | TPG Telecom Limited | 3205004 | 10 Queens Road | Melbourne |
| 124 | TPG Telecom Limited | 3216003 | 9-29 Cityview Drive | Wandana Heights |
| 125 | TPG Telecom Limited | 3337019 | 206 Coburns Road | Melton West |
| 126 | TPG Telecom Limited | 3338002 | 20-40 Rees Road | Melton South |
| 127 | TPG Telecom Limited | 3752002 | 152 Cravens Road | Mernda |
| 128 | TPG Telecom Limited | 3806003 | 100 Clyde Road | Berwick |
| 129 | TPG Telecom Limited | 3810026 | 130 Thewlis Road | Pakenham |

# Appendix B: Predicted maximum EME vs measured maximum EME

## TPG Telecom Limited

## Optus

## Telstra

# Appendix C: Active vs idle maximum beamformed EME

## TPG Telecom Limited

## Optus

## Telstra

such as the World Health Organisation (WHO) and the International Commission of Non-Ionising Radiation (ICNIRP),

In 2020, TPG and Vodafone Hutchison Australia merged to form TPG Telecom Limited.

5G mobile base station transmitters adjust their power up and down to be just sufficient to manage user demand. Beamformed radio-frequency emissions for 5G are typically very short in nature and are only present when the mobile user’s handset is actively performing a transaction using 5G, therefore an EME measurement technique that activates the 5G transmitter may provide a more accurate representation of 5G EME levels.

To address this the ACMA used the “Maximum level EME for proposed changes” figure calculated in the environmental EME Report as opposed to the current calculated EME figure.

Predicted maximum EME was based on the “Maximum level EME for proposed changes” figure calculated in the environmental EME reports for each site on the date of measurement.