# Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2021 (No. 2)

Radiocommunications Act 1992

The AUSTRALIAN COMMUNICATIONS AND MEDIA AUTHORITY makes this Variation under subsection 132(1) of the Radiocommunications Act 1992.

Dated

Australian Communications and Media Authority

1	Name of instrument

This is the Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2021 (No. 2).

2	Commencement

This instrument commences at the start of the day after the day it is registered on the Federal Register of Legislation.

Note:		The Federal Register of Legislation may be accessed, free of charge, at www.legislation.gov.au.

3	Authority

This instrument is made under subsection 132(1) of the Radiocommunications Act 1992.

4	Variations

The instrument that is specified in Schedule 1 is varied as set out in the items in that Schedule.

Schedule 1	Variations

(section 4)

Radiocommunications (Low Interference Potential Devices) Class Licence 2015 [F2015L01438]

1	Schedule 1 (table item 25, paragraphs (b) and (c))

Repeal the paragraphs, insert:

(b)    43–44, with a carrier frequency of:

(i)        43.05;

(ii)      43.15;

(iii)    43.25;

(iv)    43.35; or

(v)    43.45

2	Schedule 1 (table item 39C)

Repeal the table item, substitute:

| 39C | Fixed telecommand or telemetry transmitters | 928–935 | 25 mW | (a) The maximum radiated power spectral density must not exceed -14.5 dBm/kHz. (b) The maximum duty cycle must not exceed 0.1% averaged over one hour on any given frequency. |
| --- | --- | --- | --- | --- |

3	Schedule 1 (after table item 63)

Insert:

| 63AA | Radio Local Area Network transmitters |  |  | 5925–6425 | 250 mW | (a) The transmitter must only be used indoors. (b) The power spectral density of a transmitter must not exceed 12.5 mW EIRP per MHz. (c) Contention-based protocols for multiple access such as Carrier Sense Multiple Access (CSMA) or Multiple Access Collision Avoidance (MACA) must be implemented. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 63AB | Radio Local Area Network transmitters |  |  | 5925–6425 | 25 mW | (a) The power spectral density of a transmitter must not exceed 1.25 mW EIRP per MHz. (b) Contention-based protocols for multiple access such as Carrier Sense Multiple Access (CSMA) or Multiple Access Collision Avoidance (MACA) must be implemented. |  |