Area-wide apparatus licence device registrations

Supplementary information

November 2020

Canberra

Red Building 
Benjamin Offices
Chan Street 
Belconnen ACT

PO Box 78
Belconnen ACT 2616

T	+61 2 6219 5555
F	+61 2 6219 5353

Melbourne

Level 32 
Melbourne Central Tower
360 Elizabeth Street 
Melbourne VIC

PO Box 13112
Law Courts 
Melbourne VIC 8010

T	+61 3 9963 6800
F	+61 3 9963 6899

Sydney

Level 5 
The Bay Centre
65 Pirrama Road 
Pyrmont NSW

PO Box Q500
Queen Victoria Building 
NSW 1230

T	+61 2 9334 7700 or 1800 226 667
F	+61 2 9334 7799

Copyright notice

https://creativecommons.org/licenses/by/4.0/

With the exception of coats of arms, logos, emblems, images, other third-party material or devices protected by a trademark, this content is made available under the terms of the Creative Commons Attribution 4.0 International (CC BY 4.0) licence.

We request attribution as © Commonwealth of Australia (Australian Communications and Media Authority) 2020.

All other rights are reserved.

The Australian Communications and Media Authority has undertaken reasonable enquiries to identify material owned by third parties and secure permission for its reproduction. Permission may need to be obtained from third parties to re-use their material.

Written enquiries may be sent to:

Manager, Editorial Services
PO Box 13112
Law Courts
Melbourne VIC 8010
Email: info@acma.gov.au

Introduction	1

Supplementary information	1

## Introduction

On 26 October 2020, the ACMA finalised arrangements for area-wide apparatus licences (AWLs) in the 26 GHz and 26 GHz bands. AWLs are a ‘technology’ flexible licence which can authorise the operation of one or more radiocommunications devices within a defined geographic area at a frequency or frequencies specified on the licence, subject to the conditions included in the licence.

In accordance with the Radiocommunications Licence Conditions (Area-Wide Licence) Determination 2020 (the AWL LCD), a radiocommunications transmitter (unless exempt by the AWL LCD) must not be operated in the 24.7-30 GHz range unless it has its details recorded in the Register of Radiocommunications Licences (RRL).

## Supplementary information

In the immediate term, AWL devices are to be registered in the RRL using the existing arrangements for registering devices to be operated under a spectrum licence (using either the ACMA Lodgement Facility (ALF) or Intellidox). This means that some supplementary information is required where the relevance of certain data elements for AWL device registrations may be unclear. As the ACMA is unable to modify these elements in the immediate term the information in this document provides the ‘least-worst’ option. That said, technical elements which are required for assessing coexistence with future services will be able to be accurately recorded for AWL device registrations.

Table 1 provides supplementary information on certain device registration elements. Only the elements which do not have a clear relevance to AWL device registrations are listed here – details of elements not listed in Table 1 are available in the ACMA Lodgement Facility – User Guide.

Submission elements

| Element | Required | Supplementary information |
| --- | --- | --- |
| LegalStatement1 / Legal Declaration | YES | Use the s.145 legal statement / assessment method. Although this is not applicable for AWL device registrations, this element is mandatory, and selection of the other options will activate additional non-applicable elements. |
| LegalStatement2 | NO | Not required – leave blank |
| Level of Protection | NO | Not required – leave blank |
| Level of Protection Unit | NO | Not required – leave blank |
| Level of Protection Reference Bandwidth | YES where LOP stated | Not required – leave blank |
| Reference Bandwidth Unit | YES where LOP stated | Not required – leave blank |
| Spectrum Licence Number | YES | The area-wide apparatus licence number that the submission relates to. One licence per submission. |
| Group Status | YES | Should be always set to false, as group registration is not supported for apparatus licences |
| Effective Radius (group status) | NO | Not required – leave blank |
| Horizontal Radiated Power Pattern | YES | The maximum radiated power pattern from the device in the horizontal plane specified for every degree of azimuthal angles from 0 through to 360 degrees. The reference units should be set at dBm/MHz. |
| Non Directional Antenna / Antenna Type | Yes | A field that specifies that the antenna is non-directional. Although NGSO earth stations use a directional antenna, the azimuth/elevation (tilt) range over which an antenna points is unable to be recorded using the interim device registration arrangements. Therefore, this element should be set to ‘non-directional’ for NGSO earth stations. |
| S145 | YES | Should be set to true or selected, as other assessment methods will activate other non-applicable elements. |
| Guard Space | NO | Not required – leave blank |
| Guard Space Indicator | YES for Guard Space | Not required when S145 method is selected |
| Mitigation Value | YES for Guard Space | Not required when S145 method is selected |
| Agreement | NO | Not required – leave blank or set to ‘core condition agreement not applicable’ |
| Agreement Number | YES for Agreements | Not required – leave blank |
| Core Condition Agreement | NO | Not required – leave blank or select ‘core condition agreement not applicable’ |

It is acknowledged that the existing legal statements for spectrum licensed device registrations are not applicable for AWL device registrations. However, due to the use of the spectrum licence registration process these are unable to be changed in the immediate term. As a temporary measure, references to ‘spectrum licences’ and the Radiocommunications (Subsection 145(3) Certificates) Determination 2012 are taken to mean ‘area-wide apparatus licences’ and ‘the ACMA procedural documents relevant at the time this registration was made’ respectively.