Variation to Licence Area Plan – Mount Gambier Radio – 2022 (No. 1)

The Australian Communications and Media Authority makes the following instrument under subsection 26(2) of the Broadcasting Services Act 1992.

Dated:

Member

Member/General Manager

Australian Communications and Media Authority

1  Name

This is the Variation to Licence Area Plan – Mount Gambier Radio – 2022 (No. 1).

2  Commencement

This instrument commences at the start of the day after the day it is registered on the Federal Register of Legislation.

Note:	The Federal Register of Legislation may be accessed free of charge at www.legislation.gov.au.

3  Authority

This instrument is made under subsection 26(2) of the Broadcasting Services Act 1992.

4  Amendments

The instrument that is specified in Schedule 1 is amended as set out in the applicable items in that Schedule.

Schedule 1—Amendments

(section 4)

Licence Area Plan – Mount Gambier Radio – December 2001 (F2005B00700)

1  Name of instrument

Rename the instrument as the “Licence Area Plan – Mount Gambier Radio”.

2  Title page

Omit:

LICENCE AREA PLAN

MOUNT GAMBIER RADIO

DECEMBER 2001

substitute:

Licence Area Plan – Mount Gambier Radio

3  Heading before clauses

Omit “– DECEMBER 2001”.

4  Clauses

Number each of the clauses consecutively as clauses (1), (2), (3) and (4).

5  Clause (1)

Repeal the clause, substitute:

(1)	This plan for broadcasting services in the Mount Gambier area of South Australia and Victoria is made under subsection 26(1) of the Broadcasting Services Act 1992.

6  Clause (2)

Omit “Determination”, substitute “plan” (each occurrence).

7  Clause (3)

Omit “Determination”, substitute “plan” (each occurrence).

8  Clause (3)

Omit “Attachment 2.2 - 2.3”, substitute “Attachment 2.3”.

9  Clause (4)

Omit “Determination”, substitute “plan” (each occurrence).

10  After clause (4)

Insert:

(5)	In this plan, unless the contrary intention appears:

(a)	a reference to a schedule or an attachment is a reference to a schedule or an attachment to this plan;

(b)	a reference to a legislative instrument is a reference to that instrument as in force from time to time;

(c)	a reference to any other kind of instrument or writing is a reference to that other kind of instrument or writing as in force or in existence at the time the reference was included in this plan.

Note 1:	For references to Commonwealth Acts, see section 10 of the Acts Interpretation Act 1901; and see also subsection 13(1) of the Legislation Act 2003 for the application of the Acts Interpretation Act 1901 to legislative instruments.

Note 2:	All Commonwealth Acts and legislative instruments are registered on the Federal Register of Legislation.

11  After clause (5)

Omit “(Determined on 6 December 2001)”.

12  Before Schedule One

Omit:

(a)	the heading “SCHEDULES”, and the line of text and the eight dot points after it; and

(b)	the heading “Status Column” and the five paragraphs of text after it; and

(c)	the heading “Frequency Column”, and the dot point and the paragraph after it.

13  Schedule One

Repeal the schedule, substitute:

Schedule One

Licence Area Plan : Mount Gambier Radio

Licence Area : MT GAMBIER RA1

| Service Category | Channel/ Frequency | Service Licence No | Transmitter Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- |
| National | 1476 kHz | N/A | TS10011285 | 1.3 | Mount Gambier |
|  | 1161 kHz | N/A | TS4058001 | 1.4 | Naracoorte |
|  | 102.5 MHz | N/A | TS7565001 | 1.5 | Mount Gambier |
|  | 103.3 MHz | N/A | TS7566001 | 1.6 | Mount Gambier |
|  | 104.1 MHz | N/A | TS3023001 | 1.7 | Mount Gambier |
|  | 105.7 MHz | N/A | TS10010696 | 1.8 | Mount Gambier |
| Commercial | 963 kHz | SL4197 | TS4197001 | 1.9 | Mount Gambier |
|  | 90.5 MHz | SL4197 | TS12000915 | 1.9A | Mount Gambier |
|  | 101.7 MHz | SL4197 | TS12000916 | 1.9B | Naracoorte |
| Commercial | 96.1 MHz | SL10312 | TS10004702 | 1.10 | Mount Gambier |
|  | 100.9 MHz | SL10312 | TS10010529 | 1.11 | Naracoorte |
| Community | 104.9 MHz | SL1150784 | TS10010695 | 1.12 | Mount Gambier |
|  | 99.3 MHz | SL1150784 | TS10010698 | 1.13 | Naracoorte |

14  Schedule Two

Repeal the schedule, substitute:

Schedule Two

Licence Area Plan : Mount Gambier Radio

Licence Area : MOUNT GAMBIER RA2

| Service Category | Channel/ Frequency | Service Licence No | Transmitter Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- |
| Community | 100.1 MHz | SL3075 | TS10010697 | 2.3 | Mount Gambier |

15  Schedule Three

Repeal the schedule, substitute:

Schedule Three

Licence Area Plan : Mount Gambier Radio

Licence Area : MILLICENT RA1

| Service Category | Channel/ Frequency | Service Licence No | Transmitter Specification No | Attach No | Area Served |
| --- | --- | --- | --- | --- | --- |
| Community | 107.7 MHz | SL5703 | TS5703001 | 3.2 | Millicent |

16  Attachments

Omit all the attachments, and insert the following attachments.

Attachment 1.1

Licence Area – MT GAMBIER RA1

Commercial Radio Service Licence numbers:  SL4197 and SL10312

Community Radio Service Licence number:  SL1150784

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 5 August 2001, is:

Area Description

Mount Gambier (C)

Wattle Range (DC)

Grant (DC)

SA CD 4050807

SA CD 4050803

SA CD 4050804

SA CD 4050806

SA CD 4050808

SA CD 4050802

SA CD 4050809

SA CD 4050801

SA CD 4050603

SA CD 4050703

SA CD 4050705

SA CD 4050604

SA CD 4050704

SA CD 4050602

VIC CD 2091201

SA CD 4050706

SA CD 4050805

VIC CD 2091304

VIC CD 2091109

SA CD 4050503

VIC CD 2091110

Note:

Standard terminology used by the Australian Bureau of Statistics:

(C) 	=  	City

(CD) 	=  	Collection District

(DC) 	=  	District Council

Attachment 1.3

LICENCE AREA PLAN : Mount Gambier Radio

Category :	National

General Area Served :	Mount Gambier (SA)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS10011285

Transmitter Site :-

Nominal Location :	Broadcast Australia Site 58 Crouch Street North 
	MOUNT GAMBIER

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-37.824423	140.790216

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	MF-AM

Carrier Frequency :	1476 kHz

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| At all angles of azimuth | 0 | 220 V |

Special Condition :-

The CMF at all elevations must not exceed the specified maximum CMF at 0 degrees elevation for all angles of azimuth.

Attachment 1.4

LICENCE AREA PLAN : Mount Gambier Radio

Category :	National

General Area Served :	Naracoorte (SA)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS4058001

Transmitter Site :-

Nominal Location :	Broadcast Australia Site Donohughes Ln 
	off Messamurray Rd STEWART RANGE

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-36.940414	140.670397

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	MF-AM

Carrier Frequency :	1161 kHz

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| 0°T | 0 | 1.65 kV |
| 10°T | 0 | 1.365 kV |
| 20°T | 0 | 1 kV |
| 30°T | 0 | 595 V |
| 40°T | 0 | 205 V |
| 50°T | 0 | 200 V |
| 60°T | 0 | 470 V |
| 70°T | 0 | 670 V |
| 80°T | 0 | 790 V |
| 90°T | 0 | 825 V |
| 100°T | 0 | 790 V |
| 110°T | 0 | 670 V |
| 120°T | 0 | 470 V |
| 130°T | 0 | 200 V |
| 140°T | 0 | 205 V |
| 150°T | 0 | 595 V |
| 160°T | 0 | 1 kV |
| 170°T | 0 | 1.365 kV |
| 180°T | 0 | 1.65 kV |
| 190°T | 0 | 1.81 kV |
| 200°T | 0 | 1.835 kV |
| 210°T | 0 | 1.745 kV |
| 220°T | 0 | 1.575 kV |
| 230°T | 0 | 1.36 kV |
| 240°T | 0 | 1.15 kV |
| 250°T | 0 | 980 V |
| 260°T | 0 | 865 V |
| 270°T | 0 | 825 V |
| 280°T | 0 | 865 V |
| 290°T | 0 | 980 V |
| 300°T | 0 | 1.15 kV |
| 310°T | 0 | 1.36 kV |
| 320°T | 0 | 1.575 kV |
| 330°T | 0 | 1.745 kV |
| 340°T | 0 | 1.835 kV |
| 350°T | 0 | 1.81 kV |

Special Condition :-

The CMF at all elevations must not exceed the specified maximum CMF at 0 degrees elevation for all angles of azimuth.

Attachment 1.5

LICENCE AREA PLAN : Mount Gambier Radio

Category :	National

General Area Served :	Mount Gambier (SA)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION – FM Radio

Specification Number :	TS7565001

Transmitter Site :-

Nominal Location :	Broadcast site 11 km E of Millicent MT BURR

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-37.602350	140.482492

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	102.5 MHz

Polarisation :	Mixed

Maximum Antenna Height :	44 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 25°T | 240 kW |
| 25°T - 205°T | 100 kW |
| 205°T - 325°T | 80 kW |
| 325°T - 360°T | 240 kW |

Attachment 1.6

LICENCE AREA PLAN : Mount Gambier Radio

Category :	National

General Area Served :	Mount Gambier (SA)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS7566001

Transmitter Site :-

Nominal Location :	Broadcast site 11 km E of Millicent MT BURR

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-37.602350	140.482492

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	103.3 MHz

Polarisation :	Mixed

Maximum Antenna Height :	44 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 25°T | 240 kW |
| 25°T - 205°T | 100 kW |
| 205°T - 325°T | 80 kW |
| 325°T - 360°T | 240 kW |

Attachment 1.7

LICENCE AREA PLAN : Mount Gambier Radio

Category :	National

General Area Served :	Mount Gambier (SA)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS3023001

Transmitter Site :-

Nominal Location :	Broadcast site 11 km E of Millicent MT BURR

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-37.602350	140.482492

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	104.1 MHz

Polarisation :	Mixed

Maximum Antenna Height :	44 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 25°T | 240 kW |
| 25°T - 205°T | 100 kW |
| 205°T - 325°T | 80 kW |
| 325°T - 360°T | 240 kW |

Attachment 1.8

LICENCE AREA PLAN : Mount Gambier Radio

Category :	National

General Area Served :	Mount Gambier (SA)

Service Licence Number :	Not applicable

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10010696

Transmitter Site :-

Nominal Location :	Broadcast site 11 km E of Millicent MT BURR

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-37.602350	140.482492

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	105.7 MHz

Polarisation :	Mixed

Maximum Antenna Height :	44 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 25°T | 240 kW |
| 25°T - 205°T | 100 kW |
| 205°T - 325°T | 80 kW |
| 325°T - 360°T | 240 kW |

Attachment 1.9

LICENCE AREA PLAN : Mount Gambier Radio

Category :	Commercial

General Area Served :	Mount Gambier (SA)

Service Licence Number :	SL4197

TECHNICAL SPECIFICATION - AM Radio

Specification Number :	TS4197001

Transmitter Site :-

Nominal Location :	Jubilee Hwy West 6km NW of MOUNT GAMBIER

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-37.795568	140.721937

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	MF-AM

Carrier Frequency :	963 kHz

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Elevation | Maximum CMF |
| --- | --- | --- |
| 0°T | 0 | 950 V |
| 10°T | 0 | 840 V |
| 20°T | 0 | 750 V |
| 30°T | 0 | 530 V |
| 40°T | 0 | 380 V |
| 47°T | 13 | 250 V |
| 50°T | 0 | 170 V |
| 53°T | 16 | 200 V |
| 60°T | 0 | 120 V |
| 70°T | 0 | 300 V |
| 80°T | 0 | 470 V |
| 90°T | 0 | 600 V |
| 100°T | 0 | 750 V |
| 110°T | 0 | 840 V |
| 114°T | 5 | 900 V |
| 120°T | 0 | 840 V |
| 130°T | 0 | 950 V |
| 140°T | 0 | 950 V |
| 150°T | 0 | 840 V |
| 160°T | 0 | 840 V |
| 170°T | 0 | 750 V |
| 180°T | 0 | 670 V |
| 190°T | 0 | 530 V |
| 200°T | 0 | 420 V |
| 210°T | 0 | 210 V |
| 220°T | 0 | 84 V |
| 230°T | 0 | 240 V |
| 240°T | 0 | 420 V |
| 250°T | 0 | 600 V |
| 260°T | 0 | 750 V |
| 270°T | 0 | 840 V |
| 280°T | 0 | 950 V |
| 290°T | 0 | 1.1 kV |
| 300°T | 0 | 1.1 kV |
| 310°T | 0 | 1.1 kV |
| 320°T | 0 | 1.1 kV |
| 329°T | 27 | 900 V |
| 330°T | 0 | 1.1 kV |
| 340°T | 0 | 1.1 kV |
| 350°T | 0 | 950 V |

Special Condition :-

The CMF at all elevations must not exceed the specified maximum CMF at 0 degrees elevation for all angles of azimuth.

Availability of TS4197001:-

TS4197001 ceases to be available for SL4197 28 days after the first day on which the service provided under SL4197 is transmitted by transmitter TS12000915 in accordance with the specifications set out at Attachment 1.9A.

Attachment 1.9A

LICENCE AREA PLAN : Mount Gambier Radio

Category :	Commercial

General Area Served :	Mount Gambier (SA)

Service Licence Number :	SL4197

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS12000915

Transmitter Site :-

Nominal Location :	Broadcast site 11 km E of Millicent MT BURR

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-37.602350	140.482492

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	90.5 MHz

Polarisation :	Mixed

Maximum Antenna Height :	110 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 70°T | 20 kW |
| 70°T - 105°T | 10 kW |
| 105°T - 115°T | 5 kW |
| 115°T - 240°T | 20 kW |
| 240°T - 280°T | 5 kW |
| 280°T - 360°T | 20 kW |

Attachment 1.9B

LICENCE AREA PLAN : Mount Gambier Radio

Category :	Commercial

General Area Served :	Naracoorte (SA)

Service Licence Number :	SL4197

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS12000916

Transmitter Site :-

Nominal Location :	Council Site off Smith St and Wimmera Hwy  
	3km ENE of NARACOORTE

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-36.950761	140.769491

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	101.7 MHz

Polarisation :	Mixed

Maximum Antenna Height :	20 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 100 W |

Attachment 1.10

LICENCE AREA PLAN : Mount Gambier Radio

Category :	Commercial

General Area Served :	Mount Gambier (SA)

Service Licence Number :	SL10312

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10004702

Transmitter Site :-

Nominal Location :	Broadcast site 11 km E of Millicent MT BURR

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-37.602350	140.482492

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	96.1 MHz

Polarisation :	Mixed

Maximum Antenna Height :	110 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| 0°T - 70°T | 20 kW |
| 70°T - 105°T | 10 kW |
| 105°T - 115°T | 5 kW |
| 115°T - 240°T | 20 kW |
| 240°T - 280°T | 5 kW |
| 280°T - 360°T | 20 kW |

Attachment 1.11

LICENCE AREA PLAN : Mount Gambier Radio

Category :	Commercial

General Area Served :	Naracoorte (SA)

Service Licence Number :	SL10312

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10010529

Transmitter Site :-

Nominal Location :	Council Site off Smith St and Wimmera Hwy  
	3km ENE of NARACOORTE

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-36.950761	140.769491

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	100.9 MHz

Polarisation :	Mixed

Maximum Antenna Height :	20 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 100 W |

Attachment 1.12

LICENCE AREA PLAN : Mount Gambier Radio

Category :	Community

General Area Served :	Mount Gambier (SA)

Service Licence Number :	SL1150784

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10010695

Transmitter Site :-

Nominal Location :	Broadcast site 11 km E of Millicent MT BURR

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-37.602350	140.482492

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	104.9 MHz

Polarisation :	Mixed

Maximum Antenna Height :	50 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 20 kW |

Attachment 1.13

LICENCE AREA PLAN : Mount Gambier Radio

Category :	Community

General Area Served :	Naracoorte (SA)

Service Licence Number :	SL1150784

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10010698

Transmitter Site :-

Nominal Location :	Council Site off Smith St and Wimmera Hwy  
	3km ENE of NARACOORTE

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-36.950761	140.769491

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	99.3 MHz

Polarisation :	Mixed

Maximum Antenna Height :	20 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 100 W |

Attachment	2.1

Licence Area – MOUNT GAMBIER RA2

Community Radio Service Licence number:  SL3075

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 5 August 2001, is:

Area Description

Mount Gambier (C)

SA CD 4051305

SA CD 4051310

SA CD 4051311

SA CD 4051312

SA CD 4051304

SA CD 4051308

SA CD 4051307

SA CD 4051309

SA CD 4051302

SA CD 4051306

SA CD 4051303

SA CD 4051301

Note:

Standard terminology used by the Australian Bureau of Statistics:

(C) 	=  	City

(CD) 	=  	Collection District

Attachment 2.3

LICENCE AREA PLAN : Mount Gambier Radio

Category :	Community

General Area Served :	Mount Gambier (SA)

Service Licence Number :	SL3075

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS10010697

Transmitter Site :-

Nominal Location :	Link Site Hawkins Road MOUNT GAMBIER

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-37.841306	140.830253

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	100.1 MHz

Polarisation :	Mixed

Maximum Antenna Height	36 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 1 kW |

Attachment	3.1

Licence Area – MILLICENT RA1

Community Radio Service Licence number:  SL5703

The licence area, in terms of areas defined by the Australian Bureau of Statistics at the Census of 5 August 2001, is:

Area Description

SA CD 4051105

SA CD 4051202

SA CD 4051101

SA CD 4051203

SA CD 4051204

SA CD 4051208

SA CD 4051210

SA CD 4051209

SA CD 4051102

SA CD 4051201

SA CD 4051004

SA CD 4051103

SA CD 4051207

SA CD 4051003

SA CD 4050907

SA CD 4051206

SA CD 4051106

SA CD 4051104

SA CD 4051205

Note:

Standard terminology used by the Australian Bureau of Statistics:

(CD) 	=  	Collection District

Attachment 3.2

LICENCE AREA PLAN : Mount Gambier Radio

Category :	Community

General Area Served :	Millicent (SA)

Service Licence Number :	SL5703

TECHNICAL SPECIFICATION - FM Radio

Specification Number :	TS5703001

Transmitter Site :-

Nominal Location :	Broadcast site 11 km E of Millicent MT BURR

Nominal Co-ordinates 	Latitude	Longitude
(GDA94) :	-37.602350	140.482492

Site Tolerance :	Refer to Broadcasting Services 
	(Technical Planning) Guidelines 2017

Emission :-

Frequency Band & Mode :	VHF-FM

Carrier Frequency :	107.7 MHz

Polarisation :	Mixed

Maximum Antenna Height :	35 m

Output Radiation Pattern :-

| Bearing or Sector (clockwise direction) | Maximum ERP |
| --- | --- |
| At all angles of azimuth | 200 W |