Attachment 4a

# Planning Model for Low Power Open Narrowcasting (LPON) Services

Frequency Range

The FM radio sub-band 87.5-88.0 MHz has been made available to accommodate LPON services.

Channels

The channels centred on the frequencies 87.6 MHz, 87.8 MHz and 88.0 MHz may be used for LPON services.

The order of frequency assignment in an area is 88.0 MHz and 87.6 MHz, in either order, followed by 87.8 MHz; co-channel and other frequency availability restrictions permitting.

Frequency Availability Restrictions

There are restrictions on the availability of all, or some, of these frequencies in certain areas of Australia because of the potential for interference to the reception of broadcasting services.

VHF television channel 3

Table 1 lists VHF television channel 3 transmitter locations and protection radii.

Important note: NO LPON services are to be assigned within these protection radii.

Table 1 - Television Broadcasting Services on VHF Television Channel 3

Key: E = Easting

N = Northing

| Television Transmitter Site | Site Coordinates AMG (AGD 66) | Area Served | Effective Radiated Power | Protection Radius (kilometres) |
| --- | --- | --- | --- | --- |
| ABN3 - 3 km south of Nyngan, NSW | Zone 55 E 517100 N 6505000 | Nyngan, NSW | 20 watts | 30 |
| NBN3 – Mt Sugarloaf, NSW | Zone 56 E 363200 N 6359800 | Newcastle/Hunter River, NSW | 200,000 watts | (a) see below |
| WIN 3 – Buckeridge Lookout, NSW | Zone 56 E 237700 N 5990300 | Narooma, NSW | 1000 watts | 50 |
| WIN3 - Brokers Nose, NSW | Zone 56 E 304602 N 6195003 | Wollongong, NSW | 7900 watts | 50 |
| ABTQ3 - Mt Stuart, QLD | Zone 55 E 476825 N 7860960 | Townsville, QLD | 160,000watts | 130 |
| ABRS3 - 4 km west south west of Loxton, SA | Zone 54 E 457030 N 6186070 | Renmark/Loxton, SA | 240,000watts | 120 |
| SSW3 - Mt Lennard, WA | Zone 50 E 398943 N 6304074 | Bunbury, WA | 100,000watts | 150 |
| VEW3 - Nareling Hill, WA | Zone 50 E 561500 N 650 1600 | Baandee, W | 1000 watts | 50 |
| VEW3 - Red Hill, WA | Zone 51 E 373460 N 6546197 | Kambalda, WA | 20 watts | 30 |

(a) To afford sufficient protection to the Newcastle and Wollongong Channel 3 reception areas, the LPONS transmitters are excluded from siting in the following locations:

- Within a 50 km radius from the coordinates 34° 22’ 04’’S, 150° 52’ 30’’E (WIN-3 transmitter location).

- Within a 150 km radius from the coordinates 32° 53’ 29’’S, 151° 32’ 15’’E (NBN-3 transmitter location) except those locations within the exempted region.

The ‘exempted region’ consists of those locations which are outside the 50 km WIN-3 exclusion zone but within the 150 km NBN-3 exclusion zone, and are west of 150° 50’E and south of the line joining the coordinates 33°’S, 150° 50’E and 32° 30’S, 150°E.

FM Radio Broadcasting Services below 88.4 MHz

Table 2 lists the locations and protection radii for FM radio broadcasting transmitters using 88.1 MHz and 88.3 MHz.

Important Note:

- within the radii for 88.1 MHz transmitters, no LPON services are to be assigned;

- within the radii for 88.3 MHz transmitters, no 87.8 MHz or 88.0 MHz LPON services are to be assigned; and

- within the radii for 88.3 MHz transmitters, LPON services to operate on or below 87.6 MHz may be assigned.

Table 2 - FM Broadcasting Services on 88.1 MHz and 88.3 MHz

| Frequency (MHz) | FM Radio Callsign | Site Coordinates AMG (AGD 66) | Area Covered | Effective Radiated Power | Protection Radius (kilometres) |
| --- | --- | --- | --- | --- | --- |
| 88.1 MHz | 2ABCFM | Zone 55 E 601300 N 5991800 | Khancoban, NSW | 36 watts | 30 |
|  | 2ABCFM | Zone 55 E 616600 N 6060000 | Talbingo, NSW | 4 watts | 10 |
|  | 2RDJ | Zone 56 E 324600 N 6249200 | Burwood, NSW | 100 watts | 50 |
|  | 2RVR/T1 | Zone 55 E 314920 N 6185673 | Hay, NSW | 6000 watts | 100 |
|  | 3ABCFM | Zone 55 E 497499 N 5936495 | Bright, VIC | 50 watts | 30 |
|  | 3MFM | Zone 55 E 392200 N 5739300 | Leongatha, VIC | 2000 watts | 80 |
|  | 4HJ/T1 | Zone 55 E 555200 N 7490300 | Blair Athol, QLD | 200 watts | 50 |
|  | 8ABC | Zone 52 E 720776 N 8480826 | Tipperary Station via Daly River, NT | 2 watts | 10 |
|  | 8ABCRN | Zone 53 E 234400 N 8599900 | South Alligator, NT | 2 watts | 50 |
| 88.3 MHz | 2ABCFM | Zone 55 E 582311 N 6146329 | South West Slopes, NSW | 80,000 watts | 100 |
|  | 2UUS | Zone 56 E 284623 N 6219812 | Camden, NSW | 100 watts | 25 |
|  | 3SCB | e 55 E 327260 N 5799540 | Southern South Melbourne, VIC | 200 watts | 30 |
|  | 4ABCRR | Zone 56 E 360282 N 7310324 | Miriam Vale, QLD | 100 watts | 30 |
|  | 3ABCRN | Zone 54 E 616375 N 6031400 | Hopetoun, VIC | 500 watts | 30 |

Footnote 1 - T = translator

# LPON Transmitter Technical Characteristics

Radiated Power

Table 3

| Location* | Maximum Equivalent Radiated Power (ERP) |
| --- | --- |
| Residential | 1 Watt (1.64 watts Equivalent Isotropic Radiated Power) |
| Non-residential | 10 Watts (16.4 watts Equivalent Isotropic Radiated Power) |

* A residential area is an area within 20 kilometres of the boundary of an Urban Centre or Locality as defined by the Australian Bureau of Statistics (ABS) in the 2001 Census. A complete list of such areas can be found on the ABS website at http://www.abs.gov.au/AUSSTATS/abs@.nsf/DetailsPage/2909.02001?OpenDocument.

A non-residential area is an area at least 20 kilometres from the boundary of an Urban Centre or Locality.

Antenna

Type - Unspecified

Polarisation – Unspecified

Emission Designation

- 200KF8EHF for stereophonic or quadraphonic sound;

- 200KF3EGF monophonic sound.

Minimum Separation Distance Requirements

Co-channel and adjacent channel minimum separation requirements for LPON services are listed in Table 4.

Table 4 - Minimum Separation Distances between LPON Transmitters

In addition to conforming to the planning requirements above, LPON transmitters must maintain the minimum separation distances specified below. For example, a one watt transmitter operating at 87.6 MHz must be situated at least 10km from the nearest one watt transmitter operating at 87.6 MHz and at least 5km from the nearest one watt transmitter operating at 87.8 MHz. It can operate at the same site as a transmitter operating at 88.0 MHz (see column 3).

| LPON Carrier Frequency Separation (kHz) | Required Protection Ratio in decibels (dB) | Separation Distance between 1 watt LPON transmitters in kilometres (km) | Separation Distance between 10 watt LPON transmitters (km) | Separation Distance between 1 watt and 10 watt LPON transmitters (km) |
| --- | --- | --- | --- | --- |
| 0 | 45 | 10 | 30 | 20 |
| 200 | 27 | 5 | 15 | 10 |
| 400 | -18 | 0 | 0 | 0 |