Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2022 (No. 2)

The Australian Communications and Media Authority makes the following variation under subsection 132(1) of the Radiocommunications Act 1992.

Dated:

Member

Member/General Manager

Australian Communications and Media Authority

1  Name

This is the Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2022 (No. 2).

2  Commencement

This instrument commences at the start of the day after the day it is registered on the Federal Register of Legislation.

Note:	The Federal Register of Legislation may be accessed free of charge at www.legislation.gov.au.

3  Authority

This instrument is made under subsection 132(1) of the Radiocommunications Act 1992.

4  Variations

The instrument that is specified in Schedule 1 is varied as set out in the applicable items in that Schedule.

Schedule 1—Variations

Radiocommunications (Low Interference Potential Devices) Class Licence 2015 (F2015L01438)

1  Subsection 3A(1)

Insert:

indoors means a space on or above land that is:

(a)	enclosed by permanent walls on all sides, a permanent roof and a permanent floor; and

(b)	permanently fixed to the land.

2  Subsection 3A(1) (Note 2 to the subsection)

Omit the note, substitute:

Note 2	Words and phrases used in this Class Licence may be defined in an instrument made under section 64 of the Australian Communications and Media Authority Act 2005, including:

- datacasting service station;

- earth receive station;

- harmful interference;

- Radio Regulations;

- space station.

3  After subsection 4(3) (including the notes)

Insert:

Authorisation of earth receive stations

(4)	This Class Licence authorises a person to operate an earth receive station, subject to the following conditions:

(a)	the earth receive station must be operated:

(i)	on a frequency, or within a range of frequencies, mentioned in column 1 of an item in Schedule 1A; and

(ii)	in accordance with the limitations (if any) mentioned in column 2 of that item; and

(b)	a person who operates the earth receive station must not operate a space station, other than a space station operated in accordance with the Radio Regulations, to transmit to the earth receive station; and

(c)	a person who operates the earth receive station must not have an agreement, arrangement or understanding:

(i)	whether formal or informal, or partly formal and partly informal; and

(ii)	whether written or oral, or partly written and partly oral; and

(iii)	whether or not having legal or equitable force and whether or not based on legal or equitable rights;

with another person to operate a space station, other than a space station operated in accordance with the Radio Regulations, to transmit to the earth receive station.

Note 1	An earth receive station operated in accordance with subsection (4) will not be afforded protection from interference caused by other radiocommunication devices.

Note 2	For more information regarding paragraphs (4)(b) and (c) and Note 1, see Article No. 4.4 of the Radio Regulations, and the Rules of Procedure approved by the Radio Regulations Board of the International Telecommunication Union.

Note 3	Additional limitations in Schedule 1A may apply in relation to the operation of space stations to transmit to an earth receive station authorised under this Class Licence.

4  Schedule 1 (after table item 57)

Insert:

| 57A | Frequency hopping transmitters | 5925–6425 | 25 mW | (a) The power spectral density of the transmitter must not exceed 10 mW EIRP per MHz. (b) A minimum of 15 hopping frequencies must be used. (c) Emissions must have a maximum bandwidth of 20 MHz. (d) Emissions below 5925 MHz must be no greater than –37 dBm EIRP. |
| --- | --- | --- | --- | --- |

5  Schedule 1 (table item 61)

Omit the item, substitute:

| 61 | Radio Local Area Network transmitters | 5150–5250 | 1 W (averaged over the entire transmission burst) | The power spectral density of the transmitter must not exceed 200 mW (23 dBm) EIRP, in any direction, above 5 degrees of elevation. OR The power spectral density of the transmitter must not exceed 125 mW (21 dBm) EIRP, in any direction, above 30 degrees of elevation. |
| --- | --- | --- | --- | --- |

OR

5  Schedule 1 (after table item 61)

Insert:

| 61A | Radio Local Area Network transmitters | 5150–5250 | 1 W (averaged over the entire transmission burst) | The power spectral density of the transmitter must not exceed 200 mW (23 dBm) EIRP, in any direction, above 5 degrees of elevation. OR The power spectral density of the transmitter must not exceed 125 mW (21 dBm) EIRP, in any direction, above 30 degrees of elevation. |
| --- | --- | --- | --- | --- |

6  Schedule 1 (table item 63AA, column 4, after paragraph (c))

Insert:

(d)	Emissions below 5925 MHz must be no greater than –27 dBm EIRP.

7  Schedule 1 (table item 63AB, column 4, after paragraph (b))

Insert:

(c)	Emissions below 5925 MHz must be no greater than –37 dBm EIRP.

8  Schedule 1 (table item 63A column 4, paragraph (g))

Omit the paragraph.

9  Schedule 1 (table item 63B, column 4, paragraph (g))

Omit the paragraph.

10  After Schedule 1

Insert:

Schedule 1A	Earth receive stations

(subsection 4(4))

Table—Earth receive stations

|  | Column 1 Permitted operating frequency band (MHz) (lower limit exclusive, upper limit inclusive) | Column 2 Limitations |  |
| --- | --- | --- | --- |
| Earth receive stations for non-specific applications |  |  |  |
| 1 | 915-928 | (a) A person who operates the earth receive station must not operate a space station, other than a space station that operates in accordance with items 20, 54 or 58 of Schedule 1, to transmit to the earth receive station. (b) A person who operates the earth receive station must not have an agreement, arrangement or understanding: (i) whether formal or informal, or partly formal and partly informal; and (ii) whether written or oral, or partly written and partly oral; and (iii) whether or not having legal or equitable force and whether or not based on legal or equitable rights; with another person to operate a space station, other than a space station that operates in accordance with items 20, 54 or 58 of Schedule 1, to transmit to the earth receive station. |  |
| 2 | 2400–2483.5 | (a) A person who operates the earth receive station must not operate a space station, other than a space station that operates in accordance with items 21, 40, 43, 55, 56 or 59 of Schedule 1, to transmit to the earth receive station. (b) A person who operates the earth receive station must not have an agreement, arrangement or understanding: (i) whether formal or informal, or partly formal and partly informal; and (ii) whether written or oral, or partly written and partly oral; and (iii) whether or not having legal or equitable force and whether or not based on legal or equitable rights; with another person to operate a space station, other than a space station that operates in accordance with items 21, 40, 43, 55, 56 or 59 of Schedule 1, to transmit to the earth receive station. |  |