Coexistence between radio altimeters operating in 4200–4400 MHz and wireless broadband systems in 3400–4000 MHz

Report

FEBRUARY 2023

Canberra

Red Building 
Benjamin Offices
Chan Street 
Belconnen ACT

PO Box 78
Belconnen ACT 2616

T	+61 2 6219 5555
F	+61 2 6219 5353

Melbourne

Level 32 
Melbourne Central Tower
360 Elizabeth Street 
Melbourne VIC

PO Box 13112
Law Courts 
Melbourne VIC 8010

T	+61 3 9963 6800
F	+61 3 9963 6899

Sydney

Level 5 
The Bay Centre
65 Pirrama Road 
Pyrmont NSW

PO Box Q500
Queen Victoria Building 
NSW 1230

T	+61 2 9334 7700
F	+61 2 9334 7799

Copyright notice

https://creativecommons.org/licenses/by/4.0/

With the exception of coats of arms, logos, emblems, images, other third-party material or devices protected by a trademark, this content is made available under the terms of the Creative Commons Attribution 4.0 International (CC BY 4.0) licence.

We request attribution as © Commonwealth of Australia (Australian Communications and Media Authority) 2023.

All other rights are reserved.

The Australian Communications and Media Authority has undertaken reasonable enquiries to identify material owned by third parties and secure permission for its reproduction. Permission may need to be obtained from third parties to re-use their material.

Written enquiries may be sent to:

Manager, Editorial Services
PO Box 13112
Law Courts
Melbourne VIC 8010
Email: info@acma.gov.au

1.	Executive summary	6

The issue	6

Background	6

Observations	7

Proposed approach	8

Purpose and outline	11

2.	Background	12

Overview	12

Recent activities	15

3.	Coexistence evaluation	16

Spectrum management regulatory environment	16

International	16

Domestic	17

Interference mechanisms	17

Receiver front-end overload	17

Receiver desensitization	17

False altitude reports	18

Studies		18

Over-the-air (OTA) tests and trials	26

Historical and contemporary deployments	29

Observed historical possible interference events	29

4.	Observations, conclusions and proposed approach	31

Difference in approaches between national spectrum regulators	31

Variation in views between industry sectors	32

Alignment between theory and experience concerning interference risk	33

Performance of radio altimeters	33

Conclusions and proposed approach	34

5.	Appendix A: Wireless Broadband	47

Overview	47

Technical characteristics	47

6.	Appendix B: Radio altimeters	50

Overview	50

Technical characteristics	53

Future radio altimeter performance	54

7.	Appendix C: International developments and approaches	56

United States	56

Japan		61

United Kingdom	62

France		65

Denmark	66

Switzerland	66

Canada (initial proposed approach)	67

Canada (revised proposed approach)	69

Belgium		73

Czechia		73

Saudi Arabia	73

Brazil		74

CEPT/ECC	74

ICAO/FSMP	77

IATA		80

Eurocontrol	80

RTCA/AVSI	80

8.	Appendix D: Development of proposed interim mitigations	83

Summary	83

Airport mitigations	83

Methodology	84

Application of methodology to the Australian environment	89

Comparison with French and Canadian results	98

General interim mitigations	101

Summary of proposed mitigations	107

Frequency boundary for mitigations	109

9.	Appendix E: Historical Australian WBB deployments	111

Summary	111

Historical NBN fixed wireless access (FWA)	111

Unwired network	114

Summary of Australian 3.4–3.7 GHz spectrum-licensed deployments (January 2023)	118

10.	Appendix F: Relevant overseas WBB deployments	125

Japan	125

Denmark	131

Finland	131

Switzerland	132

Ireland	133

United Kingdom	135

11.	Appendix G: Identified runways	138

# Executive summary

## The issue

Coexistence between radio altimeters operating in the 4200–4400 MHz band and wireless broadband (WBB) systems is under consideration globally with the planned introduction of new WBB systems below (and above) the band used by radio altimeters.

Radio altimeters that operate in 4200–4400 MHz are critical sensors that provide high integrity, accurate measurements of aircraft (commercial, civil and military) height above terrain and obstacles. They are used by many aircraft (both fixed and rotary wing) to enable and enhance flight safety and operations.

Spectrum in the 3400–4200 MHz range is of particular interest globally as an early or ‘pioneer’ band for 5G WBB (in addition to 4G and earlier broadband services in parts of the range), with extensive deployments worldwide in varying parts of the band. In Australia, fixed and mobile WBB systems have operated in parts of the band between 3400–3700 MHz for over 2 decades.

## Background

Within Australia, radio altimeter and WBB coexistence has been considered during the review of spectrum management arrangements in the 3700–4200 MHz band. This review began with a discussion paper in 2019, followed by an options paper in 2020. When outcomes for this review were announced in 2021, the ACMA stated its intention to develop spectrum management arrangements for WBB systems up to 4000 MHz only.

This decision was made partly due to coexistence considerations with radio altimeters, to provide a 200 MHz guard band (4000–4200 MHz) between radio altimeters and any future WBB use. When stating this view, the ACMA also indicated that when implementing arrangements for WBB up to 4000 MHz, we would consider further what, if any, additional mitigations were necessary to achieve coexistence with radio altimeters.

To further that commitment, the ACMA conducted more investigations, including undertaking extensive engagement with industry, other parts of government and with international peers. This has included the consideration of the potential for interference and possible mitigations within the technical liaison group (TLG) (and associated Radio Altimeter Coordination Group), and separate bilateral and multilateral discussions with relevant parties. This informal phase, occurring over July 2021 to January 2022, resulted in 2 possible approaches for managing coexistence between WBB and radio altimeters. These were published in the releasing 3400–4000 MHz in remote areas consultation paper in March 2022, in which formal feedback was invited.

The 2 approaches identified in that paper were either having no additional specific mitigations required beyond the existing 200 MHz guard band, or having additional mitigations. The approach that contemplated additional mitigations was based in part on those adopted in other jurisdictions worldwide. These included measures such as exclusion and restricted zones around some airports where WBB deployments are prohibited or in some way constrained.

The ACMA considered submissions to the remote areas consultation and prepared this paper to summarise, as much as practicable, our current proposal as well as global considerations. An earlier version of this paper was shared with the TLG in October 2022 and had a minor change. This current report version has been updated with the latest global information available and any appropriate clarifications/corrections; however, the substantive proposals are largely unchanged.

The ACMA has not only been engaging on the issue with relevant sector stakeholders in close cooperation with the Civil Aviation Safety Authority (CASA), but have been collaborating using a whole-of-government approach since 2021. This forum includes high-level representation from aviation and communications areas of the Department of Infrastructure, Transport, Regional Development and Communications and the Arts (DITRDCA), CASA, Airservices Australia and Home Affairs. These agencies support a precautionary approach and continue to work towards a solution that provides certainty for both aviation operations and the implementation of new WBB services, recognising that both are important public interest elements for Australia.

## Observations

The ACMA has made the following key observations from its investigations and industry engagement:

There is a wide variation of views between domestic industry sectors:

The WBB sector is not supportive of additional mitigations beyond the 200 MHz guard band, citing no confirmed reports of interference globally.

The aviation sector is supportive of additional mitigations, citing the critical safety and operational aspects of radio altimeter use in aviation and the results of some studies that indicate the potential for interference.

The ACMA is not aware of any confirmed instances of interference to radio altimeters with an identified causal link to WBB deployments, but notes that making a causal link from any operational instance, as opposed to a controlled trial, is difficult.

There is a wide variation in approaches by national spectrum regulators in implementing WBB in bands near radio altimeters. These range from no guard band and no specific mitigations for coexistence with radio altimeters, to various-sized guard bands and additional mitigations, including exclusion and restricted areas around certain airports and other general constraints on WBB deployments. We note, however, that some regulators may not have considered this issue when developing planning arrangements.

The results of theoretical studies and controlled trials diverge from the empirical evidence gathered from real-world deployments. Some studies and trials (highly dependent on input parameters and other assumptions) indicate that, under some circumstances, interference may occur to radio altimeters. However, the absence of confirmed interference events to radio altimeters, which are causally linked to extensive WBB deployments in many countries, does not align with these theoretical observations. These result from different input parameters and testing conditions, which indicate the possibility of interference when there is none, and/or the nature of actual interference to Ras, is such that to date, it has not been properly identified or able to be causally linked with sufficient confidence.

The spectrum management coexistence performance of some current radio altimeters appears to be generally poor and not conducive to good spectrum management outcomes. This is characterised by industry testing that indicates apparent susceptibility of radio altimeters to operation of WBB devices far (100s of MHz) from the radio altimeter operating band via a mechanism where fundamental WBB emissions interact with radio altimeter operation due to poor radio altimeter filter performance. While the current use of spectrum near the radio altimeter band is changing due to the increased use for WBB under mobile and fixed service allocations, the regulatory environment in terms of service allocations has remained the same for many years. It appears that, to date, some radio altimeter devices and standards have not adequately considered the spectrum environment that they are intended to operate within. This situation is being improved through the retrofitting of filters to existing radio altimeters in the US (originally proposed to be completed by July 2023) and work underway within industry to develop new radio altimeter standards.

## Proposed approach

The theoretical studies and empirical evidence from real-world deployments regarding the coexistence of radio altimeters and WBB do not provide a conclusive view of the actual likelihood of interference to radio altimeters. Some lack of certainty and consensus on spectrum coexistence matters is not unusual given the differing perspectives of stakeholders, however, the critical nature of radio altimeters in some use-cases, makes this lack of definitive evidence more challenging than usual.

Given these circumstances, we consider it appropriate to adopt a precautionary approach to the implementation of new WBB uses into this environment. This is based on the concept that the absence of definitive data to accurately determine risk likelihood and consequence (in this risk of interference to radio altimeters) should not preclude action to mitigate the possible risk through the adoption of prudent controls (that is, regulatory measures). The consequences of excessive or unnecessary regulatory restrictions on WBB systems may, however, reduce the long-term benefits that these services can provide to the Australian community.

It is in this context that we propose the following approach. This includes aspects that are ongoing (but subject to potential future change as is any technical framework), and those of an interim, time-limited nature for WBB deployments above 3800 MHz. The proposed interim arrangements consist of those applicable to specific airports designed to address landing scenarios, and those of a general nature applicable everywhere. A detailed presentation and discussion on this approach and the supporting basis is provided in Appendix D: Development of proposed interim mitigations.

Ongoing mitigations:

- A 200 MHz guard band, 4000–4200 MHz, where WBB deployments will not be permitted (previously determined).

- (This ongoing mitigation could be implemented by a clause in RALI MS47 and an amended Radiocommunications Advisory Guidelines (Managing Interference from Spectrum Licensed Transmitters — 3.4 GHz Band) ]

- A total radiated power (TRP) limit of 48 dBm/5 MHz.

- A range of unwanted emissions limits (articulated as TRP or conducted power per antenna port). The limits can be found in the draft example spectrum licence for spectrum-licensed services and the area-wide licensing (AWL) licence condition determination for AWLs. Limit values are the same but their applicable frequency ranges and boundaries are different due to different expected allocation ranges between spectrum-licensed services and AWLs.

The ACMA had previously proposed an additional ongoing mitigation to the TLG. This was not supported by any member so the ACMA is not inclined to proceed:

- For all WBB licensees in 3400–4000 MHz, it is recommended when planning and deploying their networks, to take note of the co-existence issues that may arise due to aeronautical radio-navigation services, typically radio altimeters on aircraft, that may operate in the 4200–4400 MHz band. It is also recommended that affected licensees seek to coordinate with airports, heliports and aircraft operators to help prevent, manage and resolve interference that may arise to aeronautical radio-navigation stations.

The interim arrangements around specified airports are similar to the ‘additional mitigations’ approach (B) identified by the ACMA in its March 2022 paper, and the (original) Canadian/French approaches proposed before December 2022, referenced by a number of submissions to the ACMA consultation.

Interim mitigations:

- For deployments above 3800 MHz around runways identified in consultation with CASA:

- exclusion zones, where no WBB base stations are permitted to be deployed

- restricted zones, except in metro and regional restricted cell areas in 3950–4000 MHz, where WBB base station deployments are permitted but with obligations to meet a specified power flux density (PFD) limit.

- For deployments everywhere above 3800 MHz:

- additional WBB base station unwanted emission limits in 4200–4400 MHz, specified as an effective isotropic radiated power (EIRP) density limit

- maximum WBB base station EIRP density limits

- requiring WBB base station antennas to point or scan below the horizon only

- requiring WBB base station antennas to minimise grating lobes as much as practicable.

- Note that the definitions of the exclusion zones, restricted zones, PFD limit and unwanted emission limits may vary with the WBB frequency, to maintain compliance with the underlying derivations.

The ACMA is of the view that these interim mitigations be time limited. This is to ensure that the impacts of these mitigations on WBB (and the benefits they provide) are limited to the extent compatible with providing a reasonable opportunity for improved radio altimeter devices to be implemented in Australia. There is an expectation that the coexistence environment between radio altimeters and WBB will be significantly improved (via the mitigation of possible WBB fundamental emission interference into radio altimeters) through the deployment of improved performing radio altimeters. This would enable interim mitigations to be either removed entirely or significantly reduced so that there would be minimal restrictions on WBB.

This approach of implementing interim mitigations while radio altimeters are retrofitted is consistent with the approach adopted in the US and proposed for Canada. In these countries, timeframes have been established for retrofitting filters and/or radio altimeter units to affected radio altimeters. In the US, this was initially proposed to occur by July 2023 but extended to 1 February 2023 and in Canada, it is proposed by 31 March 2025.

To provide clarity to the aviation sector (for the timeframes to undertake retrofits as necessary) and future WBB licensees, the ACMA is proposing to remove the interim mitigations by 31 March 2025 (‘interim mitigation end date’). This would provide an additional 21 months from the original July 2023 timeframe proposed in the US in recognition of the status of developments in the band in Australia, potential equipment availability issues and time necessary to undertake such changes on aircraft in the Australian environment.

After the interim mitigation end date, CASA will determine whether any additional restrictions are necessary for aircraft operations.

This approach has been formed by the ACMA based on the currently available evidence. While the ACMA will consider the information before it at the time and may contemplate retention of modest ongoing additional mitigations after this date, or to modestly extend the date of interim mitigations, the currently available evidence does not suggest that this will be necessary. Consequently, the ACMA recommends industry work on the basis that all the interim mitigations will be removed in their entirety within the proposed time.

While we propose a clear end date for these interim arrangements, we will continue to monitor developments globally and will review any mitigations that we implement earlier, if appropriate.

## Purpose and outline

This paper consolidates the ACMA’s current understanding of the coexistence of radio altimeters and WBB systems, and outlines a proposed implementation approach. Consideration of this issue is continuing globally, therefore, the understanding of the issue will continue to evolve. This paper may be updated as new information becomes available that could influence relevant arrangements in Australia.

This paper is structured into the following chapters:

Background

- Provides a general overview of the issue and history of work conducted so far.

Coexistence evaluation

- Examines what the issue between WBB and radio altimeters is and reviews theoretical and empirical evidence for potential issues, both domestically and overseas.

Observations, conclusions and proposed approach

- Makes overall conclusions on the issue and summarises the ACMA proposal.

Appendix A: Wireless broadband

- Information on WBB and its relevant issues related to coexistence with radio altimeters.

Appendix B: Radio altimeters

- Information on the aviation usage and characteristics of radio altimeters.

Appendix C: International developments and approaches

- Information on relevant overseas activity on the issue and, where appropriate, a summary of approaches adopted.

Appendix D: Development of proposed interim mitigations

- Further ACMA work to explain how the proposed approach was developed.

Appendix E: Historical Australian WBB deployments

- An examination of domestic WBB deployments that may have had the potential to affect radio altimeter issues.

Appendix F: Relevant overseas WBB deployments

- A presentation of overseas deployment examples that may have the potential to affect radio altimeter issues.

Appendix G: Identified runways

- An initial list of where the interim mitigations are proposed to apply, based on information provided by CASA.

# Background

## Overview

This section provides an overview of the technologies and applications relevant to the replanning of the 3400–4000 MHz range.

### Wireless broadband

Information on the concepts involved in WBB is in Appendix A: Wireless broadband.

### Radio altimeters

A high-level introduction to radio altimeters and their use is in Appendix B: Radio altimeters.

### Existing band arrangements

The Australian Radiofrequency Spectrum Plan (Spectrum Plan) describes the following high-level service assignments in the band and nearby bands in Australia and other ITU-R regions.

3100–4400 MHz:

| Column 1: ITU Radio Regulations Table of Allocations |  |  | Column 2: |
| --- | --- | --- | --- |
| Region 1 | Region 2 | Region 3 | Australian Table of Allocations |
| 3 100 – 3 300 RADIOLOCATION Earth exploration–satellite (active) Space research (active) 149 428 |  |  | 3 100 – 3 300 RADIOLOCATION AUS100A Earth exploration–satellite (active) Space research (active) Fixed Mobile 149 |
| 3 300 – 3 400 RADIOLOCATION 149 429 429A 429B 430 | 3 300 – 3 400 RADIOLOCATION Amateur Fixed Mobile 149 429C 429D | 3 300 – 3 400 RADIOLOCATION Amateur 149 429 429E 429F | 3 300 – 3 400 RADIOLOCATION AUS100A Amateur Fixed Mobile 149 |
| 3 400 – 3 600 FIXED FIXED–SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 430A Radiolocation 431 | 3 400 – 3 500 FIXED FIXED–SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 431A 431B Amateur Radiolocation 433 282 | 3 400 – 3 500 FIXED FIXED–SATELLITE (space-to-Earth) Amateur Mobile 432 432B Radiolocation 433 282 432A | 3 400 – 3 600 FIXED MOBILE 432B 433A RADIOLOCATION 433 AUS101A Amateur Fixed–satellite (space-to-Earth) 282 |
|  | 3 500 – 3 600 FIXED FIXED–SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 431B Radiolocation 433 | 3 500 – 3 600 FIXED FIXED–SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 433A Radiolocation 433 |  |
| 3 600 – 4 200 FIXED FIXED–SATELLITE (space-to-Earth) Mobile | 3 600 – 3 700 FIXED FIXED–SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 434 Radiolocation 433 | 3 600 – 3 700 FIXED FIXED–SATELLITE (space-to-Earth) MOBILE except aeronautical mobile Radiolocation 435 | 3 600 – 4 200 FIXED FIXED–SATELLITE (space-to-Earth) MOBILE except aeronautical mobile |
|  | 3 700 – 4 200 FIXED FIXED–SATELLITE (space-to-Earth) MOBILE except aeronautical mobile |  |  |
| 4 200 – 4 400 AERONAUTICAL MOBILE (R) 436 AERONAUTICAL RADIONAVIGATION 438 437 439 440 |  |  | 4 200 – 4 400 AERONAUTICAL MOBILE (R) 436 AERONAUTICAL RADIONAVIGATION 438 437 440 AUS87 |

4400 –5030 MHz:

| Column 1: ITU Radio Regulations Table of Allocations |  |  | Column 2: |
| --- | --- | --- | --- |
| Region 1 | Region 2 | Region 3 | Australian Table of Allocations |
| 4 400 – 4 500 FIXED MOBILE 440A |  |  | 4 400 – 4 500 FIXED MOBILE 440A AUS67 AUS87 AUS101 |
| 4 500 – 4 800 FIXED FIXED–SATELLITE (space-to-Earth) 441 MOBILE 440A |  |  | 4 500 – 4 800 FIXED FIXED–SATELLITE (space-to-Earth) 441 MOBILE 440A AUS67 AUS87 AUS101 |
| 4 800 – 4 990 FIXED MOBILE 440A 441A 441B 442 Radio astronomy 149 339 443 |  |  | 4 800 – 4 940 FIXED AUS101A MOBILE 440A 442 AUS101A Radio astronomy 149 443 AUS67 AUS87 |
|  |  |  | 4 940 – 4 990 FIXED AUS102A MOBILE 442 AUS102A Radio astronomy 149 339 443 AUS67 AUS87 |
| 4 990 – 5 000 FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY Space research (passive) 149 |  |  | 4 990 – 5 000 FIXED AUS101A MOBILE except aeronautical mobile AUS101A RADIO ASTRONOMY Space research (passive) 149 AUS67 AUS87 |

The mobile service has been allocated as a primary service in the 3400–4000 MHz range in Australia and in ITU-R Regions 2 and 3 for more than 60 years. The potential elevation of the mobile service in 3600–3800 MHz in Region 1, from secondary to primary, is also being considered at WRC-23 under agenda item 1.3.

The initial discussion paper on 3700–4200 MHz presented the following more detailed view of the use of 3700–4200 MHz domestically, before planning decisions were made:

Existing Australian arrangements in 3700–4200 MHz

## Recent activities

### Domestic

#### History of the band review concerning the issue

A review of the 3700–4200 MHz band was first identified in the Five-year spectrum outlook 2018–22. The first discussion paper in August 2019 outlined the issue, with  an options paper following in July 2020. Outcomes were announced in January 2021.

Planning decisions for the adjacent 3400–3575 MHz range were made in December 2019, but have not been implemented.

While the planning process investigated the issue in as much depth as practicable at the time, It was announced with the 3700–4200 MHz outcomes that the issue would be further considered in the development of detailed technical frameworks for WBB.

To assist in the development of those technical frameworks, a Technical Liaison Group (TLG) was established for the whole 3400–4000 MHz range in July 2021. It concluded work on remote areas in January 2022 but could recommence if required as frameworks for other areas are considered. The TLG documents considered the issue and proposed possible methods for the issue.

A draft technical framework was released for consultation in March 2022, along with remote allocation plans. This included approaches for radio altimeters.

After considering the submissions to the consultation and developing this report, the ACMA shared the high-level proposed mitigations with CASA in July 2022. Their response included their views of appropriate mitigations and a list of applicable airports/approaches.

The TLG was reformed for September and October 2022 to consider a draft spectrum licensing technical framework for the 3400–4000 MHz range. This included an earlier version of this report and its proposed approach to mitigations. Group members provided their views before 1 November 2022.

Since 2021, the ACMA has chaired a series of inter-agency meetings to facilitate a whole-of-government consideration of the issue, involving various branches of DITRDCA, Home Affairs, CASA and Airservices.

The implementation of this planning review makes no changes to arrangements for services above the radio altimeter band, which are mainly used by Defence.

### International

An overview on activities on the issue in overseas markets, including any proposed approaches, is at Appendix C: International developments and approaches.

# Coexistence evaluation

## Spectrum management regulatory environment

## International

At the international Radio Regulations treaty level, there are currently no conditions to manage cross-border compatibility specifically between any mobile service allocation below 4200 MHz and the aeronautical radiolocation service (that is, radio altimeters) in 4200–4400 MHz. There are, however, clauses that apply to services more generally.

In Article 4 of the Radio Regulations, there are obligations for member states that may be relevant, such as Articles 4.3, 4.5 and 4.10.

Clauses in Article 3 such as Article 3.3 below, are also relevant in coexistence considerations:

- Article 3.3: Transmitting and receiving equipment intended to be used in a given part of the frequency spectrum should be designed to take into account the technical characteristics of transmitting and receiving equipment likely to be employed in neighbouring and other parts of the spectrum, provided that all technically and economically justifiable measures have been taken to reduce the level of unwanted emissions from the latter transmitting equipment and to reduce the susceptibility to interference of the latter receiving equipment.

Article 1 also defines a safety service as:

- Any radiocommunication service used permanently or temporarily for the safeguarding of human life and property.

As such, a radio altimeter would fit this definition when used for safety-critical procedures. It also defines harmful interference as:

- Interference which endangers the functioning of a radionavigation service or of other safety services or seriously degrades, obstructs, or repeatedly interrupts a radiocommunication service operating in accordance with Radio Regulations.

While there are 2 relevant World Radiocommunication Conference (WRC-23) agenda items (AI): International Mobile Telecommunications (IMT) identification for 3.6–3.8 GHz in Region 2 under AI 1.2 and the possible upgrade of the mobile service to primary for 3.6–3.8 GHz in Region 1 under AI 1.3, no relevant studies have been submitted as formal contributions in this WRC study cycle.  The current stated International Civil Aviation Organisation (ICAO) position, as of June 2022, on the issue with respect to WRC-23 is:

- It is important to note, however, that the issues raised by the radio altimeter studies are not with the regulatory allocation and identification to the mobile service (i.e., it is not pertinent to WRC-23 Agenda Item 1.3 discussions), rather to how new systems are being authorized for deployment within that service. Work continues to assess any possible measures that might be needed, both near-term and in the future, to ensure compatible operation of radio altimeters and these new mobile service systems.

While the ICAO prepared an internal study in relation to WRC-15 AI 1.1 in the previous study cycle, which included possible identification of IMT in the 3400–4200 MHz and 4400–4500 MHz ranges, it did not appear in the WRC-15 summary CPM input contribution. This study is discussed later in this section.

## Domestic

Domestically, radio altimeters are authorised for use under the Radiocommunications (Aircraft and Aeronautical Mobile Stations) Class Licence 2016. General provisions for class licences are prescribed in Part 3.4 of the Radiocommunications Act. Neither document, or any other relevant instrument, describes any specific required protection or coordination for radio altimeters, or interference management requirements. While certain sections of the Act (for example, s.192–194, 197) are relevant to the safe operation of aircraft and other systems, they relate to knowingly engaging in conduct that would, or would likely, result in interference, rather than where interference is not intended. Consequently, new regulatory provisions would be required to implement any potential interference management for radio altimeters.

## Interference mechanisms

ITU-R M.2059-0 provides an overview of appropriate interference mechanisms to consider in coexistence studies involving radio altimeters in Annex 3, including overload, desensitisation, and false altitude reports. However, the Ofcom study, discussed in the section below, determined that, in all cases, the false altitude report mechanism is effectively redundant, if overload and desensitisation is already being considered in a study on practical WBB systems. In summary, however, the mechanisms that should be considered are as follows (reproduced in part from M.2059):

## Receiver front-end overload

Receiver front-end overload occurs when sufficient power from an interfering signal saturates the front-end of a radio altimeter receiver, causing the inherent effects of non-linear behaviour, for example, harmonic distortion or intermodulation. This mechanism is often also called ‘blocking’ as the unwanted signal affects the ability of the receiver to correctly demodulate its wanted signal and ‘blocks’ it.

## Receiver desensitization

The desensitization effect is related to the intensity of the interfering signal that falls into the IF bandwidth of the radio altimeter. What complicates the issue of desensitization of a radio altimeter is that the RF spectrum related to the IF bandwidth by mixing is not constant in time, because radio altimeters operate in a homodyne configuration using a linear frequency-modulated signal. Thus, the impact of interference toward desensitization of a radio altimeter receiver is time-dependent, according to the technical characteristics of the specific radio altimeter.

The effect on a radio altimeter from in-band interference sources is related to the power of the interfering signals in the receiver IF bandwidth.

The ACMA notes that receiver overload and desensitisation performance may also depend upon the signal level received from the radio altimeter transmitter itself. This, in turn, is dependent upon assumptions around ‘loop loss’; that is, the loss from the radio altimeter transmitter to the receiver considering propagation loss, spread and ground scattering. The noise floor of the receiver may not need to be protected if the radio altimeter is receiving a strong signal from its own transmitter.

## False altitude reports

In the case of Frequency Modulation Continuous Wave (FMCW)-based radio altimeters, false altitude reports occur when interference signals are detected as frequency components during spectral frequency analysis of the overall IF bandwidth.

The magnitude of the spectral components and the bandwidth of the resulting interfering signal spectra will be dependent on the strength of the received interference level and the fraction of time the resulting difference signals remain within the IF bandwidth.

## Studies

### Introduction

This part of the Coexistence evaluation chapter considers theoretical and laboratory measurement studies that have been conducted. The following section Over the air (OTA) tests and trials reviews controlled trials, as opposed to uncontrolled empirical evidence of events, which is considered in the Observed historical possible interference events part of this chapter.

### Australia

The ACMA conducted studies as part of the options paper and TLG processes. The studies generally considered both the overload and receiver desensitisation mechanisms.

The studies covered a range of scenarios:

Single base station, static non-AAS and AAS envelope landing, similar to the ICAO study.

Single base station, dynamic ideal AAS landing.

Aggregate interference from existing base station deployments in 3575–3700 MHz for both non-AAS and ideal AAS scenarios for several metro airport approaches.

Aggregate interference from typical non-AAS BS deployment.

Aggregate interference from typical BS deployment using ideal AAS in a monte-Carlo analysis.

#### Summary

The ACMA studies generally concluded that:

Modelled non-AAS WBB systems have some theoretical potential for interference for the worst-case M.2059 or RTCA usage category 1, 2 or 3 radio altimeters.

Modelled use of AAS for WBB deployments may significantly reduce or eliminate the theoretical potential for interference for the worst-case M.2059 or RTCA usage category 1 radio altimeters.

Studies are very sensitive to the assumed radio altimeter front-end filter and to the assumed vertical radiation pattern of WBB base stations.

Existing base station deployments in 3575-3700 MHz have a low potential for interference if modelled as ideal AAS systems.

#### Limitations

The ACMA studies were limited to AAS systems scanning below the horizon with ideal M.2101 antenna patterns. Consequently, they may not be representative for some cases which have high side/grating lobes in the WBB antenna systems.

### International

#### ICAO

An input into the ICAO ACP-WG-F30 meeting of March 2014 was a Preliminary Study into Radio Altimeter Adjacent Band Compatibility concerning WRC-15 agenda item 1.1.

#### Summary

The study, using the radio altimeter parameters from the c.	ITU-R Recommendation M.5_BL_6, which was a pre-published version of M.2059-0, examined the false altitude detection mechanism from an example non-AAS LTE base station and UE under a CAT-III landing flight path.

It concluded that:

Although this preliminary study using a basic model to simulate an aircraft in one of its most critical phases of flight, the power levels received at the radio altimeter receiver are higher than desired. If an IMT allocation was made in the adjacent frequency band to the radio altimeters, there is the potential requirement for nearly 64 dB of OOB isolation at the band edge to properly protection the receiver from false altitude errors.

#### Observations

The method does not consider any radio altimeter receiver front-end filtering. For the false altitude report mechanism, this would be correct, but that mechanism appears to be incorrectly assessed.

The method appears to incorrectly use the Base station/UE power, rather than the unwanted emissions falling into the radio altimeter bandwidth. It does not appear to account for the radio altimeter chirp bandwidth, when determining if the power at the radio altimeter detector is above the false altitude detection level. The calculation then appears to be valid only for determining the overload case and, consequently, the radio altimeter overload figure should instead be used as the threshold. For the worst-case radio altimeter A2, this would be -56dBm total power. The maximum power received from the BS was determined to be -97.6dBm/100 Hz or -47.6 dBm total BS power. This is less than 10dB above the radio altimeter overload level (ignoring any possible radio altimeter receiver front-end filtering).

#### Ofcom

Ofcom performed a study as part of their planning of their Enabling opportunities for innovation – Shared access to spectrum supporting mobile technology consultation from 2018. We previously examined this study as part of an international study scan in our initial Wireless broadband and radio altimeter compatibility study – spectrum planning paper from 2020.

#### Summary

The study examined the overload, receiver desensitisation and false altitude detection mechanisms in a geometry very similar to the ICAO study, using non-AAS WBB parameters suitable for the Ofcom plan for low- and medium-power WBB deployments. It concluded that for their intended deployments, the risk was very low.

#### Observations

We observed in our 2020 study that ‘… it can be expected that the ACMA study will show a higher likelihood of interference to radio altimeters from WBB than the Ofcom study. This is largely due to the higher BS power levels and higher BS spurious levels assumed.’

#### Subsequent United Kingdom CAA Study

The UK also provided an input into the ICAO FSMP WG11 meeting, from John Mettrop of the UK Civil Aviation Authority, concerning the UK medium-power and low-power planned deployments in 3.6–3.8 GHz and 3.8–4.2 GHz ranges, respectively.

#### Summary

The study determined minimum separation distances from medium- or low-power typical deployments but with some flexes to some parameters, such as different radio altimeter front-end roll-off masks, some aggregate cases, varying aircraft pitch/roll and for M.2059 altimeter A1 vs A3.

The study made the following conclusions (verbatim):

1	This study would indicate that 5G mobile base stations operating below 3.8 GHz, especially if they use active antenna systems, which have not been considered in this study, pose a viable interference threat. However, due to the anonymisation of the data provided in the RTCA study, which is the only one currently available, the number of radio altimeters affected and the degree to which they are affected cannot be assessed

2	For the UK situation with respect to local fixed wireless access systems operating in the frequency band 3.8-4.2 GHz, especially given the restriction to rural areas and considering practical separation distances it is unlikely to be a problem for aircraft landing at an airfield.  However, there may be an issue for helicopters, especially those used by the emergency services that could land closer to a mobile mask than would occur for fixed wing aircraft.

3	In order to quantify the extent of the problem results are required for individual radio altimeters.  Once quantified then a solution(s) can be determined.

4	Given the size of the issue at 3.6-3.8 GHz in comparison to that at 3.8-4.2 GHz it would be reasonable to assume that any solution for the 3.6-3.8 GHz will also resolve the issue at 3.8-4.2 GHz

5	Noting that 5G or other similar systems, including one deployed at London Heathrow, have apparently been operating without interference being detected then the reason why there has not been any interference detected needs to be investigated in light of the results of this study.

#### Observations

Conclusion 1 notes that, because of modelling difficulties, AAS was not studied, yet concluded they should be worse, without any supporting evidence.

The aviation industry ‘6 dB safety factor’ was used in the study.

The conclusions note the contrast between theory and practice in the UK context.

#### RTCA

The RTCA released a detailed study as part of sub-committee 239 (low range radar altimeter). This included laboratory measurements of radio altimeters performed by the Aerospace vehicle systems institute (AVSI). After the release of the RTCA study, the AVSI conducted additional measurements, releasing them in stages with Volume I and II then Volume III.

#### Summary

Based on the methodology and assumptions made, the RTCA concluded that:

The results presented in this report reveal a major risk that 5G telecommunications systems in the 3.7–3.98 GHz band will cause harmful interference to radar altimeters on all types of civil aircraft—including commercial transport airplanes; business, regional, and general aviation airplanes; and both transport and general aviation helicopters. The results of the study performed clearly indicate that this risk is widespread and has the potential for broad impacts to aviation operations in the United States, including the possibility of catastrophic failures leading to multiple fatalities, in the absence of appropriate mitigations. The extent of the RF interference is summarized by the worst-case exceedance of the safe interference limit of radar altimeters by expected 5G signals in the 3.7–3.98 GHz band: 14 dB for commercial transport airplanes (as shown in Figure 10-4), 48 dB for business, regional, and general aviation airplanes (as shown in Figure 10-12), and 45 dB for helicopters (as shown in Figure 10-16). Further, the impacts are not only limited to the intentional emissions from 5G systems in the 3.7–3.98 GHz band, but also the spurious emissions from such systems within the protected 4.2–4.4 GHz radar altimeter band directly. In this latter case, the worst-case exceedance of the safe interference limit is 28 dB for business, regional, and general aviation airplanes (as shown in Figure 10-25), and 12 dB for helicopters (as shown in Figure 10-29).

#### Observations

The ACMA previously provide some comments on the report with the release of the 3700–4200 MHz outcomes paper:

The report uses similar, but more comprehensive, methodologies to those in the ACMA study and used measured radio altimeter parameters as tested by the aerospace vehicle systems institute (AVSI). The following observations about the RTCA report are made:

The Interference Tolerance Masks (ITM) determined in the RTCA analysis uses a minimum ground reflectivity of 0.01, based on a minimum testing performance standard under ED-30. The ACMA study used 3.16 based on representative conditions in Australia (a 25 dB difference). Using a very low reflectivity will result in a higher estimated potential for interference. The RTCA/AVSI view is that the altimeters should work (and be protected) in the worst case.

The ITMs used the most conservative of each parameter from any of the set of radio altimeters identified as being applicable for each specific scenario/usage category being analysed.

An extra margin of 6 dB is used in the radio altimeter laboratory tests to develop the ITMs to account for testing and advised product variations, rather than testing multiple samples of each radio altimeter to obtain a parameter distribution.

An additional 6 dB ‘aviation safety margin’ is also considered and presented in the overall results. Its application depends on the determined safety criticality of each analysed scenario.

- For many scenarios then, the RTCA study appears conservative compared with median expectations.

#### Japan MIC study

The MIC in Japan conducted a study in support of their replanning of 3700–4200 MHz.  An English language summary was provided as an input document to the ICAO FSMP WG/7 meeting in 2018. Other relevant sources for the study are:

The compatibility study between 5G base stations and radio altimeters in Japan and update of the result of measurement campaign

01.pdf (soumu.go.jp), page 12

https://www.5gamericas.org/wp-content/uploads/2021/07/Mid-Band-Spectrum-and-the-Co-Existence-with-Radio-Altimeters.pdf

https://www.soumu.go.jp/main_content/000582446.pdf (in Japanese).

#### Summary

The study examined several scenarios using radio altimeter parameters from ITU-R M.2059 and considered different base station densities in differently classified areas (for example, urban, suburban, rural) and used a statistically derived ‘beam forming antenna envelope’ to consider potential interference from base stations.

#### Observations

The ACMA 3400–4000 MHz TLG paper previously made the following observations:

It is expected that the study is more accurate for the overload mechanism than the French or Canadian cases, but still potentially conservative due to the ‘envelope’ approach. For the spurious case, use of the ITU-R M.2059 radio altimeter receiver noise floor protection to derive spurious emission requirements means it is likely conservative for that case.

#### Japan Electronic Navigation Research Institute study

The Japan Electronic Navigation Research Institute presented an information paper at the ICAO FSMP WG/15 meeting in August 2022. It presented the results of laboratory tests of a range of radio altimeters interference susceptibility against 5G systems.

#### Summary

The in and out-band electromagnetic interference susceptibility of the 12 radio altimeters for large fixed-wing aircraft and the 8 radio altimeters for the medium and small fixed-wing aircraft and the helicopters were evaluated. The measurement procedures and the interference criteria were fundamentally same as the those of described in the RTCA 5G Interference Assessment Report. The determined out-band interference threshold values of the radio altimeters for the large fixed-wing aircraft were −22 dBm at 200 ft, −29 dBm at 1000 ft, −37 dBm at more than 2000 ft. In addition, the out-band interference threshold values of the radio altimeters for the medium and small fixed-wing aircraft and helicopters were −56 dBm at 200 ft, −66 dBm at 1000 ft, −77 dBm at 2000 ft. The out-of-band (in terms of the radio altimeter) interference tolerance masks (ITMs) are similar in all cases to the RTCA ITMs.  In-band ITMs for smaller aircraft and helicopters show more variance, especially being 10dB more robust at 200 ft.

#### Observations

The Japanese ITMs, based on the worst-measured radio altimeter in each category, include a 6 dB uncertainty factor added to the measured results, but do not break this factor down. This figure seems high, given some unit-to-unit variance is already taken into account for some models tested.

While, for example, the ITMs for the out-of (in terms of radio altimeter)-band tests for large fixed-wing aircraft do not vary much with 5G frequency, this is not the case for most of the individual radio altimeters tested.

The lab tests were based on static 5G behaviour, so would be conservative compared with most real-world dynamic AAS 5G deployments.

For comparison, Figure 2 below compares the Japan ITMs (with the 6 dB factor removed) and the RTCA ITMs scaled to the 100 MHz test carrier bandwidth.

Comparison of the Japan determined  ITMs and the RTCA determined ITMs

#### France

The French regulator, National Frequency Agency (ANFR), based their proposed mitigations on some analysis that is included in a ICAO FSMP meeting WG 11 input document.

#### Summary

The French mitigations, detailed in the Appendix C: International developments and approaches section, were based on the analysis using usage category 1 ITMs from the RTCA report for an appropriate height, with some flight path tolerance, and expected WBB deployment parameters.

#### Observations

The size of the safety and precaution zones are dependent on WBB maximum EIRP (78 dBm/100 MHz for the specific example given). The analysis of the precaution zone assumes a high side/grating lobe for the WBB antenna (18dBi gain) ‘in the absence of an antenna envelope provided by the WBB operator’. This may occur in some circumstances for very large AAS beam steering angles and/or AAS arrays using non-deal (>wavelength/2) element spacing but would be considered unusual.

The size of the safety zone includes a 6dB aviation industry safety factor, but the precaution zone size calculations do not.

Given the above and our previous comments on the RTCA study, the study and consequent mitigations are likely to be conservative.

#### Initial Canada studies

ISED in Canada conducted some preliminary calculations to ascertain the level of risk for the proposed deployments of WBB, initially in the 3460–3650 MHz range but also out to consultation for the 3650–3980 MHz range.

#### Summary

The initial calculations were based on the radio altimeter characteristics in M.2059 and assumed static WBB deployments. It concluded that:

- Harmful interference to radio altimeters from signals operating outside their band of operation (4200–4400 MHz) was calculated to be possible, which could include signals from future flexible use deployment of 5G systems in the 3500 MHz band around airports and heliports, warranting further investigation into the issue.

- Because of this conclusion, ISED went to consult and then decide on mitigations very similar to those established in France, but with a PFD limit in the ‘protection zones’. While the initial calculations were based on M.2059, the PFD limit was determined based on a specific usage category 1 scenario from the RTCA report. Details on mitigations are described in Appendix C: International developments and approaches.

#### Observations

The initial assessment of risk was based on the worst-case M.2059 radio altimeter, including protecting the full receiver noise floor, regardless of the likely received level from the radio altimeter transmitter. No beamforming for AAS systems were considered. Consequently, it was likely conservative. As the subsequently determined mitigation zones are based on France and the PFD limit also based on RTCA report ITMs, the mitigations are also likely to be conservative.

Given that the French zone sizes are supposed to vary with deployed EIRP, the fixed Canadian zone sizes may not always be appropriate for their use-cases. It is also noted that WBB deployment in Canada is proposed up to 3900 MHz for high-power deployments. However, zone sizes are based on 3750 MHz specific French calculations, where they would be expected to vary, dependent on the RTCA ITM with frequency. They do not currently propose any change of zone sizes with frequency.

#### Revised Canada work released December 2022

ISED in Canada consulted on revised possible mitigations as part of their third consultation released in December 2022. Annex A was a summary of their domestic studies. This included laboratory as well as over the air tests, the latter covered in this report from 3.71. Annex A also summarises a computational model that uses the laboratory results to model a range of coexistence scenarios and estimate possible required mitigations. The revised mitigations are discussed in this report from 7.63.

- Summary

The summarised laboratory measurements followed a similar methodology to that used in the RTCA report, examining radio altimeter models available to ISED for susceptibility to both in-band and out-of-band 5G signals. Tests included measurements at 10 MHz and 100 MHz 5G bandwidths and both Time Division Duplex (TDD) and Frequency Division Duplex (FDD) simulated systems. Tests were also repeated with additional RF filtering in front of the radio altimeter receiver. Overall conclusions were:

- The laboratory study assessed the susceptibility of radio altimeters to 5G spurious and fundamental emissions.

- Results from spurious emissions testing demonstrated a high disparity in interference susceptibility between radio altimeter models. When compared to other radio altimeters, four units were more susceptible to spurious emissions at lower altitudes, of which one was also significantly more susceptible to spurious emissions than other radio altimeters under test at higher altitudes. Under study-specific test conditions, six out of eleven radio altimeters tested were found to be susceptible to 5G fundamental emissions.

- Adding band-pass filtering at the input of the radio altimeter receiver can mitigate 5G fundamental interference.

#### Observations

Noting that the summarised results do not include other factors that the RTCA study included, such as environmental, unit variation and aviation safety factor allowances, the results showed similar values and model variance as the raw RTCA ITMs. The Canadian measurements also, however, included measurements of 5G frequencies down to 3550 MHz. From the fundamental emissions results, the determined power spectral density thresholds do not scale completely linearly with 5G emission bandwidth, as might be expected for ideal systems with a gaussian noise interference source. While the summary noted that 4 radio altimeter models that were determined to be ‘susceptible to 5G fundamental emissions at all altitudes tested’, the detailed results are more nuanced, as indicated by the example below for a 100 MHz TDD 5G fundamental:

- AUTs 1, 2, 3, 8 and 9 are classed as RTCA usage category 1 models.

#### Independent study around Chicago O’Hare by Remcom

Remcom released a white paper during 2022 using their software, primarily to demonstrate the increased accuracy possible by assessing the interaction with terrain and buildings.

#### Summary

The study uses the RTCA ITMs as a basis, but models possible 5G base stations and propagation in a manner they thought appropriate. They were able to replicate the RTCA report Chicago example closely, before terrain and building assessment was included. When these factors are included, including 3D ray-tracing models, the ITMs were exceeded in some cases when they were not exceeded without these factors.

#### Observations

The study used static AAS patterns with a large downtilt (10 degree mechanical plus 20 degree electrical). This is likely to create large grating lobes that may not exist for significant portions of time in a real AAS deployment scenario.

The ray tracing additions used worst-case assumptions such as wet ground and smooth reflective surfaces that, again, may not be representative for most cases. Although their further analysis of the sensitivity of results to terrain properties shows a low variance.

Since the study is based on the RTCA ITMs, it assumes the use of worst-case ground reflectivity for the radio altimeter signal itself. If the study was introducing the more accurate factors for the 5G->radio altimeter radio path/s, not assessing more accurately the likely radio altimeter->radio altimeter radio path and behaviour means the overall study is not as accurate as it could have been.

## Over-the-air (OTA) tests and trials

### France

The September 2021 European Conference Of Postal And Telecommunications Administrations (CEPT) document ECC PT1(21)192 reported the ‘Outcome from preliminary trial on one type of radio altimeter fitted on helicopter’, considering 3 scenarios for a specific Gendarmerie helicopter.

It concluded that there was:

- No disturbance to the radio altimeter on board the EC135 of the French gendarmerie was detected during the flight, when using the 5G in the configuration for the test flight. In view of these in-flight observations, the emission of 5G NR base station had no impact on the operational behaviour of the radio altimeter fitted on the French gendarmerie's EC135 helicopters, which gives us a good level of confidence in the resilience of this type of radio altimeter to conduct its missions.

This conclusion may have influenced France in rolling back their ‘down-tilt’ mitigation requirements, ostensibly established for helicopters.

### Norway

CEPT document ECC PT1(21)184 detailed the results of an OTA trial in Norway, using a test base station near Norwegian international airport Bergen-Flesland and used 3 different helicopters and 5 different passenger aircraft. Radio altimeter types on the aircraft were identified where possible.

The test concluded that: ‘Test results showed no abnormalities on the Radio Altimeters during the test’.

During the test, the base station transmitted 100% of power with an air interface load generator. The aircraft participating in the test passed by antennas of the base station at the distance of less than 500 metres taxiing along the runway, while helicopters using the helipad passed antennas at distances of 200 metres.

### Canada

ISED, together with the National Research Council (NRC) and Transport Canada performed OTA testing of radio altimeter susceptibility to 5G emissions as part of their overall assessment program.  A summary of the OTA study is included in Annex A of their revised December 2022 consultation.

Both quantitative (measured with instruments) and qualitative (crew observation only) assessment of susceptibility were conducted, using up to 10 aircraft and 9 radio altimeter models.

The 5G base station configurations used were generally stringent worst-cases, that is, often pointing at the aircraft, so the results cannot be directly compared with expected use-cases or scenarios without careful extrapolation.

In summary, no issues were observed by flight crews in qualitative testing. In the quantitative testing, 3 radio altimeter models, corresponding to RTCA usage category 2 and 3 devices, were found to be susceptible to 5G in 29 test configurations, with an average event length of 2.3 seconds. A strong correlation was observed between the 5G power at the receive port of the radio altimeter under test and the 5G fundamental emission break point found in ISED’s laboratory study. They also noted that there was good correlation between their computational model developed from laboratory tests and the OTA results.

### United States

The Joint Interagency Five G Radar Altimeter Interference (JI-FRAI) group, which includes participation from the Department of Homeland Security (DHS), Department of Defense (DoD), Federal Communications Commission (FCC), airlines, and avionics manufacturers, was established in 2021. They are expected to produce the results of in-flight testing of 5G and radio altimeters in 2022. One member, the Institute for Telecommunications Sciences (ITS), part of the National Telecommunications and Information Administration (NTIA), has already released some information about their test methodology in the NTIA Special Publication SP-22-560.

As per the minutes of the RTCA Program Management Committee June 2021 meeting, the final report was expected in June 2022.

Defense budgeted for their contribution to the JI-FRAI testing under a series of Quick Reaction Test line items.

The ITS Report TR-22-562 for the NTIA was released during October 2022. Aside from the 5G base station pattern measurements, it contains very useful information on 5G and radio altimeter systems, and also outlines the other parts of the JI-FRAI program where reports have not yet been released. The main conclusions are para-phrased below:

    - Three models of 5G base station that are being deployed in the US 5G n77 band show distinctly visible, effective RF bandpass filtering in their emission spectra with emissions in the radio altimeter band as much as 106 dBs lower than their on-tuned intentional-radiation power in the 5G frequency band 3700–3980 MHz. Upper bounds varied between -37.5 dBm/MHz to -48.5 dBm/MHz for the models tested.

    - Airborne radiation patterns show measurably significantly less power than is found in 5G base station main antenna beams directed toward User Equipment (UEs) at ground level. The amount of power reduction in the sky is variable and needs to be examined by researchers in detail, using the collected data.

    - There is a distinct near-far effect in the airborne measurements on pairs of transportable 5G base stations, called Cells on Wheels (CoWs). This effect causes the nearer base station transmitters’ emissions to be dominant in a receiver, with more-distant transmitters’ contributions rapidly fading to insignificance.

It was also noted that the base stations tested could not exceed the interim US EIRP limit of 62 dBm/MHz (for high density areas) for most channel bandwidths. They were physico-technically limited to total EIRP of ~56 kW. The report noted: ‘No matter the selected channel bandwidth, the maximum EIRP is always +77.5 dBm for all currently available US n77 band base station transmitters.’

The report gives some guidance on how to potentially extrapolate the measured results to combine with other radio altimeter laboratory or OTA results to examine any specific radio altimeter coexistence scenario.

## Historical and contemporary deployments

### Australian

#### Wireless broadband

A discussion of relevant WBB deployments in Australia is contained in Appendix E: Historical Australian WBB deployments.

#### Defence use above 4400 MHz

Above the 4200–4400 MHz range used by radio altimeters, both the 4400–4500 MHz and 4500–4800 MHz frequency ranges, are subject to footnote AUS101 to the Spectrum Plan. The Spectrum Plan allocates those bands to fixed and mobile services on a primary basis. Authorisation of access under these footnotes is via the Defence apparatus licence type. Defence internally manages access and use under this licence type and, as a result, the ACMA does not have clear visibility of actual use. However, it is understood that use is Australia-wide, often itinerant and supports a range of services.

### International

A discussion of relevant WBB deployments overseas is in Appendix F: Relevant overseas WBB deployments.

#### Summary

There have been many deployments of WBB networks of different technologies in parts of the 3400–4000 MHz range, both in Australia and overseas. Appendix D: Development of proposed interim mitigations examines the implications when considering where a frequency boundary for possible mitigations should be placed.

## Observed historical possible interference events

### Estonia

The International Civil Aviation Organisation’s (ICAO) Frequency Spectrum Management Panel (FSMP) WG/11 meeting in March 2021 had an information document concerning potential interference to certain ATR aircraft on approach to Tallinn airport in Estonia.

- It reports a high degree of correlation of radio altimeter disturbances when passing over specific 3G/4G 800 MHz, 900 MHz, 2100 MHz base stations (note that there are no transmissions of 5G or any in the 3400–4000 MHz range from these base stations).

- If the issues were caused in part by the nearby transmissions in those operating frequency ranges, this should already have been a globally observed major issue, as WBB in these bands has been operating for many years.

### Australia

The CASA airworthiness bulletin AWB 34-020 Issue 7 – 4 March 2022 indicates a number of reported errors or discrepancies with radio altimeter systems, but notes that ‘CASA has not confirmed any radio altimeter system failure from 5G interference’.

The ACMA has received some further information on specific possible events. While some of the events, under a simple static analysis, indicate that nearby 5G base stations in the 3575–3700 MHz range have some potential to cause interference to a worst-case M.2059 radio altimeter, there is not sufficient information to be able to make any definitive conclusions about any of these historical events.

The ACMA encouraged CASA to collect a wider degree of information than that currently suggested in the AWB from each event.

### United States

It was reported in the media in February 2022 that pilots were reporting potential cases of interference to radio altimeters, but the Federal Aviation Administration (FAA) could not validate any cases at the time.

Using the FAA ASIAS and AIDS databases, as of June 2022, the search terms ‘radio altimeter’, ‘radar altimeter’, ‘LRRA’ and ‘radalt’ retrieved a total of 7 records of historical events since 1978. Only one event suggested that radio frequency interference (at a frequency of ~ 2 GHz) may have been a factor but there was no conclusion and the incident was closed. Some operational information on the particular Ground Proximity Warning System (GPWS) used by the aircraft in this event can be found in a historical Lockheed service news publication.

Broadening the AIDS database search terms to include ‘GPWS’ displayed 11 events. Some events indicated possible radio altimeter faults but did not indicate that they were interference related (excepting the same event as under 3.823.82.)

The FAA has now implemented a Report a Radio Altimeter Anomaly page.

The FAA proposed revised Airworthiness Directive related to radio altimeters included, in the background section, a summary of reports of recent radio altimeter anomalies. Out of 420 reports received:

- … roughly 100 of the anomaly reports occurring within NOTAM areas, the FAA has excluded other potential causes for the anomaly, but it could not rule out 5G C-Band interference as the potential source of the radio altimeter anomalies.

- These approximately 100 incidents included various flight deck effects such as erroneous Terrain Awareness and Warning System (TAWS) warnings, erroneous Traffic Collision Avoidance System (TCAS) warnings, erroneous landing gear warnings, and the erroneous display of radio altimeter data. Although these flight deck effects are less severe than the hazards associated with low-visibility landings, the FAA is concerned that to the extent 5G C-Band operations contributed to such events, the effects will occur more frequently as telecommunication companies continue to deploy 5G C-Band services throughout the country.

There are other aviation incident databases used in the US, such as the NASA Aviation Safety Reporting System. Using the text search term ‘radio altimeter’ results in a number of reports since January 2022, where 5G interference is stated as being a possible factor in radio altimeter events.

### UK

In the UK part of Appendix C: International developments and approaches, it is noted that the Civil Aviation Authority (CAA) has talked with fellow aviation regulators and no confirmed incidents where ‘5G interference has resulted in aircraft system malfunction or unexpected behaviour’ have occurred.

We are aware that there have been investigations into possible events around Heathrow and Bristol airports.

# Observations, conclusions and proposed approach

## Difference in approaches between national spectrum regulators

National spectrum regulators have adopted different approaches to managing coexistence between WBB and radio altimeters. There is likely to be a range of reasons why this is the case, including:

    - Timing of considerations – the timing of domestic decisions by national regulators are often different, meaning that the available information/evidence base supporting these decisions will be different. This is especially relevant in coexistence between WBB and Ras, as it is only relatively recently (the RTCA work was only published towards the end of 2020, for example) that detailed work has been undertaken to better quantify the theoretical concern. In some cases, it is likely that decisions have been made before any radio altimeter issues were identified. This different knowledge base may contribute to different decisions being made by regulators over time.

    - ‘Risk appetite’ – while the ACMA has no evidence to support this view, it is possible that different regulators have adopted a different risk management approach to coexistence between WBB and radio altimeters. Given the safety nature of radio altimeter operations in some situations, some regulators may have chosen to apply a more risk averse approach to analysis and decision-making influencing regulatory arrangements than other regulators.

    - Local specific WBB deployment models – WBB use-cases and technical deployment models are varied, which influences the associated regulatory framework and the subsequent coexistence environment. For example, high-power, wide-area WBB networks and supporting regulatory frameworks will have different coexistence considerations with radio altimeters than low-power WBB systems. It is therefore reasonable to expect that the mix of guard bands and other regulatory measures will vary from country to country.

For these reasons, it is difficult to directly compare different regulatory environments and the basis for decisions made between different regulators. Given the complexity of the issues, careful context and caveats are necessary to convey a proper comparison.

## Variation in views between industry sectors

A difference of views and perspectives between industry sectors is normal for many spectrum management considerations. Given the importance of WBB to the mobile sector and that of radio altimeters to the aviation sector (and of both to the regulator), it is unsurprising that there are strong and different sectorial views on the issue of WBB/radio altimeter coexistence.

It appears that there are some key perspectives from each sector that raise concerns with the other:

    - The apparent perspectives from parts of the WBB sector that suggest, since WBB have been deployed around the world for so long with no confirmed interference events, that the concerns raised by the aviation sector are disproportionate to the real-world risk.

    - The apparent view from parts of the aviation sector, that it is solely because the deployed mobile/fixed service environment has changed due to the advent of 5G which has created these current concerns regarding radio altimeters.

Some ACMA observations on these issues are:

    - While the absence of causally-linked interference events to radio altimeters, given current and historical worldwide deployments is a relevant consideration, caution needs to be applied as to how definitive a conclusion can be made in all cases. As outlined elsewhere, there may be reasons why no such causal link has been identified in real world, operational environments.

- Nonetheless, the ACMA does think it is material that real-world deployments have appeared to coexist with radio altimeters for some time in many areas but will remain cautious in the weight it gives this data point when making decisions. In this regard, the ACMA has closely reviewed testing undertaken overseas in Canada that was released in December 2022, and is eagerly awaiting the results of other over-the-air tests in a controlled environment known to have occurred in the US (see commentary on the Joint Interagency 5G Radar Altimeter Interference (JI-FRAI) Quick Reaction Testing (QRT) program in the NTIA ITS Report).

    - It is clear that the deployed environment using the band of interest for mobile networks has changed significantly in recent years, given the use of these bands for early 5G deployments. However, this ignores the fact that even the legacy deployed environment has been used for various WBB technologies and high-powered fixed point-to-point links (with no specific coexistence with radio altimeter arrangements in place) for many years. This contrasts with some statements that simply compare recent/new 5G use with existing low power on the ground satellite downlink only use of the band (in addition to completely ignoring use above the band used by radio altimeters).

- Perhaps more importantly, the regulatory environment that envisages fixed and mobile use of bands adjacent and nearby to the band used by radio altimeters has not changed in a substantial way for many years. The apparent possible implication, that radio altimeters are specifically designed to operate solely with the currently deployed radiocommunications environment rather than that contemplated by the regulatory environment, is noteworthy.

## Alignment between theory and experience concerning interference risk

From the various studies, trials and observations on existing deployments, there is inconsistency between the expected theoretical potential for interference and empirical evidence from real-world WBB deployments.

There has been no confirmed event of interference to a radio altimeter caused by any operationally deployed WBB system globally. The section on Coexistence evaluation and its references to appendices E and F, give examples where issues might be expected to have occurred based on some studies. However, it was not possible to identify any event that could be confirmed to have occurred because of WBB in the 3400–4000 MHz range.

The possible reasons for this divergence include:

    - Inaccuracies in modelling WBB networks and especially AAS.

    - Conservative factors in modelling coexistence between WBB networks and radio altimeters.

    - Inability to determine whether specific radio altimeter models used in studies align with those in potential real-world events.

    - The difficulty in conclusively demonstrating a real-world radio altimeter event was caused by a specific WBB transmission (or was due to radiofrequency interference at all).

## Performance of radio altimeters

The spectrum management coexistence performance of some current radio altimeters appears to be generally poor and not conducive to good spectrum management outcomes. Importantly, even if this observation only applies to a subset of the currently deployed radio altimeter fleet, it still has a significant impact on the overall coexistence picture as these lower-performing radio altimeters may need to be considered unless it can be conclusively demonstrated that they currently aren’t, or won’t be in the future, used in Australia.

This issue is characterised by industry testing that indicates that the apparent susceptibility of some radio altimeters to operation of WBB devices far (100s of MHz) from the radio altimeter operating band via a mechanism whereby fundamental WBB emissions interact with radio altimeter operation due to poor radio altimeter filter performance. From the same testing, it also appears that even relatively low levels of unwanted emissions falling within the radio altimeter passband have the theoretical possibility of affecting their operation in some scenarios.

While the current use of spectrum nearby to the radio altimeter band is changing through the increased use for WBB under mobile and fixed service allocations, the regulatory environment in terms of service allocations has remained the same for many years. It appears that, to date, some radio altimeter devices and standards have not adequately considered the spectrum environment that they are intended to operate within, with the corresponding consequences on efficient spectrum sharing and coexistence management.

While this situation is not ideal, the ACMA acknowledges that this current reality needs to be considered and cannot be ignored in its current spectrum management decision-making process.

Positively, this situation appears to have been recognised and is being improved through plans or proposals for the retrofitting of filters to existing radio altimeters in the US and Canada, and work underway within the aviation industry to develop new radio altimeter standards.

## Conclusions and proposed approach

The theoretical studies and empirical evidence from real-world deployments about the coexistence of radio altimeters and WBB do not provide a definitive picture of the true likelihood of interference to radio altimeters. Some lack of certainty and consensus on spectrum coexistence matters is not unusual, given the differing perspectives and motivations of stakeholders, noting that coexistence is often informed by theoretical modelling with associated inherent limitations. However, the critical nature of radio altimeters in some use-cases, makes this lack of definitive evidence more challenging than usual.

Given these circumstances, the ACMA currently considers it appropriate to adopt a precautionary approach to the implementation of new WBB uses into this environment. This is based on the concept that the absence of definitive data to accurately determine risk likelihood and consequence (in this case, risk of interference to radio altimeters) should not preclude action to mitigate the possible risk through the adoption of prudent controls (that is, regulatory measures). The consequences of excessive or unnecessary regulatory restrictions on WBB systems may, however, reduce the long-term benefits that these services can provide to the Australian community.

We propose to apply both ongoing and interim (time-limited) mitigations for the licensing technical frameworks in 3400–4000 MHz.

The ACMA is of the view that the interim mitigations be time limited. This is to ensure that the impacts of these mitigations on WBB (and the benefits they provide) are limited to the extent compatible with providing a reasonable opportunity for improved radio altimeter devices to be implemented in Australia. There is an expectation that the coexistence environment between radio altimeters and WBB will be significantly improved through the retrofitting of improved performing radio altimeters. This would enable interim mitigations to be either removed entirely or significantly reduced, such that there would be minimal restrictions on WBB.

This approach, of implementing interim mitigations while radio altimeters are retrofitted, is consistent with the approach adopted in the US and proposed for Canada. In these countries, timeframes have been established that will see the retrofitting of filters and/or radio altimeter units to affected radio altimeters – proposed by 1 February 2024 in the US and proposed by 31 March 2025 in Canada.

The ACMA is proposing to remove the interim mitigations by 31 March 2025 (‘interim mitigation end date’). This would provide an additional 21 months from the July 2023 timeframe adopted in the US, in recognition of the status of developments in the band in Australia, potential equipment availability issues and time necessary to undertake such changes on aircraft in the Australian environment.

It will be a matter for CASA to determine what appropriate possible restrictions would be placed on aircraft operations after the interim mitigation end date.

This approach has been formed by the ACMA based on the currently available evidence. While the ACMA will consider the information before it at the time and may contemplate retention of modest ongoing additional mitigations after this date or to modestly extend the date of interim mitigations, the currently available evidence does not suggest that this will be necessary. indicates this is unlikely to be necessary. Consequently, the ACMA recommends industry work on the basis that all the interim mitigations will be removed in their entirety within the proposed time.

This approach has been formed by the ACMA based on the currently available evidence. While the ACMA will consider the information before it at the time and may contemplate retention of modest ongoing additional mitigations after this date or to modestly extend the date of interim mitigations, the currently available evidence indicates this is unlikely to be necessary. Consequently, the ACMA recommends industry work on the basis that all the interim mitigations will be removed in their entirety within the proposed time.

The ACMA recognises that a reasonable timeframe for the implementation of improved radio altimeters depends on a range of factors. Accordingly, the ACMA is seeking views from industry on an implementation timeframe.

Ongoing mitigations

    - A 200 MHz guard band in 4000–4200 MHz where WBB deployments will not be permitted (previously determined).

    - A total radiated power (TRP) limit of 48 dBm/5 MHz.

    - A range of unwanted emission limits (articulated as a TRP or conducted power per antenna port), in addition to those that apply during the temporary mitigations. The limits can be found in the draft spectrum licence for spectrum-licensed services and the area-wide licence condition determination for AWLs. The limits for cases are the same, but frequency boundaries for their application differ between the currently proposed spectrum licence and AWL allocations due to different expected ranges of allocation.

The ACMA previously consulted on a proposal within the TLG and the proposal received no support. The proposal is included here for completeness but it is not currently proposed that the ACMA will implement this element:

    - For all WBB licensees in 3400–4000 MHz, it is recommended when planning and deploying their networks, to take note of the co-existence issues that may arise due to aeronautical radio-navigation services, typically radio altimeters on aircraft, that may operate in the 4200–4400 MHz band. It is also recommended that affected licensees seek to coordinate with airports, heliports and aircraft operators to help prevent, manage and resolve interference that may arise to aeronautical radio-navigation stations.

    - This ongoing mitigation could be implemented by a clause in RALI MS47 and an amended Radiocommunications Advisory Guidelines (Managing Interference from Spectrum Licensed Transmitters — 3.4 GHz Band).

- Ongoing unwanted emission limits

The ongoing unwanted emission limits referred to paragraph 4.22 iii are reproduced below.  As per the footnote 18, as these limits are either TRP or per antenna port based, they cannot be directly compared with the interim unwanted emission limits that are EIRP density based. However, with practical base station antenna systems, it would be expected that the ongoing limits would exceed the requirements of the interim limits for parts of the 4200–4400 MHz frequency range (especially for future spectrum-licensed services).

These unwanted emission limits include requirements for both the out-of-band (near to the 5G signal) and spurious (far from the 5G signal) domains but are not specifically identified.

While clauses covering all possible frequency offsets from base stations and frequency ranges have been reproduced, only those affecting requirements in the radio altimeter band (4200–4400 MHz) are relevant to this issue.

- Unwanted emission limits for spectrum licence allocations (expected up to 3800 MHz)

Frequency breakpoints in some of the limits have been designed to accommodate standard 3GPP NR n78 band equipment (3300–3800 MHz). The clause numbers, that start from 5, are the referenced licence core conditions. Clauses related to devices exempt from registration are low-powered devices, including handsets.

- (1) The licensee must ensure that radiocommunications transmitters operated under this licence that are not exempt from the registration requirement under Statutory Licence Condition 4 of Licence Schedule 3 do not exceed the unwanted emission limits in Core Conditions 6, 7, 9(b) and 10(b)

- The licensee must ensure that radiocommunications transmitters operating under this licence that are exempt from the registration requirement under Statutory Condition 4 of Licence Schedule 3 do not exceed the unwanted emission limits described in Core Conditions 8, 9(a) and 10(a).

- The licensee must ensure that radiocommunications receivers operated under this licence do not exceed the unwanted emission limits described in Core Conditions 11 and 12.

- The unwanted emission limits in Table 3 apply to radiocommunications transmitters with non-AAS:

  - within the 3360 MHz to 3840 MHz frequency range;

  - at frequencies outside the upper or lower frequency limits set out in Part 2 of Licence Schedule 1; and

  - offset from the upper or lower frequency limits set out in Part 2 of Licence

Schedule 1;

where:

foffset is the frequency offset from the upper or lower frequency limits set out in Part 2 of Licence Schedule 1. The closest -3dB point of the specified bandwidth to the upper or lower frequency limits of the licence is placed at foffset.

Table 3: Radiocommunications transmitter unwanted emission limits for registered devices with non-AAS

| Frequency offset range (foffset) | Mean power per antenna port (dBm) | Specified Bandwidth |
| --- | --- | --- |
| 0 kHz < foffset < 5 MHz | -7 – (7/5) f offset (MHz) | 100 kHz |
| 5 MHz < foffset < 10 MHz | -14 | 100 kHz |
| foffset > 10 MHz | -15 | 1 MHz |

- The unwanted emission limits in Table 4 apply to radiocommunications transmitters with AAS:

  - within the 3360 MHz to 3840 MHz frequency range;

  - at frequencies outside the upper or lower frequency limits set out in Part 2 of Licence Schedule 1; and

  - offset from the upper or lower frequency limits set out in Part 2 of Licence

Schedule 1;

where:

foffset is the frequency offset from the upper or lower frequency limits set out in Part 2 of Licence Schedule 1. The closest -3dB point of the specified bandwidth to the upper or lower frequency limits of the licence is placed at foffset.

Table 4: Radiocommunications transmitter unwanted emission limits for registered devices with AAS

| Frequency offset range (foffset) | Total radiated power (dBm) | Specified Bandwidth |
| --- | --- | --- |
| 0 kHz < foffset < 5 MHz | 2 – (7/5) f offset (MHz) | 100 kHz |
| 5 MHz < foffset < 10 MHz | -5 | 100 kHz |
| foffset > 10 MHz | -6 | 1 MHz |

- The unwanted emission limits in Table 5 apply:

- within the 3295 MHz to 3905 MHz frequency range;

- at frequencies outside the upper or lower frequency limits set out in Part 2 of Licence Schedule 1; and

- offset from the upper or lower frequency limits set out in Part 2 of Licence Schedule 1;

where:

foffset is the frequency offset from the upper or lower frequency limits set out in Part 2 of Licence Schedule 1. The closest -3dB point of the specified bandwidth to the upper or lower frequency limits of the licence is placed at foffset.

Table 5: Radiocommunications transmitter unwanted emission limits for devices exempt from registration

| Frequency offset range (foffset) | Total radiated power (dBm) | Specified Bandwidth |
| --- | --- | --- |
| 0 kHz < foffset < 1 MHz | -15 | 30 kHz |
| 1 MHz < foffset < 5 MHz | -10 | 1 MHz |
| 5 MHz < foffset < 100 MHz | -13 | 1 MHz |
| foffset > 100 MHz | -25 | 1 MHz |

- For radiocommunications transmitters operated under this licence:

- which are non-AAS devices that are exempt from the registration requirements under Statutory Licence Condition 4 of Licence Schedule 3 – the unwanted emission limits in Table 6 apply at frequencies outside the 3295 MHz to 3905 MHz frequency range; or

- which are devices with non-AAS that are not exempt from the registration requirements under Statutory Licence Condition 4 of Licence Schedule 3 – the unwanted emission limits in Table 6 apply at frequencies outside the 3360 MHz to 3840 MHz frequency range;

when measured over the specified bandwidth for the relevant frequency range.

Table 6: Radiocommunications transmitter unwanted emission limits for registered non-AAS devices and non-AAS devices exempt from registration

| Frequency range (f) | Mean power per antenna port (dBm) | Specified Bandwidth |
| --- | --- | --- |
| 9 kHz < f < 150 kHz | -36 | 1 kHz |
| 150 kHz < f < 30 MHz | -36 | 10 kHz |
| 30 MHz < f < 1 GHz | -36 | 100 kHz |
| 1 GHz < f < 19 GHz | -30 | 1 MHz |

- For radiocommunications transmitters operated under this licence:

  - which are AAS devices that are exempt from the registration requirements under Statutory Licence Condition 4 of Licence Schedule 3 – the unwanted emission limits in Table 7 apply at frequencies outside the 3295 MHz to 3905 MHz frequency range; or

  - which are devices with AAS that are not exempt from the registration requirements under Statutory Licence Condition 4 of Licence Schedule 3 – the unwanted emission limits in Table 7 apply at frequencies outside the 3360 MHz to 3840 MHz frequency range;

when measured over the specified bandwidth for the relevant frequency range.

Table 7: Radiocommunications transmitter unwanted emission limits for registered AAS devices and AAS devices exempt from registration

| Frequency range (f) | Total radiated power (dBm) | Specified Bandwidth |
| --- | --- | --- |
| 9 kHz < f < 150 kHz | -27 | 1 kHz |
| 150 kHz < f < 30 MHz | -27 | 10 kHz |
| 30 MHz < f < 1 GHz | -27 | 100 kHz |
| 1 GHz < f < 19 GHz | -27 | 1 MHz |

- The unwanted emission limits in Table 8 apply to radiocommunication receivers with non-AAS:

- for radiocommunications receivers that are not registered on the Register at frequencies outside the 3295 MHz to 3905 MHz frequency range; and

- for radiocommunications receivers that are registered on the Register at frequencies outside the 3360 MHz to 3840 MHz frequency range;

when measured over the specified bandwidth for the relevant frequency range.

Note 1:	Although not mandatory, the registration of radiocommunications receivers to be operated under this licence is advised because one of the matters the ACMA will take into account in settling interference disputes is the time of registration of the receiver involved in the interference.

Note 2: 	The unwanted emission limits in Core Condition 11 only apply during periods an associated radiocommunications transmitter in the device is not operating.

Table 8: Radiocommunications receiver unwanted emission limits for non-AAS receivers

| Frequency range (f) | mean power per receiver port (dBm) | Specified Bandwidth |
| --- | --- | --- |
| 30 MHz < f < 1 GHz | -57 | 100 kHz |
| 1 GHz < f < 19 GHz | -47 | 1 MHz |

- For radiocommunications receivers which are devices with AAS that are not exempt from the registration requirements under Statutory Licence Condition 4 of Licence Schedule 3 – the unwanted emission limits in Table 9 apply at frequencies outside the 3360 MHz to 3840 MHz frequency range, when measured over the specified bandwidth for the relevant frequency range, when measured over the specified bandwidth for the relevant frequency range.

Note 1:	Although not mandatory, the registration of radiocommunications receivers to be operated under this licence is advised because one of the matters the ACMA will take into account in settling interference disputes is the time of registration of the receiver involved in the interference.

Note 2:	The unwanted emission limits in Core Condition 12 only apply during periods an associated radiocommunications transmitter in the device is not operating.

Table 9: Radiocommunications receiver unwanted emission limits for AAS receivers

| Frequency range (f) | Total Radiated Power (dBm) | Specified Bandwidth |
| --- | --- | --- |
| 30 MHz < f < 1 GHz | -27 | 100 kHz |
| 1 GHz < f < 19 GHz | -21 | 1 MHz |

- Unwanted emission limits for area-wide licence allocations (currently proposed for up to 4000 MHz in remote areas initially)

Frequency breakpoints in some of the limits have been designed to accommodate standard 3GPP NR n77 band equipment (3300–4200 MHz but restricted to 4000 MHz).

Unwanted emission limits – application

(1)	The licensee of an area-wide licence must not operate a radiocommunications transmitter that is not a low risk transmitter if its unwanted emissions exceed the limits in subclauses (3), (4), (6) or (7).

(2)	The licensee of an area-wide licence must not operate a radiocommunications transmitter that is a low risk transmitter if its unwanted emissions exceed the limits in subclauses (5), (6) or (7).

Radiocommunications transmitters other than low risk transmitters

(3)	For radiocommunications transmitters without AAS, the unwanted emission limit in Table 1, measured over the specified bandwidth, applies at frequencies:

(a)	outside the upper or lower frequency limits specified in the licence; and

(b)	offset from the upper and lower frequency limits set out in the licence;

where:

foffset means the frequency offset from the upper or lower frequency limits set out in the licence. The closest -3dB point of the specified bandwidth to the upper and lower frequency limits set out in the licence is placed at foffset.

Note:  This subclause does not apply to low risk transmitters – see subclause (1).

Table 1: Unwanted emission limit – radiocommunications transmitters without AAS that are not low risk transmitters

| Column 1 | Column 2 | Column 3 |
| --- | --- | --- |
| Frequency range (foffset) | Mean power per antenna port (dBm) | Specified bandwidth |
| 0 MHz ≤ foffset < 5 MHz | -7 – (7/5) foffset (MHz) | 100 kHz |
| 5 MHz ≤ foffset < 10 MHz | -14 | 100 kHz |
| foffset ≥ 10 MHz | -15 | 1 MHz |

(4)	For radiocommunications transmitters with AAS, the unwanted emission limit in Table  2, measured over the specified bandwidth, applies at frequencies:

(a)	outside the upper or lower frequency limits specified in the licence; and

(b)	offset from the upper and lower frequency limits set out in the licence;

where:

foffset means the frequency offset from the upper or lower frequency limits set out in the licence. The closest -3dB point of the specified bandwidth to the upper and lower frequency limits set out in the licence is placed at foffset.

Note:  This subclause does not apply to low risk transmitters – see subclause (1).

Table 2: Unwanted emission limit – radiocommunications transmitters with AAS that are not low risk transmitters

| Column 1 | Column 2 | Column 3 |
| --- | --- | --- |
| Frequency range (foffset) | Total radiated power (dBm) | Specified bandwidth |
| 0 MHz ≤ foffset < 5 MHz | 2 – (7/5) foffset (MHz) | 100 kHz |
| 5 MHz ≤ foffset < 10 MHz | -5 | 100 kHz |
| foffset ≥ 10 MHz | -6 | 1 MHz |

Radiocommunications transmitters that are low risk transmitters

(5)	For low risk transmitters, the unwanted emission limit in Table 3, measured over the specific bandwidth, applies at frequencies:

(a)	outside the upper or lower frequency limits specified in the licence; and

(b)	offset from the upper and lower frequency limits set out in the licence;

where:

foffset means the frequency offset from the upper or lower frequency limits set out in the licence. The closest -3dB point of the specified bandwidth to the upper and lower frequency limits set out in the licence is placed at foffset.

Note:  This subclause only applies to low risk transmitters – see subclause (2).

Table 3: Unwanted emission limit – radiocommunications transmitters that are low risk transmitters

| Column 1 | Column 2 | Column 3 |
| --- | --- | --- |
| Frequency range (foffset) | Total radiated power (dBm) | Specified bandwidth |
| 0 MHz ≤ foffset < 1 MHz | -15 | 30 kHz |
| 1 MHz ≤ foffset < 5 MHz | -10 | 1 MHz |
| 5 MHz ≤ foffset < 100 MHz | -13 | 1 MHz |
| foffset ≥ 100 MHz | -25 | 1 MHz |

All radiocommunications transmitters

(6)	For radiocommunications transmitters without AAS, the unwanted emission limit in Table 4, measured over the specified bandwidth, applies at frequencies:

(a)	for low risk transmitters – outside the 3295 MHz to 4105 MHz frequency band; or

(b)	otherwise – outside the 3360 MHz to 4240 MHz frequency band;

where:

f means the frequency of the unwanted emission.

Note:  This subclause applies to all radiocommunications transmitters without AAS – see subclauses (1) and (2).

Table 4: Unwanted emission limit – radiocommunications transmitters without AAS, outside certain bands

| Column 1 | Column 2 | Column 3 |
| --- | --- | --- |
| Frequency range (f) | Mean power per antenna port (dBm) | Specified bandwidth |
| 9 kHz ≤ f < 150 kHz | -36 | 1 kHz |
| 150 kHz ≤ f < 30 MHz | -36 | 10 kHz |
| 30 MHz ≤ f < 1 GHz | -36 | 100 kHz |
| 1 GHz ≤ f < 19 GHz | -30 | 1 MHz |

(7)	For radiocommunications transmitters with AAS, the unwanted emission limit in Table 5, measured over the specified bandwidth, applies at frequencies:

(a)	for low risk transmitters – outside the 3295 MHz to 4105 MHz frequency band; or

(b)	otherwise – outside the 3360 MHz to 4240 MHz frequency band;

where:

f means the frequency of the unwanted emission.

Note:  This subclause applies to all radiocommunications transmitters with AAS – see subclauses (1) and (2).

Table 5: Unwanted emission limit – radiocommunications transmitters with AAS, outside certain bands

| Column 1 | Column 2 | Column 3 |
| --- | --- | --- |
| Frequency range (f) | Total radiated power (dBm) | Specified bandwidth |
| 9 kHz ≤ f < 150 kHz | -27 | 1 kHz |
| 150 kHz ≤ f < 30 MHz | -27 | 10 kHz |
| 30 MHz ≤ f < 1 GHz | -27 | 100 kHz |
| 1 GHz ≤ f < 19 GHz | -21 | 1 MHz |

Interim mitigations:

    - For deployments above 3800 MHz around identified runways:

      - exclusion zones, where no WBB services are permitted

      - restricted zones, except in the 3950–4000 MHz range in metro and regional restricted cell areas, with a PFD limit in the restricted zones.

    - For deployments above 3800 MHz everywhere:

      - additional unwanted emission EIRP density limits in 4200–4400 MHz

      - maximum EIRP density limits

      - restricting any WBB antenna system to point maximum power or scan any beam below the horizon only

      - grating lobes of WBB antenna systems should be minimised as much as is practicable.

- An ‘identified runway’ is one identified by CASA as requiring the protection of radio altimeters due to safety and/or important operational requirements. An initial list is provided at Appendix G: Identified runways.

- An ‘exclusion zone’ is a zone made of 3 contiguous areas using the following four dimensions:

      - An area extending lengthwise from the landing end of an identified runway for the ‘extension length’ indicated in Table 1 column B for highest operating frequency.

      - An area extending along the length of the runway to the far end.

      - The exclusion zone extends beyond the far end of the runway for a length equal to the ‘half-width’ in Table 1 column C.

      - The width of the exclusion zone each side of the runway centre-line, for the 3 parts in 1, 2, 3 is the ‘half-width’ using the applicable maximum operating frequency row in Table 1 column C.

- A ‘restricted zone’ is an area extending lengthwise from a landing end only of an exclusion zone, and horizontally from an identified runway centreline. The dimensions apply using the maximum operating frequency row in Table 1 column D.

- Figures 3 and 4 below are instructive for an example case at 3750 MHz.

Zone sizes example (not to scale) for a runway that requires protection in both directions

Zone sizes example (not to scale) for a runway that requires protection in only one direction

- The ‘PFD limit’ value is defined in Table 1 column F for the maximum operating frequency row. The PFD cannot exceed that value at a height above the ground of the corresponding column G of Table 1, for any percentage of the time for all combinations of elevation and azimuth angles above the horizon, for deployments within the restricted zone.

- The ‘Additional unwanted emission limits’ is defined in Table 1 column E for the minimum operating frequency row and applies across 4200–4400 MHz.

- The ‘maximum EIRP density limit’ is 62 dBm/MHz, outside of the restricted cell areas in 3950–4000 MHz.

Example table of interim mitigations in a frequency range

| A | B | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- | --- |
| Base station operating frequency (MHz) | Exclusion zone extension length (m) | Exclusion zone half-width (m) | Restricted zone size (m) (Length x half-width) | Unwanted emissions limit EIRP (dBm/MHz) | Power flux density limit (dBW/m2/MHz) | Power flux density limit restriction height (m) |
| >3800 to 3850 | 2,890 | 1,760 | 5,310 x 560 | 5.1 | -41.0 | 140 |
| >3850 to 3900 | 3,000 | 1,870 | 5,200 x 680 | 5.5 | -42.4 | 145 |
| >3900 to 3950 | 3,110 | 1,980 | 5,090 x 840 | 5.8 | -43.7 | 150 |
| >3950 to 4000 (remote areas) | 3,230 | 2,100 | 4,970 x 1,030 | 6 | -45.2 | 155 |
| >3950 to 4000 (Restricted cell use, metro and regional areas) | 610 | 12 | N/A | -36.4 | N/A | N/A |

It should be noted that any list of identified runways may need to change as airport runways are re-developed or upgraded to improved landing systems. These planning timeframes are expected to be around 5 to 10 years for consideration of new runways but can be considerably shorter for the consideration of new Instrument Landing Systems (ILS) for existing approaches. Any consideration of changes to associated regulatory arrangements would be undertaken through normal consultation mechanisms.

It should be noted that, for practical WBB equipment (outside of the restricted cell areas), the additional unwanted emissions EIRP limit may already be achieved in parts of the 4200–4400 MHz range using the ongoing unwanted emissions limits based on TRP or conducted power per antenna port limits.

Basis for the proposed mitigations

The intent and basis of each proposed mitigation is as follows:

    - A 200 MHz guard band in 4000–4200 MHz: already determined as a reasonable guard band and to provide exclusive use by incumbent services types.

    - (not proposed to be implemented) For all WBB licensees in 3400–4000 MHz, it is recommended when planning and deploying their networks, to take note of the co-existence issues that may arise due to aeronautical radio-navigation services, typically radio altimeters on aircraft, that may operate in the 4200–4400 MHz band. It is also recommended that affected licensees seek to coordinate with airports, heliports and aircraft operators to help prevent, manage and resolve interference that may arise to aeronautical radio-navigation stations. A general requirement to consider the potential for interference even where no specific mitigations apply.

    - Interim mitigations above 3800 MHz for ‘identified runways’:

      - Exclusion zones: The zone size based on the French derivation that uses the RTCA ITM for usage category 1 (UC1) plus a 6 dB safety margin. Size adapted to EIRPs necessary in Australia and the frequency range of allocation. Designed to protect aircraft landings.

      - Restricted zones: The zone size based on the French derivation that uses the RTCA ITM for UC 1 but with no safety margin. The size is adapted to EIRPs necessary in Australia and frequency range of allocation. The PFD limit calculation the same as the original Canadian method but adapted to EIRPs necessary in Australia and frequency range of allocation. Designed to protect aircraft landing approaches.

- As per Appendix D, restricted zones were determined not to be required for the restricted cell areas in 3950–4000 MHz as the restricted cell EIRP limit means that the RTCA ITM UC1 ITMs are always met outside of the exclusion zones at any practical aircraft height above ground.

    - General interim mitigations above 3800 MHz:

      - Additional unwanted emission limits in 4200–4400 MHz: These have been based on the use of RTCA ITMs for UC1 with no margin and are designed to protect aircraft landing approaches, plus also providing generally reduced risk of potential issues for all flight cases. Using an EIRP limit as opposed to a TRP/conducted power limit provides greater certainty as to the coexistence environment.

      - A maximum EIRP density limit for all WBB systems: Integral to the other calculations, but a reasonable limit to permit macro WBB deployments while also providing generally reduced risk of potential issues for all flight cases. Using an EIRP limit as opposed to a TRP limit provides greater certainty as to the coexistence environment.

      - Restricting any WBB antenna system to point maximum power or scan any beam below the horizon only: This is designed to ensure that antenna systems do not scan to handsets located on aircraft and also providing generally reduced risk of potential issues for all flight cases.

      - Grating lobes of WBB antenna systems should be minimised as much as is practicable. This is designed to ensure unwanted power from the base station is minimised in the potential directions of aircraft, to generally reduced risk of potential issues for all flight cases.

- RTCA UC1 is defined as Commercial Air Transport Aircraft in the RTCA report, being ‘commercial air transport airplanes, both single-aisle and wide-body’. The RTCA report discusses the relationship of their usage categories and ILS categories. They consider that ILS CAT II/III approaches are critical and are a common occurrence for UC1. Consequently, consistent with approaches in France, Canada and the US, the mitigations have been designed to primarily protect UC1 landings, but with some consideration to other flight scenarios and categories.

# Appendix A: Wireless Broadband

## Overview

The term wireless broadband (WBB) refers to broadband connectivity in both mobile and fixed environments and a range of use-cases.

Three WBB use-cases have been previously characterised by the ACMA in its spectrum planning activities:

WA WBB (wide area wireless broadband): this encompasses the deployment model mobile network operators (MNOs) and, in some cases, fixed wireless service providers such as NBN Co or other private WBB interests. WA WBB uses are typically supported by issuing spectrum licences covering large geographical areas.

macro cell LA WBB (local area wireless broadband): this encompasses the deployment of a limited number of WBB services requiring a medium-to-large cell size, as is often seen for services delivered by wireless internet service providers and above-ground mine sites. This use-case is typically best supported by issuing apparatus licences, such as area-wide licences (AWLs) that enable operation within a desired service area or areas.

restricted cell LA WBB: this encompasses highly localised public, private and enterprise networks, including services at warehouses, factories, airports, ports, transport hubs, hospitals, schools and smart buildings. This use-case is typically best supported by class licence or apparatus licence (such as AWL) arrangements that enable operation within a highly localised service area or areas.

Often, but not always, WBB systems utilise equipment consistent with specifications developed by the 3rd Generation Partnership Project (3GPP).

## Technical characteristics

There are several technical characteristics for WBB services that are relevant to the consideration of coexistence with radio altimeters.

- In band or ‘wanted’ emissions

In-band emissions are those necessarily required by the WBB service to deliver its data and are within the necessary wanted (channel) bandwidth of the WBB service that a receiver needs to demodulate and decode to obtain the data. For example, a 100 MHz bandwidth 5G new radio carrier provided by an MNO or a 20 MHz carrier provided by a NBN fixed wireless base station. In terms of coexistence with radio altimeters, the in-band emissions of interest are those that fall outside of the 4200–4400 MHz range and are considered for the overload mechanism.

The power level of the in-band emissions will generally be limited by either practical considerations and/or standards and regulations. Typically, in Australian technical frameworks, the total radiated power (TRP) of a carrier is restricted to at most 41 dBm/MHz, but are often lower in practice, plus whatever the net gain of the antenna system is connected to, or integrated with, the base station.

In a radio altimeter context, the ability of a radio altimeter to reject WBB in-band emissions is affected by the performance of the radio altimeters selectivity filter.

- Unwanted emissions

Unwanted emissions are those incidental (but unavoidable in practice) to the WBB in-band emissions. They occur outside of the wanted bandwidth but cannot completely be eliminated. These are often classified further into out-of-band emissions (those that are close to the wanted emissions) and spurious emissions (the others, further away from the wanted emissions). In terms of coexistence with radio altimeters, the unwanted emissions of interest are those that fall into the 4200–4400 MHz range and are applicable for the receiver de-sensitisation and false-altitude reports mechanisms (although any power presented outside of the 4200–4400 MHz range to the radio altimeter receiver can be considered for the overload mechanism).

The power level of the unwanted emissions will be limited by either practical considerations and/or standards and regulations. Permitted levels will vary and regulations generally consider different limits for out-of-band compared with spurious emissions (stricter). Where practicable, alignment with international standards is often proposed, such as the 3GPP suite of standards for 5G technologies. If required for coexistence with specific services and if it is considered practicable, regulated unwanted emissions can be made more stringent.

In a radio altimeter context, a radio altimeters selectivity filter cannot do anything about WBB unwanted emissions that fall into the 4200–4400 MHz range. There are some emerging digital signal processing (DSP) techniques within some radio altimeters that, by being able to identify the interference from the radio altimeter signal, can reduce the effect of this interference falling in-band to the radio altimeter.

What is considered or called ‘in-band’ vs ‘out-of-band’ can vary depending on your perspective. Wanted in-band WBB emissions are out-of-band as far as a radio altimeter receiver is concerned and, conversely, WBB unwanted or ‘out-of-band’ emissions may fall ‘in-band’ to a radio altimeter receiver.

ITU-R Rec SM.1541 and SM.329 provide further information and definitions.

- Antennas

Antenna systems for WBB services have advanced significantly in capability and complexity in recent years and, consequently, are often classified in standards, recommendations, and regulations into 2 broad technology types: AAS and non-AAS.

- AAS

‘Active’ or ‘Advanced’ antenna systems (AAS) in a WBB context are those that make use of dynamic multi-antenna technologies like beamforming and MIMO to improve network performance. ACMA technical frameworks often define them as:

- … an antenna system where the amplitude and/or phase between antenna elements is continually adjusted, resulting in an antenna pattern that varies in response to short term changes in the radio environment.

Often the RF electronics is integrated with the antenna. This is one of reasons why the term ‘total radiated power’ is used as a metric for base station power as there is no means to directly measure the power of the ‘transmitter’ alone.

AAS antennas are increasingly used by many WBB systems, especially with the advent of 5G.

An AAS may have a higher peak gain than a non-AAS due to the ability to form one or more beams dynamically and electronically. However, as they are dynamic and gain in any one direction varies with time (due to other characteristics of typical technologies using them such as 5G), any coexistence consideration with radio altimeters should, ideally, consider this dynamic nature.

As a result of the dynamic nature of AAS, care needs to be taken in modelling steering and scanning beams as, dependent on the design and specific circumstances, unwanted ‘side-lobe’ levels can potentially be relatively high as a result.

- Non-AAS

In a WBB context, non-AAS antennas are those that do not use dynamic beamforming or MIMO techniques. They may be a traditional ‘passive’ antenna or have some active elements but do not use them to dynamically form or steer beams.

For typical systems in the 3400–4000 MHz range, non-AAS have a peak gain of 16–20 dBi and AAS have 23-25 dBi.

- Other aspects

WBB networks may only transmit near maximum power when needed and there is a WBB receiver actively using the base station. Consequently, assumptions should be made about various time-based factors related to their time-varying use when considering coexistence.

When conducting coexistence studies between WBB and radio altimeters, it is also important to model the nature of the WBB system, especially the antenna, as close to reality as practicable, both in terms of design as well as dynamic beamforming and scanning operation and operating powers. There are currently practical limits to modelling AAS, though current working documents within ITU-R Working party 5D (IMT Systems) provide some guidance.

# Appendix B: Radio altimeters

## Overview

Radio altimeters provide high integrity, accurate measurements for aircraft height above terrain and obstacles. ITU-R M.2059 also gives a useful overview of both their function and operational use in aviation.

Radio altimeters (also known as low range radar/radio altimeters (LRRA), or by the abbreviations RALT or RADALT) are critical sensors used to enable and enhance several different safety and navigation functions throughout all phases of flight on a wide range of commercial, civil and military aircraft. Such functions include, but are not limited to, Ground Proximity Warning Systems (GPWS), Terrain Awareness Warning Systems (TAWS), Traffic Alert and Collision Avoidance Systems (TCAS) and Airborne Collision Avoidance Systems (ACAS), Wind Shear detection systems, flight control systems, auto-land systems (including auto throttle and automated landing flare and rollout) and support in Instrument Landing System (ILS) decision-making and ancillary support.

Radio altimeters measure height above terrain directly using radar principles. Radio emissions are transmitted from the radio altimeter antenna, reflected from terrain/obstacles below the aircraft and received back at the radio altimeter antenna. The received radio altimeter signals are then processed with the resulting height above terrain determined and used by various aircraft systems. Two types of radar altimeters are in use: Frequency Modulation Continuous Wave (FMCW) and Pulsed.

### Spectrum management regulatory status

Being a radar-type system, radio altimeters use the radiofrequency spectrum operating in the Aeronautical Radionavigation Service (ARNS) allocation in 4200–4400 MHz as per the International Telecommunication Union Radio Regulations and as implemented by domestic spectrum management authorities. The Radio Regulations limits (through footnote No. 5.438) use of this ARNS allocation to radio altimeters installed on aircraft and for associated transponders on the ground.

The 4200–4400 MHz band has an allocation to the Aeronautical Mobile (Route) Service (AMSIS) service. The Radio Regulations limits (through footnote No. 5.436) use of this (I(R)S) allocation to Wireless Avionics Intra-Communications (WAICs) systems that operate in accordance with recognised international standards.

### Aviation standards for radio altimeters

The first airworthy radio altimeter specification was the FAA TSO C87 from 1966.

The revised FAA TSO C87A from 2012 references compliance with EUROCAE ED-30 from 1980, with some exceptions, for new models of radio altimeters identified and manufactured after March 2012. The CASA AWB 34-020 Issue 7 notes that TSO-C87A does not provide criteria for compatibility with adjacent band operations, including potential impacts associated with wireless communications system deployments.

The RTCA DO-155 (1974) is similar, but not identical to EUROCAE ED-30. The planned RTCA-DO155A is proposed to be a technically equivalent document to the planned ED-30A to support a FAA Technical Standard Order/European Technical Standard Order (TSO/ETSO).

### Usage

As previously noted, radio altimeters assist with performing several critical and non-critical flight tasks. Taken in part from M.2059:

- Radio altimeters designed for use in automated landing systems are required to achieve an accuracy of 0.9 metres (3 feet) or more. Such elevation readings are transmitted to a pilot’s visual display and to several automatic safety components. Radio altimeters provide an essential informational component of the automatic flight control system for approach and landing, ground proximity warning system , terrain awareness and warning system , flight management guidance computer, flight control systems, electronic centralized aircraft monitoring  and engine-indicating and crew alerting system . In addition, elevation information from radio altimeters is transmitted to the traffic collision-avoidance system and automatic dependent surveillance-broadcast system, which are used to monitor the airspace around an aircraft and to warn pilots of any threat of a mid-air collision.

- Information from radio altimeters is especially critical in low-visibility conditions, but is always imperative. Generally, if a system’s check before take-off indicates that the radio altimeters are non-functioning, a flight must be suspended. If the signal from the radio altimeters is lost during flight, the collision-avoidance and other safety systems listed above are significantly impaired. If the radio altimeters are not functioning properly when an aircraft is approaching and landing, autopilot systems would be unable to function properly. Under the best situation, a crew would manually fly the approach or divert to another airport. However, this increases crew workload and degrades the approach capability, which can result in a “go around” missed approach. Such repeated landing attempts can significantly impact already congested landing patterns, increase air traffic control workload and create safety concerns. In addition, for certain category airports and weather conditions, loss of the radio altimeter system would prevent the authorized landing of the aircraft. Thereby forcing the aircraft to either fly a holding pattern until weather improves or divert to another airport. Because of the importance of radio altimeter functions, the spectrum allocated and used by these devices must be protected from harmful interference and be sufficient to meet accuracy requirements.

- Operational scenarios

- Aircraft approach and landing

- Analysing a typical landing profile from 18.5 km (10 nm) to the runway threshold for an aircraft, the avionic system components predominantly in use are the instrument/microwave landing systems, distance measurement equipment, satellite navigation systems radio altimeters, inertial reference systems and the air data computers providing barometric altitude and airspeed.  The flight-management and flight-control computers continuously monitor sensor data input and correlate this data to ensure they are within specific parameter limits, particularly that the radio altimeter height readings between the sensors are correlated to be within tolerance. Auto-throttle is engaged; a stabilized approach with controlled descent rate and speed is maintained. At a pre-established height, the glide-path vertical information sensor data is phased out of the equation by the flight-management computer and the vertical height above the runway surface is provided by the radio altimeter with aural annunciation in feet to initiate flare of the aircraft to touchdown. The flare phase is controlled by the autopilot system using information from the radio altimeter. This flight profile can be achieved in normal or low-visibility conditions.

- If an aircraft loses or receives erroneous radio altimeter data, several consequences can occur depending upon the aircraft type, airport landing requirements or classification, and weather. Loss of radio altimeter data will disable the autopilot resulting in the pilot and co-pilot manually flying and landing the aircraft. Some airport categories or certain weather conditions would prohibit the landing of some types of aircraft without altimeter data. If only one radio altimeter is operational, then the height above ground when the decision to land the aircraft is made must be adjusted to a higher altitude. If visibility is poor, then the aircraft might be forced to wait until the weather gets better or land at a different airport. If the radio altimeter signal receives harmful interference during the final stages of landing, then a hazardous or catastrophic situation could occur. At best, the flight crew workload increases significantly; at worst the aircraft, crew and passengers are placed in a catastrophic situation.

- Terrain avoidance and ground proximity warning systems

- A ground proximity warning system (GPWS) onboard an aircraft provides an automatic and very distinctive aural warning to a flight crew when the aircraft is in close proximity to ground below the aircraft. Another type of ground proximity warning system is known as the terrain avoidance warning system (TAWS) which also provides distinctive aural warnings based on the level of ground proximity threat in front of the aircraft.

- The design and intention of GPWS and TAWS is to prevent controlled flight into terrain (CFIT).  Radio altimeters are integrated into the aircraft flight management computer systems and provide critical data, such as effective aircraft height above terrain to the GPWS and TAWS. Working together, the radio altimeter, TAWS and GPWS enable safe operations when flying close to terrain, typically during low visibility operations and precision approach and landings or in mountainous areas.

- In accordance with ICAO Annex 6 – Part 1 Chapter 6, the GPWS provides time-critical alerts when flight conditions are hazardous. The TAWS with GPWS functions are required to provide the flight crew with immediate situational awareness of the aircraft’s height above the ground.

- GPWS alerts are radio altitude based, and derived from the inputs provided to the system and are available from 10 to 1 538 metres (30 to 5 000 feet).

- ICAO requirements

- ICAO Annex 6 Part 1 Chapter 6 specifies the mandatory carriage of GPWS and TAWS with forward-looking terrain functions for certain aircraft weight categories. In addition to these requirements, many Administrations’ aviation regulations and airworthiness requirements mandate the carriage of such equipment as it is directly related to airworthiness and certification dispatch requirements of an aircraft.

- ICAO Annex 6 Part 1 Chapter 6 states:

- “All turbine-engine aeroplanes of a maximum certificated take-off mass in excess of 15 000 kg or authorized to carry more than 30 passengers shall be equipped with a ground proximity warning system which has a forward-looking terrain avoidance function. (other paragraphs have similar provisions for different weight categories of aircraft.)”

## Technical characteristics

### Antenna

The antennas associated with radio altimeter models considered under ITU-R M.2059 typically have gains of 9–13 dBi with 3dB beamwidths of 40–60 degrees. Installed cable loss is typically 6dB, with up to 3 systems installed on one aircraft. In the absence of manufacturer antenna patterns, they are typically modelled as cardioids with the specified beamwidth in its operating bandwidth.

The RTCA report discussed antenna characteristics at 6.3.4 and measured the performance of 2 antennas, both in-band and in the adjacent 3.7–3.98 GHz band (Figures 6-12 and 6-11). In studies, the in-band gains should be used for receiver desensitisation and false altitude report mechanisms, and the out-of-band gains for the overload mechanism, if this is known.

### Interference protection criteria

There are no required interference protection criteria in current radio altimeter standards.

#### ITU-R Recommendation M.2059

M.2059 presented the basic parameters for some anonymised radio altimeters to be able to develop interference protection criteria for the overload, desensitisation and false altitude report mechanisms. Other assumptions would still need to be made, such as the out-of-band performance of the radio altimeter antenna, the loop loss, and any additional safety margins.

M.2059 includes Table 3 describing the RF selectivity for radio altimeters to be used in sharing studies. These characteristics are noted to offer poor selectivity for WBB systems in the adjacent 3700–4000 MHz band.

The characteristics of radio altimeter models listed in Tables 1 and 2 indicate a high degree of variability in the important receiver overload and receiver sensitivity parameters.

#### Black box testing

The AVSI conducted black box lab testing for the RTCA.  The results can be found in the links contained in the RTCA part of the coexistence evaluation section of this paper.

- As can be observed from, and was noted in the AVSI results, there is:

- a high degree in the variation of performance between different radio altimeter models

- a degree of variation between different examples of the same radio altimeter model

- a degree of variation with temperature in a radio altimeter model.

- The RTCA, when collating the characteristics into usage category groups to determine Interference Tolerance Masks (ITMs) from WBB services, took a specific approach by applying additional safety factors to cover the variability in performance.

The RTCA report grouped different radio altimeter models into specific aircraft use-cases (usage categories I, II, and III). The ITMs were then determined for each usage category. It is not known if the specific grouping of radio altimeter models into the use-cases can apply to the Australian aircraft context or if the usage categories themselves are directly comparable for Australian operations.

Japan also conducted initial limited testing of 2 pulsed (rather than FM-CW) radio altimeters of the same model, which was presented at the ICAO FSMP WG-12 meeting. It confirmed the results measured by the Japanese MIC. It concluded that the minimum interference threshold values for the in-band and the out-of-band interference test are −57.7 dBm/MHz and −86.8 dBm/MHz, respectively. Future tests, including on several FMCW Ras, were proposed.

The Japan ENRI also conducted black box testing on a range of radio altimeters, grouping them into 2 use-cases. They concluded generally good alignment with the RTCA/AVSI ITMs.

Lab testing has also been conducted by ISED in Canada and is discussed in the studies section at paragraph 3.55.

#### Comments/observations/comparisons

From examining data on radio altimeter performance, it is apparent RF susceptibility performance is highly variable between different models.

Studies based on the worst-case M.2059 radio altimeter are likely to be more conservative than those using RTCA usage category I ITMs, as the latter radio altimeters perform better than the worst M.2059 radio altimeter.

From the Canadian black box testing, fitting RF filters in front of existing radio altimeter receivers significantly improves performance around 5G fundamental rejection.

## Future radio altimeter performance

It has been acknowledged by the aviation industry that the current standard for the performance of radio altimeters (for example, DO-155 and Eurocae ED 30) were developed many years ago (1974, 1980) when the RF environment in 3400–4200 MHz was much quieter than today, and were not focused on protecting the altimeter receivers from strong RF signals.

#### Proposed new radio altimeter standards

As part of the scope of SC-239, the RTCA proposed that DO-155 be redeveloped into DO-155A by September 2022, with Eurocae ED-30A or an equivalent being developed in parallel under EUROCAE WG-119.

It is understood that the upgrade to the standard will now be a 2-phased approach, with an interim Minimum Operational Performance Standard (MOPS) for December 2022, focusing on improved out-of-band rejection in an unknown future RF environment with no beam directivity limits on WBB, and a final MOPS in December 2023, which will be a complete re-write and modernisation.

See paragraph 7.138 for a discussion of the pre-release of the RTCA DO-399 report, which is intended to support decisions on performance requirements for radio altimeters under the proposed MOPS.

The RTCA also established SC-242 in May 2022, to catalogue current spectrum standards for aviation and provide a new spectrum guidance document.

#### Timeframe to implement

Advice from the aviation industry is that, given typical life-of-type and change management processes, it would take 7–10+ years for the implementation of new radio altimeters using a revised standard. In the US, the FAA expected retrofitting interim improved radio altimeters by July 2023, but has now proposed to delay this to 1 February 2024. This is independent of compliance with the revised radio altimeter MOPS.

#### Risks associated with new standards being developed within the current deployed WBB environment

Any new standard may need to make certain assumptions about the spectrum environment in which the radio altimeters should be designed and/or certified to operate within. If the new standard being developed adopts a certain spectrum environment, which either assumes some mitigations or deployments that are not representative, then the risk is that mitigations will still be required well into the future, even for new equipment.

The minutes of the RTCA SC-239 meeting of March 2022 indicated that there was an acknowledgement that if ‘… BS equipment here is better than its spec, we are now committing to specs that are anticipated to be met worldwide, so we need to have a standard for worldwide usage’. No more recent minutes have yet been made publicly available, but it is understood the standard is now targeting the ‘best possible’ radio altimeter performance regarding out-of-band rejection.

The relevant part of Appendix C: International developments and approaches, through the ECC PT1 work item on the issue, notes that PT1 was tasked with developing ‘long term MFCN parameters for MOPS’ to be able to provide deployment WBB parameter input into the new radio altimeter standard. It recognised that there is a need to ensure the new standards are more compatible with current and future use of spectrum in adjacent frequency bands.

# Appendix C: International developments and approaches

The ICAO FSMP has also been updating an input document at each meeting, summarising the latest international developments. The latest update is from the September 2022 meeting.

## United States

The US has been undergoing a process to allocate the 3700–3980 MHz range to WBB services under the FCC Expanding Flexible Use of the 3.7 GHz to 4.2 GHz Band project. This resulted in rules being adopted in April 2020 and a subsequent spectrum auction in February 2021.

The original FCC report and order and subsequent Federal rule (see section 9 and paragraphs 366 onwards) considered the ‘coexistence with aeronautical radionavigation’. At the time, it considered that the proposed 220 MHz guard band was deemed to be sufficient, but noted the AVSI submitted work and said that:

- Specifically, the technical rules on power and emission limits the Commission sets for the 3.7 GHz Service and the spectral separation of 220 megahertz should offer all due protection to services in the 4.2–4.4 GHz band. The Commission nonetheless agrees with AVSI that further analysis is warranted on why there may even be a potential for some interference given that well-designed equipment should not ordinarily receive any significant interference (let alone harmful interference) given these circumstances.

Since then, there has been significant activity in relation to the potential issue of WBB and radio altimeter coexistence.

There has been ongoing discussion about the RTCA Report 274-20, which was referenced in the 3700–4200 MHz outcomes paper. Some of the submissions on the issue were shared with the TLG, principally from the Cellular Telecommunications and Internet Association (CTIA) and the Radio Technical Commission for Aeronautics (RTCA). In short, there is no agreement from the CTIA that the RTCA report accurately represents the potential for interference from 5G services into radio altimeters.

Most, but not all, correspondence on the issue can be found on the FCC filings for docket 18-122.

To follow the RTCA report, the aviation industry filed a submission to the FCC on 7 December 2020, proposing mitigations based on the RTCA report results.

In November 2021, the Federal Aviation Administration (FAA) released a special airworthiness bulletin (SAIB) AIR-21-18. In summary, it recommended that aviation manufacturers and operators provide radio altimeter design, deployment and usage information to federal authorities so that altimeters can be assessed to guide risk assessment of whether further mitigation is warranted above that stated in the SAIB. It requests altimeter and aircraft manufacturers provide information and conduct their own testing to determine appropriate aircraft operational restrictions. Operators are requested to manage passenger electronic devices and potential degradation of radio altimeter capabilities, ahead of a staged WBB deployment from 5 December 2021, initially restricted to 46 markets and to the 3700–3800 MHz range.

The various statements since made by the FAA on 5G can be found on a collated page, including a link to their 5G and aviation safety page.

The airline pilots association (ALPA) also has a useful history of the issue in the US.

In November 2021, US carriers Verizon and AT&T announced that they would delay their rollouts until 5 January 2022 at the request of the transportation department. On 25 November 2021, they subsequently advised the FCC that they would voluntarily limit 5G base station powers, expiring July 5 2022, unless credible evidence to warrant its continuation emerged.

The temporary limits were:

    - limiting base station EIRP to no more than the lesser of: (a) 62 dBm/MHz or (b) 48 + 20 × log10(1/sin(Ɵ)) dBm/MHz, where Ɵ is the elevation angle above the horizontal plane of the base station antenna.

    - A series of limits near public airports with paved runways and heliports:

      - Limit C-band power flux density (PFD) to a maximum of -30 dBW/m2 /MHz within the horizontal plane surface 300 feet above the established airport elevation described by swinging arcs of 1 nautical mile (6,076 feet) radius from the centre of each end of the primary surface of each paved runway and connecting the adjacent arcs by lines tangent to those arcs.

      - Limit C-band PFD to a maximum of -31 dBW/m2 /MHz at the surface of all paved runways, within the boundaries of the runway edges and runway threshold lines.

      - Limit C-band PFD to a maximum of -19 dBW/m2 /MHz at the surface of all paved aprons and paved taxiways (that is, movement and non-movement areas).

      - Limit C-band EIRP from 5G base stations to no more than 37 dBm/MHz in a rectangular area centered on the runway centerline with a length extending to 1,000 feet beyond the runway threshold at each end of the paved runway, and laterally from the extended centerline, up to and including 600 feet on either side.

      - Limit C-band EIRP from 5G base stations to no more than 55 dBm/MHz EIRP in the area from 600 feet laterally up to and including 1,000 feet laterally on either side of the runway centerline extended to 1,000 feet beyond the runway threshold at each end of the runway.

      - Base stations within the Final Approach Box (FAB), as defined below, at either end of all paved runways, will:

        - Use C-band antennas that do not exceed a centerline height equivalent to a 50:1 approach surface above the touchdown zone elevation beginning at the primary surface, where the touchdown zone elevation is the highest elevation along the first 3,000 feet of the runway at that end of the runway and the primary surface is a surface longitudinally centered on a runway that extends 200 feet beyond each end of that runway.29

        - Limit C-band EIRP above the horizon to no more than the lesser of: (a) 62 dBm/MHz or (b) 39 + [0.005788 – (Dm - 305m)] + [20 × log10(1/sin(Ɵ))] dBm/MHz, where Dm is the horizontal distance from the base station to the runway threshold and Ɵ is the elevation angle above the horizontal plane of the base station antenna.

        - For purposes of (a) and (b), the FAB is defined by an isosceles trapezoid with its short side (top) orthogonal to the runway centerline (extended beyond the runway threshold), centered on the extended runway centerline, with the top positioned 1,000 feet from the runway threshold away from the runway, with a height of 5,100 feet, and with a long side (bottom) that is 3,772 feet.

      - In addition, for all public use heliports, limit C-band PFD to no more than -16 dBW/m2/MHz on the primary surfaces of helipads.

Subsequent to the Verizon and AT&T letter, the Aerospace Industries Association (AIA) wrote to the FCC on 6 December 2021, with their own proposal for safety measures, considering that the Verizon and AT&T proposals were ‘inadequate and far too narrow to ensure the safety and economic vitality of the aviation industry’.  It retains some elements of the Verizon/AT&T proposal but adds several different elements to ‘preclude the need for an AMOC (Alternate Means of Compliance) and provide compatibility with relevant user category 1 (large air transport / cargo) platforms’ (and a subset of Category 2 regional commuter jets/turboprops).

The AIA approach defines limits on defined ‘protected services’ and also, to better cater for helicopters, it also modifies the Verizon/AT&T general EIRP limit curve.

The AIA approach was not accepted and the FAA went on to develop the AMOC approach in conjunction with buffer zones around 50 airports. This included a letter to AT&T and Verizon that documents the temporary deal.

The letter from the licensees to the US Dept of Transportation Secretary of 2 January 2022 details the temporary offer by AT&T and Verizon. This offer implies that the buffer zones are based on that used in France. The relevant figure is reproduced below:

The US buffer zone approach retains the French exclusion zone (based on 58 dBm/MHz general EIRP limit) then uses a small, restricted zone where EIRP is limited to the French example 58 dBm/MHz. Outside of that small, restricted zone, the EIRP limit is the US general temporary 62dBm/MHz. Consequently, the US buffer zone method aligns with the French exclusion zone method in part but not the French restricted zone method. Appendix D: Development of proposed interim mitigations provides detail about the French method for determining exclusion and restricted zone sizes.

The ALPA page notes:

- The FAA has developed a process by which better performing radar altimeters that are able to reject 5G interference can be approved to operate without regard to the AD and NOTAMs. These Alternate Methods of Compliance (AMOC) approvals will be specific to a combination of aircraft model and radar altimeter model.

- The method of approval will take into consideration the performance of the aircraft/radar altimeter combination, as well as the location and power of the 5G transmitter in the vicinity of the airport. Therefore, the AMOCs are being issued with a list of airports where they are effective.

- As of 4/25/2022, FAA has approved AMOCs for most airports for radar altimeters installed on Boeing 717, 737, 747, 757, 767, 777, 787, and MD-10/-11; Airbus A220, A300, A310, A319, A320, A321, A330, A340, A350, and A380; Canadair CRJ aircraft; DeHavilland Canada DHC-8; Embraer 120; and ATR-42/-72 aircraft.

- Embraer 145, 170, 175, 190, and 195 aircraft continue to see significant impacts from 5G AD/NOTAMs in operations.

- AMOCs are revised each month by FAA and distributed by aircraft manufacturers to their airline customers.

It is understood that the FAA undertook refinements during 2022 to the underlying calculations to determine compliance for an AMOC, to improve the accuracy and permit more base station deployments. It is also understood that the AMOC process was proposed to conclude in October 2022 and then only be applicable to a certain class of radio altimeter. The proposed revised Airworthiness Directive (AD) discussed at paragraph 8.25 recognised that ‘… case-by-case AMOC approvals that allow performing certain operations otherwise prohibited by an AD are no longer the most efficient way for airplane operators to show that their radio altimeters perform their intended function in the 5G C-Band environment.’, but still appears to retain it as a possible compliance mechanism (see paragraph L of the proposed revised AD).

The Aerospace Vehicle Systems Institute (AVSI) also released on 6 December 2021 to the FCC several documents related to ‘Derivation of Radar Altimeter Interference Tolerance Masks’ that were summarised in the RTCA report. They can be found through the filings for the US C-band project. These have been discussed earlier in this paper.

The FAA on June 17, 2022, made a statement that they had ‘identified a path that will continue to enable aviation and 5G C-band wireless to safely co-exist’. This included:

    - Requiring ‘operators of regional aircraft with radio altimeters most susceptible to interference to retrofit them with radio frequency filters by the end of 2022.’

    - ‘Based on progress achieved during a series of stakeholder roundtable meetings, the wireless companies offered … to continue with some level of voluntary mitigations for another year.’

    - ‘Airlines and other operators of aircraft equipped with the affected radio altimeters must install filters or other enhancements as soon as possible. Filters and replacement units for the mainline commercial fleet should be available on a schedule that would permit the work to be largely completed by July 2023. After that time, the wireless companies expect to operate their networks in urban areas with minimal restrictions.’

Subsequent to the FAA statement, the International Air Transport Association (IATA) expressed strong disappointment with the decision, labelling it ‘almost impossible to achieve.’

While we understand there has been good progress on the radio altimeter retrofit program in the US, aviation stakeholders have expressed concern at the end of 2022 that it is not possible to meet the timeframe, via a letter from the 5G coalition to various government agencies that form the National Economic Council. It was also reported in the media that the FAA has petitioned the FCC and NITA to require smaller WBB operators to also employ mitigations for radio altimeters.

There is also an FCC filing, dated 5 October 2022, from counsel for ASRI that reports on a meeting of aviation industry representatives with the Office of Engineering and Technology (OET), Office of General Counsel (OGC), and the Wireless Telecommunications Bureau (WTB) suggesting that some longer-term mitigations may still be required after the interim retrofit.

The FAA filed a notice of proposed rulemaking for an airworthiness directive (AD) on 11 January 2023, which effectively proposed a delay in the radio altimeter retrofit deadline to 1 February 2024. Comments were required by 10 February 2023. This AD also outlines the required radio altimeter susceptibility to 5G for a ‘radio altimeter tolerant airplane’, which is a ‘Tolerance to an aggregate base station conducted spurious emission level of -48 dBm/MHz in the 4200–4400 MHz radio altimeter band.’ in addition to 5G emissions below a power spectral density mask as per the figure below:

- The intent of the proposed revised AD is that the FAA are:

- … proposing interference tolerance requirements for radio altimeters that can be used across the affected fleet. Airplanes meeting these proposed minimum performance levels would be allowed to use the prohibited operations at the airports identified by an FAA Domestic Notice after July 1, 2023. Airplanes operating under 14 CFR part 121 would also be required to have a radio altimeter that meets the proposed minimum performance standards (i.e., tolerance requirements) on or before February 1, 2024.

- The FAA determined the proposed interference tolerance requirements by using the fuller understanding of specific radio altimeter capabilities the FAA gained during the AMOC process. This process revealed the radio altimeter modifications that would not require a substantial system redesign, allowing aircraft operators to readily replace radio altimeters or install filters that allowed the aircraft to operate safely in a mitigated 5G environment.

The proposed revised AD also included cost estimates to conduct a retrofit exercise in the US. These figures may be useful in estimating similar retrofit costs in Australia if the fleet makeup of radio altimeters is known.

It should be noted that, as per the proposed revised AD, aircraft with radio altimeters meeting the performance requirements of the AD, may require 5G operators to use mitigations around an airport that have been agreed with the FAA. It indicates this is an interim action and they anticipate operation with no 5G related limitations only after the FAA incorporates the new radio altimeter MOPS into their radio altimeter TSO.

We note that the proposed interim spurious limit of -48dBm/MHz may be related to the protection of RTCA usage category 2 aircraft (which the proposed revised AD would seem to cover) as the value is the same as discussed in a filing by the AIA to the FCC on 7 December 2020 in response to the original RTCA report. Also note that ISED in Canada considered this as a regulatory limit in their revised work but considered it unnecessary for the range of radio altimeters that they evaluated (see later paragraph 8.82)

We note that the proposed revised AD also removes the:

- … prohibition of Required Navigation Performance with Authorization Required (RNP AR) Instrument Approach Procedures (IAP). After further analysis, the FAA has determined that 5G C-Band interference does not create an unsafe condition for an airplane conducting RNP AR IAPs because RNP AR operations do not rely on direct radio altimeter inputs to determine arrival at altitude minimums or the flight path of the airplane.

## Japan

From the various references mentioned in the Coexistence evaluation section for Japan studies, it was concluded that the Ministry of Internal Affairs and Communications (MIC) implemented the following mitigations:

    - A 100 MHz guard band from the 4200 MHz lower edge of the radio altimeter band (that is, 4100–4200 MHz).

    - For stations operating in the 100 MHz adjacent to the guard band (4000–4100 MHz), there is a 200-metre separation of high-power 5G base stations from an aircraft approach route, which is an area of about 1 kilometre separation from an airport runway.

    - For helicopter aerodromes, 5G base stations should have a separation distance of 50 metres for macro-cell and 20 metres for small-cell stations.

    - More stringent base station spurious emissions were proposed than typical in 3GPP specifications. For stations between 4000 and 4100 MHz, base stations are required to reduce unwanted emissions into the radio altimeter band to -39 dBm/MHz, or -46 dBm/MHz for base stations with an EIRP of 25 dBm/MHz or less.

Further information from stakeholders in Japan indicates that:

    - There is no specific EIRP to protect radio altimeters but there is an overall EIRP limit of 60.3 dBm/MHz. This is a theoretical limit and actual deployments are likely to be smaller.

    - There is no down-tilt requirement.

## United Kingdom

Ofcom in the UK awarded spectrum in the 3.6–3.8 GHz range in 2020. Coexistence issues were discussed in a published annex at A8. The analysis did not include the coexistence with radio altimeters. Consequently, there were no specific mitigations. Operating parameter limits for deployment in 3.6–3.8 GHz were also in that same annex at A10, with the following relevant maximum mean transmit powers:

non-AAS base station transmit

repeater downlink transmit:

65 dBm / 5 MHz EIRP per cell

AAS base station transmit:

44 dBm / 5 MHz TRP per cell

mobile or nomadic repeater

uplink transmit:

28 dBm TRP*

fixed or installed terminal stations or

repeaters uplink transmit:

35 dBm / 5 MHz EIRP*

- * The maximum mean power relates to the EIRP or TRP of a specific piece of radio equipment, irrespective of the number of transmit antennas.

While not covered in the Annex, awarded licences have the following emission mask requirements:

Note that frame structure A is the ‘preferred frame structure’ and frame structure B is the ‘compatible frame structure’.

Ofcom considered coexistence with radio altimeters as part of their 3.8 GHz to 4.2 GHz band: Opportunities for Innovation series of consultations and decisions. They concluded that no mitigations were required for their proposed plans for medium- and low-power WBB deployment.

A shared access licence guidance document indicates the scope of the medium- and lower-power deployments permitted under 3.12 for low power and 4 for medium power:

Medium power is permitted in rural areas only with a maximum EIRP of 42dBm/carrier for carriers<=20 MHz and 36dBm/5 MHz for carriers > 20 MHz.

Medium power base station relevant out-of-band emission limits are in Table 14:

Low power is permitted outdoor with antennas limited to 10 metres height above ground and a maximum base station power of a maximum EIRP of 24dBm/carrier for carriers <=20 MHz and 18dBm/5 MHz for carriers > 20 MHz.

Low-power base station relevant out-of-band emission limits are in Table 7:

The UK Civil Aviation Authority (CAA) issued a Safety Notice in January 2022 about ‘Potential Interference Risk to Radio Altimeters from 5G mobile Technology’. It notes that:

- Conversations with other NAAs has established that there have been no confirmed instances where 5G interference has resulted in aircraft system malfunction or unexpected behaviour. It should be noted that in some states 5G infrastructure is not yet in place and that past performance is not a guarantee for future applications.

- And:

- Flight crew experiencing radio altimeter or autoflight malfunctions should not assume that this has been caused by 5G interference and should follow normal operating procedures for any malfunctions or failures. Although flight crew should be aware of the possibility of 5G interference, any malfunctions observed may well be caused by other factors such as radio altimeter and associated antenna technical failures.

## France

From information contained in the ICAO FSMP meeting WG11 input document, mitigations in the 3400–3800 MHz range in France included:

Operators must implement only downward tilt (largely removed in Mar 2021).

Operators have to take measures to avoid grating lobes as far as practicable.

Protection zones (the safety and precaution zones below) around IFR aerodromes.

Safety zone:

applied to all IFR aerodromes and some helicopter platforms

extends 910 metres from the width edge of the runway and 2,100 metres from the ends where 5G base stations are not allowed to transmit.

to protect when the aircraft is at or below 61 metres.

Precaution zone:

either side of the safety zones

to protect when the aircraft is below 305 metres.

5G base stations are coordinated (there isn’t further information on what this means)

800 metres wide and 6,100 metres from the ends of the exclusion zone

the precaution zone does not apply in the case where calculations based on the antenna gain envelope provided by the operator and worst-case location of BS outside the safety zone show that the radio altimeters remains protected under the above assumptions.

For helicopter platforms, a circular safety zone with a radius of 1 kilometre was also applied for some heliports to accommodate states aircraft that are under the responsibility of the Ministère de l’intérieur (Department Homeland Security).

For unwanted emissions:

- It was concluded that the US problematic for the unwanted emissions of the frequency band 3.7-3.98 GHz into the Radio altimeter frequency band 4.2-4.4 GHz is totally different and not applicable to the European context. DGAC therefore did not take into account the unwanted emission assessment of the RTCA report.

Zone sizes are only a representative example, based on their technical study for a BS EIRP of 78 dBm/100 MHz. The intent is that the zone sizes vary with BS deployment EIRP.

In February 2022, the ANFR announced certain changes to the mitigations (Google translation):

- In the first quarter of 2021, the stress zones (i.e. the exclusion and restricted zones) were lightened to limit them to the 17 French airports where low visibility landings are allowed, as well as to 10 helipads. The scoring (i.e. the downtilt) constraint has been restricted only to areas close to these airports and certain helipads.

## Denmark

The regulator in Denmark allocated spectrum in their 3.5 GHz band (3410–3800 MHz) in 2021.

An information memorandum gives an overview of the auction and related auction processes.

Annex F to the memorandum gives specific licence conditions for their 3.5 GHz band. There appear to be no specific mitigations for radio altimeters. EIRP is limited to 61 dBm/MHz for non-AAS base stations and TRP is limited to 41 dBm/MHz for AAS base stations (approximately 64–66 dBm/MHz EIRP, typically).

Danish digital infrastructure provider, TDC Net, claim an overall 5G coverage of 99.3% (area or population basis is not stated).

## Switzerland

Switzerland’s Federal Office of Civil Aviation (FOCA) have a useful page on the radio altimeter issue. It appears that the Swiss OFCOM regulator performs some analysis to consider the issue but conclude there is not an issue currently.

Auctions for 5G occurred in early 2019 for the 700 MHz and 3.5–3.8 GHz bands. Technical conditions, such as power limits, are not specifically referenced in the tender document, but refer to CEPT documents and recommendations. However, no specific mitigations appear to be required for radio altimeters, excepting the note at 7.36 above.

## Canada (initial proposed approach)

The Canadian regulator, the Department of Innovation, Science and Economic Development (ISED), made decisions, updated after consultation, in June 2021 about introducing WBB into the 3700–3900 MHz and in 3450–3650 MHz frequency ranges. They have been monitoring the radio altimeter issue in the latter range and extended a consultation on revised proposed technical operations. After review, they proposed an approach, similar to France, of exclusion zones and protection zones, the latter with a specified PFD limit to apply. This was initially in the range 3450–3650 MHz. They then  consulted on their 3.8 GHz band (3650–3900 MHz), and proposed to extend the mitigations up to 3900 MHz. Indoor stations were exempt from the mitigation requirements. To further manage issues with military and emergency service helicopters, with insufficient information of their altimeter fleets at this time, ISED proposed a national antenna down-tilt/vertical scan requirement. They are currently scheduled to auction the 3800 MHz band in Q4 2023.

The revised SRSP-520 — Technical Requirements for Fixed and/or Mobile Systems, Including Flexible Use Broadband Systems, in the Band 3450-3650 MHz was published in November 2021.

In August 2022, ISED commenced a consultation on a non-competitive local licensing (NCL) framework for the 3900–3980 MHz band. In the consultation, it proposes in the interim, to extend the mitigations proposed for 3450–3650 MHz and 3700–3900 MHz segments to the 3900–3980 MHz NCL segment.

- General restrictions for radio altimeters

Outdoor non-AAS and AAS fixed point-to-point and point-to-multipoint stations operating with a positive angle with reference to the horizon (that is, above) shall not exceed a maximum EIRP of 55 dBm/5 MHz. For stations with a channel bandwidth less than 5 MHz, the maximum EIRP shall not exceed 48 dBm/MHz. The EIRP of AAS fixed point-to-point and point-to-multipoint stations shall be calculated as follows:

Outdoor non-AAS and AAS base stations are required to operate their antenna systems at a negative elevation angle with reference to the horizon. Operators of AAS base stations are prohibited from vertical scanning (that is, directing energy from antenna elements to form beams) at positive elevation angles with reference to the horizon.

No licensee may operate a station within an exclusion zone.

The provisions do not apply to indoor stations.

- Exclusion zone definition

The exclusion zones are rectangular areas around airport runways where automated landing is authorized in Canada. The exclusion zones extend 910 metres on either side of the runway edge and 2,100 metres from the runway thresholds.

- Protected zone definition

The protection zones are rectangular areas that extend from the edge of the exclusion zones. For each runway, there are 2 protection zones, each extending from either end of an exclusion zone. A protection zone is 1,000 metres wide and extends 6,100 metres from each end of an exclusion zone.

- Technical and operational requirements in protection zones

All outdoor fixed (wireless access) and base stations operating within the protection zones must not exceed a PFD limit of -38.80 dBW/m2 in 1 MHz at a height of 91.44 metres (300 feet) above ground. The PFD limit shall be satisfied 100% of the time and be evaluated for all combinations of elevation and azimuth angles above the horizon relative to the location of the fixed or base station.

In cases of overlap between the exclusion zone and the protection zone, the exclusion zone takes precedence.

- Other technical limits for non-AAS systems unrelated to radio altimeters (not complete, see the SRSP)

For fixed point-to-point (P-P) stations and flexible use base stations transmitting in accordance with section 6 of this SRSP within the frequency range 3450–3650 MHz, with a channel bandwidth equal to or greater than 5 MHz, the maximum permissible EIRP is 68 dBm/5 MHz (that is, no more than 68 dBm EIRP in any 5 MHz band segment) for stations with an antenna height above average terrain (HAAT) of up to 305 metres. For stations with a channel bandwidth less than 5 MHz, the maximum permissible EIRP is 61 dBm/MHz.

For all installations with an antenna HAAT of more than 305 metres, a corresponding reduction in EIRP, according to the following formula, shall be applied:

- e.i.r.p.reduction=20log10(HAAT/305)dB

- Other technical limits for AAS systems unrelated to radio altimeters (not complete, see the SRSP)

For flexible use base stations transmitting in accordance with section 6 of this SRSP within the frequency range 3450–3650 MHz, with a channel bandwidth equal to or greater than 5 MHz, the maximum permissible TRP is 47 dBm/5 MHz (that is, no more than 47 dBm TRP in any 5 MHz band segment). For stations with a channel bandwidth less than 5 MHz, the maximum permissible TRP is 40 dBm/MHz.

Antenna height limits shall apply based on equivalent EIRP of the AAS antenna system. The equivalent EIRP in dBm/5 MHz shall be calculated as:

- (e.i.r.p.)eq=TRP+Ge+10log10(min(NTx,8))

where Ge is the gain of one antenna element in dBi, NTX is the number of transmit antenna elements and TRP is measured in dBm/5 MHz. An equivalent EIRP of 68 dBm/5 MHz applies to an antenna installation with a HAAT of 305 metres or lower.

For any installation with a HAAT above 305 metres, TRP shall be reduced according to the following formula:

- TRPreduction=20log10(HAAT/305)dB

- Unwanted emissions limits are documented in Tables 2 and 3 of RS-192:

## Canada (revised proposed approach)

In December 2022, ISED released a revised 3rd issue of the consultation and other relevant documents after completion of their comprehensive laboratory, over-the-air, computational model and other work on the issue. This resulted in numerous proposed changes described in the following paragraphs.

In 2021, when ISED developed the current exclusion and protection zones around airport runways, the interference tolerance mask (ITM) values were based solely on the Radio Technical Commission for Aeronautics (RTCA) report. The new body of work resulted in new ITM values, used to define the exclusion and protection zones around specific airports where automated landing is authorized, but the values were not significantly changed. The mitigations were determined by an assessment of scenarios using a computational model which used the laboratory study outputs, which were demonstrated to have a high degree of correlation with their OTA results. It also used mathematical 5G antenna system models rather than any OTA-based measurements (such as the US ITS measurements discussed from paragraph 3.76.)

Based on the results of the analysis, the proposed exclusion zones have been revised to be formed of 2 segments: a rectangle having the same length as the runway and a width of 640 metres centred on the centreline of the runway; and an isosceles trapezoid, with a starting base width of 640 metres centred around the runway centreline and extending 2,100 metres from the runway threshold, with an ending base width of 1,269 meters.

Similarly, the proposed protection zones are represented by isosceles trapezoids with a length of 1,000 metres from each edge of the exclusion zones, with an ending base width of 1,568 metres for the 3500 MHz band and a length of 2,500 metres from each edge of the exclusion zones, with an ending base width of 2,017 metres for the 3800 MHz band. The figure below shows an example of the proposed zone changes for Ottawa airport:

They note that the revised exclusion and protection zones described above are based on ILS surfaces, while previously, it was assumed that an aircraft flew closer to the runway centreline. Moreover, based on the information received from Transport Canada, while important to protect aircraft at 1,000 feet or less, the critical flight altitudes are at 350 feet or less, when an aircraft is in its final descent to a runway. Consequently, the proposed exclusion zone boundary was aligned with the location where the aircraft would be at 350 feet above the ground. Further, using ISED’s baseline simulators, the sizes of the exclusion and protection zones were derived based on ISED’s laboratory results of patch antennas, which are used for aircraft with category 1 radio altimeters. The exclusion and protection zone sizes were based on worst-case base station emissions, with an assumed total EIRP of 77.5 dBm in one single beam serving one user aligned in azimuth with an aircraft. This total EIRP is equivalent to a power spectral density of 61 dBm/MHz EIRP with a channel bandwidth of 45 MHz. The results of the baseline simulator provided an overestimation of 6.2 dB (median value) when compared with the results obtained in the OTA study. As such, no additional safety margin was added when determining the sizes of the exclusion and protection zones.

Based on the results of the computational analysis, new PFD values towards the sky, including equivalent EIRP values of a base station, were derived for the protection zones (see table A.11).

Table A.11: Power flux density values and equivalent EIRP skywards

In order to protect an aircraft located between 1 000 feet and 350 feet above ground, the limiting EIRP of a base station is the one to protect an aircraft at 1,000 feet, 40.19 dBm/MHz towards the sky. Using this most limiting EIRP, a new PFD value at 350 feet was derived, which is equalled to -21.31 dBW/m2/100 MHz or -34.21 dBW/m2/5 MHz.

Additionally, PFD values, including equivalent EIRP values of a base station, were derived to protect an aircraft at 325 feet or fewer, by limiting the power of a base station towards runways (see table A.12).

In order to protect an aircraft located at 325 feet above ground and lower, the limiting EIRP of a base station is the one to protect an aircraft at 325 feet, 57.25 dBm/MHz towards the runway. A PFD value of -10.45 dBW/m2/100 MHz or -23.45 dBW/m2/5 MHz at 325 feet above ground at the boundary of the exclusion zone would protect an aircraft in the critical phase of its final descent.

- Results for antenna down-tilt requirement to protect aeronautical search and rescue operations

Different combinations of mechanical and digital tilt angles of a base station antenna and vertical scanning were assessed to determine the impact on helicopters hovering between 50 feet to 1,000 feet above ground.

The results of the simulations demonstrated that the emissions from base stations in an up-tilt configuration are far less attenuated in rural areas compared to urban centres. In urban centres, where base stations would be up-tilted to serve multi-dwellings, clutter and building shadowing will reduce the emissions towards aircraft significantly. In rural areas, where most search and rescue operations take place, base station emissions will travel at larger distances since the impact of clutter or building shadowing is minimal. Consequently, the antenna down-tilt requirement does provide additional protection to aircraft flying in rural locations. Finally, the results of the simulations demonstrated that limiting the vertical scanning below the horizon had little impact when the combined digital and mechanical tilts were already kept below the horizon.

- Results for exclusion and protection zones around elevated H1 heliports

For a category A helicopter landing and departure at an elevated H1 heliport, the critical phase was defined as the area within 50 metres of the FATO centre. An ITM value of -45 dBm was used for helicopters. Based on ISED’s data on radio altimeters used on helicopter, this ITM value would sufficiently protect the vast majority of radio altimeter models at altitudes between 50 feet and 1,000 feet above ground.

Simulation results for the St. Michael’s Hospital heliport showed that an exclusion zone of an 80-metre radius around the FATO’s centre would be required to protect an aircraft from outdoor base station emissions. Moreover, the results showed that sufficient protection can be achieved by limiting the power of outdoor base stations within 500–1,000 metres of the FATO’s centre (that is, protection zone) towards these aerodromes through a PFD value of -41 dBW/m2/5 MHz at a heliport surface, within the 50-metre circular boundary measured from the FATO’s centre. Due to the potential impact of the surrounding structures, further study is necessary to determine the precise size of the proposed protection zones around elevated H1 heliports.

- Results for spurious emission levels of base stations

Finally, 3 spurious emission levels, -13, -30 and -48 dBm/MHz, were assessed for an aircraft above a base station with specific clearances of 50 feet (15.2 metres) and 35 feet (10.7 metres). The latter distance represents the typical minimum distance for a category A helicopter flying above an obstacle per rules mandated by TC. A clearance distance of 50 feet was also selected to represent the clearance of an aircraft flying above a base station located on the obstacle clearance surface around airport runways.

An ITM threshold of -91 dBm/MHz was considered as part of this analysis. This value represents the minimum breakpoint for aircraft altitudes of 50 feet and 200 feet for all models for which ISED had data, without any additional margin for unit-to-unit variation. ISED used a single-element pattern for a sub-array composed of 3 vertical sub-array elements to model spurious emissions of a 5G base station. Further, for this analysis, ISED considered the emissions of a single base station sector with a mechanical tilt of 0-degrees from the horizon and an azimuth aligned with the aircraft. Based on these parameters, table A.13 provides the maximum spurious emissions at the receiver of a radio altimeter generated by ISED’s simulators.

- ISED’s conclusion

Based on the computational analysis, isosceles trapezoidal-shaped exclusion zones of 320 metres from either side of the runway edge and extending 2,100 metres from the runway thresholds, with an ending base width of 1,269 metres, would protect category 1 aircraft landing at ILS runways. Further, isosceles trapezoidal-shaped protection zones with a length of 1,000 metres from the edge of the exclusion zones, an ending base width of 1,568 metres for 3500 MHz and a length of 2,500 metres from the edge of the exclusion zones, and an ending base width of 2,017 metres for 3800 MHz would provide protection to category 1 aircraft landing at ILS runways.

In urban areas, where base stations are more likely to be up-tilted to serve multi-dwelling units, it was determined that the impact of building shadowing and clutter significantly reduces the emissions towards aircraft. In rural areas, where most search and rescue operations take place, the antenna down-tilt restriction does provide additional protection since the impact of clutter or building shadowing is minimal. Additionally, limiting the vertical scanning below the horizon had little impact when the combined digital and mechanical tilts were already kept below the horizon.

Exclusion zones of an 80-metre radius and protection zones with a radius between 500 and 1000 metres around H1 classified heliports would protect helicopters landing or departing from these aerodromes from outdoor base stations. Within the protection zones, base station emissions towards these aerodromes would be required to meet a PFD limit of -41 dBW/m2/5 MHz.

Finally, a spurious emission level of -33 dBm/MHz would protect radio altimeters with an ITM threshold of -91 dBm/MHz (50 foot to 200-foot aircraft altitude).

All of the above proposed requirements for 5G operations in the 3450–3900 MHz band to protect radio altimeters will be put in place until 31 March 2025. After this date, mitigation measures imposed on 5G operations are intended to be removed. The 2025 sunset date will provide approximately 2 additional years from the US FAA’s July 2023 date to replace or upgrade susceptible radio altimeters. Moreover, it will provide time for aircraft to be retrofitted based on the RTCA’s MOPS (DO-155A standard), which is planned to be released by end of 2023.

## Belgium

The Belgian regulator, the Institute for Postal Services and Telecommunications (BIPT), consulted on  April 2021 on a draft decision on revised technical conditions for the 3400–3800 MHz band. It is silent on coexistence with radio altimeters. It has been reported, however, that they are applying the same mitigation approaches as in France.

Note that in Belgium, a station authorisation is required in order to operate a radio altimeter.

## Czechia

In a November 2021 Safety Brief, Prague airport said that they would respect the French safety and precaution zones, keeping them free of 5G transmitters.

## Saudi Arabia

The Saudi regulator, CITC, consulted in April 2022 on the allocation of WBB in 3500–3800 MHz and 3800–4000 MHz ranges. While most proposed technical limits are unknown at this point:

- There is an ongoing global discussion about the potential for IMT in 3800 – 4000 MHz to interfere with radio altimeters. CITC proposes to monitor the deployment of 3800 – 4000 MHz spectrum near airports. License holders are required to get approval from CITC of any planned deployments in the vicinity (Proposed: 200m) of runways as well as the approach and landing routes (Proposed:1km either side of a runway). Usage of the spectrum in these areas will be subject to specific protection parameters set by CITC, and that proposed vicinity distances are subject to change in the final IM based on ongoing co-existence studies. These approval distances are subject to change in future consultations.

## Brazil

The Belgian regulator, Anatel, made a new Act in 2021 for the Technical and Operational Requirements of Use of the Range from 3,300 MHz to 3,700 MHz. This established the basic relevant technical limits of:

for base stations, an EIRP limit of 65 dBm/10 MHz per plane of polarisation

relevant spurious emission limits for non-AAS base stations of at most -30 dBm/MHz per port in the frequency range from 1 GHz to 18.5 GHz, except for frequency bands from 3.74 GHz to 4.2 GHz and 4.5 GHz to 4.8 GHz, which should be at a maximum of -52 dBm/MHz per port

relevant spurious emission limits for AAS base stations of a maximum of -21 dBm/MHz TRP (English, Total Radiated Power) in the frequency range from 1 GHz to 18.5 GHz, except in frequency bands from 3.74 GHz to 4.2 GHz and 4.5 GHz to 4.8 GHz, which should be at most -43 dBm/MHz TRP.

In May 2022, Anatel started consultation on the installation of antennas near airports in the 3300–3700 MHz range. They proposed to:

Establish Attention Zones on a provisional and precautionary basis, where the main beams of the antennas used in base, nodal or repeater stations operating in the sub-band 3300 MHz and 3700 MHz, installed in Attention Zones, have their pointing limited between the horizon line and below.

The Attention Zone is defined by the area near certain aerodromes corresponding to the rectangle comprising the following distances:

2,100 meters from the ends of the runway

910 meters on each side of the central axis of the track.

The limit of the note provided in the caput applies to both AAS and non-AAS antennas and refers to the direction of the pointing of the main beam of the antenna, whether static or dynamic.

The location of a base station, nodal or repeater shall be referenced from the geographical coordinate of the base of the antenna support infrastructure.

## CEPT/ECC

As discussed in the various consultation papers, the EU previously made a decision in 2019 to require member states to make the 3400–3800 MHz available for terrestrial 5G services. This decision includes required technical compliance, including a ‘Block Edge Mask’ that helps describe unwanted emission limits but no specific limits on in-band power.

No relevant current CEPT document appears to apply in-band power limits or any specific mitigations for radio altimeters.

The current implementation status for each EC country can be found via CEPT, but appears incomplete (31 countries have provided no information).

The minutes of the European Communication Committee (ECC) Project Team One (PT1) on IMT matters meeting #69 in January 2021 agreed to a new work item ‘for an ECC Report containing a compatibility studies between MFCN 5G BS operating in the frequency band 3400-3800 MHz and radio altimeters operating in the frequency band 4200–4400 MHz.’ The meeting agreed to propose a target date of July 2022. The ECC PT1 Chair was requested to highlight to the ECC that urgent input from aeronautical stakeholders on radio altimeter receiver characteristics is needed in order to progress this work.

ECC PT1 meetings #68 and #69 were held in April and September 2021. Documents relevant to radio altimeters include ECC PT1(21)117, ECC PT1(21)115, ECC PT1(21)184, ECC PT1(21)188, ECC PT1(21)192, ECC PT1(210234 ANNEX VIII-14. The latter is a skeleton working document for an EC report on the Compatibility between WBB operating in 3400–3800 MHz and Radio Altimeters (RA) operating in 4200–4400 MHz. The draft meeting minutes noted the lack of progress in obtaining accurate radio altimeter parameters and what to do in their absence.

The ECC PT1 also established a correspondence group in at meeting #70 in January 2022 ‘CG RADIO ALTIMETERS 4.2-4.4 GHz’. This includes correspondence on the development of ‘long term MFCN parameters for MOPS’, that is, a draft compilation of WBB parameters and deployments that the RTCA/EUROCAE could take into account when redeveloping the radio altimeter performance standards. The January meeting draft minutes noted the following:

… continued developing the draft ECC Report on compatibility between MFCN operating in 3400-3800 MHz and Radio Altimeters (RA) operating in 4200–4400 MHz (SWG C TEMP 11 / ANNEX VIII-27). Material from relevant input documents was included in appropriate places into the working document but there was not sufficient time to discuss it, so it was inserted with square brackets and further considerations are required. Questions by the mobile industry were raised on whether received parameters for radio altimeter were the most appropriate and a list of clarifying questions was drafted. ASRI kindly agreed to work on responses to these questions. Recognising that there is a range of radio altimeter performance, participants stressed the need to agree first on realistic assumptions and scenarios before starting any compatibility studies. It was also highlighted that assumptions should represent the CEPT situation.

During the discussions it was pointed out that there are still missing parameters required for the simulations. AVSI informed the meeting that additional parameters are intended to be published in March this year. It was highlighted that a reply to Eurocae/RTCA on received radio altimeter parameters and questions regarding 5G performance would be helpful.

It was agreed to establish a Correspondence Group until the next ECC PT1 meeting (with the Terms of Reference in SWG C TEMP 12r1 / ANNEX VIII-28). The CG convener should provide an update to ECC PT1 chairman on the progress of CG activity one-week prior upcoming ECC meeting in order for PT1 chairman to inform ECC accordingly.

The draft ECC Report had been scheduled to be finalised for submission to ECC for approval for public consultation from this ECC PT1 meeting but this was the first time the group received information regarding radio altimeter characteristics, therefore more time is needed to progress the work and an extension to the deadline by 3 additional ECC PT1 meetings to work item PT1_40 will be requested from ECC (draft report for public consultation to be approved at ECC PT1 #73 in January 2023 for submission to ECC #61 in March 2023; resolution of public consultation comments at ECC PT1 #74 in April/May 2023 and final approval at ECC #62 in June/July 2023).

In the draft minutes of the ECC PT1 meeting #71 in May 2022, the ToR CG on radio altimeters indicates that the long-term MFCN parameters for MOPS needs to be finalised before 14 June 2022 for consideration of approval by the ECC. Various other input and output documents are relevant to radio altimeters from this meeting.

A ‘crossover’ meeting on radio altimeters was held on 17 June 2022 to finalise the MFCN parameters work. Draft text was developed for liaison statements to the RTCA, EUROCAE and EASA.

The draft liaison statements relating to the MFCN parameters for MOPS were sent to the RTCA and EUROCAE and EASA after approval at the 59th ECC Plenary. The minutes stated that:

- Radio altimeters 4200–4400 MHz coexistence with MFCN in 3400 - 3800 MHz

- ECC PT1 has continued developing the draft ECC Report on compatibility between MFCN operating in 3400-3800 MHz and Radio Altimeters operating in 4200–4400 MHz.

- The ECC PT1 Chair presented text for ECC to consider for inclusion in a liaison statement to the RTCA and EUROCAE Joint Working Group which is developing minimum operational performance specifications (MOPS) for radio altimeters (ECC(22)036 Annex 13). ECC PT1 agreed on the need for a liaison statement to provide information on MFCN technical characteristics so that the MOPS are more compatible with current and future use of spectrum in adjacent frequency bands; however, there had been some diverging views on some of the elements that had been proposed for inclusion in the liaison statement, as described in Section 2 of the document.

- ECC also considered document ECC(22)037 (GSA).

- ECC discussed the possible text and concluded that

- •	The liaison statement would include factual information on the base station power levels for current deployments in some CEPT countries and to mention that higher power levels were possible in the future but ECC decided not to suggest a possible upper limit on base station power. Denmark stated that it was important not to prejudge what base station development could be 30 to 40 years ahead. France was of the view that studies on the use of higher transmit powers could imply restrictions in specific areas, e.g. preventing particular pointing angle, restrictions in scanning angle or requirement to avoid beam steering above the horizon, but there was opposition from some administrations to including this view.

- •	It would include information on countries outside CEPT where this was known but this should be separate from the CEPT information and attached to a suitable disclaimer.

- •	It would request a view on critical coexistence scenarios

- A similar liaison statement would also be sent to EASA.

An EU mandate for CEPT invited:

- CEPT to assess the technical feasibility of the shared use of the 3.8–4.2 GHz frequency band by terrestrial wireless broadband systems providing local-area network connectivity with focus on vertical users and other terrestrial wireless use-cases and, on that basis, deliver harmonised technical conditions for the shared use of the band.

PT1 meeting #72 occurred in late September 2022 and continued discussions on the issue. The liaison statement discussed at paragraph 7.76  was agreed and sent.

PT1 meeting #73 occurred in January 2023. On the radio altimeter work item (PT1_40), Nokia provided an input document that proposed basic geometry considerations for defining sharing scenarios between radio altimeters and WBB in 3.4–3.8 GHz related to glideslope, obstacle free zone and base station location requirements. An input from France proposed an aircraft landing scenario for study and also summarised a calculation from the French mitigation method to derive the width of the protection/exclusion zone. Finally, the coordination group progress report on radio altimeters covers:

    - general progress report including the need to ask the PT1 plenary to extend the work deadline

    - current draft of the WBB/radio altimeter compatibility report, which is still in development but is much progressed. Some WBB characteristics are included as well as radio altimeter characteristics based on M.2059 and the RTCA report but evaluation scenarios are yet to be developed

    - a Q&A working doc on radio altimeter parameters that included some clarifications on the RTCA testing.

Meeting #73 also contained informative documents. A paper from Airbus that appears to be the basis of the scenarios examined in the draft RTCA DO-399 radio altimeter performance requirements discussed from paragraph 8.128. ASRI also proposed a draft structure for the radio altimeter report.

## ICAO/FSMP

The ICAO is a United Nations specialised agency to support diplomacy and cooperation in air transport by signatory states to the Chicago Convention (1944). Its core function is to maintain an administrative and expert bureaucracy (the ICAO Secretariat) supporting these diplomatic interactions, and to research new air transport policy and standardization innovations as directed and endorsed by governments through the ICAO Assembly, or by the ICAO Council, which the assembly elects.

The ICAO Frequency Spectrum Management Panel (FSMP) was established in 2015 to manage aeronautical frequency spectrum in order to ensure sufficient access to the resource for the provision of aeronautical communication, navigation and surveillance services (CNS) in an efficient and safe manner.

The ICAO FSMP WG/10 meeting was held in August 2020. There were several working and information papers presented to the meeting related to the radio altimeter issue, including a history at the time of activity in the US, as well as a report on the ACMA release of the 3700–4200 MHz options paper.

The ICAO FSMP WG/11 meeting was held in March 2021. There were several working and information papers presented to the meeting related to radio altimeter performance and coexistence with WBB-like services under Agenda item 3.

WG/11 included a problem statement on the issue. The only example given of autopilot failure due to RF interference was due to the operation of the ‘iron dome’ RF jamming system near Tel Aviv airport, which is not likely to be representative of the potential interference from WBB systems.

The meeting final report reported the ACMA outcomes paper and stated ‘an action item was generated (AI 11-01) to provide comments to the Australian Panel member by 31 March, 2021 to support his participating in the TLG’.

The meeting agreed an action to provide any comments on the RTCA Report to the authors.

The meeting also agreed:

- Elements for a proposed State Letter on possible interference to radio altimeters were drafted.

- A correspondence group on radio altimeters (CG-RA) was formed to collect information on a number of topics related to 5G/altimeter compatibility (AI 11-04).

- Membership was asked to provide any technical studies on radio altimeter compatibility with other services operating in nearby frequency ranges to the CG-RA.

- Noting that a work item on the subject ‘Compatibility between MFCN operating in 3400-3800 MHz and Radio Altimeters (RA) operating in 4200–4400 MHz’ had been agreed by the European Conference of Posts and Telecommunications Administrations (CEPT) Electronic Communications Committee (ECC) project team 1 (PT1), aeronautical experts from member countries were encouraged to participate in the effort to ensure aviation equities were represented. The effort is to start March 2021 and terminate in July 2022.

- The meeting noted that the use of ‘/octave’ can be interpreted in more than one way. The meeting agreed that in the context of Recommendation ITU-R M.2059, the frequency to be used in the filter rejection in terms of ‘/octave’ was the absolute frequency and not the emission or allocated bandwidth. It was noted by several members that this resulted in a relatively poor rejection level, which should be addressed by the ongoing specification work for future radio altimeters.

- It was noted that in other ITU-R Recommendations and in some countries, the frequency reference in designation of ‘/octave’ is the emission bandwidth. These are for transmitter characteristics, however.

It was noted that technical means to reduce transmitted emission levels at a given offset frequency may be less complex than the technical means necessary to reject interference at that same offset frequency. In particular, it may be difficult to achieve relevant rejection in the receiver at the frequencies in adjacent bands in which high-power emissions are transmitted while still maintaining receiver performance (for example, signal distortion, group delay) and reasonable size, weight and cost.

ICAO issued a state letter in March 2021 to highlight the issue to ICAO states and to ‘encourage you and your Administration to consider as a priority, public and aviation safety when deciding how to enable cellular broadband/5G services in radio frequency bands near the bands used by radio altimeters.’ The ACMA also received this state letter via Airservices Australia in April 2021.

WG/12 was held in October 2021 where, again, radio altimeter issues were covered under Agenda item 3. The draft report indicated that the Radio Altimeter Standards and Recommended Practices (SARPS) job card was reviewed and agreed to review the text of the job card and for participants to see if generic guidance could be developed to assist aviation authorities to analyse potential 5G impacts given local implementations.

Relevant input documents to the meeting included a summary of the status on replanning the 3700–4200 MHz band in Australia (Airservices Australia) and a presentation on the Interference Susceptibility Evaluations of Pulsed Radio Altimeters Due to 5G Mobile Base Station Signal (Electronic Navigation Research Institute, Japan). The latter recorded the test results differences between these tests, with 2 sample units of the same model as tested by the Ministry of Internal Affairs and Communications (MIC).

WG/13 from February 2022 continued to update the CG-RA report, briefed the ICAO Secretariat on the issue and included an information paper on the radio altimeter interference path loss on a specific helicopter.

The meeting included a working paper on radio altimeter work in France. This is, in effect, a summary report to the FSMP on the French helicopter trial already discussed.

Another working paper suggested the need to develop the ICAO material to ‘assist States in developing and putting in place an appropriate set of mitigations to address the potential risks to flight safety caused by 5G C-band deployments’ and to form an ad-hoc group within the FSMP to further progress this activity.

The meeting draft report considered the documents and ‘resulted in an action item to national spectrum regulators that were contributors to the work of the FSMP to provide examples of standards/systems for adjacent-band signal rejection that could be provided to the next FSMP meeting.’

WG/14 (April 2022) continued to update the CG-RA report. The meeting draft report asked that the CG-RA to continue to compile its informative report on: 5G broadband mobile terrestrial interference to radar altimeters and noted that ‘ ECC PT-1 had begun work on an interference assessment of 5G systems in 3.4-3.8 GHz and radar altimeters in 4.2-4.4 GHz, WP13 proposed a liaison statement to ECC to inform them of relevant ICAO material that might be of assistance in their deliberations. After discussion and small modifications, the meeting agreed.’

WG/15 (August 2022) again had a number of documents related to radio altimeters.

    - As noted in the studies section, IP03 presented test results of radio altimeter interference susceptibility in Japanese 5G conditions in both 3.7–4.1 GHz and 4.5–4.6 GHz ranges. Its conclusions in terms of characteristics were noted in the conclusion to be similar to the RTCA ITM values.

    - IP07, Receiver Performance Analysis: Systems Operating in the 3.7 – 4.4 GHz band, was a comparison, based on available data, of the performance of 5G and radio altimeter receivers. It would have little value in the context of their coexistence.

    - IP08 was another updated to the CG-RA report. It reported the outputs of the ECC Plenary in relation to the MFCN operational parameters in detail. It notes action in France in relation to the desire of the Fédération Française des Télécom to have a similar radio altimeter retrofit timetable to that in the US in order to remove the ‘severe and unprecedented constraints imposed on the use of frequencies in the 3.4–3.8GHz frequency band’.

    - IP09, ICAO MID Draft Guidance on Safeguarding measures to protect Radio Altimeter from potential harmful interference from Cellular 5G Communications, is a document similar in scope to this one, that:

      - summarises the issues

      - outlines the 5G implementations and short- and long-term safeguarding measures adopted in various countries

      - proposes a number of recommended minimum regulatory actions and measures that need to be taken to protect radio altimeters, by both aviation and spectrum management authorities.

The ACMA notes that our proposed interim, reviewable mitigations largely align with the desired requirements.

We note that Appendix A - French Methodology to set the dimensions of Special Protection Zones around airports has an incorrect reference, that might give the interpretation that France based their mitigations on RTCA usage category 2, not 1 as they did. There is a reference to RTCA Table 7-2. This is a table to demonstrate that the test setup did not produce significant spurious levels that would cause errors. It is not the place in the report to obtain the concluded ITMs for the usage categories.

The draft report for the meeting discussed the above inputs in relation to Agenda Item 4.

ICAO released an invitation on 1 September for the Twelfth Meeting of the Regional Aviation Safety Group – Asia and Pacific Regions. Scheduled for 17–18 November 2022, its theme will be ‘Raising awareness on potential safety hazards due to 5G interference and promoting best practices in Asia Pacific’.

## IATA

The International Air Transport Association (IATA) is ‘the prime vehicle for inter-airline cooperation in promoting safe, reliable, secure and economical air services’. They have an aviation and 5G webpage that includes their position on the issue and a global 5G status dashboard.

## Eurocontrol

Eurocontrol, or the European Organisation for the Safety of Air Navigation, is an international organisation working to achieve safe and seamless air traffic management across Europe. They released a think paper in July 2022, which examined the issue in the European context. It highlighted that more resources need to be devoted to tackling spectrum inefficiency (in aviation systems) and identified 3 strategies to improve spectrum use:

    - improving adjacent band filtering as much as possible

    - improving aviation equipment standards maintenance for legacy systems

    - getting the balance right between coordinated deployments of new CNS radio systems.

They noted that ‘incentives may be required to coordinate deployment of CNS radio systems … as the cost of development and safety certification continues to provide a high barrier to innovation’.

## RTCA/AVSI

The Radio Technical Commission for Aeronautics is a private, not-for-profit association founded in 1935. RTCA products serve as the basis for government certification of equipment used by the tens of thousands of aircraft flying daily through the world’s airspace. A Standards Development Organisation (SDO), RTCA works with the Federal Aviation Administration (FAA) to develop comprehensive, industry-vetted and endorsed standards that can be used as means of compliance with FAA regulations.

As described before, the RTCA established SC-239 to:

- … produce a report assessing the potential interference impact on radar altimeters of 5G telecommunication signals transmitted on frequencies near to the 4200–4400 MHz band. The report may include recommendations for needed mitigations to protect existing radar altimeters in operational service from such interference sources.

- RTCA SC-239 shall update equipment standard MOPS DO-155 for Low Range Radar Altimeters to guarantee the robustness of future Radar Altimeters against existing and planned IN BAND and OUT OF BAND interferences (WAIC, 5G, EAN, etc.).

The initial output was RTCA Paper No. 274-20/PMC-2073 (October 2020), with an errata in 2021.

The Aerospace Vehicle Systems Institute (AVSI) is a cooperative research environment comprising major aerospace companies and government organisations working along with academia to solve problems common to its members. The Wireless Avionics Intra-Communication (WAIC) project provided ITU-R with working papers that formed the basis for a WRC-15 agenda item to secure a specific spectrum allocation that satisfies the needs of WAIC systems.

The AVSI setup a project to conduct studies investigating RF interference with the operation of radar altimeters, for both interference sources that originate in the radio altimeter band (4.2–4.4 GHz) and sources that originate outside the radio altimeter band. The WAIC Project AFE 76s1 focused on coexistence between WAIC systems and radio altimeters operating in the radio altimeter band. The related project AFE 76s2 Out-of-band Interference with Radio Altimeters is focused on out-of-band interference (OOBI).

AFE 76s2 had the following objectives:

    - Characterise the interference power level at which altitude reported by a set of representative radio altimeters is noticeably affected, goes outside DO-155 or ARINC 707- 7b specified accuracy, and/or is not able to compute an altitude (NCD).

    - Characterise the interference signal bandwidth (equivalent to multiple emitters on adjacent channels) at which altitude reported by a set of representative radio altimeters is noticeably affected.

    - Characterise the effect of interference signal frequency separation from the 4200–4400 MHz the band edge.

Note that these OOBI tests were conducted on a standardised laboratory bench for static platform conditions and at benign environmental conditions.

The results of these tests were summarised in the RTCA report and released separately in 3 volumes as detailed earlier. The volumes, while still anonymising radio altimeter models, breaks down measured performance by radio altimeter model, where the RTCA report grouped performance into worst-performing usage categories.

To support the development of a specification for the intended MOPS, the RTCA released a final-for-comment version of DO-399 under SC-239. The closing date for comments was 2 December 2022 and the document is no longer publicly available. While the intent of the document was to provide a basis for possible specifications under the MOPS and the measurements should not be extrapolated to the performance of any possible radio altimeter developed using the eventual MOPS, it is noted that:

    - It also provides a very useful background into radio altimeter systems including loop losses.

    - It indicates that performance above the levels in DO-399 are unlikely to be achieved with any foreseeable technology.

    - All results include a 6dB safety factor.

    - The best achievable overload performance is significantly better than radio altimeters under M.2059 (and for like-for-like RTCA usage categories as well).

    - In-band performance (that is, unwanted emissions from 5G), while significantly above the M.2059 sensitivities are in a similar range to that measured in the RTCA report for like-for-like categories.

    - It also includes guidance on how to use any best available PFD (BAPFD) results to scenarios and assesses some scenarios.

The CTIA provided some feedback on the draft DO-399 to the RTCA in December 2022, on behalf of the (US) wireless industry.

# Appendix D: Development of proposed interim mitigations

## Summary

This appendix discusses the development of possible interim additional mitigations to support the coexistence between radio altimeters and WBB deployments. The ‘additional’ mitigations are based on an adapted French/Canadian approach applied at specified runway approaches, along with general mitigations applicable at all locations.

The French method of calculating exclusion and restricted zone sizes and the original Canadian method of calculating the power flux density (PFD) limit in restricted zones is summarised. These methods are then applied in an Australian context to develop proposed mitigations, including an additional consideration of the reduced EIRP in proposed restricted cell areas in metro and regional areas.

The use of an unwanted emissions limit as an additional general mitigation is summarised. This calculation is based on a similar methodology used in determining the zone sizes, but applicable to the unwanted emissions case.

Where the Canadian methodology is referenced here in Appendix D, it is the original Canadian proposal, not the revised December 2022 proposal for consultation.

## Airport mitigations

### Overview

The overall approach to mitigations for airports is based on the creation of zones around certain airport runways where additional limitations are placed on WBB deployments. The approach is summarised in the diagram and table below.

Mitigation zones around certain airport runways (not to scale)

Summary of mitigation zones around certain airport runways

| Zone | Definition |
| --- | --- |
| Exclusion zone | Area within which WBB base stations cannot operate. |
| Restricted zone | Area within which WBB base stations can operate but must meet a defined PFD level at a certain height. |

Zone size calculations are based on the methodology that the French regulator (ANFR) adopted. This methodology is then applied to Australian specific conditions (such as EIRP limits) to determine the size of the zones for Australia.

The PFD limit applicable to WBB deployments in restricted zones is calculated based on the Canadian regulator’s (ISED) method, adapted to the Australian scenario.

## Methodology

### Determination of zone sizes

The method the ANFR used to determine the appropriate size of their zones is available on their website and detailed in an ICAO meeting input paper. This approach is based on the use of the Interference Tolerance Masks (ITM) for aircraft Usage Category 1 (UC 1) radio altimeter protection requirements included in the RTCA Report. Basic free space loss calculations are then used to determine distances where emissions from WBB deployments meet the protection requirement at specific heights in aircraft approach and take-off path. As outlined in Figure 6, different geometries are used to calculate the different zone sizes.

Outline of zone sizes definitions – where both approaches to a runway require mitigations

#### Exclusion zone (half) width

The half-width of the exclusion zone is calculated using the RTCA UC 1 ITM at 200 ft (RTCA Figure 9-1), with an additional 6 dB safety margin applied. This safety margin is used in the ANFR study and is an aviation safety factor. In this scenario, the WBB base station is assumed to be at roughly the same height and pointing directly at the radio altimeter, hence the maximum EIRP is used (see Figure 7). The distance required in free space to reduce the maximum operating EIRP of the WBB base station to the required ITM is then determined as the half-width of the exclusion zone.

Calculation method dimensions for half-width of zones

The Link Budget formula for the calculation of the exclusion zone (half) width:

Path loss=PTx+GTx+GRx+Margin-ITMthres       (Equation 1)

Path loss (dB) – free space path loss required to meet protection requirement.

PTx (dBm/MHz) – power into the antenna of the WBB base station.

GTx (dBi) – gain of the WBB antenna in direction of the radio altimeter. For the exclusion zone width scenario, it is assumed this is maximum gain of the antenna (see Figure 7).

GRx (dBi) – radio altimeter antenna gain in direction of WBB base station (assumed to 0 dBi).

Margin (dB) – safety margin (6 dB).

ITMthres (dBm/MHz) – Interference Threshold Mask (ITM) value for UC 1 at 200 feet.

#### Exclusion zone extension length

The extension length of the exclusion zone is calculated using the half-width of the exclusion zone. It is the half-width of the zone plus a margin of 1,130 metres. This margin of 1,130 m is determined by the distance when an aircraft is at an altitude of 200 feet (61 metres). The aircraft approach angle used by the ANFR study is 3 degrees with 0.375 degrees margin, which, in the worst case, is 2.625 degrees. 200 metres is subtracted from this horizontal distance to account for the touchdown distance, the distance from the end of the runway to where the aircraft lands (see Figure 7).

Exclusion zone extension length m=exclusion zone half width+1,130 m (Equation 2)

#### Restricted zone (half) width

The half-width of the restricted zone is calculated using the RTCA UC1 ITM at 1,000 feet (RTCA Figure 9-1), with no additional safety margin. In this scenario, the WBB base station is assumed to be well below the radio altimeter (see Figure 7). The base station gain to the radio altimeter is, therefore, determined by worst-case grating lobes, based on an assumed general limitation on the WBB base station that it is not to scan above the horizon. The distance required in free space to reduce the maximum operating EIRP of the WBB base station to meet the required ITM is then determined as the half-width of the restricted zone. This distance is assumed to be at 45 degrees to the ground because the aircraft altitude is much higher than the base station with the zone width being the horizontal distance in the French method (see Figure 6).

The link budget for this equation is the same as for the exclusion zone width (Equation 1) but with the following parameter changes:

GTx (dBi) – gain of the WBB antenna in direction of the radio altimeter. For the restricted zone scenario, this is assumed to be a grating lobe with a gain of 18 dBi (see Figure 7)

Margin (dB) – none (0 dB)

ITMthres (dBm/MHz) – Interference Threshold Mask (ITM) value for UC 1 at 1,000 feet.

#### Restricted zone length

The length of the restricted zone is the horizontal distance when an aircraft is at an altitude of 1,000 feet (305m) with an additional 80 metre margin to account for uneven terrain and base station height. The aircraft approach angle is 3 degrees with 0.375 degrees margin, which is 2.625 degrees in the worst case. 200 metres is subtracted from this horizontal distance to account for the touchdown distance.

As the length is defined as the distance from the edge of the exclusion zone (not the runway), the restricted zone length decreases as the exclusion zone length increases:

Restricted zone length m=8,200 m-exclusion zone length (Equation 3)

Summary of zone sizes and their definitions

| Zone size | Definition | Calculation method |
| --- | --- | --- |
| Exclusion zone half-width | Distance laterally either side of a runway centreline, and beyond the opposite end of the landing end of a runway. | Free space path loss calculation based on maximum power limit and frequency. |
| Exclusion zone extension length | Distance beyond the end of the runway landing end. | Dependent on the exclusion zone half-width. |
| Restricted zone half-width | Distance laterally either side of a runway centreline. | Free space path loss calculation based on maximum power limit and frequency. |
| Restricted zone length | Distance beyond the end of the exclusion zone landing end. | Dependent on the exclusion zone length. |

Figure 7a below summarises the derivation of the geometry of the exclusion and protection zones.

- Figure 7a: Overall zone geometry

### Determination WBB power flux density (PFD) limit in restricted zones

The method ISED used to calculate the PFD limit in their protection zones is available on their website. Their calculations are based on the use of the RTCA report ITM for UC 1 aircraft, converting the power spectral density of the ITM at the aircraft altitude on the boundary of the restricted and exclusion zones to a PFD limit.

The altitude of the aircraft at the boundary between the exclusion and restricted zones is calculated, similar to the French method, using an aircraft approach angle of 3 degrees with 0.375 degrees margin, resulting in 2.625 degrees as the worst case (see Figure 8 below). The horizontal distance includes a touchdown distance of 200 metres.

Aircraft altitude at boundary of exclusion and restricted zone

This calculated altitude is used to determine the interpolated ITM value in the RTCA report for UC 1 (RTCA Figure 9-1).

  - The ITM power spectral density value (PSD) is converted to a PFD value using the calculation:

pfd=ITM-30-10log10(Ar)               (Equation 4)

Ar=c24πf2               (Equation 5)

Ar (m2) – the effective aperture of the receiver antenna

c (m/s) – the speed of light, 3.0 x 108 m/s

f (Hz) – the centre frequency of the base station

ITM (dBm/MHz) – the PSD value from the ITM

The PFD limit applies at and above the altitude of the aircraft at the boundary between the exclusion and restricted zones.

## Application of methodology to the Australian environment

The values for these mitigations adopted for France and Canada cannot directly apply to the Australian operating environment due to differences in some parameters, such as WBB maximum EIRP and operating frequency. This section applies the methodology described above using parameters relevant to the Australian environment to determine appropriate Australian mitigation values.

For example, the zone sizes used by ANFR are calculated at an operating frequency of 3700 MHz and ISED’s PFD calculations are for base stations operating at 3650 MHz. In Australia, WBB is planned to be permitted to operate up to 4000 MHz at various EIRP levels.

### Runways that require protection from only one direction

France and Canada example cases always assume that a runway requires protection for landing approaches from either end. This is not always the case. Where protection is required from only one direction, the ACMA considers the following geometry appropriate in Figure 9:

‘Single-ended’ runway zone sizes

For an approach/runway requiring protection in only one landing direction, the exclusion zone half-width is also appropriate at the far end of the runway, so that ground-level protection is provided during taxiing and pre-flights checks for take-off.

### Australian proposed EIRP limits

In calculating the exclusion zone size, ANFR used an example EIRP of 58 dBm/MHz because it is the maximum base station power allowed in the band under their proposed regulations. The ACMA has proposed a maximum EIRP density limit of 62 dBm/MHz.

In metro and regional areas, the 3950–4000 band is allocated for restricted cell use. The proposed maximum EIRP limit in this band is 17 dBm/MHz, with a maximum 34 dBm overall EIRP limit. Due to the significantly lower powers, the zone calculations will need to be adjusted. This is discussed further in section 8.46, Restricted cell use in the 3950–4000 MHz range below.

#### Example: Australian zone size calculations at 3750 MHz

The following worked example uses a base station operating at 3750 MHz, a frequency with measured RTCA ITM values, using Australian EIRP limits.

Calculation example for exclusion zone half-width (3750 MHz)

| Parameter | Variable | Value | Notes |
| --- | --- | --- | --- |
| EIRP (in 100 MHz) | PTx+GTx | 82 dBm EIRP | ACMA temporary, reviewable mitigation (ix) |
| Radio altimeter gain | GRx | 0 dBi | ANFR study refers to RTCA report 0 dBi |
| Margin | Margin | 6 dB | ANFR study used the 6 dB aviation safety factor |
| ITM threshold (in 100 MHz) | ITMthres | -19 dBm | RTCA Fundamental emissions -39 dBm/MHz at 200 feet. UC 1 3750 MHz |
| Required Path Loss | =PTx+GTx+GRx+Margin-ITMthres | 107 dB | Free Space Path Loss Model |
| Exclusion zone half-width (equivalent distance to achieve path loss) | Path Loss=32.4+20logf+20logd dB f:frequency MHz d:distance (km) | 1,432 m (rounded up to 1,440 m) | Setting the required path loss to path loss and solving for distance Recommendation ITU-R P.525-4 equation (4) |

Calculation example for restricted zone half-width (3750 MHz)

| Parameter | Variable | Value | Notes |
| --- | --- | --- | --- |
| Maximum TRP (in 100 MHz) | PTx | 57 dBm | ACMA temporary, reviewable mitigation (ix) minus ANFR antenna gain of 25 dBi |
| Base station antenna gain | GTx | 18 dBi | Maximum assumed gain of grating lobes for high elevation angles |
| Radio altimeter antenna gain | GRx | 0 dBi | ANFR study refers to RTCA report use of 0 dBi |
| Margin | Margin | 0 dB | None used by ANFR for restricted zone |
| ITM threshold (in 100 MHz) | ITMthres | -26 dBm | RTCA -46 dBm/MHz at 1,000 feet UC 1 3750 MHz |
| Required Path Loss | =PTx+GTx+GRx+Margin-ITMthres | 101 dB | Free Space Path Loss Model |
| Equivalent distance to achieve path loss | Path Loss=32.4+20logf+20logd dB f:frequency MHz d:distance (km) | 717.7 m | Setting the required path loss to path loss and solving for distance Recommendation ITU-R P.525-4 equation (4) |
| Restricted zone half-width | =d×cos45° | 507.5 m (rounded up to 510 m) | Path loss distance 45 degrees angle from the horizontal |

Using the ACMA proposed EIRP limits, the size of the exclusion zone at 3750 MHz would be 2,570 metres from the end of the runway (extension length) and a width of 1,440 metres from the centre line (half-width) (rounded up from 2,562 and 1,432 metres). The size of the restricted zone would be 5,630 metres from the end of the exclusion zone and 510 metres from the centre line (rounded up from 507 metres).

#### Example: PFD limit in restricted zones calculation at 3750 MHz

This example uses a base station operating at 3750 MHz with the Australian exclusion zone extension length in the previous example. Note that this calculation uses the unrounded exclusion zone length (2,562 metres).

The altitude of the aircraft at the boundary between the exclusion and restricted zones is calculated via the French method (an aircraft approach angle of 3 degrees with 0.375-degree margin, so 2.625 degrees worst case) including a 200-metre touchdown distance.

The height of the aircraft is 126.6 metres (415 feet) at the exclusion/restricted zone boundary, which corresponds to an interpolated Power Spectral Density (PSD) of -42.2 dBm/MHz for the ITM in the RTCA report for UC 1 at a 3750 MHz centre frequency (RTCA Figure 9-1).

A PFD limit value of -39.25 dBW/m2/MHz at a height 126.6 metres above ground is calculated for the ACMA proposed case at 3750 MHz.

Calculation example for power flux density limit (3750 MHz)

| Parameter | Value |
| --- | --- |
| Distance from aircraft landing point to boundary between exclusion and restricted zones | 2,562 m + 200 m |
| Aircraft height at boundary/PFD limit restriction height | 126.6 m (415 ft) |
| Effective aperture of the receiver antenna Ar | 5.09 x 10-4 m2 |
| ITM threshold value PSD | -42.18 dBm/MHz |
| Centre frequency f | 3750 MHz |
| PFD limit | -39.25 dBW/m2/MHz |

### Australian operating frequencies

The methodology in the preceding sections uses a specific ITM value for a WBB base station operating at 3750 MHz, to allow for close comparison with French and Canadian examples. For base stations operating at other frequencies, the ITM values will require interpolation, up to 4000 MHz, so mitigations appropriately reflect the operating frequency of the base station.

Changing the operating frequency alters both the ITM protection value and the path loss calculations. The zone dimensions and restricted zone PFD value therefore change with the operating frequency of the base station. The PFD limits are also directly dependent on the ITM value at the operating frequency.

The PFD limit is calculated using the altitude of the aircraft at the exclusion zone boundary. As a result, it is dependent on the frequency of operation as it affects the zone sizes and therefore at what altitude the aircraft crosses the boundary (which changes the free space loss calculations).

Summary of the mitigation calculations and their dependencies on frequency

| Mitigation | ITM value | Path loss calculation |
| --- | --- | --- |
| Exclusion zone dimensions | Changes with frequency | Changes with frequency |
| Restricted zone dimensions | Changes with frequency | Changes with frequency |
| Power flux density limit | Changes with frequency and altitude (because of the zone dimensions) | Does not have a path loss calculation |

#### Interpolating the ITM

The zone sizes calculations are dependent on ITM values at the frequency of operation (RTCA, section 9), however the RTCA report only contains measured ITM values at 3 frequencies (3750 MHz, 3850 MHz and 3930 MHz). If necessary, the value at other frequencies can be interpolated or extrapolated to present the full possible operating range. The values at frequencies between 3700–4000 MHz, for example, can be approximated with linear piece-wise interpolation.

Plot of interpolated ITM values

Note, as indicated in response to questions in the RTCA report, the ITMs do not consider any out-of-band receiver antenna performance changes, as the AVSI considered it to have minimal effect.

#### Zone sizes at different frequencies

To reduce the complexity of calculating the mitigations while allowing for flexible use of the band, the zone sizes can be grouped into segments. The size of the segments is to be determined. Table 8 provides an example of 50 MHz segments.

Example table of zone sizes in a frequency range

| Base station frequency (MHz) | Exclusion zone extension length (m) | Exclusion zone half-width (m) | Restricted zone size (m) (length x half-width) |
| --- | --- | --- | --- |
| >3800 to 3850 | 2,890 | 1,760 | 5,310 x 560 |
| >3850 to 3900 | 3,000 | 1,870 | 5,200 x 680 |
| >3900 to 3950 | 3,110 | 1,980 | 5,090 x 840 |
| >3950 to 4000 (remote areas only) | 3,230 | 2,100 | 4,970 x 1,030 |

The zone dimensions for each segment are the values at the highest frequency in the range. The sizes have been rounded up to the nearest 10 metres.

It would be proposed that the appropriate zone size for a base station would be the size for the highest operating frequency of the licence.

The figures below are zones for the 3900–3950 MHz frequency range over some airports. Note that in metro and regional areas, these are the worst-case zone sizes because 3950–4000 MHz is allocated for restricted cell usage.

- Figure 11: Example of 3900–3950 MHz zones over Sydney Airport

At Melbourne Airport, only runway 16 is an identified runway, therefore the zones only apply to the aircraft landing from the northward direction.

- Figure 12: Example of 3900–3950 MHz zones over Melbourne Airport

- Figure 13: Example of 3900–3950 MHz zones over Perth Airport

#### The PFD limit at different frequencies

To reduce the complexity of calculating the PFD limit, while allowing for flexible use of the band, the PFD limit can be grouped into segments. Table 9 provides an example using 50 MHz segments.

Example table of mitigations in a frequency range

| Base station frequency (MHz) | Power flux density limit (dBW/m2/MHz) | Power flux density limit restriction height (m) |
| --- | --- | --- |
| >3800 to 3850 | -41 | 140 |
| >3850 to 3900 | -42.4 | 145 |
| >3900 to 3950 | -43.7 | 150 |
| >3950 to 4000 (remote areas only) | -45.2 | 155 |

The limits in the restricted zones are the worst-case value in each range, which is at the highest frequency in the range. The PFD limits have been rounded down to the nearest 0.1 and the restriction heights rounded down to the nearest 5 meters.

It would be proposed that the appropriate limit for a base station would be the limit for the highest operating frequency of the licence.

### Restricted cell use in the 3950–4000 MHz range (metro and regional areas only)

In metro and regional areas, the 3950–4000 band is proposed to be allocated for restricted cell use. The current proposed maximum EIRP density limit in this band is 17 dBm/MHz, with a maximum 34 dBm overall EIRP limit for a transmitter.

Calculating the exclusion zone required path loss and equivalent distance with the restricted cell EIRP limit results in a worst case (at 4000 MHz) path loss of 65.88 dB, or a distance of 12 metres.

In previous cases, the extension length of the exclusion zone is the path loss distance with an additional 1,130 metres, calculated from when the horizontal distance of the aircraft is at 200 feet altitude. However, this assumes that the aircraft will enter the exclusion zone at 200 feet or above. The path loss distance in this case is less than 200 feet (61 metres).

For this shorter distance, the exclusion zone extension length can be determined by when the aircraft approaches within 12 metres of a base station. The highest altitude in which this could happen would be when the base station is directly underneath the aircraft. An aircraft is at the altitude of 37 metres (25m base station + 12m path loss distance) at 810 metres from the landing point with an approach slope of 2.625 degrees. Subtracting 200 metres for the touchdown distance results in an exclusion zone extension length of 610 metres from the runway edge.

The exclusion zone half-width of 12 metres can be less than the half-width of the physical runway. Therefore, if the physical width of a runway is greater than 24 metres, the width of the exclusion zone is increased to the width of the runway.

The restricted zone required path loss for restricted cell use is a worst case (at 4000 MHz) path loss of -52.63 dB, equal to a distance of 9 metres. The highest altitude when this could first occur is when a base station is directly under the aircraft. However, this can only occur within the exclusion zone defined above, where base stations are not allowed. For the rest of the aircraft landing from 1,000 feet altitude and 8,200 metres horizontally from the runway (the range that the restricted zone is calculated to protect), the base station EIRP level should not meet the ITM threshold. As a result, an exclusion zone is not required for restricted cell use.

In summary, for restricted cell use in 3950–4000 MHz around identified runways, there are temporary reviewable mitigations:

    - Exclusion zones, where no WBB services are permitted.

      - An ‘exclusion zone’ with an extension length of 610 metres and an extension zone half-width of 12 metres.

      - If the runway width is greater than 24 metres then the exclusion zone half-width of the area is equal to half the width of the runway.

## Comparison with French and Canadian results

### Zone size comparisons

  - The figures below compare the sizes of the French zones to the ACMA example 3750 MHz zone. Note we are not proposing zones to apply below 3800 MHz and the 3750 MHz is for comparative purposes only.

- Figure 14: ANFR zone sizes

- Figure 15: ACMA example zone sizes at 3750 MHz (not to scale)

Key factors that result in differing zone sizes are:

    - The ANFR zone sizes are calculated at 3700 MHz, while the ACMA example zone size is at 3750 MHz.

      - Lower WBB BS operating frequencies require greater distances to achieve the same path loss requirement.

      - However, the change in distance is small, for example with a fixed ITM threshold value, a change in operating frequency between 3700 MHz and 3750 MHz results in a decrease of ~12 metres.

    - The maximum EIRP used in the ANFR calculations is 58 dBm/MHz EIRP, while the proposed maximum EIRP density in Australia is 62 dBm/MHz EIRP.

      - Higher operating WBB BS power requires greater path loss, which requires greater distances.

      - This difference contributes more to the different zone sizes – at 3700 MHz with a fixed ITM threshold value, the difference between 58 dBm/MHz EIRP and 62 dBm/MHz EIRP is ~500 metres.

    - The ITM threshold value is interpolated at different frequencies in the ACMA calculation.

      - The linear piece-wise interpolation of the ITM thresholds at 3750 MHz, 3850 MHz and 3930 MHz to in-between frequencies in Figure 10 show a decreasing ITM threshold as frequency increases. A lower threshold requires greater path loss and therefore greater distances.

      - This contributes significantly to the distances – between 3700 MHz and 3850 MHz, the ITM threshold increases the required path loss by ~1 dB per 50 MHz.

Comparison between ANFR and ACMA calculations (exclusion zone)

| Parameter | ANFR example value | ACMA example value |
| --- | --- | --- |
| WBB BS Frequency | 3700 MHz | 3750 MHz |
| EIRP (in 100 MHz) PTx+GTx | 78 dBm EIRP | 82 dBm EIRP |
| Radio altimeter gain GRx | 0 dBi | 0 dBi |
| Margin | 6 dB | 6 dB |
| ITM Threshold (in 100 MHz) ITMthres | -19 dBm | -19 dBm |
| Required path loss | 103 dB | 107 dB |
| Exclusion zone half-width (equivalent distance to achieve path loss) | 903.6 m(rounded up to 910 m) | 1,432 m (rounded up to 1,440 m) |

Comparison between ANFR and ACMA calculations (restricted zone)

| Parameter | ANFR example value | ACMA example value |
| --- | --- | --- |
| WBB BS frequency | 3700 MHz | 3750 MHz |
| Maximum TRP (in 100 MHz) PTx | 53 dBm | 57 dBm |
| Base station antenna gain GTx | 18 dBi | 18 dBi |
| Radio altimeter antenna gain GRx | 0 dBi | 0 dBi |
| Margin | 0 dB | 0 dB |
| ITM threshold (in 100 MHz) ITMthres | -26 dBm | -26 dBm |
| Required path loss | 97 dB | 101 dB |
| Equivalent distance to achieve path loss | 452.9 m | 717.7 m |
| Restricted zone half-width | 320.2 m (rounded up to 400 m) | 507.5 m (rounded up to 510 m) |

### PFD limit comparison

ISED established a PFD limit of -38.80 dBW/m2/MHz at 91.44 metres above ground and the ACMA example 3750 MHz PFD limit is -39.25 dBW/m2/MHz 126.6 metres above ground (unrounded).

The differences in the Australian scenario are:

    - The aircraft height at the boundary of the exclusion and restricted zones is related to the length of the exclusion zone. The ISED limit is calculated at 91.44 metres (300 feet), whereas the ACMA altitude at 3700 MHz is 126.6 metres (415 feet).

      - The longer the exclusion zone extension length, the higher the altitude of the aircraft. The altitude of the aircraft adjusts the interpolation value of the ITM PSD interference threshold. Between 200 feet and 5,000 feet, the UC 1 ITM (Figure 9-1 of the RTCA report) has a decreasing threshold as altitude increases.

      - For base stations operating between 3800–4000 MHz, the aircraft height will increase as frequency increases. As the frequency increases, the exclusion zone extension length increases, which will increase the aircraft height at the boundary.

    - The centre frequency of the WBB base station in the ISED case is 3650 MHz, the ACMA example uses 3750 MHz.

      - This adjusts the effective aperture of the receiver antenna, which decreases the PFD limit as frequencies increase.

      - However, the change in the PFD limit is small, with less than a dB between 3700 MHz and 4000 MHz.

    - The ITM threshold value is interpolated at different frequencies and altitudes in the ACMA calculation.

      - The linear piece-wise interpolation of the ITM thresholds at 3750 MHz, 3850 MHz and 3930 MHz to in-between frequencies in Figure 10 show a decreasing ITM threshold as frequency increases.

      - An addition to interpolation between frequencies, the interpolation between aircraft altitude also shows a decreasing ITM threshold as frequency increases. A lower threshold requires greater path loss and, therefore, greater distances.

Comparison between ANFR and ACMA calculations

| Parameter | ISED example Value | ACMA example value |
| --- | --- | --- |
| Centre frequency f | 3650 MHz | 3750 MHz |
| Exclusion zone extension length from end of runway | 2,100 m | 2,562 m |
| Aircraft height at boundary/PFD limit restriction height | 91.44 , (300 ft) | 126.6 m (415 ft) |
| Effective aperture of the receiver antenna Ar | 5.38 x 10-4 m2 | 5.09 x 10-4 m2 |
| ITM threshold value PSD | -41.5 dBm/MHz | -42.18 dBm/MHz |
| Power flux density limit | -38.80 dBW/m2/MHz | -39.25 dBW/m2/MHz |

## General interim mitigations

  - In addition to specific mitigations around airports, general interim mitigations have been identified, which are proposed to apply everywhere.

### WBB base station unwanted emission limits

  - A potential unwanted emissions limit, based on using RTCA UC 1 5G spurious emissions ITM values, can be calculated using a similar method to the exclusion zone path loss calculations, but considering relevant factors for the unwanted emissions, rather than overload case:

PTx=Path loss-GTx-GRx-Margin+ITMthres               (Equation 6)

Path loss (dB) – free space path loss between the WBB base station and the radio altimeter

PTx (dBm/MHz) – power into the antenna of the WBB base station

GTx (dBi) – gain of the WBB antenna in direction of the radio altimeter

GRx (dBi) – radio altimeter antenna gain in direction of WBB base station, this is 5 dBi from the RTCA figure 6-12 of ~8dBi plus AVSI use of 3 dB cable loss.

Margin (dB) – none (0 dB)

ITMthres (dBm/MHz) – Interference Threshold Mask (ITM) value for UC 1 spurious emissions (RTCA report Figure 9-10, reproduced below).

  - Two cases are initially considered, as the ITM varies with height at an example 3750 MHz for the purposes of calculation of zone sizes.

#### At the exclusion zone and restricted zone boundary

The altitude of the aircraft at the boundary between the exclusion and restricted zones is calculated via the French method (an aircraft approach angle of 3 degrees with 0.375-degree margin) including a 200-metre touchdown distance. The height of the aircraft is then 126.6 metres (415 feet).

A worst-case base station location concerning path loss would be directly underneath the aircraft radio altimeter, with an assumed 25-metre-high base station. The minimum distance between the base station and the radio altimeter is then 101.6 metres.

For a distance of 101.6 metres and frequency of 4200.5 MHz, the path loss is 85 dB.

At an altitude of 126.6 metres (415 feet) the ITM Spurious emissions for UC 1 corresponds to -82.3 dBm/MHz.

Calculation summary (at exclusion zone/restricted zone boundary)

| Parameter | Value | Notes |
| --- | --- | --- |
| Path loss | 85 dB | Recommendation ITU-R P.525-4 equation (4) with distance 92.5 m from top of base station to aircraft at boundary and frequency 4200.5 MHz |
| Transmitter gain, GTx | 18 dBi | Maximum gain of grating lobes for high elevation angles, worst case assuming any antenna is still correlated at the radio altimeter frequencies and has the same gain |
| Radio altimeter net gain, GRx | 5 dBi | RTCA figure 6-12 of ~8dBi less AVSI use of 3 dB cable loss |
| ITM Threshold (in 1MHz), ITMthres | -82.3 dBm/MHz | RTCA Cat 1 4.2–4.4 GHz band spurious limit at 415 ft |
| Maximum Tx unwanted emissions power/TRP to meet requirement per MHz, PTx | -20.3 dBm/MHz |  |

#### A second calculation is also performed at the far end of the restricted zone

The altitude of the aircraft at the far end of the restricted zone (8,200 metres from runway) is calculated via the French method (an aircraft approach angle of 3 degrees with 0.375-degree margin) including a 200-metre touchdown distance. The height of the aircraft is then 385.1 metres (1264 ft).

A worst-case base station location concerning path loss would be directly underneath the aircraft radio altimeter, with a 25-metre-high base station. The minimum distance between the base station and the radio altimeter is 360.1 metres.

For a distance of 360.1 metres and frequency of 4200.5 MHz, the path loss is 95.99 dB.

At an altitude of 385.1 metres (1,264 feet), the ITM Spurious emissions for UC 1 corresponds to -88.2 dBm/MHz.

Calculation summary (at far end of restricted zone)

| Parameter | Value | Notes |
| --- | --- | --- |
| Path loss | 95.99 dB | Recommendation ITU-R P.525-4 equation (4) with distance 351 m from top of base station to aircraft at boundary and frequency 4200.5 MHz |
| Transmitter gain, GTx | 18 dBi | Maximum gain of grating lobes for high elevation angles, worst case assuming any antenna is still correlated at the radio altimeter frequencies and has the same gain |
| Radio altimeter net gain, GRx | 5 dBi | RTCA figure 6-12 of ~8 dBi less AVSI use of 3 dB cable loss |
| ITM Threshold (in 1MHz), ITMthres | -88.2 dBm/MHz | RTCA Cat 1 3750 MHz spurious limit at 1,234 ft |
| Maximum Tx unwanted emissions power/TRP to meet requirement per MHz, PTx | -15.2 dBm/MHz |  |

#### An EIRP limit for unwanted emissions rather than TRP

An EIRP limit for unwanted emissions would be desirable, as a TRP limit is based upon assumptions around antenna gain and sidelobes. In this case, however, use of an EIRP limit raises the following issue:

- Sidelobe antenna performance is the influencing factor for the geometry within the restricted zone but outside of the restricted zone, peak antenna gain may be more relevant.

Consequently, a third-case calculation could be done at the edge of the restricted zone half-width, assuming that the aircraft is in-line with the base station peak EIRP and determining a third possible TRP limit. The TRP/antenna combination that protects the aircraft the most out of the 3 test cases could then be used to determine an overall unwanted emissions EIRP limit.

#### At the edge of the restricted zone half-width

This case considers a base station operating at the edge of the restricted zone half-width.

- Figure 16: Test base station locations

The altitude of the aircraft on the boundary between the exclusion and restricted zones is calculated via the French method (an aircraft approach angle of 3 degrees with 0.375-degree margin) with a 200-metre touchdown distance. The height of the aircraft is then 126.6 metres (415 feet).

With a 25-meter-high base station, the vertical distance between the base station and the radio altimeter is 101.6 metres. The horizontal distance of the base station from the aircraft will be the half-width of the restricted zone, which is 510 metres. The overall distance between the aircraft and base station is therefore 520 metres.

Calculation summary (at restricted zone half-width edge)

| Parameter | Value | Notes |
| --- | --- | --- |
| Path loss | 99.1 dB | Distance 520 m from top of base station to aircraft at boundary and frequency 4200.5 MHz |
| Transmitter gain, GTx | 25 dBi | Maximum assumed gain of operating antenna. |
| Radio altimeter net gain, GRx | 5 dBi | RTCA figure 6-12 of ~8 dBi less AVSI use of 3 dB cable loss |
| ITM Threshold (in 1MHz), ITMthres | -82.3 dBm/MHz | RTCA Cat 1 4.2-4.4 GHz band spurious limit at 415 ft |
| Maximum Tx unwanted emissions power/TRP to meet requirement per MHz, PTx | -13.2 dBm/MHz |  |

  - With worst-case peak antenna gain of 25 dBi, the potential unwanted emissions EIRP limits for the 3 cases are:

Unwanted emissions test cases

| Case | Unwanted emissions TRP limit | Unwanted emissions EIRP limit |
| --- | --- | --- |
| Restricted zone near edge | -20.3 dBm/MHz | 4.7 dBm/MHz |
| Restricted zone far edge | -15.2 dBm/MHz | 9.8 dBm/MHz |
| Restricted zone half-width edge | -13.2 dBm/MHz | 11.8 dBm/MHz |

  - Consequently, the worst-case (lowest value) unwanted emissions EIRP limit is 4.7 dBm/MHz in the 4200–4400 MHz range, for the example 3750 MHz frequency. This assumes that a downtilt requirement is in place so that the peak EIRP within the restricted zone is not directed upwards to the aircraft.

#### Comparison with the draft AWL LCD unwanted emission limits

The draft Area-Wide Licence Conditions Amendment Determination 2022 defines different unwanted emissions limits, depending on the frequency offset from the base station. This is, effectively, to define different limits to the ‘out-of-band’ and the ‘spurious’ domains. The unwanted emissions limit determined above can then be potentially applied at the 4200 MHz boundary to better protect the radio altimeter operating band.

To compare limits interpreted as EIRP limits, an antenna gain of 25 dBi has been applied (for the non-AAS base station, the total EIRP assumes a 19 dBi antenna gain operating with 4 ports so 19 dBi + 6dB = 25 dBi).

Unwanted emissions limits above 4200 MHz in draft AWL LCD

|  | 4200–4210 MHz | 4210–4240 MHz | >4240 MHz |
| --- | --- | --- | --- |
| 3GPP/AWL LCD | 3 to -4 dBm/MHz mean power per antenna port non-AAS 12 to 5 dBm/MHz TRP AAS | -15 dBm/MHz mean power per antenna port non-AAS -6 dBm/MHz TRP AAS | -30 dBm/MHz mean power per antenna port non-AAS -21 dBm/MHz TRP AAS |
| Based on restricted zone calculations | -20.3 dBm/MHz TRP or total mean power into all antenna ports. |  |  |

Unwanted emissions limits above 4200 MHz in draft AWL LCD (converted to EIRP)

|  | 4200–4210 MHz | 4210–4240 MHz | >4240 MHz |
| --- | --- | --- | --- |
| 3GPP/AWL LCD | 28 to 21 dBm/MHz EIRP non-AAS 37 to 30 dBm/MHz EIRP AAS | 10 dBm/MHz EIRP non-AAS 19 dBm/MHz EIRP AAS | -5 dBm/MHz EIRP non-AAS 4 dBm/MHz EIRP AAS |
| Based on restricted zone calculations | 4.7 dBm/MHz total EIRP for AAS or non-AAS. |  |  |

We expect that WBB systems will easily be able to comply with the unwanted emissions limit, either applied specifically to the restricted zones or generally.

This aspect of WBB system performance has been examined in the US through their C-band allocation process, see FCC docket 18-122. In a Verizon letter to the Secretary of the Department of Transport on January 2 2022, it was stated that, ‘Leading equipment vendors have stated on the record that C-Band transmissions will be -30 dBm/MHz or less at 4.2 GHz’.

#### The unwanted emissions limit at different frequencies

The unwanted emissions limit is calculated with the altitude of the aircraft at the exclusion zone boundary. As a result, it is dependent on the frequency of operation as it affects the zone sizes and, therefore, at what altitude the aircraft crosses the boundary (which changes free space loss calculations).

To reduce the complexity of calculating the mitigations while allowing for flexible use of the band, the mitigations can be grouped into segments. The size of the segments is to be determined. Table 19 provides an example of 50 MHz segments.

Example mitigations in a frequency range

| Base station frequency (MHz) | Unwanted emissions limit EIRP (dBm/MHz) |
| --- | --- |
| >3800 to 3850 | 5.1 |
| >3850 to 3900 | 5.5 |
| >3900 to 3950 | 5.8 |
| >3950 to 4000 (remote areas only) | 6 |

The limits in the restricted zones are the worst-case value in each frequency range, with the unwanted emissions limit calculated using the lowest frequency in the range. They have been rounded down to the nearest 0.1.

It is proposed that the appropriate limit for a base station will be the limit for the lowest operating frequency of the licence.

## Summary of proposed mitigations

Based on the considerations in preceding sections, we propose precautionary mitigations as follows:

Summary of ongoing mitigations

| Mitigation | Purpose |
| --- | --- |
| 200 MHz guard band in 4000–4200 MHz | Already determined as a reasonable guard band and to provide exclusive use by incumbent service types. |
| Not proposed to be implemented: For all WBB licensees in 3400–4000 MHz, it is recommended when planning and deploying their networks, to take note of the co-existence issues that may arise due to aeronautical radio-navigation services, typically radio altimeters on aircraft, that may operate in the 4200–4400 MHz band. It is also recommended that affected licensees seek to coordinate with airports, heliports and aircraft operators to help prevent, manage and resolve interference that may arise to aeronautical radio-navigation stations. | A general requirement to consider potential interference even where no specific mitigations apply. This is based on similar requirements in the 1800 MHz RAG tx to coordinate spectrum licences with rail services. - This ongoing mitigation could be implemented by a clause in RALI MS47 and an amended Radiocommunications Advisory Guidelines (Managing Interference from Spectrum Licensed Transmitters — 3.4 GHz Band) |
| - A range of unwanted emission limits, TRP or per antenna port based, in addition to those that apply during the temporary mitigations using EIRP limits. The limits can be found in the draft spectrum licence for spectrum licenced services or the area-wide licence (AWL) licence condition determination. | To provide general unwanted emissions limits that align with 3GPP standards and methods as much as is practicable to enable common equipment ecosystems. |

Interim mitigations until 31 March 2025:

    - For deployments above 3800 MHz around identified runways:

- An ‘identified runway’ is one identified by CASA as requiring the protection of radio altimeters due to safety and/or important operational requirements.

Interim mitigations until 31 March 2025

| Mitigation | Purpose | Definition |
| --- | --- | --- |
| Exclusion zones, where no WBB services are permitted | Zone size based on the French derivation which uses the RTCA ITM for UC 1 plus a 6 dB safety margin. Size adapted to EIRPs necessary in Australia and the frequency ranges of allocation. Designed to protect aircraft landings and pre-flight activities. | An ‘exclusion zone’ is comprised of 3 segments: - a segment of length (exclusion zone extension length) from the landing end of an identified runway - a segment of length (exclusion zone half-width) at the opposite end of the landing end of an identified runway - a segment the length of the runway. The width of each segment is the ‘exclusion zone half-width’ either side of the identified runway’s centre-line. Dimensions are the applicable maximum operating frequency row in Table 19 columns B and C. |
| Restricted zones with a PFD limit in the restricted zones | Zone size based on the French derivation which uses the RTCA ITM for UC 1 but with no safety margin. Size adapted to EIRPs necessary in Australia and range of allocation. PFD limit calculation the same as the Canadian method but adapted to EIRPs necessary in Australia and range of allocation. Designed to protect aircraft landing approaches. | A “restricted zone” is an area extending lengthwise from each end of the exclusion zone, and horizontally from an identified runway centerline. The dimensions apply using the maximum operating frequency row in Table 19 column D. The PFD limit value is defined in Table 15 column F for the maximum operating frequency row. The PFD cannot exceed that value at a height above the ground of the corresponding column G of Table 19, for any percentage of the time for all combinations of elevation and azimuth angles above the horizon, for deployments in the restricted zone. |

    - For deployments everywhere above 3800 MHz:

| Mitigation | Purpose | Definition |
| --- | --- | --- |
| Unwanted emission limits in 4200–4400 MHz | These have been based on the use of RTCA ITMs for UC 1 with no margin and are designed to protect aircraft landing approaches, plus also providing generally reduced risk of potential issues for all flight cases. | The ‘additional unwanted emission limits’ in 4200–4400 MHz are at most the value for the minimum operating frequency row from Table 19 column E, anywhere in the 4200–4400 MHz range. |
| Maximum EIRP density limits | Integral to the other calculations, but a reasonable limit to permit macro WBB deployments while also providing generally reduced risk of potential issues for all flight cases. | The ‘maximum EIRP density limit’ is 62 dBm/MHz outside of metro and regional restricted cell areas. |
| Restricting any WBB antenna system to point maximum power or scan any beam below the horizon only | To ensure that antenna systems do not scan to handsets located on aircraft and also providing generally reduced risk of potential issues for all flight cases. |  |
| Grating lobes of WBB antenna systems should be minimised as much as is practicable | To ensure unwanted power from the base station is minimised in the potential directions of aircraft, to generally reduce risk of potential issues for all flight cases. |  |

Table 22: Example table of mitigations in a frequency range

| A | B | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- | --- |
| Base station operating frequency (MHz) | Exclusion zone extension length (m) | Exclusion zone half-width (m) | Restricted zone size (m) (length x half-width) | Unwanted emissions limit EIRP (dBm/MHz) | Power flux density limit (dBW/m2/MHz) | Power flux density limit restriction height (m) |
| >3800 to 3850 | 2890 | 1760 | 5310 x 560 | 5.1 | -41 | 140 |
| >3850 to 3900 | 3000 | 1870 | 5200 x 680 | 5.5 | -42.4 | 145 |
| >3900 to 3950 | 3110 | 1980 | 5090 x 840 | 5.8 | -43.7 | 150 |
| >3950 to 4000 (remote areas) | 3230 | 2100 | 4970 x 1030 | 6 | -45.2 | 155 |
| >3950 to 4000 (restricted cell use, metro and regional areas) | 610 | 12 | N/A | -36.4 | N/A | N/A |

It should be noted that the list of identified runways may need to change as airport runways are re-developed or upgraded to improved landing systems. It is understood that lead times can vary significantly, depending upon whether it is an upgrade to an existing runway/approach or a new runway.

It should be noted that, for practical WBB equipment (outside of the restricted cell areas), the additional spurious emissions EIRP limit may already be achieved in parts of the 4200–4400 MHz range using the ongoing unwanted emissions limits based on TRP or power per antenna port limits alone.

  - RTCA UC 1 is defined as Commercial Air Transport Aircraft in the RTCA report, being ‘commercial air transport airplanes, both single-aisle and wide-body’. The RTCA discusses the relationship of their usage categories and Instrument Landing System categories. They consider that CAT II/III approaches are critical and are a common occurrence for UC 1.  Consequently, similar to approaches in France, Canada and the US, the mitigations have been designed to primarily protect UC 1 landings, but with some consideration to other flight scenarios and categories.

## Frequency boundary for mitigations

This section considers the basis for a lower frequency boundary for interim mitigations.

The replanning of the 3700–4200 MHz band outcomes published in January 2021 considered the 3700–4200 MHz range only, but it was noted that:

- During the implementation of the outcomes of the 3700–4200 MHz band, we will consider technical, regulatory and timing synergies to promote holistic treatment across the 3400–4200 MHz range.

The ‘Approach B’ possible mitigations discussed in the TLG paper and the Allocation of AWLs in the 3.4–4.0 GHz band in remote Australia consultation  only considered possible mitigations above 3700 MHz.

As discussed in Appendix E: Historical Australian WBB deployments, there have been extensive deployments of WBB services up to 3700 MHz in Australia near major airports for many years and there have been no events with an identified causal link to a WBB service.

Overseas, there are also extensive WBB deployments (up to 3800 MHz in many European countries) without specific mitigations (for example, in the UK) and there are also no specific mitigations in Japan up to 4000 MHz. It is noted, however, that the examination of overseas deployments has some limitations due to the availability of information – in most cases, base station locations, power or exact signal levels are not readily available. While Appendix F suggests that there are many deployments where there are likely to be theoretical radio altimeter issues, the information limitations mean calculation estimates are not currently practicable.

With theoretical coexistence studies indicating that there is little difference in risk between any selected frequency boundary in the 3400–4000 MHz range, this implies that if mitigations are required above 3700 MHz, it is then likely they should also be required down to 3400 MHz.

However, given there have been no events because of Australian or overseas deployments confirmed to cause interference to radio altimeters, this is considered stronger evidence than that of theoretical studies or semi-empirical studies (such as RTCA studies supported by AVSI radio altimeter measurements). That is, the contrary is also true in that if there is no evidence of issues from actual deployments up to 3700 or 3800 MHz, then the level of risk in the 3800–4000 MHz range is also likely to be low.

However, given the safety nature of this issue, we propose to be precautionary and can consider that a mitigation boundary of 3800 MHz, based on domestic and European deployments, is likely appropriate. While we acknowledge the limitations around information available for overseas deployments, the fact that there are many deployments up to 3800 MHz without specific mitigations indicate that this risk has been considered low by those administrations.

# Appendix E: Historical Australian WBB deployments

## Summary

This section examines some historical deployments of WBB in 3400–4000 MHz and summarises their operating parameters.

## Historical NBN fixed wireless access (FWA)

There are 3,250 registrations from a 2017 RRL database search of all NBN licence holdings. The registrations were between 3400–3600 MHz. 2017 represents a snapshot of the network before a defragmentation process started, where device registrations became temporarily inaccurate.

Note: TPA refers to calculated total power into antenna and power refers to recorded power values into the antenna.

- Figure 17: Location of NBN deployments in 2017

Deployment statistics for NBN FWA

| # Bandwidth Table |  |  |
| --- | --- | --- |
| Bandwidth (MHz) |  | Percentage (%) |
| 19.9 | 952 | 29.29 |
| 20 | 2298 | 70.71 |
| # Bandwidth Distribution |  |  |
| Bandwidth (MHz) |  | Percentage (%) |
| (-0.001, 5.0] | 0 | 0 |
| (5.0, 10.0] | 0 | 0 |
| (10.0, 15.0] | 0 | 0 |
| (15.0, 20.0] | 3250 | 100 |
| # Power Table |  |  |
| Power (W) |  | Percentage (%) |
| 160 | 3243 | 99.78 |
| 80 | 7 | 0.22 |
| # Power Table (dBm/MHz) |  |  |
| Power (dBm/MHz) |  | Percentage (%) |
| 39.03 | 2298 | 70.71 |
| 39.05 | 945 | 29.08 |
| 36.04 | 7 | 0.22 |
| # TPA Table |  |  |
| Total Power into Antenna (W) |  | Percentage (%) |
| 142.60 | 1728 | 53.17 |
| 152.80 | 637 | 19.60 |
| 142.60 | 569 | 17.51 |
| 139.35 | 251 | 7.72 |
| 152.81 | 59 | 1.82 |
| 76.40 | 3 | 0.09 |
| 139.35 | 2 | 0.06 |
| 87.93 | 1 | 0.03 |
| # TPA Distribution (dBm/MHz) |  |  |
| Total Power into Antenna (dBm/MHz) |  | Percentage (%) |
| 8.53 | 1728 | 53.17 |
| 8.85 | 637 | 19.60 |
| 8.53 | 569 | 17.51 |
| 8.45 | 251 | 7.72 |
| 8.85 | 59 | 1.82 |
| 5.84 | 3 | 0.09 |
| 8.45 | 2 | 0.06 |
| 6.43 | 1 | 0.03 |
| # Antenna Gains |  |  |
| Antenna Gain (dBi) |  | Percentage (%) |
| 17.5 | 2427 | 74.68 |
| 19.6 | 823 | 25.32 |
| # EIRP |  |  |
| EIRP (W) |  | Percentage (%) |
| 8019 | 1729 | 53.20 |
| 8593 | 637 | 19.60 |
| 13005 | 569 | 17.51 |
| 12709 | 251 | 7.72 |
| 8593 | 59 | 1.82 |
| 4296 | 3 | 0.09 |
| 12709 | 2 | 0.06 |
| # EIRP dBm |  |  |
| EIRP (dBm) |  | Percentage (%) |
| 69.04 | 1729 | 53.20 |
| 69.34 | 637 | 19.60 |
| 71.14 | 569 | 17.51 |
| 71.04 | 251 | 7.72 |
| 69.34 | 59 | 1.82 |
| 66.33 | 3 | 0.09 |
| 71.04 | 2 | 0.06 |
| # EIRP dBm/MHz |  |  |
| EIRP (dBm/MHz) |  | Percentage (%) |
| 56.03 | 1729 | 53.20 |
| 56.35 | 637 | 19.60 |
| 58.13 | 569 | 17.51 |
| 58.05 | 251 | 7.72 |
| 56.35 | 59 | 1.82 |
| 53.34 | 3 | 0.09 |
| 58.05 | 2 | 0.06 |
| # Antenna Height Distributions |  |  |
| Deployment Height (M) |  | Percentage (%) |
| (-0.001, 10.0] | 4 | 0.12 |
| (10.0, 20.0] | 89 | 2.74 |
| (20.0, 30.0] | 592 | 18.22 |
| (30.0, 40.0] | 1698 | 52.25 |
| (40.0, 50.0] | 766 | 23.57 |
| (50.0, 60.0] | 89 | 2.74 |
| (60.0, 70.0] | 12 | 0.37 |
| (70.0, 80.0] | 0 | 0.00 |
| (80.0, 90.0] | 0 | 0.00 |
| (90.0, 100.0] | 0 | 0.00 |

## Unwired network

Unwired was a commercial fixed wireless network using nomadic user terminals. There are 363 registrations from a 2010 database search of AKAL Pty Ltd licences, which is the appropriate licensee at the time. The registrations are between 3475–3575 MHz. These statistics do not include backhaul registrations. It is understood that this network used Ripwave MX8 and Cisco BWX8305 equipment and some statistics has been corrected to reflect this use.

Note: TPA refers to calculated ‘total power into antenna’ and power refers to reported power values.

- Figure 18: Location of Melbourne Unwired deployments in 2010

- Figure 19: Locations of airports and runways around Melbourne

- Figure 20: Location of Sydney Unwired deployments in 2010

- Figure 21: Locations of airports and runways around Sydney

Deployment statistics for Unwired network

| # Bandwidth Table |  |  |
| --- | --- | --- |
| Bandwidth (MHz) |  | Percentage (%) |
| 5 | 336 | 92.56 |
| 2 | 27 | 7.44 |
| # TPA table |  |  |
| Total Power into Antenna (W) |  | Percentage (%) |
| 0.80 | 75 | 20.66 |
| 0.05 | 32 | 8.82 |
| 0.32 | 35 | 9.64 |
| 0.50 | 88 | 24.24 |
| 0.63 | 46 | 12.67 |
| 0.13 | 32 | 8.82 |
| 0.08 | 1 | 0.28 |
| 0.01 | 12 | 3.31 |
| 0.25 | 1 | 0.28 |
| 0.00 | 14 | 3.86 |
| 0.32 | 27 | 7.44 |
| # TPA Distribution (Watts) |  |  |
| Total Power into Antenna (W) |  | Percentage (%) |
| (-0.001, 0.1] | 59 | 16.25 |
| (0.1, 0.2] | 32 | 8.82 |
| (0.2, 0.3] | 1 | 0.28 |
| (0.3, 0.4] | 62 | 17.08 |
| (0.4, 0.5] | 0 | 0.00 |
| (0.5, 0.6] | 88 | 24.24 |
| (0.6, 1.0] | 121 | 33.33 |
| # TPA Distribution (dBm/MHz) |  |  |
| Total Power into Antenna (dBm/MHz) |  | Percentage (%) |
| (-1.001, 0.0] | 14 | 3.86 |
| (0.0, 5.0] | 12 | 3.31 |
| (5.0, 10.0] | 0 | 0.00 |
| (10.0, 15.0] | 65 | 17.91 |
| (15.0, 20.0] | 36 | 9.92 |
| (20.0, 25.0] | 236 | 65.01 |
| # Antenna Gains |  |  |
| Antenna Gain (dBi) |  | Percentage (%) |
| 16 | 363 | 100 |
| # EIRP dBm |  |  |
| EIRP (dBm) |  | Percentage (%) |
| (-0.001, 20.0] | 0 | 0.00 |
| (20.0, 25.0] | 14 | 3.86 |
| (25.0, 30.0] | 12 | 3.31 |
| (30.0, 35.0] | 32 | 8.82 |
| (35.0, 40.0] | 33 | 9.09 |
| (40.0, 45.0] | 197 | 54.27 |
| (45.0, 100.0] | 75 | 20.66 |
| # EIRP dBm/MHz |  |  |
| EIRP (dBm/MHz) |  | Percentage (%) |
| (-0.001, 10.0] | 0 | 0.00 |
| (10.0, 20.0] | 14 | 3.86 |
| (20.0, 30.0] | 45 | 12.40 |
| (30.0, 35.0] | 68 | 18.73 |
| (35.0, 40.0] | 236 | 65.01 |
| # Antenna Height Distributions |  |  |
| Deployment Height (m) |  | Percentage (%) |
| (-0.001, 25.0] | 76 | 20.94 |
| (25.0, 50.0] | 209 | 57.58 |
| (50.0, 75.0] | 22 | 6.06 |
| (75.0, 100.0] | 26 | 7.16 |
| (100.0, 125.0] | 11 | 3.03 |
| (125.0, 150.0] | 6 | 1.65 |
| (150.0, 175.0] | 13 | 3.58 |
| (175.0, 1000.0] | 0 | 0.00 |

## Summary of Australian 3.4–3.7 GHz spectrum-licensed deployments (January 2023)

This is a recent summary of all spectrum-licensed base stations in 3400–3700 MHz. Only transmitters are included in the statistics. Transmitter types may be individual antenna sectors or groups of transmitters, depending on the configuration, but most will be individual sectors. These statistics include any technology operating in the band from 8,850 unique sites, but noting that some may effectively be collocated.

An overall map of sites is not presented but updated maps around representative ‘identified runways’ are included at the end of the summary.

Distribution of sites by geographic region

| Area description | Sites | Sites |
| --- | --- | --- |
| Metro | 6,600 | 74.6% |
| Regional | 2,250 | 25.4% |

#### Licensed bandwidths

The most common (approximate) licensed bandwidths:

Distribution of common licensed bandwidths

| Licensed bandwidth +-10% (MHz) | Metro | Regional | Total |
| --- | --- | --- | --- |
| 15 | 10.0% | 42.0% | 27.4% |
| 20 | 34.3% | 53.5% | 44.8% |
| 60 | 28.2% | 0.7% | 13.2% |
| 80 | 9.0% | 1.3% | 4.8% |
| 100 | 12.9% | 0.0% | 5.9% |
| Other | 5.5% | 2.4% | 3.8% |

Distribution of licensed bandwidths

| Licensed bandwidth (BW) (MHz) | Metro | Regional | Total |
| --- | --- | --- | --- |
| 0 < BW ≤ 20 | 44.3% | 95.5% | 72.2% |
| 20 < BW ≤ 40 | 3.9% | 0.9% | 2.3% |
| 40 < BW ≤ 60 | 21.9% | 1.5% | 10.8% |
| 60 < BW ≤ 80 | 15.9% | 1.7% | 8.2% |
| 80 < BW ≤ 100 | 14.0% | 0.3% | 6.5% |

#### Transmitter power into antenna

Distribution of transmitter power into antenna in watts.

Distribution of transmit power (W)

| Transmitter power into antenna (TP) (W) | Metro | Regional | Total |
| --- | --- | --- | --- |
| TP ≤ 40 | 3.5% | 1.3% | 2.3% |
| 40 < TP ≤ 80 | 19.8% | 48.9% | 35.6% |
| 80 < TP ≤ 120 | 2.0% | 0.85% | 1.4% |
| 120 < TP ≤ 160 | 23.2% | 45.2% | 35.2% |
| 160 < TP ≤ 200 | 34.1% | 3.3% | 17.3% |
| 200 < TP ≤ 320 | 8.4 % | 0.3% | 4.0% |
| TP > 320 | 9.0% | 0.1% | 4.2% |

Distribution of transmitter power density into antenna in dBm/MHz.

Distribution of transmitter power density (dBm/MHz)

| Transmitter power into antenna (TP) (dBm/MHz) | Metro | Regional | Total |
| --- | --- | --- | --- |
| TP ≤ 20 | 0.20% | 0.0% | 0.1% |
| 20 < TP ≤ 25 | 0.1% | 0.0% | 0.0% |
| 25 < TP ≤ 30 | 2.3% | 0.7% | 1.4% |
| 30 < TP ≤ 35 | 18.0% | 4.4% | 10.6% |
| 35 < TP ≤ 40 | 73.7% | 74.6% | 74.1% |
| 40 < TP ≤ 41 | 5.6% | 20.3% | 13.6% |
| TP > 41 | 0.3% | 0.1% | 0.1% |

#### Antenna gains

Distribution of antenna gains.

Distribution of antenna gains (dBi)

| Antenna gain (AG) (dBi) | Metro | Regional | Total |
| --- | --- | --- | --- |
| AG < 17 | 0.9% | 0.2% | 0.5% |
| 17 < AG ≤ 19 | 14.8% | 33.0% | 24.6% |
| 19 < AG ≤ 21 | 20.9% | 29.0% | 25.3% |
| 21< AG ≤ 23 | 8.46% | 31.6% | 20.2% |
| 23 < AG ≤ 25 | 6.16% | 6.1% | 28.7% |
| 25 < AG ≤ 25.5 | 1.2% | 0.2% | 0.6% |
| AG > 25.5 | 0.0% | 0.0% | 0.0% |

#### EIRP

Distribution of EIRP density in dBm/MHz.

Distribution of EIRP density (dBm/MHz)

| EIRP (dBm/MHz) | Metro | Regional | Total |
| --- | --- | --- | --- |
| EIRP < 40 | 0.8% | 0.00% | 0. 4% |
| 40 < EIRP ≤ 50 | 0.7% | 0.1% | 0.4% |
| 50 < EIRP ≤ 55 | 6.2% | 3.3% | 4.6% |
| 55 < EIRP ≤ 60 | 73.8% | 94.1% | 814.8% |
| 60 < EIRP ≤ 62.5 | 17.1% | 2.2% | 9.0% |
| 62.5 < EIRP ≤ 65 | 1.2% | 0.3% | 0.7% |
| EIRP > 65 | 0.2% | 0.0% | 0.1% |

#### Deployment heights

Distribution of antenna heights.

Distribution of antenna heights (m)

| Antenna heights (AH) (m) | Metro | Regional | Total |
| --- | --- | --- | --- |
| AH ≤ 10 | 0.9% | 0.0% | 0.4% |
| 10 < AH ≤ 20 | 11.5% | 3.1% | 6.9% |
| 20 < AH ≤ 30 | 28.9% | 22.8% | 25.4% |
| 30 < AH ≤ 40 | 39.3% | 50.1% | 45.2% |
| 40 < AH ≤ 50 | 16.9% | 20.8% | 19.2% |
| 50 < AH ≤ 60 | 2.0% | 2.6% | 2.4% |
| AH > 60 | 0.4% | 0.5% | 0.5% |

#### Geographical area description

Metro: covers all capital cities (except Darwin and Hobart).

- Figure 22: Geographic area descriptions

#### Deployments around example airports that have identified runways

- Figure 23: Deployments near Perth Airport

- Figure 24: Deployments near Brisbane Airport

- Figure 25: Deployments near Sydney Airport

- Figure 26: Deployments near Melbourne Airport

- Figure 27: Deployments near Adelaide Airport

# Appendix F: Relevant overseas WBB deployments

This section examines overseas WBB deployments in bands near to the radio altimeter bands, where theoretical issues with radio altimeters could be expected.

## Japan

The following table summarises the frequency allocations in Japan in the bands adjacent to the radio altimeter band:

#### Overall deployments

Overall deployment statistics can be found online in Japanese. As of March 2021 there have been >5,600 outdoor base station deployments, but this would have increased significantly since then.

#### NTT Docomo

Docomo have been allocated the 3.6–3.7 GHz and 4.5–4.6 GHz frequency ranges. There are no specific mitigations in these frequency ranges. Coverage maps can be found on their website.

Typical coverage is show below (taken from around Tokyo for December 2022). The dark pink colour is ‘sub 6 GHz’ coverage, which for Docomo is either the 3600–3700 MHz and 4500–4600 MHz ranges, both with potential for interference to the radio altimeter 4200–4400 MHz range.

- Figure 28: NTT Docomo 5G deployments example

- Figure 29: 5G coverage for these ranges near Tokyo Narita Airport – pink areas are the 3.7/4.5 GHz ‘sub 6GHz’ coverage areas:

- Figure 30: 5G coverage for these ranges near Tokyo Haneda Airport:

- Figure 31: 5G coverage for these ranges near Kansai Airport for Osaka:

- Figure 32: 5G coverage for these ranges near Fukuoka Airport:

In all 4 examples, there are Docomo 3.6–3.7 or 4.5–4.6 GHz 5G base stations providing coverage across major airport runways. While exact base station locations and signal levels are unknown, they would suggest potential theoretical problems for radio altimeters.

#### Softbank

Similar maps are reproduced below for Softbank, who have the highest allocation without specific mitigations (3.9–4.0 GHz). Coverage maps can be found via their Japanese website.

Typical coverage is show below (taken from around Tokyo for November 2022). The dark pink colour is for the 3900–4000 MHz. Softbank coverage in this range does not appear to be particularly extensive.

- Figure 33: Softbank 5G coverage example

- Figure 34: Softbank have no 3.7 GHz band coverage around Tokyo Narita Airport

- Figure 35: Around Tokyo Haneda

- Note the relevant colour is related to 5G <3.7 GHz>

- Figure 36: Around Kansai Airport

- Figure 37: Softbank have no relevant coverage around Fukuoka Airport

In 2 of the 4 examples (Haneda, Kansai), there are Softbank 3.9–4.0 5G base stations providing coverage across major airport runways.

## Denmark

The Yousee operator is the commercial arm of TDC Net in Denmark, discussed in Appendix C: International developments and approaches. While their publicly available coverage tool does not specifically identify the 3.5 GHz band, they expect users can obtain 200–500 Mbit/s around Copenhagen Airport. This suggests strong 3.5 GHz 5G coverage there, which has no specific mitigations for radio altimeters.

- Figure 38: Denmark 5G example

## Finland

Finland has 5G in various bands, including 700 MHz, 2 GHz, 2.6 GHz, 3410–3800 MHz and 26 GHz.  However, 700 and 3410–3800 MHz were their first 5G bands. The regulator has a public interactive map. It indicates a 5G throughput of > 300 Mbps being available around Helsinki Airport, again suggesting strong 3400–3800 MHz coverage is likely there.

- Figure 39: Finland 5G example

- Helsinki Airport is within the blue circle.

## Switzerland

As per 7.37, Switzerland has 5G in the 700 MHz and 3500–3800 MHz bands. The regulator has a public database with indicative information for base station location and approximate power level. An example around Geneva airport for 5G base stations is below:

- Figure 40: Switzerland 5G example GeneVA

- While the database does not indicate the exact frequency band, given there is 300 MHz available in 3500–3800 MHz, compared with only 2 x 30 MHz UL/DL in the 700 MHz band, most base stations in urban areas would be expected to be operating in the 3500–3800 MHz range. The highlighted base station is one of several that is close to the runway, and is in the ‘high’ ERP category, which is > 1kW.

- Similarly, Zurich Airport is below. Note that in both cases, the Swiss OFCOM determined that analysis indicated that none of these base stations presented a problem for radio altimeters.

- Figure 41: Switzerland 5G example Zurich

## Ireland

The Irish regulator has allocated 3410–3435 MHz/3475–3800 MHz for 5G services and is in the process of considering the 700 MHz band and mmWave bands.  Consequently, the 3410–3800 MHz range is the only range currently used for 5G.

The information on the allocation, including technical conditions, does not appear to include any specific mitigations for radio altimeters.

#### Three

Three Ireland were allocated the 3700–3800 MHz range.

There is some Three coverage around Dublin Airport, via their coverage tool (note that the regulator coverage tool does not yet have 5G):

- Figure 42: Ireland 5G example Dublin

- Figure 43: Ireland 5G example Cork

The regulator does offer a site viewer tool. While the tool does indicate 5G (‘NR’) transmissions from some sites (for example, site CKCKA in the link example near Cork Airport for Vodaphone or Site 3_CK0370 for Three), they are all at a fair distance from the runways.

#### Meteor (Eir Mobile)

Meteor Ireland were allocated the 3620–3700 MHz range.

While their coverage tool was non-functional at the time of query, the regulator site-viewer indicated a 5G base station at Dublin Airport:

- Figure 44: Meteor 5G example

## United Kingdom

As previously discussed, Ofcom in the UK has allocated the 3.6–3.8 GHz range for 5G (as well as 700 MHz) and 3.8–4.2 GHz for medium/low power deployments.

Ofcom does not release transmission site location information for national security reasons but does offer a general mobile coverage tool. Like in other examples, the coverage tool does not differentiate between 700 MHz and 3.6 GHz 5G, but in most urban areas, it is expected that the 3.6 GHz band is the predominant use.

#### Vodaphone

Vodaphone were allocated the 3720–3760 MHz frequency range.

- Figure 45: Vodaphone 5G example Heathrow

- Figure 46: Vodaphone 5G example Gatwick

- Figure 47: Vodaphone 5G example London City

- Figure 48: Vodaphone 5G example Manchester

# Appendix G: Identified runways

This section outlines a draft initial lists where the interim mitigations are proposed to apply. Note that Norfolk Island is not part any formal WBB allocation in the band, but point-to-multipoint (PMP) apparatus licences are available upon application. This list have been provided by CASA and may be updated from time to time. Note that the list contained in draft RALI MS47 is a subset, representing the approaches applicable in remote areas only.

The details of the current locations (4) where an Instrument Landing System (ILS) (8 runways with 20 IAPs) is installed that requires radio altimeter use are:

List of identified runway approaches where ILS IAPs require use of radio altimeters

| Identifier | Location | Runway | Approach(s) | Number of IAPs |
| --- | --- | --- | --- | --- |
| YSCB | Canberra | 35 | ILS-Z SA CAT I, ILS-Z SA CAT II, ILS-Z CAT II | 3 |
| YMML | Melbourne | 16 | ILS-Z CAT II, ILS-Z CAT IIIa, ILS-Z CAT IIIb | 3 |
| YPPH | Perth | 03 | ILS-Z SA CAT I | 1 |
| YPPH | Perth | 21 | ILS-Z SA CAT II, ILS-Z CAT II, ILS-Z CAT IIIb | 3 |
| YSSY | Sydney | 16L | ILS SA CAT II, ILS SA CAT II | 2 |
| YSSY | Sydney | 16R | ILS SA CAT I, ILS SA CAT II, ILS CAT II | 3 |
| YSSY | Sydney | 34L | ILS SA CAT I, ILS SA CAT II, ILS CAT II | 3 |
| YSSY | Sydney | 34R | ILS SA CAT I, ILS SA CAT II | 2 |

The details of the current locations (17) where a Required Navigation Performance – Authorisation Required (RNP-AR) approach (32 runways with 57 IAPs) is published that requires radio altimeter use are:

Identified runway approaches where RNP-AR IAPs require radio altimeter use

| Identifier | Location | Runway | Approach(s) | Number of IAPs |
| --- | --- | --- | --- | --- |
| YMAV | Avalon | 36 | RNP U RWY 36 (AR) | 1 |
| YPAD | Adelaide | 05 | RNP W, X & Y (AR) | 3 |
| YBAS | Alice Springs | 12 | RNP M, P & U (AR) | 3 |
| YBAS | Alice Springs | 30 | RNP M, P & U (AR) | 3 |
| YBNA | Ballina | 06 | RNP X RWY 06 (AR) (0.3 and 0.1) | 1 |
| YBNA | Ballina | 24 | RNP X RWY 24 (AR) (0.3 and 0.1) | 1 |
| YBBN | Brisbane | 01L | RNP X RWY 01L (AR) (0.3 and 0.15) | 1 |
| YBBN | Brisbane | 01R | RNP M & X RWY 01R (AR) (0.3) | 2 |
| YBBN | Brisbane | 19L | RNP X RWY 19L (AR) (0.3) | 1 |
| YBBN | Brisbane | 19R | RNP X RWY 19R (AR) (0.3) | 1 |
| YBWW | Wellcamp | 12 | RNP X RWY 12 (AR) (0.3 and 0.23) | 1 |
| YBRM | Broome | 10 | RNP U RWY 10 (AR) (0.3) | 1 |
| YBRM | Broome | 28 | RNP U RWY 28 (AR) (0.3 and 0.1) | 1 |
| YBCS | Cairns | 15 | RNP W & X RWY 15 (AR) (0.3) | 2 |
| YBCS | Cairns | 33 | RNP W, X & Y RWY 33 (AR) (0.3) | 3 |
| YSCB | Canberra | 17 | RNP W, X & Y RWY 17 (AR) (0.3 and 0.11) | 3 |
| YSCB | Canberra | 35 | RNP W, X & Y RWY 35 (AR) (0.3 and 0.11) | 3 |
| YPDN | Darwin | 11 | RNP Y & X RWY 11 (AR) (0.3 and 0.15) | 2 |
| YPDN | Darwin | 29 | RNP Y & X RWY 29 (AR) (0.3 and 0.15) | 2 |
| YBCG | Gold Coast | 14 | RNP Y & W RWY 14 (AR) | 2 |
| YBCG | Gold Coast | 32 | RNP X & Y RWY 32 (AR) | 2 |
| YBHM | Hamilton Is | 14 | RNP P, U, W & Y RWY 14 (AR) | 4 |
| YBHM | Hamilton Is | 32 | RNP U, W & X RWY 32 (AR) | 3 |
| YMML | Melbourne | 16 | RNP M & P RWY 16 (AR) (0.3 and 0.11) | 2 |
| YBMA | Mount Isa | 16 | RNP U RWY 16 (AR) | 1 |
| YBMA | Mount Isa | 34 | RNP U RWY 34 (AR) | 1 |
| YSNF | Norfolk Is | 11 | RNP X RWY 11 (AR) (0.3, 0.2 and 0.1) | 1 |
| YSNF | Norfolk Is | 29 | RNP X RWY 29 (AR) (0.3, 0.2 and 0.1) | 1 |
| YBSU | Sunshine Coast | 13 | RNP W RWY 13 (AR) (0.3) | 1 |
| YBSU | Sunshine Coast | 31 | RNP W & X RWY 31 (AR) (0.3) | 2 |
| YBTL | Townsville | 01 | RNP P RWY 01 (AR) | 1 |
| YBTL | Townsville | 19 | RNP P RWY 19 (AR) | 1 |

The above locations as provided by CASA are current as of 01/12/22.

As described in the Proposed spectrum re-allocation for the 3.4 GHz and 3.7 GHz bands outcomes paper, the current implementation expectation is that there will be a separation of 250 MHz to high-powered WBB in metro and regional areas, where the 3950-4000 MHz segment will be lower-powered restricted cell use, and 200 MHz in remote areas.

Given the lack of availability of non-anonymised detailed RA device data and being able to associate them to specific aircraft, it is not possible to be more definitive on the scale of the extent of poorer performing RAs.

See the RTCA, AVSI and Japanese ‘blackbox’ testing in appendices B and C.

The ACMA proposes to adopt the approach, currently to be specified via RALI MS47, and referenced as required in any relevant legislative instruments. More detail is provided in Appendix D.

Current implementation expectations are that there will be a separation of 250 MHz to high-powered WBB deployments in metropolitan and regional areas, and 200 MHz in remote areas.

Note that the ongoing unwanted emissions limits for practical antenna systems may meet the interim EIRP-based mitigations for parts of the 4200–4400 MHz range.

We are, however, aware of activity raising the difficulty of meeting this date and the subsequent revision to 1 February 2024 by a proposed new Airworthiness Directive.

Also see paragraph 7.103 for a discussion of the other work of SC.239 in relation to RA standards development.

That is, there were no qualitative or quantitative events recorded for usage category 1 RAs models that were tested.

The ACMA acknowledges these are ACMA summaries/interpretations of perceived sectorial views and may not be precisely stated by any single party.

Noting that WBB can operate under fixed or mobile allocations without an IMT identification.

Given the lack of availability of detailed RA device data and being able to associate them to specific aircraft, it is not possible to be more definitive on the scale of the extent of poorer performing RAs.

See the RTCA, AVSI and Japanese ‘blackbox’ testing in appendices B and C.

Current implementation expectations are that there will be a separation of 250 MHz to high-powered WBB in metro and regional areas, where the 3950–4000 MHz segment will be lower-powered restricted cell use and 200 MHz in remote areas.

Note that the ongoing unwanted emissions limits for practical antenna systems may meet the interim EIRP based mitigations for parts of the 4200–4400 MHz range.

3750 MHz was used as a comparison against French and Canadian examples only and does not represent the frequency range where we propose mitigations.

For example, if a base station is registered across 3800–3900 MHz, then the applicable limit is 5.1 dBm/MHz.

If the runway width is greater than 24 metres, then the width is equal to the width of the runway.

Much of this material is sourced or based on the RTCA Report (Section 1.1).

Usage Category 1, covering commercial air transport airplanes, both single-aisle and wide-body; Usage Category 2, covering all other fixed-wing aircraft not included in Usage Category 1, including regional, business aviation, and general aviation airplanes; and Usage Category 3, covering both transport and general aviation helicopters.

See (3) Conclusions of the April 2022 ICAO FSMP CG-RA report.

Detailed in an Airbus presentation to the APT-ICAO web dialogue on radio altimeter coexistence on 23 August 2022 and confirmed in the proposed revised FAA AD discussed from paragraph 8.25.

ACMA note: effectively the ‘5G mitigated airports’ where telecommunications companies have agreed to voluntarily limit their 5G deployment at the request of the FAA.

Available in French and Dutch only.

For example, see the GSMA report, 5G-Aviation-Altimeters.pdf (gsma.com).

As expected, as the RA self-signal is generally high so the noise floor does not need to be protected.

Note the ANFR describes their restricted zones as ‘zone de precaution’ and exclusion zones as ‘zone de securite’.

Numerical figures are for a comparative example with French calculations at similar frequencies only.

If the runway width is greater than 24 metres, then the width is equal to the width of the runway.