# Planning Model for Low Power Open Narrowcasting (LPON) Services

Frequency Range

The FM radio sub-band 87.5-88.0 MHz has been made available to accommodate LPON services.

Channels

The channels centred on the frequencies 87.6 MHz, 87.8 MHz and 88.0 MHz may be used for LPON services.

The order of frequency assignment in an area is 88.0 MHz and 87.6 MHz, in either order, followed by 87.8 MHz; co-channel and other frequency availability restrictions permitting.

Frequency Availability Restrictions

There are restrictions on the availability of all, or some, of these frequencies in certain areas of Australia because of the potential for interference to the reception of broadcasting services.

FM Radio Broadcasting Services below 88.4 MHz

Table 1 lists the locations and protection radii for FM radio broadcasting transmitters using 88.1 MHz and 88.3 MHz.

Important Note:

- within the radii for 88.1 MHz transmitters, no LPON services are to be assigned.

- within the radii for 88.3 MHz transmitters, no 87.8 MHz or 88.0 MHz LPON services are to be assigned; and

- within the radii for 88.3 MHz transmitters, LPON services to operate on or below 87.6 MHz may be assigned.

Table 1 - FM Broadcasting Services on 88.1 MHz and 88.3 MHz

| Frequency (MHz) | FM Radio Callsign | Site Coordinates (AMG66) |  |  | Area Covered | Effective Radiated Power | Protection Radius (kilometres) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Zone | Easting | Northing |  |  |  |
| 88.1 MHz | 2ABCFM | 55 | 601300 | 5991800 | Khancoban, NSW | 36 watts | 30 |
|  | 2ABCFM | 55 | 616600 | 6060000 | Talbingo, NSW | 4 watts | 10 |
|  | 2BS | 55 | 726726 | 6251735 | Burraga, NSW | 2000 watts | 35 |
|  | 2RDJ | 56 | 324600 | 6249200 | Burwood, NSW | 100 watts | 50 |
|  | 2RVR/T | 55 | 314920 | 6185673 | Hay, NSW | 6000 watts | 100 |
|  | 3ABCFM | 55 | 497499 | 5936495 | Bright, VIC | 50 watts | 30 |
|  | 3MFM | 55 | 392200 | 5739300 | Leongatha, VIC | 2000 watts | 80 |
|  | 4HJ/T1 | 55 | 555200 | 7490300 | Blair Athol, QLD | 200 watts | 50 |
|  | 8ABC | 52 | 720776 | 8480826 | Tipperary Station via Daly River, NT | 2 watts | 10 |
|  | 8ABCRN | 53 | 234400 | 8599900 | South Alligator, NT | 2 watts | 50 |
| 88.3 MHz | 2ABCFM | 55 | 582311 | 6146329 | South West Slopes, NSW | 80,000 watts | 100 |
|  | 2UUS | 56 | 284623 | 6219812 | Camden, NSW | 100 watts | 25 |
|  | 3SCB | 55 | 327260 | 5799540 | Southern South Melbourne, VIC | 200 watts | 30 |
|  | 4ABCRR | 56 | 360282 | 7310324 | Miriam Vale, QLD | 100 watts | 30 |
|  | 3ABCRN | 54 | 616375 | 6031400 | Hopetoun, VIC | 500 watts | 30 |
|  | 6MER | 50 | 618547 | 6403429 | Kondinin, WA | 25 watts | 30 |
|  | 6FMS | 50 | 682335 | 7442632 | West Angelas Mine Dep B, WA | 65 watts | 30 |

# LPON Transmitter Technical Characteristics

Radiated Power

Table 2

| Location* | Maximum Equivalent Radiated Power (ERP) |
| --- | --- |
| Residential | 1 Watt (1.64 watts Equivalent Isotropic Radiated Power) |
| Non-residential | 10 Watts (16.4 watts Equivalent Isotropic Radiated Power) |

* A residential area is an area within 20 kilometres of the boundary of an Urban Centre or Locality as defined by the Australian Bureau of Statistics (ABS) in the 2021 Census. A complete list of such areas can be found on the ABS website at Urban Centre Localities page.

A non-residential area is an area at least 20 kilometres from the boundary of an Urban Centre or Locality.

Antenna

Type - Unspecified

Polarisation – Unspecified

Emission Designation

- 200KF8EHF for stereophonic or quadraphonic sound;

- 200KF3EGF monophonic sound.

Minimum Separation Distance Requirements

Co-channel and adjacent channel minimum separation requirements for LPON services are listed in Table 3.

Table 3 - Minimum Separation Distances between LPON Transmitters

In addition to conforming to the planning requirements above, LPON transmitters must maintain the minimum separation distances specified below. For example, a one watt ERP transmitter operating at 87.6 MHz must be situated at least 10km from the nearest one watt ERP transmitter operating at 87.6 MHz and at least 5km from the nearest one watt ERP transmitter operating at 87.8 MHz. It can operate at the same site as a transmitter operating at 88.0 MHz (see column 2).

| LPON Carrier Frequency Separation (kHz) | Separation Distance between 1 watt ERP LPON transmitters in kilometres (km) | Separation Distance between 10 watt ERP LPON transmitters (km) | Separation Distance between 1 watt and 10 watt ERP LPON transmitters (km) |
| --- | --- | --- | --- |
| 0 | 10 | 30 | 20 |
| 200 | 5 | 15 | 10 |
| 400 | 0 | 0 | 0 |

Translator