06 February 2026

Ms Nerida O'Loughlin PSM

Chair and Agency Head

Australian Communications and Media Authority

PO Box 78

BELCONNEN ACT 2616

Dear Ms O’Loughlin,

Re: Comments on the Additional Assurance Measures for the Coexistence of Wireless Broadband Services and Radio Altimeters Consultation Paper – December 2025

The Australian Airline Pilots’ Association (AusALPA) represents more than 7,100 professional pilots within Australia on safety and technical matters. AusALPA is also an active member of the global pilot body, the International Federation of Airline Pilots’ Associations (IFALPA), which represents over 100,000 airline pilots internationally. Our membership places a very strong expectation on our government agencies and their processes to demonstrate rational, risk- and evidence-based safety behaviour. We regard our participation in the safety-related work of Australia’s government agencies as essential to ensuring that our policymakers receive the best independent safety and technical advice.

AusALPA was actively engaged in both the Australian Communication and Media Authority (ACMA) Technical Liaison Group (TLG) and the CASA Radio Altimeter Coordination Group (RA-CG), resulting in the implementation of interim mitigations to coexist with RAs and Wireless Broadband (WBB) in June 2023.

AusALPA, along with the Australian Airports Association (AAA), Airline Operators and Aircraft Manufacturers, are united in expressing our concern over the ACMA’s proposed voluntary mitigation measures. The potential interference of aircraft Radio Altimeters by 5G transmissions are a “Safety of Life” issue and continue to have direct consequences on both our members and the travelling public.

AusALPA supports the consolidated submission made by the AAA CEO, Simon Westaway, to ACMA and the letter to the Civil Aviation Safety Authority (CASA). In addition, the Association would like to reiterate the following points:

- The maximum Effective Isotropic Radiated Power (EIRP) of any WBB installation should be limited to 62 dBm/MHz (which is higher than that used in Australia at present), or any lesser value determined acceptable by the FAA, until such time as the Radio Technical Commission for Aeronautics (RTCA) Minimum Operational Performance Standards (MOPS) have been published and Radio Altimeters (RA) have been either modified or retrofitted accordingly.

- Most 3-dimensional (3D) approaches (ILS CAT I being an exception) rely on the RA input, and therefore, it is difficult to ensure that all approaches are included on the list being produced for Australian airports (see RA-CG submission). Furthermore, new approaches are continuously being designed and introduced at various airports. It is therefore essential that the mitigation measures allow for this and that ACMA establishes a process for dealing with new approaches and the deployment of new WBB installations.

- In addition to the potential adverse effects on the approaches during low visibility conditions, the RA inputs are continuously used by aircraft systems for both landing and take-off. An example is the deployment of reversers and the activation of autobrake during landing. Consequently, any possible disruption to RA integrity by WBB installations will affect aircraft performance (increased landing distances and unable to land/take-off on the runway available in the ambient conditions), which will then cause operational limitations (landing and take-off weights; diversions; delays; cancellations, etc.), leading to financial and social loss.

- To date, the focus has been primarily on protecting operations into large airports, but as the RACG coordinated response highlights, this does not protect emergency and medical aircraft operations, especially when outside the “protected zones”. The RA-CG response details the systems used by these aircraft, whilst they carry out their functions, many of which involve saving or safeguarding life.

- AusALPA agrees with Boeing that WBB installations operating below 3700 MHz are in no way vindicated by the “lack of evidence”. As Altman and Bland state “Absence of evidence is not evidence of absence”.

If it is not possible to establish whether there is interference or if interference is being caused by WBB installations (4G or 5G), the responsible approach is to adopt the mitigation measures for all WBB installations operating at frequencies above 3700MHz.

In summary:

- AusALPA supports the consolidated submission by the AAA and thanks ACMA for the opportunity to submit our comments.

- AusALPA believes that the current mitigations that are in place should remain mandatory until such time that RTCA MOPS have been published and Australian operators have had time to retrofit their Radio Altimeters to the new technical standard.

- AusALPA remains concerned that potential interference of RAs will negatively impact the take-off and landing performance of some aircraft on any runway regardless of the environmental conditions.

- AusALPA remains concerned that emergency and medical operations are unprotected when operating outside the proposed protection zones.

- AusALPA shares the concerns expressed by Boeing that the proposed mitigations should apply to all WBB installations operating at frequencies above 3700 MHz.

Yours sincerely,

Captain Andrew Marshall				Captain Ben Bollen

President						Vice President

AusALPA						AusALPA

Tel:	61 – 2 – 8307 7777

Email:	office@ausalpa.org.au

ACMA Consultation Paper - Additional assurance measures for the coexistence of wireless broadband services and radio altimeters, December 2025

Altman DG, Bland JM. Absence of evidence is not evidence of absence. BMJ 1995;311: 485